3GPP TSG-RAN5 Meeting #99	R5-232184

# Incheon, South Korea, 22nd-26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3667 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to MAC test case for RA Based SDT / 4-step RACH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Additional TP for testing condition RSRP is below the configured sdt-RSRP-Threshold needs to be added |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added TP4 for condition RSRP is below the configured sdt-RSRP-Threshold and its associated test sequence is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE interoperability cannot be guaranteed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.1.1.13.2	RA Based SDT / 4-step RACH / Successful

7.1.1.13.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is not configured }

ensure that {

when { UE has small data to transmit and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold and RSRP is above the configured sdt-RSRP-Threshold }

then { UE shall initiate 4-step RA based SDT procedure }

}

(2)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is not configured }

ensure that {

when { UE has small data to transmit and the data volume of the pending UL data across all RBs configured for SDT is greater than sdt-DataVolumeThreshold and RSRP is above the configured sdt-RSRP-Threshold }

then { UE shall not initiate RA based SDT procedure and starts normal RRC Resume procedure }

}

(3)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is configured and Random Access resources for RA-SDT is configured }

ensure that {

when { UE initiates RA based SDT procedure }

then { UE is successfully able to send and receive subsequent SDT data }

}

(4)
with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is not configured}
ensure that {
  when { UE has small data to transmit and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold and RSRP is below the configured sdt-RSRP-Threshold}
    then { UE shall not initiate RA based SDT procedure and starts normal RRC Resume procedure }
            }

7.1.1.13.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321, clause 5.1.1b, 5.1.1c and 5.27. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.1.1b]

The MAC entity shall:

1>	if the BWP selected for Random Access procedure is configured with both set(s) of Random Access resources with MSG3 repetition indication and set(s) of Random Access resources without MSG3 repetition indication and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg3; or

1>	if the BWP selected for Random Access procedure is only configured with the set(s) of Random Access resources with MSG3 repetition indication:

2>	assume MSG3 repetition is applicable for the current Random Access procedure.

1>	else:

2>	assume MSG3 repetition is not applicable for the current Random Access procedure.

NOTE 1:	Void.

1>	if contention-free Random Access Resources have not been provided for this Random Access procedure and one or more of the features including RedCap and/or a specific NSAG(s) and/or SDT and/or MSG3 repetition is applicable for this Random Access procedure:

NOTE 2: The applicability of SDT is determined by MAC entity according to clause 5.27. The applicability of specific NSAG(s) is determined by upper layers when the Random Access procedure is initiated. The applicability of RedCap is also determined by upper layers when Random Access procedure is initiated and it is applicable to the Random Access procedures initiated by PDCCH orders and any Random Access procedure initiated by the MAC entity.

2>	if none of the sets of Random Access resources are available for any feature applicable to the current Random Access procedure (as specified in clause 5.1.1c):

3>	select the set(s) of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else if there is one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure:

3>	select this set of Random Access resources for this Random Access procedure.

2>	else (i.e. there are one or more sets of Random Access resources available that are configured with indication(s) for a subset of all features triggering this Random Access procedure):

3>	select a set of Random Access resources from the available set(s) of Random Access resources based on the priority order indicated by upper layers as specified in clause 5.1.1d for this Random Access Procedure.

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure and RedCap is applicable for the current Random Access procedure and there is one set of Random Access resources available that is only configured with RedCap indication:

2>	select this set of Random Access resources for this Random Access procedure.

1>	else:

2>	select the set of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for the current Random Access procedure.

[TS 38.321, clause 5.1.1c]

The MAC entity shall for each set of configured Random Access resources for 4-step RA type and for each set of configured Random Access resources for 2-step RA type:

1>	if RedCap indication is configured for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a Random Access procedure for which RedCap indication is not applicable.

1>	if SDT indication is configured for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure which is not triggered for SDT.

1>	if NSAG indication is configured for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure unless it is triggered for the corresponding NSAG indication.

1>	if MSG3 repetition indication is configured for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure if MSG3 repetition is not applicable.

1>	if a set of Random Access resources is not configured with any of the RedCap or SDT or NSAG(s) or MSG3 repetition indications:

2>	consider the set of Random Access resources to not associated with any feature indication.

[TS 38.321, clause 5.27]

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer. The SDT procedure can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT).

…

If RA-SDT is selected above and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI received in random access response until the RA-SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI as stored in UE Inactive AS context as specified in TS 38.331 [5] and CS-RNTI until the CG-SDT procedure is terminated.

7.1.1.13.2.3	Test description

7.1.1.13.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A and Test Mode Activated according to 38.508-1 [4] Table 4.4A.2-1 with UE test loop mode B is established IP PDU delay set to 6 seconds. IF pc_logicalChannelSR_DelayTimer the DRB is configured for SDT operation according to Table 7.1.1.13.2.3.1-1.

Table 7.1.1.13.2.3.1-1: Logical Channel Configuration Settings

| Parameter | SDT DRB |
| --- | --- |
| logicalChannelSR-DelayTimerApplied | True |
| logicalChannelSR-DelayTimer | sf512 |

7.1.1.13.2.3.2	Test procedure sequence

Table 7.1.1.13.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 2 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT( SDT Data > sdt-DataVolumeThreshold). | <-- | MAC PDU | - | - |
| 3 | The SS transmits an RRCRelease message including both full-RNTI and short-RNTI in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 5 | Check: Does the UE transmit a preamble on PRACH? | --> | PRACH Preamble | - | - |
| 6 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 5, including TC-RNTI and not including Back off Indicator subheader. | <-- | Random Access Response |  |  |
| 7 | Check: Does the UE transmit a MAC PDU containing an RRCResumeRequest message? | --> | MAC PDU ( NR RRC: : RRCResumeRequest) | 2 | P |
| 8 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (UE Contention Resolution Identity MAC CE) |  |  |
| 9 | The SS transmits a RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| - | Exception: Step 10 and 11 can happen in any order | - | - | - | - |
| 10 | The UE transmits a RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 11 | Check: Does the UE transmit a MAC PDU containing Loop backed PDU? | --> | MAC PDU (containing 1 MAC sub PDU containing RLC SDU) | 2 | P |
| 12 | The SS transmits an OPEN UE TEST LOOP message. | <-- | TC: OPEN UE TEST LOOP | - | - |
| 13 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 14 | SS transmits a CLOSE UE TEST LOOP message. | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 15 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 16 | SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 17 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT( SDT Data < sdt-DataVolumeThreshold).. | <-- | MAC PDU | - | - |
| 18 | The SS transmits a RRCRelease message including both fulI-RNTI and short-RNTI in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 19 | Check: Does the UE transmit a preamble on PRACH? | --> | PRACH Preamble | - | - |
| 20 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 17, including TC-RNTI and not including Back off Indicator subheader. | <-- | Random Access Response |  |  |
| 21 | Check: Does the UE transmit a MAC PDU containing an RRCResumeRequest message and RLC PDU on DRB with SDT configured? | --> | MAC PDU ( NR RRC: : RRCResumeRequest, RLC PDU on DRB with SDT configured) | 1 | P |
| 22 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (UE Contention Resolution Identity MAC CE) |  |  |
| - | EXCEPTION: Steps 22a1-22a3 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if pc_logicalChannelSR_DelayTimer is configured | - | - | - | - |
| 22a1 | IF pc_logicalChannelSR_DelayTimer THEN SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT (SDT Data <= sdt-DataVolumeThreshold). | <-- | MAC PDU |  |  |
| 22a2 | SS transmits an UL Grant, allowing the UE to return the RLC SDU as received in step 17a, on PDCCH with the C-RNTI assigned to the UE. | <-- | (UL Grant (C-RNTI)) | - | - |
| 22a3 | Check: Does the UE transmit a MAC PDU including one RLC SDU? | --> | MAC PDU | 3 | P |
| 23 | The SS changes the parameter ‘sdt-RSRP-Threshold-r17’ in SIB1 of NR Cell 1 to 76 and starts broadcasting updated SIB1. Note: This value should result in meeting condition ‘RSRP is below the configured sdt-RSRP-Threshold’ | - | - | - | - |
| 24 | The SS transmits an OPEN UE TEST LOOP message. | <-- | TC: OPEN UE TEST LOOP | - | - |
| 25 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 26 | SS transmits a CLOSE UE TEST LOOP message. | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 27 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 28 | SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 29 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT (SDT Data > sdt-DataVolumeThreshold). | <-- | MAC PDU | - | - |
| 30 | The SS transmits a RRCRelease message including both fulI-RNTI and short-RNTI in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 31 | The UE transmit a preamble on PRACH? | --> | PRACH Preamble | - | - |
| 32 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 5, including TC-RNTI and not including Back off Indicator subheader. | <-- | Random Access Response |  |  |
| 33 | Check: Does the UE transmit a MAC PDU containing an RRCResumeRequest message? | --> | MAC PDU ( NR RRC: : RRCResumeRequest) | 4 | P |
| 34 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (UE Contention Resolution Identity MAC CE) |  |  |
| 35 | The SS transmits a RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 36 | Exception: Step 10 and 11 can happen in any order | - | - | - | - |
| 37 | The UE transmits a RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 38 | Check: Does the UE transmit a MAC PDU containing Loop backed PDU? | --> | MAC PDU (containing 1 MAC sub PDU containing RLC SDU) | 2 | P |
| 39 | The SS transmits a RRCRelease message | <-- | NR RRC: RRCRelease | - | - |

7.1.1.13.2.3.3	Specific message contents

Table 7.1.1.13.2.3.3-1: CLOSE UE TEST LOOP (Step 14, Preamble Table 7.1.1.13.2.3.2-1 )

| Derivation Path: TS 36.508-1 [7] table 4.7A-3 condition UE test loop mode B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 1111'B | 15 seconds |  |  |

Table 7.1.1.13.2.3.3-2: SIB 1 (preamble and all steps, Table 7.1.1.13.2.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.6.1-28 with Condition SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommon | Table 7.1.1.13.2.3.3-3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sdt-LogicalChannelSR-DelayTimer-r17 | sf512 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.2.3.3-3: ServingCellConfigCommon (7.1.1.13.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon | Table 7.1.1.13.2.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.2.3.3-4: BWP-UplinkCommon (Table 7.1.1.13.2.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| AdditionalRACH-ConfigList-r17 SEQUENCE { |  |  |  |
| AdditionalRACH-Config-r17 SEQUENCE { |  |  |  |
| rach-ConfigCommon-r17 | RACH-ConfigCommon | Table 7.1.1.13.2.3.3-5 |  |
| ) |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.2.3.3-5: RACH-ConfigCommon (Table 7.1.1.13.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| featureCombinationPreamblesList-r17 SEQUENCE { |  |  |  |
| FeatureCombinationPreambles-r17 | FeatureCombinationPreambles | Table 7.1.1.13.2.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.2.3.3-6: FeatureCombinationPreambles (Table 7.1.1.13.2.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-56E |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| featureCombination-r17 ::= SEQUENCE { |  |  |  |
| smallData-r17 | true |  |  |
| } |  |  |  |
| startPreambleForThisPartition-r17 | 8 | Randomly selected |  |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 12 |  |  |
| ssb-SharedRO-MaskIndex-r17 | Not present |  |  |
| groupBconfigured-r17 | Not present |  |  |
| separateMsgA-PUSCH-Config-r17 | Not present |  |  |
| msgA-RSRP-Threshold-r17 | Not present |  |  |
| rsrp-ThresholdSSB-r17 | Not present |  |  |
| deltaPreamble-r17 | Not present |  |  |
| } |  |  |  |