3GPP TSG-RAN5 Meeting #99	R5-232321

# Incheon, Korea (Republic of), 22nd May 2023 – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5449 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to receiver reference sensitivity test cases as part of introduction of LTE Band 54 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_TDD_1670_1675MHz-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of LTE TDD Band 54 to conformance specifications |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant reference sensitivity clauses have been updated to introduce Band 54 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE conformance test specifications for LTE TDD Band 54 will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3, 7.3.4.3, 7.3.5, 7.3B.4.3, 7.3B.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.3.3	Minimum conformance requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.3-1 and Table 7.3.3-2.

Table 7.3.3-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| 2 | -102.7 | -99.7 | -98 | -95 | -93.2 | -92 | FDD |
| 3 | -101.7 | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
| 4 | -104.7 | -101.7 | -100 | -97 | -95.2 | -94 | FDD |
| 5 | -103.2 | -100.2 | -98 | -95 |  |  | FDD |
| 6 |  |  | -100 | -97 |  |  | FDD |
| 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
| 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
| 9 |  |  | -99 | -96 | -94.2 | -93 | FDD |
| 10 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| 11 |  |  | -100 | -97 |  |  | FDD |
| 12 | -101.7 | -98.7 | -97 | -94 |  |  | FDD |
| 13 |  |  | -97 | -94 |  |  | FDD |
| 14 |  |  | -97 | -94 |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 17 |  |  | -97 | -94 |  |  | FDD |
| 18 |  |  | -1007 | -977 | -95.27 |  | FDD |
| 19 |  |  | -100 | -97 | -95.2 |  | FDD |
| 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
| 21 |  |  | -100 | -97 | -95.2 |  | FDD |
| 22 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| 23 | -104.7 | -101.7 | -100 | -97 | -95.2 | -94 | FDD |
| 24 |  |  | -100 | -97 |  |  | FDD |
| 25 | -101.2 | -98.2 | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
| 26 | -102.7 | -99.7 | -97.56 | -94.56 | -92.76 |  | FDD |
| 27 | -103.2 | -100.2 | -98 | -95 |  |  | FDD |
| 28 |  | -100.2 | -98.5 | -95.5 | -93.7 | -91 | FDD |
| 30 |  |  | -99 | -96 |  |  | FDD |
| 31 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 33 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 34 |  |  | -100 | -97 | -95.2 |  | TDD |
| 35 | -106.2 | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 36 | -106.2 | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 37 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 38 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 41 |  |  | -98 | -95 | -93.2 | -92 | TDD |
| 42 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| 43 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| 44 |  | [-100.2] | [-98] | [-95] | [-93.2] | [-92] | TDD |
| 45 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 48 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| 50 |  | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 51 |  | -102.2 | -100 |  |  |  | TDD |
| 52 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| 53 | -106.2 | -102.2 | -100 | -97 |  |  | TDD |
| 54 | -106.2 | -102.2 | -100 |  |  |  | TDD |
| … |  |  |  |  |  |  |  |
| 65 | -104.2 | -101.2 | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| 66 | -104.2 | -101.2 | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| 68 |  |  | -98.5 | -95.5 | -93.7 |  | FDD |
| … |  |  |  |  |  |  |  |
| 70 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| 71 |  |  | -97.2 | -94.2 | -92.0 | -87.5 | FDD |
| 72 | -99 | -95.7 | -93.5 |  |  |  | FDD |
| 73 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |
| 74 | -104.78 | -101.78 | -99.58 | -96.58 | -94.78 | -93.58 | FDD |
| 85 |  |  | -97 | -94 |  |  | FDD |
| 87 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |
| 88 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |
| Note 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5. Note 2: The Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port Note 4: For the UE which supports both Band 3 and Band 9 the reference sensitivity level is FFS. Note 5: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. Note 6: 6 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. Note 7: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. Note 8: 8 indicates that the requirement is modified by -0.5 dB when the assigned E-UTRA channel bandwidth is confined within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

<<Unchange text skipped>>

Table 7.3.3-2: Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 1 |  |  | 25 | 50 | 75 | 100 | FDD |
| 2 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
| 3 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
| 4 | 6 | 15 | 25 | 50 | 75 | 100 | FDD |
| 5 | 6 | 15 | 25 | 251 |  |  | FDD |
| 6 |  |  | 25 | 251 |  |  | FDD |
| 7 |  |  | 25 | 50 | 75 | 751 | FDD |
| 8 | 6 | 15 | 25 | 251 |  |  | FDD |
| 9 |  |  | 25 | 50 | 501 | 501 | FDD |
| 10 |  |  | 25 | 50 | 75 | 100 | FDD |
| 11 |  |  | 25 | 251 |  |  | FDD |
| 12 | 6 | 15 | 201 | 201 |  |  | FDD |
| 13 |  |  | 201 | 201 |  |  | FDD |
| 14 |  |  | 151 | 151 |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 17 |  |  | 201 | 201 |  |  | FDD |
| 18 |  |  | 25 | 251 | 251 |  | FDD |
| 19 |  |  | 25 | 251 | 251 |  | FDD |
| 20 |  |  | 25 | 201 | 203 | 203 | FDD |
| 21 |  |  | 25 | 251 | 251 |  | FDD |
| 22 |  |  | 25 | 50 | 501 | 501 | FDD |
| 23 | 6 | 15 | 25 | 50 | 75 | 100 | FDD |
| 24 |  |  | 25 | 50 |  |  | FDD |
| 25 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
| 26 | 6 | 15 | 25 | 251 | 251 |  | FDD |
| 27 | 6 | 15 | 25 | 251 |  |  | FDD |
| 28 |  | 15 | 25 | 251 | 251 | 251 | FDD |
| 30 |  |  | 25 | 251 |  |  | FDD |
| 31 | 6 | 54 | 54 |  |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 33 |  |  | 25 | 50 | 75 | 100 | TDD |
| 34 |  |  | 25 | 50 | 75 |  | TDD |
| 35 | 6 | 15 | 25 | 50 | 75 | 100 | TDD |
| 36 | 6 | 15 | 25 | 50 | 75 | 100 | TDD |
| 37 |  |  | 25 | 50 | 75 | 100 | TDD |
| 38 |  |  | 25 | 50 | 75 | 100 | TDD |
| 39 |  |  | 25 | 50 | 75 | 100 | TDD |
| 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| 43 |  |  | 25 | 50 | 75 | 100 | TDD |
| 44 |  | 15 | 25 | 50 | 75 | 100 | TDD |
| 45 |  |  | 25 | 50 | 75 | 100 | TDD |
| 50 |  | 15 | 25 | 50 | 75 | 100 | TDD |
| 51 |  | 15 | 25 |  |  |  | TDD |
| 52 |  |  | 25 | 50 | 75 | 100 | TDD |
| 53 | 6 | 15 | 25 | 50 |  |  | TDD |
| 54 | 6 | 15 | 25 |  |  |  | TDD |
| … |  |  |  |  |  |  |  |
| 65 | 6 | 15 | 25 | 50 | 75 | 100 | FDD |
| 66 | 6 | 15 | 25 | 50 | 75 | 100 | FDD |
| 68 |  |  | 25 | 251 | 251 |  | FDD |
| … |  |  |  |  |  |  |  |
| 70 |  |  | 25 | 50 | 75 |  | FDD |
| 71 |  |  | 25 | 251 | 201 | 201 | FDD |
| 72 | 6 | 54 | 54 |  |  |  | FDD |
| 73 | 6 | 54 | 54 |  |  |  | FDD |
| 74 | 6 | 15 | 25 | 251 | 251 | 251 | FDD |
| 85 |  |  | 201 | 201 |  |  | FDD |
| 87 | 6 | 54 | 54 |  |  |  | FDD |
| 88 | 6 | 54 | 54 |  |  |  | FDD |
| Note 1: refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.4.2-1). Note 2: For the UE which supports both Band 11 and Band 21 the uplink configuration for reference sensitivity is FFS. Note 3: refers to Band 20; in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart=11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart=16. Note 4: refers to Bands 31, 72, 73, 87 and 88 in the case of 3 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 9 and in the case of 5 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 10. |  |  |  |  |  |  |  |

Unless given by Table 7.3.3-3, the minimum requirements specified in Tables 7.3.3-1, 7.3.3-1a and 7.3.3-2 shall be verified with the network signalling value NS_01 (Table 6.2.4.3-1) configured.

Table 7.3.3-3: Network signalling value for reference sensitivity

| E-UTRA Band | Network Signalling value |
| --- | --- |
| 2 | NS_03 |
| 4 | NS_03 |
| 10 | NS_03 |
| 12 | NS_06 |
| 13 | NS_06 |
| 14 | NS_06 |
| 17 | NS_06 |
| 19 | NS_08 |
| 21 | NS_09 |
| 24 | NS_56 |
| 23 | NS_03 |
| 25 | NS_03 |
| 30 | NS_21 |
| 48 | NS_27 |
| 53 | NS_45 |
| 54 | NS_62 |
| 66 | NS_03 |
| 70 | NS_03 |
| 71 | NS_35 |
| 85 | NS_06 |

The normative reference for this requirement is TS 36.101 [2] clause 7.3.1.

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 7.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions.

<<Unchange text skipped>>

7.3.4.3.10	Message contents exceptions for Band 24

1. Information element additionalSpectrumEmission is set to NS_56. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.4.3.10-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_56"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 56 (NS_56) |  |  |

2. Information element ul-CarrierFreq in the SystemInformationblockType2 as part of the cell broadcast message shall be used to provide the EARFCN of the uplink frequency according to the Tx-Rx spacing being used for the test. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.4.3.10-2: SystemInformationBlockType2: UL carrier frequency test requirement for two Tx-Rx Spacings

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ul-CarrierFreq | Per tables in 4.3.1.1.24 |  |  |

7.3.4.3.11	Message contents exceptions for Band 54

1. Information element additionalSpectrumEmission is set to NS_62. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.4.3.11-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_62"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 62 (NS_62) |  |  |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

### 7.3.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Tables 7.3.5-1 and Table 7.3.5-2.

Table 7.3.5-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 2 | -102.0 | -99.0 | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
| 3 | -101.0 | -98.0 | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
| 4 | -104.0 | -101 | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 5 | -102.5 | -99.5 | -97.3 | -94.3 |  |  | FDD |
| 6 | - | - | -99.3 | -96.3 |  |  | FDD |
| 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
| 8 | -101.5 | -98.5 | -96.3 | -93.3 |  |  | FDD |
| 9 | - | - | -98.3 | -95.3 | -93.5 | -92.3 | FDD |
| 10 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 11 | - | - | -99.3 | -96.3 |  |  | FDD |
| 12 | -101.0 | -98.0 | -96.3 | -93.3 |  |  | FDD |
| 13 |  |  | -96.3 | -93.3 |  |  | FDD |
| 14 |  | - | -96.3 | -93.3 |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 17 | - | - | -96.3 | -93.3 |  |  | FDD |
| 18 | - | - | -99.37 | -96.37 | -94.57 | - | FDD |
| 19 | - | - | -99.3 | -96.3 | -94.5 | - | FDD |
| 20 |  |  | -96.3 | -93.3 | -90.5 | -89.3 | FDD |
| 21 |  |  | -99.3 | -96.3 | -94.5 |  | FDD |
| 22 |  |  | -96.0 | -93.0 | -91.2 | -90.0 | FDD |
| 23 | -104.0 | -101 | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 24 |  |  | -99.3 | -96.3 |  |  | FDD |
| 25 | -100.5 | -97.5 | -95.8 | -92.8 | -91.0 | -89.8 | FDD |
| 26 | -102 | -99 | -96.86 | -93.86 | -926 |  | FDD |
| 27 | -102.5 | -99.5 | -97.3 | -94.3 |  |  | FDD |
| 28 |  | -99.5 | -97.8 | -94.8 | -93.0 | -90.3 | FDD |
| 30 | - | - | -98.3 | -95.3 | - | - | FDD |
| 31 | -98.3 | -95.0 | -92.8 |  |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 33 | - | - | -99,3 | -96.3 | -94.5 | -93.3 | TDD |
| 34 | - | - | -99.3 | -96.3 | -94.5 | - | TDD |
| 35 | -105.5 | -101.5 | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 36 | -105.5 | -101.5 | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 37 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 38 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 39 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 40 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
| 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| 43 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| 44 |  | [-99.5] | [-97.3] | [-94.3] | [-92.5] | [-91.3] | TDD |
| 45 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| … |  |  |  |  |  |  |  |
| 48 |  |  | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| … |  |  |  |  |  |  |  |
| 54 | -106.2 | -102.2 | -100 |  |  |  | TDD |
| … |  |  |  |  |  |  |  |
| 65 | -103.5 | -100.5 | -99.5 | -96.5 | -94 | -92.8 | FDD |
| 66 | -103.5 | -100.5 | -98.8 | -95.8 | -94 | -92.8 | FDD |
| … |  |  |  |  |  |  |  |
| 68 |  |  | -97.8 | -94.8 | -93.0 |  | FDD |
| … |  |  |  |  |  |  |  |
| 70 |  |  | -99.3 | -96.3 | -94.5 |  | FDD |
| 71 |  |  | -96.5 | -93.5 | -91.3 | -86.8 | FDD |
| 72 | -98.3 | -95.0 | -92.8 |  |  |  | FDD |
| 73 | -98.3 | -95.0 | -92.8 |  |  |  | FDD |
| 74 | -104.08 | -101.08 | -98.88 | -95.88 | -94.08 | -92.88 | FDD |
| 85 |  |  | -96.3 | -93.3 |  |  | FDD |
| 87 | -98.3 | -95.0 | -92.8 |  |  |  | FDD |
| 88 | -98.3 | -95.0 | -92.8 |  |  |  | FDD |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3.5-2) Note 2: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port Note 4: For the UE which supports both Band 3 and Band 9 the reference sensitivity level is FFS. Note 5: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. Note 6: 6 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. Note 7: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. Note 8: 8 indicates that the requirement is modified by -0.5 dB when the assigned E-UTRA channel bandwidth is confined within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

For the UE which supports inter-band carrier aggregation configurations the ΔRIB,c in Tables 7.3.3-1A, 7.3.3-1B and 7.3.3-1C shall be applied for applicable bands.

In case the UE supports more than one of the inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one inter-band carrier aggregation configurations then:

-	When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the tolerances in Table 7.3.3-1A, truncated to one decimal place that would apply for that operating band among the supported CA configurations. In case there is a harmonic relation between low band UL and high band DL (i.e. bands listed in Table 7.3A.0-0a), then the maximum tolerance among the different supported carrier aggregation configurations involving such band shall be applied

-	When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum tolerance in Table 7.3.3-1A that would apply for that operating band among the supported CA configurations

NOTE 1:	The relation to the received PSD is  with NRB is the transmission bandwidth configuration according to Table 5.4.2-1.

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.5-1 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.5-2.

NOTE 2:	Table 7.3.5-2 does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors.

Table 7.3.5-2: Uplink configuration for reference sensitivity, LCRB @ RBstart format

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz / 6 | 3 MHz / 15 | 5 MHz / 25 | 10 MHz / 50 | 15 MHz / 75 | 20 MHz / 100 | Duplex Mode |
| 1 | - | - | 25@0 | 50@0 | 75@0 | 100@0 | FDD |
| 2 | 6@0 | 15@0 | 25@0 | 50@0 | 50@25 | 50@50 | FDD |
| 3 | 6@0 | 15@0 | 25@0 | 50@0 | 50@25 | 50@50 | FDD |
| 4 | 6@0 | 15@0 | 25@0 | 50@0 | 75@0 | 100@0 | FDD |
| 5 | 6@0 | 15@0 | 25@0 | 25@25 | - | - | FDD |
| 6 | - | - | 25@0 | 25@25 | - | - | FDD |
| 7 | - | - | 25@0 | 50@0 | 75@0 | 75@25 | FDD |
| 8 | 6@0 | 15@0 | 25@0 | 25@25 | - | - | FDD |
| 9 | - | - | 25@0 | 50@0 | 50@25 | 50@50 | FDD |
| 10 | - | - | 25@0 | 50@0 | 75@0 | 100@0 | FDD |
| 11 | - | - | 25@0 | 25@25 | - | - | FDD |
| 12 | 6@0 | 15@0 | 20@5 | 20@30 | - | - | FDD |
| 13 | - | - | 20@0 | 20@0 | - | - | FDD |
| 14 | - | - | 15@0 | 15@0 | - | - | FDD |
| ... |  |  |  |  |  |  |  |
| 17 | - | - | 20@5 | 20@30 | - | - | FDD |
| 18 | - | - | 25@0 | 25@25 | 25@50 | - | FDD |
| 19 | - | - | 25@0 | 25@25 | 25@50 | - | FDD |
| 20 | - | - | 25@0 | 20@0 | 20@11 | 20@16 | FDD |
| 21 | - | - | 25@0 | 25@25 | 25@50 | - | FDD |
| 22 | - | - | 25@0 | 50@0 | 50@25 | 50@50 | FDD |
| 23 | 6@0 | 15@0 | 25@0 | 50@0 | 75@0 | 100@0 | FDD |
| 24 | - | - | 25@0 | 50@0 | - | - | FDD |
| 25 | 6@0 | 15@0 | 25@0 | 50@0 | 50@25 | 50@50 | FDD |
| 26 | 6@0 | 15@0 | 25@0 | 25@25 | 25@50 | - | FDD |
| 27 | 6@0 | 15@0 | 25@0 | 25@25 | - | - | FDD |
| 28 |  | 15@0 | 25@0 | 25@25 | 25@50 | 25@75 | FDD |
| 30 | - | - | 25@0 | 25@25 | - | - | FDD |
| 31 | 6@0 | 5@9 | 5@10 | - | - | - | FDD |
| … |  |  |  |  |  |  |  |
| 33 | - | - | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| 34 | - | - | 25@0 | 50@0 | 75@0 | - | TDD |
| 35 | 6@0 | 15@0 | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| 36 | 6@0 | 15@0 | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| 37 | - | - | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| 38 | - | - | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| 39 | - | - | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| 40 | - | - | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| 41 | - | - | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| 42 | - | - | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| 43 | - | - | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| 44 | - | 15@0 | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| 45 | - | - | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| … |  |  |  |  |  |  |  |
| 48 | - | - | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| … |  |  |  |  |  |  |  |
| 54 | 6@0 | 15@0 | 25@0 |  |  |  | TDD |
| … |  |  |  |  |  |  |  |
| 65 | 6@0 | 15@0 | 25@0 | 50@0 | 75@0 | 100@0 | FDD |
| 66 | 6@0 | 15@0 | 25@0 | 50@0 | 75@0 | 100@0 | FDD |
| … |  |  |  |  |  |  |  |
| 68 | - | - | 25@0 | 25@25 | 25@50 | - | FDD |
| … |  |  |  |  |  |  |  |
| 70 | - | - | 25@0 | 50@0 | 75@0 | - | FDD |
| 71 | - | - | 25@0 | 25@0 | 20@0 | 20@0 | FDD |
| 72 | 6@0 | 5@9 | 5@10 | - | - | - | FDD |
| 73 | 6@0 | 5@9 | 5@10 | - | - | - | FDD |
| 74 | 6@0 | 15@0 | 25@0 | 25@0 | 25@0 | 25@0 | FDD |
| 85 | - | - | 20@5 | 20@30 | - | - | FDD |
| 87 | 6@0 | 5@9 | 5@10 | - | - | - | FDD |
| 88 | 6@0 | 5@9 | 5@10 | - | - | - | FDD |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 7.3B.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions.

<<Unchange text skipped>>

##### 7.3B.4.3.4	Message contents exceptions (network signalled value "NS_[09]")

1.	Information element additionalSpectrumEmission is set to NS_[09]. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3B.4.3.4-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_[09]"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | TBD |  |  |

##### 7.3B.4.3.5	Message contents exceptions (network signalled value "NS_62")

1.	Information element additionalSpectrumEmission is set to NS_62. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3B.4.3.5-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_62"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 62 (NS_62) |  |  |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

### 7.3B.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Tables 7.3B.5-1 and Table 7.3B.5-2.

Table 7.3B.5-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 2 | -102.0 | -99.0 | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
| 3 | -101.0 | -98.0 | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
| 4 | -104.0 | -101.0 | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 5 | -102.5 | -99.5 | -97.3 | -94.3 |  |  | FDD |
| 6 | - | - | -99.3 | -96.3 |  |  | FDD |
| 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
| 8 | -101.5 | -98.5 | -96.3 | -93.3 |  |  | FDD |
| 9 | - | - | -98.3 | -95.3 | -93.5 | -92.3 | FDD |
| 10 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 11 | - | - | -99.3 | -96.3 |  |  | FDD |
| 12 | -101.0 | -98.0 | -96.3 | -93.3 |  |  | FDD |
| 13 |  |  | -96.3 | -93.3 |  |  | FDD |
| 14 |  | - | -96.3 | -93.3 |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 17 | - | - | -96.3 | -93.3 |  |  | FDD |
| 18 | - | - | -99.3 | -96.3 | -94.5 | - | FDD |
| 19 | - | - | -99.3 | -96.3 | -94.5 | - | FDD |
| 20 |  |  | -96.3 | -93.3 | -90.5 | -89.3 | FDD |
| 21 |  |  | -99.3 | -96.3 | -94.5 |  | FDD |
| 22 |  |  | -96.0 | -93.0 | -91.2 | -90.0 | FDD |
| 23 | -104.0 | -101.0 | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 24 |  |  | -99.3 | -96.3 |  |  | FDD |
| 25 | -100.5 | -97.5 | -95.8 | -92.8 | -91.0 | -89.8 | FDD |
| 26 | -102 | -99 | -96.86 | -93.86 | -926 |  | FDD |
| 27 | -102.5 | -99.5 | -97.3 | -94.3 |  |  | FDD |
| 28 |  | -99.5 | -97.8 | -94.8 | -93.0 | -90.3 | FDD |
| 30 | - | - | -98.3 | -95.3 | - | - | FDD |
| 31 | -98.3 | -95.0 | -92.8 |  |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 33 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 34 | - | - | -99.3 | -96.3 | -94.5 | - | TDD |
| 35 | -105.5 | -101.5 | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 36 | -105.5 | -101.5 | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 37 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 38 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 39 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 40 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
| 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| 43 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| 45 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| … |  |  |  |  |  |  |  |
| 48 |  |  | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| … |  |  |  |  |  |  |  |
| 54 | -106.2 | -102.2 | -100 |  |  |  | TDD |
| … |  |  |  |  |  |  |  |
| 65 | -103.5 | -100.5 | -99.5 | -96.5 | -94 | -92.8 | FDD |
| 66 | -103.5 | -100.5 | -98.8 | -95.8 | -94 | -92.8 | FDD |
| … |  |  |  |  |  |  |  |
| 68 |  |  | -97.8 | -94.8 | -93.0 |  | FDD |
| … |  |  |  |  |  |  |  |
| 72 | -98.3 | -95.0 | -92.8 |  |  |  | FDD |
| Note 1: The transmitter shall be set to PUMAX as defined in clause 6.2.5 Note 2: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port Note 4: For the UE which supports both Band 3 and Band 9 the reference sensitivity level is FFS. Note 5: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. Note 6: 6 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. |  |  |  |  |  |  |  |

<<Unchange text skipped>>

Table 7.3B.5-2: Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 1 | - | - | 25 | 50 | 75 | 100 | FDD |
| 2 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
| 3 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
| 4 | 6 | 15 | 25 | 50 | 75 | 100 | FDD |
| 5 | 6 | 15 | 25 | 251 | - | - | FDD |
| 6 | - | - | 25 | 251 | - | - | FDD |
| 7 | - | - | 25 | 50 | 751 | 751 | FDD |
| 8 | 6 | 15 | 25 | 251 | - | - | FDD |
| 9 | - | - | 25 | 50 | 501 | 501 | FDD |
| 10 | - | - | 25 | 50 | 75 | 100 | FDD |
| 11 | - | - | 25 | 251 |  |  | FDD |
| 12 | 6 | 15 | 201 | 201 |  |  | FDD |
| 13 |  |  | 201 | 201 |  |  | FDD |
| 14 |  | - | 151 | 151 |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 17 | - | - | 201 | 201 |  |  | FDD |
| 18 | - | - | 25 | 251 | 251 | - | FDD |
| 19 | - | - | 25 | 251 | 251 | - | FDD |
| 20 |  |  | 25 | 201 | 203 | 203 | FDD |
| 21 |  |  | 25 | 251 | 251 |  | FDD |
| 22 |  |  | 25 | 50 | 501 | 501 | FDD |
| 23 | 6 | 15 | 25 | 50 | 751 | 1001 | FDD |
| 24 |  |  | 25 | 50 |  |  | FDD |
| 25 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
| 26 | 6 | 15 | 25 | 251 | 251 |  | FDD |
| 27 | 6 | 15 | 25 | 251 | - | - | FDD |
| 28 |  | 15 | 25 | 251 | 251 | 251 | FDD |
| 30 | - | - | 25 | 251 | - | - | FDD |
| 31 | 6 | 54 | 54 |  |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 33 | - | - | 25 | 50 | 75 | 100 | TDD |
| 34 | - | - | 25 | 50 | 75 | - | TDD |
| 35 | 6 | 15 | 25 | 50 | 75 | 100 | TDD |
| 36 | 6 | 15 | 25 | 50 | 75 | 100 | TDD |
| 37 | - | - | 25 | 50 | 75 | 100 | TDD |
| 38 | - | - | 25 | 50 | 75 | 100 | TDD |
| 39 |  |  | 25 | 50 | 75 | 100 | TDD |
| 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| 43 |  |  | 25 | 50 | 75 | 100 | TDD |
| 45 |  |  | 25 | 50 | 75 | 100 | TDD |
| … |  |  |  |  |  |  |  |
| 48 |  |  | 25 | 50 | 75 | 100 | TDD |
| … |  |  |  |  |  |  |  |
| 54 | 6 | 15 | 25 |  |  |  | TDD |
| … |  |  |  |  |  |  |  |
| 65 | 6 | 15 | 25 | 50 | 75 | 100 | FDD |
| 66 | - | - | 25 | 50 | 75 | 100 | FDD |
| … |  |  |  |  |  |  |  |
| 68 |  |  | 25 | 251 | 251 |  |  |
| … |  |  |  |  |  |  |  |
| 72 | 6 | 54 | 54 |  |  |  | FDD |
| Note 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.4.2-1). Note 2: For the UE which supports both Band 11 and Band 21 the uplink configuration for reference sensitivity is FFS. Note 3: For Band 20; in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart=11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart=16. Note 4: 4 refers to Bands 31 and 72; in the case of 3 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 9 and in the case of 5 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 10. |  |  |  |  |  |  |  |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------