3GPP TSG-RAN5 Meeting #99	R5-232435

# Incheon, South Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0415 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to A-Galileo relative signal power |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In reality, Galileo E1 signal is +1.25 dB higher than the GPS L1 C/A signal. This can be seen by looking at the values of Minimum received power [dBW] in the GPS and Galileo specifications: https://www.gps.gov/technical/icwg/IS-GPS-200N.pdf and the https://www.gsc-europa.eu/sites/default/files/sites/all/files/Galileo_OS_SIS_ICD_v2.0.pdf While GPS L1 C/A is at -158.5 dBW, Galileo E1 is at -157.25 dBW. This should be represented in the tests, by adding a power offset of +1.25 dB for the Galileo E1 signal. Similar, the GPS L5 signal is at -153.99 dBW (when adding I and Q component) which leads to the +3.6 dB offset in the table, while the Galileo E5a, E5b and E6 are each at -155.25 dBW. This leads to a an offset factor vs. GPS L1 C/A of +3.25, not +2 as stated in the table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting the signal levels of Galileo E1, E5 and E6 signals |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs supporting Galileo may fail unjustified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depends on RAN4 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 4.2.5	UEs supporting multiple satellite signals

For UEs supporting multiple satellite signals, different minimum performance requirements may be associated with different signals. The satellite simulator shall generate all signals supported by the UE. Signals not supported by the UE do not need to be simulated. The relative power levels of each signal type for each GNSS are defined in Table 4.2.4. The individual test scenarios in clauses 6, 7 and 13 define the reference signal power level for each satellite. The power level of each simulated satellite signal type shall be set to the reference signal power level defined in each test scenario in clauses 6, 7 and 13 plus the relative power level defined in Table 4.2.4.

Table 4.2.4: Relative signal power levels for each signal type for each GNSS

|  | Galileo |  | GPS/Modernized GPS |  | GLONASS |  | QZSS |  | SBAS |  | BDS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signal power levels relative to reference power levels | E1 | 1.25 dB | L1 C/A | 0 dB | G1 | 0 dB | L1 C/A | 0 dB | L1 | 0 dB | B1I | D1 | 0 dB |
|  |  |  |  |  |  |  |  |  |  |  |  | D2 | +5 dB |
|  | E6 | +3.25 dB | L1C | +1.5 dB | G2 | -6 dB | L1C | +1.5 dB |  |  | B1C | D1 | 0 dB |
|  | E5 | +3.25 dB | L2C | -1.5 dB |  |  | L2C | -1.5 dB |  |  | B2a | D1 | 0 dB |
|  |  |  | L5 | +3.6 dB |  |  | L5 | +3.6 dB |  |  |  | D1 | 0 dB |
|  |  |  |  |  |  |  |  |  |  |  |  | D2 | +5 dB |

NOTE 1:	For test cases which involve “Modernized GPS”, the satellite simulator shall also generate the GPS L1 C/A signal if the UE supports “GPS” in addition to “Modernized GPS”.

NOTE 2:	The signal power levels in the Test Parameter Tables represent the total signal power of the satellite per channel not e.g. pilot and data channels separately.

NOTE 3:	For test cases which involve "BDS", D1 represents MEO/IGSO satellites for B1I, B1C, B3I and B2a signal types and D2 represents GEO satellites for B1I and B3I signal types.

< End of changes >