3GPP TSG-RAN5 Meeting #99	R5-232531

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2404 | rev | - | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of eMG TC 6.6.18.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To corrent and align the test configurations with TS 38.133. - To clarify the specific NR RRC message contents with Annex H.3.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Tables in initial condition and test requirement were updated. - RRC message contents were specified. - Editor`s Note was updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test configurations in this case will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.18.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

6.6.18.4	NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

6.6.18.4.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event.

6.6.18.4.2	Test applicability

This test applies to all types of NR UEs supporting concurrent gap and independent measurement gap configurations for PRS measurement capability from release 17 onwards.

6.6.18.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.18.0.2 and 6.6.18.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.18.4.

6.6.18.4.4	Test description

6.6.18.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.18.4.4.1-1.

Table 6.6.18.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.18.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.18.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.18.4-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Test environment parameters are given in Table 6.6.18.4.4.1-2.

Table 6.6.18.4.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.18.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.18.4.4.1-3.

2. Message contents are defined in clause 6.6.18.4.4.3.

3. There are three NR carriers specified in the test. Cell 1 (NR cell 1) is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.18.4.4.1-3: General test parameters for NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 3 2: Cell 2 | Two FR1 NR carrier frequencies are used for the NR cells. |
| Active cell |  | 1, 2, 3 | NR cell 1 (Pcell) | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | NR cell 2, NR cell 3 | Cell 2 is an inter-frequency neighbour cell Cell 3 is an intra-frequency neighbour cell in the positioning assistance data. |
| Gap Pattern Id |  | 1, 2, 3 | 0 for MeasGapId #0 24 for MeasGapId #1 |  |
| Measurement gap offset | ms | 1, 2, 3 | 7 for MeasGapId #0 11 for MeasGapId #1 |  |
| A3-Offset | dB | 1, 2, 3 | -6 |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | ms | 1 | 3 | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  | s | 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

6.6.18.4.4.2	Test procedure

In this test, there are three cells: NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as neighbour cell in FR1 on NR RF channel 2 and NR cell 3 as neighbour cell in FR1 on NR RF channel 1. The test parameters are given in Table 6.6.18.4.4.1-3 and Table 6.6.18.4.5-1 respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2 and NR cell 3. Cell 1 and cell 3 transmit PRS during T2.

Two measurement gap patterns (MeasGapId #0 and MeasGapId #1) are configured with the gap pattern ID #0 and #1 as defined in Table 6.6.18.4.1-3. MeasGapId #1 is configured with a higher priority than MeasGapId #0. MeasGapId #0 and MeasGapId #1 are associated with the MOs for RF channel numbers #1 and #2, respectively.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 [38] shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance of MeasGapId #1 containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.18.4.5-1.

3. SS shall transmit an RRCReconfiguration message to configure Event A3 triggered measurement report and two concurrent per-UE measurement gap patterns with MeasGapId #1 configured with a higher priority than MeasGapId #0. One of concurrent gaps used for PRS measurement in same frequency layer of serving cells while the other concurrent gap in the different frequency layer of serving cells.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.18.4.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If Cell2’s overall delays measured from the beginning of period T2 is less than 1840 ms, and report the PRS RSRP measurements for Cell 3 with respect to the reference cell in the DL-AoD assistance data, then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements, then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop. And set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.18.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.18.4.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-FREQ and GAP NEEDED Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-6 with conditions MGe Table H.3.1-7 with condition INTER-FREQ for Cell 2 and condition INTRA-FREQ for Cell 3 |
| Specific message contents exceptions for Test Configuration 6.6.1.8-1 | Table H.3.1-3 with conditions INTER-FREQ MO and MGe with associatedMeasGapSSB-r17 set to 0 for Cell 2 and condition INTRA-FREQ MO and MGe with associatedMeasGapSSB-r17 set to 2 for Cell 3 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.6.1.8-2 and 6.6.1.8-3 | Table H.3.1-3 with conditions INTER-FREQ MO and MGe with associatedMeasGapSSB-r17 set to 0 for Cell 2 and conditions INTRA-FREQ MO and MGe with associatedMeasGapSSB-r17 set to 2 for Cell 3 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.6.18.4.4.3-2: MeasConfig (Step 3)

| Derivation path: Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod SEQUENCE { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with condition INTRA-FREQ MO | NR Cell 1 and Cell 3 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | Entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with condition INTER-FREQ MO | NR Cell 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod SEQUENCE { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-DEFAULT with condition EVENT_A3 | NR Cell 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod SEQUENCE { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig |  |  | GAP NEEDED |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ | 2 entries |  |  |
| GapConfig-r17[1] SEQUENCE{ |  | Entry 1 |  |
| measGapId-r17 | 0 |  | Pattern #0 |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 7 |  | Pattern #0 |
| mgl-r17 | ms6 |  | Pattern #0 |
| mgrp-r17 | ms40 |  | Pattern #0 |
| mgta-r17 | ms0 |  | Pattern #0 |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| GapConfig-r17[2] SEQUENCE{ |  | Entry 2 |  |
| measGapId-r17 | 2 |  | Pattern #24 |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 11 |  | Pattern #24 |
| mgl-r17 | ms10 |  | Pattern #24 |
| mgrp-r17 | ms80 |  | Pattern #24 |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | true |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.18.4.5	Test requirement

Tables 6.6.18.4.5-1 define the primary level settings including test tolerances for all tests.

Table 6.6.18.4.5-1: Cell specific test parameters of NR cells for NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| TDD configuration |  |  | 1 | N/A |  | N/A |  | N/A |  |
|  |  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| BWchannel |  | MHz | 1,2,3 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | 1,2,3 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | 1,2,3 | DLBWP.0.1 |  | N/A |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  | N/A |  |
| OCNG Patterns |  |  | 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH RMC configuration |  |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  |  | 1 | CR.1.1 FDD |  | N/A |  | N/A |  |
|  |  |  | 2 | CR.1.1 TDD |  | N/A |  | N/A |  |
|  |  |  | 3 | CR.2.1 TDD |  | N/A |  | N/A |  |
| Dedicated CORESET RMC configuration |  |  | 1 | CCR.1.1 FDD |  | N/A |  | N/A |  |
|  |  |  | 2 | CCR.1.1 TDD |  | N/A |  | N/A |  |
|  |  |  | 3 | CCR.2.1 TDD |  | N/A |  | N/A |  |
| SSB parameters |  |  | 1 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  | SMTC.2 |  | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  | SMTC.1 |  | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  | SMTC.1 |  | SMTC.1 |  |
| TRS Configuration |  |  | 1 | TRS.1.1 FDD |  | N/A |  | N/A |  |
|  |  |  | 2 | TRS.1.1 TDD |  | N/A |  | N/A |  |
|  |  |  | 3 | TRS.1.2 TDD |  | N/A |  | N/A |  |
| PRS configuration |  |  | 1 | PRS.1.4 FR1 |  | N/A |  | PRS.1.4 FR1 |  |
|  |  |  | 2 | PRS.1.4 FR1 |  | N/A |  | PRS.1.4 FR1 |  |
|  |  |  | 3 | PRS.2.4 FR1 |  | N/A |  | PRS.2.4 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1 | 15 |  |  |  |  |  |
|  |  |  | 2, 3 | 30 |  |  |  |  |  |
| PRS muting configuration |  |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  | ‘01’ |  |
| EPRE ratio of PSS to SSS |  |  | 1, 2, 3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | 1, 2, 3 |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | 1, 2, 3 |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  | 1, 2, 3 |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | 1, 2, 3 |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | 1, 2, 3 |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  | 1, 2, 3 |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  | 1, 2, 3 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  | 1, 2, 3 |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98+TT |  | -98+TT |  | -98+TT |  |
| Note2 |  | dBm/SCS | 1, 2 | -98 |  | -98 |  | -98 |  |
|  |  |  | 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | 1, 2 | -94+TT | -94+TT | -Infinity | -94+TT | N/A | N/A |
|  |  |  | 3 | -91+TT | -91+TT | -Infinity | -94+TT | N/A | N/A |
| PRS-RSRP Note 3 |  | dBm/SCS Note5 | 1, 2 | -Infinity | -101+TT | N/A | N/A | -Infinity | -108+TT |
|  |  |  | 3 | -Infinity | -98+TT | N/A | N/A | -Infinity | -105+TT |
| PRS |  | dB | 1, 2, 3 | -Infinity | -3+TT | N/A | N/A | -Infinity | -10+TT |
| PRS |  | dB | 1, 2, 3 | -Infinity | -3+TT | N/A | N/A | -Infinity | -10+TT |
| IoNote3 |  | dBm/9.36 MHz | 1, 2 | -62.25+TT |  | -64.60+TT | -62.25+TT | -62.25+TT |  |
|  |  | dBm/38.16 MHz | 3 | -56.16+TT |  | -58.50+TT | -56.16+TT | -56.16+TT |  |
| Propagation Condition |  |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP/PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report for cell 2, with a measurement reporting delay less than 1840 ms from the beginning of period T2.

The PRS RSRP measurement time fulfils the requirements specified in TS 38.133[6] Clause 9.9.3.5. The UE shall perform and report the PRS RSRP measurements for Cell 3 with respect to the reference cell in the DL-AoD assistance data, Cell 1, within the time duration specified in TS 38.133[6] section 9.9.3.5 starting from the beginning of time interval T2.

The rate of the correct events for the neighbour cell observed during repeated tests shall be at least 90% with a confidence level of 95%, where the reported PRS RSRP measurement for each correct event shall be within the PRS RSRP reporting range specified in TS 38.133[6] Clause 10.1.24.3, i.e., between PRS RSRP_0 and PRS RSRP_126.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

IUE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

[End of change]