3GPP TSG-RAN5 Meeting #99	R5-232548

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source:	MediaTek Inc.

Title:	Discussion paper of handling legacy test case for IoT NTN UE

# Document for:	Endorsement

Agenda Item:	6.3.25.6

1.	Introduction and Background

At RAN#96 the Rel-17 WI UE Conformance – NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects [1] was introduced.

Generally, NB-IoT/eMTC over NTN protocol is based on the legacy protocol defined for NB-IoT/eMTC over LTE as specified in TS 36.763 and TS 36.300.

Besides the new test cases specified in the Work Plan, to have a robust test scope the existing eMTC and NB-IoT test cases could be re-used for IoT NTN UE.

Meanwhile, depending on the additional RAT supported there are two types of NB-IoT/eMTC UEs:

Type 1 UE: UE supporting NB-IoT/eMTC over both LTE and NTN

Type 2 UE: UE supporting NB-IoT/eMTC over NTN only

As there were discussions and way-forwards for vertical UE in RAN5, such as SNPN and RedCap UEs [2] [3], similar approach could be adopted for IoT NTN test cases.

2.	Discussion

The legacy protocol conformance test cases defined to test the basic NB-IoT/eMTC protocol over LTE can be executed on UEs supporting NB-IoT/eMTC over both LTE and NTN without a change.

However, currently there is no test coverage for the basic NB-IoT/eMTC protocol for UEs supporting NB-IoT/eMTC over NTN only.

Below Figure-1 shows the test coverage and execution guidance for different UE types.

Figure-1: Test coverage analysis of different UE types

To avoid a case to be executed twice, for UEs that support NB-IoT/eMTC over LTE and NTN the preferred RAT for TC execution is LTE.

3.	Proposal/Way Forward

Proposal #1a: For eMTC legacy test cases fully applicable to UEs supporting eMTC over NTN add a note to the Main behavior to mention the difference for NTN. See R5-232543 [5] for details.

Proposal #1b: For eMTC legacy test cases with test steps that are not applicable to UEs supporting eMTC over NTN add an exception for the test steps that are not applicable. See R5-232541 [6] for details.

Proposal #2a: The prose of NB-IoT legacy test cases fully applicable to UEs supporting NB-IoT over NTN can be re-used as they are.

Proposal #2b: For NB-IoT legacy test cases with test steps that are not applicable to UEs supporting NB-IoT over NTN add an exception for the test steps that are not applicable. See R5-232544 [7] for details.

Proposal #3a: Add a new table for legacy eMTC test cases fully or partially applicable to UEs supporting eMTC over NTN. See R5-232546 [8] for details.

Proposal #3b: Add a new table for legacy NB-IoT test cases fully or partially applicable to UEs supporting NB-IoT over NTN. See R5-232547 [9] for details.

4.	References

[1] 	RP-221548: New WID on UE Conformance Test Aspects – NB-IoT/eMTC support for Non-Terrestrial

Networks (NTN) including EPS aspects, MediaTek Inc.

[2]	R5-226545: Discussion paper for handling legacy test case for Redcap Only UE, RAN5#97

[3]	R5-224322: Way Forward for Rel-15 NR Tests Applicability on SNPN Only UE, RAN5#96e

[4] 	R5-231915: Update of default configuration for IoT NTN, RAN5#98

[5] 	R5-232543: Correction to NAS Test Case to enable IoT NTN test, MediaTek Inc.

[6] 	R5-232541: Correction to Idle Mode Test Case to enable IoT NTN test, MediaTek Inc.

[7] 	R5-232544: Correction to NB-IoT Test Case to enable IoT NTN test, MediaTek Inc.

[8] 	R5-232546: Applicable eMTC cases for IoT NTN, MediaTek Inc.

[9] 	R5-232547: Applicable NB-IoT cases for IoT NTN, MediaTek Inc.