3GPP TSG-RAN5 Meeting #99	R5-232602

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0311 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to SDP Message from the SS for MCData |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MCData SDP from the SS was missing elements to enable pre-established session functionality |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the PRE_ESTABLISHED_SESSION condition to Table 5.5.3.1.2-3. Added IEs to enable pre-established session functionality to Table 5.5.3.1.2-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SDP used in MCData may not be correct and cause multiple test cases to have false results |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

Table 5.5.3.1.2-3: SDP Message from the SS for MCData

| Derivation Path: RFC 4566 [27] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |
| Session description: |  |  |  |  |  |  |  |  |
| Protocol Version |  | "0" |  | v= line |  |  |  |  |
| Origin |  | Same o=line as in the previous SDP message sent by the SS except that sess-version is incremented by one |  | o= line |  |  |  |  |
| Origin |  |  |  | o= line |  |  |  | FIRST_SDP_FROM_SS |
| username |  | "-" |  | "-" indicating the concept of user IDs not being supported |  |  |  |  |
| sess-id |  | "11111111" |  | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |  |  |
| sess-version |  | "11111111" |  |  |  |  |  |  |
| nettype |  | "IN" |  |  |  |  |  |  |
| Addrtype |  | "IP4" or "IP6" depending on IP address |  |  |  |  |  |  |
| unicast-address |  | IP address of the SS |  |  |  |  |  |  |
| Session Name |  | " " |  | s= line |  |  |  |  |
| Time description |  |  |  |  |  |  |  |  |
| Timing |  |  |  | t= line |  |  |  |  |
| start-time |  | "0" |  |  |  |  |  |  |
| stop-time |  | "0" |  |  |  |  |  |  |
| Session attribute |  |  |  | a=line attribute=”ice-lite” |  | RFC 5245 [115] |  | PRE_ESTABLISHED_SESSION |
| ice-lite |  |  |  |  |  |  |  |  |
| Media description[1] |  |  |  | Media description for data |  |  |  |  |
| media description |  |  |  | m= line media = message |  | RFC 4867 [59] TS 24.282 [31] |  |  |
| media |  | "message" |  |  |  |  |  |  |
| port |  | port number assigned by the SS |  | The transport port to which the media stream is sent |  |  |  |  |
| proto |  | "TCP/MSRP" |  |  |  |  |  |  |
| fmt |  | “*” |  |  |  |  |  |  |
| Connection Data |  |  |  | c= line |  |  |  |  |
| nettype |  | "IN" |  |  |  |  |  |  |
| Addrtype |  | "IP4" or "IP6" depending on IP address |  |  |  |  |  |  |
| connection-address |  | IP address of the SS |  |  |  |  |  |  |
| media attribute |  |  |  | a= line attribute = sendonly |  |  |  | SDP_OFFER AND NOT SDS_SESSION |
| sendonly |  |  |  | No parameters associated with this line |  |  |  |  |
| media attribute |  |  |  | a= line attribute = recvonly |  |  |  | SDP_ANSWER AND NOT SDS_SESSION |
| recvonly |  |  |  | No parameters associated with this line |  |  |  |  |
| media attribute |  |  |  | a= line attribute = sendrecv |  |  |  | SDS_SESSION |
| sendrecv |  |  |  | No parameters associated with this line |  |  |  |  |
| media attribute |  |  |  | a= line attribute = path |  |  |  |  |
| path |  | MSRP URI according to RFC 4975 [120] clause 6 and 9 |  |  |  | TS 24.282 [31] |  |  |
| scheme |  | "msrp" |  |  |  |  |  |  |
| authority |  |  |  |  |  | RFC 3986 [123] clause 3.2 |  |  |
| userinfo |  | not present |  |  |  |  |  |  |
| host |  | IP address of the SS |  |  |  |  |  |  |
| port |  | same value as in the media line |  |  |  |  |  |  |
| session id |  | assigned by the SS |  |  |  |  |  |  |
| transport |  | "tcp" |  |  |  |  |  |  |
| URI-parameter |  | not present |  |  |  |  |  |  |
| media attribute |  |  |  | a= line attribute = accept-types |  | RFC 4975 [120] |  |  |
| accept-types |  |  |  |  |  |  |  |  |
| format-entry[1] |  | "application/vnd.3gpp.mcdata-signalling" |  |  |  |  |  |  |
| format-entry[2] |  | "application/vnd.3gpp.mcdata-payload" |  |  |  |  |  | MCDATA_SDS |
| media attribute |  |  |  | a= line attribute = setup |  | RFC 4145 [119] |  |  |
| setup |  | “actpass” |  |  |  |  |  | SDP_OFFER |
|  |  | "passive" |  |  |  |  |  | SDP_ANSWER |
| media attribute |  |  |  | a= line attribute = file-transfer-id |  | RFC 5547 [124] |  | MCDATA_FD |
| file-transfer-id |  | value assigned by the SS |  | randomly chosen globally unique identification (RFC 5547 [124]) |  |  |  | SDP_OFFER |
|  |  | same value as in the sdp offer |  |  |  |  |  | SDP_ANSWER |
| media attribute |  |  |  | a= line attribute = file-selector |  | RFC 5547 [124] |  | MCDATA_FD |
| file-selector |  |  |  |  |  |  |  | SDP_OFFER |
| selector[1] |  |  |  |  |  |  |  |  |
| filename |  | name of the file to be transferred |  | e.g. "TestFile.txt" |  |  |  |  |
| filesize |  | size of the file to be transferred |  |  |  |  |  |  |
| filetype |  | type of the file to be transferred |  | e.g. "text/plain" |  |  |  |  |
| hash |  |  |  |  |  |  |  |  |
| algorithm |  | "sha-1" |  |  |  |  |  |  |
| value |  | hash value of the file to be transferred |  |  |  |  |  |  |
| file-selector |  | same value as in the sdp offer |  |  |  |  |  | SDP_ANSWER |
| media attribute |  |  |  | a= line attribute = file-date |  | RFC 5547 [124] |  | MCDATA_FD AND SDP_OFFER |
| file-date |  |  |  |  |  |  |  |  |
| date-param[1] |  |  |  |  |  |  |  |  |
| type |  | "creation" |  |  |  |  |  |  |
| date-time |  | date and time when the file has been created |  | e.g. "Mon, 20 Dec 2021 15:01:31 +0100" |  | RFC 5322 [109] |  |  |
| media attribute |  |  |  | a= line attribute = key-mgmt |  |  |  | SDP_OFFER AND MCD_1to1 |
| key-mgmt |  |  |  |  |  | TS 24.379 [9] clause 6.2.1 |  |  |
| mikey |  | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2 |  | Use condition MCDATA |  | RFC 4567 [44] |  |  |
| media attribute |  |  | a=line attribute=”candidate” |  | RFC 5245 [115] |  | PRE_ESTABLISHED_SESSION |  |
| candidate |  |  | candidate for RTP |  |  |  |  |  |
| foundation | 1234 |  | arbitrarily selected |  |  |  |  |  |
| component-id | 1 |  | according to RFC 5245 [115] clause 4.1.1.1 |  |  |  |  |  |
| transport | "UDP" |  |  |  |  |  |  |  |
| priority | 2130706431 |  | RFC 5245 [115] clause 4.2: 224 * 126 + 28 * 65535 + 256 - component id |  |  |  |  |  |
| connection-address | IP address of the SS (same IP address as in the c=line for speech) |  | default candidate |  |  |  |  |  |
| port | same port number as in the m= line for speech |  |  |  |  |  |  |  |
| cand-type | "host" |  |  |  |  |  |  |  |
| media attribute |  |  | a=line attribute=”candidate” |  | RFC 5245 [115] |  | PRE_ESTABLISHED_SESSION |  |
| candidate |  |  | candidate for RTCP |  |  |  |  |  |
| foundation | 1234 |  | same as for RTP |  |  |  |  |  |
| component-id | 2 |  | according to RFC 5245 [115] clause 4.1.1.1 |  |  |  |  |  |
| transport | "UDP" |  |  |  |  |  |  |  |
| priority | 2130706430 |  | RFC 5245 [115] clause 4.2: 224 * 126 + 28 * 65535 + 256 - component id |  |  |  |  |  |
| connection-address | IP address of the SS (same IP address as in the c=line for speech) |  | default candidate |  |  |  |  |  |
| port | same port number as in the m= line for speech incremented by 1 |  |  |  |  |  |  |  |
| cand-type | "host" |  |  |  |  |  |  |  |

<END OF CHANGES>