3GPP TSG-RAN5 Meeting #99	R5-232740

# Incheon, Korea, 22th– 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2251 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.5D.3_2.3 Additional spurious emissions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_UL_MIMO_PC3_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For UE that supports UL MIMO on SUL bands, Additional spurious emissions test case needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case 6.5D.3_2.3 Additional spurious emissions for SUL with UL MIMO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Additional spurious emissions can’t be tested for UE that supports UL MIMO on SUL bands. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5D.3_2.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 CR 2252 TS 38.522 CR 0278 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5D.3_2.3	Additional spurious emissions for SUL with UL MIMO

6.5D.3_2.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5D.3_2.3.2	Test applicability

This test applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

6.5D.3_2.3.3	Minimum conformance requirements

For UE supporting UL MIMO, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products is defined as the sum of the emissions from both UE transmit antenna connectors.

The additional spurious emission requirements specified in clause 6.5.3.3.3 apply to the UE operating on SUL bands. For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements shall be met with the UL MIMO configurations described in clause 6.2D.1_1.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.5D.3 and 6.5.3.3

6.5D.3_2.3.4	Test description

6.5D.3_2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5C-1. All these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in Table 6.5D.3_2.3.4.1-1 through Table 6.5D.3_2.3.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.3_2.3.4-1: Test Configuration Table (network signalling value “NS_05”)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Use centre frequency (Fc) as specified in test parameters for SUL carrier Mid range for Non-SUL carrier |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz for SUL carrier Lowest for Non-SUL carrier |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |  |  |
| Additional Spurious for SUL test parameters for NS_05 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | SUL ChBw (MHz) | Downlink Config. | Uplink Config |  | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  |  | Modulation |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 |  |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 2 | 1925 | 10 |  |  |  | A1 |  | QPSK | Outer_Full |
| 3 | 1925 | 10 |  |  |  | A7 |  | QPSK | 42@10 |
| 4 | 1925 | 10 |  |  |  | A2 |  | QPSK | 6@40 |
| 5 | 1935 | 10 |  |  |  | A4 |  | QPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  |  | A7 |  | QPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  |  | A2 |  | QPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  |  | A2 |  | QPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  |  | A5 |  | QPSK | Outer_Full |
| 12 | 1930 | 20 |  |  |  | A1 |  | QPSK | Outer_Full |
| 13 | 1930 | 20 |  |  |  | A7 |  | QPSK | 78@28 |
| 14 | 1930 | 20 |  |  |  | A2 |  | QPSK | 6@76 |
| 15 | 1950 | 20 |  |  |  | A6 |  | QPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  |  | A3 |  | 16 QAM | Outer_Full |
| 17 | 1925 | 10 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 18 | 1925 | 10 |  |  |  | A7 |  | 16 QAM | 42@10 |
| 19 | 1925 | 10 |  |  |  | A2 |  | 16 QAM | 6@40 |
| 20 | 1935 | 10 |  |  |  | A4 |  | 16 QAM | Outer_Full |
| 21 | 1927.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 22 | 1927.5 | 15 |  |  |  | A7 |  | 16 QAM | 60@19 |
| 23 | 1927.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@56 |
| 24 | 1932.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 25 | 1932.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@68 |
| 26 | 1942.5 | 15 |  |  |  | A5 |  | 16 QAM | Outer_Full |
| 27 | 1930 | 20 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 28 | 1930 | 20 |  |  |  | A7 |  | 16 QAM | 78@28 |
| 29 | 1930 | 20 |  |  |  | A2 |  | 16 QAM | 6@76 |
| 30 | 1950 | 20 |  |  |  | A6 |  | 16 QAM | Outer_Full |
| 31 | 1922.5 | 5 |  |  |  | A3 |  | 64 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 33 | 1925 | 10 |  |  |  | A7 |  | 64 QAM | 42@10 |
| 34 | 1925 | 10 |  |  |  | A2 |  | 64 QAM | 6@40 |
| 35 | 1927.5 | 15 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 36 | 1927.5 | 15 |  |  |  | A7 |  | 64 QAM | 60@19 |
| 37 | 1927.5 | 15 |  |  |  | A2 |  | 64 QAM | 6@56 |
| 38 | 1932.5 | 15 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 39 | 1932.5 | 15 |  |  |  | A2 |  | 64 QAM | 6@68 |
| 40 | 1930 | 20 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 41 | 1930 | 20 |  |  |  | A7 |  | 64 QAM | 78@28 |
| 42 | 1930 | 20 |  |  |  | A2 |  | 64 QAM | 6@76 |
| 43 | 1922.5 | 5 |  |  |  | A3 |  | 256 QAM | Outer_Full |
| 44 | 1925 | 10 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 45 | 1925 | 10 |  |  |  | A7 |  | 256 QAM | 42@10 |
| 46 | 1927.5 | 15 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 47 | 1927.5 | 15 |  |  |  | A7 |  | 256 QAM | 60@19 |
| 48 | 1932.5 | 15 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 49 | 1930 | 20 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 50 | 1930 | 20 |  |  |  | A7 |  | 256 QAM | 78@28 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.5D.3_2.3.4-2: Test Configuration Table (network signalling value “NS_05U”)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Use centre frequency (Fc) as specified in test parameters for SUL carrier Mid range for Non-SUL carrier |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz for SUL carrier Lowest for Non-SUL carrier |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |  |  |
| Additional Spurious for SUL test parameters for NS_05 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | SUL ChBw (MHz) | Downlink Config. | Uplink Config |  | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  |  | Modulation |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 |  |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 2 | 1925 | 10 |  |  |  | A1 |  | QPSK | Outer_Full |
| 3 | 1925 | 10 |  |  |  | A7 |  | QPSK | 42@10 |
| 4 | 1925 | 10 |  |  |  | A2 |  | QPSK | 6@40 |
| 5 | 1935 | 10 |  |  |  | A4 |  | QPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  |  | A7 |  | QPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  |  | A2 |  | QPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  |  | A2 |  | QPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  |  | A5 |  | QPSK | Outer_Full |
| 12 | 1930 | 20 |  |  |  | A1 |  | QPSK | Outer_Full |
| 13 | 1930 | 20 |  |  |  | A7 |  | QPSK | 78@28 |
| 14 | 1930 | 20 |  |  |  | A2 |  | QPSK | 6@76 |
| 15 | 1950 | 20 |  |  |  | A6 |  | QPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  |  | A3 |  | 16 QAM | Outer_Full |
| 17 | 1925 | 10 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 18 | 1925 | 10 |  |  |  | A7 |  | 16 QAM | 42@10 |
| 19 | 1925 | 10 |  |  |  | A2 |  | 16 QAM | 6@40 |
| 20 | 1935 | 10 |  |  |  | A4 |  | 16 QAM | Outer_Full |
| 21 | 1927.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 22 | 1927.5 | 15 |  |  |  | A7 |  | 16 QAM | 60@19 |
| 23 | 1927.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@56 |
| 24 | 1932.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 25 | 1932.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@68 |
| 26 | 1942.5 | 15 |  |  |  | A5 |  | 16 QAM | Outer_Full |
| 27 | 1930 | 20 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 28 | 1930 | 20 |  |  |  | A7 |  | 16 QAM | 78@28 |
| 29 | 1930 | 20 |  |  |  | A2 |  | 16 QAM | 6@76 |
| 30 | 1950 | 20 |  |  |  | A6 |  | 16 QAM | Outer_Full |
| 31 | 1922.5 | 5 |  |  |  | A3 |  | 64 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 33 | 1925 | 10 |  |  |  | A7 |  | 64 QAM | 42@10 |
| 34 | 1925 | 10 |  |  |  | A2 |  | 64 QAM | 6@40 |
| 35 | 1935 | 10 |  |  |  | A4 |  | 64 QAM | Outer_Full |
| 36 | 1927.5 | 15 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 37 | 1927.5 | 15 |  |  |  | A7 |  | 64 QAM | 60@19 |
| 38 | 1927.5 | 15 |  |  |  | A2 |  | 64 QAM | 6@56 |
| 39 | 1932.5 | 15 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 40 | 1932.5 | 15 |  |  |  | A2 |  | 64 QAM | 6@68 |
| 41 | 1942.5 | 15 |  |  |  | A5 |  | 64 QAM | Outer_Full |
| 42 | 1930 | 20 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 43 | 1930 | 20 |  |  |  | A7 |  | 64 QAM | 78@28 |
| 44 | 1930 | 20 |  |  |  | A2 |  | 64 QAM | 6@76 |
| 45 | 1950 | 20 |  |  |  | A6 |  | 64 QAM | Outer_Full |
| 46 | 1922.5 | 5 |  |  |  | A3 |  | 256 QAM | Outer_Full |
| 47 | 1925 | 10 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 48 | 1925 | 10 |  |  |  | A7 |  | 256 QAM | 42@10 |
| 49 | 1935 | 10 |  |  |  | A4 |  | 256 QAM | Outer_Full |
| 50 | 1927.5 | 15 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 51 | 1927.5 | 15 |  |  |  | A7 |  | 256 QAM | 60@19 |
| 52 | 1932.5 | 15 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 53 | 1942.5 | 15 |  |  |  | A5 |  | 256 QAM | Outer_Full |
| 54 | 1930 | 20 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 55 | 1930 | 20 |  |  |  | A7 |  | 256 QAM | 78@28 |
| 56 | 1950 | 20 |  |  |  | A6 |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.5D.3_2.3.4-3: Test Configuration Table (network signalled value “NS_56”)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters for SUL carrier Lowest for non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | 15 kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |  |
|  |  |  |  |  |  | Modulation |  |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz |
| 66 | Low | 5 | N/A for A-MPR testing | N/A | 14 | CP-OFDM | QPSK |  | Edge_1RB_Left |
| 67 | Low | 5 |  |  | 6 |  | QPSK |  | Outer_Full |
| 68 | Low | 5 |  |  | 4 |  | QPSK |  | 21@4 |
| 69 | Low | 5 |  |  | 4 |  | QPSK |  | 1@4 |
| 70 | Low | 5 |  |  | 14 |  | 16 QAM |  | Edge_1RB_Left |
| 71 | Low | 5 |  |  | 6 |  | 16 QAM |  | Outer_Full |
| 72 | Low | 5 |  |  | 4 |  | 16 QAM |  | 21@4 |
| 73 | Low | 5 |  |  | 4 |  | 16 QAM |  | 1@4 |
| 74 | Low | 5 |  |  | 14 |  | 64 QAM |  | Edge_1RB_Left |
| 75 | Low | 5 |  |  | 6 |  | 64 QAM |  | Outer_Full |
| 76 | Low | 5 |  |  | 4 |  | 64 QAM |  | 21@4 |
| 77 | Low | 5 |  |  | 4 |  | 64 QAM |  | 1@4 |
| 78 | Low | 5 |  |  | 14 |  | 256 QAM |  | Edge_1RB_Left |
| 79 | Low | 5 |  |  | 6 |  | 256 QAM |  | Outer_Full |
| 80 | Low | 5 |  |  | 4 |  | 256 QAM |  | 21@4 |
| 81 | Low | 5 |  |  | 4 |  | 256 QAM |  | 1@4 |
| 82 | Low | 10 |  |  | 12 |  | QPSK |  | Edge_1RB_Left |
| 83 | Low | 10 |  |  | 8 |  | QPSK |  | Outer_Full |
| 84 | Low | 10 |  |  | 6 |  | QPSK |  | 1@3 |
| 85 | Low | 10 |  |  | 6 |  | QPSK |  | 43@9 |
| 86 | Low | 10 |  |  | 4 |  | QPSK |  | 1@35 |
| 87 | Low | 10 |  |  | 2 |  | QPSK |  | 17@35 |
| 88 | Low | 10 |  |  | 5 |  | QPSK |  | Edge_1RB_Right |
| 89 | Low | 10 |  |  | 5 |  | QPSK |  | 1@40 |
| 90 | Low | 10 |  |  | 3 |  | QPSK |  | 8@44 |
| 91 | Low | 10 |  |  | 12 |  | 16 QAM |  | Edge_1RB_Left |
| 92 | Low | 10 |  |  | 8 |  | 16 QAM |  | Outer_Full |
| 93 | Low | 10 |  |  | 6 |  | 16 QAM |  | 1@3 |
| 94 | Low | 10 |  |  | 6 |  | 16 QAM |  | 43@9 |
| 95 | Low | 10 |  |  | 4 |  | 16 QAM |  | 1@35 |
| 96 | Low | 10 |  |  | 2 |  | 16 QAM |  | 17@35 |
| 97 | Low | 10 |  |  | 5 |  | 16 QAM |  | Edge_1RB_Right |
| 98 | Low | 10 |  |  | 5 |  | 16 QAM |  | 1@40 |
| 99 | Low | 10 |  |  | 3 |  | 16 QAM |  | 8@44 |
| 100 | Low | 10 |  |  | 12 |  | 64 QAM |  | Edge_1RB_Left |
| 101 | Low | 10 |  |  | 8 |  | 64 QAM |  | Outer_Full |
| 102 | Low | 10 |  |  | 6 |  | 64 QAM |  | 1@3 |
| 103 | Low | 10 |  |  | 6 |  | 64 QAM |  | 43@9 |
| 104 | Low | 10 |  |  | 4 |  | 64 QAM |  | 1@35 |
| 105 | Low | 10 |  |  | 2 |  | 64 QAM |  | 17@35 |
| 106 | Low | 10 |  |  | 5 |  | 64 QAM |  | Edge_1RB_Right |
| 107 | Low | 10 |  |  | 5 |  | 64 QAM |  | 1@40 |
| 108 | Low | 10 |  |  | 3 |  | 64 QAM |  | 8@44 |
| 109 | Low | 10 |  |  | 12 |  | 256 QAM |  | Edge_1RB_Left |
| 110 | Low | 10 |  |  | 8 |  | 256 QAM |  | Outer_Full |
| 111 | Low | 10 |  |  | 6 |  | 256 QAM |  | 1@3 |
| 112 | Low | 10 |  |  | 6 |  | 256 QAM |  | 43@9 |
| 113 | Low | 10 |  |  | 4 |  | 256 QAM |  | 1@35 |
| 114 | Low | 10 |  |  | 2 |  | 256 QAM |  | 17@35 |
| 115 | Low | 10 |  |  | 5 |  | 256 QAM |  | Edge_1RB_Right |
| 116 | Low | 10 |  |  | 5 |  | 256 QAM |  | 1@40 |
| 117 | Low | 10 |  |  | 3 |  | 256 QAM |  | 8@44 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A Figure A.3.1.2.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement channels are set according to Table 6.5D.3_2.3.4.1-1 to Table 6.5D.3_2.3.4.1-3 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.3_2.3.4.3.

6.5D.3_2.3.4.2	Test procedure

1	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.3_2.3.4.1-1 to Table 6.5D.3_2.3.4.1-3 as appropriate. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the sum of the mean power at each UE antenna connector in the channel bandwidth of the radio access mode, which shall meet the requirements described in clauses from 6.2D.2.5, or 6.2D.3.5 as appropriate. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of transmitted power at each UE antenna connector with a measurement filter of bandwidths according to clauses 6.5.3.3.3.1 to 6.5.3.3.3.25 as appropriate. The centre frequency of the filter shall be stepped in contiguous steps according to the same table the measured power shall be verified for each step. The measurement period shall capture the active time slots.

6.5D.3_2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO and same exceptions listed in clause 6.5.3.3.4.3

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, together with exceptions as specified in clause 6.5.3.3.4.3 as appropriate for different NS values.

6.5D.3_2.3.5	Test requirement

For SUL operation, the measured power at each UE antenna connector shall meet the requirements for the specified NR band for an additional spurious emission requirement with protected bands as indicated in clause 6.5.3.3.5 for different NS values.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

<End of Changes>