3GPP TSG-RAN5 Meeting #99	R5-232743

# Incheon, Korea, May 22nd - May 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2782 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of the contents of RRC messages for L2 U2N relay related operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The contents of RRC messages for L2 U2N relay related operation should be clarified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of the contents of RRC messages |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

4.6.1	Contents of RRC messages

<Unchanged Skip>

–	RRCReconfiguration

Table 4.6.1-13: RRCReconfiguration

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |  |  |
|  |  | RadioBearerConfig with conditions SRB2 and DRB1 |  |  |  | NR, NR-DC |
|  |  | RadioBearerConfig with conditions SRB2 and REEST |  |  |  | REEST |
| secondaryCellGroup |  | CellGroupConfig |  | OCTET STRING (CONTAINING CellGroupConfig) |  | EN-DC |  |
|  |  | CellGroupConfig with condition NR-DC_SCG |  | OCTET STRING (CONTAINING CellGroupConfig) |  | NR-DC_SCG |  |
|  |  | CellGroupConfig with conditions EN-DC and PSCell_change |  | OCTET STRING (CONTAINING CellGroupConfig) |  | EN-DC_HO |  |
|  |  | CellGroupConfig with condition MEAS |  | OCTET STRING (CONTAINING CellGroupConfig) |  | EN-DC_MEAS |  |
|  |  | CellGroupConfig with condition SCell_add |  | OCTET STRING (CONTAINING CellGroupConfig) |  | EN-DC_SCell_add |  |
|  |  | Not present |  |  |  |  |  |
| measConfig |  | Not present |  |  |  |  |  |
|  |  | MeasConfig |  | Measurements configuration |  | NR_MEAS, EN-DC_MEAS, IRAT_MEAS |  |
| lateNonCriticalExtension |  | Not present |  |  |  |  |  |
| nonCriticalExtension |  | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  | NR, SCell_add, NR_MEAS, NR-DC, SIDELINK, DAPS_HO_ReleaseSource, CHO, CPC, REEST, L2RelayUE, L2RemoteUE |  |
| masterCellGroup |  | CellGroupConfig with condition SRB2_DRB1 |  | OCTET STRING (CONTAINING CellGroupConfig) |  | NR, NR-DC |
|  |  | CellGroupConfig with condition SCell_add |  | OCTET STRING (CONTAINING CellGroupConfig) |  | SCell_add |
|  |  | CellGroupConfig with condition MEAS |  | OCTET STRING (CONTAINING CellGroupConfig) |  | NR_MEAS |
|  |  | CellGroupConfig with condition REEST |  | OCTET STRING (CONTAINING CellGroupConfig) |  | REEST |
|  |  | CellGroupConfig with condition RELAY |  | OCTET STRING (CONTAINING CellGroupConfig) |  | RELAY |
| fullConfig |  | Not present |  |  |  |  |  |
| dedicatedNAS-MessageList |  | Not present |  |  |  |  |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message { |  | 1 entry |  |  |  | NR |  |
| DedicatedNAS-Message[1] |  | DedicatedNAS-Message |  | entry 1 A sequence of OCTET STRING (s) containing one or more DedicatedNAS-Message(s) |  |  |  |
| } |  |  |  |  |  |  |  |
| masterKeyUpdate |  | Not present |  |  |  |  |  |
| masterKeyUpdate SEQUENCE { |  |  |  |  |  | MasterKeyChange |  |
| keySetChangeIndicator |  | true |  |  |  |  |  |
| nextHopChainingCount |  | NextHopChainingCount |  |  |  |  |  |
| nas-Container |  | Not present |  |  |  |  |  |
|  |  | OCTET STRING including the 10 Octets value generated according to TS 24.501 [28] clause 9.11.2.9 |  |  |  | Inter_Sys_HO |  |
| } |  |  |  |  |  |  |
| dedicatedSIB1-Delivery |  | Not present |  |  |  |  |  |
| dedicatedSystemInformationDelivery |  | Not present |  |  |  |  |  |
| otherConfig |  | Not present |  |  |  |  |  |
| nonCriticalExtension |  | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  | NR-DC, SIDELINK, DAPS_HO_ReleaseSource, CHO, CPC, NE-DC,L2RelayUE, L2RemoteUE |  |
| otherConfig-v1540 |  | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| mrdc-SecondaryCellGroupConfig |  | Not present |  |  |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |  |  | NR-DC, NE-DC |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| mrdc-ReleaseAndAdd |  | Not present |  |  |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |  |  |  |  |
| nr-SCG |  | RRCReconfiguration with condition NR-DC_SCG |  | OCTET STRING (CONTAINING RRCReconfiguration) |  | NR-DC |  |
| eutra-SCG |  | RRCConnectionReconfiguration according TS 36.508 [2], table 4.6.1-8 with condition NE-DC |  | OCTET STRING (CONTAINING RRCConnectionReconfiguration) |  | NE-DC |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| radioBearerConfig2 |  | Not present |  |  |  | NE-DC |
|  |  | RadioBearerConfig with condition DRBn and SecondaryKeys |  | OCTET STRING (CONTAINING RadioBearerConfig) |  | NR-DC |
| sk-Counter |  | SK-Counter |  |  |  |  |  |
| nonCriticalExtension |  | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  | SIDELINK, DAPS_HO_ReleaseSource, CHO, CPC,L2RelayUE, L2RemoteUE |  |
| otherConfig-v1610 |  | Not present |  |  |  |  |  |
| bap-Config-r16 |  | Not present |  |  |  |  |  |
| iab-IP-AddressConfigurationList-r16 |  | Not present |  |  |  |  |  |
| conditionalReconfiguration-r16 |  | Not present |  |  |  |  |  |
|  |  | ConditionalReconfiguration |  |  |  | CHO, CPC |  |
| daps-SourceRelease-r16 |  | Not present |  |  |  |  |  |
|  |  | true |  |  |  | DAPS_HO_ReleaseSource |  |
| t316-r16 |  | Not present |  |  |  |  |  |
| needForGapsConfigNR-r16 |  | Not present |  |  |  |  |  |
| onDemandSIB-Request-r16 |  | Not present |  |  |  |  |  |
| dedicatedPosSysInfoDelivery-r16 |  | Not present |  |  |  |  |  |
| sl-ConfigDedicatedNR-r16 |  | Not present |  |  |  |  |  |
| sl-ConfigDedicatedNR-r16 CHOICE { |  |  |  |  |  | SIDELINK |  |
| setup |  | SL-ConfigDedicatedNR-r16 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| sl-ConfigDedicatedEUTRA-Info-r16 |  | Not present |  |  |  |  |  |
| smtc-r16 |  | Not present |  |  |  |  |  |
| nonCriticalExtension |  | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  | L2RelayUE, L2RemoteUE |  |
| otherConfig-v1700 |  | Not present |  |  |  |  |  |
| sl-L2RelayUE-Config-r17 |  | Not present |  |  |  |  |  |
| sl-L2RelayUE-Config-r17 CHOICE { |  |  |  |  |  |  |  |
| setup |  | SL-L2RelayUE-Config with condition SL-SRB0 and SL-SRB1 |  |  |  | L2RelayUE |  |
| } |  |  |  |  |  |  |  |
| sl-L2RemoteUE-Config-r17 |  | Not present |  |  |  |  |  |
| sl-L2RemoteUE-Config-r17 CHOICE { |  |  |  |  |  | L2RemoteUE |  |
| setup |  | SL-L2RemoteUE-Config with condition SL-SRB2 and SL-DRB1 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| dedicatedPagingDelivery-r17 |  | Not present |  |  |  |  |  |
| needForGapNCSG-ConfigNR-r17 |  | Not present |  |  |  |  |  |
| needForGapNCSG-ConfigEUTRA-r17 |  | Not present |  |  |  |  |  |
| musim-GapConfig-r17 |  | Not present |  |  |  |  |  |
| ul-GapFR2-Config-r17 |  | Not present |  |  |  |  |  |
| scg-State-r17 |  | Not present |  |  |  |  |  |
| appLayerMeasConfig-r17 |  | Not present |  |  |  |  |  |
| ue-TxTEG-RequestUL-TDOA-Config-r17 |  | Not present |  |  |  |  |  |
| nonCriticalExtension |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

<Unchanged Skip>

–	RRCSetup

Table 4.6.1-21: RRCSetup

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetup ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetup SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition SRB1 |  |  |
| masterCellGroup |  | CellGroupConfig with condition SRB1 | OCTET STRING (CONTAINING CellGroupConfig) |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | L2RemoteUE |
| sl-ConfigDedicatedNR-r17 |  | Not present |  |  |
| sl-L2RemoteUE-Config-r17 |  | SL-L2RemoteUE-Config with condition SL-SRB0 and SL-SRB1 |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of Changed Section>