3GPP TSG-RAN5 Meeting #99	R5-232761

# Incheon, South Korea, 22nd – 26th May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0535 | rev | - | Current version: | 16.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Annex A.25 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To apply default EVS configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annex A.25 has been updated to use the generic procedure A.15.1a |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The aligment to use default EVS configuration cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# A.25	Video Conference Creation / 5GS

Expected sequence

| Step |  | Direction |  |  | Message |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | UE |  | SS |  |  |  |  |
| 1 |  |  |  |  | INVITE |  | UE sends INVITE with the first SDP offer. |  |
| 2 |  |  |  |  | 100 Trying |  | SS sends a 100 Trying provisional response. |  |
| 3 |  |  |  |  | 183 Session Progress |  | SS sends an SDP answer. |  |
| 4 |  |  |  |  | PRACK |  | UE acknowledges reception of 183 Session Progress., with optional SDP offer |  |
| 5 |  |  |  |  | 200 OK |  | SS responds to PRACK. |  |
|  |  |  |  |  | Exception : Step 6-7 describe optional behaviour depending on UE implementation. The steps will be executed if no SDP offer was included in PRACK at step 4 |  |  |  |
| 6 |  |  |  |  | UPDATE |  | UE sends a second SDP offer in an UPDATE request. |  |
| 7 |  |  |  |  | 200 OK |  | SS responds to UPDATE. |  |
| 8 |  | <- |  |  | 200 OK |  | The SS responds INVITE with 200 OK and gives the final conference URI within the response |  |
| 9 |  | -> |  |  | ACK |  | The UE acknowledges the receipt of 200 OK for INVITE |  |
|  |  |  |  |  | EXCEPTION: steps 10–13 describe optional behaviour depending on UE configuration. The SS shall wait up to 3s for the SUBSCRIBE of step 10 |  |  |  |
| 10 |  | -> |  |  | SUBSCRIBE |  | UE subscribes the conference event |  |
| 11 |  | <- |  |  | 200 OK |  | SS responds to the subscription |  |
| 12 |  | <- |  |  | NOTIFY |  | SS sends the initial state of the conference event to the UE |  |
| 13 |  | -> |  |  | 200 OK |  | UE responds to the NOTIFY |  |

Specific Message Contents

The specific message contents for steps 1–7 is otherwise identical to what have been specified in Annex A.15.1a, but with the exceptions as below:

INVITE (Step 1)

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | sip:mmtel@conf-factory appended with px_IMS_HomeDomainName |
| To |  |
| addr-spec | sip:mmtel@conf-factory appended with px_IMS_HomeDomainName |

183 Session in Progress for INVITE (Step 3)

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | sip:temporary@conf-factory. appended with px_IMS_HomeDomainName |
| feature-param | isfocus |
| Record-Route |  |
| rec-route | <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: protected server port of SS;lr> |

PRACK (Step 4)

| Header/param | Value/remark |
| --- | --- |
| Require | (present, if Message-Body is present) |
| option-tag | precondition |
| Message-body | (if present, shall be the same as message body in UPDATE at step 6 of Annex A.15.1a) |

200 OK for PRACK (Step 5)

| Header/param | Value/remark |
| --- | --- |
| Require | (present, if Message-Body is present) |
| option-tag | precondition |
| Content-Type |  |
| media-type | (present, if SDP message-Body was present in PRACK at step 4) |
|  | application/sdp |
| Content-Length | (present if SDP message-Body was present in PRACK at step 4) |
| value | length of message body |
| Message-body | (present, if SDP message-Body was present in PRACK at step 4) same as message body in 200 OK for UPDATE at step 7 of Annex A.15.1a |

200 OK for INVITE (Step 8)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in Annex A.3.1 of TS 34.229-1 [2], applying conditions A1 and A5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Record-Route |  |
| rec-route | Same value as in the 183 response |
| Contact |  |
| addr-spec | sip:final@conf-factory. appended with px_IMS_HomeDomainName |
| feature-param | Isfocus |

ACK (Step 9)

Use the default message “ACK” in Annex A.2.7 of TS 34.229-1 [2], applying conditions A1 and A3, with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | sip:final@conf-factory. appended with px_IMS_HomeDomainName |

SUBSCRIBE (Step 10)

Use the default message “SUBSCRIBE for conference event package” in Annex A.5.1 of TS 34.229-1 [2], applying conditions A1 and A7.

200 OK (Step 11)

Use the default message “200 OK for SUBSCRIBE” in Annex A.5.2 of TS 34.229-1 [2], applying condition A1.

NOTIFY (Step 12)

Use the default message “MT NOTIFY for conference event package” in Annex A.5.3 of TS 34.229-1 [2], applying condition A5.

200 OK (Step 13)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in Annex A.3.1 of TS 34.229-1 [2], applying conditions A5 and A22.