3GPP TSG-RAN5 Meeting #99 Incheon	R5-232891

Incheon, Republic of Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0691 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional test case 5.3.3.2.4 4RX TDD Minimum requirements for PDCCH with intra-slot repetition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add new test case for PDCCH demodulation requirements 4RX TDD Minimum requirements for PDCCH with intra-slot repetition |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new test case for PDCCH demodulation requirements 4RX TDD Minimum requirements for PDCCH with intra-slot repetition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI cannnot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | × | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | × | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | × | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.3	PDCCH demodulation requirements

5.3.3	4RX requirements

5.3.3.1	FDD

5.3.3.2	TDD

5.3.3.2.3	4Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving

<<Skip unchanged part>>

<<Start of Change>>

5.3.3.2.4	4RX TDD Minimum requirements for PDCCH with intra-slot repetition

5.3.3.2.4.1	Test Purpose

This test verifies the demodulation performance of PDCCH with intra-slot repetition under 4 receive antenna TDD conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.2.4.3-2. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.1.4.2	Test applicability

This test applies to all types of NR UE release 17 and forward.

This test also applies to all types of EUTRA UE release 17 and forward.

5.3.3.1.4.3	Minimum conformance requirements

Table 5.3.3.2.4.3-1: Test Parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  |  | 0,1 |  |
|  |  | Repetition transmission schemes |  |  | FDM |  |
|  |  | CCE to REG mapping type |  |  | nonInterleaved |  |
|  |  | REG bundle size |  |  | 6 |  |
|  |  | Time offset/Frequency offset of the second TxRP from the first TxRP |  |  | timing offset = -0.25us, frequency offset = 300Hz |  |
|  |  | Frequency domain resource allocation for CORSET |  |  | Frequency non-overlapping |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 40 |  |
|  |  | CSI-RS offset |  | Slots | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 | 20 for CSI-RS resources 5 and 6 21 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| Note: PDCCH is transmitted from both TRxP #1 and TRxP #2 |  |  |  |  |  |  |

For the parameters specified in Table 5.3.3.2.3.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.3.3-2. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.3.2.4.3-2: Minimum performance for PDCCH with 15kHz SCS (Note 2)

| Test num. | Bandwidth(MHz) | CORESET RB (Note 4) | CORESET duration | Aggregation level | Reference Channel | Propagation Condition (Note 1) | Antenna configuration and correlation Matrix (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 40 | 48 | 2 | 2 | R.PDCCH. 2-2.2 TDD | TDLA30-10 | 2x4, ULA Low | 1 | -1.0 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Bandwidth, CORESET parameters, reference channel, Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 Note 4: CORESETs from TRxP #1 and TRxP #2 should not be overlapped |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.3.3.2.4.

5.3.3.2.4.4	Test description

5.3.3.2.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.2.1-1 and Table 5.3.3.2.4.3-1 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.3.2.4.4.3.

5.3.3.2.4.4.2	Test procedure

1.	SS transmits PDCCH with DCI format as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.3.2.4.3-1. The details of PDCCH are specified in Table 5.3.3, Table 5.3.3.2-1, Table 5.3.3.2.4.3-1 respectively. The details of PDSCH are specified in Table A.3.3.1.1-3. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.3.2.4.3-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.3.2.4.4.4-1, pass the UE. Otherwise fail the UE.

5.3.3.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.3.2.4.4.3.1	Message exceptions for SA

Table 5.3.3.2.4.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| frequencyDomainResources | 11111111 11111110 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP |  |
| Duration | 1 | SearchSpace duration of 1 symbols |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  |  |
| reg-BundleSize | n6 |  | 2 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.4.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel2 | n1 |  |  |
| } |  |  |  |

Table 5.3.3.2.4.4.3.1-3: CSI-ResourcePeriodicityAndOffset for TRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| Slots40 | 10 | For CSI-RS resources 1,2,5,6 |  |
| Slots40 | 11 | For CSI-RS resources 3,4,7,8 |  |
| } |  |  |  |

5.3.3.2.4.4.3.2	Message exceptions for NSA

Same as 5.3.3.2.4.4.3.1

5.3.3.2.4.4.4	Test requirement

Table 5.3.3.2.4.4.4-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.4.4.4-1.

Table 5.3.3.2.4.4.4-1: Test Requirements for PDCCH with 30kHz SCS (Note 2)

| Test num. | Bandwidth(MHz) | CORESET RB (Note 4) | CORESET duration | Aggregation level | Reference Channel | Propagation Condition (Note 1) | Antenna configuration and correlation Matrix (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 40 | 48 | 2 | 2 | R.PDCCH. 2-2.2 TDD | TDLA30-10 | 2x4, ULA Low | 1 | -1.0 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Bandwidth, CORESET parameters, reference channel, Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 Note 4: CORESETs from TRxP #1 and TRxP #2 should not be overlapped |  |  |  |  |  |  |  |  |  |

<<End of Change>>