3GPP TSG-RAN5 Meeting #99	R5-232899

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3764 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition to testcase 8.1.5.13.3 SDT-SRB2-Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Testcase 8.1.5.13.3 SDT-SRB2-Indication needs to be completed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Test Description in Clause 8.1.5.13.3.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Existing Testcase 8.1.5.13.3 remains incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.13.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

##### 8.1.5.13.3	RRC SDT / CG based SDT / SDT-SRB2-Indication

8.1.5.13.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state and CG based SDT procedure is ongoing }

ensure that {

when { SRB2 is configured for SDT }

then { UE shall indicate through sdt-SRB2-Indication }

}

8.1.5.13.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.331, clause 5.3.1.1, 5.3.8.3, 5.3.13.1, 5.3.13.1b, 5.3.13.2, 5.3.13.3, and TS 24.501 clause 5.3.1.4. Unless otherwise stated, these are Rel-17 requirements.

[TS 38.331, clause 5.3.1.1]

The suspension of the RRC connection is initiated by the network. When the RRC connection is suspended, the UE stores the UE Inactive AS context and any configuration received from the network, and transits to RRC_INACTIVE state. The RRC message to suspend the RRC connection is integrity protected and ciphered.

The resumption of a suspended RRC connection is initiated by upper layers when the UE needs to transit from RRC_INACTIVE state to RRC_CONNECTED state or by RRC layer to perform a RNA update or by RAN paging from NG-RAN or for SDT. When the RRC connection is resumed, network configures the UE according to the RRC connection resume procedure based on the stored UE Inactive AS context and any RRC configuration received from the network. The RRC connection resume procedure re-activates AS security and re-establishes SRB(s) and DRB(s) and/or multicast MRB(s), if configured.

Upon initiating the resume procedure for SDT, AS security (both ciphering and integrity protection) is re-activated for SRB2 (if configured for SDT) and for SRB1. In addition, AS security is also re-activated (if security is configured) for all the DRBs configured for SDT. Further, the PDCP entities of SRB1 and PDCP entities of the radio bearers configured for SDT are re-established and resumed whilst the UE remains in RRC_INACTIVE state. Transmission and reception of data and/or signalling messages over radio bearers configured for SDT can happen whilst the UE is in RRC_INACTIVE state and SDT procedure is ongoing.

In response to a request to resume the RRC connection or in response to a resume procedure initiated for SDT, the network may resume the suspended RRC connection and send UE to RRC_CONNECTED, or reject the request to resume and send UE to RRC_INACTIVE (with a wait timer), or directly re-suspend the RRC connection and send UE to RRC_INACTIVE, or directly release the RRC connection and send UE to RRC_IDLE, or instruct the UE to initiate NAS level recovery (in this case the network sends an RRC setup message).

[TS 38.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this clause 60 ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCRelease includes suspendConfig:

2>	reset MAC and release the default MAC Cell Group configuration, if any;

2>	apply the received suspendConfig except the received nextHopChainingCount;

2>	if the sdt-Config is configured:

3>	for each of the DRB in the sdt-DRB-List:

4>	consider the DRB to be configured for SDT;

3>	if sdt-SRB2-Indication is configured:

4>	consider the SRB2 to be configured for SDT;

3>	for each RLC bearer that is not suspended:

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	for SRB2 (if it is resumed) and for SRB1:

4>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

3>	if sdt-MAC-PHY-CG-Config is configured:

4>	configure the PCell with the configured grant resources for SDT and instruct the MAC entity to start the cg-SDT-TimeAlignmentTimer;

…

2>	if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1:

3>	stop the timer T319 if running;

3>	in the stored UE Inactive AS context:

4>	replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys;

4>	replace the nextHopChainingCount with the value of nextHopChainingCount received in the RRCRelease message;

4>	replace the cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message;

4>	if the suspendConfig contains the sl-UEIdentityRemote (i.e. the UE is a L2 U2N Remote UE):

…

4> else:

5>	replace the C-RNTI with the C-RNTI used in the cell (see TS 38.321 [3]) the UE has received the RRCRelease message;

5>	replace the physical cell identity with the physical cell identity of the cell the UE has received the RRCRelease message;

3>	replace the nextHopChainingCount with the value associated with the current KgNB;

3>	stop the timer T319a if running and consider SDT procedure is not ongoing;

2>	else:

3>	store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for:

-	parameters within ReconfigurationWithSync of the PCell;

-	parameters within ReconfigurationWithSync of the NR PSCell, if configured;

-	parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured;

-	servingCellConfigCommonSIB;

…

NOTE 1c:	suspendConfig is not stored as part of UE Inactive AS Context, except for the fields explicitly specified.

3>	store any previously or subsequently received application layer measurement reports for which no segment, or full message, has been submitted to lower layers for transmission;

NOTE 2:	NR sidelink communication/discovery related configurations and logged measurement configuration are not stored as UE Inactive AS Context, when UE enters RRC_INACTIVE.

2>	suspend all SRB(s) and DRB(s) and multicast MRB(s), except SRB0 and broadcast MRBs;

2>	indicate PDCP suspend to lower layers of all DRBs and multicast MRBs;

2>	release the SRAP entity, if configured;

2>	if the t380 is included:

3>	start timer T380, with the timer value set to t380;

2>	if the RRCRelease message is including the waitTime:

3>	start timer T302 with the value set to the waitTime;

3>	inform upper layers that access barring is applicable for all access categories except categories '0' and '2';

2>	if T390 is running:

3>	stop timer T390 for all access categories;

3>	perform the actions as specified in 5.3.14.4;

2>	indicate the suspension of the RRC connection to upper layers;

2>	if the UE is capable of L2 U2N Remote UE:

3>	enter RRC_INACTIVE, and perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else:

3>	enter RRC_INACTIVE and perform cell selection as specified in TS 38.304 [20];

1>	else:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with the release cause 'other'.

NOTE 3:	Whether to release the PC5 unicast link is left to L2 U2N Remote UE's implementation.

NOTE 4:	It is left to UE implementation whether to stop T430, if running, when going to RRC_INACTIVE.

[TS 38.331, clause 5.3.13.1]

The purpose of this procedure is to resume a suspended RRC connection, including resuming SRB(s), DRB(s) and multicast MRB(s) or perform an RNA update. This procedure is also used to initiate SDT in RRC_INACTIVE.

[TS 38.331, clause 5.3.13.1b]

A UE in RRC_INACTIVE initiates the resume procedure for SDT when all of the following conditions are fulfilled:

1>	the upper layers request resumption of RRC connection; and

1>	SIB1 includes sdt-ConfigCommon; and

1>	sdt-Config is configured; and

1>	all the pending data in UL is mapped to the radio bearers configured for SDT; and

1>	lower layers indicate that conditions for initiating SDT as specified in TS 38.321 [3] are fulfilled.

NOTE:	How the UE determines that all pending data in UL is mapped to radio bearers configured for SDT is left to UE implementation.

[TS 38.331, clause 5.3.13.2]

The UE initiates the procedure when upper layers or AS (when responding to RAN paging, upon triggering RNA updates while the UE is in RRC_INACTIVE, for NR sidelink communication/discovery/V2X sidelink communication as specified in clause 5.3.13.1a) requests the resume of a suspended RRC connection or requests the resume for initiating SDT as specified in clause 5.3.13.1b.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

…

1>	else:

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications, except for the parameters for which values are provided in SIB1;

2>	apply the default SRB1 configuration as specified in 9.2.1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	else:

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

1>	if sdt-MAC-PHY-CG-Config is configured:

2>	if the resume procedure is initiated in a cell that is different to the PCell in which the UE received the stored sdt-MAC-PHY-CG-Config:

3>	release the stored sdt-MAC-PHY-CG-Config;

3>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

1>	if conditions for initiating SDT in accordance with 5.3.13.1b are fulfilled:

2>	consider the resume procedure is initiated for SDT;

2>	start timer T319a when the lower layers first transmit the CCCH message;

2>	consider SDT procedure is ongoing;

1> else:

2>	start timer T319;

2>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

…

1>	initiate transmission of the RRCResumeRequest message or RRCResumeRequest1 in accordance with 5.3.13.3.

[TS 38.331, clause 5.3.13.3]

The UE shall set the contents of RRCResumeRequest or RRCResumeRequest1 message as follows:

…

1>	if the resume procedure is initiated for SDT:

2>	for each radio bearer that is configured for SDT and for SRB1:

3>	restore the RLC-BearerConfig associated with the RLC bearers of masterCellGroup and pdcp-Config from the UE Inactive AS context;

…

3>	re-establish PDCP entity for the radio bearer that is configured for SDT without triggering PDCP status report;

2>	resume all the radio bearers that are configured for SDT;

1>	submit the selected message RRCResumeRequest or RRCResumeRequest1 for transmission to lower layers.

NOTE 2:	Only DRBs with previously configured UP ciphering shall resume ciphering.

If lower layers indicate an integrity check failure while T319 is running or SDT procedure is ongoing, perform actions specified in 5.3.13.5.

If the UE is a RedCap UE and the RedCap-specific initial downlink BWP is not associated with CD-SSB, the UE may continue cell re-selection related measurements as well as cell re-selection evaluation, otherwise the UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.13.6.

[TS 24.501, clause 5.3.1.4]

…

The UE is in 5GMM-CONNECTED mode with RRC inactive indication when the UE is in:

a)	5GMM-CONNECTED mode over 3GPP access at the NAS layer; and

b)	RRC_INACTIVE state at the AS layer (see 3GPP TS 38.300 [27]).

Unless stated otherwise, the UE behaviour in 5GMM-CONNECTED mode with RRC inactive indication follows the UE behaviour in 5GMM-CONNECTED over 3GPP access, except that:

a)	the UE shall apply the mobility restrictions; and

b)	the UE shall perform the PLMN selection procedures

as in 5GMM-IDLE mode over 3GPP access.

The UE shall transition from 5GMM-CONNECTED mode over 3GPP access to 5GMM-CONNECTED mode with RRC inactive indication upon receiving an indication from the lower layers that the RRC connection has been suspended.

NOTE 1:	Any pending procedure or uplink data packet when receiving an indication from the lower layers that the RRC connection has been suspended, triggers a request to the lower layers to transition to RRC_CONNECTED state. This is also the case when the pending procedure or uplink data packet triggered a previous request to the lower layers to transition to RRC_CONNECTED state.

If the UE in 3GPP access is configured for eCall only mode as specified in 3GPP TS 31.102 [22] then:

-	if the UE with the N1 NAS signalling connection established for eCall over IMS moved to 5GMM-CONNECTED mode with RRC inactive indication, the UE shall start timer T3444; and

-	if the UE with the N1 NAS signalling connection established for a call to an HPLMN designated non-emergency MSISDN or URI for test or terminal reconfiguration service moved to 5GMM-CONNECTED mode with RRC inactive indication, the UE shall start timer T3445.

Upon:

a)	a trigger of a procedure which requires sending of a NAS message different from a REGISTRATION REQUEST message with the NG-RAN-RCU bit of the 5GS update type IE set to "UE radio capability update needed";

b)	an uplink user data packet to be sent for a PDU session with suspended user-plane resources;

c)	a trigger to request resources for 5G ProSe direct discovery over PC5 or 5G ProSe direct communication over PC5; or

d)	a trigger to request resources for V2X communication over PC5 (see 3GPP TS 23.287 [6C]);

the UE in 5GMM-CONNECTED mode with RRC inactive indication over 3GPP access shall request the lower layers to transition to RRC_CONNECTED state (see 3GPP TS 38.300 [27]).

NOTE 2:	If the UE supports Small Data Transmission (SDT) (see 3GPP TS 38.300 [27]), the following applies:

a)	if the UE due to pending uplink NAS messages or user data packets is requesting the lower layers to transition to RRC_CONNECTED state, but has not received a response from the lower layers, the UE can send the pending NAS messages or user data packets to the lower layers, and can receive multiple downlink NAS messages or multiple downlink user data packets from the lower layers while the UE remains in 5GMM-CONNECTED mode with RRC inactive indication over 3GPP access (i.e., without transitioning to 5GMM-CONNECTED mode). When the NAS layer triggers the transmission of pending uplink NAS messages or user data packets, and if the SDT is ongoing, the NAS layer will receive the response from the lower layers only after the SDT session has completed or failed;

b)	the NAS layer is not aware of the classification of NAS messages or the user data packets as belonging to the SDT session at the lower layers; and

c)	the setting of access category and the RRC establishment cause indicated to the lower layers when sending the pending uplink user data packets while the UE remains in 5GMM-CONNECTED mode with RRC inactive indication, is left to implementation.

…

The UE shall transition from 5GMM-CONNECTED mode with RRC inactive indication to 5GMM-CONNECTED mode over 3GPP access upon receiving an indication from the lower layers that the UE has transitioned to RRC_CONNECTED state (see 3GPP TS 38.300 [27]).

NOTE 3:	The AMF can be aware of the transition between 5GMM-CONNECTED mode and 5GMM-CONNECTED mode with RRC inactive indication for a UE (see 3GPP TS 23.502 [9]).

…

If the UE in 5GMM-CONNECTED mode with RRC inactive indication receives an indication from the lower layers that the RRC connection has been suspended, the UE shall stay in 5GMM-CONNECTED mode with RRC inactive indication. The UE shall re-initiate any pending procedure that had triggered the request to the lower layers to transition to RRC_CONNECTED state, if still needed.

8.1.5.13.3.3	Test Description

8.1.5.13.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell with modifications to SIB1 given in Table 8.1.5.13.3.3.3-1.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to clause 4.4A.2 Table 4.4A.2-3 of TS 38.508-1 [4] and test state parameter UE test loop mode B prepared according to clause 4.4A.3 Table 4.4A.3-1 of TS 38.508-1 [4].

8.1.5.13.3.3.2	Test procedure sequence

Table 8.1.5.13.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a CLOSE UE TEST LOOP message with UE test loop mode B selected and T_delay_modeB timer configured. | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 3 | The SS sends one IP Packet to the UE. The IP Packet is less than sdt-DataVolumeThreshold. (Note 1) | - | - | - | - |
| 4 | The UE buffers the received IP PDU and waits for T_delay_modeB timer to expire. | - | - | - | - |
| 5 | The SS transmits an RRCRelease message with suspendConfig which includes SDT-CG-Config-r17 and sdt-SRB2-Indication-r17 IEs. | <-- | NR RRC: RRCRelease | - | - |
| 6 | Check: After the timer T_delay_modeB expires, does the UE transmit an RRCResumeRequest and the loop backed IP Packet received in step 4 in a CG PUSCH occasion using SRB2? | --> | NR RRC: RRCResumeRequest | 1 | P |
| Note 1: RLC PDU for IP packet is 97 bytes (RLC SDU is 94 bytes for 3 bytes RLC header and 95 bytes for 2 bytes RLC header) and sdt-DataVolumeThreshold is 100 bytes. Therefore the size of RLC SDU is less than the sdt-DataVolumeThreshold. |  |  |  |  |  |

8.1.5.13.3.3.3	Specific message contents

Table 8.1.5.13.3.3.3-1: SIB1 (Preamble and all steps, Table 8.1.5.13.3.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sdt-ConfigCommon-r17 SEQUENCE { |  |  |  |
| sdt-RSRP-Threshold-r17 | 60 | (IE value – 156) dBm = - 96 dBm |  |
| sdt-LogicalChannelSR-DelayTimer-r17 | Not present |  |  |
| sdt-DataVolumeThreshold-r17 | byte100 |  |  |
| t319a-r17 | ms4000 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.13.3.3.3-2: CLOSE UE TEST LOOP (Step 1, Table 8.1.5.13.3.3.2-1)

| Derivation path: 38.508-1 [4] Table 4.7A-3 condition UE test loop mode B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | 0 0 0 0 0 0 1 0 | 2 seconds |  |  |

Table 8.1.5.13.3.3.3-3: RRCRelease (Step 5, Table 8.1.5.13.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE and SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| sdt-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sdt-SRB2-Indication-r17 | allowed |  |  |
| sdt-MAC-PHY-CG-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pusch-Config-r17 CHOICE { |  |  |  |
| setup | PUSCH-Config | Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig { |  |  |  |
| ConfiguredGrantConfig | ConfiguredGrantConfig | Derivation Path: TS 38.508-1 [4] Table 4.6.3-26 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 CHOICE { | Not present |  |  |
| setup SEQUENCE { |  |  |  |
| pdcch-Config-r17 CHOICE { |  |  |  |
| setup | PDCCH-Config | Derivation Path: TS 38.508-1 [4], Table 4.6.3-95 |  |
| } |  |  |  |
| pdsch-Config-r17 CHOICE { |  |  |  |
| setup | PDSCH-Config | Derivation Path: TS 38.508-1 [4], Table 4.6.3-100 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | ms5120 | TimeAlignmentTimer(5.12s) is larger than IP PDU delay timer(2s) for Loop mode B. |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | 60 | (IE value – 156) dBm = - 96 dBm |  |
| cg-SDT-TA-ValiditationConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-RSRP-ChangeThreshold-r17 | dB10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.13.3.3.3-4: RRCResumeRequest (Step 6, Table 8.1.5.13.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |
| resumeCause | mo-data |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Change>