Source: 	Huawei, Hisilicon, China Unicom

Title: 	Test point analysis for General ON/OFF Time Mask for CA in FR1

Introduction

As agreed in RAN5#78, each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations) in [1] and [2].

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for General ON/OFF Time Mask for CA (2UL CA) test case in FR1.

Discussion

General ON/OFF Time Mask for LTE 2UL inter-band CA Test case is defined as follows:

Table 6.3.4A.1.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/Afor this test | QPSK | 31 | P_6@0 | S_25@0 | - | - |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 | - | - |
| 15 | 25 |  | QPSK | 12 | S_4@0 | P_8@0 |  |  |
| 25 | 6 |  | QPSK | 13 | P_8@0 | P_5@0 |  |  |
| 25 | 15 |  | QPSK | 40 | P_25@0 | S_15@0 | - | - |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 25 | 75 |  | QPSK | 24 | P_8@0 | S_16@0 |  |  |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

General ON/OFF Time Mask for FR1 SC Test case is defined in [3] as follows:

Table 6.3.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for minimum output power | Modulation | RB allocation (NOTE 1) |
| 1 | test case | CP-OFDM QPSK | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

The minimum requirement for intra-band contiguous CA and intra-band non-contiguous CA are extracted as below:

- 6.3A.3.1	Transmit ON/OFF time mask for intra-band contiguous CA

For a intra-band contiguous carrier aggregation, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

- 6.3A.3.2	Transmit ON/OFF time mask for intra-band non-contiguous CA

For a intra-band non-contiguous carrier aggregation, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

Test Environment

Based on the baseline agreed in [1], test environment for General ON/OFF Time Mask measurement for CA (2UL CA) in FR1 can reuse the test environment defined in LTE.

Proposal 1: Define Test Environment as ‘Normal, TL/VL, TL/VH, TH/VL, TH/VH’ for General ON/OFF Time Mask for CA (2UL CA) in FR1.

  - Test Frequencies

In LTE, General ON/OFF Time Mask for 2UL inter-band CA is tested for Low and High range frequencies. This is reasonable to verify different behaviours of the UE transmitter for different frequencies due to non-linearity based on the frequency. For NR it is proposed to adopt the same selection of test frequencies.

For intra band contiguous CA, Low range and High range are tested in LTE. It’s reasonable to adopt the same selection in NR. For intra-band non-contiguous CA, only low range and high range test frequencies are defined in TS 38.508-1 for CA configurations with UL CA.

Proposal 2: Test Low and High Frequencies for General ON/OFF Time Mask measurement for CA (2UL CA) in FR1. For intra-band non-contiguous UL CA, Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA will be tested.

Test Subcarrier Spacing

The ON power is measured over one slot, and the OFF power is measured in a duration of at least one slot. The slot length varies with different SCS. The highest SCS comes with the shortest slot length and is most critical for the ON/OFF Time Mask measurement, and shall be selected. For safety the lowest SCS is also selected as defined in SC test case in FR1.

Proposal 3: Select Lowest and Highest SCS supported for UE for General ON/OFF Time Mask for CA (2UL CA) measurement in FR1.

Test Channel Bandwidth

In LTE, this test case is tested for Lowest, and Highest channel bandwidths for both PCC and SCC. This assumption seems to be reasonable to test different behaviours of the UE transmitter related to different frequency selectivities with different channel bandwidths. We believe this is also reasonable for NR 2UL CA, although generally we consider highest BW to be the worst case because of possible frequency selectivity and BW limitation issues.

Proposal 4: Test the Lowest and Highest Channel Bandwidth for General ON/OFF Time Mask measurement for CA (2UL CA) in FR1.

Modulations and Uplink and DL Configuration

In single carrier test case, the configurations with minimum MPR was tested. The same principle should be applied in CA test case.

For inter-band CA and intra-band contiguous CA, DFT-s-OFDM QPSK and Inner Full will be tested for PCC and SCC.

For intra-band non-contiguous CA, the MPR requirement are in relevant with CBW of each CC as well as the MPRIM3 requirements. The RB allocations with minimum MPR and no limitation for carrier CBW should be selected.

Proposal 5: For inter-band CA and intra-band contiguous CA, DFT-s-OFDM QPSK and Inner Full will be tested for PCC and SCC .

For intra-band non-contiguous CA, DFT-s-OFDM and the RB allocations with minimum MPR and no limitation for carrier CBW should be selected.

Number of test points for General ON/OFF Time Mask measurement for CA (2UL CA)

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 5 | 2 | 2 | 2 | 1 | 40 |

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 General ON/OFF Time Mask for CA (2UL CA) test case definition:

Proposal 1: Define Test Environment as ‘Normal, TL/VL, TL/VH, TH/VL, TH/VH’ for General ON/OFF Time Mask for CA (2UL CA) in FR1.

Proposal 2: Test Low and High Frequencies for General ON/OFF Time Mask measurement for CA (2UL CA) in FR1.

Proposal 3: Select Lowest and Highest SCS supported for UE for General ON/OFF Time Mask for CA (2UL CA) measurement in FR1.

Proposal 4: Test the Lowest and Highest Channel Bandwidth for General ON/OFF Time Mask measurement for CA (2UL CA) in FR1.

Proposal 5: For inter-band CA and intra-band contiguous CA, DFT-s-OFDM QPSK and Inner Full will be tested for PCC and SCC . For intra-band non-contiguous CA, DFT-s-OFDM and the RB allocations with minimum MPR and no limitation for carrier CBW should be selected.

References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-181832_Discussion_NR_FR1_TestPoints_General_ON_OFF_Mask_6.3.3.2