3GPP TSG-RAN5 Meeting #99	R5-233097

# Incheon , Korea, 22nd – 26th May. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2801 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating n79 test frequencies for 10MHz channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR is the implementation of proposal 2 in discussion paper R5-233095. The test frequencies for n79 10MHz doesn’t align with the principle in Annex C.3.2 and need to corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting test frequencies for n79 in 4.3.1.1.1.79-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test frequencies for n79 is invalid. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is pending on discussion paper R5-233095 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

4.3.1.1	Test frequencies for NR operating bands in FR1

4.3.1.1.1	NR operating bands in FR1

<Unchanged Text Skipped>

4.3.1.1.1.79	Reference test frequencies for NR operating band n79

Table 4.3.1.1.1.79-1: Test frequencies for NR operating band n79, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 4405.02 | 693668 | 4400.34 | 693356 | 0 | 30 | 8475 | 693696 | 4 | 4 | 0 (4) | 8 |
|  |  | & | Mid | Same test frequencies and signalling parameters as in Low range |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | 106 | Downlink | Low | 4410 | 694000 | 4400.46 | 693364 | 0 | 30 | 8476 | 693792 | 8 | 11 | 0 (4) | 15 |
|  |  | & | Mid | 4699.995 | 713333 | 4672.095 | 711473 | 102 |  | 8676 | 712992 | 7 | 0 | 0 (4) | 106 |
|  |  | Uplink | High | 4990.005 | 732667 | 4889.745 | 725983 | 504 |  | 8878 | 732384 | 5 | 5 | 0 (4) | 513 |
| 40 | 216 | Downlink | Low | 4420.02 | 694668 | 4400.58 | 693372 | 0 | 30 | 8480 | 694176 | 0 | 43 | 0 (4) | 47 |
|  |  | & | Mid | 4699.98 | 713332 | 4662.18 | 710812 | 102 |  | 8672 | 712608 | 8 | 23 | 0 (4) | 129 |
|  |  | Uplink | High | 4980 | 732000 | 4869.84 | 724656 | 504 |  | 8864 | 731040 | 0 | 4 | 0 (4) | 512 |
| 50 | 270 | Downlink | Low | 4425 | 695000 | 4400.7 | 693380 | 0 | 30 | 8480 | 694176 | 4 | 42 | 0 (4) | 46 |
|  |  | & | Mid | 4699.98 | 713332 | 4657.32 | 710488 | 102 |  | 8672 | 712608 | 8 | 50 | 0 (4) | 156 |
|  |  | Uplink | High | 4974.99 | 731666 | 4859.97 | 723998 | 504 |  | 8864 | 731040 | 10 | 58 | 0 (4) | 566 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-5 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.79-2: Test frequencies for NR operating band n79, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | Same test frequencies and signalling parameters as in High range |  |  |  |  |  |  |  |  |  |  |  |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High | 4995 | 733000 | 4809.24 | 720616 | 504 |  | 8884 | 732960 | 8 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 4410 | 694000 | 4400.82 | 693388 | 0 | 30 | 8476 | 693792 | 20 | 2 | 1 (4) | 12 |
|  |  | & | Mid | 4700.01 | 713334 | 4654.11 | 710274 | 102 |  | 8676 | 712992 | 6 | 1 | 0 (0) | 206 |
|  |  | Uplink | High | 4990.02 | 732668 | 4799.4 | 719960 | 504 |  | 8878 | 732384 | 16 | 3 | 0 (0) | 1014 |
| 40 | 106 | Downlink | Low | 4420.02 | 694668 | 4400.94 | 693396 | 0 | 30 | 8480 | 694176 | 12 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4644.21 | 709614 | 102 |  | 8672 | 712608 | 18 | 8 | 1 (4) | 228 |
|  |  | Uplink | High | 4980 | 732000 | 4779.48 | 718632 | 504 |  | 8864 | 731040 | 0 | 3 | 0 (0) | 1014 |
| 50 | 133 | Downlink | Low | 4425 | 695000 | 4401.06 | 693404 | 0 | 30 | 8480 | 694176 | 4 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4639.35 | 709290 | 102 |  | 8672 | 712608 | 6 | 22 | 1 (4) | 256 |
|  |  | Uplink | High | 4974.99 | 731666 | 4769.61 | 717974 | 504 |  | 8864 | 731040 | 10 | 26 | 1 (4) | 1068 |
| 60 | 162 | Downlink | Low | 4430.01 | 695334 | 4400.85 | 693390 | 0 | 30 | 8480 | 694176 | 18 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4634.13 | 708942 | 102 |  | 8672 | 712608 | 18 | 36 | 1 (4) | 284 |
|  |  | Uplink | High | 4969.98 | 731332 | 4759.38 | 717292 | 504 |  | 8864 | 731040 | 20 | 54 | 1 (4) | 1124 |
| 80 | 217 | Downlink | Low | 4440 | 696000 | 4400.94 | 693396 | 0 | 30 | 8480 | 694176 | 12 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4624.23 | 708282 | 102 |  | 8656 | 711072 | 6 | 0 | 1 (4) | 212 |
|  |  | Uplink | High | 4959.99 | 730666 | 4739.49 | 715966 | 504 |  | 8848 | 729504 | 2 | 46 | 1 (4) | 1108 |
| 100 | 273 | Downlink | Low | 4450.02 | 696668 | 4400.88 | 693392 | 0 | 30 | 8480 | 694176 | 16 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4614.15 | 707610 | 102 |  | 8656 | 711072 | 6 | 28 | 1 (4) | 268 |
|  |  | Uplink | High | 4950 | 730000 | 4719.42 | 714628 | 504 |  | 8832 | 727968 | 20 | 37 | 1 (4) | 1090 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-6 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of Changes>