3GPP TSG-RAN5 Meeting #99	R5-233160

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5223 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NBIOT NTN multi-TAC test case 22.2.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Combinations of system information blocks specified in clause 8.1.4.3.1.1 are not considering intra frequency scenario for GSO NTN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added system information combination 10 for GSO NTN purpose is applied in this test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.508 CR 1425 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

### 22.2.13	NB-IoT / NTN / Multi-TAC

#### 22.2.13.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED/RRC-IDLE and UE finds a cell which provides access by NB-IoT NTN RAT }

ensure that {

when { systemInformationBlockType1-NB broadcast more than one TAC per PLMN }

then { UE forwards the trackingAreaList to upper layers for mobility management procedure }

}

#### 22.2.13.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clauses 5.2.2.7 and 6.7.2. Unless otherwise stated these are Rel-17 requirements.

[TS 36.331, clause 5.2.2.7]

Upon receiving the SystemInformationBlockType1-NB, the UE shall:

…

1>	else:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, trackingAreaList and cellIdentity for the cell as received in the cellAccessRelatedInfo;

1>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE; or

1>	if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE:

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

2>	forward the trackingAreaList to upper layers, if present;

[TS 36.331, clause 6.7.2]

trackingAreaList A list of tracking area codes for the PLMN listed. For the first entry in plmn-IdentityList-v1700: If this field is present, the list of tracking area codes include the tracking area code in trackingAreaCode-r13 and the tracking area codes in trackingAreaList. If this field is absent, only trackingAreaCode-r13 applies. For other entries in plmn-IdentityList-v1700: If this field is present, the list of tracking area codes include the tracking area codes in trackingAreaList. If this field is absent, the list of tracking area codes of the preceding entry in plmn-IdentityList-v1700 applies. The total number of signalled tracking area codes across all PLMNs cannot be more than maxTAC-NB-r17.

#### 22.2.13.3	Test description

##### 22.2.13.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 and Ncell 11 as specified in TS 36.508 [18] clause 8.1.4.1.2 are configured.

-	System information combination 10 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used for both cells.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The pre-configured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	The UE is in State 3-NB Registered, Idle Mode on Ncell 1 according to TS 36.508 [18].

##### 22.2.13.3.2	Test procedure sequence

Table 22.2.13.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 22.2.13.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 [18].

Table 22.2.13.3.2-1: Ncell configuration changes over time

|  | Parameter | Unit | Ncell 1 | Ncell 11 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | NRS EPRE | dBm/15kHz | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T1 | NRS EPRE | dBm/15kHz | “Off” | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 22.2.13.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts Ncell levels according to row T1 of table 22.2.13.3.2-1. | - | - | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 11 within 3 minutes? | --> | RRCConnectionRequest-NB | 1 | F |
| 3 | Check: Does the 'Test procedure to check UE response to Paging for Control Plane CIoT MT access' as described in TS 36.508 [18], clause 8.1.5A.2 take place on Ncell 11? | - | - | 1 | - |

##### 22.2.13.3.3	Specific message contents

Table 22.2.13.3.3-1: SystemInformationBlockType1-NB for Ncell 1 (preamble and all steps, table 22.2.13.3.2-2)

| Derivation Path: TS 36.508 [18], Table 8.1.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-NTN-r17 SEQUENCE { |  |  |  |
| cellBarred-NTN-r17 | notBarred |  |  |
| plmn-IdentityList-v1700 SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-NB-v1700 SEQUENCE | 1 entry |  |  |
| PLMN-IdentityInfo-NB-v1700[1] SEQUENCE { |  | Entry 1 |  |
| trackingAreaList-r17 SEQUENCE (SIZE (1..maxTAC-NB-r17)) OF TrackingAreaCode SEQUENCE { | 2 entries |  |  |
| TrackingAreaCode[1] | TAC-1 | Entry 1 |  |
| TrackingAreaCode[2] | TAC-2 | Entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.2.13.3.3-2: SystemInformationBlockType31-NB for Ncell 11 (preamble and all steps, table 22.2.13.3.2-2)

| Derivation Path: TS 36.508 [18], Table 8.1.4.3.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  | GSO |
| positionX-r17 | -11093064 |  |  |
| positionY-r17 | 30477944 |  |  |
| positionZ-r17 | 0 |  |  |
| } |  |  |  |
| orbitalParameters SEQUENCE { |  |  | NGSO |
| semiMajorAxis-r17 | FFS |  |  |
| eccentricity-r17 | FFS |  |  |
| periapsis-r17 | FFS |  |  |
| longitude-r17 | FFS |  |  |
| inclination-r17 | FFS |  |  |
| anomaly-r17 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

<End of modified section>