3GPP TSG-RAN5 Meeting #99	R5-233165

# Incheon, South Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5456 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of UE category in test case 7.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The UE category is limited to 5 for the test points of the maximum input power test case. Thus, UEs with a category higher than 5 may not be tested. However, this is not the intention of the test specification resulting in ambiguity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The applicability of test points has been extended for higher UE categories in Table 7.4.4.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity on the test point selection will remain and UEs with a category higher than 5 may not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

## 7.4	Maximum input level

### 7.4.1	Test purpose

Maximum input level tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area near to an e-NodeB.

### 7.4.2	Test applicability

This test applies to all types of E-UTRA UE release 8 and forward.

### 7.4.3	Minimum conformance requirements

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4.3-1. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.4.3-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -252 |  |  |  |  |  |
|  |  | -273 |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.3-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is Annex A.3.2: 64QAM, R=3/4 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: Reference measurement channel is Annex A.3.2: 256QAM, R=4/5 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 7.4.1.

### 7.4.4	Test description

#### 7.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.4.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] clause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] clause 4.3.1 |  |  |  | Mid range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] clause 4.3.1 |  |  |  | Lowest, 5MHz, Highest |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| Ch BW | Mod'n | RB allocation |  |  | Mod'n | RB allocation |  | UE Category |
|  |  | FDD | TDD |  |  | FDD | TDD |  |
| 1.4MHz | 64-QAM | 6 | 6 |  | QPSK | 6 | 6 | ≥1 |
| 3MHz | 64-QAM | 15 | 15 |  | QPSK | 15 | 15 | ≥1 |
| 5MHz | 64-QAM | 25 | 25 |  | QPSK | 25 | 25 | ≥2 |
| 5MHz | 64-QAM | 25 | N/A |  | QPSK | 20 | N/A | ≥2 |
| 5MHz | 64-QAM | 25 | N/A |  | QPSK | 15 | N/A | ≥2 |
| 5MHz | 64-QAM | 25 | N/A |  | QPSK | 54 | N/A | ≥2 |
| 5MHz | 64-QAM | 18 | 18 |  | QPSK | 25 | 25 | 1 |
| 5MHz | 64-QAM | 18 | 18 |  | QPSK | 20 | 20 | 1 |
| 5MHz | 64-QAM | 18 | 18 |  | QPSK | 15 | 15 | 1 |
| 5MHz | 64-QAM | 18 | N/A |  | QPSK | 54 | N/A | 1 |
| 10MHz | 64-QAM | 50 | 50 |  | QPSK | 50 | 50 | ≥2 |
| 10MHz | 64-QAM | 50 | N/A |  | QPSK | 25 | N/A | ≥2 |
| 10MHz | 64-QAM | 50 | N/A |  | QPSK | 20 | N/A | ≥2 |
| 10MHz | 64-QAM | 50 | N/A |  | QPSK | 15 | N/A | ≥2 |
| 10MHz | 64-QAM | 17 | 17 |  | QPSK | 50 | 50 | 1 |
| 10MHz | 64-QAM | 17 | N/A |  | QPSK | 25 | N/A | 1 |
| 10MHz | 64-QAM | 17 | N/A |  | QPSK | 20 | N/A | 1 |
| 10MHz | 64-QAM | 17 | N/A |  | QPSK | 15 | N/A | 1 |
| 15MHz | 64-QAM | 75 | 75 |  | QPSK | 75 | 75 | ≥2 |
| 15MHz | 64-QAM | 75 | N/A |  | QPSK | 50 | N/A | ≥2 |
| 15MHz | 64-QAM | 75 | N/A |  | QPSK | 25 | N/A | ≥2 |
| 15MHz | 64-QAM | 75 | N/A |  | QPSK | 20 | N/A | ≥2 |
| 15MHz | 64-QAM | 17 | 17 |  | QPSK | 75 | 75 | 1 |
| 15MHz | 64-QAM | 17 | N/A |  | QPSK | 50 | N/A | 1 |
| 15MHz | 64-QAM | 17 | N/A |  | QPSK | 25 | N/A | 1 |
| 15MHz | 64-QAM | 17 | N/A |  | QPSK | 20 | N/A | 1 |
| 20MHz | 64-QAM | 100 | 100 |  | QPSK | 100 | 100 | ≥3 |
| 20MHz | 64-QAM | 100 | N/A |  | QPSK | 75 | N/A | ≥3 |
| 20MHz | 64-QAM | 100 | N/A |  | QPSK | 50 | N/A | ≥3 |
| 20MHz | 64-QAM | 100 | N/A |  | QPSK | 25 | N/A | ≥3 |
| 20MHz | 64-QAM | 100 | N/A |  | QPSK | 20 | N/A | ≥3 |
| 20MHz | 64-QAM | 83 | 83 |  | QPSK | 100 | 100 | 2 |
| 20MHz | 64-QAM | 83 | N/A |  | QPSK | 75 | N/A | 2 |
| 20MHz | 64-QAM | 83 | N/A |  | QPSK | 50 | N/A | 2 |
| 20MHz | 64-QAM | 83 | N/A |  | QPSK | 25 | N/A | 2 |
| 20MHz | 64-QAM | 83 | N/A |  | QPSK | 20 | N/A | 2 |
| 20MHz | 64-QAM | 17 | 17 |  | QPSK | 100 | 100 | 1 |
| 20MHz | 64-QAM | 17 | N/A |  | QPSK | 75 | N/A | 1 |
| 20MHz | 64-QAM | 17 | N/A |  | QPSK | 50 | N/A | 1 |
| 20MHz | 64-QAM | 17 | N/A |  | QPSK | 25 | N/A | 1 |
| 20MHz | 64-QAM | 17 | N/A |  | QPSK | 20 | N/A | 1 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band. The applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: Depending on E-UTRA band, only the appropriate Uplink RB allocation value according to table 7.3.3-2 is tested per Test Channel Bandwidth. The allocation shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.4.2-1). Note 3: For the DL signal one sided dynamic OCNG Pattern OP.1 FDD/TDD is used. Note 4: Applicable only to E-UTRA FDD Bands 31 and 72. The UL resource blocks shall be located at RBstart 10 (according to Table 7.3.3-2). |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 7.4.4.3.

< End of changes >