3GPP TSG-RAN5 Meeting #99	R5-233314

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5203 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MAC eDRX TC 7.1.6.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test |  |  |  |  |  | Date: |  |  | 2023-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The specific message contents need to align with TS 36.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The specific message contents were updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect message contents will exist in these cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.6.1.3.3, 7.1.6.1a.3.3, 7.1.6.2.3.3, 7.1.6.3.3.3, 7.1.6.4.3.3 and 7.1.6.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-232077 -> R5-233314 |  |  |  |  |  |  |  |  |

[Start of change]

7.1.6.1.3.3	Specific message contents

Table 7.1.6.1.3.3-1: specific Parameters in RRCConnectionReconfiguration (step 0A of table 7.1.6.1.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| measConfig |  | Not present |  |  |
| mobilityControlInfo |  | Not present |  |  |
| dedicatedInfoNASList |  | Not present |  |  |
| radioResourceConfigDedicated SEQUENCE { |  |  |  |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList |  | Not present |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue SEQUENCE { |  |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |  |
| maxHARQ-Tx |  | n5 |  |  |
| retxBSR-Timer |  | sf10240 |  |  |
| ttiBundling |  | FALSE |  |  |
| } |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer |  | psf20 |  |  |
| drx-InactivityTimer |  | psf6 |  |  |
| drx-RetransmissionTimer |  | psf6 |  |  |
| longDRX-CycleStartOffset CHOICE { |  |  |  |  |
| sf640 |  | 4 |  |  |
| } |  |  |  |  |
| shortDRX |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated |  | infinity |  |  |
| phr-Config CHOICE { |  |  |  |  |
| release |  | NULL |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

[Skip unchanged text]

7.1.6.1a.3.3	Specific message contents

Table 7.1.6.1a.3.3-1: specific Parameters in RRCConnectionReconfiguration (step 0A of table 7.1.6.1a.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| measConfig |  | Not present |  |  |
| mobilityControlInfo |  | Not present |  |  |
| dedicatedInfoNASList |  | Not present |  |  |
| radioResourceConfigDedicated SEQUENCE { |  |  |  |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList |  | Not present |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue SEQUENCE { |  |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |  |
| maxHARQ-Tx |  | n5 |  |  |
| retxBSR-Timer |  | sf10240 |  |  |
| ttiBundling |  | FALSE |  |  |
| } |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer |  | psf20 |  |  |
| drx-InactivityTimer |  | psf6 |  |  |
| drx-RetransmissionTimer |  | psf6 |  |  |
| longDRX-CycleStartOffset CHOICE { |  |  |  |  |
| sf640 |  | 4 |  |  |
| } |  |  |  |  |
| shortDRX |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated |  | infinity |  |  |
| phr-Config CHOICE { |  |  |  |  |
| release |  | NULL |  |  |
| } |  |  |  |  |
| drx-Config-r13 CHOICE{ |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer-v1310 |  | psf300 |  |  |
| drx-RetransmissionTimer-v1310 |  | psf40 |  |  |
| drx-ULRetransmissionTimer-r13 |  | psf320 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

[Skip unchanged text]

7.1.6.2.3.3	Specific message contents

Table 7.1.6.2.3.3-1: specific Parameters in RRCConnectionReconfiguration (step 0A of table 7.1.6.2.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| measConfig |  | Not present |  |  |
| mobilityControlInfo |  | Not present |  |  |
| dedicatedInfoNASList |  | Not present |  |  |
| radioResourceConfigDedicated SEQUENCE { |  |  |  |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList |  | Not present |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue SEQUENCE { |  |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |  |
| maxHARQ-Tx |  | n5 |  |  |
| retxBSR-Timer |  | sf10240 |  |  |
| ttiBundling |  | FALSE |  |  |
| } |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer |  | psf20 |  |  |
| drx-InactivityTimer |  | psf6 |  |  |
| drx-RetransmissionTimer |  | psf6 |  |  |
| longDRX-CycleStartOffset CHOICE { |  |  |  |  |
| sf640 |  | 4 |  |  |
| } |  |  |  |  |
| shortDRX |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated |  | infinity |  |  |
| phr-Config CHOICE { |  |  |  |  |
| release |  | NULL |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

[Skip unchanged text]

7.1.6.3.3.3	Specific message contents

Table 7.1.6.3.3.3-1: specific Parameters in RRCConnectionReconfiguration (step 1 of table 7.1.6.3.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| measConfig |  | Not present |  |  |
| mobilityControlInfo |  | Not present |  |  |
| dedicatedInfoNASList |  | Not present |  |  |
| radioResourceConfigDedicated SEQUENCE { |  |  |  |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList |  | Not present |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue SEQUENCE { |  |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |  |
| maxHARQ-Tx |  | n5 |  |  |
| retxBSR-Timer |  | sf10240 |  |  |
| ttiBundling |  | FALSE |  |  |
| } |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer |  | psf20 |  |  |
| drx-InactivityTimer |  | psf6 |  |  |
| drx-RetransmissionTimer |  | psf6 |  |  |
| longDRX-CycleStartOffset CHOICE { |  |  |  |  |
| sf640 |  | 4 |  |  |
| } |  |  |  |  |
| shortDRX { |  |  |  |  |
| shortDRX-Cycle |  | sf64 |  |  |
| drxShortCycleTimer |  | 4 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated |  | infinity |  |  |
| phr-Config CHOICE { |  |  |  |  |
| release |  | NULL |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

[Skip unchanged text]

7.1.6.4.3.3	Specific message contents

Table 7.1.6.4.3.3-1: specific Parameters in RRCConnectionReconfiguration (step 1 of table 7.1.6.4.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| measConfig |  | Not present |  |  |
| mobilityControlInfo |  | Not present |  |  |
| dedicatedInfoNASList |  | Not present |  |  |
| radioResourceConfigDedicated SEQUENCE { |  |  |  |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList |  | Not present |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue SEQUENCE { |  |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |  |
| maxHARQ-Tx |  | n5 |  |  |
| retxBSR-Timer |  | sf10240 |  |  |
| ttiBundling |  | FALSE |  |  |
| } |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer |  | psf20 |  |  |
| drx-InactivityTimer |  | psf6 |  |  |
| drx-RetransmissionTimer |  | psf6 |  |  |
| longDRX-CycleStartOffset CHOICE { |  |  |  |  |
| sf640 |  | 4 |  |  |
| } |  |  |  |  |
| shortDRX { |  |  |  |  |
| shortDRX-Cycle |  | sf64 |  |  |
| drxShortCycleTimer |  | 10 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated |  | infinity |  |  |
| phr-Config CHOICE { |  |  |  |  |
| release |  | NULL |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

[Skip unchanged text]

7.1.6.5.3.3	Specific message contents

Table 7.1.6.5.3.3-1: specific Parameters in RRCConnectionReconfiguration (step 0A of table 7.1.6.1.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| measConfig |  | Not present |  |  |
| mobilityControlInfo |  | Not present |  |  |
| dedicatedInfoNASList |  | Not present |  |  |
| radioResourceConfigDedicated SEQUENCE { |  |  |  |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList |  | Not present |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue SEQUENCE { |  |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |  |
| maxHARQ-Tx |  | n5 |  |  |
| retxBSR-Timer |  | sf10240 |  |  |
| ttiBundling |  | FALSE |  |  |
| } |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer |  | psf20 |  |  |
| drx-InactivityTimer |  | psf6 |  |  |
| drx-RetransmissionTimer |  | psf6 |  |  |
| longDRX-CycleStartOffset CHOICE { |  |  |  |  |
| sf2560 |  | 4 |  |  |
| } |  |  |  |  |
| shortDRX |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated |  | infinity |  |  |
| phr-Config CHOICE { |  |  |  |  |
| release |  | NULL |  |  |
| } |  |  |  |  |
| eDRX-Config-CycleStartOffset-r13 CHOICE { |  |  |  |  |
| setup CHOICE { |  |  |  |  |
| sf5120 |  | 0 | longDRX-Cycle =5120 and drxStartOffset = 2560*0+4 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

[End of change]