3GPP TSG-RAN5 Meeting #99	R5-233418

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1423 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Ethernet configuration for EHC testing for EUTRA common config |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_IioT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Common configuration for the E-UTRA EHC testcase was missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added configuration for ethernet type communication to be used with E-UTRA EHC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | E-UTRA EHC testcase will remain indefinite. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 36.523-1 CR 5217 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is a revision of R5-232900 and two revisions of it. |  |  |  |  |  |  |  |  |

### -------------------------------------- Start of first change -------------------------------------

### 4.7.3	Contents of ESM messages

#### -	ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT

This message is sent by the UE to the SS.

Table 4.7.3-1: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT

| Derivation Path: 24.301 clause 8.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | The same value as the value set in ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned |  |
| Activate dedicated EPS bearer context accept message identity |  | '1100 0110'B | Activate dedicated EPS bearer context accept |  |
| Protocol configuration options |  | Not present or any allowed value |  | NOT (pc_NB or pc_NonIP_PDN) |
| Protocol configuration options |  | Not present |  | pc_NB or pc_NonIP_PDN |
| Protocol configuration options |  | Not present |  | Ethernet |
| NBIFOM container |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

| Condition | Explanation |
| --- | --- |
| Ethernet | Applicable to ethernet testcases. |

#### -	ACTIVATE DEDICATED EPS BEARER CONTEXT REJECT

This message is sent by UE to the SS.

Table 4.7.3-2: ACTIVATE DEDICATED EPS BEARER CONTEXT REJECT

| Derivation Path: 24.301 clause 8.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | The same value as the value set in ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned |  |
| Activate dedicated EPS bearer context reject message identity |  | '1100 0111'B | Activate dedicated EPS bearer context reject |  |
| ESM cause |  | The value is set according to specific message content. |  |  |
| Protocol configuration options |  | Not present |  |  |
| NBIFOM container |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST

This message is sent by the SS to the UE.

Table 4.7.3-3: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST

| Derivation Path: 24.301 clause 8.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | Arbitrarily selected value between '0101'B and '1111'B. |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned | NETWORK-INITIATED |
|  |  | The same value as the value set in BEARER RESOURCE MODIFICATION REQUEST message or BEARER RESOURCE ALLOCATION REQUEST message |  | UE-INITIATED |
| Activate dedicated EPS bearer context request message identity |  | '1100 0101'B | Activate dedicated EPS bearer context request |  |
| Linked EPS bearer identity |  | The EPS bearer identity of the associated default bearer |  |  |
| Spare half octet |  | '0000'B |  |  |
| EPS QoS |  | See Reference dedicated EPS bearer context #1 in table 6.6.2-1 |  |  |
| TFT |  | See Reference dedicated EPS bearer context #1 in table 6.6.2-1 |  |  |
| Transaction identifier |  | Not present |  |  |
|  |  | Distinct value between 0 and 127 calculated on basis of the EPS bearer identity. |  | pc_UTRAN AND/OR pc_GERAN |
| Negotiated QoS |  | See Reference dedicated EPS bearer context #1 in table 6.6.2-1 |  |  |
| Negotiated LLC SAPI |  | See Reference dedicated EPS bearer context #1 in table 6.6.2-1 |  |  |
| Radio priority |  | See Reference dedicated EPS bearer context #1 in table 6.6.2-1 |  |  |
| Packet flow Identifier |  | Not present |  |  |
|  |  | '0000 0000'B | Best Effort | pc_GERAN |
| Protocol configuration options |  | See Reference dedicated EPS bearer context #1 in table 6.6.2-1 |  | NOT (pc_NB or pc_NonIP_PDN) |
| Protocol configuration options |  | Not present |  | pc_NB or pc_NonIP_PDN |
| Protocol configuration options |  | Not present |  | Ethernet |
| WLAN offload container |  | Not present |  |  |
| NBIFOM container |  | Not present |  |  |
| Extended protocol configuration options |  | Not present |  | NOT (pc_NB or pc_NonIP_PDN) |
| Extended protocol configuration options |  | See Reference dedicated EPS bearer context #1 in table 6.6.2-1 |  | pc_NB or pc_NonIP_PDN |
| Extended protocol configuration options |  | Not present |  | Ethernet |

| Condition | Explanation |
| --- | --- |
| NETWORK-INITIATED | Network initiated ESM procedures |
| UE-INITIATED | UE initiated ESM procedures |
| Ethernet | Applicable to ethernet testcases. |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT

This message is sent by the UE to the SS.

Table 4.7.3-4: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT

| Derivation Path: 24.301 clause 8.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | The same value as the value set in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned |  |
| Activate default EPS bearer context accept message identity |  | '1100 0010'B | Activate default EPS bearer context accept |  |
| Protocol configuration options |  | Not present or any allowed value |  | NOT (pc_NB or pc_NonIP_PDN) |
| Protocol configuration options |  | Not present |  | pc_NB or pc_NonIP_PDN |
| Protocol configuration options |  | Not present |  | Ethernet |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |

| Condition | Explanation |
| --- | --- |
| Ethernet | Applicable to ethernet testcases. |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ACTIVATE DEFAULT EPS BEARER CONTEXT REJECT

This message is sent by UE to the SS.

Table 4.7.3-5: ACTIVATE DEFAULT EPS BEARER CONTEXT REJECT

| Derivation Path: 24.301 clause 8.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | The same value as the value set in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned |  |
| Activate default EPS bearer context reject message identity |  | '1100 0011'B | Activate default EPS bearer context reject |  |
| ESM cause |  | The value is set according to specific message content. |  |  |
| Protocol configuration options |  | Not present |  |  |
| Extended protocol configuration options |  | Not present |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST

This message is sent by the SS to the UE.

Table 4.7.3-6: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST

| Derivation Path: 24.301 clause 8.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | An arbitrarily selected value between '0101'B and '1111'B. |  |  |
| Procedure transaction identity |  | The same value as the value set in the latest PDN CONNECTIVITY REQUEST message sent prior to this message. |  |  |
| Activate default EPS bearer context request message identity |  | '1100 0001'B | Activate default EPS bearer context request |  |
| EPS QoS |  | See Reference default EPS bearer context #1 in table 6.6.1-1 |  | NOT IMS_PDN_ConnEstab |
| EPS QoS |  | See Reference default EPS bearer context #2 in table 6.6.1-1 |  | IMS_PDN_ConnEstab |
| Access point name |  | The SS defines a Default APN |  | NOT IMS_PDN_ConnEstab AND NOT APN_Provided |
| Access point name |  | APN as provided in ESM INFORMATION RESPONSE or PDN CONNECTIVITY REQUEST |  | NOT IMS_PDN_ConnEstab AND APN_Provided |
| Access point name |  | IMS. mnc<MNC>.mcc<MCC>.gprs The <MNC> and <MCC> are set to the same values as in IMSI. |  | IMS_PDN_ConnEstab AND NOT APN_Provided |
| Access point name |  | Use APN Network Identifier provided in ESM INFORMATION RESPONSE or PDN CONNECTIVITY REQUEST message and the APN Operator Identifier mnc<MNC>.mcc<MCC>.gprs. The <MNC> and <MCC> are set to the same values as in IMSI. |  | IMS_PDN_ConnEstab AND APN_Provided |
| PDN address |  |  |  | IPv4 |
| Length of PDN address contents |  | 5 octets |  |  |
| PDN type value |  | ‘001’B | IPv4 |  |
| PDN address information |  | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  |  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| PDN address |  |  |  | IPv6 |
| Length of PDN address contents |  | 9 octets |  |  |
| PDN type value |  | ‘010’B | IPv6 |  |
| PDN address information |  | IPv6 interface identifier | The SS provides a valid IPv6 interface identifier |  |
| PDN address |  |  |  | IPv4v6 |
| Length of PDN address contents |  | 13 octets |  |  |
| PDN type value |  | ‘011’B | IPv4v6 |  |
| PDN address information (Octets 4 to 11) |  | IPv6 interface identifier | The SS provides a valid IPv6 interface identifier |  |
| PDN address information (Octets 12 to 15) |  | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  |  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| PDN address |  |  |  | CIoT_CP_NON_IP |
| Length of PDN address contents |  | 2 octets |  |  |
| PDN type value |  | ‘101’B | Non-IP PDN |  |
| PDN address information (Octets 4 to 7) |  | Coded as 0 |  | spare |  |  |  |
| PDN address |  |  |  | Ethernet |
| Length of PDN address contents |  | 3 octets |  |  |
| PDN type value |  | ‘110’B | Ethernet |  |
| PDN address information (Octets 4 to 7) |  | Coded as 0 |  |  |
| Transaction identifier |  | Not present |  |  |  |  |  |
| Transaction identifier |  | Distinct value between 0 and 127 calculated on basis of the EPS bearer identity. |  | pc_UTRAN AND/OR pc_GERAN |
| Negotiated QoS |  | See Reference default EPS bearer context #1 in table 6.6.1-1 |  |  |
| Negotiated LLC SAPI |  | See Reference default EPS bearer context #1 in table 6.6.1-1 |  |  |
| Radio priority |  | See Reference default EPS bearer context #1 in table 6.6.1-1 |  |  |
| Packet flow Identifier |  | Not present |  |  |
| Packet flow Identifier |  | '0000 0000'B | Best Effort | pc_GERAN |
| APN-AMBR |  | See Reference default EPS bearer context #1 in table 6.6.1-1 |  |  |
| ESM cause |  | Not present |  |  |
| Protocol configuration options |  | Not present |  | pc_NB or pc_NonIP_PDN |
| Protocol configuration options |  | Not present |  | Ethernet |
| Protocol configuration options |  |  |  | NOT (pc_NB or pc_NonIP_PDN) |
| Configuration protocol |  | See Reference default EPS bearer context #1 in table 6.6.1-1 |  |  |
| Container ID 1 |  | ‘0001’H |  | P-CSCF IPv6 |
| Length of container ID 1 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID 1 contents |  | IPv6 address | P-CSCF IPv6 Address |  |
| Container ID 2 |  | ‘000C’H |  | P-CSCF IPv4 |
| Length of container ID 2 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID 2 contents |  | IPv4 address | P-CSCF IPv4 Address |  |
| Container ID n |  | ‘0003’H | n assigned to next available number | DNS IPv6 |
| Length of container ID n contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n contents |  | IPv6 address | DNS IPv6 Address |  |
| Container ID n+1 |  | ‘000D’H | n assigned to next available number | DNS IPv4 |
| Length of container ID n+1 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n+1 contents |  | IPv4 address | DNS IPv4 Address |  |
| Container ID n+2 |  | ‘001D’H | n assigned to next available number | Interworking_with_5GS |
| Length of container ID n+2 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n+2 contents |  | Session-AMBR | Session-AMBR |  |
| Session AMBR |  |  |  |  |
| Unit for Session-AMBR for downlink |  | ‘000 00101’ | Value is incremented in multiples of 256 Kbps |  |
| Session-AMBR for downlink |  | ‘0000 0000 0000 0100’B | 1024 Kbps |  |
| Unit for Session-AMBR for uplink |  | ‘000 00101’ | Value is incremented in multiples of 256 Kbps |  |
| Session-AMBR for uplink |  | ‘0000 0000 0000 0100’B | 1024 Kbps |  |
| Container ID n+3 |  | ‘001C’H | n assigned to next available number | Interworking_with_5GS |
| Length of container ID n+3 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n+3 contents |  | QoS rules | QoS rules according to TS 38.508-1 [67] Table 4.8.2.1-1 | NOT IMS_PDN_ConnEstab |
| Container ID n+3 contents |  | QoS rules | QoS rules according to TS 38.508-1 [67] Table 4.8.2.1-2 | IMS_PDN_ConnEstab |
| Container ID n+4 |  | ‘001F’H | n assigned to next available number | Interworking_with_5GS |
| Length of container ID n+4 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n+4 contents |  | Qos flow descriptions | Qos flow descriptions according to TS 38.508-1 [67] 4.8.2.3-1 with condition Interworking_with_EPS | NOT IMS_PDN_ConnEstab |
| Container ID n+4 contents |  | Qos flow descriptions | Qos flow descriptions refer to TS 38.508-1 [67] 4.8.2.3-2 with condition Interworking_with_EPS | IMS_PDN_ConnEstab |
| Container ID n+5 |  | ‘001B’H | n assigned to next available number | Interworking_with_5GS |
| Length of container ID n+5 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n+5 contents (Octet 1) |  | ‘0000 0001’B | SST of S-NSSAI same as the allowed NSSAI in TS 38.508-1 [67] Table 4.7.1-7: REGISTRATION ACCEPT |  |
| Container ID n+5 contents (Octets 2 to 4) |  | PLMN ID | PLMD ID same as TS 38.508-1 [67] Table 6.3.2.2-1 |  |
| WLAN offload indication |  | Not present |  |  |
| WLAN offload indication |  |  |  | WLAN |
| WLAN offload acceptability |  | ‘0001’B | Offloading the traffic of the PDN connection via a WLAN when in S1 mode is acceptable |  |
| NBIFOM container |  | Not present |  |  |
| Header compression configuration |  | Not present or set according to specific message content |  |  |
| Control plane only indication |  | Not present |  |  |
| Control plane only indication |  | ‘0001’B |  | CIoT_ControlPlane_PDN |
| Extended protocol configuration options |  | Not present |  | NOT (pc_NB or pc_NonIP_PDN) |
| Extended protocol configuration options |  |  |  | pc_NB or pc_NonIP_PDN |
| Configuration protocol |  | See Reference default EPS bearer context #1 in table 6.6.1-1 |  |  |
| Container ID n |  | ‘0003’H | n assigned to next available number | DNS IPv6 |
| Length of container ID n contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n contents |  | IPv6 address | DNS IPv6 Address |  |
| Container ID n+1 |  | ‘000D’H | n assigned to next available number | DNS IPv4 |
| Length of container ID n+1 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n+1 contents |  | IPv4 address | DNS IPv4 Address |  |
| Extended protocol configuration options |  | Not present |  | Ethernet |
| Serving PLMN rate control |  | Not present |  |  |
| Serving PLMN rate control |  | ‘FFFF’H | Maximum number of ESM DATA TRANSPORT messages allowed per 6 min interval is not restricted | CIoT_ControlPlane_PDN |

| Condition | Explanation |
| --- | --- |
| IPv4 | If in the last PDN CONNECTIVITY REQUEST sent prior to this message, the PDN type = '001'B |
| IPv6 | If in the last PDN CONNECTIVITY REQUEST sent prior to this message, the PDN type = '010'B |
| IPv4v6 | If in the last PDN CONNECTIVITY REQUEST sent prior to this message, the PDN type = '011'B |
| IPv4-DHCP | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message, the IE Protocol configuration options contains a configuration protocol option = '000B00H' ("IPv4 address allocation via DHCPv4", length of contents = 0). Note 1: This condition is used in conjunction with IPv4 or IPv4v6 as indicated in the "PDN address row" just above. Note 2: If both messages, PDN CONNECTIVITY REQUEST and ESM INFORMATION RESPONSE, are received and contain a Protocol configuration options IE then the IE from the message which is received later shall be used. |
| P-CSCF IPv6 | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message, the Protocol configuration options and the additional parameter list was included with a “P-CSCF IPv6 Address Request” Note 1: This condition is only applicable for UEs with IMS support (TS 36.523-2 A.4.4-1/25). Note 2: If both messages, PDN CONNECTIVITY REQUEST and ESM INFORMATION RESPONSE, are received and contain a Protocol configuration options IE then the IE from the message which is received later shall be used. |
| P-CSCF IPv4 | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message, the Protocol configuration options and the additional parameter list was included with a “P-CSCF IPv4 Address Request” Note 1: This condition is only applicable for UEs with IMS support (TS 36.523-2 A.4.4-1/25). Note 2: If both messages, PDN CONNECTIVITY REQUEST and ESM INFORMATION RESPONSE, are received and contain a Protocol configuration options IE then the IE from the message which is received later shall be used. |
| DNS IPv6 | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message, the Protocol configuration options and the additional parameter list was included with a “DNS IPv6 Address Request”. Note: If both messages, PDN CONNECTIVITY REQUEST and ESM INFORMATION RESPONSE, are received and contain a Protocol configuration options IE then the IE from the message which is received later shall be used. |
| DNS IPv4 | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message, the Protocol configuration options and the additional parameter list was included with a “DNS IPv4 Address Request”. Note: If both messages, PDN CONNECTIVITY REQUEST and ESM INFORMATION RESPONSE, are received and contain a Protocol configuration options IE then the IE from the message which is received later shall be used. |
| APN_Provided | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message an Access Point Name was included NOTE: APN_Provided is equivalent to pc_Provide_IMS_APN for establishment of the first PDN connectivity during initial registration and if the first PDN is IMS (see table 4.7.3-14) |
| IMS_PDN_ConnEstab | If this message refers to establishment of connectivity to the IMS PDN (only applicable if the UE supports IMS according to TS 36.523-2 A.4.4-1/25). |
| WLAN | If the message is being sent for RAN Assisted WLAN interworking. |
| CIoT_ControlPlane_PDN | If the associated PDN connection is only for control plane CIoT optimization |
| CIoT_CP_NON_IP | If the UE requests a Non-IP type PDN |
| Interworking_with_5GS | If the UE has indicated support of N1, then the SS shall include these IEs to provide details for the interworking with 5GS. Applicable to 5G test cases defined in TS 38.523-1 [71]. |
| Ethernet | Applicable to ethernet testcases. |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

### ---------------------------------------- End of first change ------------------------------------

### --------------------------------------- Start of second change -------------------------------

#### -	PDN CONNECTIVITY REQUEST

This message is sent by the UE to the SS.

Table 4.7.3-20: PDN CONNECTIVITY REQUEST

| Derivation Path: 24.301 clause 8.3.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | Any value from 1 to 254 |  |  |
| PDN connectivity request message identity |  | '1101 0000'B | PDN connectivity request |  |
| Request type |  | '0001'B | initial request |  |
| PDN type |  | Any value between '001'B, '010'B, '011'B and '100'B | The allowed values are respectively IPv4, IPv6, IPv4v6 and "unused but interpreted as IPv6 by the network" |  |
| PDN type |  | ‘101’B | Non-IP | CIoT_CP_NON_IP |
| PDN type |  | ‘110’B | Ethernet | Ethernet |
| ESM information transfer flag |  | Not present or any allowed value |  |  |
| Access point name |  | Not present | This IE shall not be included when the PDN CONNECTIVITY REQUEST message is sent together with an ATTACH REQUEST message. |  |
| Protocol configuration options |  | Not present or any allowed value | The value received from the UE does not affect the possible verdict associated with the message when received by the SS. The SS shall remember if this IE is present and its contents because this affects subsequent SS behaviour, e.g. coding of ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST. | NOT (pc_NB or pc_NonIP_PDN) |
| Protocol configuration options |  | Not present |  | pc_NB or pc_NonIP_PDN |
| Protocol configuration options |  | Not present |  | Ethernet |
| Protocol configuration options |  | If present including at least the following container |  | Interworking_with_5GS |
| Container ID n |  | ‘001A’H | n assigned to next available number |  |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | PDU session ID | PDU session ID assigned by the UE |  |
| NBIFOM container |  | Not present or any allowed value |  |  |
| Header compression configuration |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | If present including at least the following container |  | Interworking_with_5GS |
| Container ID n |  | ‘001A’H | n assigned to next available number |  |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | PDU session ID | PDU session ID assigned by the UE |  |

| Condition | Explanation |
| --- | --- |
| CIoT_CP_NON_IP | If the UE requests a Non-IP type PDN |
| Interworking_with_5GS | If the UE has indicated support of N1, then the UE shall include this IE to provide details for the interworking with 5GS. Applicable to 5G test cases defined in TS 38.523-1 [71]. Note: The PDU session ID container shall be sent in either the Protocol configuration options or the Extended protocol configuration options IE. |
| Ethernet | Applicable to ethernet testcases. |

NOTE:	This message is sent without integrity protection before NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED NAS MESSAGE message after NAS security mode control procedure has been successfully completed.

### ------------------------------------ End of second change -----------------------------------