3GPP TSG RAN WG5#99	R5-233745

# Incheon, Korea, May 22 – 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0657 | rev | 1 | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test case 5.3.2.2.4, 2Rx TDD FR1 PDCCH performance for RedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case 5.3.2.2.4 is missing in 38.521-4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test case 5.3.2.2.4 in 38.521-4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RedCap WI cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.2.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | MU and TT in R5-232574 This CR is dependent on R4-2307238 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of R5-232568 and the result of 2 revisions. |  |  |  |  |  |  |  |  |

#### << Start of changes >>

5.3.2.2.3.5	Test requirement

Table 5.3.2.2.3.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.3.5-1.

Table 5.3.2.2.3.5-1: Minimum performance with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x2 Low | 1 | 3.9 |

##### 5.3.2.2.4	2Rx TDD FR1 PDCCH performance for RedCap

Editor's Note: This test cases is incomplete in following aspects:

##### - SNR in test requirements table is within square brackets.

5.3.2.2.4.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 1 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.2.4.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.2.4.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support NR RedCap.

5.3.2.2.4.3	Minimum conformance requirements

For the parameters specified in Table 5.3.2.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.4.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.2.2.4.3-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 20 | 48 | 1 | 4 | R.PDCCH. 2-1.5 TDD | TDLC300-100 | 1x2 Low | 1 | 3.6 |
| 2 | 20 | 48 | 1 | 8 | R.PDCCH. 2-1.6 TDD | TDLC300- 100 | 2x2 Low | 1 | 0.0 |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.3.2.2.4.

5.3.2.2.4.4	Test description

5.3.2.2.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A in Figure A.3.1.7.1 for TE diagram and clause A.3.2.3 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1 and Table 5.3.2.2.4.3-1 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without Release On, Test Mode On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.2.2.4.4.3

5.3.2.2.4.4.2	Test procedure

1.	SS transmits PDCCH with DCI format as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.2.2.4.3-1. The details of PDCCH are specified in Table 5.3.1, Table 5.3.1.1-1, Table 5.3.2.2.4.3-1 respectively. The details of PDSCH are specified in Table A.3.3.2.2-3. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.2.2.4.4.4-1 as appropriate.

3. 	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.2.2.4.4.4-1, pass the UE. Otherwise fail the UE.

5.3.2.2.4.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2 with the following exceptions:

Table 5.3.2.2.4.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP |  |
| Duration | 1 | SearchSpace duration of 1 symbol |  |
| } |  |  |  |

Table 5.3.2.2.4.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 with condition USS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel4 | n1 | AL4 | Test 1 |
| aggregationLevel8 | n1 | AL8 | Test 2 |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 | Long_DCI |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.4.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  |  |
| } |  |  |  |

5.3.2.2.4.4.4	Test requirement

Table 5.3.2.2.4.4.4-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.4.4.4-1.

Table 5.3.2.2.4.4.4-1: Test Requirement for for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 20 | 48 | 1 | 4 | R.PDCCH. 2-1.5 TDD | TDLC300-100 | 1x2 Low | 1 | 4.2 |
| 2 | 20 | 48 | 1 | 8 | R.PDCCH. 2-1.6 TDD | TDLC300- 100 | 2x2 Low | 1 | 0.9 |

#### << End of changes >>