3GPP TSG-RAN5 Meeting #99	R5-232124r1

# Incheon, South Korea, 22 – 26 May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0913 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding RedCap UE FR2 PC7 Carrier leakage requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | FR2 Redcap PC7 UE Carrier leakage requirements have not been specified in TS 38.521-2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding RedCap UE FR2 PC7 Carrier leakage requirements into clause 6.4.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test requirements for RedCap UE FR2 PC7 Carrier leakage requirement will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Add PC7 to editor’s notes. Change TTs to FFS in Table 6.4.2.2.5-7b. Remove change to Table 6.4.2.2.4.1-1 (which is covered in R5-232622) |  |  |  |  |  |  |  |  |

-------- Start of changes --------

#### 6.4.2.2	Carrier leakage

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainty and Test Tolerance are FFS for power class 1, 2, 4, 6 and 7.

- The test case is incomplete for band n259.

6.4.2.2.1	Test purpose

Carrier leakage expresses itself as unmodulated sine wave with the carrier frequency. It is an interference of approximately constant amplitude and independent of the amplitude of the wanted signal. Carrier leakage interferes with the sub carriers at its position (if allocated), especially, when their amplitude is small.

The purpose of this test is to exercise the UE transmitter to verify its modulation quality in terms of carrier leakage.

6.4.2.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.4.2.2.3	Minimum conformance requirements

Carrier leakage is an additive sinusoid waveform. The carrier leakage requirement is defined for each component carrier. The measurement interval is one slot in the time domain. The relative carrier leakage power is a power ratio of the additive sinusoid waveform to the power in the modulated waveform.

The requirement is verified with the test metric of Carrier Leakage (Link=TX beam peak direction, Meas=Link angle).

When carrier leakage is contained inside the spectrum confined within the configured UL and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4.2.2.3-1 for power class 1 UEs.

Table 6.4.2.2.3-1: Minimum requirements for relative carrier leakage power for power class 1

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 17 dBm | -25 |
| 4 dBm ≤ EIRP ≤ 17 dBm | -20 |

When carrier leakage is contained inside the spectrum occupied by the configured UL CCs and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4.2.2.3-2 for power class 2.

Table 6.4.2.2.3-2: Minimum requirements for relative carrier leakage power for power class 2

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 6 dBm | -25 |
| -13 dBm ≤ EIRP ≤ 6 dBm | -20 |

When carrier leakage is contained inside the spectrum occupied by the configured UL CCs and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4.2.2.3-3 for power class 3 UEs.

Table 6.4.2.2.3-3: Minimum requirements for relative carrier leakage power for power class 3

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 0 dBm | -25 |
| -13 dBm ≤ EIRP ≤ 0 dBm | -20 |

When carrier leakage is contained inside the spectrum occupied by the configured UL CCs and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4.2.2.3-4 for power class 4.

Table 6.4.2.2.3-4: Minimum requirements for relative carrier leakage power for power class 4

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 11 dBm | -25 |
| -13 dBm ≤ EIRP ≤11 dBm | -20 |

The normative reference for this requirement is TS 38.101-2[3] clause 6.4.2.2.

When carrier leakage is contained inside the spectrum occupied by the configured UL CCs and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4.2.2.3-6 for power class 6.

Table 6.4.2.2.3-6: Minimum requirements for relative carrier leakage power for power class 6

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 7 dBm | -25 |
| -6 dBm ≤ EIRP ≤ 7 dBm | -20 |

When carrier leakage is contained inside the spectrum occupied by the configured UL CCs and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4.2.2.3-7 for power class 7.

Table 6.4.2.2.3-7: Minimum requirements for relative carrier leakage power for power class 7

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 0 dBm | -25 |
| -13 dBm ≤ EIRP ≤ 0 dBm | -20 |

6.4.2.2.4	Test description

6.4.2.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in Table 6.4.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4.2.2.4.1-1: Test Configuration

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Mid |  |
| Test SCS as specified in Table 5.3.5-1 |  | Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | - | Modulation | RB allocation (NOTE 1, 3) |
| 1 |  | DFT-s-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 and PC6 Inner_Partial_Left_Region2 for PC1 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4 and PC6 or Table 6.1-2 for PC1. NOTE 2: Test Channel Bandwidths are checked separately for each NR band, which applicable channel bandwidths are specified in Table 5.3.5-1. NOTE 3: When the signalled DC carrier position is at Inner_Partial_Left for PC2, PC3, PC4, PC6, use Inner_Partial_Right for UL RB allocation. When the signalled DC carrier position is in Inner_Partial_Left_Region2 for PC1, use Inner_Partial_Right_Region2 for UL RB allocation. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, in Figure A.3.3.1.1 for TE diagram and section A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.4.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.4.2.2.4.3.

7.	In case the parameter 3300 or 3301 is reported from the UE via txDirectCurrentLocation IE, do not proceed to test procedure and mark the test not applicable with reasoning in the test report.

6.4.2.2.4.2	Test procedure

1.	Retrieve the LO position from the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.4.2.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the UE in the Inband Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	Send uplink power control commands to the UE using 1dB power step size to ensure that the UE EIRPTotal = EIRPθ  + EIRPφ measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level Preq, where:

-	Preq is the power level specified in Table 6.4.2.2.4.2-1 according to the power class.

-	MU is the test system uplink absolute power measurement uncertainty and is specified in Table F.1.2-1 under carrier leakage sub-clause for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 5 dB (UE power step tolerance) + (Test system uplink relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 5dB for 1dB power step size, and the Test system uplink relative power measurement uncertainty is specified in Table F.1.2-1.

Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

5.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition TxRx.

6.	Measure carrier leakage using Global In-Channel Tx-Test (Annex E) for the - and -polarization at the LO position obtained in step 1. For TDD, only slots consisting of only UL symbols are under test. Calculate CarrLeak = min(CarrLeakθ , CarrLeakφ).

7.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

NOTE 2:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no less than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.2.

Table 6.4.2.2.4.2-1: UE EIRP Preq (dBm) for carrier leakage

| Power Class | Preq (dBm) for step 3 |
| --- | --- |
| Power Class 1 | 17 |
| Power Class 2 | 6 |
| Power Class 3 | 0 |
| Power Class 4 | 11 |
| Power Class 6 | 7 |
| Power Class 7 | 0 |

Table 6.4.2.2.4.2-2: Void.

6.4.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

6.4.2.2.5	Test requirement

For each of the n carrier leakage results derived in Annex E.3.1 for θ- and φ-polarization the minimum is calculated according to

CarrLeak = min(CarrLeakθ , CarrLeakφ), where

.

Each of the n carrier leakage results CarrLeak shall not exceed the values in Table 6.4.2.2.5-1 to Table 6.4.2.2.5-4. Allocated RBs are not under test.

Table 6.4.2.2.5-1a: Test requirements for relative carrier leakage power for power class 1

| Parameter | Relative limit (dBc) |
| --- | --- |
| 17 dBm + MU < EIRP ≤ 17 dBm + MU + Uplink power control window size | -25 + TT |

Table 6.4.2.2.5-1b: Test Tolerance (carrier leakage for power class 1)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | TBD | TBD |

Table 6.4.2.2.5-2a: Test requirements for relative carrier leakage power for power class 2

| Parameter | Relative limit (dBc) |
| --- | --- |
| 6 dBm + MU < EIRP ≤ 6 dBm + MU + Uplink power control window size | -25 + TT |

Table 6.4.2.2.5-2b: Test Tolerance (carrier leakage for power class 2)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | TBD | TBD |

Table 6.4.2.2.5-3a: Test requirements for relative carrier leakage power for power class 3

| Parameter | Relative limit (dBc) |
| --- | --- |
| 0 dBm + MU < EIRP ≤ 0 dBm + MU + Uplink power control window size | -25 + TT |

Table 6.4.2.2.5-3b: Test Tolerance (carrier leakage for power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 3.54 dB | 3.62 dB |

Table 6.4.2.2.5-4a: Test requirements for relative carrier Leakage Power for power class 4

| Parameter | Relative limit (dBc) |
| --- | --- |
| 11 dBm + MU < EIRP ≤ 11 dBm + MU + Uplink power control window size | -25 + TT |

Table 6.4.2.2.5-4b: Test Tolerance (carrier leakage for power class 4)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | TBD | TBD |

Table 6.4.2.2.5-5a: FFS

Table 6.4.2.2.5-5b: FFS

Table 6.4.2.2.5-6a: Test requirements for relative carrier Leakage Power for power class 6

| Parameter | Relative limit (dBc) |
| --- | --- |
| 7 dBm + MU < EIRP ≤ 7 dBm + MU + Uplink power control window size | -25 + TT |

Table 6.4.2.2.5-6b: Test Tolerance (carrier leakage for power class 6)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | TBD | TBD |

Table 6.4.2.2.5-7a: Test requirements for relative carrier leakage power for power class 7

| Parameter | Relative limit (dBc) |
| --- | --- |
| 0 dBm + MU < EIRP ≤ 0 dBm + MU + Uplink power control window size | -25 + TT |

Table 6.4.2.2.5-7b: Test Tolerance (carrier leakage for power class 7)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | FFS dB | FFS dB |

-------- End of changes --------