3GPP TSG-RAN5 Meeting #99	R5-232275r1

# Incheon, South Korea 22 – 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2366 | rev | - | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2 test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Propagation delay compensation test cases have been defined in 38.133 but are currently missing in RAN5 test specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added minimum conformance requirements and corresponding PDC test case 7.6.13.2. - Added cell mapping configuration for above test case in Annex E.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would be missing in the test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.13.2(new), E.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0272 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Corrected AoA setup reference in Table A.7.6.13.2.1.5-1 |  |  |  |  |  |  |  |  |

### 7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2

-------- Start of first change --------

### 7.6.13.2	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD:

-	Test purpose contains [] and reference clause with X

- Measurement Uncertainties and Test Tolerances are missing

- Annex F

##### 7.6.13.2.0	Minimum conformance requirements

[38.133 clause 9.12.4.2, 9.12.5 and 10.1.25.3.1] for measurement time

When UE is configured with csi-RS-Ref-r17 in MeasObjectRxTxDiff-r17 defined in TS 38.331 [2] and provided with TRS resource configuration, the UE shall be able to measure UE Rx-Tx time difference on PCell within the measurement period TUERx-Tx_TRS, where:

TUERxTx,TRS=Nsample *Klayer3*LCM(TTRS, TDRX)

Where

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample is [4],

TTRS is the TRS resource periodicity specific for RTT-based propagation delay compensation,

TDRX is the DRX cycle length when DRX is in use, 1ms otherwise.

Klayer3 is

Ntotal / Navailable when Navailable>0

Where,

-	For a window W of duration LCM(TTRS, MGRP_max, TSMTC) and starting at the beginning of any TRS resource occasion, where TSMTC is the periodicity of SMTC for intra-frequency measurement without gap and MGRP_max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as PCell for a UE that support concurrentMeasGap-r17 and is configured with concurrent measurement gaps, otherwise MGRP_max is the MGRP of configured per-UE measurement gap or per-FR measurement gap within the same FR as PCell:

-	Ntotal is the total number of TRS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Navailable is the number of TRS resource occasions that are not overlapped with

-	For FR2: any measurement gap occasions after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.8.3 nor any SMTC occasions within the window W.

No requirements shall apply if Navailable=0.

UE is only required to perform UE Rx-Tx time difference on TRS within the active DL BWP. When UE is configured to perform UE Rx-Tx time difference measurement based on TRS, the requirements apply provided that the SCS of the TRS is same as that of the active BWP on PCell.

For FR2, the requirements apply if QCL information is provided for all the TRS resources in the resource set and for each resource one RS has QCL-TypeD.

If TRS resources overlap with Type 1A/1B/2 PPW, the UE is allowed longer measurement period to measure UE Rx-Tx time difference on PCell.

The UE Rx-Tx time difference measurement values contained in measurement report shall be based on the

measurement report mapping requirements specified in clause TS 38.133 10.1.25.3, with k = 5.

The reporting range for the absolute UE Rx-Tx time difference measurement (TUE Rx-Tx) is defined from -985024×Tc to 985024×Tc with the resolution step of 2k×Tc.

Table 7.6.13.2.0-1: Absolute UE Rx-Tx time difference measurement report mapping for k=5

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -984992 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -984992  TUE Rx-Tx < -984960 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_30782 | -32  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_30783 | 0  TUE Rx-Tx < 32 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_61563 | 984960  TUE Rx-Tx < 984992 | Tc |
| RX-TX_TIME_DIFFERENCE_61564 | 984992  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_61565 | 985024  TUE Rx-Tx | Tc |

[TS 38.133 clause 10.1.39.3] for measurement accuracy

The error in the reported value of UE Rx-Tx time difference measurement, including both the measurement error and the reporting quantization error, should be within the accuracy requirements specified in this clause.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall not apply, if:

-	NTA_offset defined in TS 38.133 Table 7.1.2-2 changes during the UE Rx-Tx measurement period or

-	if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that:

-	The UE transmits SRS within [-160, 160] msec of at least one PDC TRS resource from the serving cell (PCell).

When a serving cell change occurs during the UE Rx-Tx measurement period, the UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx measurement.

The accuracy requirements in 7.6.13.2.0-2 for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to TS 38.133 Annex B.2.13 for a corresponding Band.

AWGN propagation condition.

Table 7.6.13.2.0-2: UE Rx-Tx time difference measurement accuracy in FR2 in AWGN

| Accuracy | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | TRS Ês/Iot | Minimum TRS bandwidth | TRS SCS | IoNote 3 range |  |
|  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 4 | dB | RB | kHz | dBm / SCSTRS | dBm/BWChannel |
| [29+d+ℇ] | -3 | ≥[24] | 60 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| [14+d+ℇ] |  | ≥[64] |  | NOTE 5 | NOTE 5 |
| [7+d+ℇ] |  | ≥[132] |  | NOTE 5 | NOTE 5 |
| [15+d+ℇ] |  | ≥[32] | 120 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| [7+d+ℇ] |  | ≥[64] |  | NOTE 5 | NOTE 5 |
| [4+d+ℇ] |  | ≥[128] |  | NOTE 5 | NOTE 5 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: The Io is defined in TRS slots. The same Io range applies to TRS and non-TRS symbols. Io levels are different in TRS and non-TRS symbols within the same slot. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the TRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: d is the margin determined from Table 10.1.39.3-4. NOTE 7: ℇ is the margin for reporting quantitization error and ℇ=16 Tc. |  |  |  |  |  |

Table 7.6.13.2.0-3: Margin for UE Rx-Tx time difference measurement accuracy in FR2

| Min(TRS BW, SRS BW) (MHz) |  | Margin (Tc Note 1) |
| --- | --- | --- |
| SCS = 60 kHz | SCS = 120 kHz |  |
| ≥ 24 | N/A | [76] |
| ≥ 64 | ≥ 32 | [32] |
| ≥ 132 | ≥ 64 | [24] |
| N/A | ≥ 128 | [20] |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and TRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |

##### 7.6.13.2.1	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2 SA

7.6.13.2.1.1	Test purpose

The purpose of the test is to verify that the UE Rx-Tx measurement for RTT-based PDC meets the requirements specified in TS 38.133[6] clause 9.12.4.2 for measurement delay and clause 10.1.X.2 for measurement accuracy in AWGN propagation condition in FR2 in standalone scenario.

7.6.13.2.1.2	Test applicability

This test applies to all types of NR UE release 17 onwards supporting rtt-BasedPDC-CSI-RS-ForTracking-r17 or gNB-SideRTT-BasedPDC-r17.

7.6.13.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.13.2.0.

The normative reference for this requirement is TS 38.133 [6] clauses 9.12.4.2, 9.12.5, 10.1.39.3 and A.7.6.13.2.

7.6.13.2.1.4	Test description

7.6.13.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.13.2.1.4.1-1

Table 7.6.13.2.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 7.6.13.2-1 | 120 kHz SSB and PRS SCS, 100 MHz bandwidth, TDD duplex mode |

Configure the test requirement and the DUT according to the parameters in Table 7.6.13.2.1.4.1-2.

Table 7.6.13.2.1.4.1-2: Initial conditions for UE Tx-Rx time difference measurement for SA FR2

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.13.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1. The general test parameter settings are set up according to Table 7.6.13.2.1.4.1-3.

2. Message contents are defined in clause 7.6.13.2.1.4.3.

3. The AoA setup for this test is Setup 1 as defined in clause A.9. The UE RX Beam Peak direction has been obtained previously using one of the search procedures as described in Annex I

Table 7.6.13.2.1.4.1-3: General test parameters

| Parameter | Unit | Test configuration | Value |
| --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |
| RF Channel Number |  | 1 | 1 |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |
| SSB configuration |  | 1 | SSB.2 FR2 |
| SMTC configuration |  | 1 | SMTC.1 |
| Measurement gap |  | 1 | GP#24 or GP#13 Note 1 |
| CP length |  | 1 | Normal |
| DRX |  | 1 | OFF |
| T1 | s | 1 | 5 |
| T2 | s | 1 | 20 |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#13 is configured. |  |  |  |

7.6.13.2.1.4.2	Test procedure

The test is considered with one cell (Cell 1) in FR2.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 does not have TRS transmission during T1 and transmits TRS during T2.

The measurement control information with MeasObjectRxTxDiff set to ‘csi-RS-Ref’ as defined in clause 7.6.13.2.4.3 shall be provided to the UE during T1. The last TTI containing the RRC configuration shall be provided to the UE DT ms before the start of T2, where DT = [10] ms is the maximum processing time of the measurement request. The UE is configured to transmit SRS during T2.

The general and cell specific test parameters for PCell is given in Table 7.6.13.2.1.4.1-3 and Table 7.6.13.2.1.5-1, respectively. The test consists of two sub-tests with different TRS BW.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.13.2.1.5-1 as appropriate.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.13.2.1.5-1. T2 starts.

6. The UE shall transmit periodically MeasurementReport messages.

7. After 10s wait from Step 3, the SS shall check the result-k5-r17 reported values in the periodic MeasurementReport. The result-k5 value is outside the limits in Table 7.6.13.2.0-1 or the UE fails to report the measurement value for serving cell, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

8. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

7.6.13.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table 4.6.3-76B in 38.508-1 [14] with condition CSI-RS_REF |

7.6.13.2.1.5	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in 7.6.13.2.0

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.39.3.

The reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified [in clause 10.1.25.3.1, for k=5].

For the test to pass, the ratio of successful reported values shall be more than 90% with a confidence level of 95%.

Table 7.6.13.2.1.4.1-3 and Table 7.6.13.2.1.5-1 define the primary level settings including test tolerances for UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR2 SA.

Table A.7.6.13.2.1.5-1: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| AoA setup |  | 1 | Setup 1 as specified in clause A.9.1 |  |
| Beam AssumptionNote 7 |  | 1 | Rough |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |
| OCNG Patterns |  | 1 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.2.1 TDD |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |
| SRS configuration |  | 1 | PDC-SRS.3 |  |
| Note 2 | dBm/SCS | 1 | -89 + TT |  |
| Note 2 | dBm/15 kHz | 1 | -98 + TT |  |
| TRS | dB | 1 | -Infinity | -[2.41] + TT |
| TRS | dB | 1 | -Infinity | -[2] + TT |
| TRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -[91] + TT |
| Io | dBm/95.04 MHz | 1 | N/A | -[57.63] + TT |
| Propagation Condition |  | 1 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: TRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: TRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in TS 38.133 B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

Table 7.6.13.2.1.5-2: SRS Configuration for FR2 Propagation Delay Compensation

|  | PDC-SRS.3 |  |
| --- | --- | --- |
| Field |  | Comment |
| c-SRS | Same as NRB,c in the test case |  |
| b-SRS | 0 |  |
| b-hop | 0 | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | SSB #0 is used for SRS path loss estimation |
| usage | usagePDC-r17 |  |
| startPosition | 5 | resourceMapping setting |
| nrofSymbols | 4 |  |
| repetitionFactor | n1 | without repetition. |
| transmissionComb | n4 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| resourceType | Periodic |  |
| periodicityAndOffset-p | Sl1280, 160 | SRS transmission periodicity |

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

----- End of first change ------

----Unchanged Clauses Skipped -----

-------- Start of Second change --------

# E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7

Table E.5-1 defines the cell configuration mapping for NR/5GC FR2 test cases in chapter 7 of this test specification.

Table E.5-1: Cell configuration mapping for SA FR2 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 7.1.1.1 | NR SA FR2 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |
| 7.1.1.2 | NR SA FR2-FR2 cell re-selection | NR Cell 1 | NR Cell 23 |  |  |  |
| 7.1.1.3 | NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.4 | NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.5 | NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.6 | NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.1.1 | NR SA FR1-FR2 Inter-frequency handover; unknown target cell | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.1.2 | NR SA FR2 Intra-frequency handover; unknown target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.1.3 | NR SA FR2-FR2 Inter-frequency handover; unknown target cell | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.1.4 | NR SA FR1-FR2 synchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.3.1.5 | NR SA FR1-FR2 asynchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.3.2.1.1 | NR SA FR2 RRC re-establishment | NR Cell 1 |  |  |  |  |
| 7.3.2.1.2 | NR SA FR2-FR2 re-establishment | NR Cell 1 |  |  |  |  |
| 7.3.2.1.3 | NR SA FR2 RRC re-establishment without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.2.2.1 | NR SA FR2 contention based random access | NR Cell 1 |  |  |  |  |
| 7.3.2.2.2 | NR SA FR2 non-contention based random access | NR Cell 1 |  |  |  |  |
| 7.3.2.2.4 | NR SA FR2 2-step non-contention based random access | NR Cell 1 |  |  |  |  |
| 7.3.3.1 | NR SA FR2 conditional handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.3.2 | NR SA FR2-FR2 conditional handover | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.4.1.1 | NR SA FR2 UE transmit timing accuracy | NR Cell 1 |  |  |  |  |
| 7.4.3.1 | NR SA FR2 timing advance adjustment accuracy | NR Cell 1 |  |  |  |  |
| 7.5.1.1 | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 7.5.1.2 | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 7.5.1.3 | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |
| 7.5.1.4 | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |
| 7.5.1.9 | NR SA FR2 radio link monitoring UE scheduling restrictions | NR Cell 1 |  |  |  |  |
| 7.5.5.1 | NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.2 | NR SA FR2 SSB-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.3 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.4 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.6 | NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in non-DRX | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.5.5.7 | NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in DRX | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.5.6.1 | NR SA FR2 DCI-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |
| 7.6.1.1 | NR SA FR2 event-triggered reporting without gap in non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.2 | NR SA FR2 event-triggered reporting without gap in DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.3 | NR SA FR2 event-triggered reporting with gap in non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.4 | NR SA FR2 event-triggered reporting with gap in DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.2.1 | NR SA FR2-FR2 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.2 | NR SA FR2-FR2 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.3 | NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.4 | NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.5 | NR SA FR1-FR2 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.6 | NR SA FR1-FR2 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.7 | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.8 | NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.3.1 | NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.2 | NR SA FR2 SSB-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.3 | NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.4 | NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |
| 7.6.6.1 | NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.6.6.2 | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.6.6.3 | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.6.13.1 | UE Rx-TX time difference measurement for propagation delay compensation using PRS in FR2 | NR cell 1 |  |  |  |  |
| 7.6.13.2 | UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2 | NR Cell 1 |  |  |  |  |
| 7.7.1.1 | NR SA FR2 SS-RSRP measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |
| 7.7.1.2 | NR SA FR2-FR2 SS-RSRP measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.7.1.3.1 | NR SA FR1-FR2 SS-RSRP absolute measurement accuracy | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.7.1.3.2 | Void |  |  |  |  |  |
| 7.7.2.1 | NR SA FR2 SS-RSRQ measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.7.2.2 | NR SA FR2-FR2 SS-RSRQ measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.7.3.1 | NR SA FR2 SS-SINR measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.7.3.2 | NR SA FR2-FR2 SS-SINR measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.7.6.1 | NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.7.6.2 | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.7.6.3 | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |

----- End of Second change ------