3GPP TSG-RAN5 Meeting #99	R5-232601r1

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0310 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to MCData-Info from the UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Pre-established sessions and functional alias were added to MCData in Rel-16. The MCData-Info information element needed to be updated to accommodate these additions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the FUNCTIONAL_ALIAS condition and edited the mcdata-calling-user-id IE with the condition. Added the PRE_ESTABLISHED_SESSION condition. Added the anyExt IE with child elements "pre-established-session-ind" and "functional-alias-URI". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCData pre-established sessions and MCData functional alias may not be tested correctly and could lead to false test results. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Removed the FUNCTIONAL_ALIAS condition for the “mcdata-calling-user-id” IE in Table 5.5.3.2.1-3 |  |  |  |  |  |  |  |  |

<START OF CHANGES>

#### 5.5.3.2	MCS Info Lists

##### 5.5.3.2.1	MCS Info Lists from the UE

…

##### -	MCData

Table 5.5.3.2.1-3: MCData-Info from the UE

| Derivation Path: TS 24.282 [87], Clause D.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-access-token | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcdata-access-token> with mcdataString set to access token as assigned to the UE in the Token Response | The access token is opaque to the MCData client | TS 33.180 [94], clause B.4 RFC 6749 [77] | CONFIG GROUPCONFIG |
| request-type | not present |  |  |  |
|  | "one-to-one-sds" |  |  | MCD_1to1 |
|  | “group-sds” |  |  | MCD_grp |
| mcdata-request-uri | not present |  |  |  |
|  | Encrypted (NOTE 1) <mcdata-request-uri> with mcdataURI set to px_MCData_Group_A_ID |  |  | MCD_grp |
|  | Encrypted (NOTE 1) <mcdata-request-uri> with mcdataURI set to px_MCData_Group_A_ID |  |  | POC-SETTINGS-EVENT |
| mcdata-calling-user-id | not present |  |  |  |
|  | encrypted (NOTE 1) mcdata-calling-user-id with mcdataURI set to px_MCData_ID_User_A |  |  | FUNCTIONAL_ALIAS |
| mcdata-called-party-id | not present |  |  |  |
| mcdata-calling-group-id | not present |  |  |  |
| alert-ind | not present |  |  |  |
| originated-by | not present |  |  |  |
| mcdata-client-id | not present |  |  |  |
|  | Encrypted (NOTE 1) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) |  |  | MCD_grp |
|  | Encrypted (NOTE 1) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) |  |  | CONFIG AND PUBLISH |
|  | not present or encrypted (NOTE 1) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) | in general mcdata-client-id is not mandatory (e.g. for SIP SUBSCRIBE) |  | (CONFIG OR GROUPCONFIG) AND NOT REGISTER (NOTE 3) |
|  | Encrypted (NOTE 1) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) | mcdata-client-id is mandatory in SIP PUBLISH for MCData service settings only, according to TS 24.282 [87] clause 7.2.3 | RFC 4122 [106] | POC-SETTINGS-EVENT |
| mcdata-controller-psi | not present |  |  |  |
| anyExt |  |  |  |  |
| pre-established-session-ind | “true” | TS 24.282 [87], Clause 18.3.2.1 |  | PRE_ESTABLISHED_SESSION |
| functional-alias-URI | encrypted (NOTE 1) <functional-alias-URI> with mcdataURI set to px_MCData_ID_FA_A | set to the value of the functional alias that is used together with the "mcdata-calling-user-id" |  | FUNCTIONAL_ALIAS |
| NOTE 1: The SS shall check the mcvideo-client-id - at the first time being sent by the UE to be a valid UUID URN with a format like "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" according to RFC 4122 [106] - to be all the same UUID URN in subsequent messages. NOTE 2: Encrypted element as described in Table 5.5.3.2.1-3A NOTE 3: In contrast to MCPTT and MCVideo for MCData TS 24.282 [87] clause 7.2.1 does not specify the client-id to be included in the REGISTER request. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| MCD_1to1 | A one-to-one MCData call |
| MCD_grp | A goup MCData call |
| REGISTER | MCData-Info in SIP REGISTER request for service authorisation |
| PUBLISH | MCData-Info in SIP PUBLISH request for service authorisation |
| PRE_ESTABLISHED_SESSION | A pre-established sessions is being established |
| FUNCTIONAL_ALIAS | An active Functional Alias is used |
| For further conditions see table 5.5.1-1 |  |

Table 5.5.3.2.1-3A: Encrypted MCData info parameter sent by the UE

| Derivation Path: TS 24.282 [87] clauses D.1.2, D.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the mcdata parameter |  |  |  |

<END OF CHANGES>