3GPP TSG-RAN5 Meeting #99	R5-232678r2

# Incheon, Korea (Republic Of), 22nd May 2023 – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2422 | rev | - | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex E and F updates for CLI-based test cases including TTs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CLI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | (1) CLI test cases 4.6.5.1, 4.7.6.1, 6.6.6.1 and 6.7.8.1 have been added to the test specification, along with their corresponding TTs and Annex E and F need to be updated accordingly. (2) Annex F.1.1.2 presents the list of measurement uncertainties applicable to the different test scenarios in the RRM specification. According to 38.215, CLI-based measurements, such as SRS-RSRP are defined (and expected to be measured) in SRS resources: SRS reference signal received power (SRS-RSRP) is defined as linear average of the power contributions (in [W]) of the resource elements carrying sounding reference signals (SRS). SRS-RSRP shall be measured over the configured resource elements within the considered measurement frequency bandwidth in the configured measurement time occasions. However, the current measurement uncertainty description only considers SSB and CSI-RS based measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - (1) Updated the Tables in Annex E and F corresponding to CLI test cases 4.6.5.1, 4.7.6.1, 6.6.6.1 and 6.7.8.1. - (2) Updated the measurement uncertainty description to include the configured RBs in the SRS resource for CLI-based measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases would be incomplete and test uncertanties would not be defined for CLI-based measurement test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.2, E.4, F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: updated spec version R2: fixed a typo in the ‘Reason for change’ section of the coversheet |  |  |  |  |  |  |  |  |

<< Start of Changes >>

# E.2	Cell configuration mapping for EN-DC FR1 test cases in Chapter 4

Table E.2-1 defines the cell configuration mapping for EN-DC FR1 test cases in chapter 4 of this test specification.

Table E.2-1: Cell configuration mapping for EN-DC FR1 RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 | EN-DC FR1 contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.3.2.2.2 | EN-DC FR1 non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.3.2.2.3 | EN-DC FR1 2-step contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.3.2.2.4 | EN-DC FR1 2-step non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.4.1.1 | EN-DC FR1 UE transmit timing accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.4.3.1 | EN-DC FR1 timing advance adjustment accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.1 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.2 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.3 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.4 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.5 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.6 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.7 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.8 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.2.1 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.2.2 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.2.3 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.2.4 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.2.5 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 4.5.2.6 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 4.5.3.1 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.3.2 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.3.3 | EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.3.5 | Direct SCell activation at SCell addition of known SCell in FR1 | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.4.1 | EN-DC FR1 UE UL carrier RRC reconfiguration delay | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.5.5.1 | EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.2 | EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.3 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.4 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.5 | EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.6 | EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.6.1.1 | EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.6.1.2 | EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.5.6.2.1 | EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.7.1 | EN-DC FR1 addition and release delay of known PSCell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.8.1 | EN-DC FR1 interruptions at switching between two uplink carriers | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.1.1 | EN-DC FR1 event-triggered reporting without gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.2 | EN-DC FR1 event-triggered reporting without gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.3 | EN-DC FR1 event-triggered reporting with gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.4 | EN-DC FR1 event-triggered reporting with gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.5 | EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.6 | EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.7 | EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.8 | EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-Scell-r17 | LTE Cell 1 | NR Cell 6 | NR Cell 3 | NR Cell 12 |  |
| 4.6.2.1 | EN-DC FR1-FR1 event-triggered reporting in non-DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.2.2 | EN-DC FR1-FR1 event-triggered reporting in DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.2.5 | EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.2.6 | EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.2.9 | EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.4.1 | EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.4.2 | EN-DC FR1 SSB-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.4.3 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.4.4 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.5.1 | EN-DC FR1 SRS-RSRP measurement with non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.7.1 | EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.7.2 | EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.7.3 | EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.1.1.1 | EN-DC FR1 SS-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 489 | NR Cell 1 |  |  |
| 4.7.1.1.2 | EN-DC FR1 SS-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 489 | NR Cell 1 |  |  |
| 4.7.1.2.1 | EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.1.2.2 | EN-DC FR1-FR1 SS-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.2.1 | EN-DC FR1 SS-RSRQ measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.7.2.2.1 | EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.2.2.2 | EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.3.1 | EN-DC FR1 SS-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.7.3.2.1 | EN-DC FR1-FR1 SS-SINR absolute measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.3.2.2 | EN-DC FR1-FR1 SS-SINR relative measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.4.1.1 | EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.4.1.2 | EN-DC FR1 SSB based L1-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.4.2.1 | EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.4.2.2 | EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.5.1 | EN-DC FR1 SFTD measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.6.1 | EN-DC SRS-RSRP measurement accuracy with FR1 serving cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.7.1 | EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.7.2 | EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.7.3 | EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |

<< Skipped sections >>

# E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6

Table E.4-1 defines the cell configuration mapping for NR/5GC FR1 test cases in chapter 6 of this test specification.

Table E.4-1: Cell configuration mapping for SA FR1 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 | NR SA FR1 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |  |
| 6.1.1.2 | NR SA FR1-FR1 cell re-selection | NR Cell 6 | NR Cell 23 |  |  |  |  |
| 6.1.1.3 | NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 11 |  |  |  |  |
| 6.1.1.4 | NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 11 |  |  |  |  |
| 6.1.1.5 | NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 23 |  |  |  |  |
| 6.1.1.6 | NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 23 |  |  |  |  |
| 6.1.1.7 | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | NR Cell 11 |  |  |  |  |
| 6.1.1.8 | NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17 | NR Cell 1 | NR Cell 11 |  |  |  |  |
| 6.3.1.1 | NR SA FR1 handover with known target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.2 | NR SA FR1 handover with unknown target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.3 | NR SA FR1-FR1 Handover with unknown Target Cell | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.1.7 | NR SA FR1 synchronous DAPS handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.8 | NR SA FR1 asynchronous DAPS handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.9 | NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.1.10 | NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.1.11 | NR SA FR1 Inter-band inter-frequency synchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 6.3.1.12 | NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 6.3.2.1.1 | NR SA FR1 RRC re-establishment | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.3.2.1.2 | NR SA FR1 - FR1 RRC re-establishment | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.3.2.1.3 | NR SA FR1 RRC re-establishment without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.3.2.2.1 | NR SA FR1 contention based random access | NR Cell 1 |  |  |  |  |  |
| 6.3.2.2.2 | NR SA FR1 non-contention based random access | NR Cell 1 |  |  |  |  |  |
| 6.3.2.2.3 | NR SA FR1 2-step contention based random access | NR Cell 1 |  |  |  |  |  |
| 6.3.2.2.4 | NR SA FR1 2-step non-contention based random access | NR Cell 1 |  |  |  |  |  |
| 6.3.2.3.1 | NR SA FR1 RRC connection release with redirection | NR Cell 6 | NR Cell 2 |  |  |  |  |
| 6.3.3.1 | NR SA FR1 conditional handover | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.3.3.2 | NR SA FR1-FR1 conditional handover | NR Cell 1 | NR Cell 6 |  |  |  |  |
| 6.4.3.1 | NR SA FR1 timing advance adjustment accuracy | NR Cell 1 |  |  |  |  |  |
| 6.5.1.1 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.2 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.3 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.4 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.5 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.6 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.7 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.8 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.2.1 | NR SA FR1 interruptions during measurements on deactivated NR SCC | NR Cell 1 | NR Cell 6 |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  | inter-band |  |
| 6.5.3.1 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  | inter-band |  |
| 6.5.3.2 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  | inter-band |  |
| 6.5.3.3 | NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | NR Cell 1 | NR Cell 6 |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  | inter-band |  |
| 6.5.3.4 | Direct SCell activation at SCell addition of known SCell in FR1 | NR Cell 1 | NR Cell 6 |  |  | intra-band |  |
| 6.5.4.1 | NR SA FR1 UE UL carrier RRC reconfiguration delay | NR Cell 1 | NR Cell 33 |  |  |  |  |
| 6.5.5.1 | NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.5.2 | NR SA FR1 SSB-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.5.3 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.5.4 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.5.5 | NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | NR Cell 1 | NR Cell 10 |  |  |  |  |
| 6.5.5.6 | NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | NR Cell 1 | NR Cell 10 |  |  |  |  |
| 6.5.6.1.1 | NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.5.6.1.2 | NR SA FR1 DCI-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.6.2.1 | NR SA FR1 RRC-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.7.1 | NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA | NR Cell 1 | NR Cell 10 |  |  | inter-band |  |
| 6.5.7.2 | NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA | NR Cell 1 | NR Cell 10 |  |  | inter-band |  |
| 6.5.8.1 | UE specific CBW change on PCell in FR1 in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.1.1 | SA event triggered reporting tests without gap under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.2 | SA event triggered reporting tests without gap under DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.3 | SA event triggered reporting tests with per-UE gaps under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.4 | SA event triggered reporting tests with per-UE gaps under DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.5 | SA event triggered reporting tests without gap under non-DRX with SSB index reading | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.6 | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.7 | NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.8 | NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17 | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.2.1 | NR SA FR1-FR1 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.2.2 | NR SA FR1-FR1 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.2.5 | NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.2.6 | NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.2.12 | NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.4.1 | NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.2 | NR SA FR1 SSB-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.3 | NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.4 | NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.5 | NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 |  |  |  |  |  |
| 6.6.6.1 | NR SA FR1 SRS-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.8.1 | NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.8.2 | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.8.3 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.9.1 | NR SA FR1 Idle mode CA/DC measurement for FR1 | NR Cell 1 | NR Cell 6 |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  | inter-band |  |
| 6.6.18.1 | NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers | NR Cell 1 | NR Cell 3 | NR Cell 6 |  |  |  |
| 6.6.18.2 | NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers | NR Cell 1 | NR Cell 3 | NR Cell 6 |  |  |  |
| 6.6.20 | UE Rx-Tx time difference measurement for propagation delay compensation | NR Cell 1 |  |  |  |  |  |
| 6.6.21 | UE Rx-Tx time difference measurement for propagation delay compensation with TRS | NR Cell 1 |  |  |  |  |  |
| 6.7.1.1.1 | NR SA FR1 SS-RSRP absolute measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |  |
| 6.7.1.1.2 | NR SA FR1 SS-RSRP relative measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |  |
| 6.7.1.2.1 | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.1.2.2 | NR SA FR1-FR1 SS-RSRP relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.1.3.1 | NR SA FR1-FR2 SS-RSRP absolute measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.1.3.2 | NR SA FR1-FR2 SS-RSRP relative measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.2.1 | NR SA FR1 SS-RSRQ measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.2.2.1 | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.2.2.2 | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.3.1 | NR SA FR1 SS-SINR measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.3.2.1 | NR SA FR1-FR1 SS-SINR absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.3.2.2 | NR SA FR1-FR1 SS-SINR relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.4.1.1 | NR SA FR1 SSB based L1-RSRP absolute measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.4.1.2 | NR SA FR1 SSB based L1-RSRP relative measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.4.2.1 | NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.4.2.2 | NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.8.1 | NR SA FR1 SRS-RSRP measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.9.1 | NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.9.2 | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.9.3 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |  |

<< Skipped sections >>

### F.1.1.2	Measurement of RRM requirements

This clause defines the maximum test system uncertainty for the RRM requirements. The maximum uncertainty values allowed for the typical RRM measurement uncertainty contributors is defined in Table F.1.1.2-1 and Table F.1.1.2-2. Unless explicitly stated for a particular test case, these maximum uncertainty values should be used as starting point to perform the test tolerance analysis in TR 38.903 [22] for each of the test cases. Specific test cases might require a tighter measurement uncertainty value for some of the contributors. Exceptions to the general values in Table F.1.1.2-1 and Table F.1.1.2-2 shall be handled case by case.

Table F.1.1.2-1: Maximum allowed measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Cell BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc averaged over BWConfig | dB | ±1.5 |  |
| AWGN absolute power, Noc for Measurement PRB | dB | ±1.5 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based, or configured RBs in the SRS resource for CLI-based |
| Ratio of cell X signal / AWGN, Êsx / Noc averaged over BWConfig | dB | ±0.3 | Same as in LTE |
| Ratio of cell X signal / AWGN, Êsx / Noc for Measurement PRB | dB | ±0.3 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based, or configured RBs in the SRS resource for CLI-based |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx | Depends on fading profile, can be referenced from TS 38.101-4 [20] |
| AWGN and signal flatness* | dB | ±2.0 | Same as in LTE, can be referenced from TS 38.101-4 [20] |
| Uplink absolute power measurement | dB | ±1.5 | Same as in TS 38.521-1 [17] |
| Uplink relative power measurement | dB | ±0.7 | Same as in TS 38.521-1 [17] |
| Uplink signal transmit timing relative to downlink | Tc | ±112 |  |
| Relative transmit timing accuracy during UE timing adjustment | Tc | ±88 |  |
| Timing Advance Adjustment accuracy | Tc | ±88 |  |
| Note 1: The values in this table are specified per cell. Multi-cell test cases need to combined these values in the TT analysis in TR 38.903 Note 2:These values apply for cell BW ≤ 40 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. Note 3: Void |  |  |  |

Table F.1.1.2-2: Maximum allowed measurement uncertainty values for the test system for FR2 (up to 40GHz) and Cell BW ≤ 400 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc averaged over BWConfig | dB | PC3: ±5.654,5 PC1: FFS |  |
| AWGN absolute power, Noc for Measurement PRB | dB | PC3: ±5.654,5 PC1: FFS | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based, or configured RBs in the SRS resource for CLI-based |
| Ratio of cell X signal / AWGN, Êsx / Noc averaged over BWConfig | dB | ±0.3 |  |
| Ratio of cell X signal / AWGN, Êsx / Noc for Measurement PRB | dB | ±0.3 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based, or configured RBs in the SRS resource for CLI-based |
| ÊsxNote 3 absolute power, Noc averaged over BWConfig | dB | PC3: ±5.654,5 PC1: FFS |  |
| ÊsxNote 3 absolute power, Noc for Measurement PRB | dB | PC3: ±5.654,5 PC1: FFS | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based, or configured RBs in the SRS resource for CLI-based |
| Relative power between any two Ês levels at the same frequency | dB | ±2.0 |  |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx |  |
| AWGN and signal flatness* | dB | ±3.6 | Same as in Demod Performance test cases, can be referenced from TS 38.521-4 [20] |
| Uplink absolute power measurement | dB | TBD |  |
| Uplink relative power measurement | dB | TBD |  |
| Uplink signal transmit timing relative to downlink | Tc | ±48 |  |
| Relative transmit timing during UE timing adjustment | Tc | ±40 |  |
| Timing Advance Adjustment | Tc | ±40 |  |
| NOTE 1: The values in this table are specified per cell. Multi-cell test cases need to combine these values in the TT analysis in TR 38.903 NOTE 2: These values apply for cell BW ≤ 400 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. NOTE 3: Applies for test cases that use Ês without AWGN NOTE 4: Total Expanded MU for IFF for Max device size ≤ 30cm in TR 38.903 [22] Table E.3.1.1-2 for PC3 UEs and in Table FFS for PC1 UEs. NOTE 5: If the TT analysis for a specific test case based on this MU value results in an unsolvable conflict, making the test case untestable, even after the alternative solutions listed in TR 38.903 [22] clause A.4 have been considered for the test case in TS 38.133 [6] Annex A, the TT analysis shall be repeated using a lower MU value, taking into account the values defined in clause E.3.1 TR 38.903 [22]. The test case will be applicable for the subset of the test systems meeting this reduced MU Threshold. |  |  |  |

The maximum test system uncertainty for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.1.2-3.

The maximum test system uncertainty for the NE-DC FR1 test cases in chapter 4 is defined in Table F.1.1.2-3a.

The maximum test system uncertainty for the NR/5GC FR1 test cases in chapter 6 is defined in Table F.1.1.2-4.

The maximum test system uncertainty for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.1.2-5.

The maximum test system uncertainty for the NR/5GC FR2 test cases in chapter 7 is defined in Table F.1.1.2-6.

The maximum test system uncertainty for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases in chapter 8 is defined in Table F.1.1.2-7.

The maximum test system uncertainty for the FR1 NR sidelink test cases in chapter 9 is defined in Table F.1.1.2-8.

The maximum test system uncertainty for the NR/5GC FR1 test cases for RedCap in chapter 16 is defined in Table F.1.1.2-9.

The maximum test system uncertainty for the NR/5GC FR2 test cases for RedCap in chapter 17 is defined in Table F.1.1.2-10.

The maximum test system uncertainty for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases for RedCap in chapter 18 is defined in Table F.1.1.2-11.

Table F.1.1.2-3: Maximum test system uncertainty for RRM requirements for EN-DC FR1 test cases

| Subclause |  |  | Maximum Test System Uncertainty1 |  |  |  |  | Derivation of Test System Uncertainty |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 Contention based random access test in FR1 for PSCell in EN-DC |  |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink |  |  |  |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |  |
| 4.3.2.2.2 Non-contention based random access test in FR1 for PSCell in EN-DC |  |  | Same as 4.3.2.2.1 |  |  |  |  | Same as 4.3.2.2.1 |  |  |  |
| 4.3.2.2.3 EN-DC FR1 2-step contention based random access |  |  | Same as 4.3.2.2.1 |  |  |  |  | Same as 4.3.2.2.1 |  |  |  |
| 4.3.2.2.4 EN-DC FR1 2-step non-contention based random access |  |  | Same as 4.3.2.2.1 |  |  |  |  | Same as 4.3.2.2.1 |  |  |  |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink |  |  |  |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |  |  |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy |  |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±88Tc Timing Advance Adjustment accuracy |  |  |  |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |  |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |  |  |  |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  |  |  |  | Same as 4.5.1.1 |  |  |  |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  |  | Same as 4.5.1.1 |  |  |  |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  |  | Same as 4.5.1.1 |  |  |  |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  |  |  |  | Same as 4.5.1.1 |  |  |  |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  |  |  |  | Same as 4.5.1.1 |  |  |  |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  |  | Same as 4.5.1.1 |  |  |  |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  |  | Same as 4.5.1.1 |  |  |  |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC |  |  | ± 0.6dB (AWGN conditions) |  |  |  |  | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |  |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC |  |  | Same as 4.5.2.1 |  |  |  |  | Same as 4.5.2.1 |  |  |  |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC |  |  | Noc2 ±1.5 dB Noc3 ±1.5 dB Ês2 / Noc2 ±0.3 dB Ês3 / Noc3 ±0.3 dB |  |  |  |  | Note: Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |  |  |  |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC |  |  | Same as 4.5.2.3 |  |  |  |  | Same as 4.5.2.3 |  |  |  |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC |  |  | Noc1 ±1.5 dB Noc2 ±1.5 dB Noc3 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB |  |  |  |  | Note: Noc1 is the AWGN on E-UTRAN cell 1 frequency Ês1 / Noc1 is the ratio of E-UTRAN cell 1 signal / AWGN Noc2 is the AWGN on NR cell 2 frequency Ês2 / Noc2 is the ratio of NR cell 2 signal / AWGN Noc3 is the AWGN on E-UTRAN cell 3 frequency Ês3 / Noc3 is the ratio of E-UTRAN cell 3 signal / AWGN |  |  |  |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC |  |  | Same as 4.5.2.5 |  |  |  |  | Same as 4.5.2.5 |  |  |  |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  |  | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB |  |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle |  |  | Same as 4.5.3.1 |  |  |  |  | Same as 4.5.3.1 |  |  |  |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX |  |  | Same as 4.5.3.1 |  |  |  |  | Same as 4.5.3.1 |  |  |  |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay |  |  |  |  | Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB Noc3 ±1.5 dB Ês3 / Noc3 ±0.3 dB |  |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |  |  |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX |  |  |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX |  |  |  |  | Same as 4.5.5.1 |  |  |  |  | Same as 4.5.5.1 |  |  |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  |  |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |  |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX |  |  |  |  | Same as 4.5.5.3 |  |  |  |  | Same as 4.5.5.3 |  |  |
| 4.5.5.5 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX |  |  |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |  |
| 4.5.5.6 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX |  |  |  |  | Same as 4.5.5.5 |  |  |  |  | Same as 4.5.5.5 |  |  |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC |  |  |  |  | ±0.6 dB |  |  |  |  | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 4.5.6.1.2 EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC |  |  |  |  | Same as 4.5.6.1.1 |  |  |  |  | Same as 4.5.6.1.1 |  |  |
| 4.5.6.2.1 EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC |  |  |  |  | ±0.6 dB |  |  |  |  | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 4.5.7.1 EN-DC FR1 addition and release delay of known PSCell |  | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB |  |  |  |  | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |  |
| 4.5.8.1 EN-DC FR1 interruptions at switching between two uplink carriers |  | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB |  |  |  |  | Noc is the AWGN on cell 2 frequency Ês / Noc is the ratio of cell 2 signal / AWGN |  |  |  |  |
| 4.6.1.1 EN-DC FR1 event-triggered reporting without gap in non-DRX |  |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB |  |  |  |  | Note: Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |  |  |  |
| 4.6.1.2 EN-DC FR1 event-triggered reporting without gap in DRX |  |  | Same as 4.6.1.1 |  |  |  |  | Same as 4.6.1.1 |  |  |  |
| 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX |  |  | Same as 4.6.1.1 |  |  |  |  | Same as 4.6.1.1 |  |  |  |
| 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX |  |  | Same as 4.6.1.1 |  |  |  |  | Same as 4.6.1.1 |  |  |  |
| 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | Same as 4.6.1.1 |  |  |  |  | Same as 4.6.1.1 |  |  |  |  |
| 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | Same as 4.6.1.1 |  |  |  |  | Same as 4.6.1.1 |  |  |  |  |
| 4.6.1.7 EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  |  | Same as 4.6.1.1 |  |  |  |  | Same as 4.6.1.1 |  |  |  |
| 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX |  |  | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB |  |  |  |  | Noc2 is the AWGN on NR freq2 Noc3 is the AWGN on NR freq3 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |
| 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX |  |  | Same as 4.6.2.1 |  |  |  |  | Same as 4.6.2.1 |  |  |  |
| 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  |  | Same as 4.6.2.1 |  |  |  |  | Same as 4.6.2.1 |  |  |  |
| 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  |  | Same as 4.6.2.1 |  |  |  |  | Same as 4.6.2.1 |  |  |  |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX |  |  | Same as 4.6.4.1 |  |  |  |  | Same as 4.6.4.1 |  |  |  |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |  |  |
| 4.6.4.4 EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX |  |  | Same as 4.6.4.3 |  |  |  |  | Same as 4.6.4.3 |  |  |  |
| 4.6.4.5 EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  |  | Same as 4.6.4.1 |  |  |  |  | Same as 4.6.4.1 |  |  |  |
| 4.6.5.1 EN-DC FR1 SRS-RSRP measurement with non-DRX |  |  | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB |  |  |  |  | Average Noc is the AWGN on virtual UE frequency (NR) on NR freq1 Ês / Noc is the ratio of virtual UE signal / AWGN over the SRS resource RBs |  |  |  |
| 4.6.5.2 EN-DC FR1 CLI-RSSI measurement with non-DRX |  |  | Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB |  |  |  |  | Note: Noc2 is the AWGN on cell 2 (NR) frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |  |  |
| 4.6.7.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX |  |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  |  | Noc is the AWGN on Cell 2 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |  |  |  |
| 4.6.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX |  |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  |  | Noc is the AWGN on Cell 2 frequency Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |  |  |  |
| 4.6.7.3 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX |  |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  |  | Noc is the AWGN on Cell 2 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |  |  |  |
| 4.7.1.1.1 EN-DC FR1 SS-RSRP absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês2 / Noc ±0.3 dB Meas PRB Ês3 / Noc ±0.3 dB |  |  |  |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Ês3 / Noc is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |
| 4.7.1.1.2 EN-DC FR1 SS-RSRP relative measurement accuracy |  |  | Same as 4.7.1.1.1 |  |  |  |  | Same as 4.7.1.1.1 |  |  |  |
| 4.7.1.2.1 EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy |  |  | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês2 / Noc1 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc1 ±0.3 dB Meas PRB Ês3 / Noc2 ±0.3 dB |  |  |  |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês2 / Noc1 is the cell 2 SNR Ês3 / Noc2 is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |
| 4.7.1.2.2 EN-DC FR1-FR1 SS-RSRP relative measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  |  | Same as 4.7.1.2.1 |  |  |  |
| 4.7.2.1 EN-DC FR1 SS-RSRQ measurement accuracy |  |  | Same as 4.7.1.1.1 |  |  |  |  | Same as 4.7.1.1.1 |  |  |  |
| 4.7.2.2.1 EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  |  | Same as 4.7.1.2.1 |  |  |  |
| 4.7.2.2.2 EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  |  | Same as 4.7.1.2.1 |  |  |  |
| 4.7.3.1 EN-DC FR1 SS-SINR measurement accuracy |  |  | Same as 4.7.1.1.1 |  |  |  |  | Same as 4.7.1.1.1 |  |  |  |
| 4.7.3.2.1 EN-DC FR1-FR1 SS-SINR absolute measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  |  | Same as 4.7.1.2.1 |  |  |  |
| 4.7.3.2.2 EN-DC FR1-FR1 SS-SINR relative measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  |  | Same as 4.7.1.2.1 |  |  |  |
| 4.7.4.1.1 EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB |  |  |  |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |
| 4.7.4.1.2 EN-DC FR1 SSB based L1-RSRP relative measurement accuracy |  |  | Same as 4.7.4.1.1 |  |  |  |  | Same as 4.7.4.1.1 |  |  |  |
| 4.7.4.2.1 EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy |  |  | Same as 4.7.4.1.1 |  |  |  |  | Same as 4.7.4.1.1 |  |  |  |
| 4.7.4.2.2 EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy |  |  | Same as 4.7.4.1.1 |  |  |  |  | Same as 4.7.4.1.1 |  |  |  |
| 4.7.5.1 EN-DC FR1 SFTD measurement accuracy |  |  | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.8 dB |  |  |  |  | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 Noc2 is the AWGN on E-UTRA freq2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RB22 to RB27 |  |  |  |
| 4.7.6.1 EN-DC SRS-RSRP measurement accuracy with FR1 serving cell |  |  | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB |  |  |  |  | Average Noc is the AWGN on virtual UE frequency (NR) on NR freq1 Ês / Noc is the ratio of virtual UE signal / AWGN over the SRS resource RBs |  |  |  |
| 4.7.7.1.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB |  |  |  |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the CSI-RS#0 & CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |  |  |  |
| 4.7.7.1.2 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy |  |  | Same as 4.7.7.1.1 |  |  |  |  | Same as 4.7.7.1.1 |  |  |  |
| 4.7.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB |  |  |  |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the SSB#0 & SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |  |  |  |
| 4.7.7.3.1 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  | Same as 4.7.7.1.1 |  |  |  |  | Same as 4.7.7.1.1 |  |  |  |
| 4.7.7.3.2 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy |  |  | Same as 4.7.7.1.1 |  |  |  |  | Same as 4.7.7.1.1 |  |  |  |

Table F.1.1.2-3a Maximum test system uncertainty for RRM requirements for NE-DC FR1 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 4A.1.1.1 NE-DC FR1 addition and release delay of known PSCell | Noc ±1.5 dB Ês / Noc ±0.3 dB | Note: Ês / Noc is the ratio of E-UTRA cell signal / AWGN |
| 4A.2.1.1 NE-DC FR1 SFTD accuracy | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.8 dB | Note: Ês / Noc is the ratio of NR cell signal / AWGN Ês2 / Noc2 is the ratio of E-UTRA cell signal / AWGN |

Table F.1.1.2-4 Maximum test system uncertainty for RRM requirements for SA FR1 test cases

| Subclause | Maximum Test System Uncertainty1 |  | Derivation of Test System Uncertainty |
| --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.1.1.3 NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.1.1 |  | Same as 6.1.1.1 |
| 6.1.1.4 NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.1.1 |  | Same as 6.1.1.1 |
| 6.1.1.5 NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.1.1.6 NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.1.1.7 NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | Same as 6.1.1.1 |  | Same as 6.1.1.1 |  |
| 6.1.2.1 NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB |  | Note: Noc1 is the AWGN on cell 1 (NR) frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 (E-UTRAN) frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.1.2.2 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.1.2.3 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.1.2.4 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.1.2.5 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 | Same as 6.1.2.1 | Same as 6.1.2.1 |  |
| 6.3.1.1 NR SA FR1 handover with known target cell | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.3.1.2 NR SA FR1 handover with unknown target cell | Same as 6.3.1.1 | Same as 6.3.1.1 |  |
| 6.3.1.3 NR SA FR1-FR1 Handover with unknown Target Cell | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |
| 6.3.1.4 NR SA FR1 – E-UTRA handover with known target cell | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.3.1.5 NR SA FR1 – E-UTRA handover with unknown target cell | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.3.1.6 NR SA FR1 – UTRAN FDD handover with known target cell | Noc ±1.5 dB Ês / Noc ±0.3 dB Ioc ±0.7 dB Îor / Ioc ± 0.3dB CPICH Ec / Ior ± 0.1dB SCH Ec / Ior ± 0.1dB |  | Note: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the ratio of cell 2 CPICH code level / Ior SCH Ec / Ior is the ratio of cell 2 SCH code level / Ior |
| 6.3.1.7 NR SA FR1 synchronous DAPS handover | Same as 6.3.1.1 |  | Same as 6.3.1.1 |
| 6.3.1.8 NR SA FR1 asynchronous DAPS handover | Same as 6.3.1.1 |  | Same as 6.3.1.1 |
| 6.3.1.9 NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |
| 6.3.1.10 NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |
| 6.3.1.11 NR SA FR1 Inter-band inter-frequency synchronous DAPS handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |  |
| 6.3.1.12 NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |  |
| 6.3.2.1.1 NR SA FR1 RRC re-establishment | Same as 6.1.1.1 |  | Same as 6.1.1.1 |
| 6.3.2.1.2 NR SA FR1 – FR1 RRC re-establishment | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.3.2.1.3 NR SA FR1 RRC re-establishment without serving cell timing | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.3.2.2.1 | Noc ±1.5 dB Ês / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.3.2.2.2 | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |
| 6.3.2.2.3 | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |
| 6.3.2.2.4 | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |
| 6.3.2.3.1 NR SA FR1 RRC connection release with redirection | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.3.2.3.2 NR SA FR1 – E-UTRA RRC connection release with redirection | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.3.3.1 NR SA FR1 conditional handover | Same as 6.3.1.1 |  | Same as 6.3.1.1 |
| 6.3.3.2 NR SA FR1-FR1 conditional handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |
| 6.4.1.1 EN-DC FR1 UE transmit timing accuracy | Noc ±1.5 dB Ês1 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.4.3.1 NR SA FR1 timing advance adjustment accuracy | Noc ±1.5 dB Ês1 / Noc ±0.3 dB ±88Tc Timing Advance Adjustment accuracy |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.3 dB Fading profile ±0.7 dB |  | Noc is the AWGN on cell 1 frequency Ês / Noc is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | Noc1 ±1.5 dB Noc2 ±1.5 dB Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB |  | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |
| 6.5.3.1 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.5.3.2 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.3.3 NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.5.1 NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.5.5.2 NR SA FR1 SSB-based beam failure detection and link recovery in DRX | Same as 6.5.5.1 |  | Same as 6.5.5.1 |  |
| 6.5.5.3 NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |
| 6.5.5.4 NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | Same as 6.5.5.3 |  | Same as 6.5.5.3 |  |
| 6.5.5.5 NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB and CSI-RS Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.5.5.6 NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | Same as 6.5.5.5 |  | Same as 6.5.5.5 |  |
| 6.5.6.1.1 NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | ±0.6 dB |  | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |
| 6.5.6.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX | Same as 6.5.6.1.1 |  | Same as 6.5.6.1.1 |  |
| 6.5.6.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX | ±0.6 dB |  | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |
| 6.5.7.1 NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA | Noc1 ±1.5 dB Noc2 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |
| 6.5.7.2 NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA | Same as 6.5.7.1 |  | Same as 6.5.7.1 |  |
| 6.5.8.1 UE specific CBW change on PCell in FR1 in non-DRX | Test System uncertainty on Cell1: ±0.6 dB |  | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty: ±0.3 dB, 2. Effect of AWGN flatness and signal flatness: ±2.0 dB. Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) = 0.58dB, round up to 0.6dB. |  |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.7 NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.6.2.2 NR SA FR1-FR1 event-triggered reporting in DRX | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.3.1 NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |
| 6.6.3.3 NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |
| 6.6.4.4 NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | Same as 6.6.4.3 |  | Same as 6.6.4.3 |  |
| 6.6.4.5 NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  |
| 6.6.5.1 NR SA FR1 – UTRAN event-triggered reporting in non-DRX | Noc ±1.5 dB Ês / Noc ±0.3 dB Ioc ±0.7 dB Îor / Ioc ± 0.3dB CPICH Ec / Ior ± 0.1dB SCH Ec / Ior ± 0.1dB |  | Note: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the ratio of cell 2 CPICH code level / Ior SCH Ec / Ior is the ratio of cell 2 SCH code level / Ior |  |
| 6.6.6.1 NR SA FR1 SRS-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB |  | Average Noc is the AWGN on virtual UE frequency (NR) on NR freq1 Ês / Noc is the ratio of virtual UE signal / AWGN over the SRS resource RBs |  |
| 6.6.8.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on Cell 1 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |  |
| 6.6.8.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on Cell 1 frequency Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |  |
| 6.6.8.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on Cell 1 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |  |
| 6.6.9.1 NR SA FR1 Idle mode CA/DC measurement for FR1 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.6.15.1 NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on LTE freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB meas PRB Ês1 / Noc ±0.3 dB meas PRB Ês2 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês1 / Noc is the cell 1 SNR Ês2 / Noc is the cell 2 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy | Same as 6.7.1.1.1 |  | Same as 6.7.1.1.1 |  |
| 6.7.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB meas PRB Ês1 / Noc1 ±0.3 dB meas PRB Ês2 / Noc2 ±0.3 dB |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the cell 2 SNR Ês2 / Noc2 is the cell 3 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy | Same as 6.7.1.1.1 |  | Same as 6.7.1.1.1 |
| 6.7.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.3.1 NR SA FR1 SS-SINR measurement accuracy | Same as 6.7.1.1.1 |  | Same as 6.7.1.1.1 |
| 6.7.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.8 dB |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 1 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 |
| 6.7.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 |
| 6.7.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 |
| 6.7.5.1 NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.8 dB |  | Noc is the AWGN on E-UTRA freq1 Ês / Noc is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for RSRP #RB22 to RB27 |
| 6.7.6.1 NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Same as 6.7.5.1 |  | Same as 6.7.5.1 |
| 6.7.7.1 NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Same as 6.7.5.1 |  | Same as 6.7.5.1 |
| 6.7.8.1 NR SA FR1 SRS-RSRP measurement accuracy | Averaqe Noc ±1.5 dB Average Ês / Noc ±0.3 dB |  | Average Noc is the AWGN on virtual UE frequency (NR) on NR freq1 Ês / Noc is the ratio of virtual UE signal / AWGN over the SRS resource RBs |
| 6.7.9.1.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the CSI-RS#0 & CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |
| 6.7.9.1.2 NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy | Same as 6.7.9.1.1 |  | Same as 6.7.9.1.1 |
| 6.7.9.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the SSB#0 & SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |
| 6.7.9.3.1 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Same as 6.7.9.1.1 |  | Same as 6.7.9.1.1 |
| 6.7.9.3.2 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy | Same as 6.7.9.1.1 |  | Same as 6.7.9.1.1 |

<< Skipped sections >>

### F.1.3.2	Measurement of RRM requirements

Because the relationships between the test system uncertainties and the test tolerances are often complex, it is not always possible to give a simple derivation of the test requirement in this document. The analysis is recorded in 3GPP TR 38 903 [22].

The derivation of the test requirements for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.3.2-1.

The derivation of the test requirements for the NE-DC FR1 test cases in chapter 4 is defined in Table F.1.3.2-1a.

The derivation of the test requirements for the NR/5GC FR1 test cases in chapter 6 is defined in Table F.1.3.2-2.

The derivation of the test requirements for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.3.2-3.

The derivation of the test requirements for the NR/5GC FR2 test cases in chapter 7 is defined in Table F.1.3.2-4.

The derivation of the test requirements for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases in chapter 8 is defined in Table F.1.3.2-5.

The derivation of the test requirements for the FR1 NR sidelink test cases in chapter 9 is defined in Table F.1.3.2-6.

The derivation of the test requirements for the NR/5GC FR1 test cases for RedCap in chapter 16 is defined in Table F.1.3.2-7.

The derivation of the test requirements for the NR/5GC FR2 test cases for RedCap in chapter 17 is defined in Table F.1.3.2-8.

The derivation of the test requirements for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases for RedCap in chapter 18 is defined in Table F.1.3.2-9.

Table F.1.3.2-1: Derivation of test requirements for EN-DC FR1 RRM tests

| Test |  | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) |  | Test requirement in TS 38.533 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 |  | Absolute uplink power: Normal conditions ±9dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc |  | 2.1dB 0.7dB 112Tc 112Tc |  | Absolute uplink power: Normal conditions ±11.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |  |  |
| 4.3.2.2.2 |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  |  |
| 4.3.2.2.3 |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  |  |
| 4.3.2.2.4 |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  |  |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy |  | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês2 / Noc: +3.00dB |  | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB |  | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês2 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês2 / Noc: +3.30dB |  |  |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT |  | 0 0 0 +/- 88 Tc +/- 88 Tc |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |  |  |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |  |  |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4.1.1 |  |  |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |  |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  |  |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |  |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  |  |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |  |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  |  |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC |  | SNRs as specified |  | 0.6dB |  | Formula: SNR + TT |  |  |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC |  | Same as 4.5.2.1 |  | Same as 4.5.2.1 |  | Same as 4.5.2.1 |  |  |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC |  | Same as 4.5.2.3 |  | Same as 4.5.2.3 |  | Same as 4.5.2.3 |  |  |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC |  | Same as 4.5.2.5 |  | Same as 4.5.2.5 |  | Same as 4.5.2.5 |  |  |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  |  |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  |  |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay |  | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB |  |  |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |  |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX |  | Same as 4.5.5.1 |  | Same as 4.5.5.1 |  | Same as 4.5.5.1 |  |  |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |  |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX |  | Same as 4.5.5.3 |  | Same as 4.5.5.3 |  | Same as 4.5.5.3 |  |  |
| 4.5.5.5 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in section D.4.1.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |  |
| 4.5.5.6 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX |  | Same as 4.5.5.5 |  | Same as 4.5.5.5 |  | Same as 4.5.5.5 |  |  |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |  |
| 4.5.6.1.2 EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB |  |  |
| 4.5.6.2.1 EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |  |
| 4.5.7.1 EN-DC FR1 addition and release delay of known PSCell |  |  | For Test Configurations 1,2,4,5 Noc: -88dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS For Test Configurations 3,6 Noc: -88dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -93 dBm/SCS |  | -0.6dB 0dB -3dB -0.6dB 0dB -3dB |  |  | Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -99 dBm/SCS Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS |  |  |
| 4.5.8.1 EN-DC FR1 interruptions at switching between two uplink carriers |  |  | Noc: -104dBm/15kHz Ês / Noc: 17dB |  | 0dB 0dB |  |  | Noc: -104dBm/15kHz Ês / Noc: 17dB |  |  |
| 4.6.1.1 EN-DC FR1 event-triggered reporting without gap in non-DRX |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  |
| 4.6.1.2 EN-DC FR1 event-triggered reporting without gap in DRX |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |  |
| 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |  |
| 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |  |
| 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |  |
| 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |  |
| 4.6.1.7 EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |  |
| 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX |  | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  |  |
| 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  |  |
| 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  |  |
| 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  |  |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX |  | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping |  | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#1: RSRP_55 to RSRP_75 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |  |  |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX |  | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  |  |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB |  | 0dB 0dB 0.5dB Via mapping |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 CSI-RS#1: RSRP_55 to RSRP_75 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 CSI-RS#1: RSRP_58 to RSRP_78 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 |  |  |
| 4.6.4.4 EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX |  | Same as 4.6.4.3 |  | Same as 4.6.4.3 |  | Same as 4.6.4.3 |  |  |
| 4.6.4.5 EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  |  |
| 4.6.5.1 EN-DC FR1 SRS-RSRP measurement with non-DRX |  | Test Configuration 1: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +4dB Test Configuration 2: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +4dB |  | Test Config 1: T1: 0dB 0dB T2: 0dB +5.25dB Test Config 2: T1: 0dB 0dB T2: 0dB +4.75dB |  | Test Configuration 1: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +9.25dB Test Configuration 2: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +8.75dB |  |  |
| 4.6.5.2 EN-DC FR1 CLI-RSSI measurement with non-DRX |  |  | During T1: Noc on CLI RSSI: -116 dBm/15kHz Io on CLI RSSI : -97.43 dBm/ 1.08 MHz During T2: Noc on CLI RSSI: -108dBm/15kHz Io on CLI RSSI : -89.43 dBm/ 1.08 Mhz |  | During T1: 0dB 0dB During T2: 0dB 0.5dB |  |  | During T1: Noc: -116 dBm/15kHz Io on CLI RSSI : -97.43 dBm/ 1.08 MHz During T2: Noc: -116 dBm/15kHz Io on CLI RSSI : -88.93 dBm/ 1.08 MHz |  |  |
| 4.6.7.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ± 5.5dB Reported Differential CSI-SINR values ± 4.5dB |  | 0dB 0dB 0dB Via mapping |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_41 to SINR_64 CSI-RS#0: DIFFSINR_0 to DIFFSINR_7 |  |  |
| 4.6.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SS-SINR values ±4.5dB Reported Differential SS-SINR values ±3.5dB |  | 0dB 0dB 0dB Via mapping |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 SSB#1: SINR_43 to SINR_62 SSB#0: DIFFSINR_0 to DIFFSINR_6 |  |  |
| 4.6.7.3 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ±4dB Reported Differential CSI-SINR values ±3dB |  | 0dB 0.5dB 0dB Via mapping |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.50dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_44 to SINR_61 CSI-RS#0: DIFFSINR_0 to DIFFSINR_5 |  |  |
| 4.7.1.1.1 EN-DC FR1 SS-RSRP absolute measurement accuracy |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |
|  |  | Test 1: Noc: -106dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB |  | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  | Test 1: Noc: -107.5dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_34 to RSRP_46 RSRP_34 to RSRP_46 RSRP_35 to RSRP_47 RSRP_35 to RSRP_47 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |  |  |
|  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc: -110dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB |  | Test 1: -0.8dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  | Test 1: Noc: -110.8dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_41 to RSRP_53 Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 RSRP_38 to RSRP_50 RSRP_38 to RSRP_50 RSRP_39 to RSRP_51 RSRP_40 to RSRP_52 RSRP_40 to RSRP_52 depending on operating band |  |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |
| 4.7.1.1.2 EN-DC FR1 SS-RSRP relative measurement accuracy |  | Test 1: Noc: Test configuration 1, 2, 4, 5:-106dBm/15kHz Test configuration 3, 6:-110dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported relative SS-RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported relative SS-RSRP values: ±3dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported relative SS-RSRP values: ±3dB |  | Test 1: 0 dB 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  | Test 1: Noc: -106 dBm/15kHz Noc: -110 dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |  |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 2 and Cell 3 RSRP from Noc, Ês2 / Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |
| 4.7.1.2.1 EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_62 to RSRP_81 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 depending on operating band |  |  |
|  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |  |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |  |  |  |  |  |
| 4.7.1.2.2 EN-DC FR1-FR1 SS-RSRP relative measurement accuracy |  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported relative RSRP values: ±4.5dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported relative RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_x-7 to RSRP_x+7 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_x-31 to RSRP_x-18 |  |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 2 and Cell 3 RSRP from Noc, Ês2 / Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1.5dB wider at each end. |  |  |  |  |  |  |  |  |
| 4.7.2.1 EN-DC FR1 SS-RSRQ measurement accuracy |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |
|  |  | Test 1: Noc: -85dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -101dBm/15kHz Ês2 / Noc: -2.9dB Ês3 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping |  | Test 1: Noc: -86.5dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: Noc: -101dBm/15kHz Ês2 / Noc: -2.9dB Ês3 / Noc: -2.9dB RSRQ_46 to RSRQ_60 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |  |  |
|  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc: -91dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping |  | Test 1: Noc: -92.6dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |  |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 3 SS-RSRQ from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |  |  |  |  |  |
| 4.7.2.2.1 EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |  |
|  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 3 SS-RSRQ from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |  |  |  |  |  |
| 4.7.2.2.2 EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |  |
|  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 and Cell 3 SS-RSRQ from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1dB wider at each end. |  |  |  |  |  |  |  |  |
| 4.7.3.1 EN-DC FR1 SS-SINR measurement accuracy |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |
|  |  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -4dB Ês3 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -3.5dB Ês3 / Noc: -3.5dB SINR_28 to SINR_45 |  |  |
|  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -4dB Ês3 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  | Test 1: Noc: -93.2dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -3.5dB Ês3 / Noc: -3.5dB SINR_28 to SINR_45 |  |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |  |  |  |  |  |
| 4.7.3.2.1 EN-DC FR1-FR1 SS-SINR absolute measurement accuracy |  | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -4dB Ês3 / Noc2: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3.2dB Ês3 / Noc2: -3.2dB SINR_32 to SINR_49 |  |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |  |  |  |  |  |
| 4.7.3.2.2 EN-DC FR1-FR1 SS-SINR relative measurement accuracy |  | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -4dB Ês3 / Noc2: -4dB Reported SINR values: ±4dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3.2dB Ês3 / Noc2: -3.2dB SINR_x-11 to SINR_x+11 |  |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each for Tests 1 and 2 and 0dB wider at each end for Test 3. |  |  |  |  |  |  |  |  |
| 4.7.4.1.1 EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB RSRP_62 to RSRP_82 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_31 to RSRP_44 RSRP_31 to RSRP_45 RSRP_32 to RSRP_45 RSRP_32 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 depending on operating band |  |  |
| Test Configuration 3,6 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 RSRP_35 to RSRP_48 RSRP_35 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_37 to RSRP_51 depending on operating band |  |  |
| 4.7.4.1.2 EN-DC FR1 SSB based L1-RSRP relative measurement accuracy |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |  |
| Test Configuration 3,6 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |  |
| 4.7.4.2.1 EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy |  | Same as 4.7.4.1.1 |  | Same as 4.7.4.1.1 |  | Same as 4.7.4.1.1 |  |  |
| 4.7.4.2.2 EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy |  | Same as 4.7.4.1.2 |  | Same as 4.7.4.1.2 |  | Same as 4.7.4.1.2 |  |  |
| 4.7.5.1 EN-DC FR1 SFTD measurement accuracy |  | Noc1: -104dBm/15kHz Ês1 / Noc1: -3.0dB Noc2: -104dBm/15kHz Ês2 / Noc2: 17.0dB |  | 0dB 0.3dB 0dB 0dB |  | Noc1: -104dBm/15kHz Ês1 / Noc1: -2.7dB Noc2: -104dBm/15kHz Ês2 / Noc2: 17.0dB |  |  |
| 4.7.6.1 EN-DC FR1 SRS-RSRP measurement accuracy |  | Test Configuration 1: Test 1: Noc = -106dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3dB Test 2: Noc = -88dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±6.5dB Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3dB Test Configuration 2: Test 1: Test 1 is not used when configuring SCS 30kHz Test 2: Noc = -91dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±7dB Test 3: Noc = -114+ ΔBG_offset dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3.5dB |  | Test Config 1: Test 1: 0dB +0.5dB Via mapping Test 2: 0dB +0.5dB Via mapping Test 3: 0dB +0.75dB for BG_A +0.5dB otherwise Via mapping Test Config 2: Test 1: N/A Test 2: 0dB +0.5dB Via mapping Test 3: 0dB +1.25dB for BG_A +0.5dB otherwise Via mapping |  | Test Config 1: Test 1: Noc = -106dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 30 to 41 Test 2: Noc = -88dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 45 to 62 Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +1.75dB for BG_A Ês / Noc = +1.5dB otherwise Reported SRS-RSRP: BG_A: 23 to 33 BG_C: 23 to 34 BG_D: 24 to 34 BG_E: 24 to 35 Test Config 2: Test 1: N/A Test 2: Noc = -91dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 44 to 63 Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +2.25dB for BG_A Ês / Noc = +1.5dB otherwise Reported SRS-RSRP: BG_A: 26 to 37 BG_C: 26 to 37 BG_D: 26 to 38 BG_E: 27 to 38 |  |  |
| 4.7.7.1.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy |  |  |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_28 to L1-SINR_57 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_27 to L1-SINR_58 |  |  |
| Test Configuration 3,6 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) |  |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_28 to L1-SINR_57 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_27 to L1-SINR_58 |  |  |
| 4.7.7.1.2 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy |  |  |  |  |  |  |  |  |  |  |
| Same as 4.7.7.1.1 |  | Same as 4.7.7.1.1 |  |  | Same as 4.7.7.1.1 |  |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB L1-SINR_x-9 to L1-SINR_x+9 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB L1-SINR_x-9 to L1-SINR_x+9 |  |  |
| Same as 4.7.7.1.1 |  | Same as 4.7.7.1.1 |  |  | Same as 4.7.7.1.1 |  |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB L1-SINR_x-9 to L1-SINR_x+9 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB L1-SINR_x-9 to L1-SINR_x+9 |  |  |
| 4.7.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported SS-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported SS-SINR values: ±4.5dB (±1dB additionally for extreme conditions) |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |  |  |
| Test Configuration 3,6 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) |  |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |  |  |
| 4.7.7.3.1 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |  |  |
| Test Configuration 3,6 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) |  |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |  |  |
| 4.7.7.3.2 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy |  |  |  |  |  |  |  |  |  |  |
| Same as 4.7.7.1.2 |  | Same as 4.7.7.1.2 |  |  | Same as 4.7.7.1.2 |  |  | Same as 4.7.7.1.2 |  |  |
| Same as 4.7.7.1.2 |  | Same as 4.7.7.1.2 |  |  | Same as 4.7.7.1.2 |  |  | Same as 4.7.7.1.2 |  |  |

Table F.1.3.2-1a: Derivation of test requirements for NE-DC FR1 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 4A.1.1.1 NE-DC FR1 addition and release delay of known PSCell | Noc: -104dBm/15kHz Ês / Noc: +17dB B1 Threshold Test Configuration 1, 2, 4, 5: -96 dBm/15kHz Test Configuration 3,6: -93 dBm/15kHz | 0dB 0dB -1dB -4dB | Noc: -104dBm/15kHz Ês / Noc: +17dB Test Configuration 1, 2, 4, 5: -97 dBm/15kHz Test Configuration 3,6: -97 dBm/15kHz |
| 4A.2.1.1 NE-DC FR1 SFTD accuracy | Noc1: Test Configuration 1, 2, 4, 5: -104dBm/15kHz Test Configuration 3,6: -101dBm/15kHz Ês1 / Noc1: -3dB Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB | 0dB 0dB 0.3dB 0dB 0dB | Noc1: Test Configuration 1, 2, 4, 5: -104dBm/15kHz Test Configuration 3,6: -101dBm/15kHz Ês1 / Noc1: -2.7dB Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |

Table F.1.3.2-2: Derivation of test requirements for NR SA FR1 RRM tests

| Test |  |  | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) |  | Test requirement in TS 38.533 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |  |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB |  | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |  |
| 6.1.1.3 NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB In Signalling SSearchDeltaP = 3dB |  | During T1: 0dB -0.45dB 0dB During T2: 0dB 0dB -0.45dB In Signalling 3dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +12.55dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +12.55dB In Signalling SSearchDeltaP = 6dB |  |
| 6.1.1.4 NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB |  | During T1: 0dB -0.45dB 0dB During T2: 0dB 0dB -0.45dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +12.55dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +12.55dB |  |
| 6.1.1.5 NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 48dB Thresh_X_LowP of Cell 1 = 50dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 44dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 3dB |  | During T1: 0dB 0dB 0dB 0.3dB During T2: 0dB 0dB 0dB -2.25dB In signalling: 0dB -4dB -2dB 0dB 0dB 2dB 0dB 12dB |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -3.7dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc1: -98dBm/15kHz Ês2 / Noc2: +9.75dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 44dB Thresh_X_LowP of Cell 1 = 48dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 46dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 15dB |  |
| 6.1.1.6 NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: SSearchThresholdP of Cell 1 = 50dB SSearchThresholdP of Cell 2 = 50dB |  | During T1: 0dB 2dB -2dB 0.35dB During T2: 0dB 0dB -2dB 4dB In signalling: 0dB -16dB |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +16dB Noc2: -100dBm/15kHz Ês2 / Noc2: -3.65dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc1: -100dBm/15kHz Ês2 / Noc2: +16dB In signalling: SSearchThresholdP of Cell 1 = 50dB SSearchThresholdP of Cell 2 = 34dB |  |
| 6.1.1.7 NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |  |
| 6.1.2.1 NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 1.6dB 1.6dB During T3: 0dB -2dB 1.6dB 0.4dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB During T3: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB |  |
| 6.1.2.2 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB |  | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |  |
| 6.1.2.3 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB |  | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |  |
| 6.1.2.4 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB |  | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |  |
| 6.1.2.5 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 |  |  | Same as 6.1.2.2 |  | Same as 6.1.2.2 |  | Same as 6.1.2.2 |  |
| 6.3.1.1 NR SA FR1 handover with known target cell |  |  | TEST CONFIGURATION 1, 2 A3-Offset: 0dB |  | -1dB |  | A3-Offset: -1dB |  |
|  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |  |
|  |  |  | TEST CONFIGURATION 3 A3-Offset: 0dB |  | -1dB |  | A3-Offset: -1dB |  |
|  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |  |
| 6.3.1.2 NR SA FR1 handover with unknown target cell |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB |  |
| 6.3.1.3 NR SA FR1-FR1 Handover with unknown Target Cell |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB |  |
| 6.3.1.4 NR SA FR1 – E-UTRA handover with known target cell |  |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: -infinity During T2: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +8dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +8dB |  | During T1: 0dB 0dB 1.55dB 0dB During T2: 0dB 0dB -1.55dB 1.55dB During T3: 0dB 0dB -1.55dB 1.55dB |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.55dB Ês2 / Noc2: -infinity During T2: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -5.55dB Ês2 / Noc2: +9.55dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -5.55dB Ês2 / Noc2: +9.55dB |  |
| 6.3.1.5 NR SA FR1 – E-UTRA handover with unknown target cell |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB |  |
| 6.3.1.6 NR SA FR1 – UTRAN FDD handover with known target cell |  |  | During T1: Noc: -100dBm/15kHz Ês / Noc: 12dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -100dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T3: Noc: -100dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |  | During T1: -1.54dB 0dB 0dB 0dB 0dB 0dB During T2: -1.54dB 0dB 0dB 0dB 0dB 0dB During T3: -1.54dB 0dB 0dB 0dB 0dB 0dB |  | During T1: Noc: -101.54dBm/15kHz Ês / Noc: 12dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -101.54dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T3: Noc: -101.54dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |  |
| 6.3.1.7 NR SA FR1 synchronous DAPS handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB In signaling: A3-offset: 0dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 1dB During T3: 0dB 0dB 1dB During T4: 0dB 0dB 1dB During T5: 0dB 0dB 1dB In signaling: -1dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signaling: A3-offset: -1dB |  |
| 6.3.1.8 NR SA FR1 asynchronous DAPS handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB In signaling: A3-offset: 0dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 1dB During T3: 0dB 0dB 1dB During T4: 0dB 0dB 1dB During T5: 0dB 0dB 1dB In signaling: -1dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signaling: A3-offset: -1dB |  |
| 6.3.1.9 NR SA FR1 Intra-band inter-frequency synchronous DAPS handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB In signaling: A3-offset: 0dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 3.7dB During T3: 0dB 0dB 3.7dB During T4: 0dB 0dB 3.7dB During T5: 0dB 0dB 3.7dB In signaling: -3dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB In signaling: A3-offset: -3dB |  |
| 6.3.1.10 NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB In signaling: A3-offset: 0dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 3.7dB During T3: 0dB 0dB 3.7dB During T4: 0dB 0dB 3.7dB During T5: 0dB 0dB 3.7dB In signaling: -3dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB In signaling: A3-offset: -3dB |  |
| 6.3.1.11 NR SA FR1 Inter-band inter-frequency synchronous DAPS handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signaling: A3-offset: -6dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB During T4: 0dB 0dB 0dB During T5: 0dB 0dB 0dB In signaling: -1dB for config 1,2,4,5,9 In signaling: -4dB for config 3,6 In signaling: 2dB for config 7,8 |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signaling: -7dB for config 1,2,4,5,9 In signaling: -10dB for config 3,6 In signaling: -4dB for config 7,8 |  |
| 6.3.1.12 NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signaling: A3-offset: -4dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB During T4: 0dB 0dB 0dB During T5: 0dB 0dB 0dB In signaling: -3dB for config 1,2,4,5,9 In signaling: -6dB for config 3,6 In signaling: 0dB for config 7,8 |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signaling: -7dB for config 1,2,4,5,9 In signaling: -10dB for config 3,6 In signaling: -4dB for config 7,8 |  |
| 6.3.2.1.1 NR SA FR1 RRC re-establishment |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |  |
| 6.3.2.1.2 NR SA FR1 - FR1 RRC re-establishment |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB |  |
| 6.3.2.1.3 NR SA FR1 RRC re-establishment without serving cell timing |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |  |
| 6.3.2.2.1 |  |  | Absolute uplink power: Normal conditions ±9dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc |  | 2.1dB 0.7dB 112Tc 112Tc |  | Absolute uplink power: Normal conditions ±11.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |  |
| 6.3.2.2.2 |  |  | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |  |
| 6.3.2.2.3 |  |  | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |  |
| 6.3.2.2.4 |  |  | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |  |
| 6.3.2.3.1 NR SA FR1 RRC connection release with redirection |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |  |
| 6.3.2.3.2 NR SA FR1 – E-UTRA RRC connection release with redirection |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |  |
| 6.3.3.1 NR SA FR1 conditional handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11dB In signaling: A3-offset: 0dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB -1dB In signaling: -1dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signaling: A3-offset: -1dB |  |
| 6.3.3.2 NR SA FR1-FR1 conditional handover |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.7dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +6.7dB |  |
| 6.4.1.1 EN-DC FR1 UE transmit timing accuracy |  |  | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês1 / Noc: +3.00dB |  | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB |  | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês1 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês1 / Noc: +3.30dB |  |
| 6.4.3.1 NR SA FR1 timing advance adjustment accuracy |  |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT |  | 0 0 0 +/- 88 Tc +/- 88 Tc |  | Noc = -98 dBm/15 kHz (Config 1, 2) Noc = -95 dBm/15 kHz (Config 3) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |  |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  |  | SNR during T1: 1dB T2: -7dB T3: -15dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |  |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  |  | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4.1.1 |  |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 6.5.1.2 |  | Same as 6.5.1.2 |  | Same as 6.5.1.2 |  |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 6.5.1.2 |  | Same as 6.5.1.2 |  | Same as 6.5.1.2 |  |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 6.5.1.2 |  | Same as 6.5.1.2 |  | Same as 6.5.1.2 |  |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  |
| 6.5.3.1 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  |
| 6.5.3.2 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle |  |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.3.3 NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX |  |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.5.1 NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX |  |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |  |
| 6.5.5.2 NR SA FR1 SSB-based beam failure detection and link recovery in DRX |  |  | Same as 6.5.5.1 |  | Same as 6.5.5.1 |  | Same as 6.5.5.1 |  |
| 6.5.5.3 NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  |  | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês1 / Noc1: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |  |
| 6.5.5.4 NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX |  |  | Same as 6.5.5.3 |  | Same as 6.5.5.3 |  | Same as 6.5.5.3 |  |
| 6.5.5.5 NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX |  |  | On the SCell: q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB |  | On the SCell: Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB |  | On the SCell: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |  |
| 6.5.5.6 NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX |  |  | Same as 6.5.5.5 |  | Same as 6.5.5.5 |  | Same as 6.5.5.5 |  |
| 6.5.6.1.1 NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  |
| 6.5.6.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX |  |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T2: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T3: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |  | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T2: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T3: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |  |
| 6.5.6.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX |  |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |  | During T1: 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |  |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX |  |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX |  |  | Same as 64.6.1.1 |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX |  |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading |  |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading |  |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.7 NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.5.3.1 |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  |
| 6.5.3.2 |  |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.3.3 |  |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.7.1 NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA |  |  | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +17dB Reported CSI-RSRP values ± 10dB |  | -1.10dB -1.10dB 0dB 0dB Via mapping |  | Noc1: -105.10dBm/15kHz Noc2: -105.10dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +17dB CSI-RS#0: RSRP_64 to RSRP_84 CSI-RS#1: RSRP_67 to RSRP_87 |  |
| 6.5.7.2 NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA |  |  | Same as 6.5.7.1 |  | Same as 6.5.7.1 |  | Noc1: -105.10dBm/15kHz Noc2: -105.10dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +17dB CSI-RS#0: RSRP_67 to RSRP_87 CSI-RS#1: RSRP_67 to RSRP_87 |  |
| 6.5.8.1 UE specific CBW change on PCell in FR1 in non-DRX |  |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |  | During T1: 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |  |
| 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX |  |  | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB |  |
| 6.6.2.2 NR SA FR1-FR1 event-triggered reporting in DRX |  |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.3.1 NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX |  |  | During T1: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: 0dB Ês2 / Noc: +17.00dB |  | During T1: FFSdB FFSdB FFSdB FFSdB During T2: FFSdB FFSdB FFSdB FFSdB |  | During T1: Freq 1 Noc: -104+TT dBm/15kHz Freq 2 Noc: -104+TTdBm/15kHz Ês1 / Noc: +16+TTdB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -104+TTdBm/15kHz Freq 2 Noc: -104+TTdBm/15kHz Ês1 / Noc: 0+TTdB Ês2 / Noc: +17+TTdB |  |
| 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX |  |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |
| 6.6.3.3 NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 |  |  | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB |  |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX |  |  | During T1: Noc: -98dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping |  | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#1: RSRP_55 to RSRP_75 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |  |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX |  |  | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  |  | Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB |  | 0dB 0dB 0.5dB Via mapping |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 CSI-RS#1: RSRP_55 to RSRP_75 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 CSI-RS#1: RSRP_58 to RSRP_78 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 |  |
| 6.6.4.4 NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX |  |  | Same as 6.6.4.3 |  | Same as 6.6.4.3 |  | Same as 6.6.4.3 |  |
| 6.6.4.5 NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  |  | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  |
| 6.6.5.1 NR SA FR1 – UTRAN event-triggered reporting in non-DRX |  |  | During T1: Noc: -106dBm/15kHz Ês / Noc: 18dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -106dBm/15kHz Ês / Noc: -2dB Ioc: -70dBm/3.84MHz Îor / Ioc: -1.8dB CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  | During T1: Noc: -106dBm/15kHz Ês / Noc: 18dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -106dBm/15kHz Ês / Noc: -2dB Ioc: -70dBm/3.84MHz Îor / Ioc: -1.8dB CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |  |
| 6.6.6.1 NR SA FR1 SRS-RSRP measurement with non-DRX |  |  | Test Configuration 1: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +4dB Test Configuration 2: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +4dB |  | Test Config 1: T1: 0dB 0dB T2: 0dB +5.25dB Test Config 2: T1: 0dB 0dB T2: 0dB +4.75dB |  | Test Configuration 1: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +9.25dB Test Configuration 2: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +8.75dB |  |
| 6.6.8.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ±5.5dB Reported Differential CSI-SINR values ±4.5dB |  | 0dB 0dB 0dB Via mapping |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_41 to SINR_64 CSI-RS#0: DIFFSINR_0 to DIFFSINR_7 |  |
| 6.6.8.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SS-SINR values ±4dB Reported Differential SS-SINR values ±3dB |  | 0dB 0.5dB 0dB Via mapping |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.50dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 SSB#1: SINR_44 to SINR_61 SSB#0: DIFFSINR_0 to DIFFSINR_5 |  |
| 6.6.8.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ±4.5dB Reported Differential CSI-SINR values ±3.5dB |  | 0dB 0dB 0dB Via mapping |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_43 to SINR_62 CSI-RS#0: DIFFSINR_0 to DIFFSINR_6 |  |
| 6.6.9.1 NR SA FR1 Idle mode CA/DC measurement for FR1 |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB Reported SS-RSRP and SS-RSRQ |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB Via mapping |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB Configuration 1 and 2: Cell 1: RSRP_54 to RSRP_77 RSRQ_55 to RSRQ_72 Cell 2: RSRP_47 to RSRP_70 RSRQ_51 to RSRQ_67 Configuration 3: Cell 1: RSRP_57 to RSRP_80 RSRQ_55 to RSRQ_72 Cell 2: RSRP_50 to RSRP_73 RSRQ_51 to RSRQ_67 |  |
| 6.6.15.1 NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: -3dB During T4: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +8dB During T5: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB Thresh_X_HighP 48dB Reported SS-RSRP and SS-RSRQ |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB During T4: 0dB 0dB 0dB 0dB During T5: 0dB 0dB 0dB 0dB +4dB Via Mapping |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: -3dB During T4: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +8dB During T5: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB 52dB Configuration 1 and 2: Cell 1: RSRP_52 to RSRP_75 RSRQ_54 to RSRQ_71 Cell 2: RSRP_39 to RSRP_62 RSRQ_6 to RSRQ_27 Configuration 3: Cell 1: RSRP_55 to RSRP_78 RSRQ_54 to RSRQ_71 Cell 2: RSRP_39 to RSRP_62 RSRQ_6 to RSRQ_27 |  |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy |  |  | TEST CONFIGURATION 1, 2, |  |  |  |  |  |
|  |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB |  | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  | Test 1: Noc: -107.5dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_34 to RSRP_46 RSRP_34 to RSRP_46 RSRP_35 to RSRP_47 RSRP_35 to RSRP_47 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |  |
|  |  |  | TEST CONFIGURATION 3 |  |  |  |  |  |
|  |  |  | Test 1: Noc: -110dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB |  | Test 1: -0.8dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  | Test 1: Noc: -110.8dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_41 to RSRP_53 Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 RSRP_38 to RSRP_50 RSRP_38 to RSRP_50 RSRP_39 to RSRP_51 RSRP_40 to RSRP_52 RSRP_40 to RSRP_52 depending on operating band |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |
| 6.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy |  |  | Test 1: Noc: Test configuration 1, 2:-106dBm/15kHz Test configuration 3:-110dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported re. SS-RSRP values: ±3dB |  | Test 1: 0 dB 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  | Test 1: Noc: -106 dBm/15kHz Noc: -110 dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |
| 6.7.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_62 to RSRP_81 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 depending on operating band |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  |  |  | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0 dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |  |  |  |  |  |
| 6.7.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported relative RSRP values: ±4.5dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported relative RSRP values: ±4.5dB |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_x-7 to RSRP_x+7 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_x-31 to RSRP_x-18 |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1.5dB wider at each end. |  |  |  |  |  |  |  |  |
| 6.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |
|  | Test 1: Noc: -85dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping |  | Test 1: Noc: -86.5dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_46 to RSRQ_60 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB RSRQ_44 to RSRQ_59 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |
|  | Test 1: Noc: -91dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping |  | Test 1: Noc: -92.6dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB RSRQ_44 to RSRQ_59 |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |  |  |  |  |  |
| 6.7.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  |  |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  |  |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |  |  |  |  |  |
| 6.7.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  |  |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  |  |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 1 and Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1dB wider at each end. |  |  |  |  |  |  |  |  |
| 6.7.3.1 NR SA FR1 SS-SINR measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  |  |  | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  | Test 1: Noc: -93.2dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |  |  |  |  |  |
| 6.7.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±3.5dB |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_32 to SINR_49 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |  |  |  |  |  |
| 6.7.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±4dB |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_x-11 to SINR_x+11 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 1 and Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each for Tests 1 and 2 and 0dB wider at each end for Test 3. |  |  |  |  |  |  |  |  |
| 6.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB RSRP_62 to RSRP_82 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_31 to RSRP_44 RSRP_31 to RSRP_45 RSRP_32 to RSRP_45 RSRP_32 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 depending on operating band |  |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  |  |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 RSRP_35 to RSRP_48 RSRP_35 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_37 to RSRP_51 depending on operating band |  |
| 6.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  |  |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |
| 6.7.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 6.7.4.1.1 |  |  |  | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 |  |
| 6.7.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.2 |  |  |  | Same as 6.7.4.1.2 |  | Same as 6.7.4.1.2 |  |
| 6.7.5.1 NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Test 1: Noc: -91.65dBm/15kHz Ês / Noc: +10.0dB Reported RSRP values: ±9dB (±2dB additionally for extreme conditions) Test 2: Noc: -117dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RSRP values: ±4.5dB (±3.5dB additionally for extreme conditions) |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc: -91.65dBm/15kHz Ês / Noc: 10dB RSRP_48 to RSRP_70 Test 2: Noc: -117dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_30 RSRP_18 to RSRP_30 Depending on operating band |  |
| 6.7.6.1 NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Test 1: Noc: -83dBm/15kHz Ês / Noc: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc: -104.7dBm/15kHz Ês / Noc: -4.0dB Reported RSRQ values: ±3.5dB (±0.5dB additionally for extreme conditions) Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RSRQ values: ±3.5dB (±0.5dB additionally for extreme conditions) |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping Test 3: 0dB 0.8dB Via mapping |  | Test 1: Noc: -83dBm/15kHz Ês / Noc: -1.75dB RSRQ_4 to RSRQ_16 Test 2: Noc: -104.7dBm/15kHz Ês / Noc: -3.2dB RSRQ_0 to RSRQ_16 Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB RSRQ_0 to RSRQ_16 |  |
| 6.7.7.1 NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Test 1: Noc: -88dBm/15kHz Ês / Noc: -1.75dB Reported RS-SINR values: ±3.0dB (±1.0dB additionally for extreme conditions) Test 2: Noc: -108.5dBm/15kHz Ês / Noc: 20.0dB Reported RS-SINR values: ±3.0dB (±1.0dB additionally for extreme conditions) Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RS-SINR values: ±3.5dB (±0.5dB additionally for extreme conditions) |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0.8dB Via mapping |  | Test 1: Noc: -88dBm/15kHz Ês / Noc: -1.75dB SINR_35 to SINR_51 Test 2: Noc: -108.5dBm/15kHz Ês / Noc: 20.0dB SINR_79 to SINR_94 Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB SINR_32 to SINR_49 |  |
| 6.7.8.1 NR SA FR1 SRS-RSRP measurement accuracy | Test Configuration 1: Test 1: Noc = -106dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3dB Test 2: Noc = -88dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±6.5dB Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3dB Test Configuration 2: Test 1: Test 1 is not used when configuring SCS 30kHz Test 2: Noc = -91dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±7dB Test 3: Noc = -114+ ΔBG_offset dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3.5dB |  |  |  | Test Config 1: Test 1: 0dB +0.5dB Via mapping Test 2: 0dB +0.5dB Via mapping Test 3: 0dB +0.75dB for BG_A +0.5dB otherwise Via mapping Test Config 2: Test 1: N/A Test 2: 0dB +0.5dB Via mapping Test 3: 0dB +1.25dB for BG_A +0.5dB otherwise Via mapping |  | Test Config 1: Test 1: Noc = -106dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 30 to 41 Test 2: Noc = -88dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 45 to 62 Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +1.75dB for BG_A Ês / Noc = +1.5dB otherwise Reported SRS-RSRP: BG_A: 23 to 33 BG_C: 23 to 34 BG_D: 24 to 34 BG_E: 24 to 35 Test Config 2: Test 1: N/A Test 2: Noc = -91dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 44 to 63 Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +2.25dB for BG_A Ês / Noc = +1.5dB otherwise Reported SRS-RSRP: BG_A: 26 to 37 BG_C: 26 to 37 BG_D: 26 to 38 BG_E: 27 to 38 |  |
| 6.7.9.1.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  |  | Same as 4.7.7.1 |  | Same as 4.7.7.1 |  | Same as 4.7.7.1 |  |
| Test Configuration 3,6 |  |  | Same as 4.7.7.1 |  | Same as 4.7.7.1 |  | Same as 4.7.7.1 |  |
| 6.7.9.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  |  | Same as 4.7.7.2 |  | Same as 4.7.7.2 |  | Same as 4.7.7.2 |  |
| Test Configuration 3,6 |  |  | Same as 4.7.7.2 |  | Same as 4.7.7.2 |  | Same as 4.7.7.2 |  |
| 6.7.9.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  |  | Same as 4.7.7.3 |  | Same as 4.7.7.3 |  | Same as 4.7.7.3 |  |
| Test Configuration 3,6 |  |  | Same as 4.7.7.3 |  | Same as 4.7.7.3 |  | Same as 4.7.7.3 |  |

<< End of Changes >>