3GPP TSG-RAN5 Meeting #99	R5-232945r1

# Incheon, Korea, 22th May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2792 | rev |  | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update PDCCH-Config for MSS condtion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1.PDCCH for Multicast could reuse the CORESET#1 configured for USS, so there is not need to reconfigure the CORESET in PDCCH-ConfigMulticast. 2.14.2.1.1.2/6/9 new Multicast TC need to transmit DCI 4-2, the common configuration for PDCCH-Config only support transmit DCI 4-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.If the PDCCH-Config is used for Multicast(i.e MSS condition), CORESET#1 does not need to configure. 2.Update searchSpaceType-r17 to indicate dci-Format4-1-AndFormat4-2-r17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC need to introduce more specific message content to reconfigure PDCCH-Config. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Update r17 IE |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	PDCCH-Config

Table 4.6.3-95: PDCCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList | Not present |  | EN-DC,MSS |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 1 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet | entry 1 |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace with condition USS | entry 1 |  |
|  | SearchSpace with condition MSS | entry 1 | MSS |
|  | SearchSpace with condition SLSS | entry 1 | SLSS |
|  | SearchSpace with condition DCI_2_6 | entry 1 | DCI_2_6 |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| controlResourceSetToAddModListSizeExt-v1610 | Not present |  |  |
| controlResourceSetToReleaseListSizeExt-r16 | Not present |  |  |
| searchSpacesToAddModListExt-r16 | Not present |  |  |
| searchSpacesToAddModListExt-r16 SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-r16 { | 2 entries |  | DCI_2_6 |
| SearchSpaceExt-r16[1] SEQUENCE{ |  | entry 1 |  |
| controlResourceSetId-r16 | Not present |  |  |
| searchSpaceType-r16 | Not present |  |  |
| searchSpaceGroupIdList-r16 | Not present |  |  |
| freqMonitorLocations-r16 | Not present |  |  |
| } |  |  |  |
| SearchSpaceExt-r16[2] SEQUENCE{ |  | entry 2 |  |
| controlResourceSetId-r16 | ControlResourceSetId |  |  |
| searchSpaceType-r16 SEQUENCE { |  |  |  |
| common-r16 SEQUENCE { |  |  |  |
| dci-Format2-4-r16 | Not present |  |  |
| dci-Format2-5-r16 | Not present |  |  |
| dci-Format2-6-r16 SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceGroupIdList-r16 | Not present |  |  |
| freqMonitorLocations-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| uplinkCancellation-r16 | Not present |  |  |
| monitoringCapabilityConfig-r16 | Not present |  |  |
| searchSpaceSwitchConfig-r16 | Not present |  |  |
| searchSpacesToAddModListExt-v1700 | Not present |  |  |
| searchSpacesToAddModListExt-v1700 SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v1700 { | 1 entry |  | MSS |
| SearchSpaceExt-v1700[1] SEQUENCE { |  | entry 1 |  |
| monitoringSlotPeriodicityAndOffset-v1710 | Not present |  |  |
| monitoringSlotsWithinSlotGroup-r17 | Not present |  |  |
| duration-r17 | Not present |  |  |
| searchSpaceType-r17 SEQUENCE { |  |  |  |
| common-r17 SEQUENCE { |  |  |  |
| dci-Format4-0-r17 | Not present |  |  |
| dci-Format4-1-r17 | Not present |  |  |
| dci-Format4-2-r17 | Not present |  |  |
| dci-Format4-1-AndFormat4-2-r17 SEQUENCE { |  |  |  |
| } |  |  |  |
| dci-Format2-7-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceGroupIdList-r17 | Not present |  |  |
| searchSpaceLinkingId-r17 | Not present |  |  |
| } |  |  |  |
| monitoringCapabilityConfig-v1710 | Not present |  |  |
| searchSpaceSwitchConfig-r17 | Not present |  |  |
| pdcch-SkippingDurationList-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MSS | SearchSpace for MBS Multicast receptioin |
| SLSS | SearchSpace for SL mode 1 transmission |
| DCI_2_6 | Transmission of DCI_2_6 is required |

<End of modified section 1>