3GPP TSG-RAN5 Meeting #99	R5-233021r1

# Incheon, Korea, 22th May 2023 – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.901-5 | CR | 0033 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction in A.3.1.1.2 and default message content |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, FS_UE_5GNR_App_Data_Perf |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A.3.1.1.2 is defined as 2Rx TDD scenario but table A.3.1.1.2.3-1 refers to Demod 4Rx FDD test case. There is a need to define a new Annex for generic message exception applicable to Application Layer Data Throughput test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected reference Demod test case for A.3.1.1.2 as 5.2.2.2.1_1 2Rx TDD Added new Annex H as Default message content for Application Layer Data Throughput |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec will have inconsistencies with references to TS 38.521-4 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1.1.2, new Annex H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 fixes correct correlation matrix/antenna configuration and SNR levels aligned with Demod 5.2.2.2.1_1 2Rx TDD |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### A.3.1.1.2	5G NR /TCP Downlink Throughput /Conducted/Fading/2Rx TDD/FR1 PDSCH mapping Type A performance - 2x2 MIMO for SA and NSA

A.3.1.1.2.1	Definition

The UE application layer downlink performance for TCP is determined by the UE application layer TCP throughput.

A.3.1.1.2.2	Test Purpose

To measure the performance of the 5G NR UE using fixed reference channels and under 2 receive antenna conditions while downloading TCP based data in fading channel environment. The duplex mode is TDD.

A.3.1.1.2.3	Test Parameters

The test points to be used in this test are defined in Table A.3.1.1.2.3-1. Details of these test points are available in Annex D with the test points below referenced directly from Table D.1-2

Table A.3.1.1.2.3-1: FR1 TDD 2RX

| Subtest Number | Propagation condition | Correlation matrix and antenna configuration | TS 38.521-4 Reference |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test Case | Test Number | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | TDLA30-10 | 2x2, ULA Low | 5.2.2.2.1_1 2Rx TDD | 1-3 | 70 | 26.3 |
| 2 | TDLC300-100 | 2x2, ULA Low | 5.2.2.2.1_1 2Rx TDD | 1-4 | 30 | 2.5 |
| 3 | TDLB100-400 | 2x2, ULA Low | 5.2.2.2.1_1 2Rx TDD | 1-1 | 70 | -0.2 |
| 4 | TDLA30-10 | 2x2, ULA Low | 5.2.2.2.1_1 2Rx TDD | 2-1 | 70 | 20.8 |
| 5 | TDLA30-10 | 2x2, ULA Medium | 5.2.2.2.1_1 2Rx TDD | 3-1 | 70 | 19.0 |

Other test parameters are defined in TS 38.521-4 [3] Tables 5.2.2.2.1_1.4-1 and 5.2.2.2.1_1.4-2.

A.3.1.1.2.4	Test Description

A.3.1.1.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. Apply the Initial conditions defined in TS 38.521-4 [3] clause 5.2.2.2.1_1.3.1 with the following additional steps and/or exceptions:

1.1	Connect an application server to the IP output of the SS.

1.2	For an embedded configuration, ensure that the UE has an client test application available. For a tethered configuration, tether the UE to a laptop configured with TCP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

5.	For NSA case, the E-UTRA anchor is configured as per Annex E. Ensure the UE is in RRC_CONNECTED State with generic procedure parameters Connectivity NR for NR/5GC with Connected without Release On, Test Mode Off or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode Off  for EN-DC. Message contents are as per TS 38.521-4 [3] clause 5.2.2.2.1_1.3.3 and 5.2.2.2.1_1.3.3_2 and Tables 5.2.2.2.1_1.3.3_1-1 through 5.2.2.2.1_1.3.3_1-4.

A.3.1.1.2.4.2	Procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to TS 38.521-4 [3] Table 5.2.2.2.1.0-2. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the Modulation format and code rate, reference channel, the propagation condition, the correlation matrix and the SNR according to TS 38.521-4 [3] Tables 5.2.2.2.1_1.4-1 and 5.2.2.2.1_1.4-2 as applicable to the test points in Table A.3.1.1.2.3-1.

3.	Using the data client, begin TCP downlink data transfer from the application server. Wait for 15 seconds and then start recording the TCP throughput result. (This is iteration 1) Continue data transfer for the test duration outlined in Table A.1-1-1. Repeat transfer for iterations [2-3] within the same call as the first iteration.

4.	Repeat step 3 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

5.	Calculate and record the average application layer data throughput across three iterations. Additionally, count and record the overall number of ACK and NACK/DTX on the PUSCH/PUCCH during the test interval purely for information purposes.

6.	Record the IP address type (IPv4 or IPv6) used during the TCP data transfers.

7.	Using the values in Table 5.4.4-2 (for IPv6) and Table 5.4.4-3 (for IPv4), determine the reduction from PHY reference fractional throughput value listed in Table A.2.1.1.3-1 to obtain reference Application Layer Throughput value.

## <<< Skip unchanged sections >>>

Annex G: Applicability

This clause adds appropriate applicability references to TS 38.521-4 [3].

For FR1 testing: The applicability requirements for different number of RX antenna ports shall follow the principles defined in TS 38.521-4 [3] clauses 5.1.1.2, 5.1.1.3 and 5.1.1.4For FR2 testing: The applicability requirements for different number of RX antenna ports shall follow the principles defined in TS 38.521-4 [3] clauses 7.1.1.2, 7.1.1.3 and 7.1.1.4.

For Interworking scenarios testing the guidance from TS 38.521-4 [3] clause 9.1.1.1 applies.

Annex H: Default message content for Application Layer Data Throughput

# H.1	Radio resource control information elements

# As defined in clause TS 38.521-4 [3] with the following exceptions:

PDCCH-config

Table H.1-1: PDCCH Search Space

| Derivation Path: TS 38.508-1, Table 5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n2 | 1 for UL, 1 for DL |  |
| aggregationLevel8 | n0 |  |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  |  |
| ue-Specific SEQUENCE { |  |  |  |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Annex I: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2019-08 | RAN5#84 | R5-197487 | 6898 | - | Initial draft created | - | 0.1.0 |
| 2019-08 | RAN5#84 | R5-197482 | - | - | TP for TR 37.901-5 on Definition of 5G NR UE Application Layer Data | - | 0.1.0 |
| 2019-08 | RAN5#84 | R5-197484 | - | - | TP for TR 37.901-5 on Transport and Application Layer Protocols | - | 0.1.0 |
| 2019-08 | RAN5#84 | R5-196905 | - | - | TP for TR 37.901-5 on Parameters for Measurement | - | 0.1.0 |
| 2019-08 | RAN5#84 | R5-197483 | - | - | TP for TR 37.901-5 on Test Configurations | - | 0.1.0 |
| 2019-08 | RAN5#84 | R5-197484 | - | - | TP for TR 37.901-5 on Transport and Application Layer Protocols | - | 0.1.0 |
| 2019-08 | RAN5#84 | R5-196908 | - | - | TP for TR 37.901-5 on Test Environment | - | 0.1.0 |
| 2019-08 | RAN5#84 | R5-197485 | - | - | TP for TR 37.901-5 on Data Transfer Scenarios | - | 0.1.0 |
| 2019-08 | RAN5#84 | R5-196910 | - | - | TP for TR 37.901-5 on Test Procedure Structure and Content for App Tput measurements | - | 0.1.0 |
| 2019-08 | RAN5#84 | R5-196911 | - | - | TP for TR 37.901-5 on Test System Uncertainty and Test Tolerance | - | 0.1.0 |
| 2019-11 | RAN5#85 | R5-199537 | - | - | TP to TR 37.901-5 to introduce test points within test cases in Annex A | 0.1.0 | 0.2.0 |
| 2019-11 | RAN5#85 | R5-199538 | - | - | Text Proposal for Section 5.7 - Statistical Analysis | 0.1.0 | 0.2.0 |
| 2019-11 | RAN5#85 | R5-199539 | - | - | Text Proposal to update TC A.2.1.1 | 0.1.0 | 0.2.0 |
| 2019-11 | RAN5#85 | R5-199429 | - | - | Text Proposal to add TC A.3.1.2.1 | 0.1.0 | 0.2.0 |
| 2019-11 | RAN5#85 | R5-199540 | - | - | Text Proposal to add TC A.4.1.1 | 0.1.0 | 0.2.0 |
| 2019-11 | RAN5#85 | R5-199430 | - | - | Text Proposal to add TC A.6.1 | 0.1.0 | 0.2.0 |
| 2019-11 | RAN5#85 | R5-199431 | - | - | Text Proposal to add TC A.7.1.1.1 | 0.1.0 | 0.2.0 |
| 2019-11 | RAN5#85 | R5-199432 | - | - | Text Proposal to add TC A.9.1.1.1 | 0.1.0 | 0.2.0 |
| 2019-11 | RAN5#85 | R5-199541 | - | - | Text Proposal to update Upper Layer Parameters | 0.1.0 | 0.2.0 |
| 2019-11 | RAN5#85 | R5-199542 | - | - | Text Proposal to study and update Application Throughput Details | 0.1.0 | 0.2.0 |
| 2020-03 | RAN5#86 | R5-200781 | - | - | Text Proposal to add TC A.2.1.2 | 1.0.0 | 1.1.0 |
| 2020-03 | RAN5#86 | R5-200782 | - | - | Text Proposal to add TC A.4.1.2 | 1.0.0 | 1.1.0 |
| 2020-03 | RAN5#86 | R5-200783 | - | - | Text Proposal to update TC A.7.1.1.1 | 1.0.0 | 1.1.0 |
| 2020-03 | RAN5#86 | R5-200784 | - | - | Text Proposal to update TC A.9.1.1.1 | 1.0.0 | 1.1.0 |
| 2020-03 | RAN5#86 | R5-200789 | - | - | Text Proposal to add TC A.3.1.1.1 | 1.0.0 | 1.1.0 |
| 2020-03 | RAN5#86 | R5-200802 | - | - | Text Proposal to add TC A.3.1.1.2 | 1.0.0 | 1.1.0 |
| 2020-03 | RAN5#86 | R5-200817 | - | - | Text Proposal to add TC A.3.1.2.2 | 1.0.0 | 1.1.0 |
| 2020-03 | RAN5#86 | R5-200855 | - | - | Text Proposal to add TC A.5.1.1.1 | 1.0.0 | 1.1.0 |
| 2020-03 | RAN5#86 | R5-200860 | - | - | Text Proposal to add TC A.5.1.1.2 | 1.0.0 | 1.1.0 |
| 2020-03 | RAN5#86 | R5-200865 | - | - | Text Proposal to add TC A.5.1.2.1 | 1.0.0 | 1.1.0 |
| 2020-03 | RAN5#86 | R5-200870 | - | - | Text Proposal to add TC A.5.1.2.2 | 1.0.0 | 1.1.0 |
| 2020-06 | RAN5#87 | R5-202053 | - | - | Text proposal to add A.8.1.1 | 1.1.0 | 1.2.0 |
| 2020-06 | RAN5#87 | R5-202469 | - | - | Text Proposal to update Clause 5.5 - Test Environment | 1.1.0 | 1.2.0 |
| 2020-06 | RAN5#87 | R5-202470 | - | - | Text Proposal to update Clause 5.9 - Test Tolerances | 1.1.0 | 1.2.0 |
| 2020-06 | RAN5#87 | R5-202861 | - | - | Text Proposal to update VRC based App tput sections in TR 37.901-5 | 1.1.0 | 1.2.0 |
| 2020-06 | RAN5#87 | R5-202472 | - | - | Text Proposal to update TC A.7.1.1.1 | 1.1.0 | 1.2.0 |
| 2020-06 | RAN5#87 | R5-202473 | - | - | Text Proposal to update TC A.9.1.1.1 | 1.1.0 | 1.2.0 |
| 2020-06 | RAN5#87 | R5-202474 | - | - | Text Proposal to update TC A.8.1 | 1.1.0 | 1.2.0 |
| 2020-06 | RAN#88 | RP-201142 | - | - | Presented to RAN#88 plenary for approval | 2.0.0 | 2.0.0 |
| 2020-06 | RAN5#87 | - | - | - | Raised to v16.0.0 with editorial changes only | 2.0.0 | 16.0.0 |
| 2020-09 | RAN5#88 | R5-204318 | 0001 | - | Text Proposal to update Clause 5.5 - Test Environment | 16.0.0 | 16.1.0 |
| 2020-09 | RAN5#88 | R5-204894 | 0002 | 1 | Text Proposal to update TC A.7.1.1.1 | 16.0.0 | 16.1.0 |
| 2020-09 | RAN5#88 | R5-204895 | 0003 | 1 | Text Proposal to update TC A.9.1.1.1 | 16.0.0 | 16.1.0 |
| 2020-12 | RAN5#89 | R5-206730 | 0007 | 1 | Editorial updates to Annex in TR 37.901-5 | 16.1.0 | 16.2.0 |
| 2020-12 | RAN5#89 | R5-206795 | 0005 | 1 | Corrections to the test procedures for Application Layer Data Throughput | 16.1.0 | 16.2.0 |
| 2020-12 | RAN5#89 | R5-206796 | 0006 | 1 | Corrections across TR 37.901-5 | 16.1.0 | 16.2.0 |
| 2021-03 | RAN5#90 | R5-210593 | 0008 | - | Updates to 37.901-5 Annex A for Downlink Throughput tests with Fading and FRC scenario | 16.2.0 | 16.3.0 |
| 2021-03 | RAN5#90 | R5-210594 | 0009 | - | Updates to connection diagram for Application Layer Data Throughput | 16.2.0 | 16.3.0 |
| 2021-03 | RAN5#90 | R5-210597 | 0012 | - | Updates to Conclusion for Application Layer Data Throughput | 16.2.0 | 16.3.0 |
| 2021-03 | RAN5#90 | R5-211782 | 0010 | 1 | Updates to Impact of Modem Performance in Application Layer Throughput | 16.2.0 | 16.3.0 |
| 2021-06 | RAN5#91 | R5-214097 | 0013 | 1 | Updates to 37.901-5 Annex A for Downlink Throughput tests with Fading and FRC scenario | 16.3.0 | 16.4.0 |
| 2021-09 | RAN5#92 | R5-216121 | 0014 | 1 | Updates to 37.901-5 Annex A for Downlink Throughput tests with Variable Reference Channel | 16.4.0 | 16.5.0 |
| 2021-09 | RAN5#92 | R5-216139 | 0015 | 1 | Addition of RAN4 agreed contents for VRC scenarios to 37.901-5 | 16.4.0 | 16.5.0 |
| 2021-12 | RAN5#93 | R5-217172 | 0017 | - | Updates to 37.901-5 Annex A for Downlink Throughput tests with FR1 4Rx Variable Reference Channel | 16.5.0 | 16.6.0 |
| 2021-12 | RAN5#93 | R5-217177 | 0018 | - | Updates to 37.901-5 Annex A for Downlink Throughput tests with FR1 2Rx Variable Reference Channel | 16.5.0 | 16.6.0 |
| 2021-12 | RAN5#93 | R5-217178 | 0019 | - | Updates to 37.901-5 Annex A for Downlink Throughput tests with FR2 2Rx Variable Reference Channel | 16.5.0 | 16.6.0 |
| 2021-12 | RAN5#93 | R5-217179 | 0020 | - | Updates to 5G NR /UDP Downlink Throughput /Radiated/Fading/2Rx TDD/FR2 PDSCH mapping Type A performance - for SA and NSA | 16.5.0 | 16.6.0 |
| 2021-12 | RAN5#93 | R5-217181 | 0022 | - | Updates to Test System Uncertainty and Test Tolerance for FR2 | 16.5.0 | 16.6.0 |
| 2021-12 | RAN5#93 | R5-218278 | 0021 | 1 | Updates to test environment for radiated testing for FR2 | 16.5.0 | 16.6.0 |
| 2021-12 | RAN5#93 | R5-218279 | 0023 | 1 | Updates to 37.901-5 Annex B, Annex D and Annex E | 16.5.0 | 16.6.0 |
| 2022-03 | RAN5#94 | R5-220514 | 0025 | - | Updates to A.7.1.2.1 and A.9.1.2.1 | 16.6.0 | 16.7.0 |
| 2022-03 | RAN5#94 | R5-221784 | 0026 | 1 | Updates to A.10 and Annex.11 for Downlink Throughput tests with Variable Reference Channel | 16.6.0 | 16.7.0 |
| 2022-03 | RAN5#94 | R5-221785 | 0027 | 1 | Updates to Conclusion | 16.6.0 | 16.7.0 |
| 2022-03 | RAN5#94 | R5-221786 | 0028 | 1 | Updates to Annex.B | 16.6.0 | 16.7.0 |
| 2022-06 | RAN5#95 | R5-222564 | 0029 | - | Updates to A.7.1.1.1 and A.9.1.1.1 test points | 16.7.0 | 16.8.0 |
| 2022-09 | RAN5#96 | R5-224495 | 0031 | - | Editorial Changes to 37.901-5 | 16.8.0 | 16.9.0 |
| 2022-09 | RAN5#96 | R5-225207 | 0032 | - | Update on initial condition to avoid Activate Test Mode in TR 37.901-5 | 16.8.0 | 16.9.0 |
| 2022-09 | RAN5#96 | R5-225839 | 0030 | 1 | Updates to Radiated Static Channel Tests A.6.1.1 and A.8.1.1 | 16.8.0 | 16.9.0 |

## <<< END OF CHANGES >>>