3GPP TSG-RAN5 Meeting #99	R5-233176r1

# Incheon, South Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0293 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of applicability for FR2 CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Additional spurious emissions test cases are missing for EN-DC with FR2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional spurious emissions test cases for 2 NR CCs, 3 NR CCs and 4 NR CCs have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Additional spurious emissions CA test cases for FR2 EN-DC devices will not progress. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2, 4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 CR 1618 TS 38.521-2 CR 0940 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | associated TC CR in R5-233168 and R5-233169 r1: moved Note 1 to branch column in order to align with the agreed proposal in discussion paper R5-232823 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 4.1.2	FR2 standalone conformance test cases

Table 4.1.2-1: Applicability of RF SA FR2 conformance test cases, ref. TS 38.521-2 [2]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |
| 6.5A.3.1.1 | General spurious emissions for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 |  | Skip TC 6.5A.3.1.1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1_1.1 has been executed. |
| 6.5A.3.1.2 | General spurious emissions for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 |  | Skip TC 6.5A.3.1.2 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1_1.2 has been executed. |
| 6.5A.3.1.3 | General spurious emissions for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 |  | Skip TC 6.5A.3.1.3 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1_1.3 has been executed. |
| 6.5A.3.1.4 | Transmitter Spurious emissions for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  |  |
| 6.5A.3.1.5 | Transmitter Spurious emissions for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  |  |
| 6.5A.3.1.6 | Transmitter Spurious emissions for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  |  |
| 6.5A.3.1.7 | Transmitter Spurious emissions for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  |  |
| 6.5A.3.2.1 | Spurious emission band UE co-existence for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 CA (2UL CA) | E020 |  | Skip TC 6.5A.3.2.1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.2_1.1 has been executed. |
| 6.5A.3.2.2 | Spurious emission band UE co-existence for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 |  | Skip TC 6.5A.3.2.2 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.2_1.2 has been executed. |
| 6.5A.3.2.3 | Spurious emission band UE co-existence for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 |  | Skip TC 6.5A.3.2.3 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.2_1.3 has been executed. |
| 6.5A.3.2.4 | Spurious emission band UE co-existence for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  |  |
| 6.5A.3.2.5 | Spurious emission band UE co-existence for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  |  |
| 6.5A.3.2.6 | Spurious emission band UE co-existence for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  |  |
| 6.5A.3.2.7 | Spurious emission band UE co-existence for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  |  |
| 6.5A.3.3.1 | Additional spurious emissions for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 CA (2UL CA) | E020 | PC1 |  |
| 6.5A.3.3.2 | Additional spurious emissions for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 |  |
| 6.5A.3.3.3 | Additional spurious emissions for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 |  |
| 6.5A.3.3.4 | Additional spurious emissions for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  | NOTE 1 |
| 6.5A.3.3.5 | Additional spurious emissions for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  | NOTE 1 |
| 6.5A.3.3.6 | Additional spurious emissions for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  | NOTE 1 |
| 6.5A.3.3.7 | Additional spurious emissions for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band/CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-2. NOTE 2: Void. NOTE 3: Void. NOTE 4: Void. |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 4.1.3	NR interworking between NR FR1 and NR FR2 and between NR and LTE conformance test cases

Table 4.1.3-1: Applicability of RF EN-DC FR1 and FR2 conformance test cases, ref. TS 38.521-3 [3]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA/DC-Configurations Selection | Branch | Additional Information |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |  |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |
| 6.5B.3.4.2_1 | Spurious emission band UE co-existence for Inter-band including FR2 (>1 NR CC) |  |  |  |  |  |  |  |
| 6.5B.3.4.2_1.1 | Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (2NR CCs) | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 |  | NOTE 5 Skip TC 6.5B.3.4.2_1.1 if UE supports SA and TS 38.521-2 TC 6.5A.3.2.1 has been executed. |  |
| 6.5B.3.4.2_1.2 | Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (3NR CCs) | Rel-15 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 |  | NOTE 1 NOTE 5 Skip TC 6.5B.3.4.2_1.2 if UE supports SA and TS 38.521-2 TC 6.5A.3.2.2 has been executed. |  |
| 6.5B.3.4.2_1.3 | Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (4NR CCs) | Rel-15 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 |  | NOTE 1 NOTE 5 Skip TC 6.5B.3.4.2_1.3 if UE supports SA and TS 38.521-2 TC 6.5A.3.2.3 has been executed. |  |
| 6.5B.3.4.1_1.4 | General Spurious Emissions for Inter-band EN-DC including FR2 (5 NR CCs) | Rel-15 | C012i | UEs supporting Inter-Band EN-DC including FR2 with 5 NR UL CCs | E014a |  | NOTE 1 NOTE 5 Skip TC 6.5B.3.4.1_1.4 if UE supports SA and TS 38.521-2 TC 6.5A.3.1.4 has been executed. |  |
| 6.5B.3.4.1_1.5 | General Spurious Emissions for Inter-band EN-DC including FR2 (6 NR CCs) | Rel-15 | C012j | UEs supporting Inter-Band EN-DC including FR2 with 6 NR UL CCs | E039 |  | NOTE 1 NOTE 5 Skip TC 6.5B.3.4.1_1.5 if UE supports SA and TS 38.521-2 TC 6.5A.3.1.5 has been executed. |  |
| 6.5B.3.4.1_1.6 | General Spurious Emissions for Inter-band EN-DC including FR2 (7 NR CCs) | Rel-15 | C012k | UEs supporting Inter-Band EN-DC including FR2 with 7 NR UL CCs | E040 |  | NOTE 1 NOTE 5 Skip TC 6.5B.3.4.1_1.6 if UE supports SA and TS 38.521-2 TC 6.5A.3.1.6 has been executed. |  |
| 6.5B.3.4.1_1.7 | General Spurious Emissions for Inter-band EN-DC including FR2 (8 NR CCs) | Rel-15 | C012l | UEs supporting Inter-Band EN-DC including FR2 with 8 NR UL CCs | E041 |  | NOTE 1 NOTE 5 Skip TC 6.5B.3.4.1_1.7 if UE supports SA and TS 38.521-2 TC 6.5A.3.1.7 has been executed. |  |
| 6.5B.4.1 | Additional Spurious Emissions for Intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  |  |  |
| 6.5B.4.2 | Additional Spurious Emissions for Intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 1 |  |
| 6.5B.4.3 | Additional Spurious Emissions for Inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.5B.4.3 if UE supports SA and TS 38.521-1 TC 6.5.3.3 has been executed. |  |
| 6.5B.4.4 | Additional Spurious Emissions for Inter-band including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.5B.4.4 if UE supports SA and TS 38.521-2 TC 6.5.3.3 has been executed. |  |
| 6.5B.4.4a | Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) | Rel-16 | C012w | UEs supporting inter-band EN-DC including FR2 with 1 NR UL CC and mpr-PowerBoost-FR2-r16 | E010 | PC1 PC2 PC3 PC4 |  |
| 6.5B.4.4_1 | Additional Spurious Emissions for Inter-band including FR2 (>1 NR CC) |  |  |  |  |  |  |  |
| 6.5B.4.4_1.1 | Additional Spurious Emissions for Inter-band including FR2 (2 NR CC) | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 | PC1 NOTE 1 | NOTE 5 Skip TC 6.5B.4.4_1.1 if UE supports SA and TS 38.521-2 TC 6.5A.3.3.1 has been executed. |  |
| 6.5B.4.4_1.2 | Additional Spurious Emissions for Inter-band including FR2 (3 NR CC) | Rel-15 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 | PC1 NOTE 1 | NOTE 5 Skip TC 6.5B.4.4.1_1.2 if UE supports SA and TS 38.521-2 TC 6.5A.3.3.2 has been executed. |  |
| 6.5B.4.4_1.3 | Additional Spurious Emissions for Inter-band including FR2 (4 NR CC) | Rel-15 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 | PC1 NOTE1 | NOTE 5 Skip TC 6.5B.4.4.1_1.3 if UE supports SA and TS 38.521-2 TC 6.5A.3.3.3 has been executed. |  |
| 6.5B.5.3 | Transmit Intermodulation for Inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting Inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.5B.5.3 if UE supports SA and TS 38.521-1 TC 6.5.4 has been executed. |  |
| 6.6B.4 | Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP | Rel-15 | C011b | UEs supporting Inter-band EN-DC within FR2 with 1 NR UL CC and not beam correspondence without UL beam sweeping and release 16 and forward UEs that do not support SSB-based or CSI-RS based enhanced beam correspondence and do not support enhanced beam correspondence without UL beam sweeping | E005b |  | NOTE 1 NOTE 5 Skip TC 6.6B.4 if UE supports SA and TS 38.521-1 TC 6.6.1 has been executed. |  |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-3. NOTE 2: Void. NOTE 3: Void. NOTE 4: Void. NOTE 5: Test only one EN-DC combination per 5G NR band as LTE anchor agnostic approach is applied. |  |  |  |  |  |  |  |  |

< End of changes >