3GPP RAN5#99 - TTCN Sidebar

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Agenda Item:	5G

Source:	MCC TF160

Title:	NR RedCap Half-Duplex FDD mode

Document for:	Endorsement

# 1	Introduction

The test model and ASPs need to be enhanced to cover RedCap Half-Duplex FDD UE (HD-UE).

# 2.	Generic Data scheduling

In FDD, SCS =15kHz, the following parameters apply:

k0=0

k1=7

k2=4

The following DL data scheduling applies:

SSB#0 and #1: subframe#0

SIB1: subframe#1

SIs: subframe#5 and#6

Paging: subframe#1 and #2 depending on SSB#

Sysinfo modification: subframes #0 #1 #2 depending on SSBs = DCI format 1_0 with CRC scrambled by P-RNTI

The prose CR R5-232937 was raised (at current RAN5#99) to modify PDCCH SearchSpace for CSS and USS and prach-ConfigurationIndex.

| Subframe | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDCCH SearchSpace CSS & USS |  |  |  |  |  |  |  |  |  |  |
| DL | DCI P-RNTI / PDCCH UL grant#1 / SSBs | DCI P-RNTI / SIB1 | DCI P-RNTI / PDSCH#1 |  |  | PDCCH UL grant#2 / SI | SI | PDSCH#2 |  |  |
| UL |  |  |  | Gap T_Proc,2 | RA occasion / PUSCH#1 / HARQ of PDSCH#2 |  |  |  | Gap T_Proc,2 | PUSCH#2 / HARQ of PDSCH#1 |

## 2.1.	Proposed changes in 38.523-3

See R5-232191

## 2.2.	ASP changes

#### D.1.3.2.1	PhysicalLayer_Common

NR_CellConfigPhysicalLayerCommon_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_CellConfigPhysicalLayerCommon_Type |  |  |
| Comment | Configuration common for UL and DL |  |  |
| PhysicalCellId | PhysCellId | opt | Physical-layer cell identity according to 38.211 clause 7.4.2.1; EN-DC: corresponds to ServingCellConfigCommon.physCellId |
| DuplexMode | NR_DuplexMode_Type | opt | FDD or TDD; FDD/TDD specific parameters |

NR_DuplexMode_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DuplexMode_Type |  |
| Comment | FDD/TDD and maybe other types of duplex mode; in general FDD/TDD mode is determined from the frequency band |  |
| FDD | NR_FDD_Info_Type |  |
| TDD | NR_TDD_Info_Type |  |

NR_FDD_Info_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_FDD_Info_Type |  |  |
| Comment | FDD (paired spectrum) |  |  |
| HalfDuplexFDD | boolean | opt | "true" for RedCap half-duplex FDD. When configured, the SS shall operate as per the HD-FDD SS requirements specified in 38.523-3 clause 7.3.2.2.2. May be omitted at initial configuration when it is not applied. |

NR_TDD_UL_DL_SlotConfigList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_TDD_UL_DL_SlotConfigList_Type |
| Comment | corresponds to ServingCellConfig.tdd-UL-DL-ConfigurationDedicated |
| record of NR_ASN1_TDD_UL_DL_SlotConfig_Type |  |

NR_TDD_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_TDD_Config_Type |  |  |
| Comment | Common and dedicated TDD configuration |  |  |
| Common | NR_ASN1_TDD_UL_DL_ConfigCommon_Type | opt | Common TDD configuration as used in TS 38.213 clause 11 corresponding to ServingCellConfigCommon.tdd-UL-DL-ConfigurationCommon; shall be present for TDD at initial configuration |
| Dedicated | NR_TDD_UL_DL_SlotConfigList_Type | opt | Dedicated TDD configuration for single slots over-ruling the flexible slots of the common configuration; corresponds to ServingCellConfig.tdd-UL-DL-ConfigurationDedicated; shall be present for TDD at initial configuration: the list is empty when there is no dedicated slot configuration; (omit means "keep as it is"); NOTE: The dedicated configuration can only exist together with common configuration as a single slot configuration is related to the periodicity given by the common configuration (see TDD-UL-DL-SlotConfig field description for slotIndex in TS 38.331) |

NR_TDD_Info_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_TDD_Info_Type |  |
| Comment | cell specific parameters for TDD (unpaired spectrum) |  |
| Config | NR_TDD_Config_Type | specific TDD configuration with sets of symbols for UL and DL and possibly flexible symbols which are not specified as UL or DL (corresponding to TDD-UL-DL-ConfigurationCommon and TDD-UL-DL-ConfigDedicated according to TS 38.213 clause 11.1) |
| FullFlexible | Null_Type | No TDD configuration is provided to the UE: all slots and symbols are considered as flexible according to TS 38.213 clause 11.1 |