3GPP TSG-RAN5 Meeting #99	R5-23xxxx

# Incheon, South Korea, 22th May 2023 - 26th May 2023

Source:	Huawei, HiSilicon

Title:	Discussion on issue of TC 6.1.2.3 SIB1 PDCCH crush

- Background

The SIB1 PDCCH resources of intra-frequency cells are crushed according to the currect test specification. So UE may probably fail to demodulate the PDCCH because of the negative SNR, especially for SIG TC 6.1.2.3.

- Discussion

## 2.1 The test environment in TC 6.1.2.3

This testcase uses NR Cell 1 and NR Cell 11, these are intra-frequency neighbor cells.

And at T2, UE need to perform Cell selection to NR Cell 1, which is set to -94dB, however the neighbor cell NR Cell 11 is set to -88dB.

[TS 38.508-1 clause 4.6.3] 6.1.2.3 Cell selection / Intra NR / Serving cell becomes non-suitable (S<0, MIB Indicated barred) … Table 6.1.2.3.3.2-1: Time instances of cell power level and parameter changes for FR1 Parameter; Unit; NR Cell 1; NR Cell 11; Remark. T0; SS/PBCH SSS EPRE; dBm/SCS; -88; "Off"; The power level values are assigned to ensure the UE registered on NR Cell 1.. Qrxlevmin; dBm; -110; -. T1; SS/PBCH SSS EPRE; dBm/SCS; "Off"; -88; SrxlevNRCell 1 < 0 (NOTE 1). Qrxlevmin; dBm; -; -110. T2; SS/PBCH SSS EPRE; dBm/SCS; -92; -88; SrxlevNRCell 11 > 0, SrxlevNRCell 1 > 0. Qrxlevmin; dBm; -110; -110. cellBarred; -; notBarred; barred; Serving cell becomes barred. NOTE 1: Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3..

## 2.2	Frequency Domain Resource of SIB1 PDCCH - CORESET#0

The intra-frequency neighbor cells will broadcast the same CORESET#0 index in MIB.

[TS 38.508-1 clause 4.6.3] – ControlResourceSetZero Table 4.6.3-30: ControlResourceSetZero Derivation Path: TS 38.331 [6], clause 6.3.2. Information Element; Value/remark; Comment; Condition. ControlResourceSetZero; Set to CORESET#0 Index as defined for the frequency of the cell; For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1..

[TS 38.508-1 clause 6.2.3]

Table 6.2.3.1-1: Test frequencies for NR FDD FR1 bands using 5 MHz channel bandwidth

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | 15 | 5 | 25 | Downlink | Low, High | Same values as for Low and High range in clause 4.3.1.1.15 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 878.2 | 175640 | 873.79 | 174758 | 12 | 15 | 2197 | 175730 | 0 | 1 | 2 (4) | 17 |
|  |  |  |  |  | Mid-High | 884.8 | 176960 | 878.23 | 175646 | 24 |  | 2212 | 176930 | 8 | 1 | 0 (0) | 25 |
|  |  |  |  | Uplink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.5 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 833.2 | 166640 | 824.47 | 164894 | 36 | - | - | - | - | - | - | - |
|  |  |  |  |  | Mid-High | 839.8 | 167960 | 817.03 | 163406 | 114 | - | - | - | - | - | - | - |
| n8 | 15 | 5 | 25 | Downlink | Low, High | Same values as for Low and High range in clause 4.3.1.1.1.8 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |

…

[TS 38.213 clause 13] Table 13-1: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {15, 15} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs NRBCORESET; Number of Symbols NsymbCORESET; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 2. 2; 1; 24; 2; 4. 3; 1; 24; 3; 0. 4; 1; 24; 3; 2. 5; 1; 24; 3; 4. 6; 1; 48; 1; 12. 7; 1; 48; 1; 16. 8; 1; 48; 2; 12. 9; 1; 48; 2; 16. 10; 1; 48; 3; 12. 11; 1; 48; 3; 16. 12; 1; 96; 1; 38. 13; 1; 96; 2; 38. 14; 1; 96; 3; 38. 15; Reserved.

Observation 1: Frequency Domain Resource of SIB1 PDCCH are crushed among intra-freq cells

## 2.3	Time Domain Resource of SIB1 PDCCH – SearchSpace#0

The intra-frequency neighbor cells will broadcast the same SearchSpace#0 index in MIB to schedule SIB1.

[TS 38.508-1 clause 4.6.3] SearchSpaceZero Table 4.6.3-164: SearchSpaceZero Derivation Path: TS 38.331 [6], clause 6.3.2. Information Element; Value/remark; Comment; Condition. SearchSpaceZero; 0; Index addressing SearchSpace#0 parameter set in Tables 13.11 .. 13.15 of TS 38.213 [22].

[TS 38.213 clause 13] For operation without shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH CSS set over two slots. For SS/PBCH block with index i, the UE determines an index of slot n0 as n0=O∙2μ+i∙MmodNslotframe,μ that is in a frame with system frame number (SFN) SFNC satisfying SFNcmod2=0 if O∙2μ+i∙MNslotframe,μmod2=0, or in a frame with SFN satisfying SFNcmod2=1 if O∙2μ+i∙MNslotframe,μmod2=1 where μ∈0,1,2,3,5,6 based on the SCS for PDCCH receptions in the CORESET [4, TS 38.211]. - For μ∈{0, 1, 2, 3} and for a SS/PBCH block index i, the two slots including the associated Type0-PDCCH monitoring occasions are slots n0 and n0+1. M, O, and the index of the first symbol of the CORESET in slots n0 and n0+1 are provided by Table 13-11 and Table 13-12. - For μ=5 and for a SS/PBCH block index i, the two slots including the associated Type0-PDCCH monitoring occasions are slots n0 and n0+4. M, O, and the index of the first symbol of the CORESET in slots n0 and n0+4 are provided by Table 13-12A, where X=1.25. - For μ=6 and for a SS/PBCH block index i, the two slots including the associated Type0-PDCCH monitoring occasions are slots n0 and n0+8. M, O, and the index of the first symbol of the CORESET in slots n0 and n0+8 are provided by Table 13-12A, where X=0.625. … Table 13-11: Parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 and FR1 Index; O; Number of search space sets per slot; M; First symbol index. 0; 0; 1; 1; 0. 1; 0; 2; 1/2; {0, if i is even}, {NsymbCORESET, if i is odd}. 2; 2; 1; 1; 0. 3; 2; 2; 1/2; {0, if i is even}, {NsymbCORESET, if i is odd}. 4; 5; 1; 1; 0. 5; 5; 2; 1/2; {0, if i is even}, {NsymbCORESET, if i is odd}. 6; 7; 1; 1; 0. 7; 7; 2; 1/2; {0, if i is even}, {NsymbCORESET, if i is odd}. 8; 0; 1; 2; 0. 9; 5; 1; 2; 0. 10; 0; 1; 1; 1. 11; 0; 1; 1; 2. 12; 2; 1; 1; 1. 13; 2; 1; 1; 2. 14; 5; 1; 1; 1. 15; 5; 1; 1; 2.

If we try to calculate it for index 0 inTable 13-11,

For SSB#0, i=0, SIB 1 will broadcast in slot 0/1 in even SFN,

For SSB#1, i=1, SIB1 will broadcast in slot 1/2 in even SFN.

…

[TS 38.523-3 clause 7.3.3] All System Information messages are sent only once within the SI-window and redundancy version 0 is selected. SIB1 is broadcasted in slot#1 in frames with even SFN.

Observation 2: Time Domain Resource of SIB1 PDCCH are crushed among intra-freq cells

  - Demodulation requirement for PDCCH channel

In this testcase,

If UE selects the -88dB cell, then SNR is about +6dB (not accurate enough but very close)

If UE selects the -94dB cell, then SNR is about -6dB.

Observation 3: According to RAN4 demod requirement under aggregation level 4 specified in TS 38.101-4 clause 5.3, -6dB is too low and sticky for UEs.

  - SIB1 PDSCH channel

[TS 38.523-3 clause 7.1.2 7.1.2.2.3.1 DL scheduling scheme: Frequency domain multiplexing, RA type1, non-interleaved This scheduling scheme multiplexes the different kinds of PDSCH transmissions in the frequency domain of a single BWP by exclusively using resource allocation type 1 with non-interleaved VRB-to-PRB mapping. Assuming the resource blocks being numbered from 0 to NBWP-1 (with NBWP being the size of the BWP) the following allocation is done: Table 7.1.2.2.3.1-1: Resource allocation for frequency domain multiplexing, RA type1, non-interleaved Kind of PDSCH transmission (Note 1); CORESET#; SSB index of the cell (Note 4); Resource block allocation (Note 2). RBstart (Note 2); LRBs,max (Note 2). System information; 0; SSB#1; 0; 7. SSB#0; NBWP-7; 7. Paging (Note 3); 0; SSB#1; 7; NBWP-7. SSB#0; 0; NBWP-7. Random access response (Note 3); 0; SSB#1; 7; NBWP-7. SSB#0; 0; NBWP-7. CCCH and DCCH transmission on CSS (Note 3); 0; SSB#1; 7; NBWP-7. SSB#0; 0; NBWP-7. DCCH/DTCH transmissions (Note 3); 1; SSB#1; 7+ Offset Carrier CORESET#0 [RBs] (Note 5); NBWP-7. SSB#0; 0; NBWP-7. NOTE 1: In context of a generic 5G test model it is not relevant whether or not there is SI and Paging for a given deployment option (e.g. EN-DC). NOTE 2: LRBs ≤ LRBs,max with LRBs: number of resource blocks being eventually used for a particular transmission. LRBs and RBstart are as specified in TS 38.214 [22] clause 5.1.2.2.2. NOTE 3: In general Paging, Random access response and CCCH/DCCH/DTCH transmissions are mutual exclusive and therefore share the same allocation. NOTE 4: The SSB index used by an NR Cell is specified in TS 38.508-1 [5] Table 4.4.2-2. NOTE 5: Offset Carrier CORESET#0 [RBs] values are specified in the frequency tables in TS 38.508-1 [5]..

Observation 4: PDSCH frequency domain resource has been staggered. So there is no overlapping for SIB1 PDSCH.

- Alternative options:

Option 1: Update power level for T2 in this testcase, that’s is the simplest way at this meeting;

[TS 38.508-1 clause 4.6.3] 6.1.2.3 Cell selection / Intra NR / Serving cell becomes non-suitable (S<0, MIB Indicated barred) … Table 6.1.2.3.3.2-1: Time instances of cell power level and parameter changes for FR1 Parameter; Unit; NR Cell 1; NR Cell 11; Remark. T0; SS/PBCH SSS EPRE; dBm/SCS; -88; "Off"; The power level values are assigned to ensure the UE registered on NR Cell 1.. Qrxlevmin; dBm; -110; -. T1; SS/PBCH SSS EPRE; dBm/SCS; "Off"; -88; SrxlevNRCell 1 < 0 (NOTE 1). Qrxlevmin; dBm; -; -110. T2; SS/PBCH SSS EPRE; dBm/SCS; -92; -88; SrxlevNRCell 11 > 0, SrxlevNRCell 1 > 0. Qrxlevmin; dBm; -110; -110. cellBarred; -; notBarred; barred; Serving cell becomes barred. NOTE 1: Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3..

Option 2: Add a note in this testcase that “SS need to make sure the SIB1 PDCCH scheduling not crushed” or similar words and leave it up to SS implementation,

Option 3: Update TS 38.523-3 with either sub-option below,

Sub-option-3.1: Use different CCEs for intra-freq cells;

Sub-option-3.2: Use different slots  for intra-freq cells;