3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230xxx

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3066 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcases 7.1.1.3.2 and 7.1.1.3.2b |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 7.1.1.3.2b: - According to TS 38.523-1 Table 7.1.1.3.2b.3.2-1: Main behaviour, there are 3 DRBs established. At Step 1: The SS transmits N1 320-octet RLC SDUs on DRB1, N2 320-octet RLC SDUs on DRB2, and N3 320-octet RLC SDUs on DRB3. At Step 3: Check: Are the total number of octets of the UL RLC SDUs received at the SS for each DRB as follows: - total number of octets received for DRB1 is D1 octets +/- 10% - total number of octets received for DRB2 is 0 - total number of octets received for DRB3 is D3 octets +/- 10% otherwise? Similar requirements are placed at Steps 8 (DRB 1) and Step 12 (DRB 2) The local end ASP configuring the 3 DRBs has neither the flexibity to set the t-Reordering timer nor the OutOfOrderDelivery. For example: When DL Data is first sent on DRB 2, the PDCP SN is 0 and it should have been PDCP SN 0 for the first arriving PDU on DRB 2. However, this does not happen as per the TC requirements at Step 3, Since t-Reordering timer is infinity for all DRBs, the received PDUs for DRB 2 (Step 3, Step 12) and DRB 1 (Step 8) are buffered until PDU containing the missing PDCP SN arrives (which will not happen as per the specific test requirements) and therefore *NO UL PDUs* will be delivered to TTCN by the SS Stack for highlighted DRBs. For 7.1.1.3.2 and 7.1.1.3.2b: - The requirements at Step 3 (7.1.1.3.2) and Steps 3, 8 and 12 (7.1.1.3.2b) suggest that it is allowed to lose 10% of the octets for each DRB. This would also lead to the situation outlined above that *NO PDUs* will be delivered to TTCN by the SS Stack for those specific DRBs which have missing PDCP SN. To mitigate this, it is recommended to introduce and set outOfOrderDelivery = true in the local end ASP NR_PDCP_DRB_Config_Parameters_Type so that SS PDCP will deliver all received PDUs to TTCN immediately. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the ASP NR_PDCP_DRB_Config_Parameters_Type to introduce and set the outOfOrderDelivery to TRUE as part of the PDCP Config |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.2, 7.1.1.3.2b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the Function f_TC_7_1_1_3_2_NR5GC	3

2.2	Correction to the Function f_TC_7_1_1_3_2b_NR5GC	5

2.3	Correction to the Record NR_PDCP_DRB_Config_Parameters_Type	9

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_TC_7_1_1_3_2_NR5GC

| Function name | f_TC_7_1_1_3_2_NR5GC |
| --- | --- |
| Reason for change | The requirements at Step 3 (7.1.1.3.2) suggest that it is allowed to lose 10% of the octets for each DRB. This would also lead to the situation that *NO PDUs* will be delivered to TTCN by the SS Stack across all DRBs if there are any missing PDCP SNs for a specific DRB. To mitigate this, it is recommended to introduce and set outOfOrderDelivery = true in the local end ASP NR_PDCP_DRB_Config_Parameters_Type so that SS PDCP will deliver all received PDUs to TTCN immediately |
| Summary of change | Modified the ASP NR_PDCP_DRB_Config_Parameters_Type to introduce and set the outOfOrderDelivery to TRUE as part of the PDCP Config in test cases 7.1.1.3.2 and 7.1.1.3.2b |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_2_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling var DRB_Identity v_DRBId1; var DRB_Identity v_DRBId2; var DRB_Identity v_DRBId3; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var IntegerList_Type v_ListOfDRBs; //@sic R5-196745 sic@ var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) SDAP_Config v_SDAP_Config_DRB; //@sic R5s191054 sic@ var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; //@sic R5-197028 sic@ <<SKIPPED CODE>> //@sic r5s190783 R5s210860 sic@ v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId1,-,-,cs_SDAP_Configuration_None), f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId2,-,-,cs_SDAP_Configuration_None), f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId3,-,-,cs_SDAP_Configuration_None) }; <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_3_2_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling var DRB_Identity v_DRBId1; var DRB_Identity v_DRBId2; var DRB_Identity v_DRBId3; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var IntegerList_Type v_ListOfDRBs; //@sic R5-196745 sic@ var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) SDAP_Config v_SDAP_Config_DRB; //@sic R5s191054 sic@ var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; //@sic R5-197028 sic@ var template (value) NR_PDCP_RbConfig_Type v_PDCP_RbConfig; //WA# <<SKIPPED CODE>> //WA# v_PDCP_RbConfig.Params.Rb.Drb.SN_SizeUL := f_NR_38508_SetPDCP_SN_Size(); v_PDCP_RbConfig.Params.Rb.Drb.SN_SizeDL := f_NR_38508_SetPDCP_SN_Size(); v_PDCP_RbConfig.Params.Rb.Drb.HeaderCompression := { None := true }; v_PDCP_RbConfig.Params.Rb.Drb.IntegrityProtectionEnabled := false; v_PDCP_RbConfig.Params.Rb.Drb.OutOfOrderDelivery := true; //@sic r5s190783 R5s210860 sic@ v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId1,-,v_PDCP_RbConfig,cs_SDAP_Configuration_None), f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId2,-,v_PDCP_RbConfig,cs_SDAP_Configuration_None), f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId3,-,v_PDCP_RbConfig,cs_SDAP_Configuration_None) };

## Correction to the Function f_TC_7_1_1_3_2b_NR5GC

| Function name | f_TC_7_1_1_3_2b_NR5GC |
| --- | --- |
| Reason for change | - According to TS 38.523-1 Table 7.1.1.3.2b.3.2-1: Main behaviour, there are 3 DRBs established. At Step 1: The SS transmits N1 320-octet RLC SDUs on DRB1, N2 320-octet RLC SDUs on DRB2, and N3 320-octet RLC SDUs on DRB3. At Step 3: Check: Are the total number of octets of the UL RLC SDUs received at the SS for each DRB as follows: - total number of octets received for DRB1 is D1 octets +/- 10% - total number of octets received for DRB2 is 0 - total number of octets received for DRB3 is D3 octets +/- 10% otherwise? Similar requirements are placed at Steps 8 (DRB 1) and Step 12 (DRB 2) The local end ASP configuring the 3 DRBs has neither the flexibity to set the t-Reordering timer nor the OutOfOrderDelivery. For example: When DL Data is first sent on DRB 2, the PDCP SN is 0 and it should have been PDCP SN 0 for the first arriving PDU on DRB 2. However, this does not happen as per the TC requirements at Step 3, Since t-Reordering timer is infinity for all DRBs, the received PDUs across all DRBs are buffered until PDU containing PDCP SN 0 arrives (which will not happen) and therefore *NO PDUs* will be delivered to TTCN by the SS Stack across all DRBs. - The requirements at Steps 3, 8 and 12 (7.1.1.3.2b) suggest that it is allowed to lose 10% of the octets for each DRB. This would also lead to the situation outlined above that *NO PDUs* will be delivered to TTCN by the SS Stack across all DRBs if there are any missing PDCP SNs for a specific DRB. To mitigate this, it is recommended to introduce and set outOfOrderDelivery = true in the local end ASP NR_PDCP_DRB_Config_Parameters_Type so that SS PDCP will deliver all received PDUs to TTCN immediately |
| Summary of change | Modified the ASP NR_PDCP_DRB_Config_Parameters_Type to introduce and set the outOfOrderDelivery to TRUE as part of the PDCP Config in test cases 7.1.1.3.2 and 7.1.1.3.2b |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_2b_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling with Mapping restrictions var DRB_Identity v_DRBId1; var DRB_Identity v_DRBId2; var DRB_Identity v_DRBId3; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var IntegerList_Type v_ListOfDRBs; var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) SDAP_Config v_SDAP_Config_DRB; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig ; var template (value) NR_AllowedSCS_List_Type v_NR_AllowedSCS_List_DRB1; var template (value) NR_AllowedSCS_List_Type v_NR_AllowedSCS_List_DRB2; var template (value) NR_AllowedSCS_List_Type v_NR_AllowedSCS_List_DRB3; var boolean v_IsFR1 := false; var template (value) CellGroupConfig v_CellGroupConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var boolean v_RRC_MsgRcvd := false; var boolean v_DRB_Rcvd := false; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; var template(value) SpCellConfig v_SpCellConfig ; var template(value) BWP.locationAndBandwidth v_LocationAndBandwidth; var NR_DMRS_UL_AdditionalPosition_Type v_DMRS_UL_AdditionalPosition; var SubcarrierSpacing v_SubcarrierSpacing ; var ConfigGrant_Period_Type v_ConfigGrant_Period; var integer v_BwpSize ; var integer v_RIV; var SubFrameTiming_Type v_Timing; timer t_Watchdog := 1.0; <<SKIPPED CODE>> v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId1,-,-,cs_SDAP_Configuration_None),//@sic R5s220045 sic@ f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId2,-,-,cs_SDAP_Configuration_None), f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId3,-,-,cs_SDAP_Configuration_None) }; <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_3_2b_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling with Mapping restrictions var DRB_Identity v_DRBId1; var DRB_Identity v_DRBId2; var DRB_Identity v_DRBId3; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var IntegerList_Type v_ListOfDRBs; var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) SDAP_Config v_SDAP_Config_DRB; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig ; var template (value) NR_AllowedSCS_List_Type v_NR_AllowedSCS_List_DRB1; var template (value) NR_AllowedSCS_List_Type v_NR_AllowedSCS_List_DRB2; var template (value) NR_AllowedSCS_List_Type v_NR_AllowedSCS_List_DRB3; var boolean v_IsFR1 := false; var template (value) CellGroupConfig v_CellGroupConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var boolean v_RRC_MsgRcvd := false; var boolean v_DRB_Rcvd := false; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; var template(value) SpCellConfig v_SpCellConfig ; var template(value) BWP.locationAndBandwidth v_LocationAndBandwidth; var NR_DMRS_UL_AdditionalPosition_Type v_DMRS_UL_AdditionalPosition; var SubcarrierSpacing v_SubcarrierSpacing ; var ConfigGrant_Period_Type v_ConfigGrant_Period; var integer v_BwpSize ; var integer v_RIV; var SubFrameTiming_Type v_Timing; timer t_Watchdog := 1.0; var template (value) NR_PDCP_RbConfig_Type v_PDCP_RbConfig; //WA# <<SKIPPED CODE>> //WA# v_PDCP_RbConfig.Params.Rb.Drb.SN_SizeUL := f_NR_38508_SetPDCP_SN_Size(); v_PDCP_RbConfig.Params.Rb.Drb.SN_SizeDL := f_NR_38508_SetPDCP_SN_Size(); v_PDCP_RbConfig.Params.Rb.Drb.HeaderCompression := { None := true }; v_PDCP_RbConfig.Params.Rb.Drb.IntegrityProtectionEnabled := false; v_PDCP_RbConfig.Params.Rb.Drb.OutOfOrderDelivery := true; //@sic r5s190783 R5s210860 sic@ v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId1,-,v_PDCP_RbConfig,cs_SDAP_Configuration_None), f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId2,-,v_PDCP_RbConfig,cs_SDAP_Configuration_None), f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId3,-,v_PDCP_RbConfig,cs_SDAP_Configuration_None) }; <<SKIPPED CODE>>

## Correction to the Record NR_PDCP_DRB_Config_Parameters_Type

| Record name | NR_PDCP_DRB_Config_Parameters_Type |
| --- | --- |
| Reason for change | - According to TS 38.523-1 Table 7.1.1.3.2b.3.2-1: Main behaviour, there are 3 DRBs established. At Step 1: The SS transmits N1 320-octet RLC SDUs on DRB1, N2 320-octet RLC SDUs on DRB2, and N3 320-octet RLC SDUs on DRB3. At Step 3: Check: Are the total number of octets of the UL RLC SDUs received at the SS for each DRB as follows: - total number of octets received for DRB1 is D1 octets +/- 10% - total number of octets received for DRB2 is 0 - total number of octets received for DRB3 is D3 octets +/- 10% otherwise? Similar requirements are placed at Steps 8 (DRB 1) and Step 12 (DRB 2) The local end ASP configuring the 3 DRBs has neither the flexibity to set the t-Reordering timer nor the OutOfOrderDelivery. For example: When DL Data is first sent on DRB 2, the PDCP SN is 0 and it should have been PDCP SN 0 for the first arriving PDU on DRB 2. However, this does not happen as per the TC requirements at Step 3, Since t-Reordering timer is infinity for all DRBs, the received PDUs across all DRBs are buffered until PDU containing PDCP SN 0 arrives (which will not happen) and therefore *NO PDUs* will be delivered to TTCN by the SS Stack across all DRBs. - The requirements at Step 3 (7.1.1.3.2) and Steps 3, 8 and 12 (7.1.1.3.2b) suggest that it is allowed to lose 10% of the octets for each DRB. This would also lead to the situation outlined above that *NO PDUs* will be delivered to TTCN by the SS Stack across all DRBs if there are any missing PDCP SNs for a specific DRB. To mitigate this, it is recommended to introduce and set outOfOrderDelivery = true in the local end ASP NR_PDCP_DRB_Config_Parameters_Type so that SS PDCP will deliver all received PDUs to TTCN immediately |
| Summary of change | Modified the ASP NR_PDCP_DRB_Config_Parameters_Type to introduce and set the outOfOrderDelivery to TRUE as part of the PDCP Config in test cases 7.1.1.3.2 and 7.1.1.3.2b |
| TTCN module | NR_PDCP_TypeDefs.ttcn |
| MCC160 Comment |  |

Before Change:

type record NR_PDCP_DRB_Config_Parameters_Type { /* parameters corrsponding to NR ASN.1 PDCP-Config.drb; the following parameter are not included and may be added if needed: - integer DiscardTimer (timer value in milliseconds) - boolean StatusReportRequired - boolean OutOfOrderDelivery */ NR_PDCP_SN_Size_Type SN_SizeUL, /* PDCP-Config.drb.pdcp-SN-SizeUL */ NR_PDCP_SN_Size_Type SN_SizeDL, /* PDCP-Config.drb.pdcp-SN-SizeDL */ NR_PDCP_DRB_HeaderCompression_Type HeaderCompression, /* PDCP-Config.drb.headerCompression */ boolean IntegrityProtectionEnabled, /* PDCP-Config.drb.integrityProtection */ };

After Change:

type record NR_PDCP_DRB_Config_Parameters_Type { /* parameters corrsponding to NR ASN.1 PDCP-Config.drb; the following parameter are not included and may be added if needed: - integer DiscardTimer (timer value in milliseconds) - boolean StatusReportRequired - boolean OutOfOrderDelivery */ NR_PDCP_SN_Size_Type SN_SizeUL, /* PDCP-Config.drb.pdcp-SN-SizeUL */ NR_PDCP_SN_Size_Type SN_SizeDL, /* PDCP-Config.drb.pdcp-SN-SizeDL */ NR_PDCP_DRB_HeaderCompression_Type HeaderCompression, /* PDCP-Config.drb.headerCompression */ boolean IntegrityProtectionEnabled, /* PDCP-Config.drb.integrityProtection */ boolean OutOfOrderDelivery //WA# };