3GPP TSG-RAN WG5 IoT Adhoc Meeting #2	R5-167172

Singapore, 10 - 14 October 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | - | rev |  | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test Case: 6.2.11: E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-UEConTest |  |  |  |  |  |  | Date: |  | 10/3/2016 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Test Tolerances in Cat-M1 PRACH test cases for HD-FDD in normal coverage are not defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define Test Tolerances for Cat-M1 PRACH test cases for HD-FDD in normal coverage |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No Test Tolerances in test cases defined for evaluating Cat-M1 PRACH for HD-FDD in normal coverage |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 6.2.11 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521-2 CR xxxx |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---START OF CHANGE 1---

### 6.2.11	E-UTRAN HD-FDD - Contention based random access test for Cat-M1 UEs in normal coverage

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-

- Annex E needs to be updated.

- Some requirements in Test 1 and Test 2 are still within square brackets in TS36.133.

#### 6.2.11.1	Test purpose

The purpose of this test is to verify whether the behavior of the random access procedure of a Cat-M1 UE in Normal Coverage is according to the E-UTRAN HD-FDD contention based random access requirements specified in TS 36.133 [4] Clause 6.2.2, Clause 6.2.3 and Clause 7.1.2 in an AWGN channel model, whether the PRACH power settings and timing are within the specified limits and that the UE determines properly the enhanced coverage level based on the RSRP measurement and the configured criterion in RSRP-ThresholdsPrach as defined in TS 36.331 [5].

#### 6.2.11.2	Test applicability

This test applies to all types of E-UTRA HD-FDD UE release 13 and forward of UE category M1.

#### 6.2.11.3	Minimum conformance requirements

The random access procedure is used when establishing the layer 1 communication between the UE and E-UTRAN. The random access is specified in clause 6 of TS 36.213[8] and the control of the RACH transmission is specified in clause 5.1 of TS 36.321[11]. Contention based random access procedures can only be carried out on PCell and PSCell.

The CAT-M1 UE shall have capability to calculate PRACH transmission power according to the PRACH power formula as defined in TS 36.213 [8] clause 6.1 and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have the accuracy as defined in Table 6.3.5.1.1-1 of TS 36.101 [2]. The relative power applied to additional preambles shall have the accuracy as specified in Table 6.3.5.2.1-1 of TS 36.101 [2].

The CAT-M1 UE shall indicate a Random Access problem to upper layers if the maximum number of preambles transmission counter has been reached for the random access procedure on PCell or PSCell as specified in clause 5.1.4 in TS 36.321 [11].

The CAT-M1 UE shall stop preamble transmission if maximum number of preamble transmission counter has been reached for the random access procedure on an activated Scell as specified in clause 5.1.4 in TS 36.321 [11].

The CAT-M1 UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The Cat-M1 UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the back off time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The CAT-M1 UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the back off time expires if no Random Access Response is received within the RA Response window defined in clause 5.1.4 TS 36.321 [11].

The CAT-M1 UE shall re-transmit the msg3 upon the reception of a NACK on msg3.

The Cat-M1 UE shall send ACK if the Contention Resolution is successful.

The Cat-M1 UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the back off time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the Cat-M1 UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

The CAT-M1 UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the back off time expires if the Contention Resolution Timer expires.

In addition to the requirements defined in TS 36.133 [4] clause 6.2.1 and 6.2.2, the Cat-M1 UE shall also execute the random access procedure defined in clause 5.1 in TS 36.321 [11] using the PRACH configuration contained in PRACH-ConfigSIB in TS 36.331 [5]:

- 	Determine the enhanced coverage level based on the RSRP measurement and the configured criterion (RSRP-ThresholdsPrach in TS 36.331 [5]) as defined in section 5.1.1, TS 36.321 [11];

-	Select PRACH resources [5] configured for the corresponding enhanced coverage level as determined in the previous step;

- 	Transmits or re- transmits PRACH preamble using the selected PRACH resources and PRACH configuration.

The normative reference for this requirement is TS 36.133 [4] clause 6.2.2.1, 6.2.3, 7.1.2 and A.6.2.10.

The test parameters are given in Tables 6.2.11.5-1, 6.2.11.5-2 and 6.2.11.5-3.

#### 6.2.11.4	Test description

##### 6.2.11.4.1	Initial conditions

Test Environment: Normal, TL/VL, TL/VH, TH/VL, TH/VH; as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E Table E-1 and TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10 MHz as defined in TS 36.508 [7] clause 4.3.1.

1.    Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.18 using only main UE Tx/Rx antenna.

2.    Propagation conditions are set according to Annex B clause B.0.

3.    There is one E-UTRA HD-FDD carrier and one cell specified in the test. Cell 1 is the cell used for registration with   the power level set according to Annex C.0 and C.1 for this test.

6.2.11.4.2	Test proceduresThe test consists of a single cell. The UE has a downlink PDSCH allocated centered on the centre sub-carrier. The EUTRAN shall not explicitly signal a Random Access Preamble via dedicated signaling in the downlink to the Cat-M1 UE.

1.	Ensure the CAT-M1 UE is in State 2A-RF-CE according to TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to Tables 6.2.11.5-1, 6.2.11.5-2 and 6.2.11.5-3 as appropriate, where the CE level 0 shall be used in this test. Propagation conditions are set according to Annex B clause B.1.1.

3.	The CAT-M1 UE shall establish a connection setup with SS, the random access procedure within the connection setup is used in the test.

4.	Test 1: Correct behavior when receiving random access response

4.1.	In Test 1, the CAT-M1 UE shall send the preamble to the SS. In response to the first [4] preambles, the SS shall transmit a random access response not corresponding to the transmitted random access preamble.

4.2.	The CAT-M1 UE shall consider the random access response reception not successful then re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires if all received random access response contain random access preamble identifiers that do not match the transmitted random access preamble.

4.3.	The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after [5] preambles have been received by the SS.

4.4.	The CAT-M1 UE shall consider this random access response reception successful and transmit the msg3.

4.5.	Measure the power and timing of the first preamble and it shall not exceed the values specified in Tables 6.2.11.5-4 and 6.2.11.5-6. The power of the first preamble shall be -30 dBm. Measure the relative power and timing applied to additional preambles and it shall not exceed the values specified in Table 6.2.11.5-5. The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 6.2.11.5-6.

5.	Test 2: Correct behavior when not receiving random access response

5.1.	Repeat step 1-3.

5.2.	In Test 2, the CAT-M1 UE shall send the preamble to the SS. The SS shall not respond to the first [4] preambles.

5.3. 	The CAT-M1 UE shall consider the random access response reception not successful then re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires if no random access response is received within the RA Response window of 20 sub-frames.

5.4. 	The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after [5] preambles have been received by the SS.

5.5.   The CAT-M1 UE shall consider this random access response reception successful and transmit the msg3.

5.6.   Measure the power and timing of the first preamble and it shall not exceed the values specified Tables 6.2.11.5-4 and 6.2.11.5-6. The power of the first preamble shall be -30 dBm. Measure the relative power and timing applied to additional preambles and it shall not exceed the values specified in Table 6.2.11.5-5. The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 6.2.11.5-6.

6.	Test 3: Correct behavior when receiving a NACK on msg3

6.1.	Repeat step 1-3.

6.2. 	In Test 3, the CAT-M1 UE shall send the preamble to the SS. The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS.

6.3.	The CAT-M1 UE shall consider this random access response reception successful and transmit the msg3.

6.4.	The SS shall send NACK to all UE msg3 following a successful random access response.

6.5.	The CAT-M1 UE shall consider this contention resolution not successful then re-transmit the msg3 upon the reception of a NACK on msg3 until the maximum number of HARQ retransmission is reached.

7.	Test 4: Correct behavior when receiving an incorrect message over Temporary C-RNTI

7.1.	Repeat step 1-3.

7.2.	In Test 4, the CAT-M1 UE shall send the preamble to the SS. The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS.

7.3.	The CAT-M1 UE shall consider this random access response reception successful and transmit the msg3.

7.4. 	The SS shall send a message addressed to the Temporary C-RNTI with a UE contention resolution identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

7.5.	The CAT-M1 UE shall consider the contention resolution not successful then re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires.

8.    Test 5: Correct behavior when receiving a correct message over Temporary C-RNTI

8.1.	In Test 5, the SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS.

8.2.	The CAT-M1 UE shall consider this random access response reception successful and transmit the msg3.

8.3. 	The SS shall send a message addressed to the Temporary C-RNTI with a UE contention resolution identity included in the MAC control element matching the CCCH SDU transmitted in the msg3 uplink message.

8.4.	The CAT-M1 UE shall send ACK and consider the contention resolution successful and the random access procedure successfully completed.

9.	Test 6: Correct behavior when contention resolution timer expires

9.1.   Repeat step 1-3.

9.2.   In Test 6, the CAT-M1 UE shall send the preamble to the SS. The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS.

9.3.   The CAT-M1 UE shall consider this random access response reception successful and transmit the msg3.

9.4.   The SS shall send an ACK for msg3 but not send msg4 allowing the contention resolution timer to expire.

9.5.   The CAT-M1 UE shall consider the contention resolution not successful then re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires.

9.6.   The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS.

9.7.	The CAT-M1 UE shall consider this random access response reception successful and transmit the msg3.

10.		Test 7: PRACH Resource Selection

10.1.	Repeat step 1-3.

10.1.	In Test 7, the CAT-M1 UE shall send the preamble to the SS.

10.3.	The CAT-M1 UE shall determines the enhanced coverage level 0 based on the RSRP measurement and the configured criterion (RSRP-ThresholdsPrach in TS 36.331 [5]) as defined in section 5.1.1, TS 36,321 [11].

10.4.	The CAT-M1 UE shall selects PRACH resources configured for the corresponding enhanced coverage level 0 as determined in the previous step.

10.5.	The CAT-M1 UE shall transmit or re- transmits PRACH preamble using the selected PRACH resources, PRACH configuration and calculated PRACH transmission power for the corresponding coverage level 0.

##### 6.2.11.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions:

Table 6.2.11.4.3-1: Common Exception messages for E-UTRAN HD-FDD - Contention Based Random Access test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.6-1 Table H.2.6-2 Table H.2.6-3 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-1 |

Table 6.2.11.4.3-2: UplinkPowerControlCommon-DEFAULT: Additional E-UTRAN HD-FDD – Contention Based Random Access test requirement

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-25 UplinkPowerControlCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalPUSCH | -101 (-101 dBm) |  |  |
| } |  |  |  |

#### 6.2.11.5	Test requirement

Tables 6.2.11.5-1, 6.2.11.5-2 and 6.2.11.5-3 define the primary level settings for E-UTRAN HD-FDD - contention based random access test. Table 6.2.11.5-6 defines the uplink timing error limit including test tolerances.

For the tests specified in section 6.2.11.4.2, the power of the first preamble shall be -30 dBm to within the accuracy specified in Table 6.2.11.5-4. The relative power for preamble ramping step shall be 2 dB to within the accuracy specified in Table 6.2.11.5-5. The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 6.2.11.5-6.

Table 6.2.11.5-1: General test parameters for HD-FDD contention based random access test

| Parameter | Unit | Value | Comments |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Pattern Note 1 |  | OP.21 FDD | As defined in TS 36.133 [4] clause A.3.2.1.21. |  |
| PDSCH parameters Note 2 |  | R.10 HD-FDD | As defined in TS 36.133 [4] clause A.3.1.4.2 |  |
| MPDCCH parameters Note 2 |  | R.6 HD-FDD | As defined in TS 36.133 [4] clause A.3.1.3.2 |  |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.3 HD-FDD | As defined in TS 36.133 [4] clause A.3.1.2.3 |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| MPDCCH_RA | dB |  |  |  |
| MPDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RA Note 1 | dB |  |  |  |
| OCNG_RB Note 1 | dB |  |  |  |
|  | #### dBm/15 KHz | #### -98 |  |  |
|  | ##### dB | 3 |  |  |
| ##### Note 3 | ###### dB | 3 |  |  |
| ###### RSRP Note 3 | ###### dBm/15 KHz | -95 |  |  |
| ###### Io Note 3 | ###### dBm/9 MHz | -65.5 |  |  |
| Propagation Condition | - | AWGN |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The PDSCH and MPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Es/Iot, RSRP and Io level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |

Table 6.2.11.5-2: RACH-Configuration parameters for HD-FDD contention based random access test

| Field | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
| Parameters not per CE Levels |  |  |  |  |  |
| powerRampingStep | dB2 |  |  |  |  |
| preambleInitialReceivedTargetPower | dBm-120 |  |  |  |  |
| preambleTransMax | n6 |  |  |  |  |
| maxHARQ-Msg3Tx | 4 |  |  |  |  |
| rar-HoppingConfig | Off |  |  |  |  |
| Parameters per CE Levels |  |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 | Level 3 |  |
| ra-ResponseWindowSize (per CE) | sf20 | sf80 | sf180 | sf320 |  |
| mac-ContentionResolutionTimer (per CE) | sf80 | sf120 | sf200 | sf480 |  |
| PreambleMappingInfo {firstPreamble, lastPreamble} | {0, 9} | {10,19} | {20,29} | {30,39} |  |
| Note: For further information see Clause 6.3.2 in TS 36.331 [5]. |  |  |  |  |  |

Table 6.2.11.5-3: PRACH-Configuration parameters for HD-FDD contention based random access test

| Field | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
| Parameters not per CE Levels |  |  |  |  |  |
| rsrp-ThresholdsPrach | {24, 31, 38} |  |  |  | Corresponding to {-116, -109, -102} dBm as defined in TS36.133 [4] clause 9.1.21.5 |
| mpdcch-startSF-CSS-RA | v1 |  |  |  |  |
| referenceSignalPower | -5 dBm/15 KHz |  |  |  | As defined in clause 6.3.2 in TS 36.331 [5]. |
| maxHARQ-Msg3Tx | 4 |  |  |  | As defined in table 5.7.1-2 in TS 36.211 [9] |
| Backoff Parameter Index | 2 |  |  |  | As defined in Table 7.2-1 in TS 36.321 [11] |
| Configured UE transmitted power () | 23 dBm |  |  |  | As defined in clause 6.2.5 in TS 36.101 [2] |
| Parameters per PRACH CE Levels |  |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 | Level 3 |  |
| prach-ConfigIndex | 4 | 4 | 4 | 4 | As defined in Table 5.7.1-2 in TS 36.211 [9] |
| prach-FreqOffset | 0 | 0 | 0 | 0 |  |
| prach-StartingSubframe | sf2 | sf4 | Sf8 | sf16 |  |
| maxNumPreambleAttempt | n3 | n5 | n7 | n10 |  |
| numRepetitionPerPreambleAttempt | n1 | n4 | n16 | n64 |  |
| mpdcch-NarrowbandsToMonitor | 2 | 2 | 2 | 2 |  |
| mpdcch-NumRepetition-RA | r8 | r8 | r128 | r128 |  |
| prach-HoppingConfig | Off | Off | Off | Off |  |
| Note 1: See Clause 6.3.2 in TS 36.331 [5] for further information on the parameters in this table. |  |  |  |  |  |

Table 6.2.11.5-4: Absolute power tolerance for E-UTRAN HD-FDD - Contention Based Random Access test

| Tolerance |  |
| --- | --- |
| Normal Conditions | Extreme Conditions |
| ± 10.5 dB | ± 13.5 dB |
| Note: TS 36.133 [4] clause A.6.2.11.2 and TS 36.101 [2] clause 6.3.5.1.1 |  |

Table 6.2.11.5-5: Relative power tolerance for E-UTRAN HD-FDD - Contention Based Random Access test

| Power Step Size (Up or Down) | PRACH |  |
| --- | --- | --- |
|  | Normal Conditions | Extreme Conditions |
| ΔP [dB] | [dB] | [dB] |
| 2 ≤ ΔP < 3 | ± 3.7 | ± 5.7 |
| Note 1: F.3.2 and TS 36.133 [4] clause A.6.2.11.2 and TS 36.101 [2] clause 6.3.5.2.1 Note 2: For extreme conditions an additional ± 2.0 dB relaxation is allowed for PRACH allocations |  |  |

Table 6.2.11.5-6: Test requirements for Te Timing Error Limit for E-UTRAN HD-FDD – Contention Based Random Access test

| Downlink Bandwidth (MHz) | Te |
| --- | --- |
| 10 | 30*TS |
| Note 1: F.3.2 and TS 36.133 [4] clauses A.6.2.11.2 and 7.1.2, where Te of 1.4MHz is used for CEMode A regardless of the downlink carrier bandwidth Note 2: TS is the basic timing unit defined in TS 36.211 [9] |  |

---END OF CHANGE 1---