3GPP TSG-RAN5 Meeting #100	R5-234150

# Toulouse, France, 21th – 25th Aug 2022

Source:	NTTDOCOMO, INC

Title:	Proposal for new IMS test cases related to UAC

Agenda item:	6.4.7

Document for:	Endorsement

Background

The purpose of this document is to discuss about adding following TC related to UAC (unified access control). According to TS.38.523-1 [1], there is no test case related to Access category 7 with access identitiy 0 on the device which does not support RRC_INACTIVE in terms of checking RRC establishment cause “mo-Data”. This condition has a possibility of not being able to guarantee the UE behavior under this combination.

Reference of test cases related to UAC from TS.38.523-1 (ver17.3)[1]

-Test cases having same combination are hilighted with yellow marker.

==

| Test case number | Title | Rel | Access identity | Access Category |
| --- | --- | --- | --- | --- |
| 11.3.1 | UAC / Access Identity 0 / 0% access probability / MTSI MO speech call / SMSoIP | Rel-15 | 0 | 4,6 |
| 11.3.1a | UAC / Access Identity 0 / 0% access probability / Uplink User data transfer / RRC_INACTIVE | Rel-15 | 0 | 4,7 |
| 11.3.2 | UAC / Access Identity 0 / 0% access probability / Paging for MT Access/Emergency Call | Rel-15 | 0 | 1,2,3,4,5,6,7 |
| 11.3.3 | UAC / Access Identity 0 / AC8 / RRC_INACTIVE / RNAUpdate/RRC Resume | Rel-15 | 0 | 8 |
| 11.3.4 | UAC / Access Identity 0 / Registration procedure for mobility and periodic registration update / BarringPerPLMN/Implicit AC Barring List | Rel-15 | 0 | 3 |
| 11.3.5 | UAC / Access Identity 1 / New cell not in the country of its HPLMN/EHPLMN 0% access probability/MPS indicator / HPLMN/0%/100% accessibility AC5/MMTEL-Video call | Rel-15 | 1 | 7 |
| 11.3.6 | UAC / Access Identity 2 / New cell not in the country of its HPLMN/EHPLMN 0% access probability/MCS indicator / HPLMN/0%/100% accessibility AC7/RRC_INACTIVE | Rel-15 | 2 | 7 |
| 11.3.6a | UAC / Access Identity 2 / MCS indicator / SNPN/0%/100% accessibility AC7 / RRC_INACTIVE | Rel-16 | 2 | 7 |
| 11.3.7 | UAC / Access Identity 11..15 / High Priority Access / HPLMN/0% accessibility AC2/Emergency call | Rel-15 | 11 - 15 | 2 |
| 11.3.8 | UAC / Access Identity 0 / NR RRC_IDLE / Cell re-selection while T390 is running | Rel-15 | 0 | 3 |
| 11.3.9 | UAC / Access Identity 0 / ODAC / PLMN / RPLMN / not EPLMN | Rel-15 | 0 | 33 |
| 11.3.9a | UAC / Access Identity 0 / ODAC / SNPN / RSNPN / new SNPN | Rel-16 | 0 | 33 |
| 11.3.10 | UAC / Access Identity 0 / AC9 / 0% access probability / SIP Re-registration | Rel-16 | 0 | 9 |

==

Discussion

The purpose is to discuss about adding following TC related to Access identity 0 with AC7.

- TC: UAC / Access Identity 0 / AC7 / 0% access probability / Uplink user data transfer

Proposal

Proposal : RAN5 needs to add TC related to Access category 7 in TS 38.523-1. This TC is applied to UEs supporting 5G Core from Rel.15.

In RAN5#100, this test case are proposed as the following CR numbers.

TC: R5-234151

Test applicability: R5-234152

References

[1]	3GPP TS 38.523-1: "3rd Generation Partnership Project;Technical Specification Group Radio Access Network;

5GS;User Equipment (UE) conformance specification;Part 1: Protocol(Release 17)".