3GPP TSG-RAN5 Meeting #100	R5-234627

# Toulouse, France, 21st August – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | n/a | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NTN AMPR test configurations and requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-12 |
| Category: | n/a |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR NTN AMPR tests are missing test configurations and test requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the test point analysis in R5-234630, Add NR NTN AMPR test configurations, test procedures, and test requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR NTN AMPR tests are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5-234630 has test point analysis used in this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of Change 1 ***

### 6.2.3	UE additional maximum output power reduction

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support NR satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Annex F MU/TT is TBD

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated

with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this

specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR

satellite band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of

additionalSpectrumEmission in the relevant RRC information elements [8].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2.3.2	Test applicability

TBD

6.2.3.3	Minimum conformance requirements

6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR satellite band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-2.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2.2-1 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_24 | 6.5.3.3.13 in 3GPP TS 38.101-1 [5] | n256 | 5, 10, 15, 20 | Table 6.2.3.15-1 in 3GPP TS 38.101-1 [5] | Clause 6.2.3.7 in 3GPP TS 38.101-1 [5]2 |
| NS_02N | 6.5.3.3.2 | n255 | 5, 10, 15, 20 |  | N/A |
| NS_100 | 6.5.2.4.2 in 3GPP TS 38.101-1 [5] | n2561 |  |  | Table 6.2.3.1-2 in 3GPP TS 38.101-1 [5] |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [8] absent is default for all NTN satellite bands.]

Table 6.2.3.3.1-2: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

Table 6.2.3.3.1-3: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |

The normative reference for this requirement is TS 38.101-5 [11] clause 6.2.3.1.

6.2.3.3.2	A-MPR for NS_24

Table 6.2.3.3.2-1: A-MPR regions for NS_24

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  | Region B |  | Region C |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Rbend*12*SCS MHz | LCRB*12*SCS MHz | Rbend*12*SCS MHz | LCRB*12*SCS MHz | Rbend*12*SCS MHz | LCRB*12*SCS MHz |
| 5MHz | Fc=1992.5 |  | >3.24 |  |  |  |  |
| 5MHz | Fc=1997.5 |  | >3.24 |  |  |  |  |
| 5MHz | Fc=2002.5 |  | >2.16 | >3.78 | ≤1.98 | ≤3.6 | ≤1.98 |
| 10MHz | Fc=1985 | >5.4 |  |  |  |  |  |
| 10MHz | Fc=1995 |  | >4.5 | >7.56 | ≤4.32 | ≤7.38 | ≤4.32 |
| 10MHz | Fc=2000 | >6.84 |  | <2.88 |  | ≥3.06 ≤6.66 | >1.44 |
| 15MHz | Fc=1987.5 |  | >7.02 | >11.52 | ≤6.84 | ≤11.34 | ≤6.84 |
| 15MHz | Fc=1997.5 | >9.36 |  | <3.6 |  | ≥3.78 ≤9.18 | >1.44 |
| 20MHz | Fc=1990 | >13.5 |  | <4.5 |  | ≥4.68 ≤13.32 | >2.16 |
| 20MHz | Fc=1995 | >12.6 |  | <5.4 |  | ≥5.58 ≤12.42 | >1.44 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.3.2-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |

Table 6.2.3.3.2-2: A-MPR for NS_24

| Modulation/Waveform |  | Outer (dB) | Inner (dB) |
| --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | ≤ 1.5 | N/A |
|  | QPSK | ≤ 2 |  |
|  | 16 QAM | ≤ 3 |  |
|  | 64 QAM | ≤ 3.5 |  |
|  | 256 QAM | ≤ 5.5 |  |
| CP-OFDM | QPSK | ≤ 4 |  |
|  | 16 QAM | ≤ 4 |  |
|  | 64 QAM | ≤ 4.5 |  |
|  | 256 QAM | ≤ 7.5 |  |

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Tables 6.2.3.4.1-1 to 6.2.3.4.1-30. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.3.4.1-1: Test Configuration table for NS_100

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_100 |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |
|  |  |  |  | N/A for A-MPR test cases | DFT-s OFDM |  |  |  |
| 1 | Low | Default | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left |
| 2 | High | Default | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right |
| 3 | Default | Default | Default |  |  | PI/2 BPSK |  | Outer_Full |
| 4 | Low | Default | Default |  |  | QPSK |  | Edge_1RB_Left |
| 5 | High | Default | Default |  |  | QPSK |  | Edge_1RB_Right |
| 6 | Default | Default | Default |  |  | QPSK |  | Outer_Full |
| 7 | Low | Default | Default |  |  | 16 QAM |  | Edge_1RB_Left |
| 8 | High | Default | Default |  |  | 16 QAM |  | Edge_1RB_Right |
| 9 | Default | Default | Default |  |  | 16 QAM |  | Outer_Full |
| 10 | Low | Default | Default |  |  | 64 QAM |  | Edge_1RB_Left |
| 11 | High | Default | Default |  |  | 64 QAM |  | Edge_1RB_Right |
| 12 | Default | Default | Default |  |  | 64 QAM |  | Outer_Full |
| 13 | Low | Default | Default |  |  | 256 QAM |  | Edge_1RB_Left |
| 14 | High | Default | Default |  |  | 256 QAM |  | Edge_1RB_Right |
| 15 | Default | Default | Default |  |  | 256 QAM |  | Outer_Full |
| 16 | Low | Default | Default |  | CP-s OFDM | QPSK |  | Edge_1RB_Left |
| 17 | High | Default | Default |  |  | QPSK |  | Edge_1RB_Right |
| 18 | Default | Default | Default |  |  | QPSK |  | Outer_Full |
| 19 | Low | Default | Default |  |  | 16 QAM |  | Edge_1RB_Left |
| 20 | High | Default | Default |  |  | 16 QAM |  | Edge_1RB_Right |
| 21 | Default | Default | Default |  |  | 16 QAM |  | Outer_Full |
| 22 | Low | Default | Default |  |  | 64 QAM |  | Edge_1RB_Left |
| 23 | High | Default | Default |  |  | 64 QAM |  | Edge_1RB_Right |
| 24 | Default | Default | Default |  |  | 64 QAM |  | Outer_Full |
| 25 | Low | Default | Default |  |  | 256 QAM |  | Edge_1RB_Left |
| 26 | High | Default | Default |  |  | 256 QAM |  | Edge_1RB_Right |
| 27 | Default | Default | Default |  |  | 256 QAM |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-2: Test Configuration table for NS_24

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to uplink carrier centre frequency (Fc) in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to test parameters (5, 10, 15, 20 MHz) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |  |  |
| A-MPR test parameters for NS_24 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | Region A | Region B | Region C |
| 1 | 1992.5 | 5 | Default | N/A for A-MPR testing | DFT-s OFDM | PI/2 BPSK | Outer_Full | N/A | N/A |
| 2 | 1992.5 | 5 | Default |  |  | QPSK | Outer_Full | N/A | N/A |
| 3 | 1992.5 | 5 | Default |  |  | 16 QAM | Outer_Full | N/A | N/A |
| 4 | 1992.5 | 5 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 5 | 1992.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 6 | 1997.5 | 5 | Default |  |  | PI/2 BPSK | Outer_Full | N/A | N/A |
| 7 | 1997.5 | 5 | Default |  |  | QPSK | Outer_Full | N/A | N/A |
| 8 | 1997.5 | 5 | Default |  |  | 16 QAM | Outer_Full | N/A | N/A |
| 9 | 1997.5 | 5 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 10 | 1997.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 11-13 | 2002.5 | 5 | Default |  |  | PI/2 BPSK | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 14-16 | 2002.5 | 5 | Default |  |  | QPSK | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 17-19 | 2002.5 | 5 | Default |  |  | 16 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 20-22 | 2002.5 | 5 | Default |  |  | 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 23-25 | 2002.5 | 5 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 26 | 1985 | 10 | Default |  |  | PI/2 BPSK | Edge_1RB_Right | N/A | N/A |
| 27 | 1985 | 10 | Default |  |  | QPSK | Edge_1RB_Right | N/A | N/A |
| 28 | 1985 | 10 | Default |  |  | 16 QAM | Edge_1RB_Right | N/A | N/A |
| 29 | 1985 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right | N/A | N/A |
| 30 | 1985 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | N/A | N/A |
| 31 | 1985 | 10 | Default |  |  | PI/2 BPSK | Outer_Full | N/A | N/A |
| 32 | 1985 | 10 | Default |  |  | QPSK | Outer_Full | N/A | N/A |
| 33 | 1985 | 10 | Default |  |  | 16 QAM | Outer_Full | N/A | N/A |
| 34 | 1985 | 10 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 35 | 1985 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 36-38 | 1995 | 10 | Default |  |  | PI/2 BPSK | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 39-41 | 1995 | 10 | Default |  |  | QPSK | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 42-44 | 1995 | 10 | Default |  |  | 16 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 45-47 | 1995 | 10 | Default |  |  | 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 48-50 | 1995 | 10 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 51-53 | 2000 | 10 | Default |  |  | PI/2 BPSK | Edge_1RB_Right | Edge_1RB_Left | 36@0 |
| 54-56 | 2000 | 10 | Default |  |  | QPSK | Edge_1RB_Right | Edge_1RB_Left | 36@0 |
| 57-59 | 2000 | 10 | Default |  |  | 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 36@0 |
| 60-62 | 2000 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 36@0 |
| 63-65 | 2000 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 36@0 |
| 66 | 2000 | 10 | Default |  |  | PI/2 BPSK | Outer_Full | N/A | N/A |
| 67 | 2000 | 10 | Default |  |  | QPSK | Outer_Full | N/A | N/A |
| 68 | 2000 | 10 | Default |  |  | 16 QAM | Outer_Full | N/A | N/A |
| 69 | 2000 | 10 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 70 | 2000 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 71-73 | 1987.5 | 15 | Default |  |  | PI/2 BPSK | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 74-76 | 1987.5 | 15 | Default |  |  | QPSK | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 77-79 | 1987.5 | 15 | Default |  |  | 16 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 80-82 | 1987.5 | 15 | Default |  |  | 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 83-85 | 1987.5 | 15 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 86-88 | 1997.5 | 15 | Default |  |  | PI/2 BPSK | Edge_1RB_Right | Edge_1RB_Left | 50@0 |
| 89-91 | 1997.5 | 15 | Default |  |  | QPSK | Edge_1RB_Right | Edge_1RB_Left | 50@0 |
| 92-94 | 1997.5 | 15 | Default |  |  | 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 50@0 |
| 95-97 | 1997.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 50@0 |
| 98-100 | 1997.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 50@0 |
| 101 | 1997.5 | 15 | Default |  |  | PI/2 BPSK | Outer_Full | N/A | N/A |
| 102 | 1997.5 | 15 | Default |  |  | QPSK | Outer_Full | N/A | N/A |
| 103 | 1997.5 | 15 | Default |  |  | 16 QAM | Outer_Full | N/A | N/A |
| 104 | 1997.5 | 15 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 105 | 1997.5 | 15 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 106-108 | 1990 | 20 | Default |  |  | PI/2 BPSK | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 109-111 | 1990 | 20 | Default |  |  | QPSK | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 112-114 | 1990 | 20 | Default |  |  | 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 115-117 | 1990 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 118-120 | 1990 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 121 | 1990 | 20 | Default |  |  | PI/2 BPSK | Outer_Full | N/A | N/A |
| 122 | 1990 | 20 | Default |  |  | QPSK | Outer_Full | N/A | N/A |
| 123 | 1990 | 20 | Default |  |  | 16 QAM | Outer_Full | N/A | N/A |
| 124 | 1990 | 20 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 125 | 1990 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 126-128 | 1995 | 20 | Default |  |  | PI/2 BPSK | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 129-131 | 1995 | 20 | Default |  |  | QPSK | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 132-134 | 1995 | 20 | Default |  |  | 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 135-137 | 1995 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 138-140 | 1995 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 141 | 1995 | 20 | Default |  |  | PI/2 BPSK | Outer_Full | N/A | N/A |
| 142 | 1995 | 20 | Default |  |  | QPSK | Outer_Full | N/A | N/A |
| 143 | 1995 | 20 | Default |  |  | 16 QAM | Outer_Full | N/A | N/A |
| 144 | 1995 | 20 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 145 | 1995 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 146-147 | 1992.5 | 5 | Default |  | CP-OFDM | QPSK 16 QAM | Outer_Full | N/A | N/A |
| 148 | 1992.5 | 5 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 149 | 1992.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 150-151 | 1997.5 | 5 | Default |  |  | QPSK 16 QAM | Outer_Full | N/A | N/A |
| 152 | 1997.5 | 5 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 153 | 1997.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 154-159 | 2002.5 | 5 | Default | N/A for A-MPR testing |  | QPSK 16 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 160-162 | 2002.5 | 5 | Default |  |  | 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 163-165 | 2002.5 | 5 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 166-167 | 1985 | 10 | Default |  |  | QPSK 16 QAM | Edge_1RB_Right | N/A | N/A |
| 168 | 1985 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right | N/A | N/A |
| 169 | 1985 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | N/A | N/A |
| 170-171 | 1985 | 10 | Default |  |  | QPSK 16 QAM | Outer_Full | N/A | N/A |
| 172 | 1985 | 10 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 173 | 1985 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 174-179 | 1995 | 10 | Default |  |  | QPSK 16 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 180-182 | 1995 | 10 | Default |  |  | 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 183-185 | 1995 | 10 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 186-191 | 2000 | 10 | Default |  |  | QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 37@0 |
| 192-194 | 2000 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 37@0 |
| 195-197 | 2000 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 37@0 |
| 198-199 | 2000 | 10 | Default |  |  | QPSK 16 QAM | Outer_Full | N/A | N/A |
| 200 | 2000 | 10 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 201 | 2000 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 202-207 | 1987.5 | 15 | Default |  |  | QPSK 16 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 208-210 | 1987.5 | 15 | Default |  |  | 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 211-213 | 1987.5 | 15 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 214-219 | 1997.5 | 15 | Default |  |  | QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 51@0 |
| 220-222 | 1997.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 51@0 |
| 223-225 | 1997.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 51@0 |
| 226-227 | 1997.5 | 15 | Default |  |  | QPSK 16 QAM | Outer_Full | N/A | N/A |
| 228 | 1997.5 | 15 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 229 | 1997.5 | 15 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 230-235 | 1990 | 20 | Default |  |  | QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 236-238 | 1990 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 239-241 | 1990 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 242-243 | 1990 | 20 | Default |  |  | QPSK 16 QAM | Outer_Full | N/A | N/A |
| 244 | 1990 | 20 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 245 | 1990 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 246-251 | 1995 | 20 | Default |  |  | QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 252-254 | 1995 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 255-257 | 1995 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 258-259 | 1995 | 20 | Default |  |  | QPSK 16 QAM | Outer_Full | N/A | N/A |
| 260 | 1995 | 20 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 261 | 1995 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 262 | 1992.5 | 5 | Default |  |  | QPSK | Edge_1RB_Right | N/A | N/A |
| 263 | 1997.5 | 5 | Default |  |  | 16 QAM | Edge_1RB_Right | N/A | N/A |
| 264 | 1985 | 10 | Default |  |  | QPSK | Edge_1RB_Left | N/A | N/A |

NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: In test IDs with multiple modulations, each UL Modulation shall be tested separately against Range A, B, and C.

Editor’s note: The following lines belong at the end of subclause 6.2.3.4.1. As new tables are added to this section, these lines should always follow the tables

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0.

4.	The UL Reference Measurement channels are set according to the applicable table from Table 6.2.3.4.1-1 to Table 6.2.3.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.3.4.3.

6.2.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2.3.4.1-1 to Table 6.2.3.4.1-2. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms).

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2.3.4.1-1 to Table 6.2.3.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2.3.4.3	Message contents

6.2.3.4.3.1	Message contents exceptions for network signalling value "NS_100"

1.	Information element additionalSpectrumEmission is set to NS_100. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_100"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_100) |  |  |

6.2.3.4.3.2	Message contents exceptions for network signalling value "NS_24"

1.	Information element additionalSpectrumEmission is set to NS_24. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_24"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_24) |  |  |

6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable Table from table 6.2.3.5-1 to Table 6.2.3.5-2. The allowed A-MPR values specified in Table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.1.3-1 apply.

Table 6.2.3.5-1: Test Tolerance (UE additional maximum output power reduction)

FFS

Table 6.2.3.5-2: UE Power Class 3 test requirements (NS_100) for n256

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10, 11 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 12 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13, 14 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 16, 17 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 24 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 25, 26 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-3: UE Power Class 3 test requirements (NS_24) for n256

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 6, 11-13, 26, 31, 36-38, 51-53, 66, 71-73, 86-88, 101, 106-108, 121, 126-128, 141 |  | 23 |  | N/A |  | 1.5 |  | 0 |  | 21.5 |  | 2 |  | 2 |  | 25+TT |  | 19.5-TT |  |
| 2, 7, 14-16, 27, 32, 39-41, 54-56, 67, 74-76, 89-91, 102, 109-111, 122, 129-131, 142 |  | 23 |  | N/A |  | 2.0 |  | 0 |  | 21 |  | 2 |  | 2 |  | 25+TT |  | 19-TT |  |
| 3, 8, 17-19, 28, 33, 42-44, 57-59, 68, 77-79, 92-94, 103, 112-114, 123, 132-134, 143 |  | 23 |  | N/A |  | 3.0 |  | 0 |  | 20 |  | 2.5 |  | 2 |  | 25+TT |  | 17.5-TT |  |
| 4, 9, 20-22, 29, 34, 45-47, 60-62, 69, 80-82, 95-97, 104, 115-117, 124, 135-137, 144 |  | 23 |  | N/A |  | 3.5 |  | 0 |  | 19.5 |  | 3.5 |  | 2 |  | 25+TT |  | 16-TT |  |
| 5, 10, 23-25, 30, 35, 48-50, 63-65, 70, 83-85, 98-100, 105, 118-120, 125, 138-140, 145 |  | 23 |  | N/A |  | 5.5 |  | 0 |  | 17.5 |  | 5 |  | 2 |  | 25+TT |  | 12.5-TT |  |
| 146-147, 150-151, 154-159, 166-167, 170-171, 174-179, 186-191, 198-199, 202-207, 214-219, 226-227, 230-235, 242-243, 246-251, 258-259, 262-264 |  | 23 |  | N/A |  | 4 |  | 0 |  | 19 |  | 3.5 |  | 2 |  | 25+TT |  | 15.5-TT |  |
| 148, 152, 160-162, 168, 172, 180-182, 192-194, 200, 208-210, 220-222, 228, 236-238, 244, 252-254, 260 |  | 23 |  | N/A |  | 4.5 |  | 0 |  | 18.5 |  | 4 |  | 2 |  | 25+TT |  | 14.5-TT |  |
| 149, 153, 163-165, 169, 173, 183-185, 195-197, 201, 211-213, 223-225, 229, 239-241, 245, 255-257, 261 |  | 23 |  | N/A |  | 7.5 |  | 0 |  | 15.5 |  | 5 |  | 2 |  | 25+TT |  | 10.5-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-1. |  |  |  |  |  |  |  |  |  |

*** End of Change 1 ***