3GPP TSG-RAN5 Meeting #100	R5-234693

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2880 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test procedure for establishing unicast mode ProSe Direct communication / Initiating UE side |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test procedure for establishing unicast mode ProSe Direct communication / Initiating UE side is missing from the current spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test procedure for establishing unicast mode ProSe Direct communication / Initiating UE side. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases of NR sidelink U2N relay will be not implementable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508 CR 2882, 2883 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.9.X	Test procedure for establishing unicast mode ProSe Direct communication / Initiating UE side

4.9.X.1	Scope

The purpose of this procedure is for establishing unicast mode ProSe Direct communication / Initiating UE side.

4.9.X.2	Procedure description

4.9.X.2.1	Initial conditions

The UE is in state 1N-B, 3N-B or 4-A.

4.9.X.2.2	Procedure sequence

Table 4.9.X.2.2-1: Procedure for establishing unicast mode ProSe Direct communication / Initiating UE side

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| - | EXCEPTION: Steps 1a1 to 1b2 describe behaviour which depends on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value. | - | - |
| 1a1 | IF the UE is in state 1N-B or 4-A, the UE is configured by upper layer to establish unicast mode link. NOTE: This can be done by sending AT COMMAND +CCUTLE to close test loop function. | - | - |
| 1b1 | ELSE IF the UE is in state 3N-B, the SS transmits a CLOSE UE TEST LOOP message. | <-- | TC: CLOSE UE TEST LOOP |
| 1b2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | TC: CLOSE UE TEST LOOP COMPLETE |
| 2 | The UE sends a PROSE DIRECT LINK ESTABLISHMENT REQUEST message. | --> | PC5-S: PROSE DIRECT LINK ESTABLISHMENT REQUEST |
| 3 | The NR-SS-UE sends a PROSE DIRECT LINK SECURITY MODE COMMAND message. | <-- | PC5-S: PROSE DIRECT LINK SECURITY MODE COMMAND |
| 4 | The UE sends a PROSE DIRECT LINK SECURITY MODE COMPLETE message. | --> | PC5-S: PROSE DIRECT LINK SECURITY MODE COMPLETE |
| 5 | The NR-SS-UE sends a DIRECT LINK ESTABLISHMENT ACCEPT message. | <-- | PC5-S: PROSE DIRECT LINK ESTABLISHMENT ACCEPT |
| 6 | The UE sends an RRCReconfigurationSidelink message to establish a unicast mode SL-DRB. NOTE: Unless specifically specified in the test case prose, the UE shall select the DRB parameters as specified in the pre-configured parameters. | --> | PC5-RRC: RRCReconfigurationSidelink |
| 7 | The NR-SS-UE sends an RRCReconfigurationCompleteSidelink message. | <-- | PC5-RRC: RRCReconfigurationCompleteSidelink |
| - | EXCEPTION: Steps 8a1 to 8b2 describe behaviour which depends on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value. | - | - |
| 8a1 | IF the UE is in state 1N-B or 4-A, the SS sends AT COMMAND +CCUTLE to open test loop function | - | - |
| 8b1 | ELSE IF the UE is in state 3N-B, the SS transmits an OPEN UE TEST LOOP message. | <-- | TC: OPEN UE TEST LOOP |
| 8b2 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | TC: OPEN UE TEST LOOP COMPLETE |

4.9.X.3	Specific message contents

All specific message contents shall be according subclause 4.6, 4.7 and 4.7B with the exceptions below.

Table 4.9.X.3-1: +CCUTLE (Table 4.9.X.2.2-1, Step 1)

Derivation Path: Table 4.7B-1 with condition Close and Transmit

Table 4.9.X.3-2: CLOSE UE TEST LOOP (Table 4.9.X.2.2-1, Step 1b1)

| Derivation Path: 36.508 [2] Table 4.7A-3 with condition UE TEST LOOP MODE E(V2X Transmission) |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE test loop mode E LB setup |  |  |  |  |
| Communication Transmit or Receive |  | 0 0 0 0 0 0 0 1 | ‘01’ indicates V2X UE triggered to transmit NR sidelink communication with single spatial layer. |  |

Table 4.9.X.3-3: PROSE DIRECT LINK ESTABLISHMENT REQUEST (Table 4.9.X.2.2-1, Step 2)

Derivation Path: Table 4.7.7-1 with condition Tx

Table 4.9.X.3-4: PROSE DIRECT LINK SECURITY MODE COMMAND (Table 4.9.X.2.2-1, Step 3)

Derivation Path: Table 4.7.7-13 with condition Rx

Table 4.9.X.3-5: PROSE DIRECT LINK SECURITY MODE COMPLETE (Table 4.9.X.2.2-1, Step 4)

Derivation Path: Table 4.7.7-14 with condition Tx

Table 4.9.X.3-6: PROSE DIRECT LINK ESTABLISHMENT ACCEPT (Table 4.9.X.2.2-1, Step 5)

Derivation Path: Table 4.7.7-2 with condition Rx

Table 4.9.X.3-7: RRCReconfigurationSidelink (Table 4.9.X.2.2-1, Step 6)

| Derivation Path: Table 4.6.1A-3 with condition TX and SL_DRB |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |  |
| slrb-ConfigToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Config-r16 { |  | 1 entry |  |  |
| SLRB-Config-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-SDAP-ConfigPC5-r16 SEQUENCE { |  |  |  |  |
| sl-MappedQoS-FlowsToAddList-r16 SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16 { |  | 1 entry |  |  |
| SL-PQFI-r16[1] |  | (1..63) | entry 1 |  |
| } |  |  |  |  |
| sl-MappedQoS-FlowsToReleaseList-r16 |  | Not present |  |  |
| sl-SDAP-Header-r16 |  | present |  |  |
| } |  |  |  |  |
| sl-PDCP-ConfigPC5-r16 SEQUENCE { |  |  |  |  |
| sl-PDCP-SN-Size-r16 |  | len12bits |  |  |
| sl-OutOfOrderDelivery-r16 |  | Not present |  |  |
| } |  |  |  |  |
| sl-RLC-ConfigPC5-r16 CHOICE { |  |  |  |  |
| sl-AM-RLC-r16 SEQUENCE { |  |  |  |  |
| sl-SN-FieldLengthAM-r16 |  | size12 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-MAC-LogicalChannelConfigPC5-r16 SEQUENCE { |  |  |  |  |
| sl-LogicalChannelIdentity-r16 |  | (4..32) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.X.3-8: RRCReconfigurationCompleteSidelink (Table 4.9.X.2.2-1, Step 7)

Derivation Path: Table 4.6.1A-4 with condition RX

Table 4.9.X.3-9: +CCUTLE (Table 4.9.X.2.2-1, Step 8a1)

Derivation Path: Table 4.7B-1 with condition Open