3GPP TSG-RAN WG5 Meeting #100	R5-235127

# Toulouse, France, August 21 – 25, 2023

Agenda item:	5.3.5.11

Source: 	ZTE Corporation

Title:	Discussion on the notation modification for SUL CA configurations

Document for:	Discussion and endorsement

# 1. Introduction

In RAN4#106bis-e meeting, to align with the discussion on the new dual SUL CA combination notations, a WF on the notation for single CA combinations was approved in [1] with the proposal as below.

Proposal: Notations for single SUL CA combinations are revised as below for the three example band combinations. Further clarification and explanation on notation in the specifications can be considered.

| CA configuration | Existing notation | Proposed notation | Note |
| --- | --- | --- | --- |
| Intra-band contiguous | SUL_n41C-n80A | CA_n41C_n80A | For the two NUL carriers within CA_n41C-n80A, the SUL band n80 is corresponding to one of them in one cell. |
| Intra-band non-contiguous | SUL_n41(2A)-n99A | CA_n41(2A)_n99A | For the two NUL carriers within CA_n41(2A)-n99A, the SUL band n99 is corresponding to one of them in one cell. |
| Inter-band | CA_n1A_SUL_n78A-n80A | CA_n1A_n78A-n80A | For the two NUL carriers within CA_n1A_n78A-n80A, the SUL band n80 is corresponding to n78 in one cell. |

In RAN4#107 meeting, three corresponding CRs were approved in [2-4] for introducing new notation for single SUL CA combinations in R16, R17 and R18 respectively. In this paper, we discuss how the notation modification affects RAN5 spec.

2. Discussion

According to PRD21, there are four types of configurations related to SUL band, in which 33 configurations are SUL band combination in FR1, 17 configurations are SUL band combination with intra-band contiguous CA, 5 configurations are SUL band combinations with intra-band non-contiguous CA, and 26 configurations are SUL band combination with inter-band CA. Based on the WF in [1], the notation for the three types of “SUL FR1 with IB cont CA”, “SUL FR1 with IB non-cont CA” and “SUL FR1 with inter-band CA” should be modified accordingly.

Furthermore, for intra-band CA with SUL band, though the WF in [1] proposed to use “_” instead of “-” between NR CA band and SUL band, however with the proposal in the CRs [2-4], it is proposed that “-” would be better from the NR band pairing with SUL band perspective.

Observation 1: The notation modification of SUL CA configuration in the core spec will also affect the current RAN5 specifications.

Proposal 1: In PRD21, the configurations types of “SUL FR1 with IB cont CA”, “SUL FR1 with IB non-cont CA” and “SUL FR1 with inter-band CA” should be modified with the notations as following.

| SUL configuration with CA | Current notation | Modified notation |
| --- | --- | --- |
| Intra-band contiguous CA | SUL_n41C-n80A | CA_n41C-n80A |
| Intra-band non-contiguous CA | SUL_n41(2A)-n99A | CA_n41(2A)-n99A |
| Inter-band CA | CA_n1A_SUL_n78A-n80A | CA_n1A_n78A-n80A |

The modification of SUL CA band combination notation will also affect the current RAN5 specs.

For TS 38.521-1, currently there is no SUL band combination with intra-band non-contiguous CA completed in the spec, thus only two types of “SUL FR1 with IB cont CA” and “SUL FR1 with inter-band CA” configurations needs to be modified. The tables for delta TIB in clause 6.2C and the tables for delta RIB in clause 7.3C should also be modified accordingly.

Table 5.2C-3: Operating SUL band combination with intra-band contiguous CA in FR1 (TS 38.521-1)

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n41-n83 | n41, n83 |
| CA_n78-n80 | n78, n80 |
| CA_n78-n84 | n78, n84 |
| CA_n79-n83 | n79, n83 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

Table 5.2C-4: Operating SUL band combination with inter-band CA in FR1 (TS 38.521-1)

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1_n78-n80 | n1, n78, n80 |
| CA_n1_n78-n84 | n1, n78, n84 |
| CA_n3_n78-n80 | n3, n78, n80 |
| CA_n28_n41-n83 | n28, n41, n83 |
| CA_n28_n79-n83 | n28, n79, n83 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

Table 5.5C-3: Supported channel bandwidths per SUL band combination with intra-band contiguous CA (TS 38.521-1)

| SUL band combination with CA | SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41C-n83A | SUL_n41A-n83A | n41 | CA_n41C_BCS1 | 0 |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n78C-n80A | SUL_n78A-n80A | n78 | CA_n78C_BCS1 | 0 |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n78C-n84A | SUL_n78A-n84A | n78 | CA_n78C_BCS1 | 0 |
|  |  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n79C-n83A | SUL_n79A-n83A | n79 | CA_n79C_BCS0 | 0 |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

Table 5.5C-4: Supported channel bandwidths per SUL band combination with inter-band CA 
(TS 38.521-1)

| SUL band combination with CA | SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A_n78A-n80A | SUL_n78A-n80A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A_n78A-n84A | SUL_n78A-n84A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n3A_n78A-n80A | SUL_n78A-n80A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n28A_n41A-n83A | SUL_n41A-n83A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n28A_n79A-n83A | SUL_n79A-n83A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

Table 6.2C.2-2: ΔTIB,c for SUL band combination (Three bands) (TS 38.521-1)

| Band combination for SUL | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n1_n78-n80 | n1 | 0.6 |
|  | n78 | 0.8 |
|  | n80 | 0.6 |
| CA_n1_n78-n84 | n1 | 0.6 |
|  | n78 | 0.8 |
|  | n84 | 0.6 |
| CA_n3_n78-n80 | n3 | 0.6 |
|  | n78 | 0.8 |
|  | n80 | 0.6 |
| CA_n28_n41-n83 | n28 | 0.3 |
|  | n41 | 0.3 |
|  | n83 | 0.3 |
| CA_n28_n79-n83 | n28 | 0.5 |
|  | n79 | 0.8 |
|  | n83 | 0.5 |

Table 7.3C.0.3.2.2-1: ΔRIB,c due to SUL (three bands) (TS 38.521-1)

| Band combination for SUL | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1_n78-n80 | n1 | 0.2 |
|  | n78 | 0.5 |
| CA_n1_n78-n84 | n1 | 0.2 |
|  | n78 | 0.5 |
| CA_n3_n78-n80 | n3 | 0.2 |
|  | n78 | 0.5 |
| CA_n28_n41-n83 | n28 | 0.2 |
| CA_n28_n79-n83 | n28 | 0.2 |
|  | n79 | 0.5 |

Proposal 2: In TS 38.521-1, the notations for “SUL FR1 with IB cont CA” and “SUL FR1 with inter-band CA” should be modified as above.

For TS 38.508-2, only the tables of Supported SUL configurations with intra-band contiguous CA and Supported SUL configurations with inter-band CA will be affected.

Table A.4.3.2C.3-2: Supported SUL configurations with Intra-band contiguous CA (TS 38.508-2)

| NR SUL configuration / Item (Note 1) | Release | Supported | Supported SUL configuration in UL | Supported Bandwidth Combination Set(s) | Supported ULTxSwitching Band Pair (Note 2, 3) |
| --- | --- | --- | --- | --- | --- |
| CA_n41C-n83A | Rel-17 |  |  |  |  |
| CA_n78C-n80A | Rel-17 |  |  |  |  |
| CA_n78C-n84A | Rel-17 |  |  |  |  |
| CA_n79C-n83A | Rel-17 |  |  |  |  |
| Note 1: Notation used for SUL configurations is according to TS 38.101-1 [23] Table 5.5C-3,. e.g. ‘CA_n41C-n83A’ indicates SUL operation on NR bands n41 and n83 with DL CA Bandwidth Class C on band n41. Note 2: The ULTxSwitching capability can be reported on SUL band combinations. The UE supplier shall indicate SUL band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘nX-nY’, where both nX and nY are NR bands. For example, for SUL_n78A-n80A, N would mean not supporting ULTxSwitching, ‘n78-n80’ would mean supporting of ULTxSwitching on this band pair. Note 3: ULSwitching(Table A.4.3.2C.2-1) shall return all supported SUL Configurations where at least one SUL band pair was declared in column “Supported ULTxSwitching Band Pair". |  |  |  |  |  |

Table A.4.3.2C.3-3: Supported SUL configurations with Inter-band CA (TS 38.508-2)

| NR SUL configuration / Item (Note 1) | Release | Supported | Supported SUL configuration in UL | Supported Bandwidth Combination Set(s) | Supported ULTxSwitching Band Pair (Note 2, 3) |
| --- | --- | --- | --- | --- | --- |
| CA_n1A_n78A-n80A | Rel-17 |  |  |  |  |
| CA_n1A_n78A-n84A | Rel-17 |  |  |  |  |
| CA_n3A_n78A-n80A | Rel-17 |  |  |  |  |
| CA_n28A_n41A-n83A | Rel-17 |  |  |  |  |
| CA_n28A_n79A-n83A | Rel-17 |  |  |  |  |
| Note 1: Notation used for SUL configurations is according to TS 38.101-1 [23] Table 5.5C-4. e.g. ‘CA_n1A_n78A-n84A’ indicates SUL operation on NR bands n1, n78 and n84 with DL CA Bandwidth Class A on bands n1 and n78. Note 2: The ULTxSwitching capability can be reported on SUL band combinations. The UE supplier shall indicate SUL band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘nX-nY’, where both nX and nY are NR bands. For example, for SUL_n78A-n80A, N would mean not supporting ULTxSwitching, ‘n78-n80’ would mean supporting of ULTxSwitching on this band pair. Note 3: ULSwitching(Table A.4.3.2C.2-1) shall return all supported SUL Configurations where at least one SUL band pair was declared in column “Supported ULTxSwitching Band Pair". |  |  |  |  |  |

Proposal 3: In TS 38.508-2, the notations for the “supported SUL configurations with Intra-band contiguous CA” and the “supported configurations with Inter-band CA” should be modified as above.

# 3. Conclusion

In this proposal, we discuss the issue related to the notation modification of SUL CA configurations. Based on the discussion, we have the following observation and proposals.

Observation 1: The notation modification of SUL CA configuration in the core spec will also affect the current RAN5 specifications.

Proposal 1: In PRD21, the configurations types of “SUL FR1 with IB cont CA”, “SUL FR1 with IB non-cont CA” and “SUL FR1 with inter-band CA” should be modified with the notations as following.

| SUL configuration with CA | Current notation | Modified notation |
| --- | --- | --- |
| Intra-band contiguous CA | SUL_n41C-n80A | CA_n41C_n80A |
| Intra-band non-contiguous CA | SUL_n41(2A)-n99A | CA_n41(2A)-n99A |
| Inter-band CA | CA_n1A_SUL_n78A-n80A | CA_n1A_n78A-n80A |

Proposal 2: In TS 38.521-1, the notations for “SUL FR1 with IB cont CA” and “SUL FR1 with inter-band CA” should be modified as above.

Proposal 3: In TS 38.508-2, the notations for the “supported SUL configurations with Intra-band contiguous CA” and the “supported configurations with Inter-band CA” should be modified as above.

# 4. References

- R4-2306546, WF on the notation for single SUL CA combinations, Apple

- R4-2309084, Rel-16 CR for 38.101-1: Single SUL CA combination notation modifications, Apple

- R4-2309085, Rel-17 CR for 38.101-1: Single SUL CA combination notation modifications, Apple

- R4-2309086, Rel-18 CR for 38.101-1: Single SUL CA combination notation modifications, Apple