3GPP TSG-RAN5 Meeting #100	R5-233897r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2688 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 13.1.1.3 for cell reselection in normal coverage with UE specific DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NB-IoT cell reselection test case 13.1.1.3 is missing in 36.521-3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | NB-IoT cell reselection test case is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.1.1.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-2 CR 1009 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CR R4-2311683 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Add editor’s note Modify the format to H6 Remove Ksatellite =TBD Add R4 dependancy for Ksatellite definition |  |  |  |  |  |  |  |  |

<< Start of changes >>

13.1.1.3	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

- Initial condition and call setup procedure support satellite access are TBD

13.1.1.3.1	Test purpose

To verify that when the current and target cell operates on the same carrier frequency the Cat-NB1 UE is able to search and measure cells to meet the intra-frequency cell re-selection requirements.

13.1.1.3.2	Test applicability

This test applies to all types of NB-IoT HD-FDD category NB1 UE supporting UE specific DRX, GSO or both from release 17 and forward.

13.1.1.3.3	Minimum conformance requirements

The UE shall be able to identify new intra-frequency cells and perform NRSRP measurements of identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS36.304 within Ksatellite*Tdetect,NB_Intra_NC when Treselection= 0 .  An intra frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in 3GPP TS 36.133 Annex B.1.4 for a corresponding Band.

The UE shall measure NRSRP at least every Ksatellite*Tmeasure,NB_Intra_NC for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter NRSRP measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Ksatellite *Tmeasure,NB_Intra-NC/2

The UE shall not consider an NB-IoT neighbour cell in cell reselection if it is indicated as not allowed in the measurement control system information of the serving NB-IoT cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in 3GPP TS 36.304[6] within Ksatellite *Tevaluate,NB_intra-NC when Treselection = 0, provided that the cell is at least XdB better ranked, where ‘X’ is specified in 3GPP TS 36.133 Table 4.6A.2.4-3. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and non-serving NB-IoT intra-frequency cells.

The parameter Ksatellite is the scaling factor for measurements correspond to multiple NGSO satellites. Ksatellite = 1, if GSO satellite(s) is/are measured on the carrier. Ksatellite  equals to the number NGSO satellites to be measured if NGSO satellites are monitored.

If Treselection timer has a non zero value and the intra-frequency cell is better ranked than the serving NB-IoT cell, the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

The UE shall evaluate the intra-frequency cell re-selection criteria as defined in 3GPP TS 36.304 [6] at least every DRX cycle.

The normative reference for this requirement is TS 36.133 [4] clause 4.6A.2 and A.13.1.1.3.

13.1.1.3.4	Test description

13.1.1.3.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.1.1.3.5-1.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.93 using only UE main Tx/Rx antenna.

2.	The parameter settings for the cells are set up according to Table 13.1.1.3.4.1-2.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	There are one NB-IoT carrier and two cells specified in the test. Ncell1 is standalone cell to Cell1 and Ncell2 is standalone cell to Cell2. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

During the test, the test system shall emulate and send the GNSS signal to the test UE by AT command. The UE shall be provided with the valid information about the SAN serving cells before the test

Table 13.1.1.3.4-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GEO, HD-FDD duplex mode |

Table 13.1.1.3.4-2: General test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in normal coverage

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NB-IOT operational mode |  |  | Standalone |  |  |
| Satellite information | Config 1 |  | GEO |  |  |
| Initial condition | Active cell |  | nCell1 |  |  |
|  | Neighbour cells |  | nCell2 |  |  |
| T2 end condition | Active cell |  | nCell2 |  |  |
|  | Neighbour cells |  | nCell1 |  |  |
| Final condition | Visited cell |  | nCell1 |  |  |
| Access Barring Information |  | - | Not Sent |  | No additional delays in random access procedure. |
| NPRACH Configuration |  |  | NPRACH.R-1 |  | Refer to A.3.18 |
| DRX cycle length |  | s | 0.32 | 0.64 | The value shall be used for all cells in the test. |
| T1 |  | s | >7 |  | During T1, nCell2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that nCell2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 35 |  | T2 is defined so that cell re-selection time is taken into account. Once the UE has reselected to nCell2 (within T2) T3 starts |
| T3 |  | s | 14 |  | T3 is defined so that cell re-selection time is taken into account. |

13.1.1.3.4.2	Test procedure

The test scenario comprises of one NB-IoT carrier with 2 Ncells of different physical cell ID. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Only Ncell1 is already identified by the UE prior to the start of the test, i.e. Ncell 2 is not identified. Ncell 1 and Ncell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Ncell 2.

In the following test procedure "UE responds" means "UE starts transmitting preamble on NPRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure according to 3GPP TS 36.508 [7] clause 8.1.5A.5"

1.	Ensure the UE is in State 3A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5 in Ncell 1.

2.	Set the parameters according to T1 in Table 13.1.1.3.5-1 and 13.1.1.3.5-2. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

3.	Set Ncell2 physical cell identity = ((current Ncell 2 physical cell identity + 1) mod 14 + 2) for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 4.2.18.5-1 and 4.2.18.5-2.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Ncell 2.

6.	If the UE responds on the newly detectable cell, Ncell 2, during time duration T2 within 35 seconds from the beginning of time period T2, then count a success for the event "Re-select newly detected Ncell 2". Otherwise count a fail for the event "Re-select newly detected Ncell 2".

7.	If the UE has re-selected Ncell 2 within T2, after the re-selection or when T2 expires, continue with step 8.
Otherwise, if T2 expires and the UE has not yet re-selected Ncell 2, skip to step 12.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 13.1.1.3.5-1.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Ncell 1.

10.	If the UE responds on the already detected cell, Ncell 1, during time duration T3 within 14 seconds from the beginning of time period T3, then count a success for the event "Re-select already detected Ncell 1". Otherwise count a fail for the event "Re-select already detected Ncell 1".

11.	If the UE has re-selected Ncell 1 within T3, after the re-selection or when T3 expires, skip to step 13.
Otherwise, if T3 expires and the UE has not yet re-selected Ncell 1, continue with step 12.

12.	Switch off and on the UE and ensure the UE is in State 3A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5 in Ncell 1.

13.	Repeat step 2-12 until a test verdict has been achieved.
Each of the events "Re-select newly detected Ncell 2" and "Re-select already detected Ncell 1" is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

13.1.1.3.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.4.3, 8.1.5B and 8.1.6 using condition “standalone” with the following exceptions:

Table 13.1.1.3.4.3-1: SystemInformationBlockType2-NB:
HD-FDD Intra frequency cell reselection for Category NB1 UE for Ncell 1 and Ncell2

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.4.3.3, Table 8.1.4.3.3-1: SystemInformationBlockType2-NB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ue-SpecificDRX-CycleMin-r16 | rf32 |  | Test1 |
|  | rf64 |  | Test2 |
| } |  |  |  |

Table 13.1.1.3.4.3-2: ATTACH ACCEPT: HD-FDD Intra frequency cell reselection for Category NB1 UE for step 1

| Derivation Path: 36.508 clause 4.7.2 Table 4.7.2-1 ATTACH ACCEPT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Negotiated DRX parameter in NB-S1 mode |  |  |  |  |
| NB-S1 mode DRX value |  | DRX cycle parameter T = 32 |  | Test1 |
|  |  | DRX cycle parameter T = 64 |  | Test2 |

Table 13.1.1.3.4.3-3: TRACKING AREA UPDATE ACCEPT: HD-FDD Intra frequency cell reselection for Category NB1 UE for for step 6 and step 10

| Derivation Path: 36.508 clause 4.7.2 Table 4.7.2-24 TRACKING AREA UPDATE ACCEPT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Negotiated DRX parameter in NB-S1 mode |  |  |  |  |
| NB-S1 mode DRX value |  | DRX cycle parameter T = 32 |  | Test1 |
|  |  | DRX cycle parameter T = 64 |  | Test2 |

Table 13.1.1.3.4.3-4: ATTACH REQUEST : HD-FDD Intra frequency cell reselection for Category NB1 UE for for step 1

| Derivation Path: 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX parameter in NB-S1 mode | '0000'B | DRX value not specified and use cell specific DRX value |  |

Table 13.1.1.3.4.3-5: TRACKING AREA UPDATE REQUEST : HD-FDD Intra frequency cell reselection for Category NB1 UE for for step 6 and step 10

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| DRX parameter in NB-S1 mode | DRX cycle parameter T = 32 |  | Test1 |
|  | DRX cycle parameter T = 64 |  | Test2 |

13.1.1.3.5	Test requirement

Tables 13.1.1.3.5-3 defines the primary level settings including test tolerances for HD-FDD intra frequency cell re-selection with UE specific DRX test case.

Table 13.1.1.3.5-1: nCell 1, nCell 2 specific test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in normal coverage

| Parameter | Unit | nCell 1 |  |  | nCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | kHz | 180 |  |  | 180 |  |  |
| OCNG Pattern as defined in A.3 | - | NOP.3 FDD |  |  | NOP.3 FDD |  |  |
| NPBCH_RA | dB | -3 |  |  | -3 |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Qrxlevmin | dBm | -140 | -140 | -140 | -140 | -140 | -140 |
| Pcompensation | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Qhysts | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Qoffsets, n | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Cell_selection_and_ reselection_quality_measurement |  | NRSRP |  |  | NRSRP |  |  |
|  | dBm/15 kHz | -98+TT |  |  |  |  |  |
|  | dB | 17+TT | 13+TT | 17+TT | -infinity | 17+TT | 13+TT |
| Note2 | dB | 17+TT | -4.09+TT | 3.79+TT | -infinity | 3.79+TT | -4.09+TT |
| NRSRP Note2 | dBm/15 kHz | -81+TT | -85+TT | -81+TT | -infinity | -81+TT | -85+TT |
| Treselection | s | 0 | 0 | 0 | 0 | 0 | 0 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to nCell 1 | ms | - |  |  | 3 |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Ncell 2 and starts to send preambles on the NPRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Ncell 2.

The cell re-selection delay to a newly detectable cell shall be less than 34.32 s in test1 and test2.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on Ncell 1 and starts to send preambles on the NPRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Ncell 1.

The cell re-selection delay to an already detected cell shall be less than 13.44s in test1 and test2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect,NB_Intra_NB-IoT-NC + TSI-NB, and to an already detected cell can be expressed as: Tevaluate, NB_intra_NB-IoT-NC + TSI-NB,

Where:

Tdetect,NB_Intra_NB-IoT-NC	as specified in 3GPP TS 36.133 [4] Table 4.6A.2.2-1 in clause 4.6A.2.2.

Tevaluate, NB_intra_NB-IoT-NC	as specified in 3GPP TS 36.133 [4] Table 4.6A.2.2-1 in clause 4.6A.2.2.

TSI-NB	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 8.32 s is assumed in this test case.

This gives a total of 34.32 s, allow 35 s for the cell re-selection delay to a newly detectable cell and 13.44 s, and allow 14 s for the cell re-selection delay to an already detected cell in the test case.

For the test to pass, both events above shall pass.

The statistical pass/ fail decisions are done separately for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

<< End of changes >>