3GPP TSG-RAN5 Meeting #100	R5-234760r1

# Toulouse, France, Aug 21 - 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Annex A in 36521-4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | N/A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 16) Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) |  |
| Reason for change: |  | In the updated WP for RA5#100, Annex A has been allocated to “Measurement Channels”, and Annex A in 36521-4 needs to be added accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annex A in 36521-4 has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Plan will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Section A.3 and A.5 have been updated according to TS 36.102 Annex A. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

Annex A (normative):
Measurement Channels

A.1	General

A schematic overview of the encoding process for the reference measurement channels is provided in Figure A-1.

Figure A-1: Schematic overview of the encoding process

The throughput values defined in the measurement channels specified in Annex A, are calculated and are valid per data stream (codeword). For multi-stream (more than one codeword) transmissions, the throughput referenced in the minimum requirements is the sum of throughputs of all data streams (codewords).

The UE category entry in the definition of the reference measurement channels in Annex A is only informative and reveals the UE categories, which can support the corresponding measurement channel. Whether the measurement channel is used for testing a certain UE category or not is specified in the individual test cases.

Rate matching process in Figure A-1 is dependent on the parameter “Total number of Soft Channel bits” which has been defined for each UE category in TS 36.306 [15] clause 4.1. The SS shall use the Soft Channel bits size according to the UE category.

A.2	UL reference measurement channels

A.2.1	General

A.2.1.1	Applicability and common parameters

The following sections define the UL signal applicable to the Transmitter Characteristics (clause 6) and for the Receiver Characteristics (clause 7) where the UL signal is relevant.

The Reference channels in this section assume transmission of PUSCH and Demodulation Reference signal only. The following conditions apply:

-	1 HARQ transmission

-	Cyclic Prefix normal

-	PUSCH hopping off

-	Link adaptation off

-	Demodulation Reference signal as per TS 36.211 [3] clause 5.5.2.1.2.

Where ACK/NACK is transmitted, it is assumed to be multiplexed on PUSCH as per TS 36.212 [19] subclause 5.2.2.6.

-	ACK/NACK 1 bit

-	ACK/NACK mapping adjacent to Demodulation Reference symbol

-	ACK/NACK resources punctured into data

-	Max number of resources for ACK/NACK: 4 SC-FDMA symbols per subframe

-	No CQI transmitted, no RI transmitted

A.2.1.2	Determination of payload size

The algorithm for determining the payload size A is as follows; given a desired coding rate R and radio block allocation NRB:

1.	Calculate the number of channel bits Nch that can be transmitted during the first transmission of a given sub-frame.

2.	Find A such that the resulting coding rate is as close to R as possible, that is,

,

subject to

a)	A is a valid TB size according to clause 7.1.7 of TS 36.213 [10] assuming an allocation of NRB resource blocks.

b)	C is the number of Code Blocks calculated according to section 5.1.2 of TS 36.212 [9].

c)	For RMC-s, which at the nominal target coding rate do not cover all the possible UE categories for the given modulation, reduce the target coding rate gradually (within the same modulation), until the maximal possible number of UE categories is covered.

3.	If there is more than one A that minimises the equation above, then the larger value is chosen per default and the chosen code rate should not exceed 0.93.

A.2.1.3	Overview of UL reference measurement channels

In Table A.2.1.3-1 are listed the UL reference measurement channels specified in Annexes A.2.2 and A.2.3 of this release of TS 36.521-4. This table is informative and serves only to a better overview. The reference for the concrete reference measurement channels and corresponding implementation’s parameters as to be used for testing are annexes A.2.2 and A.2.3 as appropriate.

Table A.2.1.3-1: Overview of UL reference measurement channels

| Duplex | Table | Name |  | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD, Full RB allocation, QPSK |  |  |  |  |  |  |  |  |  |  |
| FDD | Table A.2.2.1.1-1 |  |  | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD / HD-FDD |  | Table A.2.2.1.1-1a |  |  | 1.4 | QPSK | 1/3 | 6 |  | 0 |  |
| FDD / HD-FDD |  | Table A.2.2.1.1-1b |  |  | 1.4 | QPSK | 1/3 | 6 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.1.1-1c |  |  | 1.4 | QPSK | 1/3 | 6 |  | M2 | UE UL category M2 |
| FDD, Full RB allocation, 16-QAM |  |  |  |  |  |  |  |  |  |  |
| FDD | Table A.2.2.1.2-1 |  |  | 1.4 | 16QAM | 3/4 | 6 |  | ≥ 1 |  |
| FDD / HD-FDD |  | Table A.2.2.1.2-1a |  |  | 1.4 | 16QAM | 1/3 | 5 |  | 0 |  |
| FDD / HD-FDD |  | Table A.2.2.1.2-1b |  |  | 1.4 | 16QAM | 1/3 | 5 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.1.2-1c |  |  | 1.4 | 16QAM | 1/3 | 6 |  | M2 | UE UL category M2 |
| FDD, Partial RB allocation, QPSK |  |  |  |  |  |  |  |  |  |  |
| FDD | Table A.2.2.2.1-1 |  | 1.4 |  | QPSK | 1/3 | 1 |  | ≥ 1 |  |
| FDD | Table A.2.2.2.1-1 |  | 1.4 |  | QPSK | 1/3 | 2 |  | ≥ 1 |  |
| FDD | Table A.2.2.2.1-1 |  | 1.4 |  | QPSK | 1/3 | 3 |  | ≥ 1 |  |
| FDD | Table A.2.2.2.1-1 |  | 1.4 |  | QPSK | 1/3 | 4 |  | ≥ 1 |  |
| FDD | Table A.2.2.2.1-1 |  | 1.4 |  | QPSK | 1/3 | 5 |  | ≥ 1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1a |  | 1. |  | QPSK | 1/3 | 1 |  | 0 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1a |  | 1.4 |  | QPSK | 1/3 | 2 |  | 0 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1a |  | 1.4 |  | QPSK | 1/3 | 3 |  | 0 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1a |  | 1.4 |  | QPSK | 1/3 | 4 |  | 0 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1a |  | 1.4 |  | QPSK | 1/3 | 5 |  | 0 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1b |  | 1.4 |  | QPSK | 1/3 | 1 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1b |  | 1.4 |  | QPSK | 1/3 | 2 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1b |  | 1.4 |  | QPSK | 1/3 | 3 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1b |  | 1.4 |  | QPSK | 1/3 | 4 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1b |  | 1.4 |  | QPSK | 1/3 | 5 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1b |  | 1.4 |  | QPSK | 1/3 | 6 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1c |  | 1.4 |  | QPSK | 1/3 | 1 |  | M2 | UE UL category M2 |
| FDD / HD-FDD |  | Table A.2.2.2.1-1c |  | 1.4 |  | QPSK | 1/3 | 2 |  | M2 | UE UL category M2 |
| FDD / HD-FDD |  | Table A.2.2.2.1-1c |  | 1.4 |  | QPSK | 1/3 | 3 |  | M2 | UE UL category M2 |
| FDD / HD-FDD |  | Table A.2.2.2.1-1c |  | 1.4 |  | QPSK | 1/3 | 4 |  | M2 | UE UL category M2 |
| FDD / HD-FDD |  | Table A.2.2.2.1-1c |  | 1.4 |  | QPSK | 1/3 | 5 |  | M2 | UE UL category M2 |
| FDD, Partial RB allocation, 16-QAM |  |  |  |  |  |  |  |  |  |  |
| FDD | Table A.2.2.2.2-1 |  | 1.4 |  | 16QAM | 3/4 | 1 |  | ≥ 1 |  |
| FDD | Table A.2.2.2.2-1 |  | 1.4 |  | 16QAM | 3/4 | 2 |  | ≥ 1 |  |
| FDD | Table A.2.2.2.2-1 |  | 1.4 |  | 16QAM | 3/4 | 3 |  | ≥ 1 |  |
| FDD | Table A.2.2.2.2-1 |  | 1.4 |  | 16QAM | 3/4 | 4 |  | ≥ 1 |  |
| FDD | Table A.2.2.2.2-1 |  | 1.4 |  | 16QAM | 3/4 | 5 |  | ≥ 1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.2-1a |  | 1.4 |  | 16QAM | 3/4 | 1 |  | 0 |  |
| FDD / HD-FDD |  | Table A.2.2.2.2-1a |  | 1.4 |  | 16QAM | 3/4 | 2 |  | 0 |  |
| FDD / HD-FDD |  | Table A.2.2.2.2-1a |  | 1.4 |  | 16QAM | 2/5 | 4 |  | 0 |  |
| FDD / HD-FDD |  | Table A.2.2.2.2-1b |  | 1.4 |  | 16QAM | 1/2 | 1 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.2-1b |  | 1.4 |  | 16QAM | 1/2 | 2 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.2-1b |  | 1.4 |  | 16QAM | 1/2 | 3 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.2-1b |  | 1.4 |  | 16QAM | 2/5 | 4 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.2-1c |  | 1.4 |  | 16QAM | 1/2 | 1 |  | M2 | UE UL category M2 |
| FDD / HD-FDD |  | Table A.2.2.2.2-1c |  | 1.4 |  | 16QAM | 1/2 | 2 |  | M2 | UE UL category M2 |
| FDD / HD-FDD |  | Table A.2.2.2.2-1c |  | 1.4 |  | 16QAM | 1/2 | 3 |  | M2 | UE UL category M2 |
| FDD / HD-FDD |  | Table A.2.2.2.2-1c |  | 1.4 |  | 16QAM | 1/2 | 4 |  | M2 | UE UL category M2 |
| FDD / HD-FDD |  | Table A.2.2.2.2-1c |  | 1.4 |  | 16QAM | 1/2 | 5 |  | M2 | UE UL category M2 |
| FDD, Sustained data rate |  |  |  |  |  |  |  |  |  |  |
| FDD | Table A.2.2.3-1 | R.1-1 FDD |  | 10 | QPSK | 0.31 | 40 |  | ≥ 1 |  |
| FDD | Table A.2.2.3-1 | R.1-2 FDD |  | 10 | QPSK | 0.31 | 40 |  | ≥ 1 |  |
| FDD | Table A.2.2.3-1 | R.1-3 FDD |  | 20 | QPSK | 0.31 | 90 |  | ≥ 2 |  |
| FDD | Table A.2.2.3-1 | R.1-3A FDD |  | 10 | QPSK | 0.31 | 40 |  | ≥ 1 |  |
| FDD | Table A.2.2.3-1 | R.1-4 FDD |  | 20 | QPSK | 0.31 | 40 |  | ≥ 2 |  |
| FDD | Table A.2.2.3-1 | R.1-4B FDD |  | 15 | QPSK | 0.31 | 60 |  | ≥ 1 |  |

A.2.2	Reference measurement channels for FDD

A.2.2.1	Full RB allocation

A.2.2.1.1	QPSK

Table A.2.2.1.1-1: Reference Channels for QPSK with full RB allocation

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 6 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 |
| Modulation |  | QPSK |
| Target Coding rate |  | 1/3 |
| Payload size | Bits | 600 |
| Transport block CRC | Bits | 24 |
| Number of code blocks per Sub-Frame (Note 1) |  | 1 |
| Total number of bits per Sub-Frame (Note 1) | Bits | 1728 |
| Total symbols per Sub-Frame |  | 864 |
| UE Category |  | ≥ 1 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |

Table A.2.2.1.1-1a: Reference Channels for QPSK with full/maximum RB allocation for UE category 0

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 6 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 |
| Modulation |  | QPSK |
| Target Coding rate |  | 1/3 |
| Payload size | Bits | 600 |
| Transport block CRC | Bits | 24 |
| Number of code blocks per Sub-Frame |  | 1 |
| Total number of bits per Sub-Frame | Bits | 1728 |
| Total symbols per Sub-Frame |  | 864 |
| UE Category |  | 0 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th, 6th, 12th, 13th, 14th, 20th, 21st, 22nd, 28th, 29th, 30th, 36th, 37th, and 38th subframes every 40ms. Information bit payload is available if uplink subframe is scheduled. |  |  |

Table A.2.2.1.1-1b: Reference Channels for QPSK with full/maximum RB allocation for UE category M1

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 6 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 |
| Modulation |  | QPSK |
| Target Coding rate |  | 1/3 |
| Payload size | Bits | 600 |
| Transport block CRC | Bits | 24 |
| Number of code blocks per Sub-Frame |  | 1 |
| Total number of bits per Sub-Frame | Bits | 1728 |
| Total symbols per Sub-Frame |  | 864 |
| UE Category |  | M1 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th and 6th subframes every 10ms for the channel bandwidth 5MHz/10MHz/15MHz/20MHz. For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th, and 7th subframes every 10ms for the channel bandwidth 1.4MHz/3MHz. Information bit payload is available if uplink subframe is scheduled. is total number of absolute subframes a PUSCH with repetition spans [4]. NOTE 3: For HD-FDD UE with , MPDCCH are scheduled at 0th DL subframe every +5 subframes (starting from the 0th subframe). The associated PUSCH is scheduled at the 4th to (+3)-th UL subframes every +5 subframes. Information bit payload is available if uplink subframe is scheduled. |  |  |

Table A.2.2.1.1-1c: Reference Channels for QPSK with full/maximum RB allocation for UE UL category M2

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 6 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 |
| Modulation |  | QPSK |
| Target Coding rate |  | 1/3 |
| Payload size | Bits | 600 |
| Transport block CRC | Bits | 24 |
| Number of code blocks per Sub-Frame (NOTE 1) |  | 1 |
| Total number of bits per Sub-Frame | Bits | 1728 |
| Total symbols per Sub-Frame |  | 864 |
| UE UL Category |  | M2 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th and 7th subframes every 10ms for the channel bandwidth 1.4MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |

A.2.2.1.2	16-QAM

Table A.2.2.1.2-1: Reference Channels for 16-QAM with full RB allocation

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 6 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 |
| Modulation |  | 16QAM |
| Target Coding rate |  | 3/4 |
| Payload size | Bits | 2600 |
| Transport block CRC | Bits | 24 |
| Number of code blocks per Sub-Frame (Note 1) |  | 1 |
| Total number of bits per Sub-Frame | Bits | 3456 |
| Total symbols per Sub-Frame |  | 864 |
| UE Category |  | ≥ 1 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |

Table A.2.2.1.2-1a: Reference Channels for 16-QAM with maximum RB allocation for UE category 0

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 5 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 |
| Modulation |  | 16QAM |
| Target Coding rate |  | 1/3 |
| Payload size | Bits | 872 |
| Transport block CRC | Bits | 24 |
| Number of code blocks per Sub-Frame |  | 1 |
| Total number of bits per Sub-Frame | Bits | 2880 |
| Total symbols per Sub-Frame |  | 720 |
| UE Category |  | 0 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th, 6th, 12th, 13th, 14th, 20th, 21st, 22nd, 28th, 29th, 30th, 36th, 37th, and 38th subframes every 40ms. Information bit payload is available if uplink subframe is scheduled. |  |  |

Table A.2.2.1.2-1b: Reference Channels for 16-QAM with maximum RB allocation for UE category M1

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 5 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 |
| Modulation |  | 16QAM |
| Target Coding rate |  | 1/3 |
| Payload size | Bits | 872 |
| Transport block CRC | Bits | 24 |
| Number of code blocks per Sub-Frame |  | 1 |
| Total number of bits per Sub-Frame | Bits | 2880 |
| Total symbols per Sub-Frame |  | 720 |
| UE Category |  | M1 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th, and 7th subframes every 10ms for the channel bandwidth 1.4MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |

Table A.2.2.1.2-1c: Reference Channels for 16-QAM with maximum RB allocation for UE UL category M2

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 6 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 |
| Modulation |  | 16QAM |
| Target Coding rate |  | 1/3 |
| Payload size | Bits | 1032 |
| Transport block CRC | Bits | 24 |
| Number of code blocks per Sub-Frame |  | 1 |
| Total number of bits per Sub-Frame | Bits | 3456 |
| Total symbols per Sub-Frame |  | 864 |
| UE Category |  | M2 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th and 7th subframes every 10ms for the channel bandwidth 1.4MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |

A.2.2.2	Partial RB allocation

For each channel bandwidth, various partial RB allocations are specified. The number of allocated RBs is chosen according to values specified in the Tx and Rx requirements. The single allocated RB case is included.

The allocated RBs are contiguous and start from one end of the channel bandwidth. A single allocated RB is at one end of the channel bandwidth.

A.2.2.2.1	QPSK

Table A.2.2.2.1-1: Reference Channels for QPSK with partial RB allocation

| Parameter | Ch BW | Allocated RBs | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size | Transport block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame | Total symbols per Sub-Frame | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 12 | QPSK | 1/3 | 72 | 24 | 1 | 288 | 144 | ≥ 1 |
|  | 1.4 - 20 | 2 | 12 | QPSK | 1/3 | 176 | 24 | 1 | 576 | 288 | ≥ 1 |
|  | 1.4 - 20 | 3 | 12 | QPSK | 1/3 | 256 | 24 | 1 | 864 | 432 | ≥ 1 |
|  | 1.4 - 20 | 4 | 12 | QPSK | 1/3 | 392 | 24 | 1 | 1152 | 576 | ≥ 1 |
|  | 1.4 - 20 | 5 | 12 | QPSK | 1/3 | 424 | 24 | 1 | 1440 | 720 | ≥ 1 |
|  | 3-20 | 6 | 12 | QPSK | 1/3 | 600 | 24 | 1 | 1728 | 864 | ≥ 1 |
|  | 3-20 | 8 | 12 | QPSK | 1/3 | 808 | 24 | 1 | 2304 | 1152 | ≥ 1 |
|  | 3-20 | 9 | 12 | QPSK | 1/3 | 776 | 24 | 1 | 2592 | 1296 | ≥ 1 |
|  | 3-20 | 10 | 12 | QPSK | 1/3 | 872 | 24 | 1 | 2880 | 1440 | ≥ 1 |
|  | 3-20 | 12 | 12 | QPSK | 1/3 | 1224 | 24 | 1 | 3456 | 1728 | ≥ 1 |
|  | 5-20 | 15 | 12 | QPSK | 1/3 | 1320 | 24 | 1 | 4320 | 2160 | ≥ 1 |
|  | 5-20 | 16 | 12 | QPSK | 1/3 | 1384 | 24 | 1 | 4608 | 2304 | ≥ 1 |
|  | 5-20 | 18 | 12 | QPSK | 1/3 | 1864 | 24 | 1 | 5184 | 2592 | ≥ 1 |
|  | 5-20 | 20 | 12 | QPSK | 1/3 | 1736 | 24 | 1 | 5760 | 2880 | ≥ 1 |
|  | 5-20 | 24 | 12 | QPSK | 1/3 | 2472 | 24 | 1 | 6912 | 3456 | ≥ 1 |
|  | 10-20 | 25 | 12 | QPSK | 1/3 | 2216 | 24 | 1 | 7200 | 3600 | ≥ 1 |
|  | 10-20 | 27 | 12 | QPSK | 1/3 | 2792 | 24 | 1 | 7776 | 3888 | ≥ 1 |
|  | 10-20 | 30 | 12 | QPSK | 1/3 | 2664 | 24 | 1 | 8640 | 4320 | ≥ 1 |
|  | 10-20 | 32 | 12 | QPSK | 1/3 | 2792 | 24 | 1 | 9216 | 4608 | ≥ 1 |
|  | 10-20 | 36 | 12 | QPSK | 1/3 | 3752 | 24 | 1 | 10368 | 5184 | ≥ 1 |
|  | 10-20 | 40 | 12 | QPSK | 1/3 | 4136 | 24 | 1 | 11520 | 5760 | ≥ 1 |
|  | 10-20 | 45 | 12 | QPSK | 1/3 | 4008 | 24 | 1 | 12960 | 6480 | ≥ 1 |
|  | 10-20 | 48 | 12 | QPSK | 1/3 | 4264 | 24 | 1 | 13824 | 6912 | ≥ 1 |
|  | 15 - 20 | 50 | 12 | QPSK | 1/3 | 5160 | 24 | 1 | 14400 | 7200 | ≥ 1 |
|  | 15 - 20 | 54 | 12 | QPSK | 1/3 | 4776 | 24 | 1 | 15552 | 7776 | ≥ 1 |
|  | 15 - 20 | 60 | 12 | QPSK | 1/4 | 4264 | 24 | 1 | 17280 | 8640 | ≥ 1 |
|  | 15 - 20 | 64 | 12 | QPSK | 1/4 | 4584 | 24 | 1 | 18432 | 9216 | ≥ 1 |
|  | 15 - 20 | 72 | 12 | QPSK | 1/4 | 5160 | 24 | 1 | 20736 | 10368 | ≥ 1 |
|  | 20 | 75 | 12 | QPSK | 1/5 | 4392 | 24 | 1 | 21600 | 10800 | ≥ 1 |
|  | 20 | 80 | 12 | QPSK | 1/5 | 4776 | 24 | 1 | 23040 | 11520 | ≥ 1 |
|  | 20 | 81 | 12 | QPSK | 1/5 | 4776 | 24 | 1 | 23328 | 11664 | ≥ 1 |
|  | 20 | 90 | 12 | QPSK | 1/6 | 4008 | 24 | 1 | 25920 | 12960 | ≥ 1 |
|  | 20 | 96 | 12 | QPSK | 1/6 | 4264 | 24 | 1 | 27648 | 13824 | ≥ 1 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.2.1-1a: Reference Channels for QPSK with partial RB allocation for UE category 0

| Parameter | Ch BW | Allocated RBs | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size | Trans-port block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame | Total symbols per Sub-Frame | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 12 | QPSK | 1/3 | 72 | 24 | 1 | 288 | 144 | 0 |
|  | 1.4 - 20 | 2 | 12 | QPSK | 1/3 | 176 | 24 | 1 | 576 | 288 | 0 |
|  | 1.4 - 20 | 3 | 12 | QPSK | 1/3 | 256 | 24 | 1 | 864 | 432 | 0 |
|  | 1.4 - 20 | 4 | 12 | QPSK | 1/3 | 392 | 24 | 1 | 1152 | 576 | 0 |
|  | 1.4 - 20 | 5 | 12 | QPSK | 1/3 | 424 | 24 | 1 | 1440 | 720 | 0 |
|  | 3-20 | 6 | 12 | QPSK | 1/3 | 600 | 24 | 1 | 1728 | 864 | 0 |
|  | 3-20 | 8 | 12 | QPSK | 1/3 | 808 | 24 | 1 | 2304 | 1152 | 0 |
|  | 3-20 | 9 | 12 | QPSK | 1/3 | 776 | 24 | 1 | 2592 | 1296 | 0 |
|  | 3-20 | 10 | 12 | QPSK | 1/3 | 872 | 24 | 1 | 2880 | 1440 | 0 |
|  | 3-20 | 12 | 12 | QPSK | 1/4 | 840 | 24 | 1 | 3456 | 1728 | 0 |
|  | 5-20 | 15 | 12 | QPSK | 1/5 | 872 | 24 | 1 | 4320 | 2160 | 0 |
|  | 5-20 | 16 | 12 | QPSK | 1/5 | 904 | 24 | 1 | 4608 | 2304 | 0 |
|  | 5-20 | 18 | 12 | QPSK | 1/6 | 776 | 24 | 1 | 5184 | 2592 | 0 |
|  | 5-20 | 20 | 12 | QPSK | 1/6 | 872 | 24 | 1 | 5760 | 2880 | 0 |
|  | 5-20 | 24 | 12 | QPSK | 1/8 | 872 | 24 | 1 | 6912 | 3456 | 0 |
|  | 10-20 | 25 | 12 | QPSK | 1/8 | 904 | 24 | 1 | 7200 | 3600 | 0 |
|  | 10-20 | 27 | 12 | QPSK | 1/8 | 968 | 24 | 1 | 7776 | 3888 | 0 |
|  | 10-20 | 30 | 12 | QPSK | 1/10 | 808 | 24 | 1 | 8640 | 4320 | 0 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th, 6th, 12th, 13th, 14th, 20th, 21st, 22nd, 28th, 29th, 30th, 36th, 37th, and 38th subframes every 40ms. Information bit payload is available if uplink subframe is scheduled. |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.2.1-1b: Reference Channels for QK with partial RB allocation for UE category M1

| Parameter | Ch BW | Allocated RBs | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size | Trans-port block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame | Total symbols per Sub-Frame | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 12 | QPSK | 1/3 | 72 | 24 | 1 | 288 | 144 | M1 |
|  | 1.4 - 20 | 2 | 12 | QPSK | 1/3 | 176 | 24 | 1 | 576 | 288 | M1 |
|  | 1.4 - 20 | 3 | 12 | QPSK | 1/3 | 256 | 24 | 1 | 864 | 432 | M1 |
|  | 1.4 - 20 | 4 | 12 | QPSK | 1/3 | 392 | 24 | 1 | 1152 | 576 | M1 |
|  | 1.4 - 20 | 5 | 12 | QPSK | 1/3 | 424 | 24 | 1 | 1440 | 720 | M1 |
|  | 3-20 | 6 | 12 | QPSK | 1/3 | 600 | 24 | 1 | 1728 | 864 | M1 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th and 6thsubframes every 10ms for the channel bandwidth 5MHz/10MHz/15MHz/20MHz. For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th, and 7th subframes every 10ms for the channel bandwidth 1.4MHz/3MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.2.1-1c: Reference Channels for QPSK with partial RB allocation for UE UL category M2

| Parameter | Ch BW | Allocated RBs | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size | Trans-port block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame | Total symbols per Sub-Frame | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 12 | QPSK | 1/3 | 72 | 24 | 1 | 288 | 144 | M2 |
|  | 1.4 - 20 | 2 | 12 | QPSK | 1/3 | 176 | 24 | 1 | 576 | 288 | M2 |
|  | 1.4 - 20 | 3 | 12 | QPSK | 1/3 | 256 | 24 | 1 | 864 | 432 | M2 |
|  | 1.4 - 20 | 4 | 12 | QPSK | 1/3 | 392 | 24 | 1 | 1152 | 576 | M2 |
|  | 1.4 - 20 | 5 | 12 | QPSK | 1/3 | 424 | 24 | 1 | 1440 | 720 | M2 |
|  | 3-20 | 6 | 12 | QPSK | 1/3 | 600 | 24 | 1 | 1728 | 864 | M2 |
|  | 3-20 | 9 | 12 | QPSK | 1/3 | 776 | 24 | 1 | 2592 | 1296 | M2 |
|  | 3-20 | 12 | 12 | QPSK | 1/3 | 1032 | 24 | 1 | 3456 | 1728 | M2 |
|  | 5-20 | 15 | 12 | QPSK | 1/3 | 1320 | 24 | 1 | 4320 | 2160 | M2 |
|  | 5-20 | 18 | 12 | QPSK | 1/3 | 1864 | 24 | 1 | 5184 | 2592 | M2 |
|  | 5-20 | 21 | 12 | QPSK | 1/3 | 2216 | 24 | 1 | 6068 | 3024 | M2 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th and 6th subframes every 10ms for the channel bandwidth 5MHz/10MHz/15MHz/20MHz. For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th and 7th subframes every 10ms for the channel bandwidth 1.4MHz/3MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |  |  |  |  |  |  |  |  |  |

A.2.2.2.2	16-QAM

Table A.2.2.2.2-1: Reference Channels for 16-QAM with partial RB allocation

| Parameter | Ch BW | Allocated RBs | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size | Transport block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame | Total symbols per Sub-Frame | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 12 | 16QAM | 3/4 | 408 | 24 | 1 | 576 | 144 | ≥ 1 |
|  | 1.4 - 20 | 2 | 12 | 16QAM | 3/4 | 840 | 24 | 1 | 1152 | 288 | ≥ 1 |
|  | 1.4 - 20 | 3 | 12 | 16QAM | 3/4 | 1288 | 24 | 1 | 1728 | 432 | ≥ 1 |
|  | 1.4 - 20 | 4 | 12 | 16QAM | 3/4 | 1736 | 24 | 1 | 2304 | 576 | ≥ 1 |
|  | 1.4 - 20 | 5 | 12 | 16QAM | 3/4 | 2152 | 24 | 1 | 2880 | 720 | ≥ 1 |
|  | 3-20 | 6 | 12 | 16QAM | 3/4 | 2600 | 24 | 1 | 3456 | 864 | ≥ 1 |
|  | 3-20 | 8 | 12 | 16QAM | 3/4 | 3496 | 24 | 1 | 4608 | 1152 | ≥ 1 |
|  | 3-20 | 9 | 12 | 16QAM | 3/4 | 3880 | 24 | 1 | 5184 | 1296 | ≥ 1 |
|  | 3-20 | 10 | 12 | 16QAM | 3/4 | 4264 | 24 | 1 | 5760 | 1440 | ≥ 1 |
|  | 3-20 | 12 | 12 | 16QAM | 3/4 | 5160 | 24 | 1 | 6912 | 1728 | ≥ 1 |
|  | 5-20 | 15 | 12 | 16QAM | 1/2 | 4264 | 24 | 1 | 8640 | 2160 | ≥ 1 |
|  | 5-20 | 16 | 12 | 16QAM | 1/2 | 4584 | 24 | 1 | 9216 | 2304 | ≥ 1 |
|  | 5-20 | 18 | 12 | 16QAM | 1/2 | 5160 | 24 | 1 | 10368 | 2592 | ≥ 1 |
|  | 5-20 | 20 | 12 | 16QAM | 1/3 | 4008 | 24 | 1 | 11520 | 2880 | ≥ 1 |
|  | 5-20 | 24 | 12 | 16QAM | 1/3 | 4776 | 24 | 1 | 13824 | 3456 | ≥ 1 |
|  | 10-20 | 25 | 12 | 16QAM | 1/3 | 4968 | 24 | 1 | 14400 | 3600 | ≥ 1 |
|  | 10-20 | 27 | 12 | 16QAM | 1/3 | 4776 | 24 | 1 | 15552 | 3888 | ≥ 1 |
|  | 10-20 | 30 | 12 | 16QAM | 3/4 | 12960 | 24 | 3 | 17280 | 4320 | ≥ 2 |
|  | 10-20 | 32 | 12 | 16QAM | 3/4 | 13536 | 24 | 3 | 18432 | 4608 | ≥ 2 |
|  | 10-20 | 36 | 12 | 16QAM | 3/4 | 15264 | 24 | 3 | 20736 | 5184 | ≥ 2 |
|  | 10-20 | 40 | 12 | 16QAM | 3/4 | 16992 | 24 | 3 | 23040 | 5760 | ≥ 2 |
|  | 10-20 | 45 | 12 | 16QAM | 3/4 | 19080 | 24 | 4 | 25920 | 6480 | ≥ 2 |
|  | 10-20 | 48 | 12 | 16QAM | 3/4 | 20616 | 24 | 4 | 27648 | 6912 | ≥ 2 |
|  | 15 - 20 | 50 | 12 | 16QAM | 3/4 | 21384 | 24 | 4 | 28800 | 7200 | ≥ 2 |
|  | 15 - 20 | 54 | 12 | 16QAM | 3/4 | 22920 | 24 | 4 | 31104 | 7776 | ≥ 2 |
|  | 15 - 20 | 60 | 12 | 16QAM | 2/3 | 23688 | 24 | 4 | 34560 | 8640 | ≥ 2 |
|  | 15 - 20 | 64 | 12 | 16QAM | 2/3 | 25456 | 24 | 4 | 36864 | 9216 | ≥ 2 |
|  | 15 - 20 | 72 | 12 | 16QAM | 1/2 | 20616 | 24 | 4 | 41472 | 10368 | ≥ 2 |
|  | 20 | 75 | 12 | 16QAM | 1/2 | 21384 | 24 | 4 | 43200 | 10800 | ≥ 2 |
|  | 20 | 80 | 12 | 16QAM | 1/2 | 22920 | 24 | 4 | 46080 | 11520 | ≥ 2 |
|  | 20 | 81 | 12 | 16QAM | 1/2 | 22920 | 24 | 4 | 46656 | 11664 | ≥ 2 |
|  | 20 | 90 | 12 | 16QAM | 2/5 | 20616 | 24 | 4 | 51840 | 12960 | ≥ 2 |
|  | 20 | 96 | 12 | 16QAM | 2/5 | 22152 | 24 | 4 | 55296 | 13824 | ≥ 2 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.2.2-1a: Reference Channels for 16-QAM with partial RB allocation for UE category 0

| Parameter | Ch BW | Allocated RBs | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size | Transport block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame | Total symbols per Sub-Frame | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 12 | 16QAM | 3/4 | 408 | 24 | 1 | 576 | 144 | 0 |
|  | 1.4 - 20 | 2 | 12 | 16QAM | 3/4 | 840 | 24 | 1 | 1152 | 288 | 0 |
|  | 1.4 - 20 | 4 | 12 | 16QAM | 2/5 | 904 | 24 | 1 | 2304 | 576 | 0 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th, 6th, 12th, 13th, 14th, 20th, 21st, 22nd, 28th, 29th, 30th, 36th, 37th, and 38th subframes every 40ms. Information bit payload is available if uplink subframe is scheduled. |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.2.2-1b: Reference Channels for 16-QAM with partial RB allocation for UE category M1

| Parameter | Ch BW | Allocated RBs | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size | Transport block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame | Total symbols per Sub-Frame | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 12 | 16QAM | 1/2 | 256 | 24 | 1 | 576 | 144 | M1 |
|  | 1.4 - 20 | 2 | 12 | 16QAM | 1/2 | 552 | 24 | 1 | 1152 | 288 | M1 |
|  | 1.4 - 20 | 3 | 12 | 16QAM | 1/2 | 840 | 24 | 1 | 1728 | 432 | M1 |
|  | 1.4 - 20 | 4 | 12 | 16QAM | 2/5 | 904 | 24 | 1 | 2304 | 576 | M1 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th and 6thsubframes every 10ms for the channel bandwidth 5MHz/10MHz/15MHz/20MHz. For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th, and 7th subframes every 10ms for the channel bandwidth 1.4MHz/3MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.2.2-1c: Reference Channels for 16-QAM with partial RB allocation for UE UL category M2

| Parameter | Ch BW | Allocated RBs | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size | Transport block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame | Total symbols per Sub-Frame | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 12 | 16QAM | 1/2 | 256 | 24 | 1 | 576 | 144 | M2 |
|  | 1.4 - 20 | 2 | 12 | 16QAM | 1/2 | 552 | 24 | 1 | 1152 | 288 | M2 |
|  | 1.4 - 20 | 3 | 12 | 16QAM | 1/2 | 840 | 24 | 1 | 1728 | 432 | M2 |
|  | 1.4 - 20 | 4 | 12 | 16QAM | 1/2 | 1128 | 24 | 1 | 2304 | 576 | M2 |
|  | 1.4 - 20 | 5 | 12 | 16QAM | 1/2 | 1416 | 24 | 1 | 2880 | 720 | M2 |
|  | 3 - 20 | 6 | 12 | 16QAM | 1/2 | 1736 | 24 | 1 | 3456 | 864 | M2 |
|  | 3 - 20 | 9 | 12 | 16QAM | 1/2 | 2600 | 24 | 1 | 5184 | 1296 | M2 |
|  | 3 - 20 | 12 | 12 | 16QAM | 1/2 | 3496 | 24 | 1 | 6912 | 1728 | M2 |
|  | 5 - 20 | 15 | 12 | 16QAM | 1/2 | 4264 | 24 | 1 | 8640 | 2160 | M2 |
|  | 5 - 20 | 18 | 12 | 16QAM | 1/2 | 5160 | 24 | 1 | 10368 | 2592 | M2 |
|  | 5 - 20 | 21 | 12 | 16QAM | 1/2 | 5992 | 24 | 1 | 12096 | 3024 | M2 |
|  | 5 - 20 | 24 | 12 | 16QAM | 1/2 | 6968 | 24 | 2 | 13824 | 3456 | M2 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th and 6th subframes every 10ms for the channel bandwidth 5MHz/10MHz/15MHz/20MHz. For HD-FDD , the uplink subframes are scheduled at the 5th, 6th, and 7th subframes every 10msfor the channel bandwidth 1.4MHz/3MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |  |  |  |  |  |  |  |  |  |

A.2.2.3	[FFS]

A.2.2.4	subPRB allocation

The location of allocated RB for subPRB allocation is chosen according to values specified in the Tx requirements.

Table A.2.2.4-1: Reference Channels for SubPRB allocation

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 1.4 | 1.4 |
| Allocated resource blocks |  | 1 | 1 | 1 |
| Number of subcarriers |  | 2 out of 3 | 3 | 6 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 |
| Modulation |  | π/2 BPSK | QPSK | QPSK |
| Target Coding rate |  | 1/3 | 1/3 | 1/3 |
| Payload size | Bits | 32 | 72 | 72 |
| Transport block CRC | Bits | 24 | 24 | 24 |
| Number of code blocks |  | 1 | 1 | 1 |
| Total number of bits per resource unit | Bits | 192 | 288 | 288 |
| Total symbols per resource unit |  | 192 | 144 | 144 |
| Tx time | ms | 8 | 4 | 2 |
| UE UL Category |  | M1/M2 | M1/M2 | M1/M2 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |

A.2.3	[FFS]

A.2.4	Reference measurement channels for category NB1

Table A.2.4-1 Reference Channels for category NB1

| Parameter | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 3.75 | 3.75 | 15 | 15 | 15 | 15 | 15 |
| Number of tone | 1 | 1 | 1 | 1 | 3 | 6 | 12 |
| Modulation | π/2 BPSK | π/4 QPSK | π/2 BPSK | π/4 QPSK | QPSK | QPSK | QPSK |
| Number of NPUSCH repetition (NOTE 5) | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| IMCS / ITBS | 0 / 0 | 3 / 3 | 0 / 0 | 3 / 3 | 5 / 5 | 5 / 5 | 5 / 5 |
| Payload size (bits) | 32 | 40 | 32 | 40 | 72 | 72 | 72 |
| Allocated resource unit | 2 | 1 | 2 | 1 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.33 | 0.29 | 0.33 | 0.33 | 0.33 | 0.33 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 192 | 96 | 192 | 288 | 288 | 288 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 144 | 144 | 144 |
| Tx time (ms) | 64 | 32 | 16 | 8 | 4 | 2 | 1 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: Parameters related to NPUSCH format 1 scheduling are defined in Table A.2.4-2. NOTE 3: NPDCCH is not transmitted in the subframes used for transmission of SI messages. NOTE 4: SI messages transmission should be prioritized over NPDCCH transmission in case of collision. NPDCCH transmission is postponed until the next NB-IoT downlink subframe in case NPDCCH transmission occurs in a non NB-IoT downlink subframe, where an NB-IoT downlink subframe is a subframe that does not contain NPSS/NSSS/NPBCH/SIB1-NB transmission. NOTE 5: Number of repetition NRep as defined in table 16.5.1.1-3 in TS 36.213 [20]. |  |  |  |  |  |  |  |

Following NOTEs are included in TS 36.521-1 as an addition to TS 36.101, to define examples of UL scheduling patterns based on Table A.2.4-1 and Table A.2.4-2. Overall period of UL scheduling pattern corresponds with Si-periodicity and it consists of two different patterns with dependencies on SI transmission.

-	One pattern applies for period containing SI messages transmission. This pattern has a period with same duration as SI windows length (time period where SI messages are transmitted).

-	Another pattern for period where there is no transmission of SI messages. This pattern is repeated during SI periodicity.

Figure A.2.4-1: NPDCCH scheduling based on SI transmission

NOTE 1: For 3.75kHz, 1 tone, and π/2 BPSK modulation. NPDCCH transmission is scheduled at the 18th subframe (starting from the 0th subframe) every 80ms and its corresponding NPUSCH format 1 is scheduled 9 subframes after NPDCCH transmission. NPUSCH format 1 transmission has a duration of 64 consecutive ms. This pattern applies for both periods containing and not containing SI messages transmission.

NOTE 2: For 3.75kHz, 1 tone, and π/4 QPSK modulation. NPDCCH transmissions are scheduled at the 18th, 66th and 114th subframe (starting from the 0th subframe) every 160ms for periods containing SI messages transmission and at the 2nd, 51th, 98th, 146th and 194th subframe (starting from the 0th subframe) every 240ms for periods where there is no transmission of SI messages. The corresponding NPUSCH format 1 are scheduled 9 subframes after each NPDCCH transmission. NPUSCH format 1 transmission has a duration of 32 consecutive ms.

NOTE 3: For 15kHz, 1 tone, and π/2 BPSK modulation. NPDCCH transmissions are scheduled at the 18th, 51th, 98th and 131th subframe (starting from the 0th subframe) every 160ms for periods containing SI messages transmission and at the 2nd, 34th, 66th, 98th and 131th subframe (starting from the 0th subframe) every 160ms for periods where there is no transmission of SI messages. The corresponding NPUSCH format 1 are scheduled 9 subframes after each NPDCCH transmission. NPUSCH format 1 transmission has a duration of 16 consecutive ms.

NOTE 4: For 15kHz, 1 tone, and π/4 QPSK modulation. NPDCCH transmissions are scheduled at the 18th, 42th, 66th, 98th, 122th and 146th subframe (starting from the 0th subframe) every 160ms for periods containing SI messages transmission and at the 11th, 34th, 58th, 82th and 106th subframe (starting from the 0th subframe) every 120ms for periods where there is no transmission of SI messages. The corresponding NPUSCH format 1 are scheduled 9 subframes after each NPDCCH transmission. NPUSCH format 1 transmission has a duration of 8 consecutive ms.

NOTE 5: For 15kHz, 3 tones, and QPSK modulation. NPDCCH transmissions are scheduled at the 18th, 34th, 51th, 74th, 98th, 114th, 131th and 154th subframe (starting from the 0th subframe) every 160ms for periods containing SI messages transmission and at the 11th and 34th subframe (starting from the 0th subframe) every 40ms for periods where there is no transmission of SI messages. The corresponding NPUSCH format 1 are scheduled 9 subframes after each NPDCCH transmission. NPUSCH format 1 transmission has a duration of 4 consecutive ms.

NOTE 6: For 15kHz, 6 tones, and QPSK modulation. NPDCCH transmissions are scheduled at the 18th, 34th, 51th, 66th, 98th, 114th, 131th and 146th subframe (starting from the 0th subframe) every 160ms for periods containing SI messages transmission and at the 2nd, 18th, 34th, 51th and 66th subframe (starting from the 0th subframe) every 80ms for periods where there is no transmission of SI messages. The corresponding NPUSCH format 1 are scheduled 9 subframes after each NPDCCH transmission. NPUSCH format 1 transmission has a duration of 2 consecutive ms.

NOTE 7: For 15kHz, 12 tones, and QPSK modulation. NPDCCH transmissions are scheduled at the 18th, 34th, 51th, 66th, 98th, 114th, 131th and 146th subframe (starting from the 0th subframe) every 160ms for periods containing SI messages transmission and at the 2nd, 18th, 34th, 51th and 66th subframe (starting from the 0th subframe) every 80ms for periods where there is no transmission of SI messages. The corresponding NPUSCH format 1 are scheduled 9 subframes after each NPDCCH transmission. NPUSCH format 1 transmission has a duration of 1 ms.

Table A.2.4-2: NPDCCH configuration for NPUSCH format 1 scheduling

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N0 |
| NPDCCH format |  | 1 |
| Scheduling delay () |  | 0 |
| DCI subframe repetition number |  | 00 |
| (npdcch-NumRepetitions) |  | 1 |
| G (NPDCCH-startSF-USS) |  | 8 |
| (npdcch-Offset-USS) |  | 1/4 |

A.2.5	[FFS]

A.3	DL reference measurement channels

A.3.1	General

The number of available channel bits varies across the sub-frames due to PBCH and PSS/SSS overhead. The payload size per sub-frame is varied in order to keep the code rate constant throughout a frame.

No user data is scheduled on subframes #5 in order to facilitate the transmission of system information blocks (SIB).

The algorithm for determining the payload size A is as follows; given a desired coding rate R and radio block allocation NRB

1.	Calculate the number of channel bits Nch that can be transmitted during the first transmission of a given sub-frame.

2.	Find A such that the resulting coding rate is as close to R as possible, that is,

,

subject to

a) A is a valid TB size (according to TS 36.213 [10] clause 7.1.7) assuming an allocation of NRB resource blocks

b) C is the number of Code Blocks calculated according to section 5.1.2 of TS 36.212 [19].

3.	If there is more than one A that minimizes the equation above, then the larger value is chosen per default and the chosen code rate should not exceed 0.93.

4.	For TDD, the measurement channel is based on DL/UL configuration ratio of 2DL+DwPTS (12 OFDM symbol): 2UL.

A.3.1.1	Overview of DL reference measurement channels

In Table A.3.1.1-1 are listed the DL reference measurement channels specified in Annexes A.3.2 to A.3.17 of this release of TS 36.521-1. This table is informative and serves only to a better overview. The reference for the concrete reference measurement channels and corresponding implementation’s parameters as to be used for testing are annexes A.3.2 to A.3.17 as appropriate.

Table A.3.1.1-1: Overview of DL reference measurement channels

| Duplex | Table | Name | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD, Receiver requirements |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-1 |  | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD / HD-FDD | Table A.3.2-1a |  | 1.4 | QPSK | 1/3 | 6 |  | 0 |  |
| FDD / HD-FDD | Table A.3.2-1b |  | 1.4 | QPSK | 1/3 | 4 |  | M1 |  |
| HD-FDD | Table A.3.2-1c |  | 0.2 | QPSK | 1/3 |  |  | NB1 |  |
| HD-FDD | Table A.3.2-1d |  | 0.2 | QPSK | 1/3 |  |  | NB1 |  |
| FDD / HD-FDD | Table A.3.2-1h |  | 1.4 | QPSK | 1/3 | 4 |  | M2 |  |
| FDD, Receiver requirements, Maximum input level for UE Categories 3-5 |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-3 |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| FDD, Receiver requirements, Maximum input level for UE Categories 1 |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-3a |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| FDD, Receiver requirements, Maximum input level for UE Categories 2 |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-3b |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| FDD, Receiver requirements, Maximum input level for UE Categories 0 |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-3c |  | 1.4 | 64QAM | 3/4 | 2 |  | - |  |
| FDD, Receiver requirements, Maximum input level for UE Categories M1 |  |  |  |  |  |  |  |  |  |
| FDD/HD-FDD | Table A.3.2-3d |  | 1.4 | 16QAM | 3/5 | 2 |  | - |  |
| FDD, Receiver requirements, Maximum input level for UE Categories M2 |  |  |  |  |  |  |  |  |  |
| FDD/HD-FDD | Table A.3.2-3e |  | 1.4 | 16QAM | 3/5 | 2 |  | - |  |
| FDD, Receiver requirements, Maximum input level for UE Categories 11/12/13/14/15 |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-5 |  | 1.4 | 256QAM | 4/5 | 6 |  | - |  |
| FDD, Transmitter requirements |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2A-1 |  | 1.4 | QPSK | 1/8-1/3 | 3 |  | ≥ 1 |  |
| FDD | Table A.3.2A-1a |  | 1.4 | QPSK | 1/3 | 4 |  | M1 |  |
| FDD, PDSCH Performance, Single-antenna transmission (CRS) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.1-1 | R.4 FDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD, PDSCH Performance, Single-antenna transmission (CRS), Single PRB (Channel edge) |  |  |  |  |  |  |  |  |  |
| FDD | FFS | FFS | FFS | FFS | FFS | FFS |  | FFS |  |
| FDD, PDSCH Performance (UE specific RS) without CSI-RS |  |  |  |  |  |  |  |  |  |
| FDD | FFS | FFS | FFS | FFS | FFS | FFS |  | FFS |  |
| FDD, PDCCH / PCFICH Performance |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.5.1-1 | R.16 FDD | 1.4 | PDCCH |  |  |  |  |  |
| FDD | Table A.3.5.1-2 |  | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD / TDD, PHICH Performance |  |  |  |  |  |  |  |  |  |
| FDD / TDD | Table A.3.6-1 | R.19A | 1.4 | PHICH |  |  |  |  |  |
| FDD, PMCH Performance |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.8.1-1 | R.40 FDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD, Sustained data rate (CRS) |  |  |  |  |  |  |  |  |  |
| FDD | FFS | FFS | FFS | FFS | FFS |  |  | FFS |  |
| FDD, Sustained data rate test with EPDCCH scheduling (CRS) |  |  |  |  |  |  |  |  |  |
| FDD | FFS | FFS | FFS | FFS | FFS |  |  | FFS |  |
| FDD, EPDCCH Performance |  |  |  |  |  |  |  |  |  |
| FDD | FFS | FFS | FFS | EPDCCH |  |  |  |  |  |
| FDD, MPDCCH performance |  |  |  |  |  |  |  |  |  |
| FDD | FFS | FFS | FFS | MPDCCH |  |  |  |  |  |
| FDD, Slot-PDSCH/Subslot-PDSCH |  |  |  |  |  |  |  |  |  |
| FDD | FFS | FFS | FFS | FFS | FFS | FFS |  | FFS | Slot-PDSCH |
| FDD | FFS | FFS | FFS | FFS | FFS | FFS |  | FFS | Subslot-PDSCH |
| FDD, SPDCCH Performance |  |  |  |  |  |  |  |  |  |
| FDD | FFS | FFS | FFS | SPDCCH |  |  |  |  |  |
| FDD | FFS | FFS | FFS | SPDCCH |  |  |  |  |  |

A.3.2	Reference measurement channel for receiver characteristics

Unless otherwise stated, Tables A.3.2-1, A.3.2-1a, A.3.2-1b, A.3.2-2, A.3.2-2a and A.3.2-2b are applicable for measurements on the Receiver Characteristics (clause 7) with the exception of subclause 7.4 (Maximum input level).

Unless otherwise stated, Tables A.3.2-3, A.3.2-3a, A.3.2-3b, A.3.2-4, A.3.2-4a and A.3.2-4b are applicable for subclause 7.4 (Maximum input level).

Unless otherwise stated, Tables A.3.2-1, A.3.2-1a, A.3.2-1b, A.3.2-2, A.3.2-2a and A.3.2-2b also apply for the modulated interferer used in Clauses 7.5, 7.6 and 7.8 with test specific bandwidths.

Table A.3.2-1: Fixed Reference Channel for Receiver Requirements (FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 6 |
| Subcarriers per resource block |  | 12 |
| Allocated subframes per Radio Frame |  | 9 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of HARQ Processes | Processes | 8 |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload per Sub-Frame |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 408 |
| For Sub-Frame 5 | Bits | n/a |
| For Sub-Frame 0 | Bits | 152 |
| Transport block CRC | Bits | 24 |
| Number of Code Blocks per Sub-Frame (Note 3) |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 1 |
| For Sub-Frame 5 | Bits | n/a |
| For Sub-Frame 0 | Bits | 1 |
| Binary Channel Bits Per Sub-Frame |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 1368 |
| For Sub-Frame 5 | Bits | n/a |
| For Sub-Frame 0 | Bits | 528 |
| Max. Throughput averaged over 1 frame | kbps | 341.6 |
| UE Category |  | ≥ 1 |
| Note 1: 4 symbols allocated to PDCCH for 1.4 MHz Note 2: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [3] Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |

Table A.3.2-1a: Fixed Reference Channel for Receiver Requirements for UE Category 0 (FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 6 |
| Subcarriers per resource block |  | 12 |
| Allocated subframes per Radio Frame |  | 9 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of HARQ Processes | Processes | [8] |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload per Sub-Frame |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 408 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 (Note 3) | Bits | 152 |
| Transport block CRC | Bits | 24 |
| Number of Code Blocks per Sub-Frame |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 1 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 1 |
| Binary Channel Bits Per Sub-Frame |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 1368 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 (Note 3) | Bits | 528 |
| Max. Throughput averaged over 1 frame | kbps | 341.6 |
| UE DL Category |  | 0 |
| Note 1: 4 symbols allocated to PDCCH for 1.4 MHz. Note 2: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [3]. Note 3: For Sub-Frame 0, it is assumed the 6PRBs are allocated in the centre of the channel where some REs of the same PRBs are occupied by PBCH and synchronization signals. Note 4: For HD-FDD UE, the downlink subframes are scheduled at the 0th, 1st, 2nd, 8th, 9th, 10th, 16th, 17th, 18th, 24th, 25th, 26th, 32nd, 33rd, 34th subframes every 40ms. Information bit payload is available if downlink subframe is scheduled. |  |  |

Table A.3.2-1b: Fixed Reference Channel for Receiver Requirements (FDD) for UE Category M1

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 4 |
| Subcarriers per resource block |  | 12 |
| Allocated subframes per Radio Frame |  | 2 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of HARQ Processes | Processes | 8 |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload per Sub-Frame |  |  |
| For Sub-Frames 3, 8 | Bits | 256 |
| For Sub-Frames 0,1,2,5,7,9 | Bits | N/A |
| For Sub-Frame 4 | Bits | N/A |
| For Sub-Frame 6 | Bits | N/A |
| Transport block CRC | Bits | 24 |
| Number of Code Blocks per Sub-Frame |  |  |
| For Sub-Frames 3, 8 | Bits | 1 |
| For Sub-Frames 0,1,2,5,7,9 | Bits | N/A |
| For Sub-Frame 4 | Bits | N/A |
| For Sub-Frame 6 | Bits | N/A |
| Binary Channel Bits Per Sub-Frame |  |  |
| For Sub-Frames 3, 8 | Bits | 912 |
| For Sub-Frames 0,1,2,5,7,9 | Bits | N/A |
| For Sub-Frame 4 | Bits | N/A |
| For Sub-Frame 6 | Bits | N/A |
| Max. Throughput averaged over 1 frame for FDD | kbps | 51.2 |
| Max. Throughput averaged over 1 frames for HD-FDD | kbps | 25.6 |
| UE DL Category |  | M1 |
| Note 1: 4 symbols allocated to PDCCH for 1.4 MHz Note 2: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [3]. Note 3: For HD-FDD UE, PDSCH are scheduled at the 3rd subframe every 1 radio frame for 1.4MHz channel BW. Information bit payload is available if downlink subframe is scheduled. The corresponding M-PDCCH is scheduled 2 subframes before the corresponding PDSCH transmission. Note 4: 2 resource blocks allocated to M-PDCCH |  |  |

Table A.3.2-1c: Fixed Reference Channel for Receiver Requirements (HD-FDD) without repetition – for CAT-NB1

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 0.2 |
| Number of subcarriers |  | 12 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of HARQ Processes | Processes | 1 |
| Maximum number of HARQ transmissions |  | 1 |
| Transport block size | Bits | 88 |
| Number of Sub-Frames per transport block |  | 1 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits Per Sub-Frame | Bits | 320 |
| LTE CRS port |  | N/A |
| Number of NRS ports |  | 1 |
| Number of NPDSCH repetitions |  | 0 |
| UE DL Category |  | NB1 |
| Note 1: NB-IoT in stand-alone mode has been considered here Note 2: Reference signal, Synchronization signals and NPBCH allocated as per TS 36.211 [3]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 4: Parameters related to NPDSCH scheduling are defined in Table A.3.2-1e to Table A.3.2-1g. Note 5: NPDCCH and information bit payload are not transmitted in the subframes used for transmission of SI messages. Note 6: SI messages transmission should be prioritized over NPDCCH transmission in case of collision. NPDCCH transmission is postponed until the next NB-IoT downlink subframe in case NPDCCH transmission occurs in a non NB-IoT downlink subframe, where an NB-IoT downlink subframe is a subframe that does not contain NPSS/NSSS/NPBCH/SIB1-NB transmission. |  |  |

Table A.3.2-1ca Example scheduling pattern with SI periodicity of 64 radio frames for Receiver Requirements (HD-FDD) without repetition – for CAT-NB1

| Subframe th from the 0th subframe | NPDCCH | NPDSCH | NPUSCH |
| --- | --- | --- | --- |
|  | 18, 42, 66, 98, 122, 146, 171, 194, 218, 242, 266, 291, 314, 338, 362, 386, 411, 434, 458, 482, 506, 531, 554, 578, 602, 626 | +5 from corresponding NPDCCH | +13 and +14 from corresponding NPDSCH. NPUSCH transmission occupies 2 subframes |

Table A.3.2-1d: Fixed Reference Channel for Receiver Requirements (HD-FDD) with repetition – for CAT-NB1

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 0.2 |
| Number of subcarriers |  | 12 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of HARQ Processes | Processes | 1 |
| Maximum number of HARQ transmissions |  | 1 |
| Transport block size | Bits | 88 |
| Number of Sub-Frames per transport block |  | 1 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits Per Sub-Frame | Bits | 320 |
| LTE CRS port |  | N/A |
| Number of NRS ports |  | 1 |
| Number of NPDSCH repetitions |  | TBD |
| UE DL Category |  | NB1 |
| Note 1: NB-IoT in stand-alone mode has been considered here Note 2: Reference signal, Synchronization signals and NPBCH allocated as per TS 36.211 [3]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 4: Parameters related to NPDSCH scheduling are defined in Table A.3.2-1e to Table A.3.2-1g. |  |  |

Table A.3.2-1e: General configuration for CAT-NB1

| Parameter | Unit | Value |
| --- | --- | --- |
| NB-IoT downlink subframe bitmap for anchor carrier (downlinkBitmap) |  | Not configured |
| NB-IoT downlink subframe bitmap for non-anchor carrier (downlinkBitmapNonAnchor) |  | Not configured |
| Downlink gap configuration for anchor carrier (dl-Gap) |  | Not configured |
| Downlink gap configuration for non-anchor carrier (dl-GapNonAnchor) |  | Not configured |

Table A.3.2-1f: NPDCCH configuration for NPDSCH scheduling

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N1 |
| NPDCCH format |  | 1 |
| Scheduling delay () |  | 0 |
| DCI subframe repetition number |  | 00 |
| (npdcch-NumRepetitions) |  | 1 |
| G (NPDCCH-startSF-USS) |  | 8 |
| (npdcch-Offset-USS) |  | 1/4 |

Table A.3.2-1g: NPUSCH format 2 configurations for NPDSCH scheduling

| Parameter | Unit | Value |
| --- | --- | --- |
| Scheduling delay () |  | 0 |
| (ack-NACK-NumRepetitions) |  | 1 |
| ACK/NACK resource field |  | 0 |

Table A.3.2-1h: Fixed Reference Channel for Receiver Requirements (FDD and HD-FDD) – for CAT-M2

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |  |  |  |  |  |
| Allocated resource blocks (Note 5) |  | 4 |  |  |  |  |  |
| Subcarriers per resource block |  | 12 |  |  |  |  |  |
| Allocated subframes per Radio Frame (Note 3) |  | 2 |  |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |  |
| Target Coding Rate |  | 1/3 |  |  |  |  |  |
| Number of HARQ Processes | Processes | 8 |  |  |  |  |  |
| Maximum number of HARQ transmissions |  | 1 |  |  |  |  |  |
| Information Bit Payload per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3,8 | Bits | 256 |  |  |  |  |  |
| For Sub-Frames 0,1,2,5,7,9 | Bits | N/A |  |  |  |  |  |
| For Sub-Frame 4 | Bits | N/A |  |  |  |  |  |
| For Sub-Frame 6 | Bits | N/A |  |  |  |  |  |
| Transport block CRC | Bits | 24 |  |  |  |  |  |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3,8 | Bits | 1 |  |  |  |  |  |
| For Sub-Frames 0,1,2,5,7,9 | Bits | N/A |  |  |  |  |  |
| For Sub-Frame 4 | Bits | N/A |  |  |  |  |  |
| For Sub-Frame 6 | Bits | N/A |  |  |  |  |  |
| Binary Channel Bits Per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3,8 | Bits | 912 |  |  |  |  |  |
| For Sub-Frames 0,1,2,5,7,9 | Bits | N/A |  |  |  |  |  |
| For Sub-Frame 4 | Bits | M |  |  |  |  |  |
| For Sub-Frame 6 | Bits | N/A |  |  |  |  |  |  |
| Max. Throughput averaged over 1 frame for FDD | kbps | 51.2 |  |  |  |  |  |
| Max. Throughput averaged over 1 frames for HD-FDD | kbps | 25.6 |  |  |  |  |  |
| UE DL Category |  | M2 |  |  |  |  |  |
| Note 1: 4 symbols allocated to PDCCH for 1.4 MHz Note 2: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [3]. Note 3: For HD-FDD UE, PDSCH are scheduled at the 3rd subframe every 1 radio frame for 1.4MHz channel bandwidth. Information bit payload is available if downlink subframe is scheduled. The corresponding MPDCCH is scheduled 2 subframes before the corresponding PDSCH transmission. Note 4: 2 resource blocks allocated to MPDCCH. Note 5: 4 resource blocks in each narrowband allocated to PDSCH. |  |  |  |  |  |  |  |

Table A.3.2-3: Fixed Reference Channel for Maximum input level for UE Categories 3-8 (FDD)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |  |
| Allocated resource blocks |  | 6 |  |
| Subcarriers per resource block |  | 12 |  |
| Allocated subframes per Radio Frame |  | 8 |  |
| Modulation |  | 64QAM |  |
| Target Coding Rate |  | 3/4 |  |
| Number of HARQ Processes | Processes | 8 |  |
| Maximum number of HARQ transmissions |  | 1 |  |
| Information Bit Payload per Sub-Frame |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 2984 |  |
| For Sub-Frame 5 | Bits | n/a |  |
| For Sub-Frame 0 | Bits | n/a |  |
| Transport block CRC | Bits | 24 |  |
| Number of Code Blocks per Sub-Frame (Note 3) |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  | 1 |  |
| For Sub-Frame 5 |  | n/a |  |
| For Sub-Frame 0 |  | n/a |  |
| Binary Channel Bits Per Sub-Frame |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 4104 |  |
| For Sub-Frame 5 | Bits | n/a |  |
| For Sub-Frame 0 | Bits | n/a |  |
| Max. Throughput averaged over 1 frame | kbps | 2387.2 |  |
| Note 1: 4 symbols allocated to PDCCH for 1.4 MHz Note 2: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [3] Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |

Table A.3.2-3a: Fixed Reference Channel for Maximum input level for UE Category 1 (FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 6 |
| Subcarriers per resource block |  | 12 |
| Allocated subframes per Radio Frame |  | 8 |
| Modulation |  | 64QAM |
| Target Coding Rate |  | 3/4 |
| Number of HARQ Processes | Processes | 8 |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 2984 |
| For Sub-Frame 5 | Bits | n/a |
| For Sub-Frame 0 | Bits | n/a |
| Transport block CRC | Bits | 24 |
| Number of Code Blocks per Sub-Frame (Note 3) |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  | 1 |
| For Sub-Frame 5 |  | n/a |
| For Sub-Frame 0 |  | n/a |
| Binary Channel Bits Per Sub-Frame |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 4104 |
| For Sub-Frame 5 | Bits | n/a |
| For Sub-Frame 0 | Bits | n/a |
| Max. Throughput averaged over 1 frame | kbps | 2387.2 |
| Note 1: 4 symbols allocated to PDCCH for 1.4 MHz Note 2: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [3] Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |

Table A.3.2-3b: Fixed Reference Channel for Maximum input level for UE Category 2 (FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 6 |
| Subcarriers per resource block |  | 12 |
| Allocated subframes per Radio Frame |  | 8 |
| Modulation |  | 64QAM |
| Target Coding Rate |  | 3/4 |
| Number of HARQ Processes | Processes | 8 |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 2984 |
| For Sub-Frame 5 | Bits | n/a |
| For Sub-Frame 0 | Bits | n/a |
| Transport block CRC | Bits | 24 |
| Number of Code Blocks per Sub-Frame (Note 3) |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  | 1 |
| For Sub-Frame 5 |  | n/a |
| For Sub-Frame 0 |  | n/a |
| Binary Channel Bits Per Sub-Frame |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 4104 |
| For Sub-Frame 5 | Bits | n/a |
| For Sub-Frame 0 | Bits | n/a |
| Max. Throughput averaged over 1 frame | kbps | 2387.2 |
| Note 1: 4 symbols allocated to PDCCH for 1.4 MHz Note 2: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [3] Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |

Table A.3.2-3c: Fixed Reference Channel for Maximum input level for UE DL Category 0 (FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 2 |
| Subcarriers per resource block |  | 12 |
| Allocated subframes per Radio Frame |  | 8 |
| Modulation |  | 64QAM |
| Target Coding Rate |  | 3/4 |
| Number of HARQ Processes | Processes | [8] |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 1000 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 (Note 3) | Bits | N/A |
| Transport block CRC | Bits | 24 |
| Number of Code Blocks per Sub-Frame |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  | 1 |
| For Sub-Frame 5 |  | N/A |
| For Sub-Frame 0 |  | N/A |
| Binary Channel Bits Per Sub-Frame |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 1368 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 (Note 3) | Bits | N/A |
| Max. Throughput averaged over 1 frame | kbps | 800 |
| Note 1: 4 symbols allocated to PDCCH for 1.4 MHz. Note 2: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [3]. Note 3: For Sub-Frame 0, it is assumed that the allocated 2PRBs are scheduled on the RBs other than the centre 6PRBs as most of the symbols are occupied by PBCH and synchronization signals. |  |  |

Table A.3.2-3d: Fixed Reference Channel for Maximum input level for UE DL Category M1 (FDD and HD-FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 2 |
| Subcarriers per resource block |  | 12 |
| Allocated subframes per Radio Frame |  | 2 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 3/5 |
| Number of HARQ Processes | Processes | 8 |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload |  |  |
| For Sub-Frames 3,8 | Bits | 552 |
| For Sub-Frames 0,1,2,5,7,9 | Bits | N/A |
| For Sub-Frame 4 | Bits | N/A |
| For Sub-Frame 6 | Bits | N/A |
| Transport block CRC | Bits | 24 |
| Number of Code Blocks per Sub-Frame |  |  |
| For Sub-Frames 3, 8 |  | 1 |
| For Sub-Frames 0,1,2,5,7,9 |  | N/A |
| For Sub-Frame 4 |  | N/A |
| For Sub-Frame 6 |  | N/A |
| Binary Channel Bits Per Sub-Frame |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 832 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | N/A |
| Max. Throughput averaged over 1 frame for FDD | kbps | 110.4 |
| Max. Throughput averaged over 1 frame for HD-FDD |  | 55.2 |
| Note 1: 4 symbols allocated to PDCCH for all channel bandwidths. Note 2: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [3]. Note 3: For HD-FDD UE, PDSCH are scheduled at the 3rd subframe every 1 radio frame for 1.4MHz. Information bit payload is available if downlink subframe is scheduled. The corresponding MPDCCH is scheduled 2 subframes before the corresponding PDSCH transmission. Note 4: 2 resource blocks allocated to MPDCCH. |  |  |

Table A.3.2-3e: Fixed Reference Channel for Maximum input level for UE DL Category M2 (FDD and HD-FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks (Note 5) |  | 2 |
| Subcarriers per resource block |  | 12 |
| Allocated subframes per Radio Frame (Note 3) |  | 2 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 3/5 |
| Number of HARQ Processes | Processes | 8 |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload |  |  |
| For Sub-Frames 3, 8 | Bits | 552 |
| For Sub-Frames 0,1,2,5,7,9 | Bits | N/A |
| For Sub-Frame 4 | Bits | N/A |
| For Sub-Frame 6 | Bits | N/A |
| Transport block CRC | Bits | 24 |
| Number of Code Blocks per Sub-Frame |  |  |
| For Sub-Frames 3, 8 |  | 1 |
| For Sub-Frames 0,1,2,5,7,9 |  | N/A |
| For Sub-Frame 4 |  | N/A |
| For Sub-Frame 6 |  | N/A |
| Binary Channel Bits Per Sub-Frame |  |  |
| For Sub-Frames 3, 8 | Bits | 912 |
| For Sub-Frames 0,1,2,5,7,9 |  | N/A |
| For Sub-Frame 4 | Bits | N/A |
| For Sub-Frame 6 | Bits | N/A |
| Max. Throughput averaged over 1 frame for FDD | kbps | 110.4 |
| Max. Throughput averaged over 1 frame for HD-FDD |  | 55.2 |
| Note 1: 4 symbols allocated to PDCCH for 1.4MHz channel bandwidth. Note 2: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [3]. Note 3: For HD-FDD UE, PDSCH are scheduled at the 3rd subframe every 1 radio frame for 1.4MHz. Information bit payload is available if downlink subframe is scheduled. The corresponding MPDCCH is scheduled 2 subframes before the corresponding PDSCH transmission. Note 4: 2 resource blocks allocated to MPDCCH. Note 5: 2 resource blocks allocated to PDSCH for 1.4MHz channel bandwidth. |  |  |

A.3.2A	Downlink Reference measurement channel for TX characteristics

Tables A.3.2A-1 and A.3.2A-2 describes the reference measurement channels to be used on the downlink during  Transmitter Characteristics (clause 6) for FDD. The number of allocated resource blocks have been defined (partial allocation) to allow the transmission of PBCH, PSS/SSS and system information mapped on PDSCH.

Table A.3.2A-1: Fixed DL PDSCH Dedicated Reference Channel for TX Requirements (FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 3 |
| Subcarriers per resource block |  | 12 |
| Allocated subframes per Radio Frame |  | 10 |
| Modulation |  | QPSK |
| Target Coding Rate |  | (Note 4) |
| Number of HARQ Processes | Processes | 8 |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload |  |  |
| For Sub-Frames 0, 1, 2, 3, 4, 5, 6, 7, 8, 9 | Bits | 88 |
| Transport block CRC | Bits | 24 |
| Number of Code Blocks per Sub-Frame |  | 1 |
| Code block CRC size | Bits | 0 |
| Binary Channel Bits Per Sub-Frame |  |  |
| For Sub-Frames 1, 2, 3, 4, 6, 7, 8, 9 | Bits | 684 |
| For Sub-Frames 5 |  | 540 |
| For Sub-Frames 0 |  | 264 |
| Max. Throughput averaged over 1 frame | kbps | 88 |
| UE-Category |  | ≥ 1 |
| Note 1: 4 symbols allocated to PDCCH for 1.4 MHz Note 2: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [3] Note 3: The PDSCH shall be assigned to the UE under test with a set of allocated localized virtual resource blocks starting from one end of the channel. Note 4: To ensure constant transport block size in 1.4MHz, the code rate for subframes varies approx. within {1/8-1/3} |  |  |

Table A.3.2A-1a: Fixed DL PDSCH Dedicated Reference Channel for TX Requirements (FDD) for UE Category M1

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 4 |
| Subcarriers per resource block |  | 12 |
| Allocated subframes per Radio Frame |  | 2 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of HARQ Processes | Processes | [8] |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload per Sub-Frame |  |  |
| For Sub-Frames 3, 8 | Bits | 256 |
| For Sub-Frames 0,1,2,5,7,9 | Bits | N/A |
| For Sub-Frame 4 | Bits | N/A |
| For Sub-Frame 6 | Bits | N/A |
| Transport block CRC | Bits | 24 |
| Number of Code Blocks per Sub-Frame |  |  |
| For Sub-Frames 3, 8 | Bits | 1 |
| For Sub-Frames 0,1,2,5,7,9 | Bits | N/A |
| For Sub-Frame 4 | Bits | N/A |
| For Sub-Frame 6 | Bits | N/A |
| Binary Channel Bits Per Sub-Frame |  |  |
| For Sub-Frames 3, 8 | Bits | 912 |
| For Sub-Frames 0,1,2,5,7,9 | Bits | N/A |
| For Sub-Frame 4 | Bits | N/A |
| For Sub-Frame 6 | Bits | N/A |
| Max. Throughput averaged over 1 frame for FDD | kbps | 51.2 |
| Max. Throughput averaged over 1 frames for HD-FDD | kbps | 25.6 |
| UE DL Category |  | M1 |
| Note 1: 4 symbols allocated to PDCCH for 1.4 MHz Note 2: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [3]. Note 3: For HD-FDD UE, PDSCH are scheduled at the [3rd] subframe every 1 radio frame for 1.4MHz channel BW. Information bit payload is available if downlink subframe is scheduled. The corresponding M-PDCCH is scheduled 2 subframes before the corresponding PDSCH transmission. Note 3: 2 resource blocks allocated to M-PDCCH |  |  |

A.3.3	Reference measurement channel for PDSCH performance requirements (FDD)

A.3.3.1	Single-antenna transmission (Common Reference Symbols)

Table A.3.3.1-1: Fixed Reference Channel Single Antenna Port

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | R.1 FDD | R.2 FDD | R.3 FDD |
| Channel bandwidth | MHz | 1.4 | 1.4 | 1.4 |
| Allocated resource blocks |  | Note3 | 6 | 6 |
| Allocated DL subframes per Radio Frame |  | Note 4 | Note 5 | Note 6 |
| Modulation |  | 16QAM | QPSK | QPSK |
| Target Coding Rate |  | 1/2 | 1/3 | 1/10 |
| Information Bit Payload |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 744 | 504 | 152 |
| Number of Code Blocks |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Code blocks | 1 | 1 | 1 |
| Binary Channel Bits |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 1656 (Note 7, 8) | 1656 (Note 7, 8) | 1656 (Note 7, 8) |
| Max. Throughput averaged over one period | Kbps | 149 | 15.75 | 0.950 |
| UE DL Category |  | M1 | M1 | M1 |
| Note 1: 2 symbols are allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH are allocated as per TS 36.211 [3]. Note 3: 2 symbols are allocated to PDCCH. Note 4: The downlink subframes are scheduled at the 8th and 9th subframes every 10ms (starting from 0th subframe). Information bit payload is available from the 8th to 9th subframes. The corresponding MPDCCH is scheduled 2 subframes before the corresponding PDSCH transmissions. Note 5: PDSCH subframes are scheduled at the 10th to 17th subframes every period (32ms). Information bit payload is available from the 10th to 17th subframes with repetition. (Starting from the 0th subframe). The corresponding MPDCCH is scheduled from 1st to 8th subframe every 32ms (starting from 0th subframe). Note 6: PDSCH subframes are scheduled at the 96th to 159th subframes every period (160ms). Information bit payload is available at the 96th to 159th subframes with repetition. (Starting from the 0th subframe) The corresponding MPDCCH is scheduled from 31st to 94th subframe every 160ms (starting from 0th subframe). Note 7: MPDCCH, and PDSCH are dropped when overlapped with SIB1-BR, or SIB2 or SIB3. Note 8: MPDCCH, and PDSCH are punctured in overlapping Resource Elements (RE)s with PSS/SSS/PBCH. |  |  |  |  |

Table A.3.3.1-2: Fixed Reference Channel 16QAM R=1/2

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  |  |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  |  |
| Allocated subframes per Radio Frame |  |  |
| Modulation |  |  |
| Target Coding Rate |  |  |
| Information Bit Payload |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits |  |
| For Sub-Frame 5 | Bits |  |
| For Sub-Frame 0 | Bits |  |
| Number of Code Blocks per Sub-Frame (see Note 3) |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  |  |
| For Sub-Frame 5 |  |  |
| For Sub-Frame 0 |  |  |
| Binary Channel Bits Per Sub-Frame |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits |  |
| For Sub-Frame 5 | Bits |  |
| For Sub-Frame 0 | Bits |  |
| Max. Throughput averaged over 1 frame | Mbps |  |
| UE Category |  |  |
| Note 1: 4 symbols allocated to PDCCH for 1.4 MHz Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [3] Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |

Table A.3.3.1-4: Fixed Reference Channel Single PRB (Channel Edge)

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  |  |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  |  |
| Allocated subframes per Radio Frame |  |  |
| Modulation |  |  |
| Target Coding Rate |  |  |
| Information Bit Payload |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits |  |
| For Sub-Frame 5 | Bits |  |
| For Sub-Frame 0 | Bits |  |
| Number of Code Blocks per Sub-Frame (see Note 3) |  |  |
| Binary Channel Bits Per Sub-Frame |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits |  |
| For Sub-Frame 5 | Bits |  |
| For Sub-Frame 0 | Bits |  |
| Max. Throughput averaged over 1 frame | Mbps |  |
| UE Category |  |  |
| Note 1: 4 symbols allocated to PDCCH for 1.4 MHz Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [3] Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |

Table A.3.3.1-5: FFS

Table A.3.3.1-6: Fixed Reference Channel QPSK R=1/10

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  |  |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks (Note 3) |  |  |
| Allocated subframes per Radio Frame |  |  |
| Modulation |  |  |
| Target Coding Rate |  |  |
| Information Bit Payload |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits |  |
| For Sub-Frame 5 | Bits |  |
| For Sub-Frame 0 | Bits |  |
| Number of Code Blocks per Sub-Frame (Note 3) |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  |  |
| For Sub-Frame 5 |  |  |
| For Sub-Frame 0 |  |  |
| Binary Channel Bits Per Sub-Frame |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits |  |
| For Sub-Frame 5 | Bits |  |
| For Sub-Frame 0 | Bits |  |
| Max. Throughput averaged over 1 frame | Mbps |  |
| UE Category |  |  |
| Note 1: 4 symbols allocated to PDCCH for 1.4 MHz. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [3]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |

A.3.5	Reference measurement channels for PDCCH/PCFICH performance requirements

A.3.5.1	FDD

Table A.3.5.1-1: Reference Channel FDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.16 FDD |
| Number of transmitter antennas |  | 2 |
| Channel bandwidth | MHz | 1.4 |
| Number of OFDM symbols for PDCCH | symbols | 2 |
| Aggregation level | CCE | 2 |
| DCI Format |  | Format 2 |
| Cell ID |  | 0 |
| Payload (without CRC) | Bits | 31 |

A.3.5.2	[FFS]

A.3.7	[FFS]

A.3.8	[FFS]

A.3.9	[FFS]

A.3.10	[FFS]

A.3.11	Reference Measurement Channels for MPDCCH performance requirements

A.3.11.1	FDD and half-duplex FDD

FFS

A.3.11.2	[FFS]

A.3.12	Reference measurement channels for NPDSCH performance requirements

Editor’s note: The note 2 in Table A.3.12.1.2-1 and note 1 in Table A.3.12.2.1-1 are still under discussion with RAN4.

### A.3.12.1	[FFS]

### A.3.12.2	Standalone

#### A.3.12.2.1 Single-antenna transmission

Table A.3.12.2.1-1: NPDSCH Reference Channel with 1Tx Antenna for UE Category NB1 and NB2 for FDD

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.1 FDD | R.NB.2 FDD |
| Carrier Type |  | Anchor | Non-anchor |
| Channel bandwidth | KHz | 200 | 200 |
| Allocated subframes per Radio Frame |  | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| ITBS/ISF |  | 9/3 | 6/3 |
| Target Coding Rate |  | 1/2 | 1/3 |
| Coding Rate |  | 0.5 | 0.33 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 616 | 392 |
| For Sub-Frame 0,5 | Bits | N/A | 392 |
| For Sub-Frame 4,9 | Bits | Note 2 | 392 |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 |  | 1 | 1 |
| For Sub-Frame 0,5 | Bits | N/A | 1 |
| For Sub-Frame 4,9 | Bits | Note 3 | 1 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 320 | 320 |
| For Sub-Frame 0,5 | Bits | N/A | 320 |
| For Sub-Frame 4,9 | Bits | Note 4 | 320 |
| Max. Average Throughput | Bps | Note 5 | Note 5 |
| UE Category |  | NB1,NB2 | NB1,NB2 |
| Note 1: It shall depend on the specific NPDSCH scheduling. Note 2: N/A when mod 2 = 0, otherwise 616. Note 3: N/A when mod 2 = 0, otherwise 1. Note 4: N/A when mod 2 = 0, otherwise 320. Note 5: Maximum Average Throughput equals to sum of TB(i) divided by sum of T(i), where TB(i) is the TB size of NPDSCH over ith NPDSCH scheduling period, and T(i) is the total time consisting of NPDCCH transmission duration, NPDCCH to NPDSCH scheduling delay, NPDSCH transmission duration, NPDSCH to NPUSCH format 2 scheduling delay, NPUSCH format 2 transmission duration, possible delay between NPUSCH format 2 and NPDCCH for next NPDSCH scheduling and subframes used for NPSS/NSSS/NPBCH/NB-SIB1/NB-SIB2 transmission during the ith NPDSCH scheduling period. |  |  |  |

Table A.3.12.2.1-2: NPDSCH Reference Channel with 1Tx Antenna for UE Category NB2

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.7 FDD |  |
| Carrier Type |  | Non-anchor |  |
| Channel bandwidth | KHz | 200 |  |
| Allocated subframes per Radio Frame |  | Note 1 |  |
| Modulation |  | QPSK |  |
| ITBS/ISF |  | 9/5 |  |
| Target Coding Rate |  | 1/2 |  |
| Coding Rate |  | 0.5 |  |
| Information Bit Payload |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 936 |  |
| For Sub-Frame 0,5 | Bits | 936 |  |
| For Sub-Frame 4,9 | Bits | 936 |  |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 1 |  |
| For Sub-Frame 0,5 | Bits | 1 |  |
| For Sub-Frame 4,9 | Bits | 1 |  |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 320 |  |
| For Sub-Frame 0,5 | Bits | 320 |  |
| For Sub-Frame 4,9 | Bits | 320 |  |
| Max. Average Throughput | Bps | Note 2 |  |
| UE Category |  | NB2 |  |
| Note 1: It shall depend on the specific NPDSCH scheduling. Note 2: Maximum Average Throughput equals to sum of TB(i) divided by sum of T(i), where TB(i) is the TB size of NPDSCH over ith NPDSCH scheduling period, and T(i) is the total time consisting of NPDCCH transmission duration, NPDCCH to NPDSCH scheduling delay, NPDSCH transmission duration, NPDSCH to NPUSCH format 2 scheduling delay, NPUSCH format 2 transmission duration, possible delay between NPUSCH format 2 and NPDCCH for next NPDSCH scheduling and subframes used for NPSS/NSSS/NPBCH/NB-SIB1/NB-SIB2 transmission during the ith NPDSCH scheduling period. |  |  |  |

## A.3.13	Reference measurement channels for NPDCCH performance requirements

### A.3.13.1	Half-duplex FDD

Table A.3.13.1-1 NPDCCH Reference Channel for Category NB1 and NB2 UE

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.NB.3 FDD |
| Number of NRS ports |  | 1 |
| Channel bandwidth | MHz | 0.2 |
| Aggregation level | NCCE | 2 |
| DCI Format |  | N1 |
| Payload (without CRC) | Bits | 23 |

## A.3.14	Reference measurement channels for NPBCH performance requirements for Cat NB1 UEs

Table A.3.14-1: Reference Channel for Category NB1 UE

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.1 | R.NB.2 |
| Number of transmitter antennas |  | 1 | 2 |
| Channel bandwidth | KHz | 200 | 200 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 50/1600 | 50/1600 |
| Payload (without CRC) | Bits | 34 | 34 |

A.4	CQI reference measurement channels

FFS

A.5	OFDMA Channel Noise Generator (OCNG)

A.5.1	OCNG Patterns for FDD

The following OCNG patterns are used for modelling allocations to virtual UEs (which are not under test). The OCNG pattern for each sub frame specifies the allocations that shall be filled with OCNG, and furthermore, the relative power level of each such allocation.

In each test case the OCNG is expressed by parameters OCNG_RA and OCNG_RB which together with a relative power level () specifies the PDSCH EPRE-to-RS EPRE ratios in OFDM symbols with and without reference symbols, respectively. The relative power, which is used for modelling boosting per virtual UE allocation, is expressed by:

where  denotes the relative power level of the i:th virtual UE. The parameter settings of OCNG_RA, OCNG_RB, and the set of relative power levels are chosen such that when also taking allocations to the UE under test into account, as given by a PDSCH reference channel, a transmitted power spectral density that is constant on an OFDM symbol basis is targeted.

Moreover, the OCNG pattern is accompanied by a PCFICH/PDCCH/PHICH reference channel which specifies the control region. For any aggregation and PHICH allocation, the PDCCH and any unused PHICH groups are padded with resource element groups with a power level given respectively by PDCCH_RA/RB and PHICH_RA/RB as specified in the test case such that a total power spectral density in the control region that is constant on an OFDM symbol basis is targeted.

A.5.1.1	OCNG FDD pattern 1: One sided dynamic OCNG FDD pattern

This OCNG Pattern fills with OCNG all empty PRB-s (PRB-s with no allocation of data or system information) of the DL sub-frames, when the unallocated area is continuous in frequency domain (one sided).

Table A.5.1.1-1: OP.1 FDD: One sided dynamic OCNG FDD Pattern

| Relative power level [dB] |  |  | PDSCH Data |
| --- | --- | --- | --- |
| Subframe |  |  |  |
| 0 | 5 | 1 – 4, 6 – 9 |  |
| Allocation |  |  |  |
| First unallocated PRB – Last unallocated PRB | First unallocated PRB – Last unallocated PRB | First unallocated PRB – Last unallocated PRB |  |
| 0 | 0 | 0 | Note 1 |
| Note 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of PDSCH. Note 2: If two or more transmit antennas with CRS are used in the test, the OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS according to transmission mode 2. The parameter applies to each antenna port separately, so the transmit power is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in section 7.1 in 3GPP TS 36.213. |  |  |  |

A.5.1.2	OCNG FDD Pattern 2: Two sided dynamic OCNG FDD pattern

This OCNG Pattern fills with OCNG all empty PRB-s (PRB-s with no allocation of data or system information) of the DL sub-frames, when the unallocated area is discontinuous in frequency domain (divided in two parts by the allocated area – two sided), starts with PRB 0 and ends with PRB NRB 1.

Table A.5.1.2-1: OP.1 FDD: Two sided dynamic OCNG FDD Pattern

| Relative power level  PRB [dB] |  |  | PDSCH Data |
| --- | --- | --- | --- |
| Subframe |  |  |  |
| 0 | 5 | 1 – 4, 6 – 9 |  |
| Allocation |  |  |  |
| 0 – (First allocated PRB-1) and (Last allocated PRB+1) – ( NRB 1) | 0 – (First allocated PRB-1) and (Last allocated PRB+1) – ( NRB 1) | 0 – (First allocated PRB-1) and (Last allocated PRB+1) – ( NRB 1) |  |
| 0 | 0 | 0 | Note 1 |
| Note 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameter  PRB is used to scale the power of PDSCH. |  |  |  |

A.5.2	FFS

A.5.3	OCNG Patterns for Narrowband IoT

The following OCNG patterns are used for modelling allocations to virtual narrowband IoT UEs (which are not under test). The OCNG pattern for each sub frame specifies the allocations that shall be filled with OCNG, and furthermore, the relative power level of each such allocation.

In each test case the OCNG is expressed by parameters OCNG_RA and OCNG_RB which together with a relative power level () specifies the NPDSCH EPRE-to-NRS EPRE ratios in OFDM symbols with and without Narrowband reference symbols, respectively. The relative power, which is used for modelling boosting per virtual UE allocation, is expressed by:

where  denotes the relative power level of the i:th virtual UE. The parameter settings of OCNG_RA, OCNG_RB, and the set of relative power levels are chosen such that when also taking allocations to the UE under test into account, as given by a NPDSCH or NPDCCH reference channel, a transmitted power spectral density that is constant on an OFDM symbol basis is targeted.

A.5.3.1	Narrowband IoT OCNG pattern 1

Table A.5.3.1-1: NB.OP.1 FDD: OCNG FDD Pattern 1

| Bandwidth | Relative power level [dB] | NPDCCH and corresponding NPDSCH Data |
| --- | --- | --- |
|  | Subframe |  |
|  | Unused subframes |  |
| 200KHz | 0 | Note 2 |
| Note 1: These subframes are assigned to an arbitrary number of virtual UEs with one NPDSCH per virtual UE with corresponding NPDCCH; the data transmitted over the OCNG NPDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of NPDSCH and NPDCCH. Note 2: Subframes and/or REs available for narrowband IOT DL transmission depend on the in-band, guard band or standalone mode indicated in MIB, and scheduling delay between NPDCCH, NPDSCH, NPUSCH format 2 and NPDCCH specified in test cases. |  |  |

<< END OF CHANGES >>