3GPP TSG-RAN5 Meeting #100 Incheon	R5-234879r1

Toulouse, France, 21 – 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2589 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TT update for test cases 6.5.5.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add test torlance value in the Test requirements part R1: the TT value of q1,0 and q1,1 are changed to align with R4-2312281 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test torlance value in the Test requirements part R1: update requirement basic on latest verision of R4-2312281 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI cannnot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | × | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | × | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | × | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on R4-2312281 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-234879 |  |  |  |  |  |  |  |  |

<<Start of Changes>>

6.5.5.7	NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX

Editor's Note: This test case is incomplete in following aspects:

-	TS 38.522 applicability spec update is pending

6.5.5.7.1	Test purpose

The purpose of this test is to verify that the UE properly detects TRP specific CSI-RS-based beam failure in the sets q0,0  and q0,1  configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1,0 and q1,1. The purpose is to test the downlink monitoring for beam failure detection on TRP1 within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based TRP specific beam failure detection and link recovery for an FR1 serving cell requirements in TS 38.133 [6] clause 8.18 with schedulingRequestID-BFR-r17 configured.

6.5.5.7.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting 5GS NR SA FR1, CSI-RS based RLM, TRP Specific CSI-RS-based link recovery and PCell beam failure recovery.

6.5.5.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.5.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.5.7.

6.5.5.7.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.7.1-1 shows the variation of the downlink SNR of the CSI-RS in set q0,0  and q0,1  for TRP1 and TRP2 respectively to emulate CSI-RS based beam failure on TRP1. Figure A.6.5.5.7.1-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1,0 and q1,1 of the candidate beam used for link recovery.

Figure 6.5.5.7.4-1: SNR and L1-RSRP variation for CSI-RS-based beam failure detection and link recovery testing in DRX mode

6.5.5.7.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.5.7.4.1-1.

Table 6.5.5.7.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.5.7.4.1-2.

Table 6.5.5.7.4.1-2: Initial conditions for NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.5.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1. The general test parameter settings are set up according to Table 6.5.5.7.4.1-3.

2. Message contents are defined in clause 6.5.5.7.4.3.

3. There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.5.5.7.4.1-3: General test parameters for FR1 PCell for CSI-RS-based TRP specific beam failure detection and link recovery testing in DRX mode

| Parameter |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test1 |  |
| Active PCell |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  | 1 |  |
| Duplex mode |  | Config 1 |  |  | FDD |  |
|  |  | Config 2, 3 |  |  | TDD |  |
| TDD Configuration |  | Config 1 |  |  | Not Applicable |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |
| RMSI CORESET Reference Channel |  | Config 1 |  |  | CR.1.1 FDD |  |
|  |  | Config 2 |  |  | CR.1.1 TDD |  |
|  |  | Config 3 |  |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  |  | CCR.2.1 TDD |  |
| SSB Configuration |  | Config 1 |  |  | SSB.3 FR1 |  |
|  |  | Config 2 |  |  | SSB.3 FR1 |  |
|  |  | Config 3 |  |  | SSB.4 FR1 |  |
| SMTC Configuration |  | Config 1, 2 |  |  | SMTC.1 |  |
|  |  | Config 3 |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 |  |  | 15 KHz |  |
|  |  | Config 3 |  |  | 30 KHz |  |
| PRACH Configuration |  | Config 1, 2,3 |  |  | FR1 PRACH configuration 4 |  |
| OCNG parameters |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  | 2 |  |
|  | Aggregation level |  |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  |  | REG bundle size |  |
|  | REG bundle size |  |  |  | 6 |  |
| DRX |  |  |  |  | DRX.7 |  |
| Gap pattern ID |  |  |  |  | N.A. |  |
| schedulingRequestID-BFR- r17 |  |  |  |  | Configured |  |
| Periodicity of PUCCH for SR configuration for BFR on PCell |  |  | Config 1 | slot | 5 | 5ms |
|  |  |  | Config 2, 3 |  | 10 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB |  | Config 1, 2 |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 |  |  | -95 |  |
| powerControlOffsetSS |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for TRP0 |  | Config 1 |  |  | CSI-RS.1.2 FDD |  |
|  |  | Config 2 |  |  | CSI-RS.1.2 TDD |  |
|  |  | Config 3 |  |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for TRP1 |  | Config 1 |  |  | CSI-RS.1.7 FDD |  |
|  |  | Config 2 |  |  | CSI-RS.1.6 TDD |  |
|  |  | Config 3 |  |  | CSI-RS.2.7 TDD |  |
| CSI-RS Index assigned as BFD RS (q0,0) |  |  |  |  | 0 |  |
| CSI-RS Index assigned as CBD RS (q1,0) |  |  |  |  | 1 |  |
| CSI-RS Index assigned as BFD RS (q0,1) |  |  |  |  | 2 |  |
| CSI-RS Index assigned as CBD RS (q1,1) |  |  |  |  | 3 |  |
| CSI-RS index assigned as RLM RS |  |  |  |  | 0,1,2,3 |  |
| CSI-RS configuration for CSI reporting |  | Config 1 |  |  | CSI-RS.1.1 FDD |  |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| TRS configuration |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| T310 Timer |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  | 2 |  |
| T1 |  |  |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s | 8.37 |  |
| T3 |  |  |  | s | 6.44 |  |
| T4 |  |  |  | s | 0 |  |
| T5 |  |  |  | s | 1.97 |  |
| D1 |  |  |  | s | 1.93 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |

6.5.5.7.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 6.5.5.7.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR1 and SNR2 values to T2 as specified in Table 6.5.5.7.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR1 and SNR2 values to T3 as specified in Table 6.5.5.7.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR1 and SNR2 values to T4 as specified in Table 6.5.5.7.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR1 and SNR2 values to T5 as specified in Table 6.5.5.7.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CSI transmission (according CSI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect preamble on a beam associated with the candidate beam set q10 and q11 before time point B

and

c) detects preamble on a beam associated with the candidate beam set q10 and q11 before time point F (D1 after the start of T5),

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	If iteration or the random access procedure for BFD fails, switch the UE off and on. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat steps 2-8 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.5.5.7.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.5.7.4.3-1: Common Exception messages for NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-8A with Condition CSI-RS BFD Table H.3.1-10 with Condition CSI-RS Table H.3.1-10A Table H.3.1-11 with Condition CSI-RS Table H.3.5-4 Table H.3.7-1 with Condition DRX.7 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1, |

Table 6.5.5.4.4.3-2: PDCCH Search Space for BFR

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 4 | BFR |  |
| controlResourceSetId | 2 | BFR |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Symbol 0 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 | DCI Format 1_0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.5.4.4.3-3: RLF-TimersAndConstants

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| n310 | n2 |  |  |
| } |  |  |  |

Table 6.5.5.4.4.3-4: Void

Table 6.5.5.3.4.3-5: NZP-CSI-RS-Resource

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| powerControlOffsetSS | db0 |  |  |
| } |  |  |  |

Table 6.5.5.4.4.3-6: PDCCH-Config

| Derivation Path: TS 38.501-1 [14],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 2 entries |  |  |
| ControlResourceSet[2] | ControlResourceSet | entry 2, BFR |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[2] | SearchSpace | entry 2, BFR |  |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| } |  |  |  |

Table 6.5.5.4.4.3-7: ControlResourceSet for BFR

| Derivation Path: TS 38.501-1 [14],Table 7.3.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| duration | 2 |  |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| interleaved ::= SEQUENCE { |  |  |  |
| reg-BundleSize | n6 |  |  |
| interleaverSize | n2 |  |  |
| shiftIndex | 0 |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| } |  |  |  |

Table 6.5.5.4.4.3-8:MAC-CellGroupConfig for R17 BFR

| Derivation Path: TS 38.508-1, Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestID-BFR-r17 | Not present |  |  |
| schedulingRequestID-BFR2-r17 | Not present |  |  |
| schedulingRequestConfig-v1700 | SchedulingRequest-Config |  |  |
| } |  |  |  |

6.5.5.7.5	Test requirement

Tables 6.5.5.7.4.1-3 and 6.5.5.7.5-1 define the primary level settings including test tolerances for NR SA FR1 TRP specific CSI-RS-based beam failure detection and link recovery in DRX.

Table 6.5.5.7.5-1: Cell specific test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0,0 | Config 1 | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 2 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 3 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_CSI-RS of set q0,1 | Config 1 | dB | 5.8 | 5.8 | 5.8 | 5.8 | 5.8 |
|  | Config 2 |  | 5.8 | 5.8 | 5.8 | 5.8 | 5.8 |
|  | Config 3 |  | 5.8 | 5.8 | 5.8 | 5.8 | 5.8 |
| SNR_CSI-RS of set q1,0 | Config 1 | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 2 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 3 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| CSI-RS_RP of set q1,0 | Config 1 | dB/SCS kHz | -110.2 | -110.2 | -87.8 | -87.8 | -87.8 |
|  | Config 2 |  | -110.2 | -110.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3 |  | -107.2 | -107.2 | -84.8 | -84.8 | -84.8 |
|  | Config 1 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.7.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause TS38.133 A.3.6. |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1,0.

No later than time point F occurring no later than D1 = 1920+10 ms after the start of T5, the UE shall transmit PUCCH with LRR, followed by BFR MAC CE containing a beam associated with the candidate beam set q1,0. The UE shall not transmit PUCCH with an LRR with the candidate beam set q1,0 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

<<End of Change>>