3GPP TSG-RAN WG5#102 								                   		R5-240523

Athens, Greece, 26 February 2024 – 1 March 2024

Title: 	Test frequencies for asymmetric UL/DL CBW

Agenda Item:	5.4.20

Source: 	Nokia, T-Mobile USA, Keysight Technologies, Skyworks Solutions, Inc.

Document for:	Approval

# 1	Introduction

This document is a companion to CR [1] which corrects test frequencies for band n25 asymmetric UL/DL 40/45MHz channel bandwidth (CBW).

# Discussion

In Release 17, RAN4 introduced REFSENS requirements for asymmetric Uplink / Downlink (UL/DL) operation in several NR frequency bands. Table 1 reproduces the TS 38.101-1 Table 5.3.6-1 for the example of band n25, where the UL CBW is restricted to 40MHz for operation at DL 45MHz CBW.

Table 1: Release 17 band n25 asymmetric UL/DL 40/45MHz – TS 38.101-1 Table 5.3.6-1.

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n25 | 40 | 45 | 0 |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. |  |  |  |

For asymmetric UL/DL operation, RAN4 agreed to specify REFSENS with the UL carrier configured at the default Tx-Rx frequency separation distance from the DL carrier frequency. This agreement was captured using a new footnote “Note 5” as shown in Table 2 for the example band n25 45MHz REFSENS.

Table 2: Release 17 band n25 uplink configuration for reference sensitivity – TS 38.101-1 Table 7.3.2-3.

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n71 | 15 | 25 | 501 | 501 | 501 | 501 | 481 | 401 | 401 | Note 5 |  |  |  |  |  |  | FDD |
|  | 30 |  | 241 | 241 | 241 | 241 | 241 | 201 | 201 | Note 5 |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 | 101 | 101 | Note 5 |  |  |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

This agreement is illustrated in Figure 1 and summarised in Table 3.

Figure 1: Band n25 UL/DL 40/45MHz “Low” range carrier frequencies for REFSENS

Table 3: Band n25 UL/DL 40/45MHz “Low/Mid/High” test frequencies

| UL/DL Bandwidth combination [MHz] | Range | DL carrier [MHz] | UL carrier [MHz] |
| --- | --- | --- | --- |
| 40/45 | Low | 1952.5 | 1952.5-80 = 1872.5 |
|  | Mid | 1962.5 | 1962.5-80 = 1882.5 |
|  | High | 1972.5 | 1972.5-80 = 1892.5 |

As explained in [2], we would like to remind that by “… default Tx-Rx frequency separation…” in Note 5, RAN4 means that the UL and the DL carrier frequencies should be configured at the band’s “nominal Tx-Rx frequency separation” as defined in TS 38.101-1 clause 5.4.4. In other words, the asymmetric UL/DL CBW REFSENS requirements are specified under the assumption that the maximum deviation to the Tx-Rx carrier-center frequency “ΔFTX-RX“ defined in clause 5.3.6 does not apply.

We observe in Table 4 that the test frequencies captured in TS 38.508-1 do not align with these agreements.

Table 4: Test frequencies for band n25 asymmetric UL/DL 40/45MHz – TS 38.508-1 Table 4.3.1.1.1.25-1, Table 4.3.1.1.1.25-2 and Table 4.3.1.1.1.25-3.

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] |
| --- | --- | --- | --- | --- | --- |
| 40/45 | 242 | Downlink | Low | 1952.5 | 390500 |
|  |  |  | Mid | 1962.5 | 392500 |
|  |  |  | High | 1972.5 | 394500 |
|  | 216 | Uplink | Low | 1870 | 374000 |
|  |  |  | Mid | 1880 | 376000 |
|  |  |  | High | 1890 | 378000 |

We therefore propose to adopt the following changes for band n25 [1].

Proposal 1: For Release 17, we suggest RAN5 to adopt the following changes to TS 38.508-1 for the band n25 test frequencies.

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40/45 | 242 | Downlink | Low | 1952.5 | 390500 | 1930.72 | 386144 | 0 | 15 | 4834 | 386690 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1922.36 | 384472 | 102 |  | 4856 | 388570 | 10 | 1 | 0 (0) | 103 |
|  |  |  | High | 1972.5 | 394500 | 1860 | 372000 | 504 |  | 4881 | 390510 | 2 | 0 | 0 (0) | 504 |
|  | 216 | Uplink | Low | 1872.5 | 374500 | 1853.06 | 370612 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1772.34 | 354468 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1892,5 | 378500 | 1871.98 | 374396 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

It should be noted that similar corrections are required for already defined test frequencies at least for the following bands:

- n5 UL/DL 20/25

- n8 UL/DL 20/25, 20/30, 20/35

- n71 UL/DL 20/25, 20/30, 20/35

Observation: Similar corrections on test frequencies may be needed for other asymmetrical UL/DL CBW in TS 38.508-1, for example for band n5 UL/DL 20/25MHz, n8 UL/DL 20/25, 20/30, 20/35 and n71 20/25, 20/30 and 20/35.

# Conclusion

This contribution suggests adopting the following changes to TS 38.508-1.

Proposal 1: For Release 17, we suggest RAN5 to adopt the following changes to TS 38.508-1 for the band n25 test frequencies.

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40/45 | 242 | Downlink | Low | 1952.5 | 390500 | 1930.72 | 386144 | 0 | 15 | 4834 | 386690 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1922.36 | 384472 | 102 |  | 4856 | 388570 | 10 | 1 | 0 (0) | 103 |
|  |  |  | High | 1972.5 | 394500 | 1860 | 372000 | 504 |  | 4881 | 390510 | 2 | 0 | 0 (0) | 504 |
|  | 216 | Uplink | Low | 1872.5 | 374500 | 1853.06 | 370612 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1772.34 | 354468 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1892,5 | 378500 | 1871.98 | 374396 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation: Similar corrections on test frequencies may be needed for other asymmetrical UL/DL CBW in TS 38.508-1, for example for band n5 UL/DL 20/25MHz, n8 UL/DL 20/25, 20/30, 20/35 and n71 20/25, 20/30 and 20/35.

Accompanying CR to 38.508-1 in [1] implement Rel-17 changes.

# References

[1] 	R5-240468, Test frequencies and channel bandwidth updates for band n71 and n25 in Rel-17, 3GPP TSG-RAN WG5 Meeting # 102, Athens, Greece, 26th February – 1st March 2024, Nokia, T-Mobile USA, Keysight Technologies, Skyworks Solutions, Inc.

[2] 	R5-240467, Bands n25 and n71 asymmetric UL/DL REFSENS, 3GPP TSG-RAN WG5 Meeting # 102, Athens, Greece, 26th February – 1st March 2024, Nokia, T-Mobile USA, Keysight Technologies, Skyworks Solutions, Inc.