3GPP TSG-RAN5 Meeting #102	R5-240547

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3377 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 38.523-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. The MUSIM test cases require a new MMI command. 2. The NR-DC inter-cell PSCell change sequence needs to be enhanced to document the RLC re-establishment happening on the PCell and to reconfigure the Split NR PCDP by releasing it first in the source PSCell before reconfiguring in the target PSCell. 3. The execution guidelines need to be updated to consider test specification updates and the new delivered test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Introduced new MMI command. 2. Updated the sequence in clause 7.3.5.5.1. 3. Updated clause 11. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. MUSIM test cases cannot be fully implemented. 2. The NR-DC inter-cell PSCell change will remain incomplete. 3. Execution guidelines will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 7.3.5.5.1, 11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Table 6.1-1 has been resized. Only changes highlighted in yellow are newly added. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 6	System interface

## 6.1	Upper tester interface

The Upper Tester (UT) interface is the same as specified in TS 36.523-3 [12] clause 5.

In addition, the following MMI commands are defined.

Table 6.1-1: MMI commands

| Command | Parameters |  |
| --- | --- | --- |
|  | Name | Value |
| "CAPABILITY_ENQUIRY" | "Freq. Band List" | <charstring> |
|  | "Two-Way" | "TRUE" / "FALSE" |
| "CHANGE_RAC_ID" | (none) |  |
| "CLEAR_RSNPN" | (none) |  |
| "NO_PRECONDITIONS" | (none) |  |
| "PERIODICAL_ MEASUREMENT_PSBCH_RSRP" | "Report Interval" | <charstring> |
| "SET_MUSIM_UAI" | "RRC_STATE" | "idle” / "inactive" / "outOfConnected" |
|  | "STARTING_SFN" NOTE 1 | <integer> |
|  | "STARTING_SUBFRAME" NOTE 1 | <integer> |
|  | "GAP_LENGTH" NOTE 1 | “ms3” / “ms4” / “ms6” / “ms10” / “ms20” |
|  | "GAP_REPETITION" NOTE 1 | “ms20” / “ms40” / “ms80” / “ms160” / “ms320” / “ms640” / “ms1280” / “ms2560” / “ms5120” |
|  | "GAP_OFFSET" NOTE 1 | <integer> |
| "SL_CSI_RS_CONFIGURATION" | "Freq. allocation One antenna Port" | <charstring> |
|  | “First symbol" | <charstring> |
|  | "Latency Bound" | <charstring> |
| "SL_CSI_REPORT" | (none) |  |
| "SNPN_AUTOMATIC" | (none) |  |
| "SNPN_MANUAL" | "PLMN" | <PLMN ID> |
|  | "N_ID" | <N ID> |
| "SNPN_SUBSCRIBER_DATA" | "PLMN" | <PLMN ID> |
|  | "N_ID" | <N ID> |
|  | "ACCESS_ID" | "ACCESS_ID" |
| "UEAI" | "SL-QoS Flow ID" | <charstring> |
| "UNICAST_SLDRB" | "Action" | "ESTABLISH" / "MODIFY" / "RELEASE" |
| NOTE 1: These 5 parameters make up one instance of musim-GapInfo. Up to 4 instances of musim-GapInfo make up the musim-GapPreferenceList, which is itself optional. Each of these parameters is therefore also optional. |  |  |

The following AT commands are also applied in the TTCN.

Table 6.1-2: AT Commands

Command

AT+C5GNSSAI

AT+C5GNSSAIRDP

AT+C5GQOS

AT+CGBRRREQ

AT+CLCCS

AT+CMICO

AT+CNASCREL

AT+CPAGRES

AT+CPAGTCC

AT+CPSDO

AT+CREJPAG

AT commands are referred to TS 27.007 [49].

<SECTION SKIPPED>

##### 7.3.5.5.1	Sequence of NR-DC inter-cell PSCell change

0A.	NR Source PSCell: 		Get PDCP COUNT for SRB3 (if necessary) and DRBs.

NOTE 1:	No further sending/receiving of DRB data before the PSCell change is done.

0B.		NR Target PSCell:		Inform the SS about the PSCell change and about the source PSCell id.

0C.		NR Source PSCell:		Release SRB3 (if necessary) and DRBs.

1.	NR Target PSCell:			Configuration of SRB3 (if necessary) and DRBs.

2.	NR Target PSCell:			Set PDCP COUNT for SRB3 (if necessary) and DRBs.

NOTE 2:	For AM DRBs the PDCP count is maintained. For SRB3 (if applied) and UM DRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content.

3.				Void.

4.	NR Target PSCell:			Configure RACH procedure either dedicated or C-RNTI based.

NOTE 3:	The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI.

5.	NR Target PSCell:			Activate security.

6.	NR Target PSCell:			Configure UL grant configuration ("OnSR", default grant).

NOTE 4:	Unless explicitly specified UL grant configuration keeps configured as per default at the NR source PSCell.

7.	NR PCell (at T):					Send RRCReconfiguration.

NOTE 5: If RLC re-establishment applies on SRBs and DRBs in NR PCell, RRCReconfiguration is sent at activation time T otherwise activation Time is set to ‘now’.

Steps 7A to 7E are applicable if RLC re-establishment applies on SRBs and DRBs in NR PCell.

7A.	NR PCell at T:		Stop UL grants.

7B.			NR PCell at T+5:		Release SRB1, SRB2 and DRBs.

7C.			NR PCell at T+10:	(Re-)configure SRB1, SRB2 and DRBs.

7D.	NR PCell at T+15:	Restore the PDCP counts.

NOTE 6: 	For the bearers, the PDCP count is maintained or reset depending on the RRCReconfiguration message content.

7E.			NR PCell at T+20:	Configure UL grant configuration ("OnSR", default grant).

8.	NR PCell:					Receive RRCReconfigurationComplete.

9.	NR Target PSCell:			Inform the SS about completion of the PSCell change (e.g. to trigger PDCP STATUS REPORT PDU).

10.					Void.

<SECTION SKIPPED>

# 11	Guidelines on test execution

## 11.1	Introduction

Clause 11 provides the guidelines on test executions.

## 11.2	Void

## 11.3	Test execution restrictions

### 11.3.1	Bands / band combinations

#### 11.3.1.1	Introduction

The restriction on test case execution is due to the number of frequencies available for the specific band under test specified in TS 38.508-1[5] and the number of frequencies used by the test cases specified in TS 38.523-1[8].

#### 11.3.1.2	EN-DC, NR/5GC, Inter-RAT

The list of test cases, that cannot be executed due to restrictions on bands and/or band combinations, is specified in Table 11.3.1.2-1.

Table 11.3.1.2-1: Restriction on bands / band combinations for test case execution

| Category of test case | Category of restriction | Bands / band combinations with restrictions | TS 38.523-1 [8] test cases that cannot be executed |
| --- | --- | --- | --- |
| EN-DC |  |  |  |
| Single NR carrier | More than one NR radio frequency (i.e., using NRf2 or NRf3 or NRf4) | DC_21A_n28A, DC_41A_n41A | 8.2.3.6.1a, 8.2.3.7.1a, 8.2.3.8.1a, 8.2.3.10.1, 8.2.3.11.1 |
| NR-5GC |  |  |  |
| Single RAT / Single NR carrier | More than one NR radio frequency (i.e., using NRf2 or NRf3 or NRf4) | n51, n91, n93 | 6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.5, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.2.3a, 6.1.2.7, 6.1.2.11, 6.1.2.13, 6.1.2.14, 6.1.2.16, 6.1.2.18, 6.1.2.20, 6.1.2.21, 6.1.2.22, 6.1.2.24, 6.1.2.25, 6.1.2.27, 6.3.1.1, 6.3.1.2, 6.3.1.3, 6.3.1.4, 6.3.1.5, 6.3.1.8, 6.3.1.9, 6.3.1.10, 6.3.2.1, 6.3.2.2, 6.3.2.3, 6.3.2.4, 6.3.2.5, 6.3.2.6, 6.4.1.1, 6.4.1.2, 6.4.2.2, 6.4.2.3, 6.5.1.2, 6.5.2.4, 6.5.2.6, 6.5.3.1, 6.5.3.4, 6.5.3.5, 6.5.3.7, 6.5.3.8, 6.5.3.9, 7.1.3.4.4, 7.1.3.6.5, 8.1.1.2.1, 8.1.1.3.1, 8.1.1.3.3, 8.1.1.3.7, 8.1.1.3.7b, 8.1.1.3.8, 8.1.1.3.9, 8.1.3.1.3, 8.1.3.1.6, 8.1.3.1.9, 8.1.3.1.11, 8.1.3.1.12, 8.1.3.1.14A, 8.1.3.1.15A, 8.1.3.1.20, 8.1.3.21, 8.1.4.1.2, 8.1.4.1.10, 8.1.4.3.4, 8.1.4.3.5, 8.1.5.10.2, 8.1.5.10.3, 8.1.5.11.1, 8.1.5.11.3, 8.1.5.11.4, 8.1.6.1.2.2, 8.1.6.1.2.10, 8.1.6.1.3.2, 8.1.6.1.3.7, 8.1.6.1.4.3, 8.1.6.1.4.7, 8.1.6.1.4.9, 9.1.4.1, 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.4, 9.1.5.1.8, 9.1.5.1.10, 9.1.5.1.12, 9.1.5.1.14, 9.1.5.2.10, 9.1.7.4, 9.1.9.3, 9.1.12.3, 11.3.5, 11.3.6, 11.3.6a, 11.3.8, 11.3.9, 11.3.9a, 11.3.11, 11.4.7, 11.4.8 |
| Single RAT / Single NR carrier | More than two NR radio frequencies (i.e., using NRf3 or NRf4) | n13, n14, n24, n30, n53 | 6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.5, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.2.7, 6.1.2.11, 6.1.2.20, 6.1.2.24, 6.1.2.25, 6.3.1.1, 6.3.1.2, 6.3.1.3, 6.3.1.5, 6.3.1.8, 6.4.1.1, 6.4.1.2, 6.4.2.2, 6.4.2.3, 6.5.2.4, 6.5.2.6, 8.1.1.3.3, 8.1.1.3.9, 9.1.5.1.2, 9.1.5.1.8 |
| Single RAT / NR CA inter-band | Switched allocation of PCell and SCell if SCell frequency has no UL | CA_n29A_n66A, CA_n29A_n70A, CA_n29A_n71A | 8.1.4.1.7.2, 8.1.4.1.9.2 |
| Single RAT / NR CA intra-band | Switched allocation of PCell and SCell if SCell frequency has no UL | CA_n66B | 8.1.4.1.7.1, 8.1.4.1.9.1 |
| Single RAT / NR additional BWP without CORESET#0 | No frequency specified | n29, n51, n75, n76, n91, n93, n100, n101 | 7.1.1.8.4 |
| NR/5GC Inter-RAT |  |  |  |
| NR / E-UTRA Inter-RAT | More than one E-UTRA radio frequency (i.e., using f2 or f3 or f4) | 13, 18, 31, 54, 72, 73, 87, 88 | 6.4.3.1, 8.1.1.3.4, 11.1.4, 11.1.5. |
| NR / E-UTRA Inter-RAT | More than one NR radio frequency (i.e., using NRf2 or NRf3 or NRf4) | n51, n91, n93, n94 | 6.2.1.1, 6.2.1.2, 6.2.1.3, 6.2.1.4, 8.1.1.3.7a. |
| NR / E-UTRA Inter-RAT | More than two NR radio frequencies (i.e., using NRf3 or NRf4) | n13, n14, n24, n30, n53 | 6.2.1.1 |
| NR / EN-DC Inter-RAT | More than one NR radio frequency (i.e., using NRf2 or NRf3 or NRf4) | DC_21A_n28A, DC_41A_n41A | 8.1.4.2.1.2 |

### 11.3.2	Void

### 11.3.3	NR MFBI

The following NR MFBI test case shall be executed using the combinations specified in Table 11.3.3-1 for px_NR_OverlappingNotSupportedBand_MFBI and px_NR_PrimaryBand:

6.1.2.23.

Table 11.3.3-1: NR MFBI bands combinations

| px_NR_OverlappingNotSupportedBand_MFBI | px_NR_PrimaryBand (Note) |
| --- | --- |
| n2 | n25 |
| n25 | n2 |
| n38 | n41 |
| n41 | n38 |
| n77 | n78 |
| n78 | n77 |
| n257 | n258, n261 |
| n258 | n257 |
| n261 | n257 |
| Note: The UE supports one or more of the listed MFBI bands and does not support at least one overlapping band. If the UE supports all overlapping bands, these test cases are not applicable. |  |