3GPP TSG-RAN5 Meeting #102	R5-241248

# Athens, Greece, 26th February - 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 3016 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Core Spec alignment for RRM NB-IOT NTN test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NB-IOT NTN RRM test cases 13.1.x are not aligned to core specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | NB-IOT NTN RRM test cases 13.1.x have been aligned to core specificationfor NPRACH configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-IOT NTN RRM test cases 13.1.x will remain not aligned to core specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depends on R4-2400664 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

# 13	E-UTRAN Standalone Tests for UE Category NB for Satellite Access

## 13.1	RRC_IDLE state for satellite access

### 13.1.1	Cell re-selection for Satellite Access

#### 13.1.1.1	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage

Editor's Note: This test case is incomplete in following aspects:

##### 13.1.1.1.1	Test purpose

To verify that when the current and target cell operates on the same carrier frequency the Cat-NB1 UE is able to search and measure cells to meet the intra-frequency cell re-selection requirements.

##### 13.1.1.1.2	Test applicability

This test applies to all types of NB-IoT HD-FDD category NB1 UE supporting GSO or both from release 17 and forward.

##### 13.1.1.1.3	Minimum conformance requirements

The UE shall be able to identify new intra-frequency cells and perform NRSRP measurements of identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS36.304 within Ksatellite*Tdetect,NB_Intra_NC when Treselection= 0 .  An intra frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in 3GPP TS 36.133 Annex B.1.4 for a corresponding Band.

The UE shall measure NRSRP at least every Ksatellite*Tmeasure,NB_Intra_NC for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter NRSRP measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Ksatellite*Tmeasure,NB_Intra-NC/2

The UE shall not consider an NB-IoT neighbour cell in cell reselection if it is indicated as not allowed in the measurement control system information of the serving NB-IoT cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in 3GPP TS 36.304[6] within Ksatellite*Tevaluate,NB_intra-NC when Treselection = 0, provided that the cell is at least XdB better ranked, where ‘X’ is specified in 3GPP TS 36.133 Table 4.6A.2.4-3. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and non-serving NB-IoT intra-frequency cells.

The parameter Ksatellite is the scaling factor for measurements correspond to multiple NGSO satellites. Ksatellite = 1, if GSO satellite(s) is/are measured on the carrier. Ksatellite  equals to the number NGSO satellites to be measured if NGSO satellites are monitored.

If Treselection timer has a non zero value and the intra-frequency cell is better ranked than the serving NB-IoT cell, the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

The UE shall evaluate the intra-frequency cell re-selection criteria as defined in 3GPP TS 36.304 [6] at least every DRX cycle. The DRX cycle length is 1.28 seconds.

The normative reference for this requirement is TS 36.133 [4] clause 4.6A.2 and A.13.1.1.1.

##### 13.1.1.1.4	Test description

13.1.1.1.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.1.1.1.5-1.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.93 using only UE main Tx/Rx antenna.

2.	The parameter settings for the cells are set up according to Table 13.1.1.1.4.1-2.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	There are one NB-IoT carrier and two cells specified in the test. Ncell1 is standalone cell to Cell1 and Ncell2 is standalone cell to Cell2. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

5.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

6.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

7.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

. The UE shall be provided with the valid information about the SAN serving cells before the test

Table 13.1.1.1.4-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GEO, HD-FDD duplex mode |

Table 13.1.1.1.4-2: General test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | Standalone |  |
| Satellite information | Config 1 |  | GEO |  |
| Initial condition | Active cell |  | nCell1 |  |
|  | Neighbour cells |  | nCell2 |  |
| T2 end condition | Active cell |  | nCell2 |  |
|  | Neighbour cells |  | nCell1 |  |
| Final condition | Visited cell |  | nCell1 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| s-IntraSearchP |  |  | 63 (126 dB) | to trigger intra-frequency measurement in this test |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | >7 | During T1, nCell2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that nCell2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 60 | T2 is defined so that cell re-selection time is taken into account. Once the UE has reselected to nCell2 (within T2) T3 starts |
| T3 |  | s | 15 | T3 is defined so that cell re-selection time is taken into account. |

13.1.1.1.4.2	Test procedure

The test scenario comprises of one NB-IoT carrier with 2 Ncells of different physical cell ID. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Only Ncell1 is already identified by the UE prior to the start of the test, i.e. Ncell 2 is not identified. Ncell 1 and Ncell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Ncell 2.

In the following test procedure "UE responds" means "UE starts transmitting preamble on NPRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure according to 3GPP TS 36.508 [7] clause 8.1.5A.5"

1.	Ensure the UE is in State 3A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5 in Ncell 1.

2.	Set the parameters according to T1 in Table 13.1.1.1.5-1 and 13.1.1.1.5-2. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

3.	Set Ncell2 physical cell identity = ((current Ncell 2 physical cell identity + 1) mod 14 + 2) for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 13.1.1.1.5-1.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Ncell 2.

6.	If the UE responds on the newly detectable cell, Ncell 2, during time duration T2 within 60 seconds from the beginning of time period T2, then count a success for the event "Re-select newly detected Ncell 2". Otherwise count a fail for the event "Re-select newly detected Ncell 2".

7.	If the UE has re-selected Ncell 2 within T2, after the re-selection or when T2 expires, continue with step 8.
Otherwise, if T2 expires and the UE has not yet re-selected Ncell 2, skip to step 12.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 13.1.1.1.5-1.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Ncell 1.

10.	If the UE responds on the already detected cell, Ncell 1, during time duration T3 within 15 seconds from the beginning of time period T3, then count a success for the event "Re-select already detected Ncell 1". Otherwise count a fail for the event "Re-select already detected Ncell 1".

11.	If the UE has re-selected Ncell 1 within T3, after the re-selection or when T3 expires, skip to step 13.
Otherwise, if T3 expires and the UE has not yet re-selected Ncell 1, continue with step 12.

12.	Switch off and on the UE and ensure the UE is in State 3A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5 in Ncell 1.

13.	Repeat step 2-12 until a test verdict has been achieved.
Each of the events "Re-select newly detected Ncell 2" and "Re-select already detected Ncell 1" is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

13.1.1.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.4.3, 8.1.5B and 8.1.6 using condition “standalone” with the following exception:.

Table 13.1.1.1.4.3-1: SystemInformationBlockType3-NB

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.4.3.3, Table 8.1.4.3.3-8: SystemInformationBlockType3-NB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3-NB-r13 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo-r13 SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -70 (-140 dBm) |  |  |
| q-QualMin-r13 | Not present |  |  |
| p-Max-r13 | Not present |  |  |
| s-IntraSearchP-r13 | 63 (126dB) | nCell1, nCell2 |  |
| t-Reselection-r13 | 0 |  |  |
| } |  |  |  |

##### 13.1.1.1.5	Test requirement

Tables 13.1.1.1.5-1 defines the primary level settings including test tolerances for HD-FDD intra frequency cell re-selection test case.

Table 13.1.1.1.5-1: nCell 1, nCell 2 specific test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in normal coverage

| Parameter | Unit | nCell 1 |  |  | nCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | kHz | 200 |  |  | 200 |  |  |
| OCNG Pattern as defined in A.3 | - | NOP.3 FDD |  |  | NOP.3 FDD |  |  |
| NPBCH_RA | dB | 0 |  |  | 0 |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Qrxlevmin | dBm | -140 | -140 | -140 | -140 | -140 | -140 |
| Pcompensation | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Qhysts | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Qoffsets, n | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Cell_selection_and_ reselection_quality_measurement |  | NRSRP |  |  | NRSRP |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 17 | 13 | 17.45 | -infinity | 17.45 | 13 |
| Note2 | dB | 17 | -4.53 | 4.24 | -infinity | 4.24 | -4.53 |
| NRSRP Note2 | dBm/15 kHz | -81 | -85 | -80.55- | -infinity | -80.55 | -85 |
| Treselection | s | 0 | 0 | 0 | 0 | 0 | 0 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to nCell 1 | ms | - |  |  | 3 |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Ncell 2 and starts to send preambles on the NPRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Ncell 2.

The cell re-selection delay to a newly detectable cell shall be less than 59.32 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on Ncell 1 and starts to send preambles on the NPRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Ncell 1.

The cell re-selection delay to an already detected cell shall be less than 14.82 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect,NB_Intra_NB-IoT-NC + TSI-NB, and to an already detected cell can be expressed as: Tevaluate, NB_intra_NB-IoT-NC + TSI-NB,

Where:

Tdetect,NB_Intra_NB-IoT-NC	as specified in 3GPP TS 36.133 [4] Table 4.6A.2.2-1 in clause 4.6A.2.2.

Tevaluate, NB_intra_NB-IoT-NC	as specified in 3GPP TS 36.133 [4] Table 4.6A.2.2-1 in clause 4.6A.2.2.

TSI-NB	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 8.32 s is assumed in this test case.

This gives a total of 59.32 s, allow 60 s for the cell re-selection delay to a newly detectable cell and 14.82 s, and allow 15s for the cell re-selection delay to an already detected cell in the test case.

#### 13.1.1.2	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation

Editor's Note: This test case is incomplete in following aspects:

- .

##### 13.1.1.2.1	Test purpose

To verify that when the current and target cell operates on the same carrier frequency the Cat-NB1 UE is able to search and measure cells to meet the intra-frequency cell re-selection requirements with serving cell RRM measurement relaxation.

##### 13.1.1.2.2	Test applicability

This test applies to all types of NB-IoT HD-FDD category NB1 UE supporting wake up signal, relaxed RRM measurements, GSO or both from release 17 and forward.

##### 13.1.1.2.3	Minimum conformance requirements

The cell re-selection delay to a newly detectable cell can be expressed as: Tevaluate, serv_NB-NC + Tdetect,NB_Intra_NB-IoT-NC + TSI.

<TS 36.133 4.6A.2.1>

The UE shall measure the NRSRP and NRSRQ level of the serving NB-IoT cell and evaluate the cell selection criterion S defined in [1] for the serving NB-IoT cell at least every DRX cycle.

The UE shall filter the NRSRP and NRSRQ measurements of the NB-IoT serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2.

If the UE is not configured with eDRX_IDLE cycle and has evaluated according to Table 4.2.38.3-1 in Nserv_NB-IoT-NC consecutive DRX cycles that the serving NB-IoT cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving NB-IoT cell, regardless of the measurement rules currently limiting UE measurement activities.

Table 13.1.1.2.3-1: Nserv_NB-NC

| DRX cycle length [s] | Nserv_NB-IoT-NC [number of DRX cycles] |
| --- | --- |
| 1.28 | 2 |
| 2.56 | 2 |
| 5.12 | 2 |
| 10.24 | 2 |

[TS 36.133 4.6A.2.1A]

The UE which supports wakeUpSignal [2] shall meet the requirement defined for the DRX cycle length of N*DRX_cycle in 4.6.2.1 in 3GPP TS 36.133 [4], provided the following conditions are met:

-	WUS has been configured in the serving NB-IoT cell using WUS-Config-NB-r15 [2], and

-	The serving cell measurement relaxation is signalled as n by the network using numDRX-CycleRelaxed-r15, and

-	Serving cell S criteria is met with at least 2 dB margin.

-	the relaxed monitoring criteria for neighbour cells in 3GPP TS 36.304 [1] clause 5.2.4.12.1 is fulfilled, and

where the relaxation factor N is given by Table 13.1.1.2.3-2. Otherwise the requirements defined for the configured DRX cycle length in Section 4.6.2.1A in 3GPP TS 36.133 [4] shall apply.

Table 13.1.1.2.3-2: The relaxation factor N for a UE not configured with eDRX IDLE cycle

| DRX cycle length [s] | Value |
| --- | --- |
| 1.28 | Min(n , 8) |
| 2.56 | Min(n , 4) |
| 5.12 | Min(n , 2) |
| 10.24 | 1 |
| NOTE: n is signalled by the network by using numDRX-CycleRelaxed-r15 defined in 3GPP TS 36.331 [2]. |  |

[TS 36.133 4.6A.2.2]

The UE shall be able to identify new intra-frequency cells and perform NRSRP measurements of identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in 3GPP TS36.304 within Ksatellite*Tdetect,NB_Intra_NC when Treselection= 0. An intra frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.1.4 for a corresponding Band.

The UE shall measure NRSRP at least every Ksatellite*Tmeasure,NB_Intra_NC for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter NRSRP measurements of each measured intra-frequency cell using at least [2] measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Ksatellite*Tmeasure,NB_Intra-NC/2

The UE shall not consider an NB-IoT neighbour cell in cell reselection if it is indicated as not allowed in the measurement control system information of the serving NB-IoT cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined [1] within Ksatellite*Tevaluate,NB_intra-NC when Treselection = 0, provided that the cell is at least XdB better ranked, where 'X' is specified in Table 4.6.2.4-3. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and non-serving NB-IoT intra-frequency cells.

The parameter Ksatellite is the scaling factor for measurements correspond to multiple NGSO satellites. Ksatellite = 1, if GSO satellite(s) is/are measured on the carrier. Ksatellite  equals to the number NGSO satellites to be measured if NGSO satellites are monitored.

If Treselection timer has a non-zero value and the intra-frequency cell is better ranked than the serving NB-IoT cell, the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

For UE not configured with eDRX_IDLE cycle, Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate, NB_intra_NC are specified in Table 13.1.1.2.3-3.

Table 13.1.1.2.3-3: Tdetect,NB_Intra_NB-IoT-NC, Tmeasure,NB_Intra_NB-IoT-NC and Tevaluate, NB_intra_NB-IoT-NC

| DRX cycle length [s] | Tdetect,NB_Intra_NC [s] (number of DRX cycles) | Tmeasure,NB_Intra_NB_NC [s] (number of DRX cycles) | Tevaluate,NB_intra_NB_NC [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 1.28 | 51 (40) | 1.28 (1) | 6.5 (5) |
| 2.56 | 51 (20) | 2.56 (1) | 7.68 (3) |
| 5.12 | 102 (20) | 5.12 (1) | 10.24 (2) |
| 10.24 | 102 (10) | 10.24 (1) | 20.48 (2) |

The normative reference for this requirement is TS 36.133 [4] clause 4.6A.2.1, 4.6A.2.1A, 4.6A.2.2 and A.13.1.1.2.

##### 13.1.1.2.4	Test description

13.1.1.2.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.1.1.2.5-1.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.93 using only UE main Tx/Rx antenna.

2.	The parameter settings for the cells are set up according to Table 13.1.1.2.4.1-2.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	There are one NB-IoT carrier and two cells specified in the test. Ncell1 is standalone cell to Cell1 and Ncell2 is istandalone cell to Cell2. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

The UE shall be provided with the valid information about the SAN serving cells before the test

Table 13.1.1.2.4-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GEO, HD-FDD duplex mode |

Table 13.1.1.2.4-2: General test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | Standalone |  |
| Satellite information | Config 1 |  | GEO |  |
| Initial condition | Active cell |  | nCell1 |  |
|  | Neighbour cells |  | nCell2 |  |
| T2 end condition | Active cell |  | nCell2 |  |
|  | Neighbour cells |  | nCell1 |  |
| Final condition | Visited cell |  | nCell1 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| s-IntraSearchP |  |  | 63 (126 dB) | to trigger intra-frequency measurement in this test |
| SSearchDeltaP |  | dB | 6 | Threshold for relaxed monitoring criterion as specified in 5.2.4.12.1 in [1] |
| Rmax |  |  | 128 |  |
| maxDurationFactor |  |  | one4th | WUS config. Wmax = 32 (=1/4*Rmax) |
| numPOs |  |  | n1 | WUS config. Single PO mapped to each WUS occasion |
| timeOffsetDRX |  |  | ms40 | WUS config. Gap between the end of WUS duration to the associated PO |
| numDRX-CycleRelaxed |  |  | 4 | Serving cell RRM measurement is relaxed by |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | >30 | During T1, nCell2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that nCell2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 70 | T2 is defined so that cell re-selection time is taken into account. |

13.1.1.2.4.2	Test procedure

The test scenario comprises of one NB-IoT carrier with 2 Ncells of different physical cell ID. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Only Ncell1 is already identified by the UE prior to the start of the test, i.e. Ncell 2 is not identified. Ncell 1 and Ncell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Ncell 2.

In the following test procedure "UE responds" means "UE starts transmitting preamble on NPRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure according to 3GPP TS 36.508 [7] clause 8.1.5A.5"

1.	Ensure the UE is in State 3A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5 in Ncell 1.

2.	Set the parameters according to T1 in Table 13.1.1.2.5-1. Propagation conditions are set according to Annex B clause B.1.1.

3.	Wait for 5mins so that the relaxed monitoring criteria for neighbour cells in 3GPP TS 36.304 [1] clause 5.2.4.12.1 is fulfilled. T1 starts.

4.	Set Ncell2 physical cell identity = ((current Ncell 2 physical cell identity + 1) mod 14 + 2) for one iteration of the test procedure loop.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 13.1.1.2.5-1.

6.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Ncell 2.

7.	If the UE responds on the newly detectable cell, Ncell 2, during time duration T2 within 70 seconds from the beginning of time period T2, then count a success for the event "Re-select newly detected Ncell 2". Otherwise count a fail for the event "Re-select newly detected Ncell 2".

8.	If the UE has re-selected Ncell 2 within T2, after the re-selection or when T2 expires, continue with step 10.
Otherwise, if T2 expires and the UE has not yet re-selected Ncell 2, skip to step 9.

9.	Switch off and on the UE and ensure the UE is in State 3A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5 in Ncell 1.

10.	Repeat step 2-9 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

13.1.1.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.4.3, 8.1.5B and 8.1.6 using condition “standalone” with the following exceptions:

Table 13.1.1.2.4.3-1: SystemInformationBlockType22-NB:
HD-FDD Intra frequency cell reselection for Category NB1 UE for Ncell 1

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.4.3.3, Table 8.1.4.3.3-8: SystemInformationBlockType22-NB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType22-NB-r14 ::= SEQUENCE { |  |  |  |
| dl-ConfigList-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF DL-ConfigCommon-NB-r14 SEQUENCE { |  |  |  |
| DL-ConfigCommon-NB-r14[1] SEQUENCE { |  |  |  |
| wus-Config-r15 SEQUENCE { |  |  |  |
| maxDurationFactor-r15 SEQUENCE { |  |  |  |
| maxDurationFactor-r15 | one4th |  |  |
| } |  |  |  |
| numPOs-r15 | n1 |  |  |
| numDRX-CyclesRelaxed-r15 | 4 |  |  |
| timeOffsetDRX-r15 | ms40 |  |  |
| timeOffset-eDRX-Short-r15 | ms240 |  |  |
| timeOffset-eDRX-Long-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.1.1.2.4.3-2: SystemInformationBlockType3-NB for Ncell 1

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.4.3.3, Table 8.1.4.3.3-8: SystemInformationBlockType3-NB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3-NB-r13 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo-r13 SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -70 (-140 dBm) |  |  |
| q-QualMin-r13 | Not present |  |  |
| p-Max-r13 | Not present |  |  |
| s-IntraSearchP-r13 | 63 (126dB) |  |  |
| t-Reselection-r13 | 0 |  |  |
| } |  |  |  |

##### 13.1.1.2.5	Test requirement

Tables 13.1.1.2.5-1 defines the primary level settings including test tolerances for HD-FDD intra frequency cell re-selection test case.

Table 13.1.1.2.5-1: nCell 1, nCell 2 specific test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in normal coverage

| Parameter | Unit | nCell 1 |  | nCell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| BWchannel | kHz | 200 |  | 200 |  |
| OCNG Pattern as defined in A.3 | - | NOP.3 FDD |  | NOP.3 FDD |  |
| NPBCH_RA | dB | 0 |  | 0 |  |
| NPBCH_RB | dB |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |
| Qrxlevmin | dBm | -140 | -140 | -140 | -140 |
| Pcompensation | dB | 0 | 0 | 0 | 0 |
| Qhysts | dB | 0 | 0 | 0 | 0 |
| Qoffsets, n | dB | 0 | 0 | 0 | 0 |
| Cell_selection_and_ reselection_quality_measurement |  | NRSRP |  | NRSRP |  |
|  | dBm/15 kHz | -98+ |  |  |  |
|  | dB | 17 | 7 | -infinity | 11.45 |
| Note2 | dB | 17 | -4.75 | -infinity | 3.66 |
| NRSRP Note2 | dBm/15 kHz | -81 | -91 | -infinity | -86.55 |
| Treselection | s | 0 | 0 | 0 | 0 |
| Propagation Condition |  | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1x1 |  | 1x1 |  |
| Timing offset to nCell 1 | ms | - |  | 3 |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Before the beginning of T2, UE is under relaxed monitoring where the serving cell measurement is performed every 5.12 s and the infra-frequency measurement for the neighbour cells is relaxed according to subclause 5.2.4.12.0 in 3GPP TS 36.304 [1].

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on nCell 2 and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on nCell 2.

The cell re-selection delay to a newly detectable cell shall be less than 69.56 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on nCell 2 and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on nCell 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tevaluate, serv_NB-NC + Tdetect,NB_Intra_NB-IoT-NC + TSI.

Where:

Tdetect,NB_Intra_NB-IoT-NC	See Table 4.6.2.2-1 in clause 4.6.2.2, based on the configured DRX cycle

Tevaluate, serv_NB-NC	See Table 4.6.2.2-1 in clause 4.6.2.2, based on the effective DRX cycle after relaxation; 10.24 s is assumed in this test case.

TSI	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 8.32 s is assumed in this test case.

This gives a total of 69.56 s, allow 70 s for the cell re-selection delay to a newly detectable in the test case.

For the test to pass, both events above shall pass.

The statistical pass/ fail decisions are done separately for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

#### 13.1.1.3	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX

Editor's Note: This test case is incomplete in following aspects:

##### 13.1.1.3.1	Test purpose

To verify that when the current and target cell operates on the same carrier frequency the Cat-NB1 UE is able to search and measure cells to meet the intra-frequency cell re-selection requirements.

##### 13.1.1.3.2	Test applicability

This test applies to all types of NB-IoT HD-FDD category NB1 UE supporting UE specific DRX, GSO or both from release 17 and forward.

##### 13.1.1.3.3	Minimum conformance requirements

The UE shall be able to identify new intra-frequency cells and perform NRSRP measurements of identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS36.304 within Ksatellite*Tdetect,NB_Intra_NC when Treselection= 0 .  An intra frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in 3GPP TS 36.133 Annex B.1.4 for a corresponding Band.

The UE shall measure NRSRP at least every Ksatellite*Tmeasure,NB_Intra_NC for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter NRSRP measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Ksatellite *Tmeasure,NB_Intra-NC/2

The UE shall not consider an NB-IoT neighbour cell in cell reselection if it is indicated as not allowed in the measurement control system information of the serving NB-IoT cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in 3GPP TS 36.304[6] within Ksatellite *Tevaluate,NB_intra-NC when Treselection = 0, provided that the cell is at least XdB better ranked, where ‘X’ is specified in 3GPP TS 36.133 Table 4.6A.2.4-3. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and non-serving NB-IoT intra-frequency cells.

The parameter Ksatellite is the scaling factor for measurements correspond to multiple NGSO satellites. Ksatellite = 1, if GSO satellite(s) is/are measured on the carrier. Ksatellite  equals to the number NGSO satellites to be measured if NGSO satellites are monitored.

If Treselection timer has a non zero value and the intra-frequency cell is better ranked than the serving NB-IoT cell, the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

The UE shall evaluate the intra-frequency cell re-selection criteria as defined in 3GPP TS 36.304 [6] at least every DRX cycle.

The normative reference for this requirement is TS 36.133 [4] clause 4.6A.2 and A.13.1.1.3.

##### 13.1.1.3.4	Test description

13.1.1.3.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.1.1.3.5-1.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.93 using only UE main Tx/Rx antenna.

2.	The parameter settings for the cells are set up according to Table 13.1.1.3.4.1-2.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	There are one NB-IoT carrier and two cells specified in the test. Ncell1 is standalone cell to Cell1 and Ncell2 is standalone cell to Cell2. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

5.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

6.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

7.	Deactivate UE prediction of satellite trajectory through any preconfigured means

UE shall be provided with the valid information about the SAN serving cells before the test

Table 13.1.1.3.4-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GEO, HD-FDD duplex mode |

Table 13.1.1.3.4-2: General test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in normal coverage

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NB-IOT operational mode |  |  | Standalone |  |  |
| Satellite information | Config 1 |  | GEO |  |  |
| Initial condition | Active cell |  | nCell1 |  |  |
|  | Neighbour cells |  | nCell2 |  |  |
| T2 end condition | Active cell |  | nCell2 |  |  |
|  | Neighbour cells |  | nCell1 |  |  |
| Final condition | Visited cell |  | nCell1 |  |  |
| Access Barring Information |  | - | Not Sent |  | No additional delays in random access procedure. |
| s-IntraSearchP |  |  | 63 (126 dB) |  | to trigger intra-frequency measurement in this test |
| DRX cycle length |  | s | 0.32 | 0.64 | The value shall be used for all cells in the test. |
| T1 |  | s | >7 |  | During T1, nCell2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that nCell2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 35 |  | T2 is defined so that cell re-selection time is taken into account. Once the UE has reselected to nCell2 (within T2) T3 starts |
| T3 |  | s | 14 |  | T3 is defined so that cell re-selection time is taken into account. |

13.1.1.3.4.2	Test procedure

The test scenario comprises of one NB-IoT carrier with 2 Ncells of different physical cell ID. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Only Ncell1 is already identified by the UE prior to the start of the test, i.e. Ncell 2 is not identified. Ncell 1 and Ncell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Ncell 2.

In the following test procedure "UE responds" means "UE starts transmitting preamble on NPRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure according to 3GPP TS 36.508 [7] clause 8.1.5A.5"

1.	Ensure the UE is in State 3A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5 in Ncell 1.

2.	Set the parameters according to T1 in Table 13.1.1.3.5-1 and 13.1.1.3.5-2. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

3.	Set Ncell2 physical cell identity = ((current Ncell 2 physical cell identity + 1) mod 14 + 2) for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 4.2.18.5-1 and 4.2.18.5-2.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Ncell 2.

6.	If the UE responds on the newly detectable cell, Ncell 2, during time duration T2 within 35 seconds from the beginning of time period T2, then count a success for the event "Re-select newly detected Ncell 2". Otherwise count a fail for the event "Re-select newly detected Ncell 2".

7.	If the UE has re-selected Ncell 2 within T2, after the re-selection or when T2 expires, continue with step 8.
Otherwise, if T2 expires and the UE has not yet re-selected Ncell 2, skip to step 12.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 13.1.1.3.5-1.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Ncell 1.

10.	If the UE responds on the already detected cell, Ncell 1, during time duration T3 within 14 seconds from the beginning of time period T3, then count a success for the event "Re-select already detected Ncell 1". Otherwise count a fail for the event "Re-select already detected Ncell 1".

11.	If the UE has re-selected Ncell 1 within T3, after the re-selection or when T3 expires, skip to step 13.
Otherwise, if T3 expires and the UE has not yet re-selected Ncell 1, continue with step 12.

12.	Switch off and on the UE and ensure the UE is in State 3A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5 in Ncell 1.

13.	Repeat step 2-12 until a test verdict has been achieved.
Each of the events "Re-select newly detected Ncell 2" and "Re-select already detected Ncell 1" is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

13.1.1.3.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.4.3, 8.1.5B and 8.1.6 using condition “standalone” with the following exceptions:

Table 13.1.1.3.4.3-1: SystemInformationBlockType2-NB:
HD-FDD Intra frequency cell reselection for Category NB1 UE for Ncell 1 and Ncell2

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.4.3.3, Table 8.1.4.3.3-1: SystemInformationBlockType2-NB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ue-SpecificDRX-CycleMin-r16 | rf32 |  | Test1 |
|  | rf64 |  | Test2 |
| } |  |  |  |

Table 13.1.1.3.4.3-2: ATTACH ACCEPT: HD-FDD Intra frequency cell reselection for Category NB1 UE for step 1

| Derivation Path: 36.508 clause 4.7.2 Table 4.7.2-1 ATTACH ACCEPT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Negotiated DRX parameter in NB-S1 mode |  |  |  |
| NB-S1 mode DRX value | DRX cycle parameter T = 32 |  | Test1 |
|  | DRX cycle parameter T = 64 |  | Test2 |

Table 13.1.1.3.4.3-3: TRACKING AREA UPDATE ACCEPT: HD-FDD Intra frequency cell reselection for Category NB1 UE for for step 6 and step 10

| Derivation Path: 36.508 clause 4.7.2 Table 4.7.2-24 TRACKING AREA UPDATE ACCEPT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Negotiated DRX parameter in NB-S1 mode |  |  |  |
| NB-S1 mode DRX value | DRX cycle parameter T = 32 |  | Test1 |
|  | DRX cycle parameter T = 64 |  | Test2 |

Table 13.1.1.3.4.3-4: ATTACH REQUEST : HD-FDD Intra frequency cell reselection for Category NB1 UE for for step 1

| Derivation Path: 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX parameter in NB-S1 mode | '0000'B | DRX value not specified and use cell specific DRX value |  |

Table 13.1.1.3.4.3-5: TRACKING AREA UPDATE REQUEST : HD-FDD Intra frequency cell reselection for Category NB1 UE for for step 6 and step 10

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| DRX parameter in NB-S1 mode | DRX cycle parameter T = 32 |  | Test1 |
|  | DRX cycle parameter T = 64 |  | Test2 |

Table 13.1.1.3.4.3-6: SystemInformationBlockType3-NB

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.4.3.3, Table 8.1.4.3.3-8: SystemInformationBlockType3-NB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3-NB-r13 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo-r13 SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -70 (-140 dBm) |  |  |
| q-QualMin-r13 | Not present |  |  |
| p-Max-r13 | Not present |  |  |
| s-IntraSearchP-r13 | 63 (126dB) | nCell1, nCell2 |  |
| t-Reselection-r13 | 0 |  |  |
| } |  |  |  |

##### 13.1.1.3.5	Test requirement

Tables 13.1.1.3.5-1 defines the primary level settings including test tolerances for HD-FDD intra frequency cell re-selection with UE specific DRX test case.

Table 13.1.1.3.5-1: nCell 1, nCell 2 specific test parameters for HD-FDD intra frequency cell reselection test case for Cat-NB1 UE in normal coverage

| Parameter | Unit | nCell 1 |  |  | nCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | kHz | 180 |  |  | 180 |  |  |
| OCNG Pattern as defined in A.3 | - | NOP.3 FDD |  |  | NOP.3 FDD |  |  |
| NPBCH_RA | dB | 0 |  |  | 0 |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Qrxlevmin | dBm | -140 | -140 | -140 | -140 | -140 | -140 |
| Pcompensation | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Qhysts | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Qoffsets, n | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Cell_selection_and_ reselection_quality_measurement |  | NRSRP |  |  | NRSRP |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 17 | 13 | 17.45 | -infinity | 17.45 | 13 |
| Note2 | dB | 17 | -4.53 | 4.24 | -infinity | 4.24 | -4.53 |
| NRSRP Note2 | dBm/15 kHz | -81 | -85 | -80.55 | -infinity | -80.55 | -85 |
| Treselection | s | 0 | 0 | 0 | 0 | 0 | 0 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to nCell 1 | ms | - |  |  | 3 |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Ncell 2 and starts to send preambles on the NPRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Ncell 2.

The cell re-selection delay to a newly detectable cell shall be less than 34.32 s in test1 and test2.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on Ncell 1 and starts to send preambles on the NPRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Ncell 1.

The cell re-selection delay to an already detected cell shall be less than 13.44s in test1 and test2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect,NB_Intra_NB-IoT-NC + TSI-NB, and to an already detected cell can be expressed as: Tevaluate, NB_intra_NB-IoT-NC + TSI-NB,

Where:

Tdetect,NB_Intra_NB-IoT-NC	as specified in 3GPP TS 36.133 [4] Table 4.6A.2.2-1 in clause 4.6A.2.2.

Tevaluate, NB_intra_NB-IoT-NC	as specified in 3GPP TS 36.133 [4] Table 4.6A.2.2-1 in clause 4.6A.2.2.

TSI-NB	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 8.32 s is assumed in this test case.

This gives a total of 34.32 s, allow 35 s for the cell re-selection delay to a newly detectable cell and 13.44 s, and allow 14 s for the cell re-selection delay to an already detected cell in the test case.

For the test to pass, both events above shall pass.

The statistical pass/ fail decisions are done separately for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

## 13.3	RRC connection mobility control for satellite access

### 13.3.1

#### 13.3.1.1	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage

Editor's Note: This test case is incomplete in following aspects:

##### 13.3.1.1.1	Test purpose

The purpose is to verify that the NB-IoT FDD intra-frequency RRC re-establishment delay is within the specified limits. These tests will verify the requirements for Cat-NB1 UE in TS36.133 [4] clause 6.5A.

##### 13.3.1.1.2	Test applicability

This test applies to all types of NB-IoT HD-FDD UE release 17 and forward of UE Category NB1 that supports NTN.

##### 13.3.1.1.3	Minimum conformance requirements

In RRC connected mode the UE shall be capable of sending RRCConnectionReestablishmentRequest message within Tre-establish_delay_NB-IoT seconds from the moment it detects a loss in RRC connection.  The total RRC connection delay (Tre-establish_delay_NB-IoT) shall be less than:

Tre-establish_delay_NB-IoT =  TUL_grant + TUE_re-establish_delay_NB-IoT

-	TUL_grant: It is the time required to acquire and process uplink grant from the target cell. The uplink grant is required to transmit RRCConnectionReestablishmentRequest message.

-	The UE re-establishment delay (TUE_re-establish_delay_NB-IoT) is specified in TS36.133 [4] clause 6.5A.2.1 for a UE in normal coverage.

These requirements are not applicable for UEs that only support the Control Plane CIoT EPS optimisation (see TS 24.301). Connection control in NB-IoT is defined in Clause 5.3.1.4 in TS 36.331 [5].

The UE re-establishment delay (TUE_re-establish_delay_NB-IoT) is the time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 in TS 36.331 [5] is detected by the UE to the time when the UE sends PRACH preamble to the target cell. The UE re-establishment delay (TUE_re-establish_delay_NB-IoT) requirement shall be less than:

TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB1-NC + TSI_NB1-NC + TPRACH_NB-IoT

Tsearch_NB1-NC: It is the time required by the UE to search the target cell:

If the target cell is known, then Tsearch_NB1-NC = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch_NB1-NC = 80 ms. Otherwise, Tsearch_NB1-NC = 1400 ms.

-	TSI_NB1-NC: It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [5] for the target cell for a UE in normal coverage.

-	TPRACH_NB-IoT: The additional delay caused by the random access procedure. The actual value of TPRACH_NB-IoT shall depend upon the NPRACH configuration used in the target cell and the number of repetition used by UE for sending random access to the target cell. There might be additional delay due to ramping procedure.

-	NNB-Iot-freq: It is the total number of NB-IoT frequencies to be monitored for RRC re-establishment; NNB-Iot-freq = 1 if the target cell is known.

There is no requirement if the target cell does not contain the UE context.

##### 13.3.1.1.4	Test description

There are two cells in this test case,.nCell1 and nCell2 are NB-IoT cells with different physical cell ID on the same frequency carrier. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

The UE shall be provided with the valid information about the SAN serving cells before the test.

##### 13.3.1.1.4.1	Initial conditions

The test parameters are given in table 13.3.1.1.4.1-1.

Table 13.3.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GEO, HD-FDD duplex mode |

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and TS 36.508 [7] clauses 8.1.4.2 and 8.1.3.1.

Channel Bandwidth to be tested: 200kHz as defined in 3GPP TS 36.508 [7] clause 8.1.3.1.

1.	Connect the SS and AWGN noise sources to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A, Figure A.45 using only main UE Tx/Rx antenna.

2.	The general test parameter settings are set up according to Table 13.3.1.1.4.1-2.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are defined in clause 13.3.1.1.4.3.

5.	There are two cells specified in this test. nCell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test. nCell 1 and nCell 2 are NB-IoT cells with different physical cell ID on the same frequency carrier.

6.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

8.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

Table 13.3.1.1.4.1-2: General test parameters for HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | Standalone |  |
| Initial condition | Active cell |  | nCell1 |  |
|  | Neighbour cells |  | nCell2 |  |
| Final condition | Active cell |  | nCell2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| NPDCCH repetition level |  |  | 16 | NPDCCH Rmax |
| N310 |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | Ms | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | Ms | 15000 | RRC re-establishment timer |
| DRX |  |  | OFF |  |
| T1 |  | S | 5 |  |
| T2 |  | Ms | 400 |  |
| T3 |  | S | 15 |  |

##### 13.3.1.1.4.2	Test procedure

The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, nCell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

- Ensure the UE is in State 2A-NB with UP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. nCell 1 is the active cell.

- Set the parameters according to T1 in Table 13.3.1.5-1. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

- When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 13.3.1.1.5-1. T2 starts.

- When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 13.3.1.1.5-1. T3 starts.

- If the UE starts to send NPRACH preambles to nCell 2 for sending the RRCConnectionReestablishmentRequest-NB message to nCell 2 within 10.6s from the beginning of time period T3, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

- Once the UE sends a prach in step 5, switch off the UE. Then ensure the UE is in State 2A-NB with UP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. nCell 1 is the active cell.

- Repeat step 2-6 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

##### 13.3.1.1.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 8.1.6 and clause 8.1.4.3 using condition "Standalone" and “NTN”.

##### 13.3.1.1.5	Test requirement

Table 13.3.1.1.5-1: nCell 1, nCell 2 specific test parameters for HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage

| Parameter | Unit | nCell 1 |  |  | nCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | kHz | 200 |  |  | 200 |  |  |
| NPDSCH parameters |  | R.18 HD-FDD |  |  | R.18 HD-FDD |  |  |
| NPDCCH parameters |  | R.30 HD-FDD |  |  | R.30 HD-FDD |  |  |
| NOCNG Patterns |  | NOP.3 FDD |  |  | NOP.3 FDD |  |  |
| NPBCH_RA | dB | 0 |  |  | 0 |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 7+TT | -Infinity | -Infinity | -Infinity | 4+TT | 4+TT |
| Note2 | dB | 7+TT | -Infinity | -Infinity | -Infinity | 4+TT | 4+TT |
| NRSRP Note2 | dBm/15 kHz | -91+TT | -Infinity | -Infinity | -Infinity | -94+TT | -94+TT |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to nCell 1 | ms | - |  |  | 3 |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send NPRACH preambles to cell 2 for sending the RRCConnectionReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NB-IoT FDD intra frequency cell shall be less than 10.6s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE-re-establish_delay_NB-IoT.

Where:

-	TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The NPRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

-	TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB-IoT + TSI_NB-IoT + TPRACH_NB-IoT

-	NNB-Iot-freq = 1

-	Tsearch_NB-IoT = 1400 ms

-	TSI_NB-IoT = 8320 ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target NB-IoT FDD cell.

-	TPRACH_NB-IoT = 80 ms; it is the additional delay caused by the random access procedure.

#### 13.3.1.2	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage

Editor's Note: This test case is incomplete in following aspects:

##### 13.3.1.2.1	Test purpose

The purpose is to verify that the NB-IoT FDD intra-frequency RRC re-establishment delay is within the specified limits. These tests will verify the requirements for Cat-NB1 UE in TS36.133 [4] clause 6.5A.

##### 13.3.1.2.2	Test applicability

This test applies to all types of NB-IoT HD-FDD UE release 17 and forward of UE Category NB1 that supports NTN.

Applicability requires support for the User Plane CIoT EPS Optimization for RRCConnectionReestablishment.

##### 13.3.1.2.3	Minimum conformance requirements

In RRC connected mode the UE shall be capable of sending RRCConnectionReestablishmentRequest message within Tre-establish_delay_NB-IoT seconds from the moment it detects a loss in RRC connection.  The total RRC connection delay (Tre-establish_delay_NB-IoT) shall be less than:

Tre-establish_delay_NB-IoT =  TUL_grant + TUE_re-establish_delay_NB-IoT

-	TUL_grant: It is the time required to acquire and process uplink grant from the target cell. The uplink grant is required to transmit RRCConnectionReestablishmentRequest message.

-	The UE re-establishment delay (TUE_re-establish_delay_NB-IoT) is specified in TS36.133 [4] clause 6.5A.2.2 for a UE in enhanced coverage.

These requirements are not applicable for UEs that only support the Control Plane CIoT EPS optimisation (see TS 24.301). Connection control in NB-IoT is defined in Clause 5.3.1.4 in TS 36.331 [5].

The UE re-establishment delay (TUE_re-establish_delay_NB-IoT) is the time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 in TS 36.331 [5] is detected by the UE to the time when the UE sends PRACH preamble to the target cell. The UE re-establishment delay (TUE_re-establish_delay_NB-IoT) requirement shall be less than:

TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB1-EC + TSI_NB1-EC + TPRACH_NB-IoT

-	Tsearch_NB1-EC: It is the time required by the UE to search the target cell:

-	If the target cell is known, then Tsearch_NB1-EC = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch_NB1-EC = [Ksatellite *80] ms. Otherwise, Tsearch_NB1-EC = [Ksatellite *14800] ms. Where Ksatellite = 1, if GSO satellite(s) is/are measured on the carrier.

-	TSI_NB1-EC: It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [2] for the target cell for a UE in enhanced coverage.

-	TPRACH_NB-IoT: The additional delay caused by the random access procedure. The actual value of TPRACH_NB-IoT shall depend upon the NPRACH configuration used in the target cell and the number of repetition used by UE for sending random access to the target cell. There might be additional delay due to ramping procedure.

-	NNB-Iot-freq: It is the total number of NB-IoT frequencies to be monitored for RRC re-establishment; NNB-Iot-freq = 1 if the target cell is known.

There is no requirement if the target cell does not contain the UE context.

Editor’s note: FFS Tsi-to-epochTsi-to-epoch to accommodate for the time needed between the UE acquires the NTN SIB of the target cell and the epoch time conveyed in this SIB is reached.

##### 13.3.1.2.4	Test description

There are two cells in this test case,.nCell1 and nCell2 are NB-IoT cells with different physical cell ID on the same frequency carrier. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

The UE shall be provided with the valid information about the SAN serving cells before the test.

13.3.1.2.4.1	Initial conditions

The test parameters are given in table 13.3.1.2.4.1-1.

Table 13.3.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GEO, HD-FDD duplex mode |

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and TS 36.508 [7] clauses 8.1.4.2 and 8.1.3.1.

Channel Bandwidth to be tested: 200kHz as defined in 3GPP TS 36.508 [7] clause 8.1.3.1.

1.	Connect the SS and AWGN noise sources to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A, Figure A.45 using only main UE Tx/Rx antenna.

2.	The general test parameter settings are set up according to Table 13.3.1.2.4.1-2.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are defined in clause 13.3.1.2.4.3.

5.	There are two cells specified in this test. nCell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test. nCell 1 and nCell 2 are NB-IoT cells with different physical cell ID on the same frequency carrier.

6.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

8.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

Table 13.3.1.2.4-2: General test parameters for HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | Standalone |  |
| Initial condition | Active cell |  | nCell1 |  |
|  | Neighbour cells |  | nCell2 |  |
| Final condition | Active cell |  | nCell2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| NPDCCH repetition level |  |  | 16 | NPDCCH Rmax |
| N310 |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | Ms | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | Ms | 60000 | RRC re-establishment timer |
| DRX |  |  | OFF |  |
| T1 |  | S | 5 |  |
| T2 |  | Ms | 400 |  |
| T3 |  | S | 60 |  |

13.3.1.2.4.2	Test procedure

The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, nCell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

- Ensure the UE is in State 2A-NB with UP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. nCell 1 is the active cell.

- Set the parameters according to T1 in Table 13.3.1.2.5-1. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

- When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 13.3.1.2.5-1. T2 starts.

- When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 13.3.1.2.5-1. T3 starts.

- If the UE starts to send NPRACH preambles to nCell 2 for sending the RRCConnectionReestablishmentRequest-NB message to nCell 2 within 58s from the beginning of time period T3, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

- Once the UE sends a prach in step 5, switch off the UE. Then ensure the UE is in State 2A-NB with UP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. nCell 1 is the active cell.

- Repeat step 2-6 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

##### 13.3.1.2.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 8.1.6 and clause 8.1.4.3 using condition "Standalone" and “NTN” with the following exceptions:

Table 6.1.15.4.3-1: SystemInformationBlockType1-NB

| Derivation Path: 36.508 Table 8.1.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| cellSelectionInfo-r13 SEQUENCE { |  |  |  |
| q-QualMin-r13 | -25 (-25dB) |  |  |
| } |  |  |  |
| } |  |  |  |

##### 13.3.1.2.5	Test requirement

Table 13.3.1.2.5-1: nCell 1, nCell 2 specific test parameters for HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage

| Parameter | Unit | nCell 1 |  |  | nCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | kHz | 200 |  |  | 200 |  |  |
| NPDSCH parameters |  | R.18 HD-FDD |  |  | R.18 HD-FDD |  |  |
| NPDCCH parameters |  | R.30 HD-FDD |  |  | R.30 HD-FDD |  |  |
| NOCNG Patterns |  | NOP.3 FDD |  |  | NOP.3 FDD |  |  |
| NPBCH_RA | dB | 0 |  |  | 0 |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 7+TT | -Infinity | -Infinity | -Infinity | -12.6+TT | -12.6+TT |
| Note2 | dB | 7+TT | -Infinity | -Infinity | -Infinity | -12.6+TT | -12.6+TT |
| NRSRP Note2 | dBm/15 kHz | -91+TT | -Infinity | -Infinity | -Infinity | -110.6+TT | -110.6+TT |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to nCell 1 | ms | - |  |  | 3 |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send NPRACH preambles to cell 2 for sending the RRCConnectionReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NB-IoT FDD intra frequency cell shall be less than 58 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE-re-establish_delay_NB-IoT.

Where:

-	TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The NPRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

-	TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB-IoT + TSI_NB-IoT + TPRACH_NB-IoT

-	NNB-Iot-freq = 1

-	Tsearch_NB-IoT = 14800 ms

-	TSI_NB-IoT = 41560 ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target NB-IoT FDD cell.

-	TPRACH_NB-IoT = 1280 ms; it is the additional delay caused by the random access procedure.

# < Unchanged sections omitted >

## 13.4	Timing and signalling characteristics for satellite access

### 13.4.1	UE transmit timing for satellite access

#### 13.4.1.1	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access

Editor's Note: This test case is incomplete in following aspects:

Connection diagram is TBD.

##### 13.4.1.1.1	Test purpose

The purpose of this test is to verify that the Category NB1 UE under normal coverage is capable of following the frame timing change of the connected eNodeB and that the UE initial transmits timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in TS 36.133 [4] clause 7.20A.

##### 13.4.1.1.2	Test applicability

This test applies to all types of NB-IoT HD-FDD UE release 17 and forward of UE Category NB1 that supports NTN.

##### 13.4.1.1.3	Minimum conformance requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in 3GPP TS 36.133 [4] clause 7.20A and shown in Table 13.4.1.1.3-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×TsNTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA_Ref for NPRACH is defined as 0. NTA_Ref+NTAoffset+NTA,common+NTA, UE-specificNTA_Ref+NTAoffset+NTA,common+NTA, UE-specific (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in 3GPP TS 36.133 clause 7.22A was applied. NTA_Ref for other channels is not changed until next timing advance is received.

Table 13.4.1.1.3-1: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 97– TGNSS_margin*TS |
| Note 1: TS is the basic timing unit defined in 3GPP TS 36.211 [9] |  |

TGNSS_margin is the margin for the GNSS position definition error considered in the core requirement, which needs to be substracted for the test requirement, due to the usuage of AT commands in the test. 
TGNSS_margin = [5.12]TS

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for NPUSCH the UE shall, when no repetitions are configured on the uplink or the repetition period is R=1, be capable of changing the transmission timing according to the received downlink frame of the serving NB-IoT cell except when the timing advance in 3GPP TS 36.133 clause 7.22A is applied such that the UE transmission timing error shall be less than or equal to ±Te, where the timing error limit value Te is specified in table 13.4.1.1.3-1.

When no repetition period is configured, or the configured repetition period is R=1, all adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change, apart from a change of NTA, UE-specificNTA, UE-specific due to satellite position update and NTA,commonNTA,common between the previous transmission and the current transmission, in one adjustment shall be 58.33*TS seconds.

2)	The minimum aggregate adjustment rate, apart from a change of NTA, UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, shall be 7*TS per 1second.

3)	The maximum aggregate adjustment rate, apart from a change of NTA, UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, shall be 58.33*TS per 200ms.

when a repetition is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary, as defined above.

The normative reference for this requirement is 3GPP TS 36.133 [4] clause 7.20A and A.13.4.1.1.

##### 13.4.1.1.4	Test description

There is one NB-IoT SAN cell (Ncell 1), which is the active cell in the test. The transmit timing is verified by the UE transmitting NPUSCH.

Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The UE shall be provided with the valid information about the SAN serving cells before the test.

13.4.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 13.4.1.1.4.1-1.

Table 13.4.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table Annex E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: 200 KHz as defined in 3GPP TS 36.508 [7] clause 8.1.3.1.

1.	Connection diagram is TBD.

2.	The general test parameter settings are set according to Table 13.4.1.1.4.1-2.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are defined in clause 13.4.1.1.4.3.

5.	There is one cell specified in this test. Ncell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test.

6.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

7.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

Table 13.4.1.1.4.1-2: General Test Parameters for E-UTRAN HD-FDD UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| NB-IoT Operation mode |  |  | Standalone |
| DRX |  |  | OFF |
| Satellite information | Config 1 |  | SSC.1 |
|  | Config 2 |  | SSC.2 |
| NPDCCH repetition level |  |  | 1 |
| npdcch-StartSF-USS Note 1 |  |  | v8 |
| npdcch-NumRepetitions-r13 Note 1 |  |  | r1 |
| NPUSCH repetition level |  |  | 1 |
| Note 1: For further information see clause 6.7.3.2 in TS 36.331 [5]. |  |  |  |

13.4.1.1.4.2	Test procedure

The test consists of a single cell Ncell 1, the UE shall be fully synchronized to Ncell 1. The transmit timing accuracy is verified related to the downlink frame timing of Ncell 1. The downlink timing of Ncell 1 is changed and the changes in UE transmit timing are observed. The transmit timing is verified by the UE transmitting NPUSCH used as a measurement reference facilitating the SS timing estimation.

1.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

2.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5.

3.	Set the parameters according to Table 13.4.1.1.5-1. Propagation conditions are set according to Annex B clause B.1.

4.	SS sends uplink scheduling information via NPDCCH DCI format N0 for C_RNTI to schedule uplink subframes.

5.	The UE shall transmit NPUSCH, and the SS shall measure the UE transmit timing offset (nTs).

6.	Using the value of n measured in step 4, the SS calculates the adjustment of the downlink transmit timing for the Ncell 1:

-	if n < 0, by +(144 - |n|)TS compared to that in step 4.

-	 if n ≥ 0, by -(144 - |n|)TS compared to that in step 4.

7.	Before applying the downlink transmit timing adjustment, the SS sends NPDCCH including uplink grant, measures the timing of the resulting NPUSCH, and verifies that it is within the limits specified in Table 13.4.1.1.5-2 with respect to the first detected path (in time) of the corresponding downlink frame of NB-IoT cell 1. The SS also records the measured value for use in step 8.

8.	Immediately after Step 6, the downlink timing adjustment is performed monotonically in multiple steps of |∆T| ≤ 9×TS per 256ms until the above required total timing change is achieved, during which no grant is transmitted for the UE.

9.	Immediately after Step 7, the SS sends NPDCCH including uplink grant for NPUSCH transmission and immediately after receiving NPUSCH the test system repeatedly sends NPDCCH including uplink grant for NPUSCH transmission. The SS samples the UE Transmit Timing once per NPUSCH transmission. The SS shall check that the maximum time adjustment step size between the first NPUSCH transmission in step 8 and the NPUSCH transmission in step 6 is within Rule 1 as specified in clause 13.4.1.1.5. And using the first NPUSCH transmission in step 8 and subsequent NPUSCH transmissions, the SS shall check the difference in timing between one NPUSCH transmission to the next is within Rule 1 as specified in clause 13.4.1.1.5. To check that the minimum adjustment rate is within Rule 2 as specified in clause 13.4.1.1.5, the SS shall measure the change in NPUSCH transmission timing over a 1008ms sliding window, with step size 8ms. To check that the maximum adjustment rate is within Rule 3 as specified in clause 13.4.1.1.5, the SS shall measure the change in NPUSCH transmission timing over a 192ms sliding window, with step size 8ms. The three rules apply until the UE transmit timing offset is within the limits specified in Table 13.4.1.1.5-2 with respect to the first detected path (in time) of the corresponding downlink frame of Ncell 1.

10.	The SS shall check that the UE transmit timing offset stays within the limits specified in Table 13.4.1.1.5-2 with respect to the first detected path (in time) of the corresponding downlink frame of Ncell 1.

11.	Repeat steps 1-10 with ephemeris values for minimum Doppler, maximum positive Doppler and maximum negative Doppler replacing ephemeris in step 1 by corresponding tables in TS 36.508 [12] clause 8.4.6.2.2 for the type of satellite under test.

13.4.1.1.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 8.1.6 and clause 8.1.4.3 using condition "Standalone" and “NTN” with the following exceptions:

Table 13.4.1.1.4.3-1: NPDCCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.6.3, Table 8.1.6.3-3: NPDCCH-ConfigDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDCCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| npdcch-NumRepetitions-r13 | r1 | Set NPDCCH repetition to 1 |  |
| npdcch-StartSF-USS-r13 | V8 |  |  |
| npdcch-Offset-USS-r13 | zero |  |  |
| } |  |  |  |

##### 13.4.1.1.5	Test requirement

Table 13.4.1.1.5-1: Cell specific Test Parameters for E-UTRAN HD-FDD UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 1 |
| RF Channel Number |  | 1 |
| BWchannel | kHz | 200 |
| NPDSCH parameter |  | R.18 HD-FDD |
| NPDCCH parameter |  | R.30 HD-FDD |
| NOCNG Patterns |  | NOP.3 FDD |
| NPBCH_RA | dB | 0 |
| NPBCH_RB |  |  |
| NPSS_RA |  |  |
| NSSS_RA |  |  |
| NPDCCH_RA |  |  |
| NPDCCH_RB |  |  |
| NPDSCH_RA |  |  |
| NPDSCH_RB |  |  |
| NOCNG_RA Note1 |  |  |
| NOCNG_RB Note1 |  |  |
|  | dBm/15 kHz | -88 |
|  | dB | 4.3 |
|  | dB | 4.3 |
| Antenna Configuration |  | 1x1 |
| Propagation condition | - | AWGN |
| Note 1 NOCNG shall be used such that the cell is fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

The UE transmit timing offset shall be within the requirements in Table 13.4.1.1.5-2.

Table 13.4.1.1.5-2: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | (97 – TGNSS_margin +3)*TS |
| NOTE 1: TS is the basic timing unit defined in 3GPP TS 36.211 [9] |  |

The reference point for the UE initial transmit timing control test requirement shall be the downlink timing minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×TsNTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts.

The UE shall be capable of changing the transmission timing according to the received downlink frame. When the transmission timing error between the UE and the reference cell exceeds Te the UE is required to adjust its timing to within Te.

All adjustments made to the UE uplink timing shall follow three rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be (58.33+0.5)*TS seconds.

2)	The minimum aggregate adjustment rate shall be (7-3.6)*TS per (1+0.008) second.

3)	The maximum aggregate adjustment rate shall be (58.33+1.1)*TS per (200-8)ms.

#### 13.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access

Editor's Note: This test case is incomplete in following aspects:- Connection diagram is TBD.

- How to get UE location is TBD.

##### 13.4.1.2.1	Test purpose

The purpose of this test is to verify that the Category NB1 UE under enhanced coverage is capable of following the frame timing change of the connected eNode B, that the UE initial transmit timing accuracy is within the specified limits and that the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission. This test will verify the requirements in TS 36.133 [4] clause 7.20A.

##### 13.4.1.2.2	Test applicability

This test applies to all types of NB-IoT HD-FDD UE release 17 and forward of UE Category NB1 that supports NTN.

##### 13.4.1.2.3	Minimum conformance requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in 3GPP TS 36.133 [4] clause 7.20A and shown in Table 13.4.1.2.3-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×TsNTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA_Ref for NPRACH is defined as 0. NTA_Ref+NTAoffset+NTA,common+NTA, UE-specificNTA_Ref+NTAoffset+NTA,common+NTA, UE-specific (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in 3GPP TS 36.133 clause 7.22A was applied. NTA_Ref for other channels is not changed until next timing advance is received.

Table 13.4.1.2.3-1: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 97– TGNSS_margin*TS |
| Note 1: TS is the basic timing unit defined in 3GPP TS 36.211 [9] |  |

TGNSS_margin is the margin for the GNSS position definition error considered in the core requirement, which needs to be substracted for the test requirement, due to the usuage of AT commands in the test. 
TGNSS_margin = [5.12]TS

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for NPUSCH the UE shall, when no repetitions are configured on the uplink or the repetition period is R=1, be capable of changing the transmission timing according to the received downlink frame of the serving NB-IoT cell except when the timing advance in 3GPP TS 36.133 clause 7.22A is applied such that the UE transmission timing error shall be less than or equal to ±Te, where the timing error limit value Te is specified in table 13.4.1.2.3-1.

When no repetition period is configured, or the configured repetition period is R=1, all adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change, apart from a change of NTA, UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, in one adjustment shall be 58.33*TS seconds.

2)	The minimum aggregate adjustment rate, apart from a change of NTA, UE-specific due to satellite position update and NTA,commonNTA,common between the previous transmission and the current transmission, shall be 7*TS per 1second.

3)	The maximum aggregate adjustment rate, apart from a change of NTA, UE-specific due to satellite position update and NTA,commonNTA,common between the previous transmission and the current transmission, shall be 58.33*TS per 200ms.

when a repetition is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary, as defined above.

The normative reference for this requirement is 3GPP TS 36.133 [4] clause 7.20A and A.13.4.1.2.

##### 13.4.1.2.4	Test description

There is one NB-IoT SAN cell (Ncell 1), which is the active cell in the test. The transmit timing is verified by the UE transmitting NPUSCH. The test consists of two subtests of Test 1 and Test 2 respectively.

During Test 1, DRX configuration is disabled. The UE transit timing offset shall be verified for the first transmission in a repetition period (R>1) for NPUSCH.

During Test 2, DRX configuration is enabled. In the on-duration part of every DRX cycle, the UE transmit timing offset shall be verified for the first transmission in a repetition period (R>1) for NPUSCH.Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The UE shall be provided with the valid information about the SAN serving cells before the test.

13.4.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 13.4.1.2.4.1-1.

Table 13.4.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table Annex E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: 200 KHz as defined in 3GPP TS 36.508 [7] clause 8.1.3.1.

1.	Connection diagram is TBD.

2.	The general test parameter settings are set according to Table 13.4.1.2.4.1-2.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are defined in clause 13.4.1.2.4.3.

5.	There is one cell specified in this test. Ncell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test.

6.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

7.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

Table 13.4.1.2.4.1-2: General Test Parameters for E-UTRAN HD-FDD UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| NB-IoT Operation mode |  |  | Standalone | Standalone |
| Satellite information | Config 1 |  | SSC.1 | SSC.1 |
|  | Config 2 |  | SSC.2 | SSC.2 |
| DRX |  |  | OFF | ON |
| NPDCCH repetition level |  |  | 32 | 32 |
| npdcch-StartSF-USS Note 2 |  |  | v2 | v2 |
| npdcch-NumRepetitions-r13 Note 2 |  |  | r32 | r32 |
| NPUSCH resource units |  |  | 1 | 1 |
| NPUSCH repetition level |  |  | 128 | 128 |
| NPUSCH subcarrier spacing |  | kHz | 15 | 15 |
| NPUSCH number of subcarriers |  |  | 1 | 1 |
| NPUSCH modulation |  |  |  /4QPSK |  /4QPSK |
| NPUSCH Transport block size |  | Bits | 40 | 40 |
| Note 1: DRX related parameters are defined in Table 13.4.1.2.5-2. Note 2: For further information see clause 6.7.3.2 in 3GPP TS 36.331 [5]. |  |  |  |  |

13.4.1.2.4.2	Test procedure

The test consists of a single cell Ncell 1, the UE shall be fully synchronized to Ncell 1. The transmit timing accuracy is verified related to the downlink frame timing of Ncell 1. The downlink timing of Ncell 1 is changed and the changes in UE transmit timing are observed. The transmit timing is verified by the UE transmitting NPUSCH used as a measurement reference facilitating the SS timing estimation.

1.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

2.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5.

3.	Set Test 1 or Test 2 parameters according to Table 13.4.1.2.5-1 as appropriate. Propagation conditions are set according to Annex B clause B.1.

4.	SS sends uplink scheduling information via NPDCCH DCI format N0 for C_RNTI to schedule uplink subframes.

5.	The UE shall transmit NPUSCH. The SS shall measure the UE transmit timing offset (nTs).

6.	The SS verifies the transmission timing offset in a repetition period (R>1) for NPUSCH. If it’s the first transmission timing offset in the repetition period, the SS verifies that it’s within the limits specified in Table 13.4.1.2.5-3 with respect to the first detected path (in time) of the corresponding downlink frame of Ncell 1. If it’s not the first transmission timing offset in the repetition period, the SS verifies that the UE transmit timing offset doesn’t change. If UE adjusts the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission, this subtest fails, and goto step 9. For Test 2, in the on-duration part of every DRX cycle, the UE transmit timing offset in a repetition period (R>1) for NPUSCH shall be verified.

7.	Using the value of n measured in step 4, the SS calculates the total adjustment value of the downlink transmit timing for the Ncell 1:

-	if n < 0, by +(144 - |n|)TS compared to that in step 4.

-	 if n ≥ 0, by -(144 - |n|)TS compared to that in step 4.

8.	After 16ms from the initial NPUSCH transmission in step 4, the test system adjusts the downlink transmit timing for the Ncell 1, using the value of n measured in step 4. The downlink timing adjustment is performed monotonically in multiple steps of |∆T| ≤ 9×TS per 256ms until the above required total timing change is achieved, during which no grant is transmitted for the UE.

9.	The SS shall check that the UE first transmit timing offset in the repetition period (R>1) stays within the limits specified in Table 13.4.1.2.5-3 with respect to the first detected path (in time) of the corresponding downlink frame of Ncell 1. For Test 2, in the on-duration part of every DRX cycle, the UE transmit timing offset in a repetition period (R>1) for NPUSCH shall be verified.

10.	The SS shall transmit RRCConnectionRelease-NB message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

11.	After the RRC connection release, the SS:

- transmits in Ncell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5 (if the paging fails, switches off and on the UE and ensures the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5),

or

- switches off and on the UE and ensures the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5.

12.	Repeat step 2-10 for each subtest in Table 13.4.1.2.5-1 as appropriate.

13.	Repeat step 1-12 with ephemeris values for minimum Doppler, maximum positive Doppler and maximum negative Doppler replacing ephemeris in step 1 by corresponding tables in TS 36.508 [12] clause 8.4.6.2.2 for the type of satellite under test.

13.4.1.2.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 8.1.6 and clause 8.1.4.3 using condition "Standalone" and “NTN” with the following exceptions:

Table 13.4.1.2.4.3-1: NPDCCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.6.3, Table 8.1.6.3-3: NPDCCH-ConfigDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDCCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| npdcch-NumRepetitions-r13 | r32 | Set NPDCCH repetition to 32 |  |
| npdcch-StartSF-USS-r13 | V2 |  |  |
| npdcch-Offset-USS-r13 | zero |  |  |
| } |  |  |  |

Table 13.4.1.2.4.3-2: NB-IoT Physical layer parameters for DCI format N0

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.3.6, Table 8.1.3.6.1.1-1: NB-IoT Physical layer parameters for DCI format N0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | Value | Value in binary | Comment | Condition |
| Repetition number | 7 | “111” | Set NPUSCH repetition to 128 |  |
| DCI subframe repetition number | 32 | “11” | Set NPDCCH repetition to 32 |  |

Table 13.4.1.2.4.3-3: RRCConnectionSetup-NB

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.6.1, Table 8.1.6.1-14: RRCConnectionSetup-NB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13{ |  |  |  |
| srb-ToAddModList-r13 SEQUENCE (SIZE (1)) OF SEQUENCE {} |  |  |  |
| mac-MainConfig CHOICE { | MAC-MainConfig-NB-SRB |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.4.1.2.4.3-4: MAC-MainConfig-NB-SRB

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.8.2, Table 8.1.8.2.1.5-1: MAC-MainConfig-NB-SRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-MainConfig-NB-SRB ::= SEQUENCE { |  |  |  |
| drx-Config-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| onDurationTimer-r13 | pp1 |  |  |
| drx-InactivityTimer-r13 | pp0 |  |  |
| drx-RetransmissionTimer-r13 | pp0 |  |  |
| drx-Cycle-r13 | sf2048 |  |  |
| drx-StartOffset-r13 | 0 |  |  |
| drx-ULRetransmissionTimer-r13 | pp0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 13.4.1.2.5	Test requirement

Table 13.4.1.2.5-1: Cell specific Test Parameters for E-UTRAN HD-FDD UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |
| RF Channel Number |  | 1 | 1 |
| BWchannel | kHz | 200 | 200 |
| NPDSCH parameter |  | R.18 HD-FDD | R.18 HD-FDD |
| NPDCCH parameter |  | R.30 HD-FDD | R.30 HD-FDD |
| NOCNG Patterns |  | NOP.3 FDD | NOP.3 FDD |
| NPBCH_RA | dB | 0 | 0 |
| NPBCH_RB |  |  |  |
| NPSS_RA |  |  |  |
| NSSS_RA |  |  |  |
| NPDCCH_RA |  |  |  |
| NPDCCH_RB |  |  |  |
| NPDSCH_RA |  |  |  |
| NPDSCH_RB |  |  |  |
| NOCNG_RA Note1 |  |  |  |
| NOCNG_RB Note1 |  |  |  |
|  | dBm/15 kHz | -88 | -88 |
|  | dB | -10.7 | -10.7 |
|  | dB | -10.7 | -10.7 |
| Antenna Configuration |  | 1x1 | 1x1 |
| Propagation condition | - | AWGN | AWGN |
| Note 1 NOCNG shall be used such that the cell is fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 13.4.1.2.5-2: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 for E-UTRAN HD-FDD Category NB1 UE Standalone mode under enhanced coverage for Satellite Access

| Field | Value | Comment |
| --- | --- | --- |
|  | Test 2 |  |
| onDurationTimer | pp1 |  |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| longDRX-CycleStartOffset | sf2048 |  |
| shortDRX | disable |  |
| Note 1: For further information see clause 6.7.3 in TS 36.331 [5]. |  |  |

The UE transmit timing offset shall be within the requirements in Table 13.4.1.2.5-3.

Table 13.4.1.2.5-3: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | (97 – TGNSS_margin +3)*TS |
| NOTE 1: TS is the basic timing unit defined in 3GPP TS 36.211 [9] |  |

The reference point for the UE initial transmit timing control test requirement shall be the downlink timing minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×TsNTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts.

The UE shall be capable of changing the transmission timing according to the received downlink frame. When the transmission timing error between the UE and the reference cell exceeds Te, the UE is required to adjust its timing to within Te.

If DRX configuration is enabled, the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

#### 13.4.1.3	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access

Editor's Note: This test case is incomplete in following aspects:

- initial conditions contains [].

##### 13.4.1.3.1	Test purpose

The purpose of this test is to verify that the Category NB1 UE, which is not supporting the capability of ntn-SegmentedPrecompensationGaps-r17, under enhanced coverage is capable of following the frame timing change of the connected eNode B, that the UE initial transmit timing accuracy is within the specified limits and that the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary. This test will verify the requirements in TS 36.133 [4] clause 7.20A

##### 13.4.1.3.2	Test applicability

This test applies to all types of NB-IoT HD-FDD UE release 17 and forward of UE Category NB1 that supports NGSO.

##### 13.4.1.3.3	Minimum conformance requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in 3GPP TS 36.133 [4] clause 7.20A and shown in Table 13.4.1.3.3-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×TsNTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA_Ref for NPRACH is defined as 0. NTA_Ref+NTAoffset+NTA,common+NTA, UE-specificNTA_Ref+NTAoffset+NTA,common+NTA, UE-specific (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in 3GPP TS 36.133 clause 7.22A was applied. NTA_Ref for other channels is not changed until next timing advance is received.

Table 13.4.1.3.3-1: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 97– TGNSS_margin*TS |
| Note 1: TS is the basic timing unit defined in 3GPP TS 36.211 [9] |  |

TGNSS_margin is the margin for the GNSS position definition error considered in the core requirement, which needs to be substracted for the test requirement, due to the usuage of AT commands in the test. 
TGNSS_margin = [5.12]TS

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for NPUSCH the UE shall, when no repetitions are configured on the uplink or the repetition period is R=1, be capable of changing the transmission timing according to the received downlink frame of the serving NB-IoT cell except when the timing advance in 3GPP TS 36.133 clause 7.22A is applied such that the UE transmission timing error shall be less than or equal to ±Te, where the timing error limit value Te is specified in table 13.4.1.3.3-1.

When no repetition period is configured, or the configured repetition period is R=1, all adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change, apart from a change of NTA, UE-specificNTA, UE-specific due to satellite position update and NTA,commonNTA,common between the previous transmission and the current transmission, in one adjustment shall be 58.33*TS seconds.

2)	The minimum aggregate adjustment rate, apart from a change of NTA, UE-specificNTA, UE-specific due to satellite position update and NTA,commonNTA,common between the previous transmission and the current transmission, shall be 7*TS per 1second.

3)	The maximum aggregate adjustment rate, apart from a change of NTA, UE-specificNTA, UE-specific due to satellite position update and NTA,commonNTA,common between the previous transmission and the current transmission, shall be 58.33*TS per 200ms.

when a repetition is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary, as defined above.

The normative reference for this requirement is 3GPP TS 36.133 [4] clause 7.20A and A.13.4.1.3.

##### 13.4.1.3.4	Test description

There is one NB-IoT SAN cell (Ncell 1), which is the active cell in the test. The transmit timing is verified by the UE transmitting NPUSCH. The UE shall be provided with the valid information about the SAN serving cells before the test.

13.4.1.3.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table Annex E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: 200 KHz as defined in 3GPP TS 36.508 [7] clause 8.1.3.1.

1.	Connection diagram is TBD.

2.	The general test parameter settings are set according to Table 13.4.1.3.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are defined in clause 13.4.1.3.4.3.

5.	There is one cell specified in this test. Ncell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test.

6.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

7.	Deactivate UE prediction of satellite trajectory through any preconfigured means

Table 13.4.1.3.4.1-1: General Test Parameters for E-UTRAN HD-FDD UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access

| Parameter | Unit | Value | Commnet |
| --- | --- | --- | --- |
|  |  | Test 1 |  |
| NB-IoT Operation mode |  | Standalone |  |
| Satellite information |  | SSC.2 | For NGSO |
| DRX |  | OFF |  |
| NPDCCH repetition level |  | 32 |  |
| npdcch-StartSF-USS Note 2 |  | v2 |  |
| npdcch-NumRepetitions-r13 Note 2 |  | r32 |  |
| NPUSCH resource units |  | 1 |  |
| NPUSCH repetition level |  | 128 |  |
| NPUSCH subcarrier spacing | kHz | 15 |  |
| NPUSCH number of subcarriers |  | 1 |  |
| NPUSCH modulation |  |  /4QPSK |  |
| NPUSCH Transport block size | Bits | 40 |  |
| npusch-TxDuration | ms | 64 |  |
| Note 1: Void Note 2: For further information see clause 6.7.3.2 in TS 36.331 [5]. |  |  |  |

13.4.1.3.4.2	Test procedure

The test consists of a single cell Ncell 1, the UE shall be fully synchronized to Ncell 1. The transmit timing accuracy is verified related to the downlink frame timing of Ncell 1. The downlink timing of Ncell 1 is changed and the changes in UE transmit timing are observed. The transmit timing is verified by the UE transmitting NPUSCH used as a measurement reference facilitating the SS timing estimation.

1.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

2.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5.

3.	Set the parameters according to Table 13.4.1.3.5-1 as appropriate. Propagation conditions are set according to Annex B clause B.1.

4.	SS sends uplink scheduling information via NPDCCH DCI format N0 for C_RNTI to schedule uplink subframes.

5.	The UE shall transmit NPUSCH. The SS shall measure the UE transmit timing offset (nTs).

6.	The SS verifies NPUSCH transmission timing offset. If it’s the initial transmission or at the start of a transmission segment boundary, the SS verifies that it’s within the limits specified in Table 13.4.1.3.5-2 with respect to the first detected path (in time) of the corresponding downlink frame of Ncell 1. If it’s neither the initial transmission nor at the start of a transmission segment boundary, the SS verifies that the UE transmit timing offset doesn’t change. If UE adjusts the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary, this test fails.

7.	Using the value of n measured in step 4, the SS calculates the total adjustment value of the downlink transmit timing for the Ncell 1:

-	if n < 0, by +(144 - |n|)TS compared to that in step 4.

-	 if n ≥ 0, by -(144 - |n|)TS compared to that in step 4.

8.	After 16ms from the initial NPUSCH transmission in step 4, the test system adjusts the downlink transmit timing for the Ncell 1, using the value of n measured in step 4. The downlink timing adjustment is performed monotonically in multiple steps of |∆T| ≤ 9×TS per 256ms until the above required total timing change is achieved, during which no grant is transmitted for the UE.

9.	The SS shall check that the UE transmit timing offset at initial transmission or at the start of a transmission segment boundary, stays within the limits specified in Table 13.4.1.3.5-2 with respect to the first detected path (in time) of the corresponding downlink frame of Ncell 1.

10.	Repeat steps 1-9 with ephemeris values for minimum Doppler, maximum positive Doppler and maximum negative Doppler replacing ephemeris in step 1 by corresponding tables in TS 36.508 [12] clause 8.4.6.2.2 for the type of satellite under test.

13.4.1.3.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 8.1.6 and clause 8.1.4.3 using condition "Standalone" and “NTN” with the following exceptions:

Table 13.4.1.3.4.3-1: NPDCCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.6.3, Table 8.1.6.3-3: NPDCCH-ConfigDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDCCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| npdcch-NumRepetitions-r13 | r32 | Set NPDCCH repetition to 32 |  |
| npdcch-StartSF-USS-r13 | V2 |  |  |
| npdcch-Offset-USS-r13 | zero |  |  |
| } |  |  |  |

Table 13.4.1.3.4.3-2: NB-IoT Physical layer parameters for DCI format N0

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.3.6, Table 8.1.3.6.1.1-1: NB-IoT Physical layer parameters for DCI format N0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | Value | Value in binary | Comment | Condition |
| Repetition number | 7 | “111” | Set NPUSCH repetition to 128 |  |
| DCI subframe repetition number | 32 | “11” | Set NPDCCH repetition to 32 |  |

##### 13.4.1.3.5	Test requirement

Table 13.4.1.3.5-1: Cell specific Test Parameters for E-UTRAN HD-FDD UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access

| Parameter | Unit | Value | Commnet |
| --- | --- | --- | --- |
|  |  | Test 1 |  |
| RF Channel Number |  | 1 |  |
| BWchannel | kHz | 200 |  |
| NPDSCH parameter |  | R.18 HD-FDD |  |
| NPDCCH parameter |  | R.30 HD-FDD |  |
| NOCNG Patterns |  | NOP.3 FDD |  |
| NPBCH_RA | dB | 0 |  |
| NPBCH_RB |  |  |  |
| NPSS_RA |  |  |  |
| NSSS_RA |  |  |  |
| NPDCCH_RA |  |  |  |
| NPDCCH_RB |  |  |  |
| NPDSCH_RA |  |  |  |
| NPDSCH_RB |  |  |  |
| NOCNG_RA Note1 |  |  |  |
| NOCNG_RB Note1 |  |  |  |
|  | dBm/15 kHz | -88 |  |
|  | dB | -10.7 |  |
|  | dB | -10.7 |  |
| Antenna Configuration |  | 1x1 |  |
| Propagation condition | - | AWGN |  |
| Note 1 NOCNG shall be used such that the cell is fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

The UE transmit timing offset shall be within the requirements in Table 13.4.1.3.5-2.

Table 13.4.1.3.5-2: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | (97– TGNSS_margin+3)*TS |
| NOTE 1: TS is the basic timing unit defined in 3GPP TS 36.211 [9] |  |

The reference point for the UE initial transmit timing control test requirement shall be the downlink timing minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×TsNTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts.

The UE shall be capable of changing the transmission timing according to the received downlink frame. When the transmission timing error between the UE and the reference cell exceeds Te, the UE is required to adjust its timing to within Te.

# < Unchanged sections omitted >

## 13.6	Measurement performance requirements for UE for satellite access

### 13.6.1	Void

### 13.6.2	Channel quality reporting accuracy for satellite access

#### 13.6.2.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

##### 13.6.2.1.1	Test purpose

The purpose of this test is to verify that the downlink channel quality reporting accuracy is within the specified limits. This test will verify the requirements in TS36.133[4] Section 9.1.22A.8for NB-IoT SAN PCell.

##### 13.6.2.1.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD category NB1 UEs supporting GSO or NGSO or both from release 17 and forwards.

##### 13.6.2.1.3	Minimum conformance requirements

The requirements for accuracy of downlink channel quality reporting in this clause apply to the serving cell on the anchor carrier and non-anchor carrier for UE Category NB1.

The accuracy requirements in Table 13.6.2.1.3-1 are valid under the following conditions:

-	Cell specific reference signals are transmitted either from one port.

-	Conditions defined in TS 36.102 Clause 7.3B for reference sensitivity are fulfilled.

-	NRSRP|dBm according to TS36.133 Annex B.3.25A for a corresponding Band.

Table 13.6.2.1.3-1: Downlink channel quality reporting accuracy for UE Category NB1

| NPDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 2 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups NOTE 3 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -6 dB | NFDD_SAB_ | -122.9 | N/A | -70 |
| R/4 NOTE 1 | >1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R NOTE 1 | ≤1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/8 NOTE 1 | >1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| NOTE 1: R is the reported NPDCCH repetition level that UE has reported in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA/NR operating band groups are as defined in TS36.133 Section 3.5. |  |  |  |  |  |  |

The normative reference for this requirement is 3GPP TS 36.133 [4] clause A.13.6.2.1.

##### 13.6.2.1.4	Test description

13.6.2.1.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.6.2.1.5-2.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.18 using only UE main Tx/Rx antenna.

2.	Propagation conditions are set according to Annex B clause B.0.

3.	There is one NB-IoT cell specified in the test. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

4.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

5.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

6.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

The UE shall be provided with the valid information about the SAN serving cells before the test.

Table 13.6.2.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

13.6.2.1.4.2	Test procedure

The test scenario comprises of one NB-IoT carrier with 1 Ncell.

1.	 Ensure the UE is in State 3A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 in Ncell 1.

2.	 Set the parameters according to Tables 13.6.2.1.5-1 and 13.6.2.1.5-2. Propagation conditions are set according to Annex B clause B.1.1.

3.	The UE shall transmit msg3 with the NPDCCH repetition number R report according to TS36.133 Table 9.1.22.15-1 after successful reception of the RAR.

4.	The SS transmits NPDSCH via NPDCCH DCI format N1 according to the UE reported NPDCCH repetition number R. The SS sends downlink MAC padding bits on the DL RMC.

5.	Measure the Pm-dsg for a duration of 4500 NPDCCH. SS counts the number of NACKs, ACKs and statDTXs on the UL NPUSCH. Pm-dsg is the ratio according to the formula (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is ≤ 1%, continue with step 6. Otherwise fail the UE.

6.	The SS transmits NPDSCH via NPDCCH DCI format N1 according to NPDCCH repetition number R/4. The SS sends downlink MAC padding bits on the DL RMC.

7.	Measure the Pm-dsg for a duration of 4500 NPDCCH. SS counts the number of NACKs, ACKs and statDTXs on the UL NPUSCH. Pm-dsg is the ratio according to the formula (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is > 1%, pass the UE. Otherwise fail the UE.

13.6.2.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.4, 8.1.5B and 8.1.6 with following exceptions.

Table 13.6.2.1.4.3-1: Common Exception messages for NB1 test requirements

| Default Message Contents |  |
| --- | --- |
| NPDCCH standalone message contents | Table A.10.1.2-1 |
| NPDSCH standalone message contents | Table A.10.2.2-1 |

Table 13.6.2.1.4.3-2: NPUSCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 36.508 clause Table 8.1.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-r13 | r1 | Default |  |
| npusch-AllSymbols-r13 | TRUE | Default |  |
| groupHoppingDisabled-r13 | Not present | Default |  |
| } |  |  |  |

Table 13.6.2.1.4.3-3: SystemInformationBlockType2-NB

| Derivation Path: 36.508 Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  | FDD |
| cqi-Reporting-r14 | True |  |  |
| } |  |  |  |
| } |  |  |  |

##### 13.6.2.1.5	Test requirement

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables 13.6.2.1.5-1 and 13.6.2.1.5-2. This test is divided into two parts. Firstly, UE should report the correct R value to ensure the pm-dsg under 1%. Secondly, the pm-dsg should larger than 1% when R/4 or R/8 is used.

Table 13.6.2.1.5-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |
| CP Length |  |  | Normal |
| DRX |  |  | OFF |
| NPUSCH repetition level |  |  | 1 |
| Satellite information | Config 1 | SSC.1 | GSO |
|  | Config 2 | SSC.2 | NGSO |

Table 13.6.2.1.5-2: nCell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| BWchannel | kHz | 200 |
| NPDCCH parameter |  | R.30 HD-FDD |
| NPDCCH repetition level for RAR |  | 4 |
| NPBCH_RB | dB | 0 |
| NPSS_RA | dB |  |
| NSSS_RA | dB |  |
| NPDCCH_RA | dB |  |
| NPDCCH_RB | dB |  |
| NPDSCH_RA | dB |  |
| NPDSCH_RB | dB |  |
| OCNG_RANote1 | dB |  |
| OCNG_RBNote1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
| NRS | dB | -5.4 |
| Propagation condition |  | AWGN |
| Antenna Configuration |  | 1x1 |
| Scheduling delay in RAR (IDelay) Note3 |  | 0 |
| Channel quality IE Note4 |  | CQI-NPDCCH-NB |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See section 16.3.3 in TS 36.213 [23]. Note 4: See TS 36.331 [2]. |  |  |

The pass/fail decision is as specified in the test procedure in clause 13.6.2.1.4.2.

#### 13.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- MU/TT is TBD

##### 13.6.2.2.1	Test purpose

The purpose of this test is to verify that the downlink channel quality reporting accuracy is within the specified limits. This test will verify the requirements in TS36.133[4] Section 9.1.22A.8 for NB-IoT SAN PCell.

##### 13.6.2.2.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD category NB1 UEs supporting GSO or NGSO or both from release 17 and forwards.

##### 13.6.2.2.3	Minimum conformance requirements

The requirements for accuracy of downlink channel quality reporting in this clause apply to the serving cell on the anchor carrier and non-anchor carrier for UE Category NB1.

The accuracy requirements in Table 13.6.2.2.3-1 are valid under the following conditions:

-	Cell specific reference signals are transmitted either from one port.

-	Conditions defined in TS 36.102 Clause 7.3B for reference sensitivity are fulfilled.

-	NRSRP|dBm according to TS36.133 Annex B.3.25A for a corresponding Band.

Table 13.6.2.2.3-1: Downlink channel quality reporting accuracy for UE Category NB1

| NPDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 2 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups NOTE 3 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/4 NOTE 1 | >1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R NOTE 1 | ≤1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/8 NOTE 1 | >1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| NOTE 1: R is the reported NPDCCH repetition level that UE has reported in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA/NR operating band groups are as defined in TS36.133 Section 3.5. |  |  |  |  |  |  |

The normative reference for this requirement is 3GPP TS 36.133 [4] clause A.13.6.2.2.

##### 13.6.2.2.4	Test description

13.6.2.2.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.6.2.2.5-2.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.18 using only UE main Tx/Rx antenna.

2.	Propagation conditions are set according to Annex B clause B.0.

3.	There is one NB-IoT cell specified in the test. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

4.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

5.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

6.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

The UE shall be provided with the valid information about the SAN serving cells before the test.

Table 13.6.2.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

13.6.2.2.4.2	Test procedure

The test scenario comprises of one NB-IoT carrier with 1 Ncell.

1.	 Ensure the UE is in State 3A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 in Ncell 1.

2.	 Set the parameters according to Tables 13.6.2.2.5-1 and 13.6.2.2.5-2. Propagation conditions are set according to Annex B clause B.1.1.

3.	The UE shall transmit msg3 with the NPDCCH repetition number R report according to TS36.133 Table 9.1.22.15-1 after successful reception of the RAR.

4.	The SS transmits NPDSCH via NPDCCH DCI format N1 according to the UE reported NPDCCH repetition number R. The SS sends downlink MAC padding bits on the DL RMC.

5.	Measure the Pm-dsg for a duration of 4500 NPDCCH. SS counts the number of NACKs, ACKs and statDTXs on the UL NPUSCH. Pm-dsg is the ratio according to the formula (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is ≤ 1%, continue with step 6. Otherwise fail the UE.

6.	The SS transmits NPDSCH via NPDCCH DCI format N1 according to NPDCCH repetition number R/8. The SS sends downlink MAC padding bits on the DL RMC.

7.	Measure the Pm-dsg for a duration of 4500 NPDCCH. SS counts the number of NACKs, ACKs and statDTXs on the UL NPUSCH. Pm-dsg is the ratio according to the formula (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is > 1%, pass the UE. Otherwise fail the UE.

13.6.2.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.4, 8.1.5B and 8.1.6 with following exceptions.

Table 13.6.2.2.4.3-1: Common Exception messages for NB1 test requirements

| Default Message Contents |  |
| --- | --- |
| NPDCCH standalone message contents | Table A.10.1.2-1 |
| NPDSCH standalone message contents | Table A.10.2.2-1 |

Table 13.6.2.2.4.3-2: NPUSCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 36.508 clause Table 8.1.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-r13 | r1 | Default |  |
| npusch-AllSymbols-r13 | TRUE | Default |  |
| groupHoppingDisabled-r13 | Not present | Default |  |
| } |  |  |  |

Table 13.6.2.2.4.3-3: SystemInformationBlockType2-NB

| Derivation Path: 36.508 Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  | FDD |
| cqi-Reporting-r14 | True |  |  |
| } |  |  |  |
| } |  |  |  |

##### 13.6.2.2.5	Test requirement

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables 13.6.2.2.5-1 and 13.6.2.2.5-2. This test is divided into two parts. Firstly, UE should report the correct R value to ensure the pm-dsg under 1%. Secondly, the pm-dsg should larger than 1% when R/4 or R/8 is used.

Table 13.6.2.2.5-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |
| CP Length |  |  | Normal |
| DRX |  |  | OFF |
| NPUSCH repetition level |  |  | 1 |
| Satellite information | Config 1 | SSC.1 | GSO |
|  | Config 2 | SSC.2 | NGSO |

Table 13.6.2.2.5-2: nCell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| BWchannel | kHz | 200 |
| NPDCCH parameter |  | R.30 HD-FDD |
| NPDCCH repetition level for RAR |  | 16 |
| NPBCH_RB | dB | 0 |
| NPSS_RA | dB |  |
| NSSS_RA | dB |  |
| NPDCCH_RA | dB |  |
| NPDCCH_RB | dB |  |
| NPDSCH_RA | dB |  |
| NPDSCH_RB | dB |  |
| OCNG_RANote1 | dB |  |
| OCNG_RBNote1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
| NRS | dB | -12 |
| Propagation condition |  | AWGN |
| Antenna Configuration |  | 1x1 |
| Scheduling delay in RAR (IDelay) Note3 |  | 0 |
| Channel quality IE Note4 |  | CQI-NPDCCH-NB |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See section 16.3.3 in TS 36.213 [23]. Note 4: See TS 36.331 [2]. |  |  |

The pass/fail decision is as specified in the test procedure in clause 13.6.2.2.4.2.

#### 13.6.2.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

##### 13.6.2.3.1	Test purpose

The purpose of this test is to verify that the downlink channel quality reporting accuracy on non-anchor carrier is within the specified limits. This test will verify the requirements in TS36.133[4] Section 9.1.22A.8 for NB-IoT SAN PCell.

##### 13.6.2.3.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD category NB1 UEs supporting GSO or NGSO or both from release 17 and forwards.

Applicability requires support for sending DL channel quality reporting for a non-anchor carrier.

##### 13.6.2.3.3	Minimum conformance requirements

The requirements for accuracy of downlink channel quality reporting in this clause apply to the serving cell on the anchor carrier and non-anchor carrier for UE Category NB1.

The accuracy requirements in Table 13.6.2.3.3-1 are valid under the following conditions:

-	Cell specific reference signals are transmitted either from one port.

-	Conditions defined in TS 36.102 Clause 7.3B for reference sensitivity are fulfilled.

-	NRSRP|dBm according to TS36.133 Annex B.3.25A for a corresponding Band.

Table 13.6.2.3.3-1: Downlink channel quality reporting accuracy for UE Category NB1

| NPDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 2 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups NOTE 3 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/4 NOTE 1 | >1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R NOTE 1 | ≤1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/8 NOTE 1 | >1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| NOTE 1: R is the reported NPDCCH repetition level that UE has reported in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA/NR operating band groups are as defined in TS36.133 Section 3.5. |  |  |  |  |  |  |

The normative reference for this requirement is 3GPP TS 36.133 [4] clause A.13.6.2.3.

##### 13.6.2.3.4	Test description

13.6.2.3.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.6.2.3.5-2.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.18 using only UE main Tx/Rx antenna.

2.	Propagation conditions are set according to Annex B clause B.0.

3.	There is one NB-IoT cell specified in the test. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test. In Ncell 1 one anchor carrier and one non-anchor carrier are configured.

4.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

5.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

6.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

The UE shall be provided with the valid information about the SAN serving cells before the test.

Table 13.6.2.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

13.6.2.3.4.2	Test procedure

The test scenario comprises of 2 NB-IoT carriers with 1 Ncell.

The test procedures are the same as test procedures defined in 13.6.2.1.4.2 but performed on the non-anchor carrier.

13.6.2.3.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.4, 8.1.5B and 8.1.6 with following exceptions.

Table 13.6.2.3.4.3-1: Common Exception messages for NB1 test requirements

| Default Message Contents |  |
| --- | --- |
| NPDCCH standalone message contents | Table A.10.1.2-1 |
| NPDSCH standalone message contents | Table A.10.2.2-1 |

Table 13.6.2.3.4.3-2: SystemInformationBlockType22-NB

| Derivation Path: 36.508 Table 8.1.4.3.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType22-NB-r14 ::= SEQUENCE { |  |  |  |
| ul-ConfigList-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigCommon-NB-r14 SEQUENCE { |  |  |  |
| UL-ConfigCommon-NB-r14[1] SEQUENCE { |  |  |  |
| nprach-ParametersList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r14 SEQUENCE { |  |  |  |
| NPRACH-Parameters-NB-r14[1] SEQUENCE { |  |  |  |
| nprach-Parameters-r14 SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms40 |  |  |
| nprach-StartTime-r14 | ms8 |  |  |
| nprach-SubcarrierOffset-r14 | n0 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | one |  |  |
| npdcch-NumRepetitions-RA-r14 | r4 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v8 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r14[2] SEQUENCE { |  |  |  |
| nprach-Parameters-r14 SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms40 |  |  |
| nprach-StartTime-r14 | ms8 |  |  |
| nprach-SubcarrierOffset-r14 | n0 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | one |  |  |
| npdcch-NumRepetitions-RA-r14 | r8 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v2 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r14[3] SEQUENCE { |  |  |  |
| nprach-Parameters-r14 SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms40 |  |  |
| nprach-StartTime-r14 | ms8 |  |  |
| nprach-SubcarrierOffset-r14 | n0 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | one |  |  |
| npdcch-NumRepetitions-RA-r14 | r32 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v2 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nprach-ProbabilityAnchorList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ProbabilityAnchor-NB-r14 SEQUENCE { |  |  |  |
| NPRACH-ProbabilityAnchor-NB-r14[1] SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | zero |  |  |
| } |  |  |  |
| NPRACH-ProbabilityAnchor-NB-r14[2] SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | zero |  |  |
| } |  |  |  |
| NPRACH-ProbabilityAnchor-NB-r14[3] SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | zero |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 13.6.2.3.4.3-3: NPUSCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 36.508 clause Table 8.1.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-r13 | r1 | Default |  |
| npusch-AllSymbols-r13 | TRUE | Default |  |
| groupHoppingDisabled-r13 | Not present | Default |  |
| } |  |  |  |

Table 13.6.2.3.4.3-4: SystemInformationBlockType2-NB

| Derivation Path: 36.508 Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  | FDD |
| cqi-Reporting-r14 | True |  |  |
| } |  |  |  |
| } |  |  |  |

##### 13.6.2.3.5	Test requirement

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy on non-anchor carrier is tested by using the parameters in Tables 13.6.2.3.5-1 and 13.6.2.3.5-2. This test is divided into two parts. Firstly, UE should report the correct R value to ensure the pm-dsg under 1%. Secondly, the pm-dsg should larger than 1% when R/4 or R/8 is used.

Table 13.6.2.3.5-1: General Test Parameters for Downlink channel quality reporting accuracy test on non-anchor carrier for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |
| CP Length |  |  | Normal |
| DRX |  |  | OFF |
| NPUSCH repetition level |  |  | 1 |
| NPDCCH carrier index (npdcch-CarrierIndex-r14) |  |  | 1 (Note 1) |
| Satellite information | Config 1 | SSC.1 | GSO |
|  | Config 2 | SSC.2 | NGSO |
| Note 1: Index of non-anchor carrier configured in SystemInformationBlockType22-NB (See TS 36.331 [2]). |  |  |  |

Table 13.6.2.3.5-2: nCell specific Test Parameters for Downlink channel quality reporting accuracy test on non-anchor carrier for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| BWchannel | kHz | 200 |
| NPDCCH parameter |  | R.30 HD-FDD |
| NPDCCH repetition level for RAR on non-anchor |  | 4 |
| NPBCH_RB | dB | 0 |
| NPSS_RA | dB |  |
| NSSS_RA | dB |  |
| NPDCCH_RA | dB |  |
| NPDCCH_RB | dB |  |
| NPDSCH_RA | dB |  |
| NPDSCH_RB | dB |  |
| OCNG_RANote1 | dB |  |
| OCNG_RBNote1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
| NRS | dB | -5.4 |
| Propagation condition |  | AWGN |
| Antenna Configuration |  | 1x1 |
| Scheduling delay in RAR (IDelay) Note3 |  | 0 |
| Channel quality IE Note4 |  | CQI-NPDCCH-NB |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See section 16.3.3 in TS 36.213 [23]. Note 4: See TS 36.331 [2]. |  |  |

The pass/fail decision is as specified in the test procedure in clause 13.6.2.3.4.2.

#### 13.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

##### 13.6.2.4.1	Test purpose

The purpose of this test is to verify that the downlink channel quality reporting accuracy on non-anchor carrier is within the specified limits. This test will verify the requirements in TS36.133[4] Section 13.6.2.4.1 for NB-IoT SAN PCell.

##### 13.6.2.4.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD category NB1 UEs supporting GSO or NGSO or both from release 17 and forwards.

##### 13.6.2.4.3	Minimum conformance requirements

The requirements for accuracy of downlink channel quality reporting in this clause apply to the serving cell on the anchor carrier and non-anchor carrier for UE Category NB1.

The accuracy requirements in Table 13.6.2.4.3-1 are valid under the following conditions:

-	Cell specific reference signals are transmitted either from one port.

-	Conditions defined in TS 36.102 Clause 7.3B for reference sensitivity are fulfilled.

-	NRSRP|dBm according to TS36.133 Annex B.3.25A for a corresponding Band.

Table 13.6.2.4.3-1: Downlink channel quality reporting accuracy for UE Category NB1

| NPDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 2 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups NOTE 3 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/4 NOTE 1 | >1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R NOTE 1 | ≤1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/8 NOTE 1 | >1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| NOTE 1: R is the reported NPDCCH repetition level that UE has reported in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA/NR operating band groups are as defined in TS36.133 Section 3.5. |  |  |  |  |  |  |

The normative reference for this requirement is 3GPP TS 36.133 [4] clause A.13.6.2.4.

##### 13.6.2.4.4	Test description

13.6.2.4.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.6.2.4.5-2.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.18 using only UE main Tx/Rx antenna and without fading.

2.	Propagation conditions are set according to Annex B clause B.0.

3.	There is one NB-IoT cell specified in the test. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test. In Ncell 1 one anchor carrier and one non-anchor carrier are configured.

4.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

5.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

6.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

The UE shall be provided with the valid information about the SAN serving cells before the test.

Table 13.6.2.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

13.6.2.4.4.2	Test procedure

The test scenario comprises of 2 NB-IoT carriers with 1 Ncell.

The test procedures are the same as test procedures defined in 13.6.2.2.4.2 but performed on the non-anchor carrier.

13.6.2.4.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.4, 8.1.5B and 8.1.6 with following exceptions.

Table 13.6.2.3.4.3-1: Common Exception messages for NB1 test requirements

| Default Message Contents |  |
| --- | --- |
| NPDCCH standalone message contents | Table A.10.1.2-1 |
| NPDSCH standalone message contents | Table A.10.2.2-1 |

Table 13.6.2.3.4.3-2: SystemInformationBlockType22-NB

| Derivation Path: 36.508 Table 8.1.4.3.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType22-NB-r14 ::= SEQUENCE { |  |  |  |
| ul-ConfigList-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigCommon-NB-r14 SEQUENCE { |  |  |  |
| UL-ConfigCommon-NB-r14[1] SEQUENCE { |  |  |  |
| nprach-ParametersList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r14 SEQUENCE { |  |  |  |
| NPRACH-Parameters-NB-r14[1] SEQUENCE { |  |  |  |
| nprach-Parameters-r14 SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms40 |  |  |
| nprach-StartTime-r14 | ms8 |  |  |
| nprach-SubcarrierOffset-r14 | n0 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | one |  |  |
| npdcch-NumRepetitions-RA-r14 | r1 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v8 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r14[2] SEQUENCE { |  |  |  |
| nprach-Parameters-r14 SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms40 |  |  |
| nprach-StartTime-r14 | ms8 |  |  |
| nprach-SubcarrierOffset-r14 | n0 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | one |  |  |
| npdcch-NumRepetitions-RA-r14 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v2 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r14[3] SEQUENCE { |  |  |  |
| nprach-Parameters-r14 SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms40 |  |  |
| nprach-StartTime-r14 | ms8 |  |  |
| nprach-SubcarrierOffset-r14 | n0 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | one |  |  |
| npdcch-NumRepetitions-RA-r14 | r32 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v2 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nprach-ProbabilityAnchorList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ProbabilityAnchor-NB-r14 SEQUENCE { |  |  |  |
| NPRACH-ProbabilityAnchor-NB-r14[1] SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | zero |  |  |
| } |  |  |  |
| NPRACH-ProbabilityAnchor-NB-r14[2] SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | zero |  |  |
| } |  |  |  |
| NPRACH-ProbabilityAnchor-NB-r14[3] SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | zero |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 13.6.2.4.4.3-3: NPUSCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 36.508 clause Table 8.1.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-r13 | r1 | Default |  |
| npusch-AllSymbols-r13 | TRUE | Default |  |
| groupHoppingDisabled-r13 | Not present | Default |  |
| } |  |  |  |

Table 13.6.2.4.4.3-4: SystemInformationBlockType2-NB

| Derivation Path: 36.508 Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  | FDD |
| cqi-Reporting-r14 | True |  |  |
| } |  |  |  |
| } |  |  |  |

##### 13.6.2.4.5	Test requirement

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy on non-anchor carrier is tested by using the parameters in Tables 13.6.2.4.5-1 and 13.6.2.4.5-2. This test is divided into two parts. Firstly, UE should report the correct R value to ensure the pm-dsg under 1%. Secondly, the pm-dsg should larger than 1% when R/4 or R/8 is used.

Table 13.6.2.4.5-1: General Test Parameters for Downlink channel quality reporting accuracy test on non-anchor carrier for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |
| CP Length |  |  | Normal |
| DRX |  |  | OFF |
| NPUSCH repetition level |  |  | 1 |
| NPDCCH carrier index (npdcch-CarrierIndex-r14) |  |  | 1 (Note 1) |
| Satellite information | Config 1 | SSC.1 | GSO |
|  | Config 2 | SSC.2 | NGSO |
| Note 1: Index of non-anchor carrier configured in SystemInformationBlockType22-NB (See TS 36.331 [2]). |  |  |  |

Table 13.6.2.4.5-2: nCell specific Test Parameters for Downlink channel quality reporting accuracy test on non-anchor carrier for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| BWchannel | kHz | 200 |
| NPDCCH parameter |  | R.30 HD-FDD |
| NPDCCH repetition level for RAR on non-anchor |  | 16 |
| NPBCH_RB | dB | 0 |
| NPSS_RA | dB |  |
| NSSS_RA | dB |  |
| NPDCCH_RA | dB |  |
| NPDCCH_RB | dB |  |
| NPDSCH_RA | dB |  |
| NPDSCH_RB | dB |  |
| OCNG_RANote1 | dB |  |
| OCNG_RBNote1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
| NRS | dB | -12 |
| Propagation condition |  | AWGN |
| Antenna Configuration |  | 1x1 |
| Scheduling delay in RAR (IDelay) Note3 |  | 0 |
| Channel quality IE Note4 |  | CQI-NPDCCH-NB |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See section 16.3.3 in TS 36.213 [23]. Note 4: See TS 36.331 [2]. |  |  |

The pass/fail decision is as specified in the test procedure in clause 13.6.2.4.4.2.

< End of changes >