3GPP TSG-RAN5 Meeting #102	R5-241295

# Athens, Greece, 26th February 2024 – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3056 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cleanup on FR1 CG-SDT test 6.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some test parameters are still in brackets. These have been already cleared in 38.133. This CR clears them in 38.533. Some content from agreed CR R5-237960 is still missing in the specification. This CR also re-adds this content. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed brackets from affected parameters in test 6.2.1 - Removed editor’s notes - Re-added missing content from agreed R5-237960 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### 6.2.1	NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

#### 6.2.1.1	Test Purpose

The purpose of this test is to partly verify that the UE properly perform TA validation for CG-SDT transmission in clause 6.2.0.1.

#### 6.2.1.2	Test Applicability

This test applies to all types of NR UE release 17 and forward, supporting NR SA, CG-SDT and long DRX cycle.

#### 6.2.1.3	Minimum Conformance Requirement

The minimum requirements are specified in clause 6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.2.1.

#### 6.2.1.4	Test Description

The test includes two sub-tests, Sub-test#1 for testing valid TA where UE can initiate CG-SDT transmission, and Sub-test#2 for testing invalid TA where UE does not initiate CG-SDT transmission. Subtest#2 is only tested if Sub-test#1 has passed. For each sub-test, UE is configured with CG-SDT configuration when entering RRC_INACTIVE state. Sub-test#1 consists of four successive time periods, with time duration of T1, T2, T3 and T4, respectively. Sub-test#2 consists of two successive time periods, with time duration of T5 and T6, respectively. There is one cell, which is the active NR cell in FR1.  Figure 6.2.1.4-1 shows the variation of the RSRP over the duration of Sub-test#1, and Figure 6.2.1.4-2 shows the variation of the RSRP over the duration of Sub-test#2.

Figure 6.2.1.4-1: RSRP variation model for Sub-test#1 for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

In Sub-test#1:

Prior to the time point TA, the UE shall be fully synchronized to PCell (Cell 1), be registered to the cell and have entered RRC_CONNECTED state.

Before starting the test at time point TA, test equipment configures RSRP to P0.

At time point TB, RSRP is changed from P0 to P1.

At time point TC, which is W1 after time point TB, UE expect to receive RRC release with CG SDT configuration and RRC status is changed to INACTIVE status.

At time point TD, RSRP is changed from P1 to P0.

At time point TE, RSRP is changed from P0 to P2.

Test equipment triggers UL data arrival at UE lower layer at time point TF. After time point TF, test equipment observes whether UE transmits with CG-SDT no later than TG, which is W3 after TF.

After time point TG, RRC status is changed from RRC INACTIVE to RRC CONNECTED.

Figure 6.2.1.4-2: RSRP variation model for Sub-test#2 for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

In Sub-test#2:

Prior to the time point TA, the UE shall pass Sub-test#1 and have entered RRC connected mode. Otherwise, Sub-test#2 shall not be executed.

From time point TA to time point TD, RSRP is set to P2.

At time point TC, which is W1 after time point TB, UE expect to receive RRC release with CG SDT configuration and RRC status is changed to INACTIVE status.

At time point TD, RSRP is changed from P2 to P0.

Test equipment triggers UL data arrival at UE lower layer at time point TF. TF is 3360ms after TD. After time point TF, test equipment observes whether UE transmits with CG-SDT no later than TG which is W3 after TF.

W1 equals to 640ms and W2 equals to 640ms, based on requirements in clause 6.2.0. W3 is 860ms.

##### 6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.2.1.4.1-1.

Table 6.2.1.4.1-1: NR configuration for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Config | Description |
| --- | --- |
| 6.2.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.2.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.2.1-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.2.1.4.1-2.

Table 6.2.1.4.1-2: Initial conditions for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 without fading | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.2.1.4.3.

2.	The power levels and settings for Cell 1 are set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 6.2.1.4.1-3. Supplementary timeline events are given in Table 6.2.1.4.1-4.

Table 6.2.1.4.1-3: General test parameters for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| TDD Configuration |  | 1 | N/A |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| BWchannel | MHz | 1,2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| PDSCH Reference measurement channel |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| SSB configuration |  | 1,2,3 | SSB.1 FR1 |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  | 1,2,3 | SMTC.1 |  |
| DRX configuration |  | 1,2,3 | DRX.7 |  |
| T1 | s | 1,2,3 | 0.4 |  |
| T2 | s | 1,2,3 | 1.28 |  |
| T3 | s | 1,2,3 | 2.72 |  |
| T4 | s | 1,2,3 | 1.5 |  |
| T5 | s | 1,2,3 | 1.68 |  |
| T6 | s | 1,2,3 | 4.22 |  |
| cg-SDT-RSRP-ChangeThreshold | dB | 1,2,3 | 14 |  |
| cg-SDT-RSRP-ThresholdSSB | dBm | 1,2 | -117 | Modified by TT analysis |
|  | dBm | 3 | -119 | Modified by TT analysis |
| cg-SDT-TimeAlignmentTime |  | 1,2,3 | infinity |  |
| CG-SDT resource period | ms |  | 320 | configuredGrantConfig Peridocity at 38.331 [13] Config 1,2 : sym320x14, config 3 : sym640x14 |
| Propagation condition |  | 1,2,3 | AWGN |  |

Table 6.2.1.4.1-4: Supplementary timeline event derivations for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Time Period | Unit | Test configuration | Value | Derivation |
| --- | --- | --- | --- | --- |
| T_delay_modeB | ms | 1,2,3 | 4000 | Test configuration for step 4 and step 9 |
| W1 | ms | 1,2,3 | 640 | Test configuration |
| W2 | ms | 1,2,3 | 640 | Test configuration |
| W3 | ms | 1,2,3 | 860 | Test configuration |
| TAtoTB | ms | 1,2,3 | 400 | Sub-test#1: TAtoTB = T1 Sub-test#2: TAtoTB = T5 – 2*W1 |
| TBtoTC | ms | 1,2,3 | 640 | TBtoTC = W1 |
| TCtoTD | ms | 1,2,3 | 640 | TCtoTD = W1 |
| TDtoTE | ms | 1,2,3 | 2720 | Sub-test#1: TDtoTE = T3 Sub-test#2: TDtoTE = T6 – W2 – W3 |
| TEtoTF | ms | 1,2,3 | 640 | TEtoTF = W2 |
| TFtoTG | ms | 1,2,3 | 860 | TFtoTG = W3 |
| Note 1: These timeline derivations are useful to understand different timeline events other than T1, T2, T3, T4, T5 and T6. Calculations are informative. |  |  |  |  |

##### 6.2.1.4.2	Test Procedure

There is one cell in the test, the FR1 PCell, which is the active NR cell. The test parameters for the PCell are given in Table 6.2.1.4.1-1, Table 6.2.1.4.1-3, and Table 6.2.1.5-1, for Sub-test#1 and Sub-test#2, respectively. Prior to the start of each of the Sub-tests, the UE shall be fully synchronized to PCell (Cell 1). According to Table 6.2.1.5-1, in time intervals T1 to T6 the SS configures the SS-RSRP by fixing Noc and adjusting SNR correspondingly.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Loop Function On and Activate UE Test Loop Mode B according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

2.	Set the parameters of NR Cell according to T1 in Table 6.2.1.5-1. Propagation conditions are set according to clause C.2.3. Sub-test#1 starts. T1 starts.

3.	When T1 expires the SS shall change the SS-RSRP value to T2 as specified in Table 6.2.1.5-1. T2 starts.

4.	W1=640ms seconds after the start of T2 (time event TC), the SS shall send CLOSE UE TEST LOOP message for UE Test Loop Mode B and configuring T_delay_modeB as per Table 6.2.1.4.3-5, and transmit one PDCP SDU (which will get buffered at the UE) immediately followed by an RRCRelease message containing CG-SDT configuration for the UE to change its RRC state to RRC_INACTIVE, according to section 6.2.1.4.3.

5.	When T2 expires the SS shall change the SS-RSRP value to T3 as specified in Table 6.2.1.5-1. T3 starts.

6.	When T3 expires the SS shall change the SS-RSRP value to T4 as specified in Table 6.2.1.5-1. T4 starts.

7.	T_delay_modeB expires after the start of T4, at time event TF. If the UE transmits PUSCH at the next CG-SDT resource no later than TG, which is W3 seconds after TF, then the number of successful tests for CG-SDT Sub-test#1 is increased by one. Otherwise, the number of failed tests for CG-SDT Sub-test#1 is increased by one and proceed with step 12.

8.	The SS shall transmit RRCRelease. Set the parameters of NR Cell according to T5 in Table 6.2.1.5-1. Propagation conditions are set according to clause C.2.3. Afterwards, the SS shall ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Loop Function On and Activate UE Test Loop Mode B according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell. Sub-test#2 starts. T5 starts.

9.	W1=640ms seconds before the end of T5 (time event TC), the SS shall send CLOSE UE TEST LOOP message for UE Test Loop Mode B and configuring T_delay_modeB as per Table 6.2.1.4.3-5, and transmit one PDCP SDU (which will get buffered at the UE) immediately followed by an RRCRelease message containing CG-SDT configuration for the UE to change its RRC state to RRC_INACTIVE, according to section 6.2.1.4.3.

10.	When T5 expires the SS shall change the SS-RSRP value to T6 as specified in Table 6.2.1.5-1. T6 starts.

11.	T_delay_modeB expires after the start of T6, at time event TF. If the UE transmits PUSCH at the next CG-SDT resource no later than TG, which is W3 seconds after TF, then the number of failed tests for CG-SDT Sub-test#2 is increased by one. Otherwise, the number of successful tests for CG-SDT Sub-test#2 is increased by one. Proceed with step 12.

12.	The SS shall switch off and then on the UE and then proceed with step 13.

13.	Repeat steps 1-11 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved for both Sub-tests. If Sub-test#1 fails, there is no need to achieve any confidence level for Sub-test#2.

6.2.1.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 6.2.1.4.3-1: Common Exception messages for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | 38.508-1 [14] Table 4.6.1-16 with conditions NR_RRC_INACTIVE and SDT |

Table 6.2.1.4.3-2: SDT-MAC-PHY-CG-Config-r17 for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SDT-MAC-PHY-CG-Config-r17::= SEQUENCE { |  |  |  |
| cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF CG-SDT-Config-LCH-restriction-r17 { | 1 entry |  |  |
| logicalChannelIdentity-r17 | LogicalChannelIdentity with condition DRB1 |  |  |
| configuredGrantType1Allowed-r17 | true |  |  |
| allowedCG-List-r17 | Not present |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 CHOICE { |  |  |  |
| setup | BWP-UplinkDedicatedSDT-r17 |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-SUL-r17 | Not present |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 |  |  |  |
| setup | BWP-DownlinkDedicatedSDT-r17 |  |  |
| } |  |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | infinity |  |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | 39 | -117dBm | Config 1,2 |
|  | 37 | -119dBm | Config 3 |
| cg-SDT-TA-ValiditationConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-RSRP-ChangeThreshold-r17 | dB14 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.1.4.3-3: BWP-UplinkDedicatedSDT-r17 for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicatedSDT-r17::= SEQUENCE { |  |  |  |
| pusch-Config-r17 CHOICE { |  |  |  |
| setup | PUSCH-Config |  |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig | 1 entry |  |  |
| } |  |  |  |

Table 6.2.1.4.3-4: ConfiguredGrantConfig for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| periodicity | sym320x14 |  | Config 1,2 |
|  | sym640x14 |  | Config 3 |
| } |  |  |  |

Table 6.2.1.4.3-5: CLOSE UE TEST LOOP for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Derivation Path: TS 38.509 [16], clause 6.3.1 |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| Protocol discriminator | 1 1 1 1 |  |
| Skip indicator | 0 0 0 0 |  |
| Message type | 1 0 0 0 0 0 0 0 |  |
| UE test loop mode | 0 0 0 0 0 0 0 1 | UE test loop mode B |
| UE test loop mode B LB setup |  |  |
| IP PDU delay | 0 0 0 0 0 1 0 0 | T_delay_modeB duration in seconds (4 seconds) |

#### 6.2.1.5	Test Requirement

Table 6.2.1.5-1 defines the cell specific primary level settings including test tolerances for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT).

Table 6.2.1.5-1: Cell-specific test parameters for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Parameter | Config | Unit | SSB#0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T6 |
|  | 1,2 | dBm/15kHz | -98.95 | -98.95 | -101 | -101 | -99.5 | -105 |
|  | 3 | dBm/15kHz | -98.2 | -98.2 | -101.25 | -101.25 | -103.7 | -107.95 |
|  | 1,2 | dBm/SSB SCS | -98.95 | -98.95 | -101 | -101 | -99.5 | -105 |
|  | 3 | dBm/SSB SCS | -95.2 | -95.2 | -98.25 | -98.25 | -100.7 | -104.95 |
|  | 1,2 | dB | 0 | 10 | 0 | 15 | 19.6 | -0.95 |
|  | 3 | dB | 0 | 10.5 | 0 | 15.5 | 18 | -2.8 |
| SS RSRP | 1,2 | dBm/SSB SCS | -98.95 | -88.95 | -101 | -86 | -79.9 | -105.95 |
|  | 3 | dBm/SSB SCS | -95.2 | -84.7 | -98.25 | -82.75 | -82.7 | -107.75 |
| Io | 1,2 | dBm/9.36MHz | -67.99 | -60.58 | -70.04 | -57.91 | -51.9 | -74.49 |
|  | 3 | dBm/38.16MHz | -61.13 | -53.27 | -64.18 | -51.57 | -51.58 | -72.06 |
|  | 1,2 | dB | 0 | 10 | 0 | 15 | 19.6 | -0.95 |
|  | 3 | dB | 0 | 10.5 | 0 | 15.5 | 18 | -2.8 |

The UE behaviour in each test during time durations shall be as follows:

During Sub-test#1, UE shall transmit PUSCH at CG-SDT resource within 860ms after time point TF.

During Sub-test#2, after passing Sub-test#1, UE shall not transmit PUSCH at CG-SDT resources after TF until the end of the test at time point TG.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

<< Skipped sections >>

### F.1.1.2	Measurement of RRM requirements

<< Skipped tables >>

Table F.1.1.2-4 Maximum test system uncertainty for RRM requirements for SA FR1 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.1.1.3 NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 6.1.1.4 NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 6.1.1.5 NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 6.1.1.6 NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 6.1.1.7 NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 6.1.1.8 NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17 | Same as 6.1.1.2 | Same as 6.1.1.2 |  |
| 6.1.2.1 NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 (NR) frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 (E-UTRAN) frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.1.2.2 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 6.1.2.3 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 6.1.2.4 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 6.1.2.5 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 6.2.1 NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT) | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Ês / Noc ±0.3 dB | Note: Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.3.1.1 NR SA FR1 handover with known target cell | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.3.1.2 NR SA FR1 handover with unknown target cell | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 6.3.1.3 NR SA FR1-FR1 Handover with unknown Target Cell | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.3.1.4 NR SA FR1 – E-UTRA handover with known target cell | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 6.3.1.5 NR SA FR1 – E-UTRA handover with unknown target cell | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 6.3.1.6 NR SA FR1 – UTRAN FDD handover with known target cell | Noc ±1.5 dB Ês / Noc ±0.3 dB Ioc ±0.7 dB Îor / Ioc ± 0.3dB CPICH Ec / Ior ± 0.1dB SCH Ec / Ior ± 0.1dB | Note: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the ratio of cell 2 CPICH code level / Ior SCH Ec / Ior is the ratio of cell 2 SCH code level / Ior |
| 6.3.1.7 NR SA FR1 synchronous DAPS handover | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 6.3.1.8 NR SA FR1 asynchronous DAPS handover | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 6.3.1.9 NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 6.3.1.10 NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 6.3.1.11 NR SA FR1 Inter-band inter-frequency synchronous DAPS handover | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 6.3.1.12 NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 6.3.2.1.1 NR SA FR1 RRC re-establishment | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 6.3.2.1.2 NR SA FR1 – FR1 RRC re-establishment | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 6.3.2.1.3 NR SA FR1 RRC re-establishment without serving cell timing | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.3.2.2.1 | Noc ±1.5 dB Ês / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.3.2.2.2 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 6.3.2.2.3 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 6.3.2.2.4 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 6.3.2.3.1 NR SA FR1 RRC connection release with redirection | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 6.3.2.3.2 NR SA FR1 – E-UTRA RRC connection release with redirection | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 6.3.3.1 NR SA FR1 conditional handover | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 6.3.3.2 NR SA FR1-FR1 conditional handover | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 6.4.1.1 EN-DC FR1 UE transmit timing accuracy | Noc ±1.5 dB Ês1 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.4.3.1 NR SA FR1 timing advance adjustment accuracy | Noc ±1.5 dB Ês1 / Noc ±0.3 dB ±88Tc Timing Advance Adjustment accuracy | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.3 dB Fading profile ±0.7 dB | Noc is the AWGN on cell 1 frequency Ês / Noc is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.9 SA FR1 radio link monitoring out-of-sync Test for PSCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | Noc1 ±1.5 dB Noc2 ±1.5 dB Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.5.3.1 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.5.3.2 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.3.3 NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.3.4 NR SA FR1 direct SCell activation at SCell addition of known SCell | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.3.5 NR SA FR1 direct SCell activation at handover with known SCell | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc1 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc1 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.5.3.10 NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 for CSI-RSRP Meas PRB are # RB0 to # RB105. |
| 6.5.3.11 NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640 ms SCell measurement cycle | Same as 6.5.3.10 | Same as 6.5.3.10 |
| 6.5.5.1 NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.5.5.2 NR SA FR1 SSB-based beam failure detection and link recovery in DRX | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 6.5.5.3 NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| 6.5.5.4 NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 6.5.5.5 NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB and CSI-RS Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.5.5.6 NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | Same as 6.5.5.5 | Same as 6.5.5.5 |
| 6.5.5.7 NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 6.5.6.1.1 NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | ±0.6 dB | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 6.5.6.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX | Same as 6.5.6.1.1 | Same as 6.5.6.1.1 |
| 6.5.6.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX | ±0.6 dB | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 6.5.7.1 NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA | Noc1 ±1.5 dB Noc2 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.5.7.2 NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA | Same as 6.5.7.1 | Same as 6.5.7.1 |
| 6.5.7A.2 NR SA FR1 DL interruptions at switching between two uplink carriers in TDD-TDD CA | Same as 6.5.7.1 | Same as 6.5.7.1 |
| 6.5.7B.2 NR SA FR1 DL interruptions at switching between two uplink bands in TDD-TDD CA | Noc1 ±1.5 dB Noc2 ±1.5 dB Noc3 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |
| 6.5.8.1 UE specific CBW change on PCell in FR1 in non-DRX | Test System uncertainty on Cell1: ±0.6 dB | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty: ±0.3 dB, 2. Effect of AWGN flatness and signal flatness: ±2.0 dB. Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) = 0.58dB, round up to 0.6dB. |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.7 NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.8 NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.2.2 NR SA FR1-FR1 event-triggered reporting in DRX | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.12 NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 | Same as 6.6.2.1 | Same as 6.6.2.1 |  |
| 6.6.3.1 NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 6.6.3.3 NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| 6.6.4.4 NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 6.6.4.5 NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 6.6.5.1 NR SA FR1 – UTRAN event-triggered reporting in non-DRX | Noc ±1.5 dB Ês / Noc ±0.3 dB Ioc ±0.7 dB Îor / Ioc ± 0.3dB CPICH Ec / Ior ± 0.1dB SCH Ec / Ior ± 0.1dB | Note: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the ratio of cell 2 CPICH code level / Ior SCH Ec / Ior is the ratio of cell 2 SCH code level / Ior |
| 6.6.6.1 NR SA FR1 SRS-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB | Average Noc is the AWGN on virtual UE frequency (NR) on NR freq1 Ês / Noc is the ratio of virtual UE signal / AWGN over the SRS resource RBs |  |
| 6.6.8.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on Cell 1 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |
| 6.6.8.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on Cell 1 frequency Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |
| 6.6.8.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on Cell 1 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |
| 6.6.9.1 NR SA FR1 Idle mode CA/DC measurement for FR1 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.15.1 NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on LTE freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.17.1 NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.6.17.2 NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation | Same as 6.6.17.1 | Same as 6.6.17.1 |
| 6.6.18.1 NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Noc3 is the AWGN on NR freq3 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.18.2 NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers | Same as 6.6.18.1 | Same as 6.6.18.1 |
| 6.6.19.1 NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.19.3 NR SA FR1 NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 6.6.19.4 NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX | Same as 6.6.1.8 | Same as 6.6.1.8 |
| 6.6.20.1 UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB | Noc1 is the AWGN on NR freq Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.21.1 UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB meas PRB Ês1 / Noc ±0.3 dB meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês1 / Noc is the cell 1 SNR Ês2 / Noc is the cell 2 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 6.7.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB meas PRB Ês1 / Noc1 ±0.3 dB meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the cell 2 SNR Ês2 / Noc2 is the cell 3 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 6.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 6.7.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 6.7.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 6.7.3.1 NR SA FR1 SS-SINR measurement accuracy | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 6.7.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 6.7.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 6.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 1 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 6.7.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 6.7.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 6.7.5.1 NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.8 dB | Noc is the AWGN on E-UTRA freq1 Ês / Noc is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for RSRP #RB22 to RB27 |
| 6.7.6.1 NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Same as 6.7.5.1 | Same as 6.7.5.1 |
| 6.7.7.1 NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Same as 6.7.5.1 | Same as 6.7.5.1 |
| 6.7.8.1 NR SA FR1 SRS-RSRP measurement accuracy | Averaqe Noc ±1.5 dB Average Ês / Noc ±0.3 dB | Average Noc is the AWGN on virtual UE frequency (NR) on NR freq1 Ês / Noc is the ratio of virtual UE signal / AWGN over the SRS resource RBs |  |
| 6.7.9.1.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the CSI-RS#0 & CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |
| 6.7.9.1.2 NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |
| 6.7.9.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the SSB#0 & SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |
| 6.7.9.3.1 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |
| 6.7.9.3.2 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |

<< Skipped sections >>

### F.1.3.2	Measurement of RRM requirements

<< Skipped tables >>

Table F.1.3.2-2: Derivation of test requirements for NR SA FR1 RRM tests

| Test |  | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |
| 6.1.1.3 NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB In Signalling SSearchDeltaP = 3dB | During T1: 0dB -0.45dB 0dB During T2: 0dB 0dB -0.45dB In Signalling 3dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +12.55dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +12.55dB In Signalling SSearchDeltaP = 6dB |
| 6.1.1.4 NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB -0.45dB 0dB During T2: 0dB 0dB -0.45dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +12.55dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +12.55dB |
| 6.1.1.5 NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 48dB Thresh_X_LowP of Cell 1 = 50dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 44dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 3dB | During T1: 0dB 0dB 0dB 0.3dB During T2: 0dB 0dB 0dB -2.25dB In signalling: 0dB -4dB -2dB 0dB 0dB 2dB 0dB 12dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -3.7dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc1: -98dBm/15kHz Ês2 / Noc2: +9.75dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 44dB Thresh_X_LowP of Cell 1 = 48dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 46dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 15dB |
| 6.1.1.6 NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: SSearchThresholdP of Cell 1 = 50dB SSearchThresholdP of Cell 2 = 50dB | During T1: 0dB 2dB -2dB 0.35dB During T2: 0dB 0dB -2dB 4dB In signalling: 0dB -16dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +16dB Noc2: -100dBm/15kHz Ês2 / Noc2: -3.65dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc1: -100dBm/15kHz Ês2 / Noc2: +16dB In signalling: SSearchThresholdP of Cell 1 = 50dB SSearchThresholdP of Cell 2 = 34dB |
| 6.1.1.7 NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |
| 6.1.1.8 NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17 |  | Same as 6.1.1.2 | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 6.1.2.1 NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 1.6dB 1.6dB During T3: 0dB -2dB 1.6dB 0.4dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB During T3: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB |
| 6.1.2.2 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |
| 6.1.2.3 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |
| 6.1.2.4 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |
| 6.1.2.5 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 |  | Same as 6.1.2.2 | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 6.2.1 NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT) |  | For configs 1,2: cg-SDT-RSRP-ChangeThreshold: 14dB cg-SDT-RSRP-ThresholdSSB: -110dBm SNR during T1: 0dB T2: 8dB T3: 0dB T4: 18dB T5: 18dB T6: 0dB AWGN during T1: -100dB T2: -100dB T3: -100dB T4: -100dB T5: -100dB T6: -100dB For config 3: cg-SDT-RSRP-ChangeThreshold: 14dB cg-SDT-RSRP-ThresholdSSB: -110dBm SNR during T1: 0dB T2: 8dB T3: 0dB T4: 18dB T5: 18dB T6: 0dB AWGN during T1: -100dB T2: -100dB T3: -100dB T4: -100dB T5: -100dB T6: -100dB | For configs 1,2: 0dB -7dB SNR offset during T1: 0dB T2: +2dB T3: 0dB T4: -3dB T5: +1.6dB T6: -0.95dB AWGN during T1: +1.05dB T2: +1.05dB T3: -1dB T4: -1dB T5: +0.5dB T6: -5dB For config 3: 0dB -9dB SNR offset during T1: 0dB T2: +2.5dB T3: 0dB T4: -2.5dB T5: 0dB T6: -2.8dB AWGN during T1: +1.8dB T2: +1.8dB T3: -1.25dB T4: -1.25dB T5: -3.7dB T6: -7.95dB | For configs 1,2: cg-SDT-RSRP-ChangeThreshold: 14dB cg-SDT-RSRP-ThresholdSSB: -117dBm SNR during T1: 0dB T2: 10dB T3: 0dB T4: 15dB T5: 19.6dB T6: -0.95dB AWGN during T1: -98.95dB T2: -98.95dB T3: -101dB T4: -101dB T5: -99.5dB T6: -105dB For config 3: cg-SDT-RSRP-ChangeThreshold: 14dB cg-SDT-RSRP-ThresholdSSB: -119dBm SNR during T1: 0dB T2: 10.5dB T3: 0dB T4: 15.5dB T5: 18dB T6: -2.8dB AWGN during T1: -98.2dB T2: -98.2dB T3: -101.25dB T4: -101.25dB T5: -103.7dB T6: -107.95dB |
| 6.3.1.1 NR SA FR1 handover with known target cell |  | TEST CONFIGURATION 1, 2 A3-Offset: 0dB | -1dB | A3-Offset: -1dB |
|  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |
|  |  | TEST CONFIGURATION 3 A3-Offset: 0dB | -1dB | A3-Offset: -1dB |
|  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |
| 6.3.1.2 NR SA FR1 handover with unknown target cell |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB |
| 6.3.1.3 NR SA FR1-FR1 Handover with unknown Target Cell |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB |
| 6.3.1.4 NR SA FR1 – E-UTRA handover with known target cell |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: -infinity During T2: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +8dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +8dB | During T1: 0dB 0dB 1.55dB 0dB During T2: 0dB 0dB -1.55dB 1.55dB During T3: 0dB 0dB -1.55dB 1.55dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.55dB Ês2 / Noc2: -infinity During T2: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -5.55dB Ês2 / Noc2: +9.55dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -5.55dB Ês2 / Noc2: +9.55dB |
| 6.3.1.5 NR SA FR1 – E-UTRA handover with unknown target cell |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB |
| 6.3.1.6 NR SA FR1 – UTRAN FDD handover with known target cell |  | During T1: Noc: -100dBm/15kHz Ês / Noc: 12dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -100dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T3: Noc: -100dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB | During T1: -1.54dB 0dB 0dB 0dB 0dB 0dB During T2: -1.54dB 0dB 0dB 0dB 0dB 0dB During T3: -1.54dB 0dB 0dB 0dB 0dB 0dB | During T1: Noc: -101.54dBm/15kHz Ês / Noc: 12dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -101.54dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T3: Noc: -101.54dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |
| 6.3.1.7 NR SA FR1 synchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB In signaling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 1dB During T3: 0dB 0dB 1dB During T4: 0dB 0dB 1dB During T5: 0dB 0dB 1dB In signaling: -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signaling: A3-offset: -1dB |
| 6.3.1.8 NR SA FR1 asynchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB In signaling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 1dB During T3: 0dB 0dB 1dB During T4: 0dB 0dB 1dB During T5: 0dB 0dB 1dB In signaling: -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signaling: A3-offset: -1dB |
| 6.3.1.9 NR SA FR1 Intra-band inter-frequency synchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB In signaling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 3.7dB During T3: 0dB 0dB 3.7dB During T4: 0dB 0dB 3.7dB During T5: 0dB 0dB 3.7dB In signaling: -3dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB In signaling: A3-offset: -3dB |
| 6.3.1.10 NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB In signaling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 3.7dB During T3: 0dB 0dB 3.7dB During T4: 0dB 0dB 3.7dB During T5: 0dB 0dB 3.7dB In signaling: -3dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB In signaling: A3-offset: -3dB |
| 6.3.1.11 NR SA FR1 Inter-band inter-frequency synchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signaling: A3-offset: -6dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB During T4: 0dB 0dB 0dB During T5: 0dB 0dB 0dB In signaling: -1dB for config 1,2,4,5,9 In signaling: -4dB for config 3,6 In signaling: 2dB for config 7,8 | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signaling: -7dB for config 1,2,4,5,9 In signaling: -10dB for config 3,6 In signaling: -4dB for config 7,8 |
| 6.3.1.12 NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signaling: A3-offset: -4dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB During T4: 0dB 0dB 0dB During T5: 0dB 0dB 0dB In signaling: -3dB for config 1,2,4,5,9 In signaling: -6dB for config 3,6 In signaling: 0dB for config 7,8 | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signaling: -7dB for config 1,2,4,5,9 In signaling: -10dB for config 3,6 In signaling: -4dB for config 7,8 |
| 6.3.2.1.1 NR SA FR1 RRC re-establishment |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |
| 6.3.2.1.2 NR SA FR1 - FR1 RRC re-establishment |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB |
| 6.3.2.1.3 NR SA FR1 RRC re-establishment without serving cell timing |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |
| 6.3.2.2.1 |  | Absolute uplink power: Normal conditions ±9dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc | 2.1dB 0.7dB 112Tc 112Tc | Absolute uplink power: Normal conditions ±11.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |
| 6.3.2.2.2 |  | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 6.3.2.2.3 |  | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 6.3.2.2.4 |  | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 6.3.2.3.1 NR SA FR1 RRC connection release with redirection |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |
| 6.3.2.3.2 NR SA FR1 – E-UTRA RRC connection release with redirection |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |
| 6.3.3.1 NR SA FR1 conditional handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11dB In signaling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB -1dB In signaling: -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signaling: A3-offset: -1dB |
| 6.3.3.2 NR SA FR1-FR1 conditional handover |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.7dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +6.7dB |
| 6.4.1.1 EN-DC FR1 UE transmit timing accuracy |  | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês1 / Noc: +3.00dB | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês1 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês1 / Noc: +3.30dB |
| 6.4.3.1 NR SA FR1 timing advance adjustment accuracy |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT | 0 0 0 +/- 88 Tc +/- 88 Tc | Noc = -98 dBm/15 kHz (Config 1, 2) Noc = -95 dBm/15 kHz (Config 3) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4.1.1 |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode |  | Same as 6.5.1.1 | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode |  | Same as 6.5.1.2 | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Same as 6.5.1.1 | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Same as 6.5.1.2 | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  | Same as 6.5.1.1 | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  | Same as 6.5.1.2 | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 6.5.1.9 SA FR1 radio link monitoring out-of-sync Test for PSCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion |  | SNR during T1: 1dB T2: 1dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: 1.8dB T3: -15.8dB |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.3.1 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.3.2 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle |  | Same as 6.5.3.1 | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.3.3 NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX |  | Same as 6.5.3.1 | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.3.4 NR SA FR1 direct SCell activation at SCell addition of known SCell |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.3.5 NR SA FR1 direct SCell activation at handover with known SCell |  | During T1: Noc1: -98 dBm/15 kHz Noc2: -98 dBm/15 kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB Ês3 / Noc1: +8dB During T2: Noc1: -98 dBm/15 kHz Noc2: -98 dBm/15 kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB Ês3 / Noc1: +8dB During T3: Noc1: -98 dBm/15 kHz Noc2: -98 dBm/15 kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB Ês3 / Noc1: +8dB In signaling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB In signaling: 0dB | During T1: Noc1: -98 dBm/15 kHz Noc2: -98 dBm/15 kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB Ês3 / Noc1: +8dB During T2: Noc1: -98 dBm/15 kHz Noc2: -98 dBm/15 kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB Ês3 / Noc1: +8dB During T3: Noc1: -98 dBm/15 kHz Noc2: -98 dBm/15 kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB Ês3 / Noc1: +8dB In signaling: A3-offset = -15dB |
| 6.5.3.10 NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB In signaling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB In signaling: 0 dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB In signaling: A3-offset = -15dB |
| 6.5.3.11 NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640 ms SCell measurement cycle |  | Same as 6.5.3.10 | Same as 6.5.3.10 | Same as 6.5.3.10 |
| 6.5.5.1 NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |
| 6.5.5.2 NR SA FR1 SSB-based beam failure detection and link recovery in DRX |  | Same as 6.5.5.1 | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 6.5.5.3 NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês1 / Noc1: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |
| 6.5.5.4 NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX |  | Same as 6.5.5.3 | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 6.5.5.5 NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX |  | On the SCell: q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB | On the SCell: Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB | On the SCell: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |
| 6.5.5.6 NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX |  | Same as 6.5.5.5 | Same as 6.5.5.5 | Same as 6.5.5.5 |
| 6.5.5.7 NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX |  | q0,0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q0,1 CSI-RS SNR during: T1-T5: 5dB q1,0 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1,0 CSI-RS during T2: Noc2: -98dBm/15kHz Ês1 / Noc1: -10dB q1,0 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês1 / Noc1: +10dB | q0,0 Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q0,1 Offset during: T1-T5: +0.8dB q1,0 CSI-RS during T1: 0dB -0.2dB q1,0 CSI-RS during T2: 0dB -0.2dB q1,0 CSI-RS during T3, T4 and T5: 0dB +0.2dB | q0,0 SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q0,1 SNR during: T1-T5: +5.8dB q1,0 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1,0 CSI-RS during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1,0 CSI-RS during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |
| 6.5.6.1.1 NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.6.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T2: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T3: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T2: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T3: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |
| 6.5.6.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB | During T1: 0dB 0dB | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX |  | Same as 64.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.7 NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.5.3.1 |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.3.2 |  | Same as 6.5.3.1 | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.3.3 |  | Same as 6.5.3.1 | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.7.1 NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA |  | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +17dB Reported CSI-RSRP values ± 10dB | -1.10dB -1.10dB 0dB 0dB Via mapping | Noc1: -105.10dBm/15kHz Noc2: -105.10dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +17dB CSI-RS#0: RSRP_64 to RSRP_84 CSI-RS#1: RSRP_67 to RSRP_87 |
| 6.5.7.2 NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA |  | Same as 6.5.7.1 | Same as 6.5.7.1 | Noc1: -105.10dBm/15kHz Noc2: -105.10dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +17dB CSI-RS#0: RSRP_67 to RSRP_87 CSI-RS#1: RSRP_67 to RSRP_87 |
| 6.5.7A.2 NR SA FR1 DL interruptions at switching between two uplink carriers in TDD-TDD CA |  | Same as 6.5.7.2 | Same as 6.5.7.2 | Same as 6.5.7.2 |
| 6.5.7B.2 NR SA FR1 DL interruptions at switching between two uplink bands in TDD-TDD CA |  | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +14dB Ês3 / Noc: +14dB Reported CSI-RSRP values ± 10dB | -1.10dB -1.10dB -1.10dB 0dB 0dB 0dB Via mapping | Noc1: -105.10dBm/15kHz Noc2: -105.10dBm/15kHz Noc3: -105.10dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +14dB Ês3 / Noc: +14dB CSI-RS#0: RSRP_67 to RSRP_87 CSI-RS#1: RSRP_67 to RSRP_87 CSI-RS#2: RSRP_67 to RSRP_87 |
| 6.5.8.1 UE specific CBW change on PCell in FR1 in non-DRX |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB | During T1: 0dB 0dB | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |
| 6.6.1.8 NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17 |  | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês2 / Noc: +4.00dB |
| 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX |  | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB |
| 6.6.2.2 NR SA FR1-FR1 event-triggered reporting in DRX |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.12 NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.3.1 NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX |  | During T1: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: 0dB Ês2 / Noc: +17.00dB | During T1: 0dB 0dB +1.55dB 0dB During T2: 0dB 0dB -1.55dB +1.55dB | During T1: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: +17.55dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -104+TTdBm/15kHz Freq 2 Noc: -104+TTdBm/15kHz Ês1 / Noc: -1.55dB Ês2 / Noc: +18.55dB |
| 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX |  | Same as 6.6.3.1 | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 6.6.3.3 NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 |  | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX |  | During T1: Noc: -98dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#1: RSRP_55 to RSRP_75 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX |  | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  | Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB | 0dB 0dB 0.5dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 CSI-RS#1: RSRP_55 to RSRP_75 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 CSI-RS#1: RSRP_58 to RSRP_78 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 |
| 6.6.4.4 NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX |  | Same as 6.6.4.3 | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 6.6.4.5 NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 6.6.5.1 NR SA FR1 – UTRAN event-triggered reporting in non-DRX |  | During T1: Noc: -106dBm/15kHz Ês / Noc: 18dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -106dBm/15kHz Ês / Noc: -2dB Ioc: -70dBm/3.84MHz Îor / Ioc: -1.8dB CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB | During T1: Noc: -106dBm/15kHz Ês / Noc: 18dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -106dBm/15kHz Ês / Noc: -2dB Ioc: -70dBm/3.84MHz Îor / Ioc: -1.8dB CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |
| 6.6.6.1 NR SA FR1 SRS-RSRP measurement with non-DRX |  | Test Configuration 1: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +4dB Test Configuration 2: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +4dB | Test Config 1: T1: 0dB 0dB T2: 0dB +5.25dB Test Config 2: T1: 0dB 0dB T2: 0dB +4.75dB | Test Configuration 1: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +9.25dB Test Configuration 2: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +8.75dB |
| 6.6.8.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ±5.5dB Reported Differential CSI-SINR values ±4.5dB | 0dB 0dB 0dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_41 to SINR_64 CSI-RS#0: DIFFSINR_0 to DIFFSINR_7 |
| 6.6.8.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SS-SINR values ±4dB Reported Differential SS-SINR values ±3dB | 0dB 0.5dB 0dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.50dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 SSB#1: SINR_44 to SINR_61 SSB#0: DIFFSINR_0 to DIFFSINR_5 |
| 6.6.8.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ±4.5dB Reported Differential CSI-SINR values ±3.5dB | 0dB 0dB 0dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_43 to SINR_62 CSI-RS#0: DIFFSINR_0 to DIFFSINR_6 |
| 6.6.9.1 NR SA FR1 Idle mode CA/DC measurement for FR1 |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB Reported SS-RSRP and SS-RSRQ | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB Via mapping | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB Configuration 1 and 2: Cell 1: RSRP_54 to RSRP_77 RSRQ_55 to RSRQ_72 Cell 2: RSRP_47 to RSRP_70 RSRQ_51 to RSRQ_67 Configuration 3: Cell 1: RSRP_57 to RSRP_80 RSRQ_55 to RSRQ_72 Cell 2: RSRP_50 to RSRP_73 RSRQ_51 to RSRQ_67 |
| 6.6.15.1 NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: -3dB During T4: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +8dB During T5: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB Thresh_X_HighP 48dB Reported SS-RSRP and SS-RSRQ | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB During T4: 0dB 0dB 0dB 0dB During T5: 0dB 0dB 0dB 0dB +4dB Via Mapping | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: -3dB During T4: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +8dB During T5: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB 52dB Configuration 1 and 2: Cell 1: RSRP_52 to RSRP_75 RSRQ_54 to RSRQ_71 Cell 2: RSRP_39 to RSRP_62 RSRQ_6 to RSRQ_27 Configuration 3: Cell 1: RSRP_55 to RSRP_78 RSRQ_54 to RSRQ_71 Cell 2: RSRP_39 to RSRP_62 RSRQ_6 to RSRQ_27 |
| 6.6.17.1 NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG |  | During T1, T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1, T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |
| 6.6.17.2 NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation |  | Same as 6.6.17.1 | Same as 6.6.17.1 | Same as 6.6.17.1 |
| 6.6.18.1 NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers |  | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Freq 3 Noc3: -98dBm/15kHz Freq 1 Ês1 / Noc1: +4.00dB Freq 2 Ês2 / Noc2: -infinity Freq 3 Ês3 / Noc3: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Freq 3 Noc3: -98dBm/15kHz Freq 1 Ês1 / Noc1: +4.00dB Freq 2 Ês2 / Noc2: +7.00dB Freq 3 Ês3 / Noc3: +7.00dB | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Freq 3 Noc3: -98dBm/15kHz Freq 1 Ês1 / Noc1: +4.00dB Freq 2 Ês2 / Noc2: -infinity Freq 3 Ês3 / Noc3: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Freq 3 Noc3: -98dBm/15kHz Freq 1 Ês1 / Noc1: +4.00dB Freq 2 Ês2 / Noc2: +7.00dB Freq 3 Ês3 / Noc3: +7.00dB |
| 6.6.18.2 NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers |  | Same as 6.6.18.1 | Same as 6.6.18.1 | Same as 6.6.18.1 |
| 6.6.19.1 NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1 |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.19.2 NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.19.3 NR SA FR1 NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG |  | Same as 6.6.3.1 | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 6.6.19.4 NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX |  | Same as 6.6.1.8 | Same as 6.6.1.8 | Same as 6.6.1.8 |
| 6.6.20.1 UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA |  | TEST CONFIGURATION 1, 2 |  |  |
|  |  | During T1: Noc : -98dBm/15kHz Ês / Noc: infinity During T2: Noc: -98dBm/15kHz Ês / Noc: -2dB | During T1: 0 dB 0 dB During T2: 0 dB 0 dB | During T1: Noc : -98dBm/15kHz Ês / Noc: infinity During T2: Noc: -98dBm/15kHz Ês / Noc: -2dB |
|  |  | TEST CONFIGURATION 3 |  |  |
|  |  | During T1: Noc: -95dBm/30kHz Ês / Noc: infinity During T2: Noc1: -95dBm/30kHz Ês1 / Noc1: -2dB | During T1: 0 dB 0 dB During T2: 0 dB 0 dB | During T1: Noc: -95dBm/30kHz Ês / Noc: infinity During T2: Noc1: -95dBm/30kHz Ês1 / Noc1: -2dB |
| 6.6.21.1 UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA |  | TEST CONFIGURATION 1, 2 |  |  |
|  |  | During T1: Noc : -98 dBm/15kHz Ês / Noc: infinity During T2: Noc: -98 dBm/15kHz Ês / Noc: -3 dB | During T1: 0 dB 0 dB During T2: 0 dB 0 dB | During T1: Noc : -98 dBm/15kHz Ês / Noc: infinity During T2: Noc: -98 dBm/15kHz Ês / Noc: -3 dB |
|  |  | TEST CONFIGURATION 3 |  |  |
|  |  | During T1: Noc: -95 dBm/30kHz Ês / Noc: infinity During T2: Noc1: -95 dBm/30kHz Ês1 / Noc1: -3 dB | During T1: 0 dB 0 dB During T2: 0 dB 0 dB | During T1: Noc: -95 dBm/30kHz Ês / Noc: infinity During T2: Noc1: -95 dBm/30kHz Ês1 / Noc1: -3 dB |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy |  | TEST CONFIGURATION 1, 2, |  |  |
|  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping | Test 1: Noc: -107.5dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_34 to RSRP_46 RSRP_34 to RSRP_46 RSRP_35 to RSRP_47 RSRP_35 to RSRP_47 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |
|  |  | TEST CONFIGURATION 3 |  |  |
|  |  | Test 1: Noc: -110dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -0.8dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping | Test 1: Noc: -110.8dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_41 to RSRP_53 Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 RSRP_38 to RSRP_50 RSRP_38 to RSRP_50 RSRP_39 to RSRP_51 RSRP_40 to RSRP_52 RSRP_40 to RSRP_52 depending on operating band |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |
| 6.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy |  | Test 1: Noc: Test configuration 1, 2:-106dBm/15kHz Test configuration 3:-110dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported re. SS-RSRP values: ±3dB | Test 1: 0 dB 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping | Test 1: Noc: -106 dBm/15kHz Noc: -110 dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |
| 6.7.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_62 to RSRP_81 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 depending on operating band |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0 dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |  |
| 6.7.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported relative RSRP values: ±4.5dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported relative RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_x-7 to RSRP_x+7 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_x-31 to RSRP_x-18 |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1.5dB wider at each end. |  |  |  |  |
| 6.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc: -85dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -86.5dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_46 to RSRQ_60 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB RSRQ_44 to RSRQ_59 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc: -91dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -92.6dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB RSRQ_44 to RSRQ_59 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |  |
| 6.7.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |  |
| 6.7.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 1 and Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1dB wider at each end. |  |  |  |  |
| 6.7.3.1 NR SA FR1 SS-SINR measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping | Test 1: Noc: -93.2dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |  |
| 6.7.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_32 to SINR_49 |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |  |
| 6.7.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±4dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_x-11 to SINR_x+11 |
| The derivation of the SINR values takes into account the uncertainty in Cell 1 and Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each for Tests 1 and 2 and 0dB wider at each end for Test 3. |  |  |  |  |
| 6.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB RSRP_62 to RSRP_82 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_31 to RSRP_44 RSRP_31 to RSRP_45 RSRP_32 to RSRP_45 RSRP_32 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 depending on operating band |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 RSRP_35 to RSRP_48 RSRP_35 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_37 to RSRP_51 depending on operating band |
| 6.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 |
| 6.7.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 6.7.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.2 |  | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 |
| 6.7.5.1 NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Test 1: Noc: -91.65dBm/15kHz Ês / Noc: +10.0dB Reported RSRP values: ±9dB (±2dB additionally for extreme conditions) Test 2: Noc: -117dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RSRP values: ±4.5dB (±3.5dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc: -91.65dBm/15kHz Ês / Noc: 10dB RSRP_48 to RSRP_70 Test 2: Noc: -117dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_30 RSRP_18 to RSRP_30 Depending on operating band |
| 6.7.6.1 NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Test 1: Noc: -83dBm/15kHz Ês / Noc: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc: -104.7dBm/15kHz Ês / Noc: -4.0dB Reported RSRQ values: ±3.5dB (±0.5dB additionally for extreme conditions) Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RSRQ values: ±3.5dB (±0.5dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping Test 3: 0dB 0.8dB Via mapping | Test 1: Noc: -83dBm/15kHz Ês / Noc: -1.75dB RSRQ_4 to RSRQ_16 Test 2: Noc: -104.7dBm/15kHz Ês / Noc: -3.2dB RSRQ_0 to RSRQ_16 Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB RSRQ_0 to RSRQ_16 |
| 6.7.7.1 NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Test 1: Noc: -88dBm/15kHz Ês / Noc: -1.75dB Reported RS-SINR values: ±3.0dB (±1.0dB additionally for extreme conditions) Test 2: Noc: -108.5dBm/15kHz Ês / Noc: 20.0dB Reported RS-SINR values: ±3.0dB (±1.0dB additionally for extreme conditions) Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RS-SINR values: ±3.5dB (±0.5dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0.8dB Via mapping | Test 1: Noc: -88dBm/15kHz Ês / Noc: -1.75dB SINR_35 to SINR_51 Test 2: Noc: -108.5dBm/15kHz Ês / Noc: 20.0dB SINR_79 to SINR_94 Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB SINR_32 to SINR_49 |
| 6.7.8.1 NR SA FR1 SRS-RSRP measurement accuracy | Test Configuration 1: Test 1: Noc = -106dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3dB Test 2: Noc = -88dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±6.5dB Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3dB Test Configuration 2: Test 1: Test 1 is not used when configuring SCS 30kHz Test 2: Noc = -91dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±7dB Test 3: Noc = -114+ ΔBG_offset dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3.5dB |  | Test Config 1: Test 1: 0dB +0.5dB Via mapping Test 2: 0dB +0.5dB Via mapping Test 3: 0dB +0.75dB for BG_A +0.5dB otherwise Via mapping Test Config 2: Test 1: N/A Test 2: 0dB +0.5dB Via mapping Test 3: 0dB +1.25dB for BG_A +0.5dB otherwise Via mapping | Test Config 1: Test 1: Noc = -106dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 30 to 41 Test 2: Noc = -88dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 45 to 62 Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +1.75dB for BG_A Ês / Noc = +1.5dB otherwise Reported SRS-RSRP: BG_A: 23 to 33 BG_C: 23 to 34 BG_D: 24 to 34 BG_E: 24 to 35 Test Config 2: Test 1: N/A Test 2: Noc = -91dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 44 to 63 Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +2.25dB for BG_A Ês / Noc = +1.5dB otherwise Reported SRS-RSRP: BG_A: 26 to 37 BG_C: 26 to 37 BG_D: 26 to 38 BG_E: 27 to 38 |
| 6.7.9.1.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Same as 4.7.7.1 | Same as 4.7.7.1 | Same as 4.7.7.1 |
| Test Configuration 3,6 |  | Same as 4.7.7.1 | Same as 4.7.7.1 | Same as 4.7.7.1 |
| 6.7.9.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Same as 4.7.7.2 | Same as 4.7.7.2 | Same as 4.7.7.2 |
| Test Configuration 3,6 |  | Same as 4.7.7.2 | Same as 4.7.7.2 | Same as 4.7.7.2 |
| 6.7.9.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Same as 4.7.7.3 | Same as 4.7.7.3 | Same as 4.7.7.3 |
| Test Configuration 3,6 |  | Same as 4.7.7.3 | Same as 4.7.7.3 | Same as 4.7.7.3 |

<< End of Changes >>