3GPP TSG-RAN WG5 Meeting #102	R5-241792

Athens, Greece, February 26th – March 1st, 2024            (R5-241413r3, revision of R5-241413r2)

Agenda Item:	5.3.10.9

Source:	THALES

Title: 	Neighbor GSO satellite ephemeris file for NTN NR UE testing

Document for:	Discussion

# Introduction

The main objective of this technical discussion document is to present an updated methodology to generate satellite Ephemeris for NTN NR UE testing that incorporates both SIB19 (SIB31/(SIB32)/SIB33 for NB-IoT) state vector and agreement reached at RAN4#106-bis with respect to NTN UE testing.

The data required by the Test Equipment to implement SIB19 (NTN dedicated SIB from 5G NR Rel-17 and Rel-18) or SIB31/(SIB32)/SIB33 (NTN dedicated SIB for NB-IoT starting from Rel-17 and Rel-18) is based on two potential information: ECEF information and/or Orbital information. Effectively, it has been agreed in RAN1/RAN2 that UEs are provided with satellite ephemeris in ECI of the date (namely ECEF) in either state vector (position velocity) or orbital information (Kepler elements).

The testing methodology from this discussion document can apply for test preparation of NTN NR UE either for:

- Testing at the time of the current date using GNSS real time resources, or

- Testing at a previous date earlier than the current date (i.e. 1 day) using stored GNSS information or GNSS information to be retrieved from AT commands.

The recovery, the interpretation and processing of location information by the UE (pre-configured location, GNSS information or AT commands) is out scope of this document.

This approach is in line with maximum errors described in R1-2111122 "Considerations on UL timing and frequency synchronization in NTN" or the testing process described in R4-2302517 "RRM test cases methodology and configuration for SAN NTN assistance information parameters".

In addition to Range and Doppler effect evaluation, the delay, delay shift considered. Furthermore, the state vector (position, velocity) are given in SIB19/SIB31/(SIB32)/SIB33 format using the dedicated transfer functions.

This work represents an incremental update of previous RAN5 contribution R5-233941 “Satellite Ephemeris file generation methodology for NTN NR UE testing” (THALES) and R5-237213 “GSO satellite Ephemeris file generation methodology for NTN NR UE testing” (THALES). This contribution further provides information with respect to neighbor GSO satellite ephemeris data and related channel Doppler/Delay values. As a matter of fact, the serving GSO satellite ephemeris data has been already provided in R5-237213.

Is important to mention that the process and provided data are applicable/are valid for both 5G NR NTN (SIB19) and NB-IoT NTN (SIB31/(SIB32)/SIB33).

Note: The process and provided data remains applicable for different frequency ranges.

# Recall of RRC protocol (TS 38.331) in Rel-17

The specification TS 38.331 "Technical Specification Group Radio Access Network - NR - Radio Resource Control (RRC) protocol specification" indicates the required satellite ephemeris information to be included in SIB and transmitted/broadcasted by the satellite. The UE uses this information (namely SIB19 for NTN 5GNR) to perform UL Doppler and timing advance pre-compensation.

-- ASN1START

-- TAG-EPHEMERISINFO-START

EphemerisInfo-r17 ::=          CHOICE {

positionVelocity-r17           PositionVelocity-r17,

orbital-r17                    Orbital-r17

}

PositionVelocity-r17 ::=       SEQUENCE {

positionX-r17                  PositionStateVector-r17,

positionY-r17                  PositionStateVector-r17,

positionZ-r17                  PositionStateVector-r17,

velocityVX-r17                 VelocityStateVector-r17,

velocityVY-r17                 VelocityStateVector-r17,

velocityVZ-r17                 VelocityStateVector-r17

}

Orbital-r17 ::=                SEQUENCE {

semiMajorAxis-r17              INTEGER (0..8589934591),

eccentricity-r17               INTEGER (0..1048575),

periapsis-r17                  INTEGER (0..268435455),

longitude-r17                  INTEGER (0..268435455),

inclination-r17                INTEGER (-67108864..67108863),

meanAnomaly-r17                INTEGER (0..268435455)

}

PositionStateVector-r17 ::= INTEGER (-33554432..33554431)

VelocityStateVector-r17 ::= INTEGER (-131072..131071)

-- TAG-EPHEMERISINFO-STOP

-- ASN1STOP

With the following description:

EphemerisInfo field descriptions

Eccentricity Satellite orbital parameter: eccentricity e, see NIMA TR 8350.2 [71]. Unit is radian. Step of 1.431 * 10-8. Actual value = field value * (1.431 * 10-8).

Inclination Satellite orbital parameter: inclination i, see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

Longitude Satellite orbital parameter: longitude of ascending node , see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

meanAnomaly Satellite orbital parameter: Mean anomaly M at epoch time, see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

Periapsis Satellite orbital parameter: argument of periapsis , see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

positionX, positionY, positionZ X, Y, Z coordinate of satellite position state vector in ECEF. Unit is meter. Step of 1.3 m. Actual value = field value * 1.3.

semiMajorAxis Satellite orbital parameter: semi major axis , see NIMA TR 8350.2 [71]. Unit is meter. Step of 4.249 * 10-3 m. Actual value = 6500000 + field value * (4.249 * 10-3).

velocityVX, velocityVY, velocityVZ X, Y, Z coordinate of satellite velocity state vector in ECEF. Unit is meter/second. Step of 0.06 m/s. Actual value = field value * 0.06.

Those transfer functions previously presented are directly incorporated in the GMAT script used for emulating satellite movement, relative speed and velocity with respect to ground UE.

# Agreement on NTN UE testing

The test of NTN UE for NGSO satellite has been discussed at several occasions during RAN4#108 Toulouse. Meanwhile, after the RAN4#108 meeting, RAN#101 decided the following Way Forward in RP-232682 “Summary of offline discussions for NTN testing”:

Way Forward:

- RAN tasks RAN4 to discuss and conclude in Oct meeting on

  - The condition for testing the frequency error requirements to verify NGSO UE pre-compensation functionality for Doppler shift, which will be implemented in the RAN5 NTN specifications

    - For example, setting an artificial fixed Doppler shift, which is randomly selected out of a range of Doppler shift values decided by RAN4, as the condition in a test

  - The condition for RRM uplink timing test cases to verify NGSO UE delay pre-compensation functionality, which can be implemented in RAN5 NTN specifications

    - For example, setting an artificial fixed delay, which is randomly selected out of a range of delay values decided by RAN4, as the condition in a test

    - Note: For all the other NTN RRM test cases (except NGSO scenario in UE timing TCs), zero doppler shift and zero timing shift are assumed to be configured for RAN5 conformance testing

- Further discuss TE-emulated channel model with delay and Doppler shifts matching the satellite propagator model in future release i.e. Rel-19

- No changes to the RAN4 core requirements

More discussion took place before the RAN4#108-bis meeting and during the meeting, which generated the following documents:

- R4-2317963, Topic summary for [108bis][332] LS_BSRF_RAN_task;

- R4-2316953, Ad-hoc meeting minutes on LS_BSRF_RAN_task.

At the end, the final agreements for NGSO/GSO testing reached at RAN4#108-bis were:

(priority) Issue 1-1: [NGSO][RF] Range of Doppler shift for RF frequency error requirements

agreement

- For one of the cases in the freq error test, the range of absolute value of Doppler is within [0.1 or 7.5ppm] greater than zero and less than or equal to 24ppm.

  - The range can be further limited, considering other factors like elevation angles and satellite height.

  - The exact value(s) and the number of values (>1) to be selected are left to RAN5.

    - At least the worst-case value needs to be selected

  - UE should derive the amount of Doppler to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

    - The same ephemeris info will be maintained during each single frequency error measurement of the TE

Issue 1-2: [NGSO][RF] Assumption of delay shift for RF frequency error requirements

Agreement:

- Constant value from the same ephemeris (i.e., orbit emulation) as the Doppler shift is derived

- UE is assumed to derive the amount of delay to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

- The requirement is defined assuming the UE pre-compensates the amount of delay

(priority) Issue 1-3: [NGSO][RRM] Range of Delay shift for RRM UL timing accuracy test cases

Agreement:

- The range of the one-way delay from UE to satellite is within 2ms (lowest value for LEO orbit 600km) to 6.67ms (highest for LEO orbit 1200 km)

  - Assuming that the absolute elevation angle is larger than 30 degree

  - The range can be further limited, considering other factors like elevation angles and satellite height.

  - The set of values and the number of values is to be introduced by RAN4.  RAN5 to select from this set according to RAN4’s test case revision.

    - At least the worst-case value needs to be selected

  - UE should derive the amount of time delay to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

    - The same ephemeris info will be maintained during each single timing accuracy measurement of the TE

Issue 1-4: [NGSO][RRM] Assumption of Doppler shift for RRM UL timing accuracy test cases

Agreement:

- Constant value from the same ephemeris (i.e., orbit emulation) as the delay is derived

- UE is assumed to derive the amount of Doppler to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

- The requirement is defined assuming the UE pre-compensates the amount of Doppler

Issue 1-5: [NGSO][RRM] Assumption of Doppler and timing shifts for RRM test cases other than UL timing accuracy

Agreement: Constant non-zero time delay, with constant non-zero Doppler value per satellite

Sub-topic 1-2: GSO testing

Agreement:  Apply the same principles for GSO as agreed for NGSO

Finally, the following Way Forward has been approved as follows at RAN4#108-bis:

- R4-2316967	WF on NTN RAN task

- Chair:  Issue 2-5 indicates non-zero Doppler for GSO.  In addition, zero Doppler shall be considered for GEO.

# Serving GSO satellite data

The UE selected location is given by TS 36.508 (and respectively by TS 38.508-1):

The generated report file for SIB19/SIB31 contains satellite ephemeris for each time of the propagation, for 2.56 sec simulation step and 7° inclination angle (worst case for GSO), including Doppler and Delay values:

The following [units] have been considered: satellite elevation in UE frame in [degree], one-way Delay in [ms], Doppler in [ppm], so the results remain relevant and representative also for other NTN (FR1 or FR2) radio frequencies to be included in future Releases.

## Further considerations for serving GSO satellite

For any given UE location, the daily elevation variation remains rather low compared to NGSO cases. However, the UE location is limited by the Earth visibility domain with the minimum angle of 10° or 30°.

As an indication, the Earth visibility domain as seen by a satellite in geostationary position and 0° inclination for a minimum elevation of 0° is given the following figure:

Figure 1. Visibility domain from a geostationary satellite, Minimum UE elevation=0

The following table gives some representative correspondence between the maximum UE latitude and the minimum elevation angle between the UE and the satellite (with the satellite GEO longitude at UE longitude):

Table 1. Maximum UE latitude with satellite longitude at UE longitude

| Minimum UE Elevation (degree) | Maximum UE latitude with satellite longitude at UE longitude (spherical Earth) |
| --- | --- |
| 0 | 81.30 |
| 5 | 76.33 |
| 10 | 71.43 |
| 15 | 66.60 |
| 20 | 61.83 |
| 25 | 57.12 |
| 30 | 52.47 |

As also provided in previous THALES contributions, GSO data (including ephemeris, elevation, Doppler, Delay values and their variation in time as seen by the UE) has been provided for other UE latitude different from 25.08439333 degrees (e.g. 55 degrees as seen below) in order to consider an elevation angle target of 30 degrees (and which would not have been possible to achieve with a UE latitude of 25.08439333 degrees):

Therefore, for this above generated file only, UE coordinates were:

- Latitude 55 degrees;

- Longitude 121.56076999 degrees;

- Altitude 0 m.

Otherwise, for the other generated files, the reference UE coordinates are considered such as:

- Latitude 25.08439333 degrees;

- Longitude 121.56076999 degrees;

- Altitude 0 m.

## Further considerations for the Maximum Doppler of the serving GSO satellite

The maximum Doppler during simulation is about ±0.1 ppm. As a matter of fact, this is a common value for geostationary satellite with a fixed (or static) UE location.

Although TR 38.811 shows higher values, is important to mention that these higher results were obtained with UE in motion embarked on an aircraft:

Table 2. Extract of TR 38.811 Rel-15

Finally, the Figure 2 below shows satellite movement and Doppler values seen by the UE during the simulation for a fixed UE position (24 hours have been simulated):

Figure 2. Satellite orbit evolution during 24hrs with UE Doppler

In Figure 2 is represented the “8”-shape variation of the satellite longitude and latitude and also the associated non-zero Doppler variation as seen by the UE during a 24h period. Because of different parameters such as (non-zero) inclination angle, Moon/Sun attraction for instance and (non-zero) orbit eccentricity, and in practice also because of Earth homogeneity/ irregular shape/ density and difficulty of perfectly placing/ controlling & keeping the satellite into orbit (which may be a very expensive process) the satellite orbit evolution is not perfectly circular. For these reasons, perfect circular orbits and zero Doppler conditions are very difficult to achieve. Even for GEO case there is usually a non-zero orbit eccentricity and a residual inclination angle which introduces a Doppler effect. Therefore, in practice, zero Doppler condition should be considered and treated as a non-representative case for NTN.

# Serving GSO satellite data – Option 1

In Figure 2 it can be also seen that the satellite longitude varies around the UE longitude (UE position selected as reference). With respect to this observation, the neighbor satellite position was therefore selected such as the longitude of the neighbor satellites is +/-30 degrees longitude with respect to (same initial) reference UE longitude located at:

- Latitude 25.08439333 degrees;

- Longitude 121.56076999 degrees;

- Altitude 0 m.

For instance, the position of one of the neighbor satellite (with longitude at 91.56076999 degrees) is described below in Figure 3:

Figure 3. Neighbor GSO at longitude 91.56076999 degrees

The generated files for the 2 neighbor satellites for Option 1 (with longitudes at 91.56076999 and 151.56076999 degrees respectively) are given below:

Given said that, is important to test with different serving and neighbor GSO in more dynamic conditions since the relative Doppler between serving and neighbor GSO for Option 1 (delta serving-neighbor Doppler value experienced by the UE for Option 1) may reach up to ± 0.2 ppm with a fixed (or static) UE location condition. For a moving UE case with GSO, Doppler conditions will be higher as previously mentioned.

# Serving GSO satellite data – Option 2

After received offline comments from other companies, results have been included also for neighbor GSO data for a different UE position corresponding to (serving GSO data for this configuration has been already provided during the previous RAN5#101 meeting):

-        Latitude 55 degrees;

-        Longitude 121.56076999 degrees;

-        Altitude 0 m.

## Option 2a

As for previous example from Option 1 (where UE latitude at 25.08439333 degrees), the neighbor satellite position was therefore selected such as the longitude of the neighbor satellites is +/-30 degrees longitude with respect to the UE reference longitude i.e. 121.56076999 degrees.

For instance, the position of the neighbor satellite with longitude at 91.56076999 degrees is described below in Figure 4:

Figure 4. Neighbor GSO at longitude 91.56076999 degrees

Moreover, the position of the neighbor satellite with longitude at 151.56076999 degrees is described below in Figure 5:

Figure 5. Neighbor GSO at longitude 151.56076999 degrees

Note: the latitude scale on both Figures 4 and 5 is: 90, 60, 30, 0, -30, -60, -90.

The generated files for the 2 neighbor satellites for Option 2a (with longitudes at 91.56076999 and 151.56076999 respectively) are given below:

Given said that, the delta serving-neighbor Doppler value (experienced by the UE) for Option 2a may reach up to ± 0.34 ppm with a fixed (or static) UE location condition. For a moving UE case with GSO, Doppler conditions will be higher as previously mentioned.

## Option 2b

For the Option 2b, the neighbor satellite position was also selected such as the longitude of the neighbor satellites is +/-20 degrees longitude with respect to the UE reference longitude i.e. 121.56076999 degrees. This configuration was added after discussion with other companies in order to meet 30° min elevation angle condition for the UE testing requirement – which was decided in a previous meeting (for Option 2a the min elevation angle was lower than 30°).

The generated files for the 2 neighbor satellites for Option 2b (with longitudes at 101.56076999 and 141.56076999 respectively) are given below:

Given said that, the delta serving-neighbor Doppler value (experienced by the UE) for Option 2b may reach up to ± 0.34 ppm with a fixed (or static) UE location condition. For a moving UE case with GSO, Doppler conditions will be higher as previously mentioned.

# Overall Conclusion

Observation 1: Two options have been considered for the UE reference position:

- [Option 1]

  - Latitude 25.08439333 degrees;

  - Longitude 121.56076999 degrees;

  - Altitude 0 m.

- [Option 2a & 2b]

  - Latitude 55 degrees;

  - Longitude 121.56076999 degrees;

  - Altitude 0 m.

Proposal 1: [Option 1] For GSO NTN UE testing, RAN5 can use the following example file for the serving GSO satellite:

Proposal 2: [Option 1] For GSO NTN UE testing, RAN5 can use the following example files for the neighbor GSO satellites (-30 and 30 degrees longitude from the central point of longitude of serving GSO satellites):

Proposal 3: [Option 2a & 2b] For GSO NTN UE testing, RAN5 can use the following example file for the serving GSO satellite:

Proposal 4: [Option 2a] For GSO NTN UE testing, RAN5 can use the following example files for the neighbor GSO satellites (-30 and 30 degrees longitude from the central point of longitude of serving GSO satellites):

Proposal 5: [Option 2b] For GSO NTN UE testing, RAN5 can use the following example files for the neighbor GSO satellites (-20 and 20 degrees longitude from the central point of longitude of serving GSO satellites):

Proposal 6: Intermediate GSO ephemeris values can be also selected, depending on different company willingness to test with different (non-zero) Doppler/ (non-fixed) Delay values.

Proposal 7: The Ephemeris/Elevation/Delay/Doppler provided data are applicable/are valid for both 5G NR NTN (SIB19) and NB-IoT NTN (SIB31/(SIB32)/SIB33).

Proposal 8: The provided set of Ephemeris/Elevation/Delay/Doppler can be used to test different frequencies since Doppler values are given in [ppm] (in an agnostic manner with respect to radio frequency value) and therefore the methodology is applicable also for future Releases.

Proposal 9: Because of Moon/Sun attraction for instance, even in the case of GSO with zero inclination (i.e. GEO) some orbit eccentricity may still exists and the Doppler cannot be considered zero. Therefore, using a zero Doppler condition is unrealistic and may be interpreted as a fake test.

Proposal 10: If the test equipment vendors and UE/chipset providers still insist to test under zero Doppler conditions, they would have to consider very specific set of satellite ephemeris data (correlated to a UE reference position) for which Doppler zero condition is met. However, these zero Doppler conditions represent very particular testing cases and therefore non-representative for satellite communications in general and for NTN in particular.

Proposal 11: Zero Doppler condition should be considered and treated as a non-representative case for NTN.