3GPP TSG-RAN5 Meeting #106	R5-251548

# Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3261 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of A-MPR for UL-MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_eMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test requirement for Test IDs 1 and 2 in Table 6.2D.3.5-11a is incorrect. Several values in Table 6.2D.3.5-17 are wrong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test requirements have been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will remain incorrect and inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of one revision of R5-251139. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

6.2D.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2D.3.5-1 to table 6.2D.3.5-19. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2D.1.3-1 apply.

Table 6.2D.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

< Unchanged tables omitted >

Table 6.2D.3.5-11a: UE Power Class 3 test requirements (NS_03/NS_03U) for band n2, n25 supporting ULFPTx

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 1.5 | 20 | 5 | 3 | 25+TT | 15-TT |
|  | NS_03U | 23 | 0.5 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 5 | 3 | 25+TT | 16.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 4, 5 | NS_03; NS_03U | 23 | 1 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 6 | NS_03; NS_03U | 23 | 1 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 7, 8 | NS_03; NS_03U | 23 | 2 | 3 | 1.5 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 9 | NS_03; NS_03U | 23 | 2 | 3 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 10, 11 | NS_03; NS_03U | 23 | 2.5 | 3.5 | 1.5 | 18 | 5 | 3 | 25+TT | 13-TT |
| 12 | NS_03; NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 13, 14 | NS_03; NS_03U | 23 | 4.5 | 5.5 | 1.5 | 16 | 5 | 3 | 25+TT | 11-TT |
| 15 | NS_03; NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 16, 17 | NS_03; NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 18 | NS_03; NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 19, 20 | NS_03; NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 21 | NS_03; NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 22, 23 | NS_03; NS_03U | 23 | 3.5 | 4.5 | 1.5 | 17 | 5 | 3 | 25+TT | 12-TT |
| 24 | NS_03; NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 25, 26 | NS_03; NS_03U | 23 | 6.5 | 7.5 | 1.5 | 14 | 6 | 3 | 25+TT | 8-TT |
| 27 | NS_03; NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

< Unchanged tables omitted >

Table 6.2D.3.5-17: UE Power Class 3 test requirements (NS_50) for band n39

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 6.5 | 0 | 16.5 | 5 | 3 | 25.0 + TT | 11.5 - TT |
| 2 | 23 | 0 | 3 | 0 | 0 | 20 | 5 | 3 | 25.0 + TT | 15.0 - TT |
| 3 | 23 | 0 | 3 | 0 | 0 | 20 | 5 | 3 | 25.0 + TT | 15.0 - TT |
| 4 | 23 | 0 | 3 | 6.5 | 0 | 16.5 | 5 | 3 | 25.0 + TT | 11.5 - TT |
| 5 | 23 | 0 | 3 | 0 | 0 | 20 | 5 | 3 | 25.0 + TT | 15.0 - TT |
| 6 | 23 | 0 | 3 | 0 | 0 | 20 | 5 | 3 | 25.0 + TT | 15.0 - TT |
| 7 | 23 | 0 | 3 | 12 | 0 | 11 | 6 | 3 | 25.0 + TT | 5.0 - TT |
| 8 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25.0 + TT | 13.5 - TT |
| 9 | 23 | 0 | 3 | 8 | 0 | 15 | 6 | 3 | 25.0 + TT | 9.0 - TT |
| 10 | 23 | 0 | 3 | 3.5 | 0 | 19.5 | 5 | 3 | 25.0 + TT | 14.5 - TT |
| 11 | 23 | 0 | 3 | 5 | 0 | 18 | 5 | 3 | 25.0 + TT | 13.0 - TT |
| 12 | 23 | 0 | 3 | 12 | 0 | 11 | 6 | 3 | 25.0 + TT | 5.0 - TT |
| 13 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25.0 + TT | 13.5 - TT |
| 14 | 23 | 0 | 3 | 8 | 0 | 15 | 6 | 3 | 25.0 + TT | 9.0 - TT |
| 15 | 23 | 0 | 3 | 3.5 | 0 | 19.5 | 5 | 3 | 25.0 + TT | 14.5 - TT |
| 16 | 23 | 0 | 3 | 5 | 0 | 18 | 5 | 3 | 25.0 + TT | 13.0 - TT |
| 17 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25.0 + TT | 11.5 - TT |
| 18 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25.0 + TT | 11.5 - TT |
| 19 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25.0 + TT | 11.5 - TT |
| 20 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25.0 + TT | 11.5 - TT |
| 21 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25.0 + TT | 11.5 - TT |
| 22 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25.0 + TT | 11.5 - TT |
| 23 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6 | 3 | 25.0 + TT | 5.0 - TT |
| 24 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25.0 + TT | 11.5 - TT |
| 25 | 23 | 0 | 6.5 | 8 | 0 | 15 | 6 | 3 | 25.0 + TT | 9.0 - TT |
| 26 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25.0 + TT | 11.5 - TT |
| 27 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25.0 + TT | 11.5 - TT |
| 28 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6 | 3 | 25.0 + TT | 5.0 - TT |
| 29 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25.0 + TT | 11.5 - TT |
| 30 | 23 | 0 | 6.5 | 8 | 0 | 15 | 6 | 3 | 25.0 + TT | 9.0 - TT |
| 31 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25.0 + TT | 11.5 - TT |
| 32 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25.0 + TT | 11.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

< End of changes >