3GPP TSG-RAN5 Meeting #106	R5-250158r1

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0073 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Annex F for NB-IoT NTN test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The MU and TT analysis need to be added into Annex F for TC 6.5A.3.3 and 6.5B.3.3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Adding MU and TT analysis for TC 6.5A.3.3 and 6.5B.3.3 2. Removed the minimum requirements of A-SEM from additional spurious emission TC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-SEM TC cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F1.2, F3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 1st revision update MU value |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change1 >

F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2A.1 UE maximum output power for category M1 | Same as clause 6.2.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 6.2A.2 UE maximum output power reduction for category M1 | Same as clause 6.2.3EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 6.2A.3 UE additional maximum output power reduction for category M1 UE | Same as clause 6.2.4EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 6.2A.4 Configured transmitted Power for category M1 | Same as clause 6.2.5EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 6.2B.1 UE maximum output power for category NB1 and NB2 | Same as clause 6.2.2F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 6.2B.2 UE maximum output power reduction for category NB1 and NB2 | Same as clause 6.2.3F in TS 36.521-1[14] for FDD band with “f ≤ 3.0GHz”. |  |
| 6.2B.3 UE additional maximum output power reduction for category NB1 and NB2 UE | Same as clause 6.2.3F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 6.2B.4 Configured transmitted Power for category NB1 and NB2 | Same as clause 6.2.5F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 6.3A.1 UE Minimum output power for category M1 | Same as clause 6.3.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 6.3A.2 Transmit OFF power for category M1 | Same as clause 6.3.3EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 6.3A.3.1 General ON/OFF time mask for category M1 | Same as clause 6.3.4EA.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 6.3A.3.2.1 PRACH time mask for UE category M1 | Same as clause 6.3.4EA.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 6.3A.3.2.2 SRS time mask for UE category M1 | Same as clause 6.3.4EA.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 6.3A.4.1 Power Control Absolute power tolerance for UE category M1 | Same as clause 6.3.5EA.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 6.3A.4.2 Power Control Relative power tolerance for UE category M1 | Same as clause 6.3.5EA.2 in TS 36.521-1 [14] |  |
| 6.3A.4.3 Aggregate power control tolerance for UE category M1 | Same as clause 6.3.5EA.3 in TS 36.521-1 [14] |  |
| 6.3B.1 UE Minimum output power for category NB1 and NB2 | Same as clause 6.3.2F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz” |  |
| 6.3B.2 Transmit OFF power for category NB1 and NB2 | Same as clause 6.3.3F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz” |  |
| 6.3B.3.1 General ON/OFF time mask for category NB1 and NB2 | Same as clause 6.3.4F.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz” |  |
| 6.3B.3.2 NPRACH time mask for category NB1 and NB2 | Same as clause 6.3.4F.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz” |  |
| 6.3B.4.1 Power Control Absolute power tolerance for category NB1 and NB2 | Same as clause 6.3.5F.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz” |  |
| 6.3B.4.2 Power Control Relative power tolerance for category NB1 and NB2 | Same as clause 6.3.5F.2 in TS 36.521-1 [14] |  |
| 6.3B.4.3 Aggregate power control tolerance for category NB1 and NB2 | Same as clause 6.3.5F.3 in TS 36.521-1 [14] |  |
| 6.4A.1_1 Frequency error with GSO ephemeris for UE category M1 | Same as clause 6.5.1EA in TS 36.521-1 [14] |  |
| 6.4A.1_2 Frequency error with NGSO ephemeris for UE category M1 | Same as clause 6.5.1EA in TS 36.521-1 [14] |  |
| 6.4A.2.1 Error Vector Magnitude (EVM) for category M1 | Same as clause 6.5.2.1EA.1 in TS 36.521-1 [14] |  |
| 6.4A.2.2 Carrier leakage for category M1 | Same as clause 6.5.2.2EA in TS 36.521-1 [14] |  |
| 6.4A.2.3 In-band emissions for non allocated RB for category M1 | Same as clause 6.5.2.3EA in TS 36.521-1 [14] |  |
| 6.4A.2.4 EVM equalizer spectrum flatness for category M1 | Same as clause 6.5.2.4EA in TS 36.521-1 [14] |  |
| 6.4B.1_1 Frequency error with GSO ephemeris for UE category NB1 and NB2 | Same as clause 6.5.1F in TS 36.521-1 [14] |  |
| 6.4B.1_2 Frequency error with NGSO ephemeris for UE category NB1 and NB2 | Same as clause 6.5.1F in TS 36.521-1 [14] |  |
| 6.4B.2.1 Error Vector Magnitude (EVM) for Category NB1 and NB2 | Same as clause 6.5.2.1F.1 in TS 36.521-1 [14] |  |
| 6.4B.2.2 Carrier leakage for Category NB1 and NB2 | Same as clause 6.5.2.2F in TS 36.521-1 [14] |  |
| 6.4B.2.3 In-band emissions for Category NB1 and NB2 | Same as clause 6.5.2.3F in TS 36.521-1 [14] |  |
| 6.5A.2 Occupied bandwidth for category M1 | Same as clause 6.6.1EA in TS 36.521-1 [14] for FDD band with “channel bandwidth = 1.4MHz”. |  |
| 6.5A.3.2 Spectrum emission mask for category M1 | Same as clause 6.6.2.1EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz” |  |
| 6.5A.3.3 Additional Spectrum Emission Mask for category M1 | ± 2.0 dB, f ≤ 3.0GHz |  |
| 6.5A.3.4 Adjacent Channel Leakage Ratio for category M1 | Same as clause 6.6.2.3EA in TS 36.521-1 [14] |  |
| 6.5A.4.2 Transmitter Spurious emissions for category M1 | Same as clause 6.6.3EA.1 in TS 36.521-1 [14] for FDD band for spurious frequencies up to 12.75 GHz |  |
| 6.5A.4.3 Spurious emission band UE co-existence for category M1 | Same as clause 6.6.3EA.2 in TS 36.521-1 [14] for FDD band with “results > -60 dBm, f ≤ 3.0GHz” |  |
| 6.5A.4.4 Additional spurious emissions for category M1 | ± 2.0 dB, f ≤ 3.0GHz |  |
| 6.5B.2 Occupied bandwidth for category NB1 and NB2 | Same as clause 6.6.1F in TS 36.521-1 [14] |  |
| 6.5B.3.2 Spectrum emission mask for category NB1 and NB2 | Same as clause 6.6.2.1F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz” |  |
| 6.5B.3.3 Additional Spectrum Emission Mask for category NB1 and NB2 | ± 2.0 dB, f ≤ 3.0GHz |  |
| 6.5B.3.4 Adjacent Channel Leakage Ratio for category NB1 and NB2 | Same as clause 6.6.2.3F in TS 36.521-1 [14] |  |
| 6.5B.4.2 Transmitter Spurious emissions for category NB1 and NB2 | Same as clause 6.6.3F.1 in TS 36.521-1[14] for f < 5th harmonic of the upper frequency edge of the UL operating band in GHz |  |
| 6.5B.4.3 Spurious emission band UE co-existence for category NB1 and NB2 | Same as clause 6.6.3F.2 in TS 36.521-1 [14] for FDD band with “results > -60 dBm, f ≤ 3.0GHz” |  |
| 6.5B.4.4 Additional spurious emissions for category NB1 and NB2 | ± 2.0 dB, f ≤ 3.0GHz |  |
| 6.6B Transmit intermodulation for category NB1 and NB2 | Same as clause 6.7F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz” |  |

<End of Change1 >

<Skipped unchanged >

<Start of Change2 >

F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Test | Minimum Requirement in TS 36.102 | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |
| --- | --- | --- | --- |
| 6.2A.1 UE maximum output power for category M1 | Same as clause 6.2.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz, Power class 3” and “f ≤ 3.0GHz, Power class 5”. | Same as clause 6.2.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz, Power class 3” and “f ≤ 3.0GHz, Power class 5”. | Same as clause 6.2.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz, Power class 3” and “f ≤ 3.0GHz, Power class 5”. |
| 6.2A.2 UE maximum output power reduction for category M1 | Same as clause 6.2.3EA in TS 36.521-1 [14] for FDD band with “Power class 3, f ≤ 3.0GHz” and “Power class 5, f ≤ 3.0GHz”. | Same as clause 6.2.3EA in TS 36.521-1 [14] for FDD band with “Power class 3, f ≤ 3.0GHz” and “Power class 5, f ≤ 3.0GHz”. | Same as clause 6.2.3EA in TS 36.521-1 [14] for FDD band with “Power class 3, f ≤ 3.0GHz” and “Power class 5, f ≤ 3.0GHz”. |
| 6.2A.3 UE additional maximum output power reduction for category M1 UE | Same as clause 6.2.4EA in TS 36.521-1 [14] | Same as clause 6.2.4EA in TS 36.521-1 [14]] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.2.4EA in TS 36.521-1 [14] |
| 6.2A.4 Configured transmitted Power for category M1 | Same as clause 6.2.5EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.2.5EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.2.5EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.2B.1 UE maximum output power for category NB1 and NB2 | Same as clause 6.2.2F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz, Power class 3” and “f ≤ 3.0GHz, Power class 5” | Same as clause 6.2.2F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.2.2F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.2B.2 UE maximum output power reduction for category NB1 and NB2 | Same as clause 6.2.3F in TS 36.521-1 [14] for FDD band with “Power class 3, f ≤ 3.0GHz” and “Power class 5, f ≤ 3.0GHz”. | Same as clause 6.2.3F in TS 36.521-1 [14] for FDD band with “Power class 3, f ≤ 3.0GHz” and “Power class 5, f ≤ 3.0GHz”. | Same as clause 6.2.3F in TS 36.521-1 [14] for FDD band with “Power class 3, f ≤ 3.0GHz” and “Power class 5, f ≤ 3.0GHz”. |
| 6.2B.3 UE additional maximum output power reduction for category NB1 and NB2 UE | Same as clause 6.2.3F in TS 36.521-1[14] for FDD band with “f ≤ 3.0GHz” | Same as clause 6.2.3F in TS 36.521-1[14] for FDD band with “f ≤ 3.0GHz” | Same as clause 6.2.3F in TS 36.521-1[14] for FDD band with “f ≤ 3.0GHz” |
| 6.2B.4 Configured transmitted Power for category NB1 and NB2 | Same as clause 6.2.5F in TS 36.521-1 [14] | Same as clause 6.2.5F in TS 36.521-1 [14] | Same as clause 6.2.5F in TS 36.521-1 [14] |
| 6.3A.1 UE Minimum output power for category M1 | Same as clause 6.3.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3A.2 Transmit OFF power for category M1 | Same as clause 6.3.3EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.3EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.3EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3A.3.1 General ON/OFF time mask for category M1 | Same as clause 6.3.4EA.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4EA.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4EA.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3A.3.2.1 PRACH time mask for UE category M1 | Same as clause 6.3.4EA.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4EA.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4EA.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3A.3.2.2 SRS time mask for UE category M1 | Same as clause 6.3.4EA.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4EA.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4EA.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3A.4.1 Power Control Absolute power tolerance for UE category M1 | Same as clause 6.3.5EA.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.5EA.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.5EA.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3A.4.2 Power Control Relative power tolerance for UE category M1 | Same as clause 6.3.5EA.2 in TS 36.521-1 [14]. | Same as clause 6.3.5EA.2 in TS 36.521-1 [14]. | Same as clause 6.3.5EA.2 in TS 36.521-1 [14]. |
| 6.3A.4.3 Aggregate power control tolerance for UE category M1 | Same as clause 6.3.5EA.3 in TS 36.521-1 [14]. | Same as clause 6.3.5EA.3 in TS 36.521-1 [14]. | Same as clause 6.3.5EA.3 in TS 36.521-1 [14]. |
| 6.3B.1 UE Minimum output power for category NB1 and NB2 | Same as clause 6.3.2F in TS 36.521-1 [14] | Same as clause 6.3.2F in TS 36.521-1 [14] | Same as clause 6.3.2F in TS 36.521-1 [14] |
| 6.3B.2 Transmit OFF power for category NB1 and NB2 | Same as clause 6.3.3F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.3F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.3F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3B.3.1 General ON/OFF time mask for category NB1 and NB2 | Same as clause 6.3.4F.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4F.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4F.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3B.3.2 NPRACH time mask for category NB1 and NB2 | Same as clause 6.3.4F.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4F.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.4F.2 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3B.4.1 Power Control Absolute power tolerance for category NB1 and NB2 | Same as clause 6.3.5F.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.5F.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.3.5F.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 6.3B.4.2 Power Control Relative power tolerance for category NB1 and NB2 | Same as clause 6.3.5F.2 in TS 36.521-1 [14] | Same as clause 6.3.5F.2 in TS 36.521-1 [14] | Same as clause 6.3.5F.2 in TS 36.521-1 [14] |
| 6.3B.4.3 Aggregate power control tolerance for category NB1 and NB2 | Same as clause 6.3.5F.3 in TS 36.521-1 [14] | Same as clause 6.3.5F.3 in TS 36.521-1 [14] | Same as clause 6.3.5F.3 in TS 36.521-1 [14] |
| 6.4A.1_1 Frequency error with GSO ephemeris for UE category M1 | Same as clause 6.5.1EA in TS 36.521-1 [14] | Same as clause 6.5.1EA in TS 36.521-1 [14] | Same as clause 6.5.1EA in TS 36.521-1 [14] |
| 6.4A.1_2 Frequency error with NGSO ephemeris for UE category M1 | Same as clause 6.5.1EA in TS 36.521-1 [14] | Same as clause 6.5.1EA in TS 36.521-1 [14] | Same as clause 6.5.1EA in TS 36.521-1 [14] |
| 6.4A.2.1 Error Vector Magnitude (EVM) for category M1 | Same as clause 6.5.2.1EA.1 in TS 36.521-1 [14] | Same as clause 6.5.2.1EA.1 in TS 36.521-1 [14] | Same as clause 6.5.2.1EA.1 in TS 36.521-1 [14] |
| 6.4A.2.2 Carrier leakage for category M1 | Same as clause 6.5.2.2EA in TS 36.521-1 [14] | Same as clause 6.5.2.2EA in TS 36.521-1 [14] | Same as clause 6.5.2.2EA in TS 36.521-1 [14] |
| 6.4A.2.3 In-band emissions for non allocated RB for category M1 | Same as clause 6.5.2.3EA in TS 36.521-1 [14] | Same as clause 6.5.2.3EA in TS 36.521-1 [14] | Same as clause 6.5.2.3EA in TS 36.521-1 [14] |
| 6.4A.2.4 EVM equalizer spectrum flatness for category M1 | Same as clause 6.5.2.4EA in TS 36.521-1 [14] | Same as clause 6.5.2.4EA in TS 36.521-1 [14] | Same as clause 6.5.2.4EA in TS 36.521-1 [14] |
| 6.4B.1_1 Frequency error with GSO ephemeris for UE category NB1 and NB2 | Same as clause 6.5.1F in TS 36.521-1 [14] | Same as clause 6.5.1F in TS 36.521-1 [14] | Same as clause 6.5.1F in TS 36.521-1 [14] |
| 6.4B.1_2 Frequency error with NGSO ephemeris for UE category NB1 and NB2 | Same as clause 6.5.1F in TS 36.521-1 [14] | Same as clause 6.5.1F in TS 36.521-1 [14] | Same as clause 6.5.1F in TS 36.521-1 [14] |
| 6.4B.2.1 Error Vector Magnitude (EVM) for Category NB1 and NB2 | Same as clause 6.5.2.1F.1 in TS 36.521-1 [14] | Same as clause 6.5.2.1F.1 in TS 36.521-1 [14] | Same as clause 6.5.2.1F.1 in TS 36.521-1 [14] |
| 6.4B.2.2 Carrier leakage for Category NB1 and NB2 | Same as clause 6.5.2.2F in TS 36.521-1 [14] | Same as clause 6.5.2.2F in TS 36.521-1 [14] | Same as clause 6.5.2.2F in TS 36.521-1 [14] |
| 6.4B.2.3 In-band emissions for Category NB1 and NB2 | Same as clause 6.5.2.3F in TS 36.521-1 [14] | Same as clause 6.5.2.3F in TS 36.521-1 [14] | Same as clause 6.5.2.3F in TS 36.521-1 [14] |
| 6.5A.2 Occupied bandwidth for category M1 | Same as clause 6.6.1EA in TS 36.521-1 [14] for FDD band with “channel bandwidth = 1.4MHz”. | Same as clause 6.6.1EA in TS 36.521-1 [14] | Same as clause 6.6.1EA in TS 36.521-1 [14] |
| 6.5A.3.2 Spectrum emission mask for category M1 | Same as clause 6.6.2.1EA in TS 36.521-1 [14] for FDD band with “channel bandwidth = 1.4MHz”. | Same as clause 6.6.2.1EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.6.2.1EA in TS 36.521-1 [14] |
| 6.5A.3.3 Additional Spectrum Emission Mask for category M1 | NS_02N and NS_03N -2 dBm / 4kHz -5 dBm / 4kHz -12 dBm / 4kHz -13 dBm / 4kHz -15 dBm / 4kHz NS_04N -2 dBm / 30kHz -26 dBm / 30kHz NS_05N -5 dBm / 30kHz -8.5 dBm / 30kHz -15 dBm / 30kHz -23 dBm / 30kHz -26 dBm / 30kHz | Same as clause 6.6.2.2EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.6.2.2EA in TS 36.521-1 [14] |
| 6.5A.3.4 Adjacent Channel Leakage Ratio for category M1 | Same as clause 6.6.2.3EA in TS 36.521-1 [14] | Same as clause 6.6.2.3EA in TS 36.521-1 [14] | Same as clause 6.6.2.3EA in TS 36.521-1 [14] |
| 6.5A.4.2 Transmitter Spurious emissions for category M1 | Same as clause 6.6.3EA.1 in TS 36.521-1 [14] for FDD band for spurious frequencies up to 12.75 GHz. | Same as clause 6.6.3EA.1 in TS 36.521-1 [14] | Same as clause 6.6.3EA.1 in TS 36.521-1 [14] |
| 6.5A.4.3 Spurious emission band UE co-existence for category M1 | Same as clause 6.6.3EA.2 in TS 36.521-1 [14] | Same as clause 6.6.3EA.2 in TS 36.521-1 [14] | Same as clause 6.6.3EA.2 in TS 36.521-1 [14] |
| 6.5A.4.4 Additional spurious emissions for category M1 | -26 dBm / 700kHz -50 dBm / 700kHz -16 dBm / 1MHz -40 dBm / 1MHz Frequencies as detailed in core requirement | 0 dB | Formula: Minimum Requirement + TT |
| 6.5B.2 Occupied bandwidth for category NB1 and NB2 | Same as clause 6.6.1F in TS 36.521-1 [14] | Same as clause 6.6.1F in TS 36.521-1 [14] | Same as clause 6.6.1F in TS 36.521-1 [14] |
| 6.5B.3.2 Spectrum emission mask for category NB1 and NB2 | Same as clause 6.6.2.1F in TS 36.521-1 [14] | Same as clause 6.6.2.1F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 6.6.2.1F in TS 36.521-1 [14] |
| 6.5B.3.3 Additional Spectrum Emission Mask for category NB1 and NB2 | NS_02N and NS_03N -2 dBm / 4kHz -5 dBm / 4kHz -12 dBm / 4kHz -13 dBm / 4kHz -15 dBm / 4kHz NS_04N -2 dBm / 30kHz -26 dBm / 30kHz NS_05N -5 dBm / 30kHz -8.5 dBm / 30kHz -15 dBm / 30kHz -23 dBm / 30kHz -26 dBm / 30kHz | Same as caluse 6.6.2.2 in TS 36.521-1 [14] | Same as caluse 6.6.2.2 in TS 36.521-1 [14] |
| 6.5B.3.4 Adjacent Channel Leakage Ratio for category NB1 and NB2 | Same as clause 6.6.2.3F in TS 36.521-1 [14] | Same as clause 6.6.2.3F in TS 36.521-1 [14] | Same as clause 6.6.2.3F in TS 36.521-1 [14] |
| 6.5B.4.2 Transmitter Spurious emissions for category NB1 and NB2 | Same as clause 6.6.3F.1 in TS 36.521-1[14] for f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | Same as clause 6.6.3F in TS 36.521-1[14] for f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | Same as clause 6.6.3F in TS 36.521-1[14] for f < 5th harmonic of the upper frequency edge of the UL operating band in GHz |
| 6.5B.4.3 Spurious emission band UE co-existence for category NB1 and NB2 | Same as 6.6.3F.2 in TS 36.521-1 [14] | Same as 6.6.3F.2 in TS 36.521-1 [14] | Same as 6.6.3F.2 in TS 36.521-1 [14] |
| 6.5B.4.4 Additional spurious emissions for category NB1 and NB2 | -26 dBm / 700kHz -50 dBm / 700kHz -16 dBm / 1MHz -40 dBm / 1MHz Frequencies as detailed in core requirement | 0 dB | Formula: Minimum Requirement + TT |
| 6.6B Transmit intermodulation for category NB1 and NB2 | Same as clause 6.7F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz” | Same as clause 6.7F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz” | Same as clause 6.7F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz” |

<End of Change2 >