3GPP TSG-RAN5 Meeting #107	R5-252456r2

# St Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1602 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new LTE IMS data channel test case 12.30 - MO Voice call without precondition and BDC failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This new IMS data channel (IMS DC) test case is added to test: UE send SDP offer with IMS DC and NW reply SDP answer using port 0 in m line for IMS DC. The UE can establish voice call without precondition even if IMS DC establishment failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new LTE IMS data channel test case 12.30 IMS Data Channel / DCMTSI MO Voice call / Bootstrap Data Channel failure / without preconditions at both originating UE and terminating UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG_RTC_IMS_dataCH-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.30 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0369 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: 1.Correct the reference in the Media description for audio in re-INVITE message. 2nd revision: Editorial update(adding several spaces) |  |  |  |  |  |  |  |  |

<Start of modified section 1>

12.30	IMS Data Channel / DCMTSI MO Voice call / Bootstrap Data Channel failure / without preconditions at both originating UE and terminating UE

12.30.1	Definition

Test to verify that the UE correctly performs IMS mobile originated voice call setup without preconditions when IMS Bootstrap data channel setups failure because Network replies the SDP answer without media-level attributes “a = dcmap”. This process is described in 3GPP TS 24.186 [159] and TS 26.114 [66].

12.30.2	Conformance requirement

[TS 24.186 clause 9.2.3.1]

Upon generating an initial INVITE request or a re-INVITE request, the UE supporting IMS data channel and is allowed to use IMS data channel and if the UE determined its home network supports the IMS data channel capability, the UE shall include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type in the Contact header field to the remote UE and use +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4], regardless of IMS data channel media description being part of the SDP or not. The UE may include in the initial INVITE request an Accept-Contact header field containing the "sip.app-subtype" media feature tag defined in IETF RFC 5688 [5] with a value of "webrtc-datachannel" as specified in 3GPP TS 24.173 [10].

[TS 24.186 clause 9.3.2.1.2]

If the UE initiates an MMTel session with IMS data channels, the UE:

1)	shall generate an initial INVITE request in accordance with 3GPP TS 24.229 [9] and 3GPP TS 24.173 [10];

2)	shall include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type with +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4] in the Contact header field;

3)	may include an Accept-Contact header field containing the "sip.app-subtype" media feature tag defined in IETF RFC 5688 [5] with a value of "webrtc-datachannel" as specified in 3GPP TS 26.114 [4]; and

4)	if the configuration described in clause 9.3.2.1.1, allows the establishment of bootstrap data channels simultaneously with the setup of the MMTel session, shall include an SDP offer containing the media descriptions for the MMTel media according 3GPP TS 24.173 [10] and data channel media descriptions for both the local and remote bootstrap data channels in accordance with 3GPP TS 26.114 [4].

[TS 24.186 clause 9.3.2.1.3.1]

If the UE wants to establish a bootstrap data channel, the UE shall take into account the data channel configuration as specified in clause 9.3.2.1.1, and if the UE determines that the configuration allows the establishment of IMS data channels after the establishment of the MMTel session, the UE shall:

1)	generate a re-INVITE request in accordance with 3GPP TS 24.229 [9] and 3GPP TS 24.173 [10];

2)	include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type with +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4] in the Contact header field; and

3)	include an updated SDP offer that contains data channel media descriptions for both the local and remote bootstrap data channels according to 3GPP TS 26.114 [4].

[TS 26.114 clause 6.2.10.1]

A bootstrap data channel shall be configured as ordered, reliable, with normal SCTP multiplexing priority. The sub-protocol for a bootstrap data channel shall be HTTP (not encapsulating HTTP in TCP), represented by the following, example SDP "a=dcmap" line, which therefore shall be present in each data channel media description in an SDP offer from a DCMTSI client in terminal:

a=dcmap:0 subprotocol="http"

[TS 26.114 clause 6.2.10.3]

An answering DCMTSI client in terminal that desires to reject the entire SCTP association for all offered data channels shall set the port to 0 (zero) on the corresponding "m=application" line in SDP, as described in [172]. An SCTP association that initially, or as a result of session modification, has no open data channels ("a=dcmap" lines) should be rejected or closed by modifying the session, setting port number to 0 (zero).

[TS 26.114 clause 6.2.10.4]

An offering DCMTSI client in terminal receiving an SDP answer where the data channel SCTP association is accepted (port is not 0) may use any offered stream ID that has a corresponding "a=dcmap" line in the SDP answer, as described by section 6.5 in [172]. Data channels with "a=dcmap" lines in the SDP offer that are not included in the SDP answer must be considered as rejected and shall not be used, as described by section 6.5 in [172]. The "req-app-id" parameter of the "a=3gpp-req-app" attribute is used to identify the application for which the application data channels are added/updated.

Reference(s)

3GPP TS 24.186 [159], clauses 9.2.3.1, 9.3.2.1.2 and 9.3.2.1.3.1. 3GPP TS 26.114 [66], clauses 6.2.10.1, 6.2.10.3 and 6.2.10.4.

12.30.3	Test purpose

1)	To verify that UE establishes IMS data channel when initiating MO Voice call or after the establishment of MO Voice call; and

2)	To verify that UE includes media feature tag +sip.app-subtype="webrtc-datachannel" in (re-)INVITE message.

3)	To verify that UE includes correct SDP offer for IMS data channel media negotiation in (re-)INVITE message.

4) 	To verify that UE proceeds the voice call without preconditions when IMS Bootstrap data channel setups failure because Network replies the SDP answer without media-level attributes “a = dcmap”

12.30.4	Method of test

Initial conditions

UE contains either SIM application (GIBA), ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security). UE is configured to not use the precondition mechanism. UE is configured to use IMS data channel.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-9)	UE executes the procedures described in TS 36.508 [94] table 4.5A.6.3-1 steps 1 to 14. If INVITE in step2 does not include SDP offer containing media lines for IMS data channel, Step 10a1-10a4 are excuted to establish IMS data channel after voice call establishment. If INVITE in step2 include SDP offer containing media lines for both voice and IMS data channel, SS reject IMS data channel by replying the SDP answer without media-level attributes “a = dcmap”.

10a1)	UE send re-INVITE with an SDP offer containing media lines for both voice and IMS data channel.

10a2)	UE SS responds with a 100 Trying provisional response.

10a3)	UE SS responds re-INVITE with 200 OK, and SS reject IMS data channel by repling the SDP answer without media-level attributes “a = dcmap”.

10a4)	UE acknowledge the receipt of 200 OK for re-INVITE.

11-12)	Test procedure for MT release of IMS call defined in annex C.33 steps 1 to 4 is executed.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-9 |  |  | Steps 1-9 defined in annex C.21f | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.6.3-1 for a UE with E-UTRA support. |
| - | - |  | EXCEPTION: Step 10a1 to 10a4 executed if INVITE in step2 does not include SDP offer containing media lines for IMS data channel. |  |
| 10a1 |  |  | INVITE | UE send re-INVITE with an SDP offer containing media lines for both voice and IMS data channel. |
| 10a2 |  |  | 100 Trying | SS responds with a 100 Trying provisional response. |
| 10a3 |  |  | 200 OK | SS responds re-INVITE with 200 OK. |
| 10a4 |  |  | ACK | UE acknowledge the receipt of 200 OK for re-INVITE. |
| 11-12 |  |  | Steps 1-2 defined in annex C.33 | MT release of IMS call. Referred from C.33. |

Specific Message Contents

INVITE (Step 2)

Use the default message “INVITE” in annex C.21f with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Step 2 of C.21f except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body |  | SDP body is same as Step 2 of C.21f or same as Step 2 of C.21f except adding at least one Media description for IMS bootstrap data channel as follow: Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel c=IN (addrtype) (connection-address for UE) [Note 1] Attributes for media for IMS data channel: a=max-message-size:(max-message-size-value) [Note 2] a=sctp-port:(sctp-port-value) [Note 3] a=fingerprint:(hash-func) (fingerprint) [Note 4] a=setup:actpass a=tls-id:(tls-id-value) a=dcmap: (dcmap-stream-id) subprotocol="http” [Note 5,6] Note 1: At least one "c=" field shall be present. Note 2: Optional attribute. If the SDP ”max-message-size” attribute is not present the default value is 64K. Note 3: SCTP port range is between 0 and 65535(both included).Leading zeroes must not be used. Note 4: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 5: Multiple data channels may be mapped to a single data channel SDP media description, each with a corresponding "a=dcmap" SDP attribute and stream IDs that are unique within that media description. Note 6: dcmap-stream-id range is between 0 and 999(both included). |

183 Session Progress (Step 4)

Use the default message “183 Session Progress” in annex C.21f with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Step 4 of C.21f except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body | A1 A2 | SDP body is same as Step 4 of C.21f except adding one Media description as follow: Media description for IMS data channel: m=application 0 UDP/DTLS/SCTP webrtc-datachannel Attributes for media for IMS data channel: Not present SDP answer is same as Step 4 of C.21f |

| Condition | Explanation |
| --- | --- |
| A1 | INVITE includes SDP offer containing media lines both for voice and IMS data channel in Step 2. |
| A2 | INVITE does not include SDP offer containing media lines for IMS data channel in Step 2. |

200OK (Step 8)

Use the default message “200OK for INVITE” in annex C.21f with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Step 8 of C.21f except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |

Re-INVITE (Step 10a1)

Use the default message “INVITE” in annex A.21 with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Annex A.2.1 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body |  | SDP body is same as Step 2 of C.21f (INVITE) with following exception: Session description: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one Media description for audio: Same as in previous SDP sent by the SS in the 183 Session Progress in Step 4 of C.21f. Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel c=IN (addrtype) (connection-address for UE) [Note 1] Attributes for media for IMS data channel: a=max-message-size:(max-message-size-value) [Note 2] a=sctp-port:(sctp-port-value) [Note 3] a=fingerprint:(hash-func) (fingerprint) [Note 4] a=setup:actpass a=tls-id:(tls-id-value) a=dcmap: (dcmap-stream-id) subprotocol="http” [Note 5,6] Note 1: At least one "c=" field shall be present. Note 2: Optional attribute. If the SDP ”max-message-size” attribute is not present the default value is 64K. Note 3: SCTP port range is between 0 and 65535(both included).Leading zeroes must not be used. Note 4: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 5: Multiple data channels may be mapped to a single data channel SDP media description, each with a corresponding "a=dcmap" SDP attribute and stream IDs that are unique within that media description. Note 6: dcmap-stream-id range is between 0 and 999(both included). |

100 Trying (Step 10a2)

Use the default message “100 Trying” in annex A.2.2 with condition A1.

200OK (Step 10a3)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with condition A23.

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Annex A.3.1 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Content-Type media-type |  | application/sdp |
| Content-Length value |  | length of message-body |
| Message-body |  | SDP body is same as Step 4 of C.21f with follow exception: Session description: "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one Media description for IMS data channel: m=application 0 UDP/DTLS/SCTP webrtc-datachannel Attributes for media for IMS data channel: Not present |

ACK (Step 10a4)

Use the default message “ACK” in annex A.2.7 with condition A5.

<End of modified section 1>