3GPP TSG-RAN5 Meeting #107	R5-252458r1

# St Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1604 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new LTE IMS data channel test case 12.32 - MO Voice call without precondition and BDC success |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This new IMS data channel (IMS DC) test case is added to test: UE send SDP offer with IMS DC and NW reply SDP answer with IMS DC. The UE can establish voice call without precondition + IMS DC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new LTE IMS data channel test case 12.32 IMS Data Channel / DCMTSI MO Voice call / Bootstrap Data Channel success / without preconditions at both originating UE and terminating UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG_RTC_IMS_dataCH-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.32 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0369 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: 1.Correct the reference in the Media description for audio in re-INVITE message. 2.Update the Comment column in the expected sequence. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

12.32	IMS Data Channel / DCMTSI MO Voice call / Bootstrap Data Channel success / without preconditions at both originating UE and terminating UE

12.32.1	Definition

Test to verify that the UE correctly performs IMS mobile originated voice call setup without preconditions and IMS Bootstrap data channel setup. This process is described in 3GPP TS 23.228 [3] ,3GPP TS 24.186 [159] and TS 26.114 [66].

12.32.2	Conformance requirement

[TS 24.186 clause 9.2.3.1]

Upon generating an initial INVITE request or a re-INVITE request, the UE supporting IMS data channel and is allowed to use IMS data channel and if the UE determined its home network supports the IMS data channel capability, the UE shall include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type in the Contact header field to the remote UE and use +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4], regardless of IMS data channel media description being part of the SDP or not. The UE may include in the initial INVITE request an Accept-Contact header field containing the "sip.app-subtype" media feature tag defined in IETF RFC 5688 [5] with a value of "webrtc-datachannel" as specified in 3GPP TS 24.173 [10].

[TS 24.186 clause 9.3.2.1.2]

If the UE initiates an MMTel session with IMS data channels, the UE:

1)	shall generate an initial INVITE request in accordance with 3GPP TS 24.229 [9] and 3GPP TS 24.173 [10];

2)	shall include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type with +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4] in the Contact header field;

3)	may include an Accept-Contact header field containing the "sip.app-subtype" media feature tag defined in IETF RFC 5688 [5] with a value of "webrtc-datachannel" as specified in 3GPP TS 26.114 [4]; and

4)	if the configuration described in clause 9.3.2.1.1, allows the establishment of bootstrap data channels simultaneously with the setup of the MMTel session, shall include an SDP offer containing the media descriptions for the MMTel media according 3GPP TS 24.173 [10] and data channel media descriptions for both the local and remote bootstrap data channels in accordance with 3GPP TS 26.114 [4].

[TS 24.186 clause 9.3.2.1.3.1]

If the UE wants to establish a bootstrap data channel, the UE shall take into account the data channel configuration as specified in clause 9.3.2.1.1, and if the UE determines that the configuration allows the establishment of IMS data channels after the establishment of the MMTel session, the UE shall:

1)	generate a re-INVITE request in accordance with 3GPP TS 24.229 [9] and 3GPP TS 24.173 [10];

2)	include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type with +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4] in the Contact header field; and

3)	include an updated SDP offer that contains data channel media descriptions for both the local and remote bootstrap data channels according to 3GPP TS 26.114 [4].

[TS 26.114 clause 6.2.10.1]

A bootstrap data channel shall be configured as ordered, reliable, with normal SCTP multiplexing priority. The sub-protocol for a bootstrap data channel shall be HTTP (not encapsulating HTTP in TCP), represented by the following, example SDP "a=dcmap" line, which therefore shall be present in each data channel media description in an SDP offer from a DCMTSI client in terminal:

a=dcmap:0 subprotocol="http"

[TS 26.114 clause 6.2.10.4]

An offering DCMTSI client in terminal receiving an SDP answer where the data channel SCTP association is accepted (port is not 0) may use any offered stream ID that has a corresponding "a=dcmap" line in the SDP answer, as described by section 6.5 in [172].

[TS 23.228 clause AC.7.1]

Figure AC.7.1-1depicts a signalling flow diagram for establishing a bootstrap data channel in a person-to-person use case. The MF anchors the bootstrap data channel, and the originating network is offering a bootstrap data channel to the remote peer as well for application download.

NOTE 1:	Some SIP signalling not relevant for the procedure, is not shown in the call flow below.

Figure AC.7.1-1: Bootstrap Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

1.	UE#1 sends the SIP INVITE request with an initial SDP to the IMS AS, through P-CSCF and S-CSCF in the originating network. The initial SDP contains offers for the bootstrap data channel establishment request with bootstrap DC stream ID. In this example procedure, the SDP contains both bootstrap data channel offers for originating side and terminating side.

NOTE 2:	This SIP INVITE can also be a SIP re-INVITE performed after the initial IMS audio session is setup. The SIP re-INVITE can be initiated either from UE#1 or UE#2.

2.	IMS AS validates user subscription data to determine whether the data channel call request should be notified to DCSF.

If the IMS AS determined, based on the user profile, that data channel call request needs to be notified to DCSF, the IMS AS selects a DCSF for this user based on local configuration or discovery and selection of a DCSF instance via NRF.

If the IMS AS determined, based on the user profile, that the data channel call request need not to be notified to DCSF, or DCSF decides that DC request is not allowed, the IMS AS proceeds with normal IMS procedures to setup the MMTel session without performing Data Channel bootstrapping, by deleting DC related media information and sending the updated SIP INVITE to the originating S-CSCF.

3.	IMS AS notifies the DCSF of the DC call event by sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, Calling ID, Called ID, Session Case, Event initiator, Media InfoList, DC Stream ID) request to the DCSF.

4.	After receiving the DC control request, the DCSF determines the policy about how to process the bootstrap data channel establishment request based on the related parameters in the Data Channel control request (e.g. CallingID, CalledID, DC Stream ID) and/or DCSF service specific policy.

5.	Since the SessionEstablishmentRequestEvent indicates that served user is offered local bootstrap media, DCSF, based on its policies reserves originating side MDC1 media information, as well as the terminating side MDC1 remote bootstrap media (targeting remote UE), which are used to receive UE request for application downloading from MF.

6.	DCSF invokes the Nimsas_MediaControl_MediaInstruction (Session ID, Media Instruction Set) operation based on its policies instructing the IMS AS how to set up bootstrap data channel with MF both for originating and terminating side. The MediaInstructionSet provided by the DSCF, includes its MDC1 media endpoint addresses created in step 5, DC Stream ID, and the replacement HTTP URL representing the application list offered via the MDC1 interface.

In this scenario, the DCSF instructs the IMS AS to terminate bootstrap data channel establishment request on originating MF, and initiate remote bootstrap data channel establishment request(targeting remote UE) as well as forwarding remote bootstrap data channel establishment request of served user (targeting remote DCSF) towards terminating network.

7.	The IMS AS selects a MF based on local configuration or discovers and selects a MF instance supporting DC media function via NRF.

8.	IMS AS invokes Nmf_MRM_Create(List of Media Termination Descriptors) service operation to instruct MF to allocate required data channel media resources. IMS AS request creation of two different Media Terminations, one representing the local bootstrap media to be terminated and the other representing the remote bootstrap media to be offered to remote UE. Each Media Termination includes information required to allocate resources in both Mb and the MDC1 interfaces. The MF responds with the negotiated data channel media resource information to IMS AS.

NOTE 3:	The MF media resource allocation in step 8 could be done with one or multiple service invocations.

9.	IMS AS responds to the MediaInstruction request received in step 6. The response may include the atomic success result of operation and also includes negotiated data channel media resource information for MDC1.

10.	The DCSF stores the media resource information and responds to the Notify Request received in step 3.

11-13.	IMS AS sends the INVITE which includes the updated SDP offer adding media information of MF via the originating S-CSCF to remote network side and UE#2. In this scenario, the SDP offer for bootstrap data channel to UE#2 is included.

14.	UE#2 and terminating network returns an 18X response with the SDP answer to bootstrap data channel to originating network. According to the received SDP answer, IMS AS may notify the DCSF using Nimsas_SessionEventControl_Notify, and then instruct MF to update data channel media resource information for UE#2.

15-16:	UE#2 and terminating network returns a 200 OK response.

17.	The IMS AS notifies the successful session establishment event, Nimsas_SessionEventControl Notify (SessionEstablishmentSuccessEvent, Session ID, Media Info List) to DCSF.

18.	The DCSF responds to the Nimsas notification request.

19.	200 OK forwarded to UE#1 which indicates the bootstrap data channel has been established.

20.	The originating network P-CSCF executes QoS procedure for bootstrap data channel as specified in TS 23.203 [54] and TS 23.503 [95].

21-24:	The bootstrap data channels have been established between originating MF and UE#1/UE#2. The UEs send application request messages to MF to request a data channel application or an application list if multiple DC applications are available, via the established bootstrap data channel with its data channel capabilities. The MF replaces the root URL with the replacement URL received in steps 8 and forwards the message to received media point of DCSF. The DCSF provides the application list and proper data channel applications further to UE#1 and UE#2 based on their data channel capabilities and their choices through MF.

The bootstrap data channels have also been established between terminating MF and UE#1/UE#2. The data channel application is requested and downloaded to UE#1 and UE#2 from terminating DCSF.

Steps 20-24 may be executed after step 14, if the SDP answer in 200 OK to the PRACK and UPDATE messages contain the information required to establish bootstrap data channels.

NOTE 4:	IMS-AGW needs to allow the establishment of bootstrap data channels based on the information in the 200 OK to PRACK and UPDATE messages.

25.	Subsequent procedures continue.

NOTE 5:	In the DC application list, the DCSF may provide DC applications supported by both UEs, or only supported by the UE who sends the application request message. The details of how to provide the application list to the UE and how to use it by the UE are not defined in this specification.

Reference(s)

3GPP TS 24.186 [159], clauses 9.2.3.1, 9.3.2.1.2 and 9.3.2.1.3.1. 3GPP TS 26.114 [66], clauses 6.2.10.1 and 6.2.10.4. 3GPP TS 23.228 [3], clauses AC.7.1.

12.32.3	Test purpose

1)	To verify that UE setups IMS Bootstrap data channel successfully when initiating MO Voice call or after the establishment of MO Voice call; and

2)	To verify that UE includes media feature tag +sip.app-subtype="webrtc-datachannel" in (re-)INVITE message.

3)	To verify that UE includes correct SDP offer for IMS data channel media negotiation in (re-)INVITE message.

12.32.4	Method of test

Initial conditions

UE contains either SIM application (GIBA), ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security). UE is configured to not use the precondition mechanism. UE is configured to use IMS data channel.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-9)	UE executes the procedures described in TS 36.508 [94] table 4.5A.6.3-1 steps 1 to 14. If INVITE in step2 does not include SDP offer containing media lines for IMS data channel, Step 10a1-10a4 are excuted to establish IMS data channel after voice call establishment. If INVITE in step2 include SDP offer containing media lines for both voice and IMS data channel, SS accept IMS data channel by replying the SDP answer without media-level attributes “a = dcmap”, and the qos flow for IMS bootstrap data channel (specified in TS 36.508[94] table 4.5A.32.3-1 ) is established after 200OK for INVITE..

10a1)	UE send re-INVITE with an SDP offer containing media lines for both voice and IMS data channel.

10a2)	SS responds with a 100 Trying provisional response.

10a3)	SS responds re-INVITE with 200 OK, and SS reject IMS data channel by repling the SDP answer without media-level attributes “a = dcmap” , then SS establishs qos flow for IMS bootstrap data channel (specified in TS 36.508[94] table 4.5A.32-1..

10a4)	UE acknowledge the receipt of 200 OK for re-INVITE.

11-12)	Test procedure for MT release of IMS call defined in annex C.33 steps 1 to 4 is executed.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-9 |  |  | Steps 1-9 defined in annex C.21f | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.6.3-1 for a UE with E-UTRA support. If INVITE in step2 include SDP offer containing media lines for both voice and IMS data channel, the qos flow for IMS bootstrap data channel is established after 200OK for INVITE message.Referred from 36.508 [94] table 4.5A.32.3-1. |
| - | - |  | EXCEPTION: Step 10a1 to 10a4 executed if INVITE in step2 does not include SDP offer containing media lines for IMS data channel. |  |
| 10a1 |  |  | INVITE | UE send re-INVITE with an SDP offer containing media lines for both voice and IMS data channel. |
| 10a2 |  |  | 100 Trying | SS responds with a 100 Trying provisional response. |
| 10a3 |  |  | 200 OK | SS responds re-INVITE with 200 OK. Qos flow for IMS bootstrap data channel is established after 200OK is sent. Referred from 36.508 [94] table 4.5A.32.3-1. |
| 10a4 |  |  | ACK | UE acknowledge the receipt of 200 OK for re-INVITE. |
| 11-12 |  |  | Steps 1-2 defined in annex C.33 | MT release of IMS call. Referred from C.33. |

Specific Message Contents

INVITE (Step 2)

Use the default message “INVITE” in annex C.21f with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Step 2 of C.21f except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body |  | SDP body is same as Step 2 of C.21f or same as Step 2 of C.21f except adding at least one Media description for IMS bootstrap data channel as follow: Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel c=IN (addrtype) (connection-address for UE) [Note 1] Attributes for media for IMS data channel: a=max-message-size:(max-message-size-value) [Note 2] a=sctp-port:(sctp-port-value) [Note 3] a=fingerprint:(hash-func) (fingerprint) [Note 4] a=setup:actpass a=tls-id:(tls-id-value) a=dcmap: (dcmap-stream-id) subprotocol="http” [Note 5,6] Note 1: At least one "c=" field shall be present. Note 2: Optional attribute. If the SDP ”max-message-size” attribute is not present the default value is 64K. Note 3: SCTP port range is between 0 and 65535(both included).Leading zeroes must not be used. Note 4: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 5: Multiple data channels may be mapped to a single data channel SDP media description, each with a corresponding "a=dcmap" SDP attribute and stream IDs that are unique within that media description. Note 6: dcmap-stream-id range is between 0 and 999(both included). |

183 Session Progress (Step 4)

Use the default message “183 Session Progress” in annex C.21f with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Step 4 of C.21f except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body | A1 A2 | SDP body is same as Step 4 of C.21f except adding one Media description as follow: Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel [Note 1] b=AS:500 Attributes for media for IMS data channel: a=max-message-size:64000 a=sctp-port:(sctp-port-value) [Note 2] a=setup:passive a=fingerprint: (hash-func) (fingerprint) [Note 3] a=tls-id: (tls-id-value) [Note 4] a=dcmap: (dcmap-stream-id) subprotocol="http" [Note 5] SDP body is same as Step 4 of C.21f Note 1: Transport port is the port number of the SS. Note 2: The sctp-port-value is the SCTP port of the SS. Note 3: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 4: The tls-id-value is random number generated by SS. Note 5: The dcmap-stream-id is the same as the dcmap-stream-id in the INVITE message. |

| Condition | Explanation |
| --- | --- |
| A1 | INVITE includes SDP offer containing media lines both for voice and IMS data channel in Step2. |
| A2 | INVITE does not include SDP offer containing media lines for IMS data channel in Step2. |

200OK (Step 8)

Use the default message “200OK for INVITE” in annex C.21f with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Step 8 of C.21f except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |

Re-INVITE (Step 10a1)

Use the default message “INVITE” in annexA.21 with condtion A5.

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Annex A.2.1 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body |  | SDP body is same as Step 2 of C.21f (INVITE) with following exception: Session description: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one Media description for audio: Same as in previous SDP sent by the SS in the 183 Session Progress in Step 4 of C.21f. Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel c=IN (addrtype) (connection-address for UE) [Note 1] Attributes for media for IMS data channel: a=max-message-size:(max-message-size-value) [Note 2] a=sctp-port:(sctp-port-value) [Note 3] a=fingerprint:(hash-func) (fingerprint) [Note 4] a=setup:actpass a=tls-id:(tls-id-value) a=dcmap: (dcmap-stream-id) subprotocol="http” [Note 5,6] Note 1: At least one "c=" field shall be present. Note 2: Optional attribute. If the SDP ”max-message-size” attribute is not present the default value is 64K. Note 3: SCTP port range is between 0 and 65535(both included).Leading zeroes must not be used. Note 4: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 5: Multiple data channels may be mapped to a single data channel SDP media description, each with a corresponding "a=dcmap" SDP attribute and stream IDs that are unique within that media description. Note 6: dcmap-stream-id range is between 0 and 999(both included). |

100 Trying (Step 10a2)

Use the default message “100 Trying” in annexA.2.2 with condition A1.

200OK (Step 10a3)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annexA.3.1 with condition A23.

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Annex A.3.1 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Content-Type media-type |  | application/sdp |
| Content-Length value |  | length of message-body |
| Message-body |  | SDP body is same as Step 4 of C.21f with follow exception: Session description: "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel [Note 1] b=AS:500 Attributes for media for IMS data channel: a=max-message-size:64000 a=sctp-port:(sctp-port-value) [Note 2] a=setup:passive a=fingerprint: (hash-func) (fingerprint) [Note 3] a=tls-id: (tls-id-value) [Note 4] a=dcmap: (dcmap-stream-id) subprotocol="http" [Note 5] Note 1: Transport port is the port number of the SS. Note 2: The sctp-port-value is the SCTP port of the SS. Note 3: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 4: The tls-id-value is random number generated by SS. Note 5: The dcmap-stream-id is the same as the dcmap-stream-id in the re-INVITE message. |

ACK (Step 10a4)

Use the default message “ACK” in annexA.2.7 with condition A5.

<End of modified section 1>