3GPP TSG-RAN5 Meeting #107	R5-252643r1

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1610 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of EPS eCall test case 21.27 for CEN alignments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New test case 21.27 needs to be added as part of eCallCEN-UEConTest WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 21.27. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCallCEN-UEConTest WP will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.27 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 34.229-2 CR 0371 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-252643r1: 1. Added SIM references in UE initial conditions. 2. Comments column against steps 2-5 and 11-23 updated in Expected sequence. |  |  |  |  |  |  |  |  |

<Start of modified section>

## 21.27	eCall capable / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 486 (Busy Here) with positive ACK / PSAP Callback / Success

### 21.27.1	Definition

Test to verify that on reception of reject cause 486 Busy here with positive acknowledgement for automatically initiated INVITE for eCall over IMS, UE considers MSD transmission as successful and wait for PSAP callback. The process consists of normal registration, emergency registration and MSD included in INVITE during call setup.

### 21.27.2	Conformance requirement

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

NOTE 2:	To ensure the identification and the related routing (e.g. to a test centre) of non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes, "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual" are not used for such calls. Additionally the usage of any other service URN to establish non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes needs to be in accordance with local operator policy.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall over IMS support defined in 3GPP TS 36.331 [19F] or in 3GPP TS 38.331 [19G], the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 1:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall act as follows:

iii)	if the UE is in limited service state, perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

iv)	if the UE is not in limited service state, report the response and successful MSD transfer to upper layers; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

NOTE 2:	For case 3-a-iv). upper layers can wait for an incoming call from the PSAP as defined in CEN EN 17184:2024 [300] if incoming IMS calls are possible or if the upper layers can instigate CS fallback to receive incoming calls over CS. If incoming calls cannot be received by the UE, upper layers can perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

### 21.27.3	Test purpose

1)	To verify that the UE considers MSD transmission as successful after receiving 486 (Busy here) SIP error message with positive acknowledgement for automatically initiated INVITE for eCall over IMS.

2)	To verify that the UE checks the header fields in incoming IMS call for PSAP callback and responds with 200 OK including correctly composed Recv-Info and Accept header fields.

### 21.27.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 36.508 [94] table 4.9.3.5-1. The UE has discovered P-CSCF and registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE subjected to national regulations in CEN EN 17184:2024 for IMS eCall over EPS (TS 34.229-2 [5] A.4.5/67)

1)	Automatic eCall over IMS initiated at UE.

2-5) Emergency registration according to C.20 is executed.

6)	SS waits for UE to send an INVITE request.

7)	SS sends 486 Busy here.

8)	UE sends ACK.

9)	SS releases the RRC connection and waits for 10 seconds.

10) SS sends SIP INVITE with correctly composed Accept, Priority and Recv-Info header.

11) SS expects call setup with correctly composed 200 OK with Accept and Recv-Info headers and sends ACK.

12) SS sends INFO request to transfer MSD.

13) SS expects and receives 200 OK.

14) SS expects and receives INFO request from the UE with MSD contents.

15) SS responds with 200 OK for INFO request from UE.

16) SS sends BYE to the UE.

17) SS expects and receives 200 OK for BYE from the UE.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate an automatic eCall |  |
| 2-5 | - |  | Steps 1-4 defined in Annex C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1. |
| 6 | -> |  | Step 1 defined in C.47 | Automatic eCall is initiated. |
| 7 | <- |  | 486 Busy here | The SS sends 486 Busy here. |
| 8 | -> |  | ACK | UE sends ACK. |
| 9 | - |  | SS releases the RRC connection |  |
| 10 |  |  |  | SS waits for 10 seconds |
| 11-23 | - |  | Steps 1-13 defined in C.11 | MT call establishment. Referred from 36.508 [94] table 4.5A.7.3-1. |
| 24-27 | - |  | Steps 4-7 defined in C.47 | MSD transfer. |
| 28-31 | - |  | Steps 1-4 defined in C.33 | SS releases the call. |

Specific Message Contents

INVITE (Step 6, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A21.

486 Busy Here (Step 7)

As specified in Annex A.2.21 with condition A1

ACK (Step 8)

As specified in Annex A.2.7 with condition A4

INVITE (Step 11, step 1 of C.11)

Use INVITE as specified for step 1 in annex C.11 with condition A12.

200 OK for INVITE (Step 22, step 12 of C.11)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with condition A24.

BYE (Step 28, step 1 of C.33)

Use the default message “BYE” in annex A.2.8 with condition A9.

### 21.27.5	Test requirements

The UE shall send requests and responses as described in clause 21.27.4.

<End of modified section>