3GPP TSG-RAN5 Meeting #107	R5-252769r1

# Malta, May 19 - 23 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3946 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR1 and EUTRA - FR2 testability issue CR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The list of test cases that UE does not have to pass the test case due to mix of E-UTRA/FR1 and FR2 carriers is missing in TS 38.533. Discussion paper was submitted to RAN5#107 to introduce such list to TS 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New annex (A.19 Test Cases involving E-UTRA/FR1 and FR2 carriers) is added. Test cases with the E-UTRA/FR1 and FR2 testability issue are listed in tables A.19.2-1, A.19.3-1, A.19.4-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Cases involving E-UTRA/FR1 and FR2 carriers with the testabilty issue will remain missing in TS 38.533. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.19 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1: In the revision the titles of the tables were corrected and test cases that are not complete in 38.533 were removed. A.19.5 for NR-DC was added. Note 1 was added to the tables. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/// Start of changes

# A.18	Spatial Relation Configuration

## A.18.1	Introduction

This clause provides the configurations for spatial relation towards either SSB or CSI-RS. The spatial relation defined in this clause are configured in each test when applicable to indicate spatial setting for certain UL signals with the referenceSignal configured in the spatial relation.

## A.18.2	Spatial Relation

Table A.18.2-1: PUCCH Spatial Relation

| Parameter | PUCCH.SRI.0 | PUCCH.SRI.1 |
| --- | --- | --- |
| pucch-SpatialRelationInfoId | Id0 | Id1 |
| referenceSignal | SSB0 | SSB1 |
| PUCCH-PathlossReferenceRS | SSB0 | SSB1 |
| Note 1: referenceSignal configurations towards which the spatial relation are configured in a test-specific manner. |  |  |

Table A.18.2-2: SRS Spatial Relation

| Parameter | SRS.SRI0 | SRS.SRI1 |
| --- | --- | --- |
| srs-SpatialRelationInfoId | Id0 | Id1 |
| referenceSignal | SSB0 | SSB1 |
| Note 1: referenceSignal configurations towards which the spatial relation are configured in a test-specific manner. |  |  |

# A.19	Test Cases involving E-UTRA/FR1 and FR2 carriers

## A.19.1	Introduction

The following applies to UE when undergoing tests with a mix of E-UTRA/NR FR1 and NR FR2 carriers in clauses 5, 7 and 8.

## A.19.2	Principle of Testing in EN-DC

For test cases in clause 5 listed in Table A.19.2-1, the following applies:

-	UE does not have to pass the test case.

Table A.19.2-1: Test cases with the E-UTRA/FR1 – FR2 testability issue (EN-DC)

| Clause | Test case slogan |
| --- | --- |
| 5.5.2.7 | E-UTRAN - NR FR2 interruptions at E-UTRA SRS carrier based switching |
| 5.5.3.6 | Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2 |
| 5.5.6.1.2 | EN-DC FR2 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC |
| 5.6.2.5 | EN-DC FR1-FR2 event-triggered reporting in non-DRX |
| 5.6.2.6 | EN-DC FR1-FR2 event-triggered reporting in DRX |
| 5.6.2.7 | EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection |
| 5.6.2.8 | EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection |
| 5.7.1.3 | EN-DC FR1-FR2 SS-RSRP measurement accuracy |
| Note 1: The list of the test case is based on TS 38.133 [6] Table A.3.13A.2-1 and updated based on the RAN5 testability analysis. |  |

### A.19.3	Principle of Testing in SA

For test cases in clause 7 listed in Table A.19.3-1, the following applies:

-	UE does not have to pass the test case.

Table A.19.3-1: Test cases with the E-UTRA/FR1 – FR2 testability issue (SA)

| Clause | Test case slogan |
| --- | --- |
| 7.3.1.1 | NR SA FR1-FR2 Inter-frequency handover; unknown target cell |
| 7.3.1.4 | NR SA FR1-FR2 synchronous DAPS handover |
| 7.3.1.5 | NR SA FR1-FR2 asynchronous DAPS handover |
| 7.5.3.2 | NR SA FR1-FR2 inter-band SCell activation and deactivation delay |
| 7.5.6.1.2 | NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX |
| 7.5.7.1 | NR SA FR2 addition and release delay of known PSCell |
| 7.5.7.2 | NR SA FR2 addition and release delay of unknown PSCell |
| 7.5.12.1 | NR SA FR2 Addition and Release Delay of PSCell |
| 7.5.14.1 | NR SA FR2 PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 |
| 7.6.2.5 | NR SA FR1-FR2 event-triggered reporting in non-DRX |
| 7.6.2.6 | NR SA FR1-FR2 event-triggered reporting in DRX |
| 7.6.2.7 | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection |
| 7.6.2.8 | NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used |
| 7.6.3.6 | NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used |
| 7.7.1.3 | NR SA FR1-FR2 SS-RSRP measurement accuracy |
| Note 1: The list of the test case is based on TS 38.133 [6] Table A.3.13A.3-1 and updated based on the RAN5 testability analysis. |  |

### A.19.4	Principle of Testing in E-UTRA

For test cases in clause 8 listed in Table A.19.4-1, the following applies:

-	UE does not have to pass the test case.

Table A.19.4-1: Test cases with the E-UTRA/FR1 – FR2 testability issue (E-UTRA)

| Clause | Test case slogan |
| --- | --- |
| 8.2.2.2 | E-UTRA – NR FR2 Early Measurement Reporting |
| 8.4.2.5 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX |
| 8.4.2.6 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX |
| 8.4.2.7 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX |
| 8.4.2.8 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX |
| 8.4.3.1 | E-UTRAN - NR SA FR2 event-triggered reporting without gap with interruption |
| Note 1: The list of the test case is based on TS 38.133 [6] Table A.3.13A.4-1 and updated based on the RAN5 testability analysis. |  |

### A.19.5	Principle of Testing in NR-DC

For test cases in clause 7 listed in Table A.19.5-1, the following applies:

-	UE does not have to pass the test case.

Table A.19.5-1: Test cases with the E-UTRA/FR1 – FR2 testability issue (NR-DC)

| Clause | Test case slogan |
| --- | --- |
| 7.3.3.5 | NR SA FR2 NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC |
| Note 1: The list of the test case is based on TS 38.133 [6] Table A.3.13A.5-1 and updated based on the RAN5 testability analysis. |  |

Annex B (normative):
Conditions for RRM requirements applicability for operating bands

/// End of changes