3GPP TSG-RAN5 Meeting #107	R5-252879r1

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0978 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Core alignment to correct aggregation level and number of candidates for SDR test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Table 5.5A-1 defines different aggregation levels depending on the channel bandwidth and subcarrier spacing under test. However, for some testing scenarios, the number of resources required to configure the aggregation level and number of candidates is higher than the number of resources available in the coreset considering HARQ feedback is configured in PUSCH channel for SDR. SCS (kHz); ChBw (MHz); AggLevel; Candidates; Required resources in Coreset; Available resources in Coreset. 15; 15; 8; 2; 96; 78. 30; 5; 8; 2; 96; 6. 30; 10; 4; 2; 48; 24. 30; 25; 8; 2; 96; 60. 30; 30; 8; 2; 96; 78. 30; 35; 8; 2; 96; 90. For channel bandwidth 5 MHz and SCS 30 kHz, coreset size is only 6 CCE, but it is needed at least 12 CCE, in the case of using minimum AggregationLevel set to One with 2 candidates, so, there is not a valid configuration to test this scenario. |
| Summary of change: |  | - Modify aggregation level for current invalid testing scenarios to ensure enough PDCCH resources for DL and UL DCI in same slot as requested in SDR test scenario: SCS (kHz); ChBw (MHz); AggLevel; Candidates; Required resources in Coreset; Available resources in Coreset. 15; 15; 4; 2; 48; 78. 30; 10; 2; 2; 24; 24. 30; 25; 4; 2; 48; 60. 30; 30; 4; 2; 48; 78. 30; 35; 4; 2; 48; 90. - Include a note to indicate that channel bandwidth 5 MHz and subcarrier spacing 30 kHz is not a valid testing scenario. - Remove entry in the Reference Channel for SDR test cases for 5 MHz for the TDD cases. |
| Consequences if not approved: |  | Test configuration for some (ChBw, SCS) combination will remain incorrect and SDR scenarios will not ensure HARQ feedback in PUSCH for all UL slots. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.1, 5.5A.1, 9.4B.1.1, Annex A.3.2_1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CR R4-2507687. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-252879: - removing 30kHz/5MHz changes as there is NOT any band in TS 38.101-1 defining 30 kHz for 5MHz CBW - Removing 35MHz/30kHz as 35MHz is NOT define in Rel-15 and Rel-16, it is defined in Rel-17 and onwards |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### 5.5.1	FR1 Sustained downlink data rate performance for single carrier

5.5.1.1	Test Purpose

The purpose of the test is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. The sustained downlink data rate shall be verified in terms of the success rate of delivered PDCP SDU(s) by Layer 2. The test case below specifies the RF conditions and the required success rate of delivered TB by Layer 1 to meet the sustained data rate requirement

5.5.1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

5.5.1.3	Minimum conformance requirements

The requirements in this clause are applicable to the FR1 single carrier case.

The TB success rate shall be higher than 85% when PDSCH is scheduled with MCS defined for the channel bandwidth with the downlink physical channel setup according to Annex C.3.1.

The TB success rate is defined as 100%*NDL_correct_rx / (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks.

The common test parameters are specified in Table 5.5.1.3-1. The parameters specified in Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in Table 5.5.1.3-3 are applicable for tests on TDD bands.

Unless otherwise stated, no user data is scheduled on slot #0, 10 and 11 within 20 ms for SCS 15 kHz.

Unless otherwise stated, no user data is scheduled on slot #0, 20 and 21 within 20 ms for SCS 30 kHz.

Table 5.5.1.3-1: Common test parameters for FDD and TDD bands

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| EPRE ratio of PTRS to PDSCH |  |  | dB | N/A |
| Channel bandwidth |  |  | MHz | Select channel bandwidth as per test parameter selection clause 5.5.1.3.1 |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
| Cross carrier scheduling |  |  |  | Not configured |
| Active DL BWP index |  |  |  | 1 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  |  | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
|  | Cyclic prefix |  |  | Normal |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  |  | Symbols #0 |
|  | Number of PRBs in CORESET |  |  | Table 5.5.1.3-4 |
|  | Number of PDCCH candidates and aggregation levels |  |  | 2/AL2 for 15 kHz / 5 MHz, 30 kHz / 10 MHz and 30 kHz / 15 MHz 2/AL4 for 15 kHz / 10 MHz, 15 kHz / 15 MHz, 30 kHz / 20 MHz, 30 kHz / 25 MHz and 30 kHz / 30 MHz 2/AL8 for other greater combinations |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI State |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | For number of Tx=1: No precoding;For number of Tx=2: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of precoder indices 0 and 2 For number of Tx=4: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of i_1,1 in {1,2,3,5,6,7} and i_2 in {0,2} |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | WB |
|  | Resource allocation type |  |  | Type 0 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 1 |
|  | Length |  |  | 1 |
|  | Antenna ports indexes |  |  | {1000} for 1 Layer CCs {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for 1 layer and 2 layers CCs 2 for 4 Layers CCs |
| PTRS configuration |  |  |  | PTRS is not configured |
| CSI-RS for tracking | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 3 for CSI-RS resource 1,2,3,4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | For number of Tx=1: ‘No CDM’ For number of Tx>1: 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | For number of Tx=1: No precoding; For number of Tx>1: Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot with equal probability of each applicable i1, i2 combination |
| Symbols for all unused Res |  |  |  | OCNG Annex A.5 |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |
| Antenna configuration | 1 layer CCs |  |  | 1x2 or 1x4 |
|  | 2 layers CCs |  |  | 2x2 or 2x4 |
|  | 4 layers CCs |  |  | 4x4 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Es |  |  | dBm/Hz | -112 for MCS indexes in Table 5.5.1.3-5: -110.5 For 1024QAM MCS indexes in Table 5.5.2.3-1: |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |  |  |  |  |

## <<< Skip unchanged sections >>>

### 5.5A.1	FR1 Sustained downlink data rate performance for carrier aggregation

##### 5.5A.1.1	FR1 SDR performance for CA (2DL CA)

5.5A.1.1.1	Test Purpose

The purpose of the test is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. The sustained downlink data rate shall be verified in terms of the success rate of delivered PDCP SDU(s) by Layer 2. The test case below specifies the RF conditions and the required success rate of delivered TB by Layer 1 to meet the sustained data rate requirement.

5.5A.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supports 2DL CA

5.5A.1.1.3	Minimum conformance requirements

The Sustained Data Rate (SDR) requirements in this clause are applicable to the FR1 CA.

The TB success rate shall be higher than 85% when PDSCH is scheduled with MCS defined for the selected CA bandwidth combination and with the downlink physical channel setup according to Annex C.2.1.

The TB success rate is defined as 100%*NDL_correct_rx / (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks.

The common test parameters are specified in Table 5.5A.1.1.3-1. The parameters specified in Table 5.5A.1.1.3-2 are applicable for tests on FDD CCs and parameters specified in Table 5.5A.1.1.3-3 are applicable for tests on TDD CCs.

Unless otherwise stated, no user data is scheduled on slot #0, 10 and 11 within 20 ms for SCS 15 kHz.

Unless otherwise stated, no user data is scheduled on slot #0, 20 and 21 within 20 ms for SCS 30 kHz.

Table 5.5A.1.1.3-1: Common test parameters for FDD and TDD component carriers

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| EPRE ratio of PTRS to PDSCH |  |  | dB | N/A |
| Channel bandwidth |  |  | MHz | Channel bandwidth from selected CA bandwidth combination |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
| Cross carrier scheduling |  |  |  | Not configured |
| Active DL BWP index |  |  |  | 1 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  |  | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
|  | Cyclic prefix |  |  | Normal |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  |  | Symbols #0 |
|  | Number of PRBs in CORESET |  |  | Table 5.5A.1.1.3-4 |
|  | Number of PDCCH candidates and aggregation levels |  |  | 2/AL2 for 15 kHz / 5 MHz, 30 kHz / 10 MHz and 30 kHz / 15 MHz 2/AL4 for 15 kHz / 10 MHz, 15kHz / 15 MHz, 30 kHz / 20 MHz, 30 kHz / 25 MHz and 30 kHz / 30 MHz 2/AL8 for other greater combinations |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI State |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | For number of Tx=1: No precoding;For number of Tx=2: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of precoder indices 0 and 2 For number of Tx=4: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of i_1,1 in {1,2,3,5,6,7} and i_2 in {0,2} |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | wideband |
|  | Resource allocation type |  |  | Type 0 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 1 |
|  | Length |  |  | 1 |
|  | Antenna ports indexes |  |  | {1000} for 1 Layer CCs {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for 1 layer and 2 layers CCs 2 for 4 Layers CCs |
| PTRS configuration |  |  |  | PTRS is not configured |
| CSI-RS for tracking | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 3 for CSI-RS resource 1,2,3,4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | For number of Tx=1: ‘No CDM’ For number of Tx>1: 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | For number of Tx=1: No precoding; For number of Tx>1: Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12] |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 OP.1 TDD as defined in Annex A.5.2.1 |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |
| Antenna configuration | 1 layer CCs |  |  | 1x2 or 1x4 |
|  | 2 layers CCs |  |  | 2x2 or 2x4 |
|  | 4 layers CCs |  |  | 4x4 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |  |  |  |  |

## <<< Skip unchanged sections >>>

#### 9.4B.1.1	Sustained downlink data rate performance for EN-DC within FR1

9.4B.1.1.1	Test Purpose

The purpose of the test is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. The sustained downlink data rate shall be verified in terms of the success rate of delivered PDCP SDU(s) by Layer 2. The test case below specifies the RF conditions and the required success rate of delivered TB by Layer 1 to meet the sustained data rate requirement

9.4B.1.1.2	Test Applicability

This test applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

9.4B.1.1.3	Minimum conformance requirements

During the test, the PDSCH performance on both the NR cell(s) and LTE cell(s) shall be verified.

The TB success rate shall be higher than 85% when NR PDSCH is scheduled with MCS defined for the selected EN-DC bandwidth combination and with the downlink physical channel setup according to Annex C.3.1 and when E-UTRA PDSCH is scheduled with FRC defined for the selected EN-DC bandwidth combination and with the downlink physical channel setup according to Annex C.3.2 from TS 36.101 [X].

The TB success rate is defined as 100%*NDL_correct_rx / (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks.

The common test parameters for NR cell are specified in Table 9.4B.1.1.3-1. The parameters specified in Table 9.4B.1.1.3-2 are applicable for tests on FDD NR cell and parameters specified in Table 9.4B.1.1.3-3 are applicable for tests on TDD NR cell.

Unless otherwise stated, no user data is scheduled on slot #0, 10 and 11 within 20 ms for SCS 15 kHz for NR cell.

Unless otherwise stated, no user data is scheduled on slot #0, 20 and 21 within 20 ms for SCS 30 kHz for NR cell.

Table 9.4B.1.1.3-1: Common test parameters for FDD or TDD NR band

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| EPRE ratio of PTRS to PDSCH |  |  | dB | N/A |
| Channel bandwidth |  |  | MHz | Channel bandwidth from selected CA bandwidth combination |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
| Cross carrier scheduling |  |  |  | Not configured |
| Active DL BWP index |  |  |  | 1 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  |  | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
|  | Cyclic prefix |  |  | Normal |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  |  | Symbols #0 |
|  | Number of PRBs in CORESET |  |  | Table 9.4B.1.1.3-4 |
|  | Number of PDCCH candidates and aggregation levels |  |  | 2/AL2 for 15 kHz / 5 MHz, 30 kHz / 10 MHz and 30 kHz / 15 MHz 2/AL4 for 15 kHz / 10 MHz, 15 kHz / 15 MHz, 30 kHz / 20 MHz, 30 kHz / 25 MHz and 30 kHz / 30 MHz 2/AL8 for other greater combinations |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI State |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | For 2Tx: Single Panel Type I, Random precoder chosen from precoder index 0 and 2, selection updated per slot For 4Tx: Single Panel Type I, Random precoder chosen from precoders with i_1,1 in {1,2,3,5,6,7} and i_2 in {0,2}, selection updated per slot |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | WB |
|  | Resource allocation type |  |  | Type 0 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 1 |
|  | Length |  |  | 1 |
|  | Antenna ports indexes |  |  | {1000} for 1 Layer CCs {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for 1 layer and 2 layers CCs 2 for 4 Layers CCs |
| PTRS configuration |  |  |  | PTRS is not configured |
| CSI-RS for tracking | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 3 for CSI-RS resource 1,2,3,4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | ‘FD-CDM2’ |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | ‘FD-CDM2’ |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity |
| Symbols for all unused REs |  |  |  | OCNG Annex A.5 |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |
| Antenna configuration | 1 layer CCs |  |  | 1x2 or 1x4 |
|  | 2 layers CCs |  |  | 2x2 or 2x4 |
|  | 4 layers CCs |  |  | 4x4 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |  |  |  |  |

## <<< Skip unchanged sections >>>

#### A.3.2_1.2.1	Reference measurement channels for SCS 30 kHz FR1

Table A.3.2_1.2.1-1: Sustained Downlink Data Rate Reference Channel for TDD 30kHz SCS FR1 (64QAM)

| Parameter | Channel bandwidth | Subcarrier spacing | Allocated resource blocks | Number of consecutive PDSCH symbols for allocated full DL slots (Note 1) | MCS Index (Note 2) | Modulation | Target Coding Rate | Number of MIMO layers | LDPC Base Graph | Information Bit Payload per Slot for allocated full DL slots (Note 1) | Transport block CRC per Slot for allocated full DL slots (Note 1) | Number of Code Blocks per Slot for allocated full DL slots (Note 1, 6) | Binary Channel Bits per Slot for allocated full DL slots (Note 1) | Max. Throughput averaged over 2 frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | kHz | PRBs | Symbols |  |  |  |  |  | Bits | Bits | CBs | Bits | Mbps |
|  | 10 | 30 | 24 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 9480 | 24 | 2 | 20736 | 11.85 |
|  | 15 | 30 | 38 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 14856 | 24 | 2 | 32832 | 18.57 |
|  | 20 | 30 | 51 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 19968 | 24 | 3 | 44064 | 24.96 |
|  | 25 | 30 | 65 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 25608 | 24 | 4 | 56160 | 32.01 |
|  | 30 | 30 | 78 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 30728 | 24 | 4 | 67392 | 38.41 |
|  | 35 | 30 | 92 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 35856 | 24 | 5 | 79488 | 44.82 |
|  | 40 | 30 | 106 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 42016 | 24 | 5 | 91584 | 52.52 |
|  | 45 | 30 | 119 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 47112 | 24 | 6 | 102816 | 58.89 |
|  | 50 | 30 | 133 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 52224 | 24 | 7 | 114912 | 65.28 |
|  | 60 | 30 | 162 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 63528 | 24 | 8 | 139968 | 79.41 |
|  | 80 | 30 | 217 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 86040 | 24 | 11 | 187488 | 107.55 |
|  | 90 | 30 | 245 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 96264 | 24 | 12 | 211680 | 120.33 |
|  | 100 | 30 | 273 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 106576 | 24 | 13 | 235872 | 133.22 |
|  | 10 | 30 | 24 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 13576 | 24 | 2 | 20736 | 16.97 |
|  | 15 | 30 | 38 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 21504 | 24 | 3 | 32832 | 26.88 |
|  | 20 | 30 | 51 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 28680 | 24 | 4 | 44064 | 35.85 |
|  | 25 | 30 | 65 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 36896 | 24 | 5 | 56160 | 46.12 |
|  | 30 | 30 | 78 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 44040 | 24 | 6 | 67392 | 55.05 |
|  | 35 | 30 | 92 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 51216 | 24 | 7 | 79488 | 64.02 |
|  | 40 | 30 | 106 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 59432 | 24 | 8 | 91584 | 74.29 |
|  | 45 | 30 | 119 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 67584 | 24 | 9 | 102816 | 84.48 |
|  | 50 | 30 | 133 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 73776 | 24 | 9 | 114912 | 92.22 |
|  | 60 | 30 | 162 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 90176 | 24 | 11 | 139968 | 112.72 |
|  | 80 | 30 | 217 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 122976 | 24 | 15 | 187488 | 153.72 |
|  | 90 | 30 | 245 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 139376 | 24 | 17 | 211680 | 174.22 |
|  | 100 | 30 | 273 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 151608 | 24 | 18 | 235872 | 189.51 |
|  | 10 | 30 | 24 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 14600 | 24 | 2 | 20736 | 18.25 |
|  | 15 | 30 | 38 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 23040 | 24 | 3 | 32832 | 28.8 |
|  | 20 | 30 | 51 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 30728 | 24 | 4 | 44064 | 38.41 |
|  | 25 | 30 | 65 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 38936 | 24 | 5 | 56160 | 48.67 |
|  | 30 | 30 | 78 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 47112 | 24 | 6 | 67392 | 58.89 |
|  | 35 | 30 | 92 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 55304 | 24 | 7 | 79488 | 69.13 |
|  | 40 | 30 | 106 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 64552 | 24 | 8 | 91584 | 80.69 |
|  | 45 | 30 | 119 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 71688 | 24 | 9 | 102816 | 89.61 |
|  | 50 | 30 | 133 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 79896 | 24 | 10 | 114912 | 99.87 |
|  | 60 | 30 | 162 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 98376 | 24 | 12 | 139968 | 122.97 |
|  | 80 | 30 | 217 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 131176 | 24 | 16 | 187488 | 163.97 |
|  | 90 | 30 | 245 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 147576 | 24 | 18 | 211680 | 184.47 |
|  | 100 | 30 | 273 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 163976 | 24 | 20 | 235872 | 204.97 |
|  | 10 | 30 | 24 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 18432 | 24 | 3 | 20736 | 23.04 |
|  | 15 | 30 | 38 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 29192 | 24 | 4 | 32832 | 36.49 |
|  | 20 | 30 | 51 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 38936 | 24 | 5 | 44064 | 48.67 |
|  | 25 | 30 | 65 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 50184 | 24 | 6 | 56160 | 62.73 |
|  | 30 | 30 | 78 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 59432 | 24 | 8 | 67392 | 74.29 |
|  | 35 | 30 | 92 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 69672 | 24 | 9 | 79488 | 87.09 |
|  | 40 | 30 | 106 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 81976 | 24 | 10 | 91584 | 102.47 |
|  | 45 | 30 | 119 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 92200 | 24 | 11 | 102816 | 115.25 |
|  | 50 | 30 | 133 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 102416 | 24 | 13 | 114912 | 128.02 |
|  | 60 | 30 | 162 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 125016 | 24 | 15 | 139968 | 156.27 |
|  | 80 | 30 | 217 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 167976 | 24 | 20 | 187488 | 209.97 |
|  | 90 | 30 | 245 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 188576 | 24 | 23 | 211680 | 235.72 |
|  | 100 | 30 | 273 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 208976 | 24 | 25 | 235872 | 261.22 |
|  | 10 | 30 | 24 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 18960 | 24 | 3 | 41472 | 23.7 |
|  | 15 | 30 | 38 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 29704 | 24 | 4 | 65664 | 37.13 |
|  | 20 | 30 | 51 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 39936 | 24 | 5 | 88128 | 49.92 |
|  | 25 | 30 | 65 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 51216 | 24 | 7 | 112320 | 64.02 |
|  | 30 | 30 | 78 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 61480 | 24 | 8 | 134784 | 76.85 |
|  | 35 | 30 | 92 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 71688 | 24 | 9 | 158976 | 89.61 |
|  | 40 | 30 | 106 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 83976 | 24 | 10 | 183168 | 104.97 |
|  | 45 | 30 | 119 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 94248 | 24 | 12 | 205632 | 117.81 |
|  | 50 | 30 | 133 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 104496 | 24 | 13 | 229824 | 130.62 |
|  | 60 | 30 | 162 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 127080 | 24 | 16 | 279936 | 158.85 |
|  | 80 | 30 | 217 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 172176 | 24 | 21 | 374976 | 215.22 |
|  | 90 | 30 | 245 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 192624 | 24 | 23 | 423360 | 240.78 |
|  | 100 | 30 | 273 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 213176 | 24 | 26 | 471744 | 266.47 |
|  | 10 | 30 | 24 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 27144 | 24 | 4 | 41472 | 33.93 |
|  | 15 | 30 | 38 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 43032 | 24 | 6 | 65664 | 53.79 |
|  | 20 | 30 | 51 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 57376 | 24 | 7 | 88128 | 71.72 |
|  | 25 | 30 | 65 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 73776 | 24 | 9 | 112320 | 92.22 |
|  | 30 | 30 | 78 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 88064 | 24 | 11 | 134784 | 110.08 |
|  | 35 | 30 | 92 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 102416 | 24 | 13 | 158976 | 128.02 |
|  | 40 | 30 | 106 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 118896 | 24 | 15 | 183168 | 148.62 |
|  | 45 | 30 | 119 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 135296 | 24 | 17 | 205632 | 169.12 |
|  | 50 | 30 | 133 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 147576 | 24 | 18 | 229824 | 184.47 |
|  | 60 | 30 | 162 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 180376 | 24 | 22 | 279936 | 225.47 |
|  | 80 | 30 | 217 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 245976 | 24 | 30 | 374976 | 307.47 |
|  | 90 | 30 | 245 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 278776 | 24 | 34 | 423360 | 348.47 |
|  | 100 | 30 | 273 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 303240 | 24 | 36 | 471744 | 379.05 |
|  | 10 | 30 | 24 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 29192 | 24 | 4 | 41472 | 36.49 |
|  | 15 | 30 | 38 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 46104 | 24 | 6 | 65664 | 57.63 |
|  | 20 | 30 | 51 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 61480 | 24 | 8 | 88128 | 76.85 |
|  | 25 | 30 | 65 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 77896 | 24 | 10 | 112320 | 97.37 |
|  | 30 | 30 | 78 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 94248 | 24 | 12 | 134784 | 117.81 |
|  | 35 | 30 | 92 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 110632 | 24 | 14 | 158976 | 138.29 |
|  | 40 | 30 | 106 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 129128 | 24 | 16 | 183168 | 161.41 |
|  | 45 | 30 | 119 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 143400 | 24 | 18 | 205632 | 179.25 |
|  | 50 | 30 | 133 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 159880 | 24 | 19 | 229824 | 199.85 |
|  | 60 | 30 | 162 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 196776 | 24 | 24 | 279936 | 245.97 |
|  | 80 | 30 | 217 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 262376 | 24 | 32 | 374976 | 327.97 |
|  | 90 | 30 | 245 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 295176 | 24 | 36 | 423360 | 368.97 |
|  | 100 | 30 | 273 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 327888 | 24 | 39 | 471744 | 409.86 |
|  | 10 | 30 | 24 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 36896 | 24 | 5 | 41472 | 46.12 |
|  | 15 | 30 | 38 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 58384 | 24 | 7 | 65664 | 72.98 |
|  | 20 | 30 | 51 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 77896 | 24 | 10 | 88128 | 97.37 |
|  | 25 | 30 | 65 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 100392 | 24 | 12 | 112320 | 125.49 |
|  | 30 | 30 | 78 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 118896 | 24 | 15 | 134784 | 148.62 |
|  | 35 | 30 | 92 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 139376 | 24 | 17 | 158976 | 174.22 |
|  | 40 | 30 | 106 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 163976 | 24 | 20 | 183168 | 204.97 |
|  | 45 | 30 | 119 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 184424 | 24 | 22 | 205632 | 230.53 |
|  | 50 | 30 | 133 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 204976 | 24 | 25 | 229824 | 256.22 |
|  | 60 | 30 | 162 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 250056 | 24 | 30 | 279936 | 312.57 |
|  | 80 | 30 | 217 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 335976 | 24 | 40 | 374976 | 419.97 |
|  | 90 | 30 | 245 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 376896 | 24 | 45 | 423360 | 471.12 |
|  | 100 | 30 | 273 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 417976 | 24 | 50 | 471744 | 522.47 |
|  | 10 | 30 | 24 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 37896 | 24 | 5 | 76032 | 47.37 |
|  | 15 | 30 | 38 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 60456 | 24 | 8 | 120384 | 75.57 |
|  | 20 | 30 | 51 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 81976 | 24 | 10 | 161568 | 102.47 |
|  | 25 | 30 | 65 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 104496 | 24 | 13 | 205920 | 130.62 |
|  | 30 | 30 | 78 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 125016 | 24 | 15 | 247104 | 156.27 |
|  | 35 | 30 | 92 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 147576 | 24 | 18 | 291456 | 184.47 |
|  | 40 | 30 | 106 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 167976 | 24 | 20 | 335808 | 209.97 |
|  | 45 | 30 | 119 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 188576 | 24 | 23 | 376992 | 235.72 |
|  | 50 | 30 | 133 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 213176 | 24 | 26 | 421344 | 266.47 |
|  | 60 | 30 | 162 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 258144 | 24 | 31 | 513216 | 322.68 |
|  | 80 | 30 | 217 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 344376 | 24 | 41 | 687456 | 430.47 |
|  | 90 | 30 | 245 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 393272 | 24 | 47 | 776160 | 491.59 |
|  | 100 | 30 | 273 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 434280 | 24 | 52 | 864864 | 542.85 |
|  | 10 | 30 | 24 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 53288 | 24 | 7 | 76032 | 66.61 |
|  | 15 | 30 | 38 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 83976 | 24 | 10 | 120384 | 104.97 |
|  | 20 | 30 | 51 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 112648 | 24 | 14 | 161568 | 140.81 |
|  | 25 | 30 | 65 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 143400 | 24 | 18 | 205920 | 179.25 |
|  | 30 | 30 | 78 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 172176 | 24 | 21 | 247104 | 215.22 |
|  | 35 | 30 | 92 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 204976 | 24 | 25 | 291456 | 256.22 |
|  | 40 | 30 | 106 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 237776 | 24 | 29 | 335808 | 297.22 |
|  | 45 | 30 | 119 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 262376 | 24 | 32 | 376992 | 327.97 |
|  | 50 | 30 | 133 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 295176 | 24 | 36 | 421344 | 368.97 |
|  | 60 | 30 | 162 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 360488 | 24 | 43 | 513216 | 450.61 |
|  | 80 | 30 | 217 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 483464 | 24 | 58 | 687456 | 604.33 |
|  | 90 | 30 | 245 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 540776 | 24 | 65 | 776160 | 675.97 |
|  | 100 | 30 | 273 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 606504 | 24 | 72 | 864864 | 758.13 |
|  | 10 | 30 | 24 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 57376 | 24 | 7 | 76032 | 71.72 |
|  | 15 | 30 | 38 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 90176 | 24 | 11 | 120384 | 112.72 |
|  | 20 | 30 | 51 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 120936 | 24 | 15 | 161568 | 151.17 |
|  | 25 | 30 | 65 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 155776 | 24 | 19 | 205920 | 194.72 |
|  | 30 | 30 | 78 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 184424 | 24 | 22 | 247104 | 230.53 |
|  | 35 | 30 | 92 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 221376 | 24 | 27 | 291456 | 276.72 |
|  | 40 | 30 | 106 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 254176 | 24 | 31 | 335808 | 317.72 |
|  | 45 | 30 | 119 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 286976 | 24 | 35 | 376992 | 358.72 |
|  | 50 | 30 | 133 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 319784 | 24 | 38 | 421344 | 399.73 |
|  | 60 | 30 | 162 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 385272 | 24 | 46 | 513216 | 481.59 |
|  | 80 | 30 | 217 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 516312 | 24 | 62 | 687456 | 645.39 |
|  | 90 | 30 | 245 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 590128 | 24 | 71 | 776160 | 737.66 |
|  | 100 | 30 | 273 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 655800 | 24 | 78 | 864864 | 819.75 |
|  | 10 | 30 | 24 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 67584 | 24 | 9 | 76032 | 84.48 |
|  | 15 | 30 | 38 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 106576 | 24 | 13 | 120384 | 133.22 |
|  | 20 | 30 | 51 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 143400 | 24 | 18 | 161568 | 179.25 |
|  | 25 | 30 | 65 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 184424 | 24 | 22 | 205920 | 230.53 |
|  | 30 | 30 | 78 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 221376 | 24 | 27 | 247104 | 276.72 |
|  | 35 | 30 | 92 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 258144 | 24 | 31 | 291456 | 322.68 |
|  | 40 | 30 | 106 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 295176 | 24 | 36 | 335808 | 368.97 |
|  | 45 | 30 | 119 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 335976 | 24 | 40 | 376992 | 419.97 |
|  | 50 | 30 | 133 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 376896 | 24 | 45 | 421344 | 471.12 |
|  | 60 | 30 | 162 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 458896 | 24 | 55 | 513216 | 573.62 |
|  | 80 | 30 | 217 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 606504 | 24 | 72 | 687456 | 758.13 |
|  | 90 | 30 | 245 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 688776 | 24 | 82 | 776160 | 860.97 |
|  | 100 | 30 | 273 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 770568 | 24 | 92 | 864864 | 963.21 |
| Note 1: Allocated full DL slots are with slot index i, if mod(i,10) = 0,1,2,3,4,5,6 and i is not in {0,20,21} for i = 0,1,...,39. So total number of allocated slots per 2 frames is 25. Note 2: MCS Index is based on MCS Table defined in TS38.214 when 256QAM is not enabled. MCS 18 and 19 are equivalent to MCS 11 and 12 in 256QAM table, respectively. Note 3: Number of DMRS REs per RB = 12,12,24,24 for number of MIMO layers = 1,2,3,4, respectively Note 4: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 5: Overhead parameter for TBS determination is 0. Note 6: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.2_1.2.1-2: Sustained Downlink Data Rate Reference Channel for TDD 30kHz SCS FR1(256QAM)

| Parameter | Channel bandwidth | Subcarrier spacing | Allocated resource blocks | Number of consecutive PDSCH symbols for allocated full DL slots (Note 1) | MCS Index (Note 2) | Modulation | Target Coding Rate | Number of MIMO layers | LDPC Base Graph | Information Bit Payload per Slot for allocated full DL slots (Note 1) | Transport block CRC per Slot for allocated full DL slots (Note 1) | Number of Code Blocks per Slot for allocated full DL slots (Note 1, 6) | Binary Channel Bits per Slot for allocated full DL slots (Note 1) | Max. Throughput averaged over 2 frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | kHz | PRBs | Symbols |  |  |  |  |  | Bits | Bits | CBs | Bits | Mbps |
|  | 10 | 30 | 24 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 18432 | 24 | 3 | 27648 | 23.04 |
|  | 15 | 30 | 38 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 29192 | 24 | 4 | 43776 | 36.49 |
|  | 20 | 30 | 51 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 38936 | 24 | 5 | 58752 | 48.67 |
|  | 25 | 30 | 65 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 50184 | 24 | 6 | 74880 | 62.73 |
|  | 30 | 30 | 78 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 59432 | 24 | 8 | 89856 | 74.29 |
|  | 35 | 30 | 92 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 69672 | 24 | 9 | 105984 | 87.09 |
|  | 40 | 30 | 106 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 81976 | 24 | 10 | 122112 | 102.47 |
|  | 45 | 30 | 119 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 92200 | 24 | 11 | 137088 | 115.25 |
|  | 50 | 30 | 133 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 102416 | 24 | 13 | 153216 | 128.02 |
|  | 60 | 30 | 162 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 125016 | 24 | 15 | 186624 | 156.27 |
|  | 80 | 30 | 217 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 167976 | 24 | 20 | 249984 | 209.97 |
|  | 90 | 30 | 245 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 188576 | 24 | 23 | 282240 | 235.72 |
|  | 100 | 30 | 273 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 208976 | 24 | 25 | 314496 | 261.22 |
|  | 10 | 30 | 24 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 18960 | 24 | 3 | 27648 | 23.7 |
|  | 15 | 30 | 38 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 30216 | 24 | 4 | 43776 | 37.77 |
|  | 20 | 30 | 51 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 40976 | 24 | 5 | 58752 | 51.22 |
|  | 25 | 30 | 65 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 52224 | 24 | 7 | 74880 | 65.28 |
|  | 30 | 30 | 78 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 62504 | 24 | 8 | 89856 | 78.13 |
|  | 35 | 30 | 92 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 73776 | 24 | 9 | 105984 | 92.22 |
|  | 40 | 30 | 106 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 83976 | 24 | 10 | 122112 | 104.97 |
|  | 45 | 30 | 119 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 94248 | 24 | 12 | 137088 | 117.81 |
|  | 50 | 30 | 133 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 106576 | 24 | 13 | 153216 | 133.22 |
|  | 60 | 30 | 162 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 129128 | 24 | 16 | 186624 | 161.41 |
|  | 80 | 30 | 217 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 172176 | 24 | 21 | 249984 | 215.22 |
|  | 90 | 30 | 245 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 196776 | 24 | 24 | 282240 | 245.97 |
|  | 100 | 30 | 273 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 217128 | 24 | 26 | 314496 | 271.41 |
|  | 10 | 30 | 24 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 24576 | 24 | 3 | 27648 | 30.72 |
|  | 15 | 30 | 38 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 38936 | 24 | 5 | 43776 | 48.67 |
|  | 20 | 30 | 51 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 52224 | 24 | 7 | 58752 | 65.28 |
|  | 25 | 30 | 65 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 67584 | 24 | 9 | 74880 | 84.48 |
|  | 30 | 30 | 78 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 79896 | 24 | 10 | 89856 | 99.87 |
|  | 35 | 30 | 92 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 94248 | 24 | 12 | 105984 | 117.81 |
|  | 40 | 30 | 106 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 108552 | 24 | 13 | 122112 | 135.69 |
|  | 45 | 30 | 119 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 122976 | 24 | 15 | 137088 | 153.72 |
|  | 50 | 30 | 133 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 135296 | 24 | 17 | 153216 | 169.12 |
|  | 60 | 30 | 162 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 167976 | 24 | 20 | 186624 | 209.97 |
|  | 80 | 30 | 217 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 225480 | 24 | 27 | 249984 | 281.85 |
|  | 90 | 30 | 245 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 254176 | 24 | 31 | 282240 | 317.72 |
|  | 100 | 30 | 273 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 278776 | 24 | 34 | 314496 | 348.47 |
|  | 10 | 30 | 24 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 36896 | 24 | 5 | 55296 | 46.12 |
|  | 15 | 30 | 38 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 58384 | 24 | 7 | 87552 | 72.98 |
|  | 20 | 30 | 51 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 77896 | 24 | 10 | 117504 | 97.37 |
|  | 25 | 30 | 65 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 100392 | 24 | 12 | 149760 | 125.49 |
|  | 30 | 30 | 78 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 118896 | 24 | 15 | 179712 | 148.62 |
|  | 35 | 30 | 92 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 139376 | 24 | 17 | 211968 | 174.22 |
|  | 40 | 30 | 106 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 163976 | 24 | 20 | 244224 | 204.97 |
|  | 45 | 30 | 119 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 184424 | 24 | 22 | 274176 | 230.53 |
|  | 50 | 30 | 133 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 204976 | 24 | 25 | 306432 | 256.22 |
|  | 60 | 30 | 162 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 250056 | 24 | 30 | 373248 | 312.57 |
|  | 80 | 30 | 217 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 335976 | 24 | 40 | 499968 | 419.97 |
|  | 90 | 30 | 245 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 376896 | 24 | 45 | 564480 | 471.12 |
|  | 100 | 30 | 273 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 417976 | 24 | 50 | 628992 | 522.47 |
|  | 10 | 30 | 24 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 37896 | 24 | 5 | 55296 | 47.37 |
|  | 15 | 30 | 38 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 60456 | 24 | 8 | 87552 | 75.57 |
|  | 20 | 30 | 51 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 81976 | 24 | 10 | 117504 | 102.47 |
|  | 25 | 30 | 65 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 104496 | 24 | 13 | 149760 | 130.62 |
|  | 30 | 30 | 78 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 125016 | 24 | 15 | 179712 | 156.27 |
|  | 35 | 30 | 92 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 147576 | 24 | 18 | 211968 | 184.47 |
|  | 40 | 30 | 106 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 167976 | 24 | 20 | 244224 | 209.97 |
|  | 45 | 30 | 119 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 188576 | 24 | 23 | 274176 | 235.72 |
|  | 50 | 30 | 133 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 213176 | 24 | 26 | 306432 | 266.47 |
|  | 60 | 30 | 162 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 258144 | 24 | 31 | 373248 | 322.68 |
|  | 80 | 30 | 217 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 344376 | 24 | 41 | 499968 | 430.47 |
|  | 90 | 30 | 245 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 393272 | 24 | 47 | 564480 | 491.59 |
|  | 100 | 30 | 273 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 434280 | 24 | 52 | 628992 | 542.85 |
|  | 10 | 30 | 24 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 49176 | 24 | 6 | 55296 | 61.47 |
|  | 15 | 30 | 38 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 77896 | 24 | 10 | 87552 | 97.37 |
|  | 20 | 30 | 51 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 104496 | 24 | 13 | 117504 | 130.62 |
|  | 25 | 30 | 65 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 135296 | 24 | 17 | 149760 | 169.12 |
|  | 30 | 30 | 78 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 159880 | 24 | 19 | 179712 | 199.85 |
|  | 35 | 30 | 92 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 188576 | 24 | 23 | 211968 | 235.72 |
|  | 40 | 30 | 106 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 217128 | 24 | 26 | 244224 | 271.41 |
|  | 45 | 30 | 119 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 245976 | 24 | 30 | 274176 | 307.47 |
|  | 50 | 30 | 133 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 270576 | 24 | 33 | 306432 | 338.22 |
|  | 60 | 30 | 162 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 335976 | 24 | 40 | 373248 | 419.97 |
|  | 80 | 30 | 217 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 450984 | 24 | 54 | 499968 | 563.73 |
|  | 90 | 30 | 245 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 507984 | 24 | 61 | 564480 | 634.98 |
|  | 100 | 30 | 273 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 557416 | 24 | 67 | 628992 | 696.77 |
|  | 10 | 30 | 24 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 73776 | 24 | 9 | 101376 | 92.22 |
|  | 15 | 30 | 38 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 118896 | 24 | 15 | 160512 | 148.62 |
|  | 20 | 30 | 51 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 159880 | 24 | 19 | 215424 | 199.85 |
|  | 25 | 30 | 65 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 200808 | 24 | 24 | 274560 | 251.01 |
|  | 30 | 30 | 78 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 241720 | 24 | 29 | 329472 | 302.15 |
|  | 35 | 30 | 92 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 286976 | 24 | 35 | 388608 | 358.72 |
|  | 40 | 30 | 106 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 327888 | 24 | 39 | 447744 | 409.86 |
|  | 45 | 30 | 119 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 368872 | 24 | 44 | 502656 | 461.09 |
|  | 50 | 30 | 133 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 409616 | 24 | 49 | 561792 | 512.02 |
|  | 60 | 30 | 162 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 507984 | 24 | 61 | 684288 | 634.98 |
|  | 80 | 30 | 217 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 671976 | 24 | 80 | 916608 | 839.97 |
|  | 90 | 30 | 245 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 770568 | 24 | 92 | 1034880 | 963.21 |
|  | 100 | 30 | 273 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 852696 | 24 | 102 | 1153152 | 1065.87 |
|  | 10 | 30 | 24 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 79896 | 24 | 10 | 101376 | 99.87 |
|  | 15 | 30 | 38 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 125016 | 24 | 15 | 160512 | 156.27 |
|  | 20 | 30 | 51 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 167976 | 24 | 20 | 215424 | 209.97 |
|  | 25 | 30 | 65 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 213176 | 24 | 26 | 274560 | 266.47 |
|  | 30 | 30 | 78 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 258144 | 24 | 31 | 329472 | 322.68 |
|  | 35 | 30 | 92 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 303240 | 24 | 36 | 388608 | 379.05 |
|  | 40 | 30 | 106 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 352440 | 24 | 42 | 447744 | 440.55 |
|  | 45 | 30 | 119 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 393272 | 24 | 47 | 502656 | 491.59 |
|  | 50 | 30 | 133 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 434280 | 24 | 52 | 561792 | 542.85 |
|  | 60 | 30 | 162 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 540776 | 24 | 65 | 684288 | 675.97 |
|  | 80 | 30 | 217 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 721000 | 24 | 86 | 916608 | 901.25 |
|  | 90 | 30 | 245 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 803304 | 24 | 96 | 1034880 | 1004.13 |
|  | 100 | 30 | 273 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 901344 | 24 | 107 | 1153152 | 1126.68 |
|  | 10 | 30 | 24 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 90176 | 24 | 11 | 101376 | 112.72 |
|  | 15 | 30 | 38 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 143400 | 24 | 18 | 160512 | 179.25 |
|  | 20 | 30 | 51 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 192624 | 24 | 23 | 215424 | 240.78 |
|  | 25 | 30 | 65 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 245976 | 24 | 30 | 274560 | 307.47 |
|  | 30 | 30 | 78 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 295176 | 24 | 36 | 329472 | 368.97 |
|  | 35 | 30 | 92 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 344376 | 24 | 41 | 388608 | 430.47 |
|  | 40 | 30 | 106 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 401640 | 24 | 48 | 447744 | 502.05 |
|  | 45 | 30 | 119 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 450984 | 24 | 54 | 502656 | 563.73 |
|  | 50 | 30 | 133 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 500136 | 24 | 60 | 561792 | 625.17 |
|  | 60 | 30 | 162 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 606504 | 24 | 72 | 684288 | 758.13 |
|  | 80 | 30 | 217 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 819256 | 24 | 97 | 916608 | 1024.07 |
|  | 90 | 30 | 245 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 934152 | 24 | 111 | 1034880 | 1167.69 |
|  | 100 | 30 | 273 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 1032192 | 24 | 123 | 1153152 | 1290.24 |
| Note 1: Allocated full DL slots are with slot index i, if mod(i,10) = 0,1,2,3,4,5,6 and i is not in {0,20,21} for i = 0,1,...,39. So total number of allocated slots per 2 frames is 25. Note 2: MCS Index is based on MCS Table defined in TS38.214 when 256QAM is enabled. Note 3: Number of DMRS REs per RB = 12,12,24,24 for number of MIMO layers = 1,2,3,4, respectively Note 4: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 5: Overhead parameter for TBS determination is 0. Note 6: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.2_1.2.1-3: Sustained Downlink Data Rate Reference Channel for TDD 30kHz SCS FR1(1024QAM)

| Parameter | Channel bandwidth | Subcarrier spacing | Allocated resource blocks | Number of consecutive PDSCH symbols for allocated full DL slots (Note 1) | MCS Index (Note 2) | Modulation | Target Coding Rate | Number of MIMO layers | LDPC Base Graph | Information Bit Payload per Slot for allocated full DL slots (Note 1) | Transport block CRC per Slot for allocated full DL slots (Note 1) | Number of Code Blocks per Slot for allocated full DL slots (Note 1, 6) | Binary Channel Bits per Slot for allocated full DL slots (Note 1) | Max. Throughput averaged over 2 frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | kHz | PRBs | Symbols |  |  |  |  |  | Bits | Bits | CBs | Bits | Mbps |
|  | 10 | 30 | 24 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 27144 | 24 | 4 | 34560 | 33.93 |
|  | 15 | 30 | 38 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 43032 | 24 | 6 | 54720 | 53.79 |
|  | 20 | 30 | 51 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 57376 | 24 | 7 | 73440 | 71.72 |
|  | 25 | 30 | 65 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 73776 | 24 | 9 | 93600 | 92.22 |
|  | 30 | 30 | 78 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 88064 | 24 | 11 | 112320 | 110.08 |
|  | 35 | 30 | 92 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 104496 | 24 | 13 | 132480 | 130.62 |
|  | 40 | 30 | 106 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 120936 | 24 | 15 | 152640 | 151.17 |
|  | 45 | 30 | 119 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 135296 | 24 | 17 | 171360 | 169.12 |
|  | 50 | 30 | 133 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 151608 | 24 | 18 | 191520 | 189.51 |
|  | 60 | 30 | 162 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 184424 | 24 | 22 | 233280 | 230.53 |
|  | 80 | 30 | 217 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 245976 | 24 | 30 | 312480 | 307.47 |
|  | 90 | 30 | 245 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 278776 | 24 | 33 | 352800 | 348.47 |
|  | 100 | 30 | 273 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 311368 | 24 | 37 | 393120 | 389.21 |
|  | 10 | 30 | 24 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 28680 | 24 | 4 | 34560 | 35.85 |
|  | 15 | 30 | 38 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 45096 | 24 | 6 | 54720 | 56.37 |
|  | 20 | 30 | 51 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 61480 | 24 | 8 | 73440 | 76.85 |
|  | 25 | 30 | 65 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 77896 | 24 | 10 | 93600 | 97.37 |
|  | 30 | 30 | 78 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 94248 | 24 | 12 | 112320 | 117.81 |
|  | 35 | 30 | 92 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 110632 | 24 | 14 | 132480 | 138.29 |
|  | 40 | 30 | 106 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 127080 | 24 | 16 | 152640 | 158.85 |
|  | 45 | 30 | 119 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 143400 | 24 | 18 | 171360 | 179.25 |
|  | 50 | 30 | 133 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 159880 | 24 | 19 | 191520 | 199.85 |
|  | 60 | 30 | 162 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 192624 | 24 | 23 | 233280 | 240.78 |
|  | 80 | 30 | 217 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 262376 | 24 | 32 | 312480 | 327.97 |
|  | 90 | 30 | 245 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 295176 | 24 | 35 | 352800 | 368.97 |
|  | 100 | 30 | 273 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 327888 | 24 | 39 | 393120 | 409.86 |
|  | 10 | 30 | 24 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 54296 | 24 | 7 | 69120 | 67.87 |
|  | 15 | 30 | 38 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 86040 | 24 | 11 | 109440 | 107.55 |
|  | 20 | 30 | 51 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 114776 | 24 | 14 | 146880 | 143.47 |
|  | 25 | 30 | 65 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 147576 | 24 | 18 | 187200 | 184.47 |
|  | 30 | 30 | 78 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 176208 | 24 | 21 | 224640 | 220.26 |
|  | 35 | 30 | 92 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 208976 | 24 | 25 | 264960 | 261.22 |
|  | 40 | 30 | 106 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 241720 | 24 | 29 | 305280 | 302.15 |
|  | 45 | 30 | 119 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 270576 | 24 | 33 | 342720 | 338.22 |
|  | 50 | 30 | 133 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 303240 | 24 | 36 | 383040 | 379.05 |
|  | 60 | 30 | 162 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 368872 | 24 | 44 | 466560 | 461.09 |
|  | 80 | 30 | 217 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 491800 | 24 | 59 | 624960 | 614.75 |
|  | 90 | 30 | 245 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 557416 | 24 | 66 | 705600 | 696.77 |
|  | 100 | 30 | 273 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 622760 | 24 | 74 | 786240 | 778.45 |
| Note 1: Allocated full DL slots are with slot index i, if mod(i,10) = 0,1,2,3,4,5,6 and i is not in {0,20,21} for i = 0,1,...,39. So total number of allocated slots per 2 frames is 25. Note 2: MCS Index is based on MCS Table defined in TS38.214 when 1024QAM is enabled. Note 3: Number of DMRS REs per RB = 12,12,24,24 for number of MIMO layers = 1,2,3,4, respectively Note 4: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 5: Overhead parameter for TBS determination is 0. Note 6: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES >>>