3GPP TSG-RAN5 Meeting #108	R5-253792

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0834 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RF baseline implementation capabilities for new PC2 R18 EN-DC combos within FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The RF baseline implementation capabilities for these PC2 EN-DC configurations are not complete. DC_1A_n77(2A), DC_3A_n77(2A), DC_11A_n77A, DC_41C_n77A, DC_42A_n77A, DC_42C_n77A, DC_1A-11A_n77A, DC_1A-41A_n77(2A), DC_1A-41C_n77A, DC_1A-41C_n77(2A), DC_1A-42A_n77A, DC_1A-42C_n77A, DC_3A-41A_n77(2A), DC_3A-41C_n77A, DC_3A-41C_n77(2A), DC_3A-42A_n77A, DC_3A-42C_n77A |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding these PC2 EN-DC configurations in Table A.4.3.2B.2.3.1-3, Table A.4.3.2B.2.3.1-3a and Table A.4.3.2B.2.3.2-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The RF baseline implementation capabilities for these PC2 EN-DC configurations will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2B.2.3.1, A.4.3.2B.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

A.4.3.2B.2.3.1	Inter-band EN-DC within FR1 (two bands)

Table A.4.3.2B.2.3.1-1: Downlink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and two bands (for one or more of the supported DC configurations in Table A.4.3.2B.2.3.1-2)

| Item | DL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | Inter-band EN-DC within FR1 BW Class Combination A_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_A_A |  |
| 2 | Inter-band EN-DC within FR1 BW Class Combination A_(2A) (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_A_(2A) |  |
| 3 | Inter-band EN-DC within FR1 BW Class Combination A_B (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_A_B |  |
| 4 | Inter-band EN-DC within FR1 BW Class Combination A_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_A_C |  |
| 5 | Inter-band EN-DC within FR1 BW Class Combination (2A)_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_(2A)_A |  |
| 6 | Inter-band EN-DC within FR1 BW Class Combination (2A)_(2A) (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_(2A)_(2A) |  |
| 7 | Inter-band EN-DC within FR1 BW Class Combination (2A)_B (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_(2A)_B |  |
| 8 | Inter-band EN-DC within FR1 BW Class Combination (3A)_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_(3A)_A |  |
| 9 | Inter-band EN-DC within FR1 BW Class Combination B_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_B_A |  |
| 10 | Inter-band EN-DC within FR1 BW Class Combination C_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_C_A |  |
| 11 | Inter-band EN-DC within FR1 BW Class Combination C_(2A) (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_C_(2A) |  |
| 12 | Inter-band EN-DC within FR1 BW Class Combination C_B (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_C_B |  |
| 13 | Inter-band EN-DC within FR1 BW Class Combination C_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_C_C |  |
| 14 | Inter-band EN-DC within FR1 BW Class Combination D_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_D_A |  |
| 15 | Inter-band EN-DC within FR1 BW Class Combination D_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_D_C |  |
| 16 | Inter-band EN-DC within FR1 BW Class Combination E_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_E_A |  |
| 17 | Inter-band EN-DC within FR1 BW Class Combination E_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_E_C |  |

Table A.4.3.2B.2.3.1-1a: Uplink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and two bands (for one or more of the supported configurations in Table A.4.3.2B.2.3.1-2)

| Item | UL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL Inter-band EN-DC within FR1 BW Class Combination A_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_UL_inter_band_EN_DC_FR1_2B_Class_A_A |  |
| 2 | UL Inter-band EN-DC within FR1 BW Class Combination A_B (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_UL_inter_band_EN_DC_FR1_2B_Class_A_B |  |
| 3 | UL Inter-band EN-DC within FR1 BW Class Combination A_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_UL_inter_band_EN_DC_FR1_2B_Class_A_C |  |
| 4 | UL Inter-band EN-DC within FR1 BW Class Combination (2A)_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_UL_inter_band_EN_DC_FR1_2B_Class_(2A)_A |  |
| 5 | UL Inter-band EN-DC within FR1 BW Class Combination C_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_UL_inter_band_EN_DC_FR1_2B_Class_C_A |  |

Table A.4.3.2B.2.3.1-2: Supported Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration / Item | Release | Supported | Supported EN-DC Bandwidth Class(es) in UL | Supported ULTxSwitching Band Pair | Supported uplinkTxSwitching-DL-Interruption-r16 (Note 1) | Supported simultaneousRxTx (Note 2) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n3A | Rel-16 |  |  |  |  |  |
| DC_1A_n5A | Rel-16 |  |  |  |  |  |
| DC_1A_n7A | Rel-16 |  |  |  |  |  |
| DC_1A_n28A | Rel-15 |  |  |  |  |  |
| DC_1A_n41A | Rel-16 |  |  |  |  | Yes |
| DC_1A_n77A | Rel-15 |  |  |  | Not supported | Yes |
| DC_1A_n78A | Rel-15 |  |  |  | Not supported | Yes |
| DC_1A_n78C | Rel-15 |  |  |  | Not supported | Yes |
| DC_1A-1A_n78A | Rel-17 |  |  |  | Not supported |  |
| DC_1A_n79A | Rel-15 |  |  |  | Not supported | Yes |
| DC_2A_n5A | Rel-15 |  |  |  |  |  |
| DC_2A-2A_n5A | Rel-16 |  |  |  |  |  |
| DC_2A_n41A | Rel-16 |  |  |  |  |  |
| DC_2C_n41A | Rel-16 |  |  |  |  |  |
| DC_2A_n66A | Rel-15 |  |  |  |  |  |
| DC_2A-2A_n66A | Rel-16 |  |  |  |  |  |
| DC_2A_n71A | Rel-15 |  |  |  |  |  |
| DC_2A_n77A | Rel-17 |  |  |  |  |  |
| DC_2A_n77(2A) | Rel-17 |  |  |  |  |  |
| DC_2A-2A_n77A | Rel-17 |  |  |  |  |  |
| DC_2A-2A_n77(2A) | Rel-17 |  |  |  |  |  |
| DC_2A_n78A | Rel-15 |  |  |  |  |  |
| DC_3A_n1A | Rel-16 |  |  |  |  |  |
| DC_3A_n8A | Rel-16 |  |  |  |  |  |
| DC_3A_n7A | Rel-15 |  |  |  |  |  |
| DC_3A_n5A | Rel-16 |  |  |  |  |  |
| DC_3A_n28A | Rel-15 |  |  |  |  |  |
| DC_3A_n41A | Rel-16 |  |  |  | Not supported | Yes |
| DC_3A_n77A | Rel-15 |  |  |  | Not supported | Yes |
| DC_3A_n78A | Rel-15 |  |  |  | Not supported | Yes |
| DC_3A_n78C | Rel-15 |  |  |  | Not supported | Yes |
| DC_3A_n79A | Rel-15 |  |  |  | Not supported | Yes |
| DC_3A_n82A | Rel-15 |  |  |  |  |  |
| DC_3C_n77A | Rel-17 |  |  |  | Not supported |  |
| DC_3C_n77(2A) | Rel-17 |  |  |  | Not supported |  |
| DC_3C_n78A | Rel-15 |  |  |  | Not supported |  |
| DC_5A_n2A | Rel-16 |  |  |  |  |  |
| DC_5A_n66A | Rel-15 |  |  |  |  |  |
| DC_5A_n77A | Rel-17 |  |  |  |  |  |
| DC_5A_n77(2A) | Rel-17 |  |  |  |  |  |
| DC_5A_n78A | Rel-15 |  |  |  | Not supported | Yes |
| DC_5A_n78C | Rel-17 |  |  |  | Not supported | Yes |
| DC_7A_n1A | Rel-16 |  |  |  |  |  |
| DC_7A_n3A | Rel-16 |  |  |  |  |  |
| DC_7A_n5A | Rel-16 |  |  |  |  |  |
| DC_7A_n8A | Rel-16 |  |  |  |  |  |
| DC_7A_n28A | Rel-15 |  |  |  |  |  |
| DC_7A_n78A | Rel-15 |  |  |  |  | Yes |
| DC_7A_n66A | Rel-15 |  |  |  |  |  |
| DC_7C_n66A | Rel-15 |  |  |  |  |  |
| DC_7C_n78A | Rel-15 |  |  |  |  | Yes |
| DC_8A_n1A | Rel-16 |  |  |  |  |  |
| DC_8A_n3A | Rel-16 |  |  |  |  |  |
| DC_8A_n20A | Rel-16 |  |  |  |  |  |
| DC_8A_n41A | Rel-16 |  |  |  | Not supported | Yes |
| DC_8A_n77A | Rel-15 |  |  |  | Not supported | Yes |
| DC_8A_n77(2A) | Rel-16 |  |  |  | Not supported | Yes |
| DC_8A_n78A | Rel-15 |  |  |  | Not supported | Yes |
| DC_11A_n77A | Rel-15 |  |  |  | Not supported | Yes |
| DC_11A_n78A | Rel-15 |  |  |  | Not supported | Yes |
| DC_11A_n79A | Rel-15 |  |  |  |  | Yes |
| DC_12A_n2A | Rel-16 |  |  |  |  |  |
| DC_12A_n5A | Rel-15 |  |  |  |  |  |
| DC_12A_n66A | Rel-15 |  |  |  |  |  |
| DC_12A_n77A | Rel-17 |  |  |  |  |  |
| DC_12A_n77(2A) | Rel-17 |  |  |  |  |  |
| DC_12A_n78A | Rel-16 |  |  |  |  |  |
| DC_13A_n2A | Rel-16 |  |  |  |  |  |
| DC_13A_n66A | Rel-15 |  |  |  |  |  |
| DC_13A_n77A | Rel-17 |  |  |  |  |  |
| DC_14A_n2A | Rel-16 |  |  |  |  |  |
| DC_14A_n66A | Rel-16 |  |  |  |  |  |
| DC_14A_n77A | Rel-17 |  |  |  |  |  |
| DC_14A_n77(2A) | Rel-17 |  |  |  |  |  |
| DC_18A_n77A | Rel-15 |  |  |  | Not supported | Yes |
| DC_18A_n78A | Rel-15 |  |  |  | Not supported | Yes |
| DC_18A_n79A | Rel-15 |  |  |  |  | Yes |
| DC_19A_n1A | Rel-17 |  |  |  |  |  |
| DC_19A_n77A | Rel-15 |  |  |  |  | Yes |
| DC_19A_n77(2A) | Rel-17 |  |  |  |  | Yes |
| DC_19A_n78A | Rel-15 |  |  |  | Not supported | Yes |
| DC_19A_n78(2A) | Rel-17 |  |  |  | Not supported | Yes |
| DC_19A_n79A | Rel-15 |  |  |  | Not supported | Yes |
| DC_20A_n1A | Rel-16 |  |  |  |  |  |
| DC_20A_n3A | Rel-16 |  |  |  |  |  |
| DC_20A_n7A | Rel-16 |  |  |  |  |  |
| DC_20A_n8A | Rel-15 |  |  |  |  |  |
| DC_20A_n28A | Rel-15 |  |  |  |  |  |
| DC_20A_n78A | Rel-15 |  |  |  |  | Yes |
| DC_20A_n83A | Rel-16 |  |  |  |  |  |
| DC_21A_n1A | Rel-17 |  |  |  |  |  |
| DC_21A_n28A | Rel-17 |  |  |  |  |  |
| DC_21A_n77A | Rel-15 |  |  |  |  | Yes |
| DC_21A_n77(2A) | Rel-17 |  |  |  |  | Yes |
| DC_21A_n78A | Rel-15 |  |  |  | Not supported | Yes |
| DC_21A_n78(2A) | Rel-17 |  |  |  | Not supported | Yes |
| DC_21A_n79A | Rel-15 |  |  |  | Not supported | Yes |
| DC_25A_n41A | Rel-15 |  |  |  |  |  |
| DC_26A_n41A | Rel-16 |  |  |  |  |  |
| DC_26A_n77A | Rel-16 |  |  |  |  | Yes |
| DC_26A_n78A | Rel-16 |  |  |  |  | Yes |
| DC_26A_n79A | Rel-16 |  |  |  |  | Yes |
| DC_28A_n3A | Rel-16 |  |  |  |  |  |
| DC_28A_n5A | Rel-16 |  |  |  |  |  |
| DC_28A_n7A | Rel-16 |  |  |  |  |  |
| DC_28A_n51A | Rel-15 |  |  |  |  |  |
| DC_28A_n77A | Rel-15 |  |  |  | Not supported | Yes |
| DC_28A_n78A | Rel-15 |  |  |  | Not supported | Yes |
| DC_28A_n79A | Rel-15 |  |  |  |  | Yes |
| DC_30A_n2A | Rel-16 |  |  |  |  |  |
| DC_30A_n5A | Rel-15 |  |  |  |  |  |
| DC_30A_n77A | Rel-17 |  |  |  |  |  |
| DC_30A_n77(2A) | Rel-17 |  |  |  |  |  |
| DC_30A_n66A | Rel-15 |  |  |  |  |  |
| DC_38A_n78A | Rel-15 |  |  |  |  | Yes |
| DC_39A_n41A | Rel-16 |  |  |  | Not supported | Yes from Rel-18 |
| DC_39A_n79A | Rel-15 |  |  |  | Not supported | Yes |
| DC_40A_n1A | Rel-16 |  |  |  |  |  |
| DC_40A_n41A | Rel-16 |  |  |  |  | Yes from Rel-18 |
| DC_40A_n78A | Rel-16 |  |  |  |  |  |
| DC_40A_n79A | Rel-16 |  |  |  | Not supported | Yes |
| DC_40C_n78A | Rel-16 |  |  |  |  |  |
| DC_40C_n79A | Rel-16 |  |  |  | Not supported | Yes |
| DC_41A_n28A | Rel-16 |  |  |  |  | Yes |
| DC_41A_n77A | Rel-16 |  |  |  |  |  |
| DC_41A_n78A | Rel-16 |  |  |  |  |  |
| DC_41A_n79A | Rel-15 |  |  |  | Not supported | Yes |
| DC_42A_n1A | Rel-17 |  |  |  |  |  |
| DC_42C_n1A | Rel-17 |  |  |  |  |  |
| DC_42A_n77A | Rel-15 |  |  |  |  |  |
| DC_42A_n78A | Rel-15 |  |  |  |  |  |
| DC_42A_n79A | Rel-15 |  |  |  |  |  |
| DC_48A_n5A | Rel-16 |  |  |  |  |  |
| DC_48A_n46A | Rel-16 |  |  |  |  |  |
| DC_48A_n66A | Rel-16 |  |  |  |  |  |
| DC_66A_n2A | Rel-16 |  |  |  |  |  |
| DC_66A-66A_n2A | Rel-16 |  |  |  |  |  |
| DC_66A_n5A | Rel-15 |  |  |  |  |  |
| DC_66A-66A_n5A | Rel-16 |  |  |  |  |  |
| DC_66A_n41A | Rel-16 |  |  |  |  |  |
| DC_66A_n71A | Rel-15 |  |  |  |  |  |
| DC_66A_n77A | Rel-17 |  |  |  |  |  |
| DC_66A_n77(2A) | Rel-17 |  |  |  |  |  |
| DC_66A-66A_n77A | Rel-17 |  |  |  |  |  |
| DC_66A-66A_n77(2A) | Rel-17 |  |  |  |  |  |
| DC_66A-66A-66A_n77A | Rel-17 |  |  |  |  |  |
| DC_66A-66A-66A_n77(2A) | Rel-18 |  |  |  |  |  |
| DC_66A_n78A | Rel-15 |  |  |  |  |  |
| DC_71A_n2A | Rel-17 |  |  |  |  |  |
| DC_71A_n66A | Rel-16 |  |  |  |  |  |
| Note 1: A UE that supports ULTxSwitching on a band pair might report the uplinkTxSwitching-DL-Interruption-r16 capability on the same band pair. If UE doesn’t report this capability, no DL interruption is allowed during UL Tx switching. For certain band configurations DL interruption is not allowed according to Note 14 in Table 5.5B.4.1-1 of TS 38.101-3 [25], therefore the corresponding entry is prefilled by ‘Not Supported’. Note 2: For configurations with Note 7 in Table 5.5B.4.1-1 of TS 38.521-3 [7], UE capability simultaneousRxTxInterBandENDC is mandatory, therefore the corresponding entry is prefilled with ‘Yes’. |  |  |  |  |  |  |

Table A.4.3.2B.2.3.1-3: Inter-band EN-DC within FR1 (two bands) PC2 UE RF Baseline Implementation Capabilities

| Item | EN-DC configuration | Inter-band EN-DC within FR1 (two bands) PC2 UE RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_39A_n41A | LTE Frequency band: 1880-1920 MHz NR Frequency band: 2496-2690 MHz | 38.101-3, 6.2B.1.3 | Rel-16 | pc_Band39_nrBand41_PC2_Supp |  |
| 2 | DC_39A_n79A | LTE Frequency band: 1880-1920 MHz NR Frequency band: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-16 | pc_Band39_nrBand79_PC2_Supp |  |
| 3 | DC_41A_n79A | LTE Frequency band: 2496-2690 MHz NR Frequency band: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-16 | pc_Band41_nrBand79_PC2_Supp |  |
| 4 | DC_3A_n78A | LTE Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-16 | pc_Band3_nrBand78_PC2_Supp |  |
| 5 | DC_3A_n41A | LTE Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band: 2496-2690 MHz | 38.101-3, 6.2B.1.3 | Rel-16 | pc_Band3_nrBand41_PC2_Supp |  |
| 6 | DC_1A_n78A | LTE Frequency band: 1920-1980 MHz (UL),2110- 2170 MHz (DL) NR Frequency band: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band1_nrBand78_PC2_Supp |  |
| 7 | Void |  |  |  |  |  |
| 8 | DC_8A_n78A | LTE Frequency band: 703-748 MHz (UL), 758-803 MHz (DL) NR Frequency band: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band8_nrBand78_PC2_Supp |  |
| 9 | DC_2A_n77A | LTE Frequency band: 1850-1910 MHz (UL),1930-1990 MHz (DL) NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band2_nrBand77_PC2_Supp |  |
| 10 | DC_5A_n77A | LTE Frequency band: 824-849 MHz (UL),869- 894 MHz (DL) NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band5_nrBand77_PC2_Supp |  |
| 11 | DC_13A_n77A | LTE Frequency band: 777-787 MHz (UL),746-756 MHz (DL) NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band13_nrBand77_PC2_Supp |  |
| 12 | DC_66A_n77A | LTE Frequency band: 1710-1780 MHz (UL),2110-2200 MHz (DL) NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band66_nrBand77_PC2_Supp |  |
| 13 | DC_12A_n77A | LTE Frequency band: 699-716 MHz (UL),729- 746 MHz (DL) NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band12_nrBand77_PC2_Supp |  |
| 14 | DC_14A_n77A | LTE Frequency band: 788-798 MHz (UL),758-768 MHz (DL) NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band14_nrBand77_PC2_Supp |  |
| 15 | DC_30A_n77A | LTE Frequency band: 2305-2315 MHz (UL),2350-2360 MHz (DL) NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band30_nrBand77_PC2_Supp |  |
| 16 | DC_28A_n78A | LTE Frequency band: 703-748 MHz (UL),758- 803 MHz (DL) NR Frequency band: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band28_nrBand78_PC2_Supp |  |
| 17 | DC_1A_n79A | LTE Frequency band: 1920-1980 MHz (UL), 2110- 2170 MHz (DL) NR Frequency band: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_nrBand79_PC2_Supp |  |
| 18 | DC_3A_n79A | LTE Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_nrBand79_PC2_Supp |  |
| 19 | DC_19A_n78A | LTE Frequency band: 830-845 MHz (UL),875-890 MHz (DL) NR Frequency band: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band19_nrBand78_PC2_Supp |  |
| 20 | DC_19A_n79A | LTE Frequency band: 830-845 MHz (UL),875-890 MHz (DL) NR Frequency band: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band19_nrBand79_PC2_Supp |  |
| 21 | DC_21A_n78A | LTE Frequency band: 1447.9-1462.9 MHz (UL), 1495.9-1510.9 MHz (DL) NR Frequency band: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band21_nrBand78_PC2_Supp |  |
| 22 | DC_21A_n79A | LTE Frequency band: 1447.9-1462.9 MHz (UL), 1495.9-1510.9 MHz (DL) NR Frequency band: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band21_nrBand79_PC2_Supp |  |
| 23 | DC_3A_n77A | LTE Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_nrBand77_PC2_Supp |  |
| 24 | DC_18A_n77A | LTE Frequency band: 815-830 MHz (UL), 860-875 MHz (DL) NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band18_nrBand77_PC2_Supp |  |
| 25 | DC_28A_n77A | LTE Frequency band: 703-748 MHz (UL),758- 803 MHz (DL) NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band28_nrBand77_PC2_Supp |  |
| 26 | DC_1A_n41A | LTE Frequency band: 1920-1980 MHz (UL), 2110- 2170 MHz (DL) NR Frequency band: 2496-2690 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_nrBand41_PC2_Supp |  |
| 27 | DC_41A_n77A | LTE Frequency band: 2496-2690 MHz NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band41_nrBand77_PC2_Supp |  |
| 28 | DC_1A_n77A | LTE Frequency band: 1920-1980 MHz (UL), 2110- 2170 MHz (DL) NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_nrBand77_PC2_Supp |  |
| 29 | DC_2A_n41A | LTE Frequency band: 1850-1910 MHz (UL),1930-1990 MHz (DL) NR Frequency band: 2496-2690 MHz | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band2_nrBand41_PC2_Supp |  |
| 30 | DC_66A_n41A | LTE Frequency band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) NR Frequency band: 2496-2690 MHz | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band66_nrBand41_PC2_Supp |  |
| A | DC_1A_n77(2A) | LTE Frequency band: 1920-1980 MHz (UL), 2110- 2170 MHz (DL) NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_nrBand77_PC2_Supp |  |
| B | DC_3A_n77(2A) | LTE Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_nrBand77_PC2_Supp |  |
| C | DC_11A_n77A | LTE Frequency band: 1427.9-1447.9 MHz (UL), 1475.9-1495.9 MHz (DL) NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band11_nrBand77_PC2_Supp |  |
| D | DC_41C_n77A | LTE Frequency band: 2496-2690 MHz NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band41_nrBand77_PC2_Supp |  |
| E | DC_42A_n77A | LTE Frequency band: 3400-3600 MHz NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band42_nrBand77_PC2_Supp |  |
| F | DC_42C_n77A | LTE Frequency band: 3400-3600 MHz NR Frequency band: 3300-4200 MHz1 | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band42_nrBand77_PC2_Supp |  |
| NOTE 1: In the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz |  |  |  |  |  |  |

Table A.4.3.2B.2.3.1-3a: Inter-band EN-DC within FR1 (two bands) NR part power class UE RF Baseline Implementation Capabilities (Rel-16 and forward)

| Item | EN-DC configuration | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Supported NR part power class |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_39A_n41A | DC_39A_n41A NR part power class | 38.306, 4.2.7.1 | Rel-16 | pc_Band39_nrBand41_powerClassNRPart_r16 |  |
| 2 | DC_39A_n79A | DC_39A_n79A NR part power class | 38.306, 4.2.7.1 | Rel-16 | pc_Band39_nrBand79_powerClassNRPart_r16 |  |
| 3 | DC_41A_n79A | DC_41A_n79A NR part power class | 38.306, 4.2.7.1 | Rel-16 | pc_Band41_nrBand79_powerClassNRPart_r16 |  |
| 4 | DC_3A_n78A | DC_3A_n78A NR part power class | 38.306, 4.2.7.1 | Rel-16 | pc_Band3_nrBand78_powerClassNRPart_r16 |  |
| 5 | DC_3A_n41A | DC_3A_n41A NR part power class | 38.306, 4.2.7.1 | Rel-16 | pc_Band3_nrBand41_powerClassNRPart_r16 |  |
| 6 | DC_1A_n78A | DC_1A_n78A NR part power class | 38.306, 4.2.7.1 | Rel-17 | pc_Band1_nrBand78_powerClassNRPart_r17 |  |
| 7 | DC_8A_n78A | DC_8A_n78A NR part power class | 38.306, 4.2.7.1 | Rel-17 | pc_Band8_nrBand78_powerClassNRPart_r17 |  |
| 8 | DC_2A_n77A | DC_2A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-17 | pc_Band2_nrBand77_powerClassNRPart_r17 |  |
| 9 | DC_5A_n77A | DC_5A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-17 | pc_Band5_nrBand77_powerClassNRPart_r17 |  |
| 10 | DC_13A_n77A | DC_13A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-17 | pc_Band13_nrBand77_powerClassNRPart_r17 |  |
| 11 | DC_66A_n77A | DC_66A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-17 | pc_Band66_nrBand77_powerClassNRPart_r17 |  |
| 12 | DC_12A_n77A | DC_12A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-17 | pc_Band12_nrBand77_powerClassNRPart_r17 |  |
| 13 | DC_14A_n77A | DC_14A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-17 | pc_Band14_nrBand77_powerClassNRPart_r17 |  |
| 14 | DC_30A_n77A | DC_30A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-17 | pc_Band30_nrBand77_powerClassNRPart_r17 |  |
| 15 | DC_3A_n77A | DC_3A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-18 | pc_Band3_nrBand77_powerClassNRPart_r18 |  |
| 16 | DC_18A_n77A | DC_18A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-18 | pc_Band18_nrBand77_powerClassNRPart_r18 |  |
| 17 | DC_28A_n77A | DC_28A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-18 | pc_Band28_nrBand77_powerClassNRPart_r18 |  |
| 18 | DC_1A_n41A | DC_1A_n41A NR part power class | 38.306, 4.2.7.1 | Rel-18 | pc_Band1_nrBand41_powerClassNRPart_r18 |  |
| 19 | DC_41A_n77A | DC_41A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-18 | pc_Band41_nrBand77_powerClassNRPart_r18 |  |
| 20 | DC_1A_n77A | DC_1A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-18 | pc_Band1_nrBand77_powerClassNRPart_r18 |  |
| A | DC_1A_n77(2A) | DC_1A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-18 | pc_Band1_nrBand77_powerClassNRPart_r18 |  |
| B | DC_3A_n77(2A) | DC_3A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-18 | pc_Band3_nrBand77_powerClassNRPart_r18 |  |
| C | DC_11A_n77A | DC_11A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-18 | pc_Band11_nrBand77_powerClassNRPart_r18 |  |
| D | DC_41C_n77A | DC_41A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-18 | pc_Band41_nrBand77_powerClassNRPart_r18 |  |
| E | DC_42A_n77A | DC_42A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-18 | pc_Band42_nrBand77_powerClassNRPart_r18 |  |
| F | DC_42C_n77A | DC_42A_n77A NR part power class | 38.306, 4.2.7.1 | Rel-18 | pc_Band42_nrBand77_powerClassNRPart_r18 |  |

Table A.4.3.2B.2.3.1-3b: Inter-band EN-DC within FR1 (two bands) maxNumberSRS-Ports-PerResource UE RF Baseline Implementation Capabilities (Rel-15)

| Item | EN-DC configuration | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Supported maxNumberSRS-Ports-PerResource |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_39A_n41A | DC_39A_n41A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band39_nrBand41_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 2 | DC_39A_n79A | DC_39A_n79A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band39_nrBand79_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 3 | DC_41A_n79A | DC_41A_n79A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band41_nrBand79_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 4 | DC_3A_n78A | DC_3A_n78A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band3_nrBand78_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 5 | DC_3A_n41A | DC_3A_n41A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band3_nrBand41_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 6 | DC_1A_n78A | DC_1A_n78A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band1_nrBand78_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 7 | DC_8A_n78A | DC_8A_n78A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band8_nrBand78_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 8 | DC_2A_n77A | DC_2A_n77A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band2_nrBand77_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 9 | DC_5A_n77A | DC_5A_n77A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band5_nrBand77_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 10 | DC_13A_n77A | DC_13A_n77 maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band13_nrBand77_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 11 | DC_66A_n77A | DC_66A_n77A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band66_nrBand77_maxNumberSRS-Ports-PerResource_NR_r15 |  |

Table A.4.3.2B.2.3.1-3c: Inter-band EN-DC within FR1 (two bands) NR part power class UE RF Baseline Implementation Capabilities (Rel-15) (maxNumberSRS-Ports-PerResource=n2 in NR standalone operation mode, maxNumberSRS-Ports-PerResource=n1 for EN-DC on NR band)

| Item | EN-DC configuration | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Supported NR part power class |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_39A_n41A | DC_39A_n41A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band39_nrBand41_powerClassNRPart_r15 |  |
| 2 | DC_39A_n79A | DC_39A_n79A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band39_nrBand79_powerClassNRPart_r15 |  |
| 3 | DC_41A_n79A | DC_41A_n79A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band41_nrBand79_powerClassNRPart_r15 |  |
| 4 | DC_3A_n78A | DC_3A_n78A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band3_nrBand78_powerClassNRPart_r15 |  |
| 5 | DC_3A_n41A | DC_3A_n41A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band3_nrBand41_powerClassNRPart_r15 |  |
| 6 | DC_1A_n78A | DC_1A_n78A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band1_nrBand78_powerClassNRPart_r15 |  |
| 7 | DC_8A_n78A | DC_8A_n78A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band8_nrBand78_powerClassNRPart_r15 |  |
| 8 | DC_2A_n77A | DC_2A_n77A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band2_nrBand77_powerClassNRPart_r15 |  |
| 9 | DC_5A_n77A | DC_5A_n77A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band5_nrBand77_powerClassNRPart_r15 |  |
| 10 | DC_13A_n77A | DC_13A_n77A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band13_nrBand77_powerClassNRPart_r15 |  |
| 11 | DC_66A_n77A | DC_66A_n77A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band66_nrBand77_powerClassNRPart_r15 |  |

Table A.4.3.2B.2.3.1-4: UE Power Class implementation Capabilities for inter-band EN-DC within FR1 (two bands)

| Item | UE Power Class implementation Capabilities | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UE Power Class 2 for Inter-band EN-DC within FR1 (two bands) | 38.101-3, 6.2B.1.3 | Applicable to the bands in Table A.4.3.2B.2.3.1-3 |
| 2 | UE Power Class 3 for Inter-band EN-DC within FR1 (two bands) | 38.101-3, 6.2B.1.3 | Applicable to the bands in Table A.4.3.2B.2.3.1-2 |

Table A.4.3.2B.2.3.1-5: Inter-band EN-DC within FR1 (two bands) with UL MIMO capability

| Item | EN-DC configuration | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_3A_n78A | n78 band: UL MIMO | 38.101-3, 6.2H.1.3 | Rel-17 | pc_Band3_nrBand78_UL_MIMO |  |
| 2 | DC_7A_n78A | n78 band: UL MIMO | 38.101-3, 6.2H.1.3 | Rel-17 | pc_Band7_nrBand78_UL_MIMO |  |
| 3 | DC_8A_n78A | n78 band: UL MIMO | 38.101-3, 6.2H.1.3 | Rel-17 | pc_Band8_nrBand78_UL_MIMO |  |
| 4 | DC_20A_n78A | n78 band: UL MIMO | 38.101-3, 6.2H.1.3 | Rel-17 | pc_Band20_nrBand78_UL_MIMO |  |
| 5 | DC_28A_n78A | n78 band: UL MIMO | 38.101-3, 6.2H.1.3 | Rel-17 | pc_Band28_nrBand78_UL_MIMO |  |
| 6 | DC_40A_n78A | n78 band: UL MIMO | 38.101-3, 6.2H.1.3 | Rel-17 | pc_Band40_nrBand78_UL_MIMO |  |
| 7 | DC_41A_n78A | n78 band: UL MIMO | 38.101-3, 6.2H.1.3 | Rel-17 | pc_Band41_nrBand78_UL_MIMO |  |

Table A.4.3.2B.2.3.1-6: Inter-band EN-DC within FR1 (two bands) with Tx Diversity capability

| Item | EN-DC configuration | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_3A_n78A | n78 band: Tx Diversity | 38.101-3, 6.2L.1.3 | Rel-17 | pc_Band3_nrBand78_TxD |  |
| 2 | DC_7A_n78A | n78 band: Tx Diversity | 38.101-3, 6.2L.1.3 | Rel-17 | pc_Band7_nrBand78_TxD |  |
| 3 | DC_8A_n78A | n78 band: Tx Diversity | 38.101-3, 6.2L.1.3 | Rel-17 | pc_Band8_nrBand78_TxD |  |
| 4 | DC_20A_n78A | n78 band: Tx Diversity | 38.101-3, 6.2L.1.3 | Rel-17 | pc_Band20_nrBand78_TxD |  |
| 5 | DC_28A_n78A | n78 band: Tx Diversity | 38.101-3, 6.2L.1.3 | Rel-17 | pc_Band28_nrBand78_TxD |  |
| 6 | DC_40A_n78A | n78 band: Tx Diversity | 38.101-3, 6.2L.1.3 | Rel-17 | pc_Band40_nrBand78_TxD |  |
| 7 | DC_41A_n78A | n78 band: Tx Diversity | 38.101-3, 6.2L.1.3 | Rel-17 | pc_Band41_nrBand78_TxD |  |

A.4.3.2B.2.3.2	Inter-band EN-DC within FR1 (three bands)

Table A.4.3.2B.2.3.2-1: Downlink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and three bands (for one or more of the supported DC configurations in Table A.4.3.2B.2.3.2-2)

| Item | DL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic |
| --- | --- | --- | --- |
| 1 | Inter-band EN-DC within FR1 BW Class Combination A-A_A (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-A_A |
| 2 | Inter-band EN-DC withinFR1 BW Class Combination A-A_B (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-A_B |
| 3 | Inter-band EN-DC within FR1 BW Class Combination A-A_C (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-A_C |
| 4 | Inter-band EN-DC within FR1 BW Class Combination A-C_A (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-C_A |
| 5 | Inter-band EN-DC within FR1 BW Class Combination A-C_C (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-C_C |
| 6 | Inter-band EN-DC withinFR1 BW Class Combination A-D_A (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-D_A |
| 7 | Inter-band EN-DC within FR1 BW Class Combination A-E_A (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-E_A |
| 8 | Inter-band EN-DC within FR1 BW Class Combination A_A-A (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A_A-A |
| 9 | Inter-band EN-DC within FR1 BW Class Combination C-A_A (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_C-A_A |
| 10 | Inter-band EN-DC within FR1 BW Class Combination C-C_A (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_C-C_A |
| 11 | Inter-band EN-DC within FR1 BW Class Combination A_(n)AA (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A_(n)AA |
| 12 | Inter-band EN-DC within FR1 BW Class Combination (2A)-A_A (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_(2A)-A_A |
| 13 | Inter-band EN-DC within FR1 BW Class Combination (2A)-C_A (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_(2A)-C_A |
| 14 | Inter-band EN-DC within FR1 BW Class Combination A-A_(2A) (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-A_(2A) |
| 15 | Inter-band EN-DC within FR1 BW Class Combination A-C_(2A) (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-C_(2A) |

Table A.4.3.2B.2.3.2-1a: Uplink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and three bands (for one or more of the supported configurations in Table A.4.3.2B.2.3.2-2)

| Item | UL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL Inter-band EN-DC within FR1 BW Class Combination A_A (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_UL_inter_band_EN_DC_FR1_3B_Class_A_A |  |
| 2 | UL Inter-band EN-DC within FR1 BW Class Combination C_A (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_UL_inter_band_EN_DC_FR1_3B_Class_C_A |  |
| 3 | UL Inter-band EN-DC within FR1 BW Class Combination C_B (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_UL_inter_band_EN_DC_FR1_3B_Class_C_B |  |
| 4 | UL Inter-band EN-DC within FR1 BW Class Combination (n)AA (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_UL_inter_band_EN_DC_FR1_3B_Class_(n)AA |  |

Table A.4.3.2B.2.3.2-2: Supported Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration / Item (Note 1, 3, 5) | Release | Supported | Supported EN-DC Bandwidth Class(es) in UL (Note 2, 4) |
| --- | --- | --- | --- |
| DC_1A-3A_n28A | Rel-15 |  |  |
| DC_1A-3A_n41A | Rel-16 |  |  |
| DC_1A-3A_n77A | Rel-15 |  |  |
| DC_1A-3A_n78A | Rel-15 |  |  |
| DC_1A-3C_n77A | Rel-17 |  |  |
| DC_1A-3C_n77(2A) | Rel-17 |  |  |
| DC_1A-3C_n78A | Rel-15 |  |  |
| DC_1A-3C_n78(2A) | Rel-16 |  |  |
| DC_1A-1A-3A_n78A | Rel-17 |  |  |
| DC_1A-1A-3C_n78A | Rel-17 |  |  |
| DC_1A-1A-5A_n78A | Rel-17 |  |  |
| DC_1A-3A_n79A | Rel-15 |  |  |
| DC_1A-5A_n78C | Rel-17 |  |  |
| DC_1A-7A_n3A | Rel-16 |  |  |
| DC_1A-7A_n28A | Rel-15 |  |  |
| DC_1A-7A_n78A | Rel-15 |  |  |
| DC_1A-8A_n3A | Rel-16 |  |  |
| DC_1A-8A_n77A | Rel-16 |  |  |
| DC_1A-8A_n77(2A) | Rel-16 |  |  |
| DC_1A-8A_n78A | Rel-15 |  |  |
| DC_1A-8A_n78(2A) | Rel-17 |  |  |
| DC_1A-18A_n77A | Rel-15 |  |  |
| DC_1A-19A_n77(2A) | Rel-17 |  |  |
| DC_1A-19A_n78A | Rel-15 |  |  |
| DC_1A-19A_n78(2A) | Rel-17 |  |  |
| DC_1A-19A_n79A | Rel-15 |  |  |
| DC_1A-20A_n3A | Rel-16 |  |  |
| DC_1A-20A_n8A | Rel-16 |  |  |
| DC_1A-20A_n28A | Rel-15 |  |  |
| DC_1A-20A_n78A | Rel-15 |  |  |
| DC_1A-21A_n28A | Rel-17 |  |  |
| DC_1A-21A_n77(2A) | Rel-17 |  |  |
| DC_1A-21A_n78A | Rel-15 |  |  |
| DC_1A-21A_n78(2A) | Rel-17 |  |  |
| DC_1A-21A_n79A | Rel-15 |  |  |
| DC_1A-28A_n3A | Rel-16 |  |  |
| DC_1A-28A_n5A | Rel-16 |  |  |
| DC_1A-28A_n78C | Rel-15 |  |  |
| DC_1A_n28A-n78A | Rel-15 |  |  |
| DC_1A_n28A-n79A | Rel-17 |  |  |
| DC_1A-41A_n28A | Rel-16 |  |  |
| DC_1A-41C_n28A | Rel-16 |  |  |
| DC_1A-41A_n41A | Rel-16 |  |  |
| DC_1A-41A_n77A | Rel-15 |  |  |
| DC_1A-42A_n78A | Rel-15 |  |  |
| DC_1A-42C_n78A | Rel-15 |  |  |
| DC_1A-42D_n78A | Rel-15 |  |  |
| DC_1A-42E_n78A | Rel-15 |  |  |
| DC_1A-42A_n79A | Rel-15 |  |  |
| DC_1A-42C_n79A | Rel-15 |  |  |
| DC_1A-42D_n79A | Rel-15 |  |  |
| DC_1A-42E_n79A | Rel-15 |  |  |
| DC_1A_n78A-n79A | Rel-15 |  |  |
| DC_2A-5A_n2A | Rel-16 |  |  |
| DC_2A-5A_n66A | Rel-15 |  |  |
| DC_2A-2A-5A_n66A | Rel-17 |  |  |
| DC_2A-5A_n77A | Rel-17 |  |  |
| DC_2A-5A_n77(2A) | Rel-17 |  |  |
| DC_2A-2A-5A_n77A | Rel-17 |  |  |
| DC_2A-2A-5A_n77(2A) | Rel-18 |  |  |
| DC_2A_n5A-n77A | Rel-17 |  |  |
| DC_2A-13A_n77A | Rel-17 |  |  |
| DC_2A-2A-14A_n66A | Rel-16 |  |  |
| DC_2A-2A-66A_n66A | Rel-16 |  |  |
| DC_2A-14A_n2A | Rel-16 |  |  |
| DC_2A-14A_n66A | Rel-16 |  |  |
| DC_2A-14A_n77A | Rel-17 |  |  |
| DC_2A-2A-14A_n77A | Rel-17 |  |  |
| DC_2A-14A_n77(2A) | Rel-17 |  |  |
| DC_2A-2A-14A_n77(2A) | Rel-18 |  |  |
| DC_2A-66A_n2A | Rel-16 |  |  |
| DC_2A-66A_n41A | Rel-16 |  |  |
| DC_2A-66A_n66A | Rel-16 |  |  |
| DC_2A-66A-66A_n66A | Rel-17 |  |  |
| DC_2A-2A-66A-66A_n66A | Rel-17 |  |  |
| DC_2A-66A_n5A | Rel-16 |  |  |
| DC_2A-2A-66A_n5A | Rel-16 |  |  |
| DC_2A-66A-66A_n5A | Rel-16 |  |  |
| DC_2A-2A-66A-66A_n5A | Rel-16 |  |  |
| DC_2A-66A_n71A | Rel-15 |  |  |
| DC_2A-66A_n77A | Rel-17 |  |  |
| DC_2A-66A_n77(2A) | Rel-17 |  |  |
| DC_2A-2A-66A_n77A | Rel-17 |  |  |
| DC_2A-2A-66A_n77(2A) | Rel-18 |  |  |
| DC_2A-66A-66A_n2A | Rel-17 |  |  |
| DC_2A-66A-66A_n77A | Rel-17 |  |  |
| DC_2A-66A-66A_n77(2A) | Rel-18 |  |  |
| DC_2A-2A-66A-66A_n77A | Rel-17 |  |  |
| DC_2A-(n)71AA | Rel-15 |  |  |
| DC_3A_n1A-n78A | Rel-17 |  |  |
| DC_3A_n1A-n79A | Rel-17 |  |  |
| DC_3A-5A_n78C | Rel-17 |  |  |
| DC_3A-7A_n1A | Rel-16 |  |  |
| DC_3A-7A_n5A | Rel-16 |  |  |
| DC_3A-7A_n8A | Rel-16 |  |  |
| DC_3A-7A_n28A | Rel-15 |  |  |
| DC_3A-7A_n78A | Rel-15 |  |  |
| DC_3A-8A_n1A | Rel-16 |  |  |
| DC_3A-8A_n28A | Rel-16 |  |  |
| DC_3A-8A_n77A | Rel-16 |  |  |
| DC_3A-8A_n77(2A) | Rel-16 |  |  |
| DC_3A-8A_n78A | Rel-15 |  |  |
| DC_3A-8A_n78(2A) | Rel-17 |  |  |
| DC_3A-18A_n77A | Rel-16 |  |  |
| DC_3A-18A_n78A | Rel-16 |  |  |
| DC_3A-19A_n1A | Rel-17 |  |  |
| DC_3A-19A_n77(2A) | Rel-17 |  |  |
| DC_3A-19A_n78A | Rel-15 |  |  |
| DC_3A-19A_n78(2A) | Rel-17 |  |  |
| DC_3A-19A_n79A | Rel-15 |  |  |
| DC_3A-20A_n1A | Rel-16 |  |  |
| DC_3A-20A_n8A | Rel-16 |  |  |
| DC_3A-20A_n28A | Rel-15 |  |  |
| DC_3A-20A_n78A | Rel-15 |  |  |
| DC_3A-21A_n1A | Rel-17 |  |  |
| DC_3A-21A_n28A | Rel-17 |  |  |
| DC_3A-21A_n77(2A) | Rel-17 |  |  |
| DC_3A-21A_n78A | Rel-15 |  |  |
| DC_3A-21A_n78(2A) | Rel-17 |  |  |
| DC_3A-21A_n79A | Rel-15 |  |  |
| DC_3A-28A_n78A | Rel-15 |  |  |
| DC_3A_n28A-n78A | Rel-15 |  |  |
| DC_3A_n28A-n79A | Rel-17 |  |  |
| DC_3A-40A_n1A | Rel-16 |  |  |
| DC_3A-41A_n28A | Rel-17 |  |  |
| DC_3A-41C_n28A | Rel-16 |  |  |
| DC_3A-41A_n41A | Rel-16 |  |  |
| DC_3A-41A_n77A | Rel-16 |  |  |
| DC_3A-41A_n77(2A) | Rel-16 |  |  |
| DC_3A-41C_n77A | Rel-16 |  |  |
| DC_3A-42A_n1A | Rel-17 |  |  |
| DC_3A-42C_n1A | Rel-17 |  |  |
| DC_3A-42A_n78A | Rel-15 |  |  |
| DC_3A-42C_n78A | Rel-15 |  |  |
| DC_3A-42D_n78A | Rel-15 |  |  |
| DC_3A-42E_n78A | Rel-15 |  |  |
| DC_3A-42A_n79A | Rel-15 |  |  |
| DC_3A-42C_n79A | Rel-15 |  |  |
| DC_3A-42D_n79A | Rel-15 |  |  |
| DC_3A-42E_n79A | Rel-15 |  |  |
| DC_3A_n78A-n79A | Rel-15 |  |  |
| DC_3C-8A_n77A | Rel-17 |  |  |
| DC_3C-8A_n77(2A) | Rel-17 |  |  |
| DC_5A-66A_n2A | Rel-16 |  |  |
| DC_5A-66A-66A_n2A | Rel-16 |  |  |
| DC_5A-66A_n66A | Rel-16 |  |  |
| DC_5A-66A_n77A | Rel-17 |  |  |
| DC_5A-66A_n77(2A) | Rel-17 |  |  |
| DC_5A-66A-66A_n77A | Rel-17 |  |  |
| DC_5A-66A-66A_n77(2A) | Rel-18 |  |  |
| DC_5A-7A_n78A | Rel-15 |  |  |
| DC_7A-5A_n78A | Rel-16 |  |  |
| DC_7A-8A_n1A | Rel-16 |  |  |
| DC_7A-8A_n3A | Rel-16 |  |  |
| DC_7A-20A_n1A | Rel-16 |  |  |
| DC_7A-20A_n3A | Rel-16 |  |  |
| DC_7A-20A_n8A | Rel-16 |  |  |
| DC_7A-20A_n28A | Rel-15 |  |  |
| DC_7A-20A_n78A | Rel-15 |  |  |
| DC_7A-28A_n5A | Rel-16 |  |  |
| DC_7A_n28A-n78A | Rel-15 |  |  |
| DC_7C-5A_n78A | Rel-16 |  |  |
| DC_7C_n28A-n78A | Rel-16 |  |  |
| DC_13A_n2A-n77A | Rel-17 |  |  |
| DC_13A-66A_n2A | Rel-16 |  |  |
| DC_13A-66A_n77A | Rel-17 |  |  |
| DC_14A-66A_n2A | Rel-16 |  |  |
| DC_14A-66A-66A_n2A | Rel-16 |  |  |
| DC_14A-66A_n66A | Rel-16 |  |  |
| DC_14A-66A_n77A | Rel-17 |  |  |
| DC_14A-66A-66A_n77A | Rel-17 |  |  |
| DC_14A-66A_n77(2A) | Rel-17 |  |  |
| DC_14A-66A-66A_n77(2A) | Rel-18 |  |  |
| DC_18A-41C_n3A | Rel-16 |  |  |
| DC_18A-41A_n77A | Rel-16 |  |  |
| DC_18A-41C_n77A | Rel-16 |  |  |
| DC_18A-41A_n78A | Rel-16 |  |  |
| DC_18A-41C_n78A | Rel-16 |  |  |
| DC_19A_n1A-n78A | Rel-17 |  |  |
| DC_19A_n1A-n79A | Rel-17 |  |  |
| DC_19A-21A_n1A | Rel-17 |  |  |
| DC_19A-21A_n77(2A) | Rel-17 |  |  |
| DC_19A-21A_n78A | Rel-15 |  |  |
| DC_19A-21A_n78(2A) | Rel-17 |  |  |
| DC_19A-21A_n79A | Rel-15 |  |  |
| DC_19A-42A_n1A | Rel-17 |  |  |
| DC_19A-42C_n1A | Rel-17 |  |  |
| DC_19A-42A_n78A | Rel-15 |  |  |
| DC_19A-42A_n79A | Rel-15 |  |  |
| DC_19A-42C_n78A | Rel-15 |  |  |
| DC_19A-42C_n79A | Rel-15 |  |  |
| DC_19A_n78A-n79A | Rel-15 |  |  |
| DC_20A_n28A-n78A | Rel-15 |  |  |
| DC_21A_n1A-n78A | Rel-17 |  |  |
| DC_21A_n1A-n79A | Rel-17 |  |  |
| DC_21A_n28A-n77A | Rel-17 |  |  |
| DC_21A_n28A-n78A | Rel-17 |  |  |
| DC_21A_n28A-n79A | Rel-17 |  |  |
| DC_21A-42A_n1A | Rel-17 |  |  |
| DC_21A-42C_n1A | Rel-17 |  |  |
| DC_21A-42A_n78A | Rel-15 |  |  |
| DC_21A-42C_n78A | Rel-15 |  |  |
| DC_21A-42A_n79A | Rel-15 |  |  |
| DC_21A-42C_n79A | Rel-15 |  |  |
| DC_21A_n78A-n79A | Rel-15 |  |  |
| DC_28A_n7A-n78A | Rel-16 |  |  |
| DC_66A_n2A-n77A | Rel-17 |  |  |
| DC_66A_n5A-n77A | Rel-17 |  |  |
| DC_66A-(n)71AA | Rel-15 |  |  |
| Note 1: Notation used for inter-band EN-DC Bands is according to TS 38.101-3 [25] Table 5.5B.4.2-1, e.g. ‘DC_1A-3C_n78A’ indicates EN-DC operation on E-UTRA CA configuration CA_1A-3C with E-UTRA DL Bandwidth Classes A, C for the E-UTRA bands 1 and 3 respectively and NR band n78 with NR DL CA Bandwidth Class A. Note 2: See UL_nCC(table_index) in Note 2 of Table 4.0-3 in TS 38.522 [9]. Note 3: See DL_nCC(table_index) in Note 4 of Table 4.0-3 in TS 38.522 [9]. Note 4: See UL_NR_nCC(table_index) in Note 3 of Table 4.0-3 in TS 38.522 [9]. Note 5: See DL_NR_nCC(table_index) in Note 5 of Table 4.0-3 in TS 38.522 [9]. |  |  |  |

Table A.4.3.2B.2.3.2-3: Inter-band EN-DC within FR1 (three bands) PC2 UE RF Baseline Implementation Capabilities

| Item | EN-DC configuration | Inter-band EN-DC within FR1 (three bands) PC2 UE RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_1A-3A_n78A | LTE Frequency band 1: 1920-1980 MHz (UL),2110- 2170 MHz (DL) LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band n78: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band3_nrBand78_PC2_Supp |  |
| 2 | DC_1A-3A_n79A | LTE Frequency band 1: 1920-1980 MHz (UL),2110- 2170 MHz (DL) LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band n79: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band3_nrBand79_PC2_Supp |  |
| 3 | DC_1A-19A_n78A | LTE Frequency band 1: 1920-1980 MHz (UL),2110- 2170 MHz (DL) LTE Frequency band 19: 830-845 MHz (UL),875-890 MHz (DL) NR Frequency band n78: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band19_nrBand78_PC2_Supp |  |
| 4 | DC_1A-19A_n79A | LTE Frequency band 1: 1920-1980 MHz (UL),2110- 2170 MHz (DL) LTE Frequency band 19: 830-845 MHz (UL),875-890 MHz (DL) NR Frequency band n79: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band19_nrBand79_PC2_Supp |  |
| 5 | DC_1A-21A_n78A | LTE Frequency band 1: 1920-1980 MHz (UL),2110- 2170 MHz (DL) LTE Frequency band 21: 1447.9-1462.9 MHz (UL), 1495.9-1510.9 MHz (DL) NR Frequency band n78: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band21_nrBand78_PC2_Supp |  |
| 6 | DC_1A-21A_n79A | LTE Frequency band 1: 1920-1980 MHz (UL),2110- 2170 MHz (DL) LTE Frequency band 21: 1447.9-1462.9 MHz (UL), 1495.9-1510.9 MHz (DL) NR Frequency band n79: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band21_nrBand79_PC2_Supp |  |
| 7 | DC_1A-42A_n79A | LTE Frequency band 1: 1920-1980 MHz (UL),2110- 2170 MHz (DL) LTE Frequency band 42: 3400-3600 MHz NR Frequency band n79: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band42_nrBand79_PC2_Supp |  |
| 8 | DC_1A_n78A-n79A | LTE Frequency band 1: 1920-1980 MHz (UL),2110- 2170 MHz (DL) NR Frequency band n78: 3300-3800 MHz NR Frequency band n79: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_nrBand78_nrBand79_PC2_Supp |  |
| 8A | DC_2A_n5A-n77A | LTE frequency band 2: 1850-1910 MHz (UL), 1930-1990 MHz (DL) NR frequency band n5: 824-849 MHz (UL), 869-894 MHz (DL) NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band2_nrBand5_nrBand77_PC2_Supp |  |
| 8B | DC_2A-13A_n77A | LTE frequency band 2: 1850-1910 MHz (UL), 1930-1990 MHz (DL) LTE frequency band 13: 777-787 MHz (UL),746-756 MHz (DL) NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band2_Band13_nrBand77_PC2_Supp |  |
| 8C | DC_2A-66A_n77A | LTE frequency band 2: 1850-1910 MHz (UL), 1930-1990 MHz (DL) LTE frequency band 66: 1710-1780 MHz (UL), 2110-2200 MHz (DL) NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band2_Band66_nrBand77_PC2_Supp |  |
| 9 | DC_3A-19A_n78A | LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) LTE Frequency band 19: 830-845 MHz (UL),875-890 MHz (DL) NR Frequency band n78: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_Band19_nrBand78_PC2_Supp |  |
| 10 | DC_3A-19A_n79A | LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) LTE Frequency band 19: 830-845 MHz (UL),875-890 MHz (DL) NR Frequency band n79: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_Band19_nrBand79_PC2_Supp |  |
| 11 | DC_3A-21A_n78A | LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) LTE Frequency band 21: 1447.9-1462.9 MHz (UL), 1495.9-1510.9 MHz (DL) NR Frequency band n78: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_Band21_nrBand78_PC2_Supp |  |
| 12 | DC_3A-21A_n79A | LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) LTE Frequency band 21: 1447.9-1462.9 MHz (UL), 1495.9-1510.9 MHz (DL) NR Frequency band n79: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_Band21_nrBand79_PC2_Supp |  |
| 13 | DC_3A-42A_n79A | LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) LTE Frequency band 42: 3400-3600 MHz NR Frequency band n79: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_Band42_nrBand79_PC2_Supp |  |
| 14 | DC_3A_n78A-n79A | LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band n78: 3300-3800 MHz NR Frequency band n79: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_nrBand78_nrBand79_PC2_Supp |  |
| 14A | DC_13A_n2A-n77A | LTE frequency band 13: 777-787 MHz (UL),746-756 MHz (DL) NR frequency band n2: 1850-1910 MHz (UL), 1930-1990 MHz (DL) NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band13_nrBand2_nrBand77_PC2_Supp |  |
| 14B | DC_13A-66A_n77A | LTE frequency band 13: 777-787 MHz (UL),746-756 MHz (DL) LTE frequency band 66: 1710-1780 MHz (UL), 2110-2200 MHz (DL) NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band13_Band66_nrBand77_PC2_Supp |  |
| 15 | DC_19A-21A_n78A | LTE Frequency band 19: 830-845 MHz (UL),875-890 MHz (DL) LTE Frequency band 21: 1447.9-1462.9 MHz (UL), 1495.9-1510.9 MHz (DL) NR Frequency band n78: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band19_Band21_nrBand78_PC2_Supp |  |
| 16 | DC_19A-21A_n79A | LTE Frequency band 19: 830-845 MHz (UL),875-890 MHz (DL) LTE Frequency band 21: 1447.9-1462.9 MHz (UL), 1495.9-1510.9 MHz (DL) NR Frequency band n79: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band19_Band21_nrBand79_PC2_Supp |  |
| 17 | DC_19A-42A_n79A | LTE Frequency band 19: 830-845 MHz (UL),875-890 MHz (DL) LTE Frequency band 42: 3400-3600 MHz NR Frequency band n79: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band19_Band42_nrBand79_PC2_Supp |  |
| 18 | DC_19A_n78A-n79A | LTE Frequency band 19: 830-845 MHz (UL),875-890 MHz (DL) NR Frequency band n78: 3300-3800 MHz NR Frequency band n79: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band19_nrBand78_nrBand79_PC2_Supp |  |
| 19 | DC_21A-42A_n79A | LTE Frequency band 21: 1447.9-1462.9 MHz (UL), 1495.9-1510.9 MHz (DL) LTE Frequency band 42: 3400-3600 MHz NR Frequency band n79: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band21_Band42_nrBand79_PC2_Supp |  |
| 20 | DC_21A_n78A-n79A | LTE Frequency band 21: 1447.9-1462.9 MHz (UL), 1495.9-1510.9 MHz (DL) NR Frequency band n78: 3300-3800 MHz NR Frequency band n79: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band21_nrBand78_nrBand79_PC2_Supp |  |
| 21 | DC_66A_n2A-n77A | LTE frequency band 66: 1710-1780 MHz (UL), 2110-2200 MHz (DL) NR frequency band n2: 1850-1910 MHz (UL), 1930-1990 MHz (DL) NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band66_nrBand2_nrBand77_PC2_Supp |  |
| 22 | DC_66A_n5A-n77A | LTE frequency band 66: 1710-1780 MHz (UL), 2110-2200 MHz (DL) NR frequency band n5: 824-849 MHz (UL), 869-894 MHz (DL) NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band66_nrBand5_nrBand77_PC2_Supp |  |
| 23 | DC_1A-3A_n77A | LTE Frequency band 1: 1920-1980 MHz (UL),2110- 2170 MHz (DL) LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band3_nrBand77_PC2_Supp |  |
| 24 | DC_1A-41A_n77A | LTE Frequency band 1: 1920-1980 MHz (UL),2110- 2170 MHz (DL) LTE Frequency band 41: 2496-2690 MHz NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band41_nrBand77_PC2_Supp |  |
| 25 | DC_1A-3A_n41A | LTE Frequency band 1: 1920-1980 MHz (UL),2110- 2170 MHz (DL) LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band n41: 2496-2690 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band3_nrBand41_PC2_Supp |  |
| 26 | DC_1A-18A_n77A | LTE Frequency band 1: 1920-1980 MHz (UL),2110- 2170 MHz (DL) LTE Frequency band 18: 815-830 MHz (UL), 860-875 MHz (DL) NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band18_nrBand77_PC2_Supp |  |
| 27 | DC_3A-18A_n77A | LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) LTE Frequency band 18: 815-830 MHz (UL), 860-875 MHz (DL) NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_Band18_nrBand77_PC2_Supp |  |
| 28 | DC_3A-41A_n77A | LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) LTE Frequency band 41: 2496-2690 MHz NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_Band41_nrBand77_PC2_Supp |  |
| A | DC_1A-11A_n77A | LTE Frequency band 1: 1920-1980 MHz (UL), 2110- 2170 MHz (DL) LTE Frequency band 11: 1427.9-1447.9 MHz (UL), 1475.9- 1495.9 MHz (DL) NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band11_nrBand77_PC2_Supp |  |
| B | DC_1A-41A_n77(2A) | LTE Frequency band 1: 1920-1980 MHz (UL),2110- 2170 MHz (DL) LTE Frequency band 41: 2496-2690 MHz NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band41_nrBand77_PC2_Supp |  |
| C | DC_1A-41C_n77A | LTE Frequency band 1: 1920-1980 MHz (UL),2110- 2170 MHz (DL) LTE Frequency band 41: 2496-2690 MHz NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band41_nrBand77_PC2_Supp |  |
| D | DC_1A-41C_n77(2A) | LTE Frequency band 1: 1920-1980 MHz (UL),2110- 2170 MHz (DL) LTE Frequency band 41: 2496-2690 MHz NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band41_nrBand77_PC2_Supp |  |
| E | DC_1A-42A_n77A | LTE Frequency band 1: 1920-1980 MHz (UL), 2110- 2170 MHz (DL) LTE Frequency band 42: 3400-3600 MHz NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band42_nrBand77_PC2_Supp |  |
| F | DC_1A-42C_n77A | LTE Frequency band 1: 1920-1980 MHz (UL), 2110- 2170 MHz (DL) LTE Frequency band 42: 3400-3600 MHz NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band1_Band42_nrBand77_PC2_Supp |  |
| G | DC_3A-41A_n77(2A) | LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) LTE Frequency band 41: 2496-2690 MHz NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_Band41_nrBand77_PC2_Supp |  |
| H | DC_3A-41C_n77A | LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) LTE Frequency band 41: 2496-2690 MHz NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_Band41_nrBand77_PC2_Supp |  |
| I | DC_3A-41C_n77(2A) | LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) LTE Frequency band 41: 2496-2690 MHz NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_Band41_nrBand77_PC2_Supp |  |
| J | DC_3A-42A_n77A | LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) LTE Frequency band 42: 3400-3600 MHz NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_Band42_nrBand77_PC2_Supp |  |
| K | DC_3A-42C_n77A | LTE Frequency band 3: 1710-1785 MHz (UL), 1805-1880 MHz (DL) LTE Frequency band 42: 3400-3600 MHz NR frequency band n77: 3300-4200 MHz | 38.101-3, 6.2B.1.3 | Rel-18 | pc_Band3_Band42_nrBand77_PC2_Supp |  |

-------- End of changes --------