3GPP TSG-RAN WG5 Meeting #108	R5-254261

# Bengaluru, India, 25th Aug 2025 – 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3547 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to default RF test frequencies for n256 and addition of RF test frequencies to verify RF aspects of variable Tx-Rx spacing for NR NTN bands n255 and n256 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar, Viasat |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 specifications allow configuration UL and DL center frequency independently. Normally RAN4 defines minimum requirements with a single default Tx-Rx separation. Satellite industry has a strong need to use different Tx-Rx spacing in different deployments. Therefore, RAN4 specified variable Tx-Rx spacing for NTN bands. Additionally, for n255 and n256, SCS=30 kHz the test frequencies are incorrect and based on incorrect number of PRBs |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adds RF test frequencies for n256 for tests to be performed with flexible Tx-Rx separation - Corrects the number of PRBs and test frequency parameters for n256 for SCS of 30 kHz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance of variable Tx-Rx spacing during handover cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.9.1.1, new clause 4.3.1.9.1.1a, 4.3.1.9.1.2, new clause 4.3.1.9.1.2a, Annex C.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.9.1	NR NTN operating bands

4.3.1.9.1.1	Reference test frequencies for NR NTN operating band n256

Table 4.3.1.9.1.1-1: Test frequencies for NR operating band n256 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2172.5 | 434500 | 2170.25 | 434050 | 0 | 15 | 5429 | 434410 | 0 | 0 | 0(0) | 0 |
|  |  |  | Mid | 2185 | 437000 | 2164.39 | 432878 | 102 |  | 5464 | 437090 | 0 | 1 | 2(4) | 107 |
|  |  |  | High | 2197.5 | 439500 | 2104.53 | 420906 | 504 |  | 5493 | 439470 | 8 | 1 | 0(0) | 505 |
|  |  | Uplink | Low | 1982.5 | 396500 | 1980.25 | 396050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1902.03 | 380406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2007.5 | 401500 | 2004.17 | 400834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2175 | 435000 | 2170.32 | 434064 | 0 | 15 | 5430 | 434430 | 2 | 0 | 0(0) | 0 |
|  |  |  | Mid | 2185 | 437000 | 2161.96 | 432392 | 102 |  | 5458 | 436610 | 2 | 1 | 2(4) | 107 |
|  |  |  | High | 2195 | 439000 | 2099.6 | 419920 | 504 |  | 5480 | 438490 | 10 | 1 | 0(0) | 505 |
|  |  | Uplink | Low | 1985 | 397000 | 1980.32 | 396064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1899.6 | 379920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2005 | 401000 | 1999.24 | 399848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 2177.5 | 435500 | 2170.39 | 434078 | 0 | 15 | 5431 | 434450 | 4 | 0 | 0(0) | 0 |
|  |  |  | Mid | 2185.0 | 437000 | 2159.53 | 431906 | 102 |  | 5452 | 436130 | 4 | 1 | 2(4) | 107 |
|  |  |  | High | 2192.5 | 438500 | 2094.67 | 418934 | 504 |  | 5470 | 437570 | 8 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1987.5 | 397500 | 1980.39 | 396078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1897.17 | 379434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2002.5 | 400500 | 1994.31 | 398862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 2180 | 436000 | 2170.46 | 434092 | 0 | 15 | 5432 | 434650 | 6 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2185 | 437000 | 2157.1 | 431420 | 102 |  | 5446 | 435650 | 6 | 1 | 2(4) | 107 |
|  |  |  | High | 2190 | 438000 | 2089.74 | 417948 | 504 |  | 5457 | 436590 | 10 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1990 | 398000 | 1980.46 | 396092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1894.74 | 378948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2000 | 400000 | 1989.38 | 397876 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.1-2: Test frequencies for NR operating band n256 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2175 | 435000 | 2170.68 | 435136 | 0 | 15 | 5436 | 434910 | 18 | 0 | 0(5) | 10 |
|  |  |  | Mid | 2185 | 437000 | 2143.96 | 428792 | 102 |  | 5464 | 437090 | 6 | 0 | 3(8) | 220 |
|  |  |  | High | 2195 | 439000 | 2009.24 | 401848 | 504 |  | 5486 | 438970 | 14 | 0 | 1(6) | 1020 |
|  |  | Uplink | Low | 1985 | 397000 | 1980.68 | 396136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1809.24 | 361848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2005 | 401000 | 1998.52 | 399704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 2177.5 | 435500 | 2170.66 | 434132 | 0 | 15 | 5443 | 434930 | 2 | 0 | 1(6) | 12 |
|  |  |  | Mid | 2185 | 437000 | 2141.44 | 428288 | 102 |  | 545455 | 436370 | 6 | 0 | 0(5) | 214 |
|  |  |  | High | 2192.5 | 438500 | 2004.22 | 400844 | 504 |  | 5476 | 438050 | 18 | 0 | 2(7) | 1022 |
|  |  | Uplink | Low | 1987.5 | 397500 | 1980.66 | 396132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1806.72 | 361344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2002.5 | 400500 | 1993.5 | 398700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 2180 | 436000 | 2170.82 | 434164 | 0 | 15 | 5435 | 434890 | 2 | 0 | 0(5) | 10 |
|  |  |  | Mid | 2185 | 437000 | 2139.1 | 427820 | 102 |  | 5446 | 435890 | 2 | 0 | 0(5) | 214 |
|  |  |  | High | 2190 | 438000 | 1999.38 | 399876 | 504 |  | 5463 | 437070 | 14 | 0 | 2(7) | 1022 |
|  |  | Uplink | Low | 1990 | 398000 | 1980.82 | 396164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1804.38 | 427820 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2000 | 400000 | 1988.66 | 397732 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.1-3: Test frequencies for NR operating band n256 and SCS 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2175 | 435000 | 2171.04 | 434208 | 0 | 15 | - | 434568 |
|  |  |  | Mid | 2185 | 437000 | 2107.6 | 421520 | 102 |  | - | 436568 |
|  |  |  | High | 2195 | 439000 | 1828.16 | 365632 | 504 |  | - | 438568 |
|  |  | Uplink | Low | 1985 | 397000 | 1981.04 | 396208 | 0 | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1628.16 | 325632 | 504 |  | - | - |
|  |  |  | High | 2005 | 401000 | 1996.72 | 399344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2177.5 | 435500 | 2171.02 | 434204 | 0 | 15 | - | 434564 |
|  |  |  | Mid | 2185 | 437000 | 2105.08 | 421016 | 102 |  | - | 436064 |
|  |  |  | High | 2192.5 | 438500 | 1823.14 | 364628 | 504 |  | - | 437564 |
|  |  | Uplink | Low | 1987.5 | 397500 | 1981.02 | 396204 | 0 | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1625.64 | 325128 | 504 |  | - | - |
|  |  |  | High | 2002.5 | 400500 | 1991.7 | 398340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 2180 | 436000 | 2171.36 | 434272 | 0 | 15 | - | 434632 |
|  |  |  | Mid | 2185 | 437000 | 2102.92 | 420584 | 102 |  | - | 435632 |
|  |  |  | High | 2190 | 438000 | 1818.48 | 363696 | 504 |  | - | 436632 |
|  |  | Uplink | Low | 1990 | 398000 | 1981.36 | 396272 | 0 | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1623.48 | 324696 | 504 |  | - | - |
|  |  |  | High | 2000 | 400000 | 1987.04 | 397408 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.9.1.1a	Reference test frequencies for NR NTN operating band n256 with flexible Tx-Rx separation

Table 4.3.1.9.1.1a-1: Test frequencies for NR operating band n256 and SCS 15 kHz with flexible Tx-Rx separation

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2172.5 | 434500 | 2170.25 | 434050 | 0 | 15 | 5429 | 434410 | 0 | 0 | 0(0) | 0 |
|  |  |  | Mid | 2185 | 437000 | 2164.39 | 432878 | 102 |  | 5464 | 437090 | 0 | 1 | 2(4) | 107 |
|  |  |  | High | 2197.5 | 439500 | 2104.53 | 420906 | 504 |  | 5493 | 439470 | 8 | 1 | 0(0) | 505 |
|  |  | Uplink | Low | 2007.5 | 401500 | 2005.25 | 401050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1996 | 399200 | 1903.03 | 380606 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1982.5 | 396500 | 1979.17 | 395834 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 2180 | 436000 | 2170.46 | 434092 | 0 | 15 | 5432 | 434650 | 6 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2185 | 437000 | 2157.1 | 431420 | 102 |  | 5446 | 435650 | 6 | 1 | 2(4) | 107 |
|  |  |  | High | 2190 | 438000 | 2089.74 | 417948 | 504 |  | 5457 | 436590 | 10 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 2000 | 400000 | 1990.46 | 398092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1996 | 399200 | 1895.74 | 379148 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1990 | 398000 | 1979.38 | 395876 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.1a-2: Test frequencies for NR operating band n256 and SCS 30 kHz with flexible Tx-Rx separation

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 51 | Downlink | Low | 2180 | 436000 | 2170.82 | 434164 | 0 | 15 | 5435 | 434890 | 2 | 0 | 0(5) | 10 |
|  |  |  | Mid | 2185 | 437000 | 2139.1 | 427820 | 102 |  | 5446 | 435890 | 2 | 0 | 0(5) | 214 |
|  |  |  | High | 2190 | 438000 | 1999.38 | 399876 | 504 |  | 5463 | 437070 | 14 | 0 | 2(7) | 1022 |
|  |  | Uplink | Low | 2000 | 398000 | 1990.82 | 398164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1996 | 399200 | 1805.38 | 361076 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1990 | 400000 | 1978.66 | 395732 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.1a-3: Test frequencies for NR operating band n256 and SCS 60 kHz with flexible Tx-Rx separation

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 24 | Downlink | Low | 2180 | 436000 | 2171.36 | 434272 | 0 | 15 | - | 434632 |
|  |  |  | Mid | 2185 | 437000 | 2102.92 | 420584 | 102 |  | - | 435632 |
|  |  |  | High | 2190 | 438000 | 1818.48 | 363696 | 504 |  | - | 436632 |
|  |  | Uplink | Low | 2000 | 400000 | 1991.36 | 434272 | 0 | - | - | - |
|  |  |  | Mid | 1996 | 399200 | 1624.48 | 324896 | 504 |  | - | - |
|  |  |  | High | 1990 | 398000 | 1977.04 | 395408 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.9.1.2	Reference test frequencies for NR NTN operating band n255

<< NEXT CHANGE >>

4.3.1.9.1.2	Reference test frequencies for NR NTN operating band n255

Table 4.3.1.9.1.2-1: Test frequencies for NR operating band n255 with SCS 15 kHz and SSB SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1527.5 | 305500 | 1525.25 | 305050 | 0 | 15 | 3818 | 305530 | 4 | 1 | 1(2) | 3 |
|  |  |  | Mid | 1542 | 308400 | 1521.39 | 304278 | 102 |  | 3855 | 308430 | 4 | 1 | 1(2) | 105 |
|  |  |  | High | 1556.5 | 311300 | 1463.53 | 292706 | 504 |  | 3892 | 311330 | 4 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1629 | 325800 | 1626.75 | 325350 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1550.53 | 310106 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1658 | 331600 | 1654.67 | 330934 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 1530 | 306000 | 1525.32 | 305064 | 0 | 15 | 3819 | 305550 | 6 | 1 | 1(2) | 3 |
|  |  |  | Mid | 1542 | 308400 | 1518.96 | 303792 | 102 |  | 3849 | 307950 | 6 | 1 | 1(2) | 105 |
|  |  |  | High | 1554 | 310800 | 1458.6 | 291720 | 504 |  | 3879 | 310350 | 6 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1631.5 | 326300 | 1626.82 | 325364 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1548.1 | 309620 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1655.5 | 331100 | 1649.74 | 329948 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1532.5 | 306500 | 1525.39 | 305078 | 0 | 15 | 3820 | 305570 | 8 | 1 | 1(2) | 3 |
|  |  |  | Mid | 1542 | 308400 | 1516.53 | 303306 | 102 |  | 3843 | 307470 | 8 | 1 | 1(2) | 105 |
|  |  |  | High | 1551.5 | 310300 | 1453.67 | 290734 | 504 |  | 3866 | 309370 | 8 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1634 | 326800 | 1626.89 | 325378 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1545.67 | 309134 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1653 | 330600 | 1644.81 | 328962 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1535 | 307000 | 1525.46 | 305092 | 0 | 15 | 3818 | 305530 | 2 | 0 | 1(2) | 2 |
|  |  |  | Mid | 1542 | 308400 | 1514.1 | 302820 | 102 |  | 3837 | 306990 | 10 | 1 | 1(2) | 105 |
|  |  |  | High | 1549 | 309800 | 1448.74 | 289748 | 504 |  | 3856 | 308450 | 6 | 1 | 2(4) | 509 |
|  |  | Uplink | Low | 1636.5 | 327300 | 1626.96 | 325392 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1543.24 | 308648 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1650.5 | 330100 | 1639.88 | 327976 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.2-2: Test frequencies for NR operating band n255 with SCS 30 kHz and SSB SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1530 | 306000 | 1525.68 | 325436 | 0 | 15 | 3825 | 306030 | 10 | 0 | 2(7) | 14 |
|  |  |  | Mid | 1542 | 308400 | 1500.96 | 300192 | 102 |  | 3855 | 308430 | 10 | 0 | 2(7) | 218 |
|  |  |  | High | 1554 | 310800 | 1368.24 | 273648 | 504 |  | 3885 | 310830 | 10 | 0 | 2(7) | 1022 |
|  |  | Uplink | Low | 1631.5 | 326300 | 1627.18 | 325436 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1457.74 | 300192 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1655.5 | 331100 | 1649.02 | 273648 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 1532.5 | 306500 | 1525.66 | 305132 | 0 | 15 | 3826 | 306050 | 18 | 0 | 2(7) | 14 |
|  |  |  | Mid | 1542 | 308400 | 1498.44 | 299688 | 102 |  | 3849 | 307950 | 18 | 0 | 2(7) | 218 |
|  |  |  | High | 1551.5 | 310300 | 1363.22 | 272644 | 504 |  | 3872 | 309850 | 18 | 0 | 2(7) | 1022 |
|  |  | Uplink | Low | 1634 | 326800 | 1627.16 | 325432 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1455.22 | 299688 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1653 | 330600 | 1644 | 272644 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 1535 | 307000 | 1525.82 | 305164 | 0 | 15 | 3824 | 306010 | 18 | 0 | 1(6) | 12 |
|  |  |  | Mid | 1542 | 308400 | 1496.1 | 299220 | 102 |  | 3843 | 307470 | 14 | 0 | 2(7) | 218 |
|  |  |  | High | 1549 | 309800 | 1358.38 | 271676 | 504 |  | 3859 | 308690 | 2 | 0 | 0(5) | 1018 |
|  |  | Uplink | Low | 1636.5 | 327300 | 1627.32 | 325464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1452.88 | 299220 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1650.5 | 330100 | 1639.16 | 271676 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.2-3: Test frequencies for NR operating band n255 with SCS 60 kHz and SSB SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1530 | 306000 | 1526.04 | 305208 | 0 | 15 | - | 305568 |
|  |  |  | Mid | 1542 | 308400 | 1464.6 | 292920 | 102 |  | - | 307968 |
|  |  |  | High | 1554 | 310800 | 1187.16 | 237432 | 504 |  | - | 310368 |
|  |  | Uplink | Low | 1631.5 | 326300 | 1627.54 | 325508 | 0 | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1276.66 | 255332 | 504 |  | - | - |
|  |  |  | High | 1655.5 | 331100 | 1647.22 | 329444 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 1532.5 | 306500 | 1526.02 | 305204 | 0 | 15 | - | 305564 |
|  |  |  | Mid | 1542 | 308400 | 1462.08 | 292416 | 102 |  | - | 307464 |
|  |  |  | High | 1551.5 | 310300 | 1182.14 | 236428 | 504 |  | - | 309364 |
|  |  | Uplink | Low | 1634 | 326800 | 1627.52 | 325504 | 0 | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1274.14 | 254828 | 504 |  | - | - |
|  |  |  | High | 1653 | 330600 | 1642.2 | 328440 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 1535 | 307000 | 1526.36 | 305272 | 0 | 15 | - | 305632 |
|  |  |  | Mid | 1542 | 308400 | 1459.92 | 291984 | 102 |  | - | 307032 |
|  |  |  | High | 1549 | 309800 | 1177.48 | 235496 | 504 |  | - | 308432 |
|  |  | Uplink | Low | 1636.5 | 327300 | 1627.86 | 325572 | 0 | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1271.98 | 254396 | 504 |  | - | - |
|  |  |  | High | 1650.5 | 330100 | 1637.54 | 327508 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.9.1.2a	Reference test frequencies for NR NTN operating band n255 with flexible Tx-Rx separation

Table 4.3.1.9.1.2a-1: Test frequencies for NR operating band n255 with SCS 15 kHz and SSB SCS 15 kHz with flexible Tx-Rx separation

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1556.5 | 311300 | 1554.25 | 310850 | 0 | 15 | 3892 | 311330 | 4 | 1 | 1(2) | 3 |
|  |  |  | Mid | 1542 | 308400 | 1521.39 | 304278 | 102 |  | 3855 | 308430 | 4 | 1 | 1(2) | 105 |
|  |  |  | High | 1527.5 | 305500 | 1434.53 | 286906 | 504 |  | 3818 | 305530 | 4 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1629 | 325800 | 1626.75 | 325350 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1642 | 328500 | 1549.53 | 304278 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1658 | 331600 | 1654.67 | 330934 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1549 | 309800 | 1539.46 | 307892 | 0 | 15 | 3856 | 308450 | 6 | 0 | 2(4) | 4 |
|  |  |  | Mid | 1542 | 308400 | 1514.1 | 302820 | 102 |  | 3837 | 306990 | 10 | 1 | 1(2) | 105 |
|  |  |  | High | 1535 | 307000 | 1434.74 | 286948 | 504 |  | 3818 | 305530 | 2 | 0 | 1(2) | 506 |
|  |  | Uplink | Low | 1636.5 | 327300 | 1626.96 | 325392 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1642 | 328500 | 1542.24 | 302820 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1650.5 | 330100 | 1639.88 | 327976 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.2a-2: Test frequencies for NR operating band n255 with SCS 30 kHz and SSB SCS 15 kHz with flexible Tx-Rx separation

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 106 | Downlink | Low | 1549 | 309800 | 1539.82 | 307964 | 0 | 15 | 3859 | 308690 | 2 | 0 | 0(5) | 10 |
|  |  |  | Mid | 1542 | 308400 | 1496.1 | 299220 | 102 |  | 3843 | 307470 | 14 | 0 | 2(7) | 218 |
|  |  |  | High | 1535 | 307000 | 1344.38 | 268876 | 504 |  | 3824 | 306010 | 18 | 0 | 1(6) | 1020 |
|  |  | Uplink | Low | 1636.5 | 327300 | 1627.32 | 325464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1642.5 | 328500 | 1451.88 | 299220 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1650.5 | 330100 | 1639.16 | 327832 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.2a-3: Test frequencies for NR operating band n255 with SCS 60 kHz and SSB SCS 15 kHz with flexible Tx-Rx separation

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 24 | Downlink | Low | 1549 | 309800 | 1540.36 | 308072 | 0 | 15 | - | 308432 |
|  |  |  | Mid | 1542 | 308400 | 1459.92 | 291984 | 102 |  | - | 307032 |
|  |  |  | High | 1535 | 307000 | 1163.48 | 232696 | 504 |  | - | 305632 |
|  |  | Uplink | Low | 1636.5 | 327300 | 1627.86 | 325572 | 0 | - | - | - |
|  |  |  | Mid | 1642.5 | 328500 | 1270.98 | 254196 | 504 |  | - | - |
|  |  |  | High | 1650.5 | 330100 | 1637.54 | 327508 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

<< NEXT CHANGE >>

### C.2.1.1	Determination of test frequencies for Low-, Mid- and High-Range

Downlink:

| FDL_LowRange = Ceil((FDL_Low + CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.1.1-Eq1 |
| --- | --- |
| FDL_MidRange = Round((FDL_Low + BWDL/2) / ΔFRaster) * ΔFRaster | C.2.1.1-Eq2 |
| FDL_HighRange = Floor((FDL_High - CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.1.1-Eq3 |

FDL_LowRange is rounded up and FDL_HighRange is rounded down to obey to the minimum guard band according to clause 5.3.3 of TS 38.101-1 [7] and TS 38.101-2 [8].

Uplink:

| FUL_LowRange = FDL_LowRange - FTx-Rx_separation | C.2.1.1-Eq4 |
| --- | --- |
| FUL_MidRange = FDL_MidRange - FTx-Rx_separation | C.2.1.1-Eq5 |
| FUL_HighRange = FDL_HighRange - FTx-Rx_separation | C.2.1.1-Eq6 |

For those FDD frequency bands with two UL sub-bands, FTx-Rx_separation shall be chosen using the following criteria:

- For Low-Range and Mid-Range, use FTx-Rx_separation for lower UL sub-band.

- For High-Range, use FTx-Rx_separation for higher UL sub-band.

For NTN frequency bands, the test frequencies for flexible Tx-Rx separation tests, FTx-Rx_separation shall be chosen using the following criteria:

- For Low-Range, use smallest FTx-Rx_separation.

- For Mid-Range, use default FTx-Rx_separation - 1 MHz.

- For High-Range, use largest FTx-Rx_separation.

<< END OF CHANGES >>